3GPP TSG RAN meeting #98-e									RP-222736

Electronic Meeting, December 12-16, 2022

## Status Report to TSG

Agenda item:			9.5.1.4

| WI / SI Name | Perf. part: Support of reduced capability NR devices |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_redcap-Perf |  |  |  |  |
| Unique ID | 900262 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220966 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: | Performance part: 12/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: | Performance Part: 100% | Testing part: |  |

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Johan BERGMAN |
|  | Company | Ericsson |
|  | Email | johan.bergman@ericsson.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	RAN4#104-bis-e

170 contributions were submitted to this meeting (for details see agenda item 4.6 in Tdoc list).

RAN4 carried our thee following offline email discussions:

- [104-bis-e][207] NR_redcap_RRM_1

  - Summarized in R4-2217140

- [104-bis-e][208] NR_redcap_RRM_2

  - Summarized in R4-2217141

- [104-bis-e][323] NR_RedCap_Demod

  - Summaized in R4-2217487

RAN4 made the following agreements related to RRM performance requirements.

- Test cases list for HO test cases in FR1 Test Index; Test. 1; Intra-frequency handover from FR1 CD-SSB to FR1 CD-SSB; known target cell for UE (1Rx, 2Rx). 2; Intra-frequency handover from FR1 NCD-SSB to FR1 NCD-SSB; unknown target cell for UE (1Rx, 2Rx). 3; Inter-frequency handover from FR1 CD-SSB to FR1 NCD-SSB; unknown target cell for UE (1Rx, 2Rx). - Test cases list for HO test cases in FR2 Test Index; Test. 1; Intra-frequency handover from FR2 CD-SSB to FR2 CD-SSB; known target cell for UE (2Rx). 2; Intra-frequency handover from FR2 NCD-SSB to FR2 NCD-SSB; unknown target cell for UE (2Rx). - Test cases list for NCD-SSB measurement test cases Test Index; Test. 1; SA event triggered reporting tests without gap under non-DRX (1Rx, 2Rx). 2; SA event triggered reporting tests with per-UE gaps under non-DRX (1Rx, 2Rx). 3; SA event triggered reporting tests without gap under non-DRX with SSB index reading (1Rx, 2Rx). 4; SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading (1Rx, 2Rx). - Value and sign of following list of offsets were agreed: Type of threshold; Threshold [dB]. 1; rsrp-ThresholdSSB,; + 1. 2; msgA-RSRP-ThresholdSSB,; + 1. 3; msgA-RSRP-Threshold; + 1. 4; absThreshSS-BlocksConsolidation; + 1. 5; sdt-RSRP-Threshold; + 1. 6; s-SearchDeltaP-r16; -1. 7; s-SearchDeltaP-Stationary-r17; -1. 8; s-SearchThresholdP-r16; + 1. 9; s-SearchThresholdQ-r16; + 1. 10; s-SearchThresholdP2-r17; + 1. 11; s-SearchThresholdQ2-r17; + 1. 12; Qrxlevmin and Qqualmin; 1 dB. - Following SSB test configuration was agreed: - Intra-frequency measurement Test Index; Test; Test Configuration. 1; SA event triggered reporting tests for NCD-SSB without gap under non-DRX; d. 2; SA event triggered reporting tests for NCD-SSB with per-UE gaps under non-DRX. 3; SA event triggered reporting tests for NCD-SSB without gap under non-DRX with SSB index reading. 4; SA event triggered reporting tests for NCD-SSB with per-UE gaps under non-DRX with SSB index reading. - HO Test Index; Test; Test Configuration. 1; Intra-frequency handover from FR1 CD-SSB to FR1 CD-SSB; known target cell for 1 Rx UE; D by replacing NCD-SSB with CD-SSB. 2; Intra-frequency handover from FR1 NCD-SSB to FR1 NCD-SSB; unknown target cell for 1 Rx UE; d. 3; Inter-frequency handover from FR1 CD-SSB to FR1 NCD-SSB; unknown target cell for 1 Rx UE; D on the neighbour cell carrier. - BWP switching Test Index; Test; Test Configuration. 1; DCI-based and Timer-based Active BWP Switch: NR FR1 DL active BWP switch of Cell with non-DRX in SA (1Rx, 2Rx); c. 2; RRC-based Active BWP Switch: NR FR1 DL active BWP switch of Cell with non-DRX in SA (1Rx, 2Rx); Follow legacy BWP switch configuraitons but 40MHz BWP does not apply. - All test cases except those related to NCD-SSB based handover and measurements planned for phase 2 were agreed.

RAN4 made the following agreements related to UE demodulation requirements (R4-2217429).

- Confirm the agreement in RAN4#104: the requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements - RAN4 sets the final PDSCH/PDCCH/PBCH demodulation requirements according to the simulation results summary R4-2216234. - Set the following configuration for CQI reporting test in static/fading condition for FR1 FDD 15kHz - CSI-RS periodicity and offset: 10/5 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: 10ms - Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests - Set the static channel matrix in the frequency domain as H1RX=11. - Set SNR test point X=3dB lower than 2Rx test case, that is, SNR=5/6dB and SNR=11/12dB. - Define the MIMO correlation matrix for 4x1 ULA high. - Rhigh=1.00000.98830.95420.89990.98831.00000.98830.95420.95420.98831.00000.98830.89990.95420.98831.0000 - Use the following configuration for RI reporting test - FR1 FDD - CQI table 1 - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.1-1 - FR1 TDD - CQI table 1 - CBW/SCS: 20MHz/30kHz - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.2-1 - FR2 - Reuse the requirements - Set the following configuration for RI reporting test for FR1 FDD 15kHz - CSI-RS periodicity and offset: 10/5 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: 10ms

##### 2.4.1.2	RAN4#105

60 contributions were submitted to this meeting (for details see agenda item 6.6 in Tdoc list).

For UE demodulation requirements, RAN4 agreed the Big CR to 38.101-4: Correction of RedCap UE demodulation and CSI reporting requirement (R4-2219117).

RAN4 made the following agreements related to RRM performance requirements.

- Updated test case list in R4-2219829 was approved.

- Phase two and remaining phase 1 test cases were agreed.

#### 2.4.2	Remaining Open issues

None

## 4.	References

RAN4#104-bis-e

170 contributions (for details see agenda item 4.6 in Tdoc list)

RAN4#105

60 contributions (for details see agenda item 6.6 in Tdoc list)