3GPP TSG-RAN WG5 Meeting #97	R5-226366

Toulouse, France, Nov 14th - 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2607 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of Test freqs for R16 pending CADC configs from cl 4.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-1 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the endorsed proposals 1-3 in R5-223337, R16 pending CADC configurations in cl 4.3.1 need to be removed |  |  |  |  |  |  |  |  |
| Summary of change: |  | R16 pending CADC configurations in cl 4.3.1 have been removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The pending configurations may lead to misunderstanding that these configs are of interest to the industry or even ready for validation/certification which is not the fact. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.2.1, 4.3.1.1.3, 4.3.1.2.3.1.2, 4.3.1.4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

4.3.1.1.2	NR inter-band CA configurations in FR1

4.3.1.1.2.1	NR inter-band CA configurations in FR1 (two bands)

Table 4.3.1.1.2.1-1: Inter-band NR CA configurations (FR1, two bands)

| NR CA configuration | Uplink NR CA configuration | NR CA downlink configuration band 1 (Note 3) | NR CA downlink configuration band 2 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | Applicable for protocol testing (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1A | n3A | n1A | n3A | Yes |
| CA_n1A-n8A | CA_n1A-n8A | n1A | n8A | n1A | n8A | Yes |
| CA_n1(2A)-n3A | - | CA_n1(2A) | n3A | n1A | n3A | No |
| CA_n1(2A)-n5A | - | CA_n1(2A) | n5A | - | - | No |
| CA_n1(2A)-n8A | - | CA_n1(2A) | n8A | - | - | No |
| CA_n1A-n77A | - | n1A | n77A | - | - | Yes |
| CA_n1A-n78A | CA_n1A-n78A | n1A | n78A | n1A | n78A | Yes |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1A | CA_n78(2A) | n1A | n78A | No |
| CA_n1A-n78C | CA_n1A-n78A | n1A | CA_n78C | n1A | n78A | No |
| CA_n1(2A)-n78A | - | CA_n1(2A) | n78A | - | - | No |
| CA_n1A-n79A | CA_n1A-n79A | n1A | n79A | n1A | n79A | Yes |
| CA_n2A-n5A | CA_n2A-n5A | n2A | n5A | n2A | n5A | Yes |
| CA_n2A-n48A | CA_n2A-n48A | n2A | n48A | n2A | n48A | Yes |
| CA_n2A-n66A | CA_n2A-n66A | n2A | n66A | n2A | n66A | Yes |
| CA_n2A-n77A | CA_n2A-n77A | n2A | n77A | n2A | n77A | Yes |
| CA_n3A-n5A | CA_n3A-n5A | n3A | n5A | n3A | n5A | Yes |
| CA_n3(2A)-n5A | - | CA_n3(2A) | n5A | - | - | No |
| CA_n3(2A)-n8A | - | CA_n3(2A) | n8A | - | - | No |
| CA_n3A-n41A | CA_n3A-n41A | n3A | n41A | n3A | n41A | Yes |
| CA_n3A-n77A | - | n3A | n77A | - | - | Yes |
| CA_n3A-n78A | CA_n3A-n78A | n3A | n78A | n3A | n78A | Yes |
| CA_n3A-n78(2A) | CA_n3A-n78A | n3A | CA_n78(2A) | n3A | n78A | No |
|  | CA_n78(2A) | n3A | CA_n78(2A) | - | CA_n78(2A) | No |
| CA_n3(2A)-n78A | - | CA_n3(2A) | n78A | - | - | No |
| CA_n5A-n7A | - | n5A | n7A | - | - | Yes |
| CA_n5A-n48A | CA_n5A-n48A | n5A | n48A | n5A | n48A | Yes |
| CA_n5A-n66A | CA_n5A-n66A | n5A | n66A | n5A | n66A | Yes |
| CA_n5A-n78A | - | n5A | n78A | - | - | Yes |
| CA_n5A-n78(2A) | CA_n5A-n78A | n5A | CA_n78(2A) | n5A | n78A | No |
| CA_n7A-n78A | - | n7A | n78A | - | - | Yes |
| CA_n8A-n75A | - | n8A | n75A | - | - | Yes |
| CA_n8A-n78A | CA_n8A-n78A | n8A | n78A | n8A | n78A | Yes |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8A | CA_n78(2A) | n8A | CA_n78(2A) | No |
| CA_n24A-n41A | CA_n24A-n41A | n24A | n41A | n24A | n41A | Yes |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24A | CA_n41(2A) | n24A | n41A | No |
| CA_n24A-n48A | CA_n24A-n48A | n24A | n48A | n24A | n48A | Yes |
| CA_n24A-n48B | CA_n24A-n48A | n24A | CA_n48B | n24A | n48A | No |
| CA_n24A-n48(2A) | CA_n24A-n48A | n24A | CA_n48(2A) | n24A | n48A | No |
| CA_n24A-n77A | CA_n24A-n77A | n24A | n77A | n24A | n77A | Yes |
| CA_n24A-n77C | CA_n24A-n77A | n24A | CA_n77C | n24A | n77A | No |
| CA_n26A-n66A | CA_n26A-n66A | n26A | n66A | n26A | n66A | Yes |
| CA_n26A-n66(2A) | CA_n26A-n66A | n26A | CA_n66(2A) | n26A | n66A | No |
| CA_n26A-n70A | CA_n26A-n70A | n26A | n70A | n26A | n70A | Yes |
| CA_n28A-n41A | CA_n28A-n41A | n28A | n41A | n28A | n41A | Yes |
| CA_n28A-n75A | - | n28A | n75A | - | - | Yes |
| CA_n28A-n78A | CA_n28A-n78A | n28A | n78A | n28A | n78A | Yes |
| CA_n28A-n79A | CA_n28A-n79A | n28A | n79A | n28A | n79A | Yes |
| CA_n29A-n66A | - | n29A | n66A (Note 1) | - | - | Yes |
| CA_n29B-n66B | - | n29A | CA_n66B (Note 1) | - | - | No |
| CA_n29A-n66(2A) | - | n29A | CA_n66(2A) (Note 1) | - | - | No |
| CA_n29A-n70A | - | n29A | n70A (Note 1) | - | - | Yes |
| CA_n29A-n71A | - | n29A | n71A (Note 1) | - | - | Yes |
| CA_n41A-n78A | - | n41A | n78A | - | - | Yes |
| CA_n41A-n79A | CA_n41A-n79A | n41A | n79A | n41A | n79A | Yes |
| CA_n41C-n79A | CA_n41A-n79A | CA_n41C | n79A | n41A | n79A | No |
|  | CA_n41C | CA_n41C | n79A | CA_n41C | - | No |
| CA_n48A-n66A | CA_n48A-n66A | n48A | n66A | n48A | n66A | Yes |
| CA_n48A-n66(2A) | CA_n48A-n66A | n48A | CA_n66(2A) | n48A | n66A | No |
| CA_n48A-n70A | CA_n48A-n70A | n48A | n70A | n48A | n70A | Yes |
| CA_n48A-n71A | CA_n48A-n71A | n48A | n71A | n48A | n71A | Yes |
| CA_n48A-n71(2A) | CA_n48A-n71A | n48A | CA_n71(2A) | n48A | n71A | No |
| CA_n48B-n66A | CA_n48A-n66A | CA_n48B | n66A | n48A | n66A | No |
| CA_n48B-n70A | CA_n48A-n70A | CA_n48B | n70A | n48A | n70A | No |
| CA_n48B-n71A | CA_n48A-n71A | CA_n48B | n71A | n48A | n71A | No |
| CA_n48(2A)-n66A | CA_n48A-n66A | CA_n48(2A) | n66A | n48A | n66A | No |
| CA_n48(2A)-n66(2A) | CA_n48A-n66A | CA_n48(2A) | CA_n66(2A) | n48A | n66A | No |
| CA_n48(2A)-n70A | CA_n48A-n70A | CA_n48(2A) | n70A | n48A | n70A | No |
| CA_n48(2A)-n71A | CA_n48A-n71A | CA_n48(2A) | n71A | n48A | n71A | No |
| CA_n48(2A)-n71(2A) | CA_n48A-n71A | CA_n48(2A) | CA_n71(2A) | n48A | n71A | No |
| CA_n66A-n70A | - | n66A | n70A | - | - | Yes |
| CA_n66B-n70A | - | CA_n66B | n70A | - | - | No |
| CA_n66(2A)-n70A | - | CA_n66(2A) | n70A | - | - | No |
| CA_n66A-n71A | CA_n66A-n71A | n66A | n71A | n66A | n71A | Yes |
| CA_n66A-n71(2A) | CA_n66A-n71A | n66A | CA_n71(2A) | n66A | n71A | No |
| CA_n66B-n71A | CA_n66A-n71A | CA_n66B | n71A | n66A | n71A | No |
| CA_n66(2A)-n71A | CA_n66A-n71A | CA_n66(2A) | n71A | n66A | n71A | No |
| CA_n66(2A)-n71(2A) | CA_n66A-n71A | CA_n66(2A) | CA_n71(2A) | n66A | n71A | No |
| CA_n70A-n71A | CA_n70A-n71A | n70A | n71A | n70A | n71A | Yes |
| CA_n70A-n71(2A) | CA_n70A-n71A | n70A | CA_n71(2A) | n70A | n71A | No |
| CA_n75A-n78A | - | n75A | n78A (Note 1) | - | - | Yes |
| CA_n76A-n78A | - | n76A | n78A (Note 1) | - | - | Yes |
| CA_n77A-n79A | - | n77A | n79A | - | - | Yes |
| CA_n78A-n79A | - | n78A | n79A | - | - | Yes |
| Note 1: This band is used as PCell. Note 2: Protocol testing is limited to NR CA configurations with 2CC. Note 3: PCell is configured on this band unless otherwise stated. |  |  |  |  |  |  |

<<<UNCHANGED PARTS SKIPPED >>>

4.3.1.1.3	NR intra-band contiguous CA in FR1

4.3.1.1.3.1 – 4.3.1.1.3.39	FFS

4.3.1.1.3.40	NR Intra-band contiguous configurations CA_n40

4.3.1.1.3.40.1	CA_n40B

Editor's note:	CA_n40B Test frequencies are FFS.

Table 4.3.1.1.3.40.1-1: NR Intra-Band contiguous CA configuration CA_n40B (PCC=CC1 and SCC=CC2), CC1 SCS = 15kHz, CC2 SCS = 15kHz

| CBW combination [MHz] | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Table 4.3.1.1.3.40.1-2: NR Intra-Band contiguous CA configuration CA_n40B (PCC=CC1 and SCC=CC2), CC1 SCS = 30kHz, CC2 SCS = 30kHz

| CBW combination [MHz] | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.3.1.1.3.41	NR Intra-band contiguous configurations CA_n41

4.3.1.1.3.41.1	CA_n41C

Editor's note:	Test frequencies for CA_n41C with mixed numerology with SCS CC1=15 kHz and SCS CC2=30 kHz or 60 kHz; and SCS CC1=30 kHz and SCS CC2= 30 kHz or 60kHz is FFS.

Table 4.3.1.1.3.41.1-1: NR Intra-Band contiguous CA configuration CA_n41C (PCC=CC1 and SCC=CC2), CC1 SCS = 30kHz, CC2 SCS = 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40+80 | CC1 | 40 | 30 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 |  | 6252 | 500190 |  | 4 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2497.2 | 499440 | 102 |  |  | 6345 | 507630 |  | 18 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2590.17 | 518034 | 2389.65 | 477930 | 504 |  |  | 6438 | 515070 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=59.82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2575.83 | 515166 | 2536.77 | 507354 | 0 | 30 |  | 6351 | 508110 |  | 12 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2612.82 | 522564 | 2537.04 | 507408 | 102 |  |  | 6444 | 515550 |  | 2 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 40+100 | CC1 | 40 | 30 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 |  | 6252 | 500190 |  | 4 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2487.21 | 497442 | 102 |  |  | 6321 | 505710 |  | 20 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2570.28 | 514056 | 2369.76 | 473952 | 504 |  |  | 6387 | 510990 |  | 10 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=69.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2585.73 | 517146 | 2536.59 | 507318 | 0 | 30 |  | 6351 | 508110 |  | 0 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2612.73 | 522546 | 2526.87 | 505374 | 102 |  |  | 6420 | 513630 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0 (0) | 1008 |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 |  | 6252 | 500190 |  | 18 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2502.33 | 500466 | 102 |  |  | 6357 | 508590 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2605.02 | 521004 | 2399.64 | 479928 | 504 |  |  | 6462 | 516990 |  | 18 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2575.98 | 515196 | 2546.82 | 509364 | 0 | 30 |  | 6378 | 510270 |  | 14 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2617.95 | 523590 | 2552.07 | 510414 | 102 |  |  | 6483 | 518670 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 |  | 6252 | 500190 |  | 18 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2492.34 | 498468 | 102 |  |  | 6333 | 506670 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2585.13 | 517026 | 2379.75 | 475950 | 504 |  |  | 6414 | 513150 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2585.88 | 517176 | 2546.82 | 509364 | 0 | 30 |  | 6378 | 510270 |  | 14 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2617.86 | 523572 | 2542.08 | 508416 | 102 |  |  | 6456 | 516510 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 |  | 6252 | 500190 |  | 18 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2482.35 | 496470 | 102 |  |  | 6309 | 504750 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2565.24 | 513048 | 2359.86 | 471972 | 504 |  |  | 6363 | 509070 |  | 6 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2595.78 | 519156 | 2546.64 | 509328 | 0 | 30 |  | 6378 | 510270 |  | 2 |  | 0 | 3(3) | 6 |
|  |  |  |  |  | & | Mid | 2617.77 | 523554 | 2531.91 | 506382 | 102 |  |  | 6432 | 514590 |  | 0 |  | 0 | 2(2) | 208 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0(0) | 1008 |
| 60+50 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2568 | 513600 | 2502.12 | 500424 | 102 |  |  | 6357 | 508590 |  | 10 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2610.03 | 522006 | 2399.43 | 479886 | 504 |  |  | 6462 | 516990 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2580.96 | 516192 | 2557.02 | 511404 | 0 | 30 |  | 6402 | 512190 |  | 22 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2622.96 | 524592 | 2562.3 | 512460 | 102 |  |  | 6507 | 520590 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  |  | 6612 | 528990 |  | 20 |  | 0 | 0 (0) | 1008 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2497.11 | 499422 | 102 |  |  | 6345 | 507630 |  | 0 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2599.98 | 519996 | 2389.38 | 477876 | 504 |  |  | 6438 | 515070 |  | 14 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2586 | 517200 | 2556.84 | 511368 | 0 | 30 |  | 6402 | 512190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2622.99 | 524598 | 2557.11 | 511422 | 102 |  |  | 6495 | 519630 |  | 0 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2487.12 | 497424 | 102 |  |  | 6321 | 505710 |  | 2 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2580.09 | 516018 | 2369.49 | 473898 | 504 |  |  | 6387 | 510990 |  | 4 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2595.9 | 519180 | 2556.84 | 511368 | 0 | 30 |  | 6402 | 512190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2622.9 | 524580 | 2547.12 | 509424 | 102 |  |  | 6471 | 517710 |  | 2 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2477.13 | 495426 | 102 |  |  | 6294 | 503550 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2560.2 | 512040 | 2349.6 | 469920 | 504 |  |  | 6339 | 507150 |  | 2 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2605.8 | 521160 | 2556.66 | 511332 | 0 | 30 |  | 6402 | 512190 |  | 22 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2622.81 | 524562 | 2536.95 | 507390 | 102 |  |  | 6444 | 515550 |  | 8 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0 (0) | 1008 |
| 80+40 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2573.01 | 514602 | 2497.23 | 499446 | 102 |  |  | 6345 | 507630 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2610.18 | 522036 | 2389.68 | 477936 | 504 |  |  | 6438 | 515070 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=59.82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 2595.84 | 519168 | 2576.76 | 515352 | 0 | 30 |  | 6453 | 516270 |  | 18 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.83 | 526566 | 2577.03 | 515406 | 102 |  |  | 6543 | 523470 |  | 0 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  |  | 6636 | 530910 |  | 2 |  | 0 | 0 (0) | 1008 |
| 80+50 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2568 | 513600 | 2492.22 | 498444 | 102 |  |  | 6333 | 506670 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2600.13 | 520026 | 2379.63 | 475926 | 504 |  |  | 6414 | 513150 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2600.88 | 520176 | 2576.94 | 515388 | 0 | 30 |  | 6453 | 516270 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.86 | 526572 | 2572.2 | 514440 | 102 |  |  | 6531 | 522510 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  |  | 6612 | 528990 |  | 20 |  | 0 | 0 (0) | 1008 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2487.21 | 497442 | 102 |  |  | 6321 | 505710 |  | 20 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2590.08 | 518016 | 2369.58 | 473916 | 504 |  |  | 6387 | 510990 |  | 22 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2605.92 | 521184 | 2576.76 | 515352 | 0 | 30 |  | 6453 | 516270 |  | 18 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.89 | 526578 | 2567.01 | 513402 | 102 |  |  | 6519 | 521550 |  | 4 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2477.22 | 495444 | 102 |  |  | 6294 | 503550 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2570.01 | 514002 | 2349.51 | 469902 | 504 |  |  | 6336 | 506910 |  | 0 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=79.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2616 | 523200 | 2576.94 | 515388 | 0 | 30 |  | 6453 | 516270 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.98 | 526596 | 2557.2 | 511440 | 102 |  |  | 6495 | 519630 |  | 18 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2467.23 | 493446 | 102 |  |  | 6270 | 501630 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2550.12 | 510024 | 2329.62 | 465924 | 504 |  |  | 6288 | 503070 |  | 22 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2625.9 | 525180 | 2576.76 | 515352 | 0 | 30 |  | 6453 | 516270 |  | 18 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.89 | 526578 | 2547.03 | 509406 | 102 |  |  | 6468 | 517470 |  | 0 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0 (0) | 1008 |
| 100+40 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2573.01 | 514602 | 2487.15 | 497430 | 102 |  |  | 6321 | 505710 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2600.28 | 520056 | 2369.7 | 473940 | 504 |  |  | 6387 | 510990 |  | 14 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=69.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 2615.73 | 523146 | 2596.65 | 519330 | 0 | 30 |  | 6501 | 520110 |  | 20 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.73 | 528546 | 2586.93 | 517386 | 102 |  |  | 6570 | 525630 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  |  | 6636 | 530910 |  | 2 |  | 0 | 0 (0) | 1008 |
| 100+50 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2568 | 513600 | 2482.14 | 496428 | 102 |  |  | 6306 | 504510 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2590.23 | 518046 | 2359.65 | 471930 | 504 |  |  | 6363 | 509070 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2620.77 | 524154 | 2596.83 | 519366 | 0 | 30 |  | 6501 | 520110 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.76 | 528552 | 2582.1 | 516420 | 102 |  |  | 6558 | 524670 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  |  | 6612 | 528990 |  | 20 |  | 0 | 0 (0) | 1008 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2477.13 | 495426 | 102 |  |  | 6294 | 503550 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2580.18 | 516036 | 2349.6 | 469920 | 504 |  |  | 6339 | 507150 |  | 2 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2625.81 | 525162 | 2596.65 | 519330 | 0 | 30 |  | 6501 | 520110 |  | 20 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.79 | 528558 | 2576.91 | 515382 | 102 |  |  | 6543 | 523470 |  | 8 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2467.14 | 493428 | 102 |  |  | 6270 | 501630 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2560.11 | 512022 | 2329.53 | 465906 | 504 |  |  | 6288 | 503070 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2635.89 | 527178 | 2596.83 | 519366 | 0 | 30 |  | 6501 | 520110 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.88 | 528576 | 2567.1 | 513420 | 102 |  |  | 6519 | 521550 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<<UNCHANGED PARTS SKIPPED >>>

4.3.1.2.3.1.2	CA_n257C

Editor’s note:	CA_n257C Test frequencies are FFS.

Table 4.3.1.2.3.1.2-1: NR Intra-Band contiguous CA configuration CA_n257C, SCS=120 kHz, ΔFRaster 120 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

<<<UNCHANGED PARTS SKIPPED >>>

###### 4.3.1.4.1.5	Inter-band EN-DC configurations within FR1 (five bands)

Table 4.3.1.4.1.5-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC Configuration | E-UTRA downlink configuration | NR downlink configuration | E-UTRA uplink configuration | NR uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A-7A-20A_n28A | DC_1A_n28A | CA_1A-3A-7A-20A | n28A | 1A | n28A | No |
|  | DC_3A_n28A | CA_1A-3A-7A-20A | n28A | 3A | n28A | No |
|  | DC_7A_n28A | CA_1A-3A-7A-20A | n28A | 7A | n28A | No |
|  | DC_20A_n28A | CA_1A-3A-7A-20A | n28A | 20A | n28A | No |
| DC_1A-3A-7A-20A_n78A | DC_1A_n78A | CA_1A-3A-7A-20A | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3A-7A-20A | n78A | 3A | n78A | No |
|  | DC_7A_n78A | CA_1A-3A-7A-20A | n78A | 7A | n78A | No |
|  | DC_20A_n78A | CA_1A-3A-7A-20A | n78A | 20A | n78A | No |
| DC_1A-3A-7A-28A_n78A | DC_1A_n78A | CA_1A-3A-7A-28A | n78A | 1A | n78A | No |
|  | DC_3A_n78A | CA_1A-3A-7A-28A | n78A | 3A | n78A | No |
|  | DC_7A_n78A | CA_1A-3A-7A-28A | n78A | 7A | n78A | No |
|  | DC_28A_n78A | CA_1A-3A-7A-28A | n78A | 28A | n78A | No |
| DC_1A-3A-7A_n28A-n78A | DC_1A_n28A | CA_1A-3A-7A | CA_n28A-n78A | 1A | n28A | No |
|  | DC_1A_n78A | CA_1A-3A-7A | CA_n28A-n78A | 1A | n78A | No |
|  | DC_3A_n28A | CA_1A-3A-7A | CA_n28A-n78A | 3A | n28A | No |
|  | DC_3A_n78A | CA_1A-3A-7A | CA_n28A-n78A | 3A | n78A | No |
|  | DC_7A_n28A | CA_1A-3A-7A | CA_n28A-n78A | 7A | n28A | No |
|  | DC_7A_n78A | CA_1A-3A-7A | CA_n28A-n78A | 7A | n78A | No |
| <<<UNCHANGED PARTS SKIPPED >>> |  |  |  |  |  |  |

<< END OF CHANGES >>