3GPP TSG-RAN5 Meeting #97	R5-227475

# Toulouse, France, 14th – 18th November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0418 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MAC implementation capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The applicability for some Direct SCell activation test cases reqires capabilites for: Direct SCell activation for MCG and MSC inactive and SCG but they are missing from the table of MAC Implementation Capabilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New capabilites pc_directMCG_SCellActivation_r17, pc_directMCG_SCellActivationResume_r17, pc_directSCG_SCellActivationResume_r17, were added to Table A.4.3.5-1: UE MAC Implementation Capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability for the Direct SCell activation test cases will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### A.4.3.5	MAC Implementation Capabilities

Table A.4.3.5-1: UE MAC Implementation Capabilities

| Item |  | UE MAC Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | M |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Support long DRX cycle |  | 38.306, 4.2.6 |  | Rel-15 |  | pc_longDRX_Cycle |  | Yes |  |  |  |  |  |
| 2 |  | Support short DRX cycle |  | 38.306, 4.2.6 |  | Rel-15 |  | pc_shortDRX_Cycle |  | Yes |  |  |  |  |  |
| 3 |  | Support skipping of UL transmission for an uplink grant indicated on PDCCH if no data is available for transmission |  | 38.306, 4.2.6 |  | Rel-15 |  | pc_skipUplinkTxDynamic |  | No |  |  |  |  |  |
| 4 |  | Supports the logicalChannelSR-DelayTimer |  | 38.306, 4.2.6 |  | Rel-15 |  | pc_logicalChannelSR_DelayTimer |  | No |  |  |  |  |  |
| 5 |  | Supports DRX adaptation |  | 38.306, 4.2.6 |  | Rel-16 |  | pc_DRX_Adaptation |  | No |  |  |  |  |  |
| 6 |  | Support LCH-based prioritization |  | 38.306, 4.2.6 |  | Rel-16 |  | pc_lch_PriorityBasedPrioritization_r16 |  | No |  |  |  |  |  |
| 7 |  | Supports autonomous transmission of the MAC PDU generated for a deprioritized configured uplink grant |  | 38.306, 4.2.6 |  | Rel-16 |  | pc_autonomousTransmission_r16 |  | No |  |  |  |  |  |
| 8 |  | Supports the bit rate recommendation message from the gNB to the UE as specified in TS 38.321 |  | 38.306, 4.2.6 |  | Rel-15 |  | pc_recommendedBitRate |  | No |  |  |  |  |  |
| 9 |  | Supports the bit rate recommendation query message from the UE to the gNB as specified in TS 38.321. |  | 38.306, 4.2.6 |  | Rel-15 |  | pc_recommendedBitRateQuery |  | No |  |  |  | This field is only applicable if the UE supports pc_recommendedBitRate. |  |
| 10 |  | Support PUSCH transmissions on multiple configured uplink grants |  | 38.306, 4.2.6 |  | Rel-16 |  | pc_multipleConfiguredGrants_r16 |  | No |  |  |  |  |  |
| 11 |  | Support the selection of logical channels for each UL grant based on RRC configured restriction |  | 38.306, 4.2.6 |  | Rel-15 |  | pc_lcp_Restriction |  | No |  |  |  |  |  |
| 12 |  | Support direct NR SCG SCell activation, as specified in TS 38.321, upon SCell addition and upon reconfiguration with sync of the SCG, both performed via an RRCReconfiguration message received via SRB3 or contained in an RRC(Connection)Reconfiguration message received via SRB1, as specified in TS 38.331 and TS 36.331 |  | 38.306, 4.2.6 |  | Rel-16 |  | pc_directSCG_SCellActivation_r16 |  | No |  |  |  | A UE indicating support of directSCG-SCellActivation-r16 shall indicate support of EN-DC or support of NGEN-DC as specified in TS 36.331 or support of NR-DC as specified in TS 38.331. |  |
| NN |  | Support direct NR MCG SCell activation, as specified in TS 38.321, upon SCell addition, upon reconfiguration with sync of the MCG, as specified in TS 38.331. |  | 38.306, 4.2.6 |  | Rel-17 |  | pc_directMCG_SCellActivation_r17 |  | No |  |  |  |  |  |
| RR |  | Support direct NR MCG SCell activation, as specified in TS 38.321, upon reception of an RRCResume message, as specified in TS 38.331. |  | 38.306, 4.2.6 |  | Rel-17 |  | pc_directMCG_SCellActivationResume_r17 |  | No |  |  |  |  |  |
| UU |  | Support direct NR SCG SCell activation, as specified in TS 38.321: - upon reception of an RRCReconfiguration included in an RRCConnectionResume message, as specified in TS 38.331 and TS 36.331, if the UE indicates support of EN-DC or NGEN-DC, and support of resumeWithSCG-Config-r16 as specified in TS 36.331, - upon reception of an RRCReconfiguration included in an RRCResume message, as specified in TS 38.331, if the UE indicates support of NR-DC and of resumeWithSCG-Config-r16 as specified in TS 38.331. |  | 38.306, 4.2.6 |  | Rel-17 |  | pc_directSCG_SCellActivationResume_r17 |  | No |  |  |  | A UE indicating support of directSCG-SCellActivationResume-r16 shall indicate support of EN-DC or NGEN-DC and support of resumeWithSCG-Config-r16 as specified in TS 36.331 or indicate support of NR-DC and of resumeWithSCG-Config-r16 as specified in TS 38.331. |  |