3GPP TSG-RAN5 Meeting #97	R5-226936

# Toulouse , France , 14th – 18th Nov. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0231 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding applicability for new test cases for SUL with UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_UL_MIMO_PC3_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New test cases for SUL with UL MIMO are introduced. Test case applicability need to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating table 4.1.1 for new introduced test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test applicability is absent for many test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title |  | Release |  | Applicability |  |  |  | Tested Bands/CA-Configurations Selection |  | Branch |  | Additional Information |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Condition |  | Comment |  |  |  |  |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  | PC1 PC2 PC3 |  |  |  |
| 6.2.2 | UE maximum output power reduction |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  | PC1 PC2 PC3 |  | Test execution is not necessary if TS 38.521-1 TC 6.5.2.4.1 is executed. Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.3 or 6.5B.2.3.3.1 has been executed. |  |
| 6.2.3 | UE additional maximum output power reduction |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  | PC1 PC2 PC3 |  | Test execution is not necessary if TS 38.521-1 TC 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed. Skip TC 6.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.3 has been executed or TS 38.521-3 TCs 6.5B.2.3.2, 6.5B.2.3.3.2 and 6.5B.4.3 have been executed. |  |
| 6.2.4 | Configured transmitted power |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  | PC1 PC2 PC3 |  |  |  |
| 6.2A.1.1 | UE maximum output power for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 E015a |  | Inter-band CA: PC2, PC3 |  |  |  |
| 6.2A.2.1 | UE maximum output power reduction for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  | Inter-band CA: PC3 Intra-band contiguous CA: PC2, PC3 Intra-band non-contiguous CA: PC3 |  | Test execution is not necessary if TS 38.521-1 TC 6.5A.2.4.1.1 is executed. |  |
| 6.2A.3.1 | UE additional maximum output power reduction for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  | Inter-band CA: PC3 |  | Test execution is not necessary if TS 38.521-1 TC 6.5A.2.3 and 6.5A.3.3 are executed. |  |
| 6.2A.4.1 | Configured transmitted power for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  | Inter-band CA: PC3 Intra-band contiguous CA: PC2, PC3 |  |  |  |
| 6.2C.1 | Configured transmitted power for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.2C.3 | UE maximum output power for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.2C.4 | UE maximum output power reduction for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  | Test execution is not necessary if TS 38.521-1 TC 6.5C.2.4.1 is executed. |  |
| 6.2C.5 | UE additional maximum output power reduction for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.2D.1 | UE maximum output power for UL MIMO |  | Rel-15 |  | N/A |  | Not recommended due to no test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4. |  |  |  |  |  | Maximum Output Power for UL MIMO is tested as part of the MPR test case with using MPR=1.5dB suggested by RAN4. |  |
| 6.2D.1_1 | UE maximum output power for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 6.2D.2 | UE maximum output power reduction for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  | PC2 PC3 |  | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.4.1 is executed. |  |
| 6.2D.3 | UE additional maximum output power reduction for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  | Test execution is not necessary if TS 38.521-1 TC 6.5D.2.3 and 6.5D.3.3 are executed. |  |
| 6.2D.4 | Configured transmitted power for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.2D.4_1 | Configured transmitted power for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 6.2E.2.1 | UE maximum output power reduction for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  | Test execution is not necessary if TS 38.521-1 TC 6.5E.2.4.1 is executed. |  |
| 6.2E.2.2 | UE maximum output power reduction for V2X / concurrent operation |  | FFS |  | FFS |  | FFS |  | FFS |  |  |  |  |  |
| 6.2F.1 | UE maximum output power for shared spectrum channel access |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 6.2F.3 | UE additional maximum output power reduction for shared spectrum access |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 6.2G.1 | UE maximum output power for Tx Diversity |  | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.2G.2 | UE maximum output power reduction for Tx Diversity |  | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.2G.3 | UE additional maximum output power reduction |  | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and t supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.2I.1 | Maximum output power for RedCap |  | Rel-17 |  | C177 |  | RedCap UEs supporting 5GS FR1 |  | D018 |  |  |  | NOTE 1 |  |
| 6.2I.2 | UE maximum output power reduction for RedCap |  | Rel-17 |  | C177 |  | RedCap UEs supporting 5GS FR1 |  | D018 |  |  |  | NOTE 1 |  |
| 6.2I.3 | UE additional maximum output power reduction for RedCap |  | Rel-17 |  | C177 |  | RedCap UEs supporting 5GS FR1 |  | D018 |  |  |  | NOTE 1 |  |
| 6.2I.4 | Configured output power for RedCap |  | Rel-17 |  | C177 |  | RedCap UEs supporting 5GS FR1 |  | D018 |  |  |  | NOTE 1 |  |
| 6.3.1 | Minimum output power |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  |  |  |
| 6.3.3.2 | General ON/OFF time mask |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.3.3.2 if UE supports NSA and TS 38.521-3 TC 6.3B.3.1 or 6.3B.3.2 or 6.3B.3.3 has been executed. |  |
| 6.3.3.4 | PRACH time mask |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.3.3.4 if UE supports NSA and TS 38.521-3 TC 6.3B.4.1 or 6.3B.4.2 or 6.3B.4.3 has been executed. |  |
| 6.3.3.6 | SRS time mask |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  |  |  |
| 6.3.4.2 | Absolute power tolerance |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  |  |  |
| 6.3.4.3 | Relative power tolerance |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  |  |  |
| 6.3.4.4 | Aggregate power tolerance |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  |  |  |
| 6.3A.1.1 | Minimum output power for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  |  |  |
| 6.3A.3.1 | Transmit ON/OFF time mask for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  |  |  |
| 6.3A.3.1_1 | Time mask for switching between two uplink carriers |  | Rel-16 |  | C051 |  | UEs supporting 5GS FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching |  | E019 |  |  |  |  |  |
| 6.3A.4.1 | Absolute power tolerance for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  |  |  |
| 6.3A.4.2 | Power control relative power tolerance for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  |  |  |
| 6.3A.4.3 | Aggregate power tolerance for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  |  |  |
| 6.3C.1 | Minimum output power for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.3C.3.1 | General transmit ON/OFF time mask for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.3C.3.2 | General transmit ON/OFF time mask for switching between two uplink carriers |  | Rel-16 |  | C178 |  | UEs supporting 5GS FR1 and SUL and dynamic UL Tx switching |  | D003 |  |  |  |  |  |
| 6.3C.4.1 | Absolute power tolerance for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.3C.4.2 | Relative power tolerance for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.3C.4.3 | Aggregate power tolerance for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.3D.1 | Minimum output power for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.3D.1_1 | Minimum output power for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 6.3D.3 | Transmit ON/OFF time mask for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.3D.3_1 | Transmit ON/OFF time mask for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 6.3D.4.1 | Absolute power tolerance for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.3D.4.2 | Relative power tolerance for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.3D.4.3 | Aggregate power tolerance for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.3E.1.1 | Minimum output power for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| 6.3E.1.1D | Minimum output power for V2X / non-concurrent operation / SL-MIMO |  | Rel-16 |  | C100 |  | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO |  | D016 |  |  |  | NOTE 1 |  |
| 6.3G.1 | Minimum output power for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.3G.2 | Transmit OFF power for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.3G.3.1 | General ON/OFF time mask for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.3G.3.2 | PRACH time mask for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.3G.3.3 | SRS time mask for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.3G.4.2 | Relative power tolerance for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.3G.4.3 | Aggregate power tolerance for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.4.1 | Frequency error |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.4.1 if UE supports NSA and TS 38.521-3 TC 6.4B.1.1 or 6.4B.1.2 or 6.4B.1.3 has been executed. |  |
| 6.4.2.1 | Error vector magnitude |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.4.2.1 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.1 or 6.4B.2.2.1 or 6.4B.2.3.1 has been executed. |  |
| 6.4.2.1a | Error Vector Magnitude including symbols with transient period |  | Rel-16 |  | C156 |  | UEs supporting 5GS FR1 AND Band supporting enhancedUL-TransientPeriod and not supporting txDiversity-r16 |  | D001 |  |  |  | NOTE 1 |  |
| 6.4.2.2 | Carrier leakage |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.4.2.2 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.2 or 6.4B.2.2.2 or 6.4B.2.3.2 has been executed. |  |
| 6.4.2.3 | In-band emissions |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.4.2.3 if UE supports NSA and TS 38.521-3 TC 6.4B.2.2.3 or 6.4B.2.3.3 has been executed. |  |
| 6.4.2.4 | EVM equalizer spectrum flatness |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.4.2.4 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.4 or 6.4B.2.2.4 or 6.4B.2.3.4 has been executed. |  |
| 6.4.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK |  | Rel-15 |  | C050 |  | UEs supporting 5GS FR1 Power Class 3 and pi/2-BPSK modulation scheme for power boosting in FR1and not supporting txDiversity-r16 |  | D017 |  |  |  |  |  |
|  |  |  | Rel-16 |  | C111 |  | UEs supporting 5GS FR1 and pi/2-BPSK modulation scheme and low PAPR DMRSand not supporting txDiversity-r16 |  | D001 |  |  |  |  |  |
| 6.4A.1.1 | Frequency error for CA (2UL CA) |  | Rel-15 |  | FFS |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | FFS |  |  |  | NOTE 1 |  |
| 6.4A.2.1.1 | Error vector magnitude for CA (2UL CA) |  | Rel-15 |  | FFS |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | FFS |  |  |  | NOTE 1 |  |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) |  | Rel-15 |  | FFS |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | FFS |  |  |  | NOTE 1 |  |
| 6.4A.2.3.1 | In-band emissions for CA (2UL CA) |  | Rel-15 |  | FFS |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | FFS |  |  |  | NOTE 1 |  |
| 6.4C.1 | Frequency error for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.4C.2.1 | Error vector magnitude for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.4C.2.2 | Carrier leakage for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.4C.2.3 | In-band emissions for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.4C.2.4 | EVM equalizer spectrum flatness for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.4C.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK for SUL |  | Rel-16 |  | C112 |  | UEs supporting 5GS FR1 and SUL and pi/2-BPSK modulation scheme and low PAPR DMRS |  | D003 |  |  |  |  |  |
| 6.4D.1 | Frequency error for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.4D.2.1 | Error vector magnitude for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.4D.2.2 | Carrier leakage for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.4D.2.3 | In-band emissions for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.4D.2.4 | EVM equalizer spectrum flatness for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.4D.3 | Time alignment error for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.4D.4 | Requirements for coherent UL MIMO |  | FFS |  | FFS |  | FFS |  | FFS |  |  |  | NOTE 1 |  |
| 6.4E.2.2.1 | Error Vector Magnitude for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  | NOTE 1 |  |
| 6.4E.2.2.1D | Error Vector Magnitude for V2X / non-concurrent operation / SL-MIMO |  | Rel-16 |  | C100 |  | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO |  | D016 |  |  |  | NOTE 1 |  |
| 6.4E.2.4.1 | In-band emissions for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  | NOTE 1 |  |
| 6.4E.2.4.1D | In-band emissions for V2X / non-concurrent operation / SL-MIMO |  | Rel-16 |  | C100 |  | UEs supporting 5GS FR1 and NR sidelink and SL-MIMO |  | D016 |  |  |  | NOTE 1 |  |
| 6.4G.1 | Frequency Error for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.4G.2.2 | Carrier Leakage for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.4G.2.3 | In-band emissions for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.4G.2.4 | EVM equalizer spectrum flatness for Tx Diversity | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 6.5.1 | Occupied bandwidth |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.5.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.2 or 6.5B.1.3 has been executed. |  |
| 6.5.2.2 | Spectrum emission mask |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  | PC1 PC2 PC3 |  | Skip TC 6.5.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.2.1 or 6.5B.2.3.1 has been executed. |  |
| 6.5.2.3 | Additional spectrum emission mask |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  | PC1 PC2 PC3 |  | NOTE 1 Skip TC 6.5.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.2 has been executed. |  |
| 6.5.2.4.1 | NR ACLR |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  | PC1 PC2 PC3 |  | Skip TC 6.5.2.4.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.3.1 has been executed. |  |
| 6.5.2.4.2 | UTRA ACLR |  | Rel-15 |  | C001a |  | UEs supporting 5GS FR1 PC3 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.5.2.4.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.3.2 has been executed. |  |
| 6.5.3.1 | General spurious emissions |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.5.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.1.1 or 6.5B.3.2.1 has been executed. |  |
| 6.5.3.2 | Spurious emissions for UE co-existence |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  |  |  |
| 6.5.3.3 | Additional spurious emissions |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.5.3.3 if UE supports NSA and TS 38.521-3 TC 6.5B.4.3 has been executed. |  |
| 6.5.4 | Transmit intermodulation |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  |  |  | Skip TC 6.5.4 if UE supports NSA and TS 38.521-3 TC 6.5B.5.3 has been executed. |  |
| 6.5A.1.1 | Occupied bandwidth for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  | NOTE 1 |  |
| 6.5A.2.2.1 | Spectrum emission mask for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  | Inter-band CA: PC3 Intra-band contiguous CA: PC2, PC3 |  |  |  |
| 6.5A.2.4.1.1 | NR ACLR for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  | NOTE 1 |  |
| 6.5A.2.4.2.1 | UTRA ACLR for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  | NOTE 1 |  |
| 6.5A.3.1.1 | General spurious emissions for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  | NOTE 1 |  |
| 6.5A.3.2.1 | Spurious emissions for UE co-existence for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  | NOTE 1 |  |
| 6.5A.4.1 | Transmit intermodulation for CA (2UL CA) |  | Rel-15 |  | C004 |  | UEs supporting 5GS FR1 and CA (2UL CA) |  | E015 |  |  |  | NOTE 1 |  |
| 6.5C.1 | Occupied bandwidth for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.5C.2.2 | Spectrum emission mask for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.5C.2.3 | Additional spectrum emission mask for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.5C.2.4.1 | NR ACLR for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.5C.2.4.2 | UTRA ACLR for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.5C.3.1 | General spurious emissions for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.5C.3.2 | Spurious emissions for UE co-existence for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.5C.3.3 | Additional spurious emissions for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.5C.4 | Transmit intermodulation for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 6.5D.1 | Occupied bandwidth for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.2.2 | Spectrum Emission Mask for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.2.3 | Additional spectrum emission mask for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.2.4.1 | NR ACLR for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.2.4.2 | UTRA ACLR for UL MIMO |  | Rel-15y |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.3.1 | General spurious emissions for UL MIMO |  | Rel-15 only |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.3.2 | Spurious emissions for UE co-existence for UL MIMO |  | Rel-15 only |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.3.3 | Additional spurious emissions for UL MIMO |  | Rel-15 only |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.3_1.1 | General spurious emissions for UL MIMO (Rel-16 onward) |  | Rel-16 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.3_1.2 | Spurious emission for UE co-existence for UL MIMO (Rel-16 onward) |  | Rel-16 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.3_1.3 | Additional spurious emissions for UL MIMO(Rel-16 onward) |  | Rel-16 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5D.4 | Transmit intermodulation for UL MIMO |  | Rel-15 |  | C003 |  | UEs supporting 5GS FR1 and UL MIMO |  | D022 |  |  |  |  |  |
| 6.5E.2.2.1 | Spectrum emission mask for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| 6.5E.2.3.1 | Additional Spectrum emission mask for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  | NOTE 1 |  |
| 6.5E.2.4.1 | Adjacent channel leakage ratio for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  |  |  |
| 6.5E.3.2.1 | Spurious emissions for UE co-existence for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  | NOTE 1 |  |
| 6.5E.3.3.1 | Additional spurious emissions requirements for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  | NOTE 1 |  |
| 6.5F.2.2 | Spectrum emission mask for operation with shared spectrum channel access |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 6.5F.2.4 | Adjacent channel leakage ratio for operation with shared spectrum channel access |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 6.5F.3.1 | General spurious emissions |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 6.5G.2.3 | Adjacent channel leakage ratio |  | Rel-15 |  | C001g |  | UEs supporting 5GS FR1 and supporting txDiversity-r16 |  | D001 |  | PC1.5 PC2 PC3 |  |  |  |
| 7 | Receiver Characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.2 | Reference sensitivity power level |  | Rel-15 |  | C001h |  | UEs supporting 5GS FR1 not supporting txDiversity-r16 |  | D001 |  | 2Rx 4Rx |  |  |  |
| 7.3A.1 | Reference sensitivity power level for 2DL CA without exception |  | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 |  |  |  |  |  |
| 7.3A.1_1 | Reference sensitivity power level for 2DL CA exceptions |  | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 E015a |  |  |  |  |  |
| 7.3A.2 | Reference sensitivity level for CA (3DL CA) |  | FFS |  | FFS |  | FFS |  | FFS |  |  |  | NOTE 1 |  |
| 7.3A.3 | Reference sensitivity level for CA (4DL CA) |  | FFS |  | FFS |  | FFS |  | FFS |  |  |  | NOTE 1 |  |
| 7.3C.2 | Reference sensitivity power level for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 7.3D.2 | Reference sensitivity power level for UL MIMO |  | Rel-15 |  | C003a |  | UEs supporting 5GS FDD FR1 and UL MIMO |  | D012 |  |  |  |  |  |
| 7.3D.2_1 | Reference sensitivity power level for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 7.3E.2 | Reference sensitivity for V2X / non-concurrent operation |  | Rel-16 |  | C079 |  | UEs supporting 5GS FR1 and NR sidelink |  | D016 |  |  |  | NOTE 1 |  |
| 7.3F.2 | Reference sensitivity for shared spectrum channel access |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 7.4 | Maximum input level |  | Rel-15 |  | C001 |  | UEs supporting 5GS FR1 |  | D001 |  |  |  | Skip TC 7.4 if UE supports NSA and TS 38.521-3 TC 7.4B.3 or 7.4B.4 has been executed. |  |
| 7.4A.1 | Maximum input level for CA (2DL CA) |  | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 |  |  |  |  |  |
| 7.4A.2 | Maximum input level for CA (3DL CA) |  | Rel-16 |  | C033 |  | UEs supporting 5GS FR1 and CA (3DL CA) |  | E017 |  |  |  |  |  |
| 7.4A.3 | Maximum input level for CA (4DL CA) |  | FFS |  | FFS |  | FFS |  | FFS |  |  |  | NOTE 1 |  |
| 7.4D | Maximum input level for UL MIMO |  | Rel-15 |  | C003a |  | UEs supporting 5GS FDD FR1 and UL MIMO |  | D012 |  |  |  |  |  |
| 7.4D_1 | Maximum input level for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 7.5 | Adjacent channel selectivity |  | Rel-15 |  | C001 |  | UEs supporting 5GS FR1 |  | D001 |  |  |  | NOTE 1 Skip TC 7.5 if UE supports NSA and TS 38.521-3 TC 7.5B.2 or 7.5B.3 has been executed. |  |
| 7.5A.1 | Adjacent channel selectivity for CA (2DL CA) |  | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 |  |  |  |  |  |
| 7.5A.2 | Adjacent channel selectivity for 3DL CA |  | FFS |  | FFS |  | FFS |  | FFS |  |  |  | NOTE 1 |  |
| 7.5A.3 | Adjacent channel selectivity for 4DL CA |  | FFS |  | FFS |  | FFS |  | FFS |  |  |  | NOTE 1 |  |
| 7.5D | Adjacent channel selectivity for UL MIMO |  | Rel-15 |  | C003a |  | UEs supporting 5GS FDD FR1 and UL MIMO |  | D012 |  |  |  |  |  |
| 7.5D_1 | Adjacent channel selectivity for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 7.5F.1 | Adjacent channel selectivity for shared spectrum channel access |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 7.6.2 | In-band blocking |  | Rel-15 |  | C001 |  | UEs supporting 5GS FR1 |  | D001 |  |  |  | Skip TC 7.6.2 if UE supports NSA and TS 38.521-3 TC 7.6B.2.2 or 7.6B.2.3 has been executed. |  |
| 7.6.3 | Out-of-band blocking |  | Rel-15 |  | C001 |  | UEs supporting 5GS FR1 |  | D001 |  |  |  |  |  |
| 7.6.4 | Narrow band blocking |  | Rel-15 |  | C001 |  | UEs supporting 5GS FR1 |  | D004 |  |  |  | Skip TC 7.6.4 if UE supports NSA and TS 38.521-3 TC 7.6B.4.2 or 7.6B.4.3 has been executed. |  |
| 7.6A.2.1 | In-band blocking for CA (2DL CA) |  | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 |  |  |  |  |  |
| 7.6A.2.2 | In-band blocking for CA (3DL CA) |  | Rel-16 |  | C033 |  | UEs supporting 5GS FR1 and CA (3DL CA) |  | E017 |  |  |  |  |  |
| 7.6A.2.3 | In-band blocking for CA (4DL CA) |  | Rel-16 |  | C036 |  | UEs supporting 5GS FR1 and CA (4DL CA) |  | E018 |  |  |  | NOTE 1 Skip TC 7.6A.2.3 if UE supports NSA and TS 38.521-3 TC 7.6B.2.3_1.3 has been executed. |  |
| 7.6A.3.1 | Out-of-band blocking for CA (2DL CA) |  | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 |  |  |  |  |  |
| 7.6A.3.2 | Out-of-band blocking for CA (3DL CA) |  | Rel-16 |  | C033 |  | UEs supporting 5GS FR1 and CA (3DL CA) |  | E017 |  |  |  |  |  |
| 7.6A.3.3 | Out-of-band blocking for CA (4DL CA) |  | Rel-16 |  | C036 |  | UEs supporting 5GS FR1 and CA (4DL CA) |  | E018 |  |  |  |  |  |
| 7.6A.4.1 | Narrow band blocking for CA (2DL CA) |  | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 |  |  |  |  |  |
| 7.6A.4.2 | Narrow band blocking for CA (3DL CA) |  | Rel-16 |  | C033 |  | UEs supporting 5GS FR1 and CA (3DL CA) |  | E017 |  |  |  |  |  |
| 7.6A.4.3 | Narrow band blocking for CA (4DL CA) |  | Rel-16 |  | C036 |  | UEs supporting 5GS FR1 and CA (4DL CA) |  | E018 |  |  |  | Skip TC 7.6A.4.3 if UE supports NSA and TS 38.521-3 TC 7.6B.4.3_1.3 has been executed. |  |
| 7.6C.2 | In-band blocking for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 7.6C.2_1 | Inband Blocking for SUL with DL CA |  | Rel-17 |  | C078 |  | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) |  | E031 |  |  |  |  |  |
| 7.6C.3 | Out-of-band blocking for SUL |  | Rel-15 |  | C002 |  | UEs supporting 5GS FR1 and SUL |  | D003 |  |  |  |  |  |
| 7.6C.3_1 | Out-of-band blocking for SUL with DL CA |  | Rel-17 |  | C078 |  | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) |  | E031 |  |  |  |  |  |
| 7.6D.2 | In-band blocking for UL MIMO |  | Rel-15 |  | C003a |  | UEs supporting 5GS FDD FR1 and UL MIMO |  | D012 |  |  |  |  |  |
| 7.6D.2_1 | In-band blocking for UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 7.6D.3 | Out-of-band blocking for UL MIMO |  | Rel-15 |  | C003a |  | UEs supporting 5GS FDD FR1 and UL MIMO |  | D012 |  |  |  |  |  |
| 7.6D.3_1 | Out-of-band blocking for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 7.6D.4 | Narrow band blocking for UL MIMO |  | Rel-15 |  | C003a |  | UEs supporting 5GS FDD FR1 and UL MIMO |  | D012 |  |  |  |  |  |
| 7.6D.4_1 | Narrow band blocking for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 7.6F.2.1 | In-band blocking for shared spectrum channel access |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 7.6F.3.1 | Out-of-band blocking for shared spectrum channel access |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 7.7 | Spurious response |  | Rel-15 |  | C001 |  | UEs supporting 5GS FR1 |  | D001 |  |  |  |  |  |
| 7.7A.1 | Spurious response for CA (2DL CA) |  | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 |  |  |  |  |  |
| 7.7A.2 | Spurious response for CA (3DL CA) |  | Rel-16 |  | FFS |  | UEs supporting 5GS FR1 and CA (3DL CA) |  | FFS |  |  |  | NOTE 1 |  |
| 7.7A.3 | Spurious response for CA (4DL CA) |  | Rel-16 |  | FFS |  | UEs supporting 5GS FR1 and CA (4DL CA) |  | FFS |  |  |  | NOTE 1 |  |
| 7.7D | Spurious response for UL MIMO |  | Rel-15 |  | C003a |  | UEs supporting 5GS FDD FR1 and UL MIMO |  | D012 |  |  |  |  |  |
| 7.7D_1 | Spurious response for SUL with UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 7.7F.1 | Spurious response for shared spectrum channel access |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 7.8.2 | Wide band Intermodulation |  | Rel-15 |  | C001 |  | UEs supporting 5GS FR1 |  | D001 |  |  |  | Skip TC 7.8.2 if UE supports NSA and TS 38.521-3 TC 7.8B.2.2 or 7.8B.2.3 has been executed. |  |
| 7.8A.2.1 | Wide band Intermodulation for CA (2DL CA) |  | Rel-15 |  | C031 |  | UEs supporting 5GS FR1 and CA (2DL CA) |  | E016 |  |  |  |  |  |
| 7.8A.2.2 | Wide band Intermodulation for CA (3DL CA) |  | Rel-16 |  | C033 |  | UEs supporting 5GS FR1 and CA (3DL CA) |  | E017 |  |  |  |  |  |
| 7.8A.2.3 | Wide band Intermodulation for CA (4DL CA) |  | Rel-16 |  | C036 |  | UEs supporting 5GS FR1 and CA (4DL CA) |  | E018 |  |  |  |  |  |
| 7.8D.2 | Wide band Intermodulation for UL MIMO |  | Rel-15 |  | C003a |  | UEs supporting 5GS FDD FR1 and UL MIMO |  | D012 |  |  |  |  |  |
| 7.8D.2_1 | Wide band Intermodulation for UL MIMO |  | Rel-17 |  | C179 |  | UEs supporting 5GS FR1 and SUL and UL MIMO |  | D024 |  |  |  |  |  |
| 7.8F.2 | Wide band Intermodulation |  | Rel-16 |  | C001c |  | UEs supporting 5GS FR1 and operation with shared spectrum channel access |  | D018 |  |  |  |  |  |
| 7.9 | Spurious emissions |  | Rel-15 |  | C001 |  | UEs supporting 5GS FR1 |  | D001 |  |  |  | Skip TC 7.9 if UE supports NSA and TS 38.521-3 TC 7.9B.1 or 7.9B.2 or 7.9B.3 has been executed. |  |
| 7.9A.1 | Spurious emissions for CA (2DL CA) |  | Rel-15 |  | C005 |  | UEs supporting 5GS FR1 and inter-band 2DL CA with a DL-only band |  | E002 |  |  |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.1.1-1a: Void

Table 4.1.1-1b: Void

Table 4.1.1-1c: Void

<Unchanged Skipped>

<End of Changes>