3GPP TSG-RAN5 Meeting #97	R5-227727

# Toulouse, France,  14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0719 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of CA_n48A-n77A and CA_n71A-n77A test point analyses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DISH Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CA_n48A-n77A and CA_n71A-n77A are ongoing in PRD21, but missing from this section of specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added analyses information of CA_n48A-n77A and CA_n71A-77A into reference sensitivity TP analysis table and CA_n71A-n77A into UL CA spurious emissions table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TP analyses for CA_n48A-n77A and CA_n71A-n77A conformance requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Number of TPs corrected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-227279 |  |  |  |  |  |  |  |  |

### 4.1.3	Test point analysis per NR CA configuration

#### 4.1.3.1	Reference Sensitivity test cases for FR1 NR CA

Editor's note:

-	Not all CA configurations completed in TS 38.521-1 refsens test cases are listed in Table 4.1.3.1-1 through Table 4.1.3.1-5.

This clause contains information on test point analysis, test frequency selection and status for FR1 NR CA test cases in TS 38.521-1 [2] clause 7. The analyses are performed per CA configuration in Table 4.1.3.1-1 through Table 4.1.3.1 4. The principles for selection of reference sensitivity test points for NR CA is given in Annex B including a test point analysis check list in B.9.

Table 4.1.3.1-1: Reference Sensitivity test cases per CA configuration (single band)

| CA config | Single band refsens exception, victim band (Note 2) | Requirement coverage (Note 2) | Test point analysis updated |
| --- | --- | --- | --- |
| CA_n41C |  | NE | RAN5#89-e |
| CA_n48B |  | NE | RAN5#94-e |
| CA_n66B |  | NE | RAN5#86-e |
| CA_n66(2A) |  | NE | RAN5#86-e |
| CA_n71(2A) |  | NE | RAN5#94-e |
| CA_n78C |  | NE | RAN5#87-e |
| NOTE 1: Void. NOTE 2: Notations used: NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 |  |  |  |

Table 4.1.3.1-2: Reference Sensitivity test cases per CA configuration (two bands)

| CA config |  | Two band refsens exception, victim band (Note 1) |  | Requirement coverage (Note 1) |  | Test point analysis updated |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  | n1 (IMD3) n3(CBI) |  | IMD,CBI |  | RAN5#95-e |  |
| CA_n1A-n8A |  | n1 (IMD4) |  | IMD |  | RAN5#95-e |  |
| CA_n1A-n77A |  | n77 (HD2) |  | HD |  | RAN5#89-e |  |
| CA_n1A-n78A |  | n1 (IMD4) |  | IMD |  | RAN5#94 |  |
| CA_n2A-n48A |  | n48 (HD2), n2(IMD4) |  | HD, IMD |  | RAN5#96-e |  |
| CA_n2A-n66A |  | n2(IMD3), n66(IMD5) |  | IMD |  | RAN5#96-e |  |
| CA_n2A-n77A |  | n77 (HD2), n2 (HM), n2(IMD2), n2(IMD4), n2(IMD5), n2(IMD7) |  | HD, HM, IMD |  | RAN5#96-e |  |
| CA_n3A-n5A |  | n3 (IMD4 ¦2*fn3-2*fn5¦) n5 (IMD2 ¦fn3-fn5¦) |  | IMD |  | RAN5#95-e |  |
| CA_n3A-n77A |  | n78 (HD2) |  | HD |  | RAN5#84 |  |
| CA_n3A-n78A |  | n3 (IMD2 ¦fn78-fn3¦) n3 (IMD4 ¦fn78-3*fn3¦) n78 (HD2) |  | HD, IMD |  | RAN5#87-e |  |
| CA_n5A-n66A |  | n5(IMD2) |  | IMD |  | RAN5#96-e |  |
| CA_n5A-n77A |  | n77 (HD4), n77 (HD5), n5 (HM), n5(IMD4), n5(IMD5) |  | HD, HM,, IMD |  | RAN5#96-e |  |
| CA_n5A-n78A |  | n78 (HD2) |  | HD |  | RAN5#94-e |  |
| CA_n7A-n78A |  | n7 and n78 (CBI) |  | CBI |  | RAN5#94-e |  |
| CA_n8A-n78A |  | n8 (IMD4 ¦fn78-3*fn8¦) n78 (HD4) |  | HD, IMD |  | RAN5#87-e |  |
| CA_n26A-n66A |  | n26 (IMD2) |  | IMD |  | RAN5#94-e |  |
| CA_n26A-n70A |  | n26 (IMD2) |  | IMD |  | RAN5#94-e |  |
| CA_n28A-n41A |  |  |  | NE |  | RAN5#91-e |  |
| CA_n28A-n79A |  |  |  | NE |  | RAN5#91-e |  |
| CA_n29A-n66A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n29A-n70A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n29A-n71A |  | n29 (CBI) |  | CBI |  | RAN5#95-e |  |
| CA_n41A-n79A |  |  |  | NE |  | RAN5#83 |  |
| CA_n48A-n66A |  | n48 (HD2), n66 (IMD5) |  | HD, IMD |  | RAN5#94-e |  |
| CA_n48A-n66(2A) |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48A-n70A |  | n70 (IMD2) |  | IMD |  | RAN5#94-e |  |
| CA_n48A-n71A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n48A-n71(2A) |  | - |  | NE |  | RAN5#95-e |  |
| CA_n48A-n77A |  | - |  | NE |  | RAN5#97-e |  |
| CA_n48B-n66A |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48B-n70A |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48B-n71A |  | - |  | NE |  | RAN5#95-e |  |
| CA_n48(2A)-n66A |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48(2A)-n66(2A) |  | (NOTE 2) |  | NE |  | RAN5#96-e |  |
| CA_n48(2A)-n70A |  | (NOTE 2) |  | NE |  | RAN5#95-e |  |
| CA_n48(2A)-n71A |  | - |  | NE |  | RAN5#95-e |  |
| CA_n48(2A)-n71(2A) |  | - |  | NE |  | RAN5#96-e |  |
| CA_n66A-n70A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n66A-n71A |  | n66 (IMD4) |  | IMD |  | RAN5#87-e |  |
| CA_n70A-n71A |  | n70 (HD3), n70 (IMD4) |  | HD, IMD |  | RAN5#85 |  |
| CA_n71A-n77A |  | n71 (IMD5) |  | IMD |  | RAN5#97-e |  |
| NOTE 1: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 NOTE 2: Exception for this band is tested with two carrier case |  |  |  |  |  |  |  |

Table 4.1.3.1-3: Reference Sensitivity test cases per CA configuration (three bands)

| CA config |  | Three band refsens exception, victim band (Note 1) |  | Requirement coverage (Note 1) |  | Test point analysis updated |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26A-n66A-n70A |  |  |  | NE |  | RAN5#94-e |  |
| CA_n26A-n66(2A)-n70A |  |  |  | NE |  | RAN5#96-e |  |
| CA_n29A-n66A-n70A |  |  |  | NE |  | RAN5#89-e |  |
| CA_n48A-n66A-n71(2A) |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48A-n66(2A)-n70A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48A-n66(2A)-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48A-n70A-n71(2A) |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48B-n66A-n70A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48B-n66A-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48B-n70A-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48(2A)-n66A-n70A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48(2A)-n66A-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n48(2A)-n70A-n71A |  | - |  | NE |  | RAN5#96-e |  |
| CA_n66A-n70A-n71A |  |  |  | NE |  | RAN5#87-e |  |
| NOTE 1: Notations used NE: Non-exception as defined in TS 38.101-1 [5], meaning single carrier NR requirements apply. HD: UL Harmonic Distortion, as defined in TS 38.101-1 [5], 7.3A.4 HM: RX Harmonic Mixing, as defined in TS 38.101-1 [5], 7.3A.4 IMD: Intermodulation Distortion, as defined in TS 38.101-1 [5], 7.3A.5 CBI: Cross Band Isolation, as defined in TS 38.101-1 [5], 7.3A.6 |  |  |  |  |  |  |  |

Table 4.1.3.1-4: Reference Sensitivity test cases per CA configuration (four bands)

| CA config | Four band refsens exception, victim band (Note 1) | Requirement coverage (Note 1) | Test point analysis updated |
| --- | --- | --- | --- |
| TBD |  | TBD | TBD |
| NOTE 1: No exceptions can occur for four bands. |  |  |  |

Table 4.1.3.1-5: Reference Sensitivity test cases per CA configuration (five bands)

| CA config | Five band refsens exception, victim band (Note 1) | Requirement coverage (Note 1) | Test point analysis updated |
| --- | --- | --- | --- |
| TBD |  | TBD | TBD |
| NOTE 1: No exceptions can occur for five bands. |  |  |  |

Table 4.1.3.1-6: Test frequency selection per band pair for UL harmonics exception

| Band pair (Note 2) |  | Frequency |  | Channel BW [MHz] |  | Comment |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1A / n77A |  | Mid / Low |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | Mid / 3900 |  | 20/ Highest |  | Exception Note 2 of TS 38.101 table 7.3A.4-1 |  |  |
|  |  | Mid / 3870 |  | 20/20 |  | Exception Note 3 of TS 38.101 table 7.3A.4-1 |  |  |
| n2A/n48A |  | UL 1860 MHz / DL 3700 MHz |  | 20/20 |  | Exception Note 3 of TS 38.521 Table 7.3A.0.4-1 |  |  |
|  |  | UL 1860 MHz / DL 3400 MHz |  | 20/100 |  | HD avoid |  |  |
| n2A/n77A |  | UL 1860 MHz / DL 3720 MHz |  | 20/100 |  | Exception Note 2 of TS 38.101 Table 7.3A.4-1 |  |  |
|  |  | UL 1860 MHz / DL 3700 MHz |  | 20/20 |  | Exception Note 3 of TS 38.521 Table 7.3A.0.4-1 |  |  |
|  |  | UL 1860 MHz / DL 3800 MHz |  | 20/100 |  | HD avoid |  |  |
| n3A / n77A |  | TBD/TBD |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | TBD/TBD |  | Highest/Highest |  | Exception Note 2 of TS38.101 table 7.3A.4-1 |  |  |
|  |  | TBD/TBD |  | 20/20 |  | Exception Note 3 of TS38.101 table 7.3A.4-1 |  |  |
| n3A / n78A |  | Mid/High |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | Mid/3495 MHz |  | Highest/Highest |  | Exception Note 2 of TS38.101 table 7.3A.4-1 |  |  |
|  |  | Mid/3465 MHz |  | 20/20 |  | Exception Note 3 of TS38.101 table 7.3A.4-1 |  |  |
| n5A/n77A |  | UL 834 MHz / DL 3336 MHz |  | 20/100 |  | Exception Note 4 of TS 38.101-1 table 7.3A.4-1 |  |  |
|  |  | UL 834 MHz / DL 4170 MHz |  | 20/100 |  | Exception Note 5 TS 38.101-1 table 7.3A.4-1 |  |  |
|  |  | UL 834 MHz/DL 3800 MHz |  | 20/100 |  | HD Avoid |  |  |
| n5A / n78A |  | Mid/High |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 in TS 38.521-1 [2] |  |  |
|  |  | Mid/3346 MHz |  | Highest/Highest |  | Exception Note 4 of TS 38.101 [5] table 7.3A.4-1 |  |  |
|  |  | Mid/3316 MHz |  | 20/20 |  | Exception Note 5 of TS 38.101 [5] table 7.3A.4-1 |  |  |
| n8A / n78A |  | Mid/High |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | Mid/3590 MHz |  | Highest/Highest |  | Exception Note 5 of TS38.101 table 7.3A.4-1 |  |  |
| n70A / n71A |  | Mid/High |  | Highest/Highest |  | Non-exception. To be used in test cases 7.3A.1 to 7.3A.4 |  |  |
|  |  | Low/Low |  | Highest / 10 |  | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 20@10 |  |  |
|  |  | Low/Low |  | 5 / 5 |  | Exception Note 9 of TS38.101 table 7.3A.4-1 n71 UL RB allocation 8@10 |  |  |
| NOTE 1: This selection is used in test case 7.3A.1_1 unless otherwise stated. NOTE 2: Aggressor band in bold. |  |  |  |  |  |  |  |

#### 4.1.3.2	Spurious emissions test cases for FR1 UL CA

In this case, it is sufficient to verify the minimum requirements in frequency ranges affected by 2nd and 3rd order intermodulation products. The frequency ranges and UL RB allocations used in the test are calculated here.

The analyses are performed per CA configuration and are stored as zip-files as defined in annex A.

Table 4.1.3.2-1: Frequency range analysis availability per CA configuration

| CA config | Number of test point | Justification | Comments |
| --- | --- | --- | --- |
| CA_n1A-n78A | 5 | 38.521-1_TpAnalysisSpur(CA_n1A-n78A)v2.zip | RAN5#88e |
| CA_n2A-n77A | 3 | 38.521-1_TpAnalysisSpur(CA_n2A-n77A)_r1.zip | Added at RAN5#96e |
| CA_n3A-n78A | 3 | TpAnalysisSpur(n3A-n78A).zip | RAN5#82 |
| CA_n5A-n77A | 7 | 38.521-1_TpAnalysisSpur(CA_n5A-n77A)_r1.zip | Added at RAN5#96e |
| CA_n8A-n78A | 4 | TpAnalysisSpur(n8A-n78A).zip | RAN5#82 |
| CA_n24A-n41A | 5 | 38.521-1_TpAnalysisSpur(CA_n24A-n41A).zip | RAN5#95e |
| CA_n24A-n48A | 2 | 38.521-1_TpAnalysisSpur(CA_n24A-n48A).zip | RAN5#95e |
| CA_n24A-n77A | 4 | 38.521-1_TpAnalysisSpur(CA_n24A-n77A).zip | RAN5#95e |
| CA_n26A-n66A | 1 | 38.521-1_TpAnalysisSpur(CA_n26A-n66A).zip | Added at RAN5#96e |
| CA_n26A-n70A | 1 | 38.521-1_TpAnalysisSpur(CA_n26A-n70A).zip | Added at RAN5#96e |
| CA_n28A-n41A | 4 | 38.521-1_TpAnalysisSpur(CA_n28A-n41A).zip | RAN5#91e |
| CA_n41A-n79A | 7 | TpAnalysisSpur(n41A-n79A).zip | RAN5#83 |
| CA_n48A-n66A | 1 | 38.521-1_TpAnalysisSpur(CA_n48A-n66A).zip | Added at RAN5#96e |
| CA_n48A-n70A | 2 | 38.521-1_TpAnalysisSpur(CA_n48A-n70A).zip | Added at RAN5#96e |
| CA_n48A-n71A | 2 | 38.521-1_TpAnalysisSpur(CA_n48A-n71A).zip | Added at RAN5#96e |
| CA_n66A-n71A | 1 | 38.521-1_TpAnalysisSpur(CA_n66A-n71A).zip | Added at RAN5#96e |
| CA_n66A-n77A | 10 | 38.521-1_TpAnalysisSpur(CA_n66A-n77A)_r1.zip | Added at RAN5#96e |
| CA_n70A-n71A | 1 | 38.521-1_TpAnalysisSpur(CA_n70A-n71A).zip | Added at RAN5#96e |
| CA_n71A-n77A | 7 | 38.521-1_TpAnalysisSpur(CA_n71A-n77A).zip | Added at RAN5#97 |

[END OF MODIFICATIONS]

<< Add attached .zip files to TR 38.905: “38.521-1_TpAnalysisSpur(CA_n71A-n77A).zip” >>