3GPP TSG-RAN5 Meeting #97-e	R5-226915

# Toulouse, France, 14th – 18th Nov. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2025 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating Annex F for TxD test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MU and TT information is missing for several TxD test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding MU and TT information for TxD test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.1 UE maximum output power | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2.2 Maximum Power Reduction (MPR) | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2.3 UE additional maximum output power reduction | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.2.4 Configured transmitted power | f ≤ 3.0GHz ±0.7 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.3 dB, BW ≤ 20MHz ±1.5 dB, 20MHz < BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| < Unchanged Lines Skipped > |  |  |
| 6.2F.3 UE additional maximum output power reduction for shared spectrum access | Same as 6.2F.1 |  |
| 6.2G.1 UE maximum output power for Tx Diversity | Same as 6.2.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.2.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2G.2 UE maximum output power reduction for Tx Diversity | Same as 6.2.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.2.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2G.3 UE additional maximum output power reduction for Tx Diversity | Same as 6.2.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.2.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.2G.4 Configured transmitted power for Tx Diversity | Same as 6.2.4 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.2.4 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3.1 Minimum output power | f ≤ 3.0GHz ±1.0 dB, BW ≤ 40MHz ±1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.3 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.5 dB, BW ≤ 40MHz ±1.8 dB, 40MHz < BW ≤ 100MHz |  |
| 6.3.2 Transmit OFF power | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |
| 6.3.3.2 General ON/OFF time mask | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |
| 6.3.3.4 PRACH time mask | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |
| 6.3.3.6 SRS time mask | f ≤ 3.0GHz ±1.5 dB, BW ≤ 40MHz ±1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.8 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 80MHz ±2.2 dB, 80MHz < BW ≤ 100MHz |  |
| 6.3.4.2 Absolute power tolerance | f ≤ 3.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±1.4 dB, BW ≤ 40MHz ±1.9 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.0 dB, BW ≤ 20MHz ±2.1 dB, 20MHz < BW ≤ 40MHz ±2.2 dB, 80MHz < BW ≤ 100MHz | Test System uncertainty = SQRT (UL Meas Uncer2 + DL Meas Uncer2) |
| 6.3.4.3 Relative power tolerance | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for step 2.1 same as 6.2.1. Absolute Uplink power measurement for step 1.1 same as 6.3.1. |  |
| 6.3.4.4 Aggregate power tolerance | ±0.7 dB, BW ≤ 40MHz ±1.0 dB, 40MHz < f ≤ 100MHz |  |
| < Unchanged Lines Skipped > |  |  |
| 6.3G.1 Minimum output power for Tx Diversity | Same as 6.3.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3G.2 Transmit OFF power for Tx Diversity | Same as 6.3.2 for each antenna |  |
| 6.3G.3.1 General ON/OFF time mask for Tx Diversity | ON power: Same as 6.2G.1 OFF power: Same as 6.3G.2 |  |
| 6.3G.3.2 PRACH time mask for Tx Diversity | Same as 6.3.3.4 for each antenna |  |
| 6.3G.3.3 SRS time mask for Tx Diversity | Same as 6.3.3.6 for each antenna |  |
| 6.3G.4.1 Absolute power tolerance for Tx Diversity | Same as 6.3.4.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3G.4.2 Relative power tolerance for Tx Diversity | Same as 6.3.4.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.3G.4.3 Aggregate power tolerance for Tx Diversity | Same as 6.3.4.4 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna port in 6.3.4.4 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.4.1 Frequency Error | ±15 Hz, f ≤ 3.0GHz ±36 Hz, f > 3.0GHz DL Signal level: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.4.2.1 Error Vector Magnitude | For up to 256QAM: f ≤ 6.0GHz, BW ≤ 100MHz 15 dBm < PUL PUSCH, PUCCH, PRACH: ±1.5 % -25 dBm < PUL ≤ 15 dBm PUSCH, PUCCH, PRACH: ±2.5 % -40dBm ≤ PUL ≤ -25dBm PUSCH, PUCCH, PRACH: ±3.0 % Absolute Uplink power measurement same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4.2.1a Error Vector Magnitude including symbols with transient period | Same as 6.4.2.1 |  |
| 6.4.2.2 Carrier Leakage | f ≤ 3.0GHz ±0.8 dB, BW ≤ 40MHz ±1.5 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±0.8 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for step 2 and step 4 same as 6.2.1. Absolute Uplink power measurement for step 6 and step 8 same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4.2.3 In-band emissions | f ≤ 3.0GHz ±0.8 dB, BW ≤ 40MHz ±1.5 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±0.8 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±1.0 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz Absolute Uplink power measurement for steps 1.2, 1.4, 2.2, and 2.4 same as 6.2.1. Absolute Uplink power measurement for steps 1.6, 1.8, 2.6, and 2.8 same as 6.3.1. Relative Uplink power measurement same as 6.3.4.3. |  |
| 6.4.2.4 EVM equalizer spectrum flatness | ±1.4 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.4.2.5 EVM equalizer spectrum flatness for Pi/2 BPSK | Same as 6.4.2.4 |  |
| < Unchanged Lines Skipped > |  |  |
| 6.4G.1 Frequency Error for Tx Diversity | Same as 6.4.1 for each antenna |  |
| 6.4G.2.1 Error Vector Magnitude for Tx Diversity | FFS |  |
| 6.4G.2.2 Carrier Leakage for Tx Diversity | Same as 6.4.2.2 for each antenna Absolute Uplink power measurement for step 2 and step 4 same as 6.2G.1. Absolute Uplink power measurement for step 6 and step 8 same as 6.3G.1. Relative Uplink power measurement same as 6.3G.4.3. |  |
| 6.4G.2.3 In-band emissions for Tx Diversity | Same as 6.4.2.3 for each antenna Absolute Uplink power measurement for steps 1.2 and 1.4 same as 6.2G.1. Absolute Uplink power measurement for steps 1.6 and 1.8 same as 6.3G.1. Relative Uplink power measurement same as 6.3G.4.3. |  |
| 6.4G.2.4 EVM equalizer spectrum flatness for Tx Diversity | ±1.4 dB, BW ≤ 40MHz ±1.6 dB, 40MHz < BW ≤ 100MHz |  |
| 6.5.1 Occupied bandwidth | 1.5% of channel bandwidth |  |
| 6.5.2.2 Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.5.2.3 Additional spectrum emission mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.5.2.4.1 NR ACLR | ±0.8 dB, f ≤ 4.0GHz ±1.0 dB, 4.0GHz < f ≤ 6.0GHz |  |
| 6.5.2.4.2 UTRA ACLR | ±0.8 dB, f ≤ 4.0GHz ±1.0 dB, 4.0GHz < f ≤ 6.0GHz |  |
| 6.5.3.1 General spurious emissions | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |
| 6.5.3.2 Spurious emission for UE co-existence | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |
| 6.5.3.3 Additional spurious emissions | for results > -60 dBm: ±2.0 dB, 9kHz < f ≤ 3GHz ±2.5 dB, 3GHz < f ≤ 4GHz ±4.0 dB, 4GHz < f ≤ 19GHz ±6.0 dB, 19GHz < f ≤ 26GHz |  |
| 6.5.4 Transmit intermodulation | f ≤ 3.0GHz ±2.7 dB, BW ≤ 40MHz ±3.1 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±3.7 dB, BW ≤ 40MHz ±4.0 dB, 40MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±5.1 dB, BW ≤ 40MHz ±5.3 dB, 40MHz < BW ≤ 100MHz | Overall system uncertainty comprises four quantities: 1. Wanted signal setting error 2. CW Interferer level error 3. Wanted signal meas. error 4. Intermodulation product measurement error The relative level of the wanted signal and the CW interferer has 2 x effect on the intermodulation product. Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared to provide the combined effect. Test System uncertainty = SQRT [(2 x SQRT (Wanted setting_error2 + CW_level_error2)) 2 + Wanted_level_meas error2 + Intermodulation product measurement error2] |
| < Unchanged Lines Skipped > |  |  |
| 6.5D.4 Transmit intermodulation for UL MIMO | Same as 6.5.4 for each antenna |  |
| 6.5G.1 Occupied bandwidth for Tx Diversity | Same as 6.5.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.2.1 Spectrum emission mask for Tx Diversity | Same as 6.5.2.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.2.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.2.2 Additional spectrum emission mask for Tx Diversity | Same as 6.5.2.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.2.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.2.3.1 NR ACLR for Tx Diversity | Same as 6.5.2.4.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.2.4.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.2.3.2 UTRA ACLR for Tx Diversity | Same as 6.5.2.4.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.2.4.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.3.1 General spurious emissions for Tx Diversity | Same as 6.5.3.1 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.3.1 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.3.2 Spurious emissions for UE co-existence for Tx Diversity | Same as 6.5.3.2 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.3.2 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.3.3 Additional spurious emissions for Tx Diversity | Same as 6.5.3.3 for the sum of power at each of UE antenna connector | MU is for the sum of power at each of UE antenna connector, and is the same as the MU of single antenna connector in 6.5.3.3 with SNR assumption reduced by 3dB compared to the single antenna case. |
| 6.5G.4 Transmit intermodulation for Tx Diversity | Same as 6.5.4 for each antenna |  |

<Unchanged Text Skipped>

F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1 UE maximum output power | f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit - TT |
| 6.2.2 Maximum Power Reduction (MPR) | f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit - TT |
| 6.2.3 UE additional maximum output power reduction | f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit - TT |
| 6.2.4 Configured transmitted power | f ≤ 3.0GHz 0.7 dB, BW ≤ 40MHz 1.0 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.0 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit - TT |
| < Unchanged Lines Skipped > |  |  |
| 6.2G.1 UE maximum output power for Tx Diversity | Same as 6.2.1 for the sum of power at each of UE antenna connector | Same as 6.2.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.2G.2 UE maximum output power reduction for Tx Diversity | Same as 6.2.2 for the sum of power at each of UE antenna connector | Same as 6.2.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.2G.3 UE additional maximum output power reduction for Tx Diversity | Same as 6.2.3 for the sum of power at each of UE antenna connector | Same as 6.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.2G.4 Configured transmitted power for Tx Diversity | Same as 6.2.4 for the sum of power at each of UE antenna connector | Same as 6.2.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.3.1 Minimum output power | f ≤ 3.0GHz 1.0 dB, BW ≤ 40MHz 1.3 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.3 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.3.2 Transmit OFF power | f ≤ 3.0GHz 1.5 dB, BW ≤ 40MHz 1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.8 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.3.3.2 General ON/OFF time mask | f ≤ 3.0GHz 1.5 dB, BW ≤ 40MHz 1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.8 dB, BW ≤ 100MHz | OFF Power: Minimum requirement + TT ON Power: –Same as 6.2.1 |
| 6.3.3.4 PRACH time mask | f ≤ 3.0GHz 1.5 dB, BW ≤ 40MHz 1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.8 dB, BW ≤ 100MHz | OFF Power: Minimum requirement + TT ON Power: Upper limit + TT, Lower limit - TT |
| 6.3.3.6 SRS time mask | f ≤ 3.0GHz 1.5 dB, BW ≤ 40MHz 1.7 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.8 dB, BW ≤ 100MHz | OFF Power: Minimum requirement + TT ON Power: Upper limit + TT, Lower limit - TT |
| 6.3.4.2 Absolute power tolerance | UL Power ≥ 0dBm f ≤ 3.0GHz 1.0 dB, BW ≤ 40MHz 1.4 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 1.4 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit – TT |
| 6.3.4.3 Relative power tolerance | 0.7 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit – TT |
| 6.3.4.4 Aggregate power tolerance | 0.7 dB, BW ≤ 100MHz | Upper limit + TT, Lower limit – TT |
| < Unchanged Lines Skipped > |  |  |
| 6.3G.1 Minimum output power for Tx Diversity | Same as 6.3.1 for the sum of power at each of UE antenna connector | Same as 6.3.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3G.2 Transmit OFF power for Tx Diversity | Same as 6.3.2 for each antenna | Same as 6.3.2 Uplink power measurement applies to each Tx antenna connector |
| 6.3G.3.1 General ON/OFF time mask for Tx Diversity | ON power: Same as 6.2G.1 OFF power: Same as 6.3G.2 | ON power: Same as 6.2G.1 OFF power: Same as 6.3G.2 |
| 6.3G.3.2 PRACH time mask for Tx Diversity | Same as 6.3.3.4 for each antenna | Same as 6.3.3.4 Uplink power measurement applies to each Tx antenna connector |
| 6.3G.3.3 SRS time mask for Tx Diversity | Same as 6.3.3.6 for each antenna | Same as 6.3.3.6 Uplink power measurement applies to each Tx antenna connector |
| 6.3G.4.1 Absolute power tolerance for Tx Diversity | Same as 6.3.4.2 for the sum of power at each of UE antenna connector | Same as 6.3.4.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3G.4.2 Relative power tolerance for Tx Diversity | Same as 6.3.4.3 for the sum of power at each of UE antenna connector | Same as 6.3.4.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.3G.4.3 Aggregate power tolerance for Tx Diversity | Same as 6.3.4.4 for the sum of power at each of UE antenna connector | Same as 6.3.4.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |
| 6.4.1 Frequency Error | 15 Hz | Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: REFSENS + TT |
| 6.4.2.1 Error Vector Magnitude | For up to 64QAM 0% For 256QAM f ≤ 6.0GHz, BW ≤ 100MHz 0.3%, 15dBm < PUL 0.8%, -25dBm < PUL ≤ 15dBm, 1.1%, -40dBm ≤ PUL ≤ -25dBm | Minimum requirement + TT |
| 6.4.2.1a Error Vector Magnitude including symbols with transient period | Same as 6.4.2.1 | Same as 6.4.2.1 |
| 6.4.2.2 Carrier Leakage | 0.8 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.4.2.3 In-band emissions | 0.8 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.4.2.4 EVM equalizer spectrum flatness | 1.4 dB, BW ≤ 100MHz | Minimum requirement + TT |
| 6.4.2.5 EVM equalizer spectrum flatness for Pi/2 BPSK | Same as 6.4.2.4 | Minimum requirement + TT |
| < Unchanged Lines Skipped > |  |  |
| 6.4G.1 Frequency Error for Tx Diversity | Same as 6.4.1 for each antenna | Same as 6.4.1 |
| 6.4G.2.1 Error Vector Magnitude for Tx Diversity | FFS | FFS |
| 6.4G.2.2 Carrier Leakage for Tx Diversity | Same as 6.4.2.2 for each antenna | Same as 6.4.2.2 |
| 6.4G.2.3 In-band emissions for Tx Diversity | Same as 6.4.2.3 for each antenna | Same as 6.4.2.3 |
| 6.4G.2.4 EVM equalizer spectrum flatness for Tx Diversity | Same as 6.4.2.4 for each antenna | Same as 6.4.2.4 |
| 6.5.1 Occupied bandwidth | 0 kHz | Minimum requirement + TT |
| 6.5.2.2 Spectrum Emission Mask | 1.5 dB, f ≤ 3.0GHz 1.8 dB, 3.0GHz < f ≤ 6.0GHz | Minimum requirement + TT |
| 6.5.2.3 Additional spectrum emission mask | 1.5 dB, f ≤ 3.0GHz 1.8 dB, 3.0GHz < f ≤ 6.0GHz | Minimum requirement + TT |
| 6.5.2.4.1 NR ACLR | Absolute requirement 0 dB Relative requirement 0.8 dB | Absolute requirement ACLR Minimum Requirement + TT Relative requirement ACLR Minimum Requirement - TT |
| 6.5.2.4.2 UTRA ACLR | Same as 6.5.2.4.1 | Same as 6.5.2.4.1 |
| 6.5.3.1 General spurious emissions | 0 dB | Minimum requirement + TT |
| 6.5.3.2 Spurious emission for UE co-existence | 0 dB | Minimum requirement + TT |
| 6.5.3.3 Additional spurious emissions | 0 dB | Minimum requirement + TT |
| 6.5.4 Transmit intermodulation | 0 dB | CW interferer Minimum Requirement - TT |
| < Unchanged Lines Skipped > |  |  |
| 6.5D.4 Transmit intermodulation for UL MIMO | Same as 6.5.4 for each antenna | Same as 6.5.4 |
| 6.5G.1 Occupied bandwidth for Tx Diversity | Same as 6.5.1 for the sum of power at each of UE antenna connector | Same as 6.5.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.2.1 Spectrum emission mask for Tx Diversity | Same as 6.5.2.2 for the sum of power at each of UE antenna connector | Same as 6.5.2.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.2.2 Additional spectrum emission mask for Tx Diversity | Same as 6.5.2.3 for the sum of power at each of UE antenna connector | Same as 6.5.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.2.3.1 NR ACLR for Tx Diversity | Same as 6.5.2.4.1 for the sum of power at each of UE antenna connector | Same as 6.5.2.4.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.2.3.2 UTRA ACLR for Tx Diversity | Same as 6.5.2.4.2 for the sum of power at each of UE antenna connector | Same as 6.5.2.4.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.3.1 General spurious emissions for Tx Diversity | Same as 6.5.3.1 for the sum of power at each of UE antenna connector | Same as 6.5.3.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.3.2 Spurious emissions for UE co-existence for Tx Diversity | Same as 6.5.3.2 for the sum of power at each of UE antenna connector | Same as 6.5.3.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.3.3 Additional spurious emissions for Tx Diversity | Same as 6.5.3.3 for the sum of power at each of UE antenna connector | Same as 6.5.3.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over both Tx antenna connectors |
| 6.5G.4 Transmit intermodulation for Tx Diversity | Same as 6.5.4 for each antenna | Same as 6.5.4 |

<End of Changes>