3GPP TSG-RAN5 Meeting #97	R5-227139

# Toulouse, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2661 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MBS IE CFR-ConfigMCCH-MTCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CFR-ConfigMCCH-MTCH is FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the CFR-ConfigMCCH-MTCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CFR-ConfigMCCH-MTCH is FFS, and 14.1.1.1 need this IE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	CFR-ConfigMCCH-MTCH

Table 4.6.7-2: CFR-ConfigMCCH-MTCH

| Derivation Path: TS 38.331 [6], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CFR-ConfigMCCH-MTCH-r17 ::= SEQUENCE { |  |  |  |
| locationAndBandwidthBroadcast-r17 | Not present |  |  |
| locationAndBandwidthBroadcast-r17 CHOICE { |  |  | SIB1_BWP |
| sameAsSib1ConfiguredLocationAndBW | NULL |  |  |
| } |  |  |  |
| pdsch-ConfigMCCH-r17 | Not present |  |  |
| commonControlResourceSetExt-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SIB1_BWP | CFR for broadcast has the same location and size as the locationAndBandwidth for initial BWP configured in SIB1 |

<End of modified section 1>