3GPP TSG-RAN5 Meeting #97	R5-227168

# Toulouse, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3400 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR TC 7.1.1.4.2.4 to test RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RedCap UE use 12bit SN for AM RLC and PDCP, so the RLC header is 2 bytes and PDCP header is 2 bytes. - maximum channel bandwidth for FR1 RedCap UE is 20MHz, FR2 RedCap UE is 100MHz |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add Table 7.1.1.4.2.4.3.2-2AA to define the Number of downlink PDCP SDUs and PDCP SDU size used as test data for RedCap UE. - Limit the maximum test channel bandwidth for FR1 to 20Mhz for RedCap UE. - Limit the maximum test channel bandwidth for FR2 to 100Mhz for RedCap UE. - correct typo in Table 7.1.1.4.2.4.3.2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC or a non-conformant UE may pass the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.4.2.4	UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled

7.1.1.4.2.4.1	Test Purpose (TP)

(1)

with { UE in  RRC_CONNECTED state and mcs-Table is set as ‘qam256‘ }

ensure that {

when { UE has pending data for transmission and receives DCI format 0_1 indicating resource allocation type 0  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE transmits MAC PDU's on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

(2)

with { UE in  RRC_CONNECTED state and mcs-Table is set as ‘qam256‘ }

ensure that {

when { UE has pending data for transmission and receives DCI format 0_1 indicating resource allocation type 1  a resource block assignment correspondent to   physical resource blocks , Time domain resource assignment and a modulation and coding }

then { UE transmits MAC PDU's on PUSCH as per Modulation Coding scheme, time domain resource allocation  and PRB's }

}

7.1.1.4.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212 clause 7.3.1.1.1, TS 38.214 clause 6.1.2.1, 6.1.2.2, 6.1.2.2.1, 6.1.2.2.2, 6.1.4.1, 5.1.3.1, 6.1.4.2 and 5.1.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.212, clause 7.3.1.1.2]

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or new-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator – 0 or 3 bits, as defined in Subclause 10.1 of [5, TS38.213].

-	UL/SUL indicator – 0 bit for UEs not configured with SUL in the cell or UEs configured with SUL in the cell but only PUCCH carrier in the cell is configured for PUSCH transmission; 1 bit for UEs configured with SUL in the cell as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bit width for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active UL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Subclause 6.1.2.2.1 of [6, TS 38.214],

-	bits if only resource allocation type 1 is configured, or  bits if both resource allocation type 0 and 1 are configured.

-	If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the  LSBs provide the resource allocation as defined in Subclause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bit width of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bit width of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Subclause 6.1.2.1 of [6, TS38.214]. The bit width for this field is determined as bits, where I the number of entries in the higher layer parameter pusch-AllocationList.

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if the higher layer parameter frequencyHopping is not configured;

-	1 bit according to Table 7.3.1.1.2-34 otherwise, only applicable to resource allocation type 1, as defined in Subclause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme – 5 bits as defined in Subclause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	1st downlink assignment index – 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook.

-	2nd downlink assignment index – 0 or 2 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks;

-	0 bit otherwise.

-	TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS38.213]

-	SRS resource indicator – or  bits, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', and  is the maximum number of supported layers for the PUSCH.

-	 bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'nonCodeBook';

-	 bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook'.

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank, and codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters transformPrecoder, maxRank, and codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters maxRank and codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, and according to the values of higher layer parameters maxRank and codebookSubset.

-	Antenna ports – number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if transformPrecoder=enabled, dmrs-Type=1, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-7, if transformPrecoder=enabled, dmrs-Type=1, and maxLength=2;

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transformPrecoder=disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transformPrecoder=disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transformPrecoder=disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transformPrecoder=disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bit width of this field equals , where  is the “Antenna ports” bit width derived according to dmrs-UplinkForPUSCH-MappingTypeA and  is the “Antenna ports” bit width derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  and .

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with SUL in the cell; 3 bits for UEs configured SUL in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Subclause 6.1.1.2 of [6, TS 38.214].

-	CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize.

-	CBG transmission information (CBGTI) – 0, 2, 4, 6, or 8 bits determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for PUSCH.

-	PTRS-DMRS association – number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured and transformPrecoder=disabled, or if transformPrecoder=enabled, or if maxRank=1;

-	2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field.

If “Bandwidth part indicator” field indicates a bandwidth part other than the active bandwidth part and the “PTRS-DMRS association” field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the “PTRS-DMRS association” field is not present for the indicated bandwidth part.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

-	DMRS sequence initialization – 0 if the higher layer parameter transformPrecoder=enabled; 1 bit if the higher layer parameter transformPrecoder=disabled and both scramblingID0 and scramblingID1 are configured in DMRS-UplinkConfig, for  selection defined in Subclause 6.4.1.1.1.1 of [4, TS 38.211].

-	UL-SCH indicator – 1 bit. A value of “1” indicates UL-SCH shall be transmitted on the PUSCH and a value of “0” indicates UL-SCH shall not be transmitted on the PUSCH.

For a UE configured with SUL in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_1 for the SUL is not equal to the number of information bits in format 0_1 for the non-SUL, zeros shall be appended to smaller format 0_1 until the payload size equals that of the larger format 0_1.

Table 7.3.1.1.2-1: Bandwidth part indicator

| Value of BWP indicator field | Bandwidth part |
| --- | --- |
| 2 bits |  |
| 00 | First bandwidth part configured by higher layers |
| 01 | Second bandwidth part configured by higher layers |
| 10 | Third bandwidth part configured by higher layers |
| 11 | Fourth bandwidth part configured by higher layers |

Table 7.3.1.1.2-2: Precoding information and number of layers, for 4 antenna ports, if transformPrecoder=disabled and maxRank = 2 or 3 or 4

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |
| 19 | 1 layer: TPMI=11 | 19 | 1 layer: TPMI=11 |  |  |
| 20 | 2 layers: TPMI=6 | 20 | 2 layers: TPMI=6 |  |  |
| … | … | … | … |  |  |
| 27 | 2 layers: TPMI=13 | 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 | 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 | 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 | 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 | 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layers: TPMI=12 |  |  |  |  |
| … | … |  |  |  |  |
| 47 | 1 layers: TPMI=27 |  |  |  |  |
| 48 | 2 layers: TPMI=14 |  |  |  |  |
| … | … |  |  |  |  |
| 55 | 2 layers: TPMI=21 |  |  |  |  |
| 56 | 3 layers: TPMI=3 |  |  |  |  |
| … | … |  |  |  |  |
| 59 | 3 layers: TPMI=6 |  |  |  |  |
| 60 | 4 layers: TPMI=3 |  |  |  |  |
| 61 | 4 layers: TPMI=4 |  |  |  |  |
| 62-63 | reserved |  |  |  |  |

Table 7.3.1.1.2-3: Precoding information and number of layers for 4 antenna ports, if transformPrecoder= enabled, or if transformPrecoder=disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |  |  |
| … | … | … | … |  |  |
| 11 | 1 layer: TPMI=11 | 11 | 1 layer: TPMI=11 |  |  |
| 12 | 1 layers: TPMI=12 | 12-15 | reserved |  |  |
| … | … |  |  |  |  |
| 27 | 1 layers: TPMI=27 |  |  |  |  |
| 28-31 | reserved |  |  |  |  |

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder=disabled and maxRank = 2

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 | 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 | 3 | reserved |
| 4 | 1 layer: TPMI=3 |  |  |
| 5 | 1 layer: TPMI=4 |  |  |
| 6 | 1 layer: TPMI=5 |  |  |
| 7 | 2 layers: TPMI=1 |  |  |
| 8 | 2 layers: TPMI=2 |  |  |
| 9-15 | reserved |  |  |

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transformPrecoder= enabled, or if transformPrecoder= disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |  |  |
| 3 | 1 layer: TPMI=3 |  |  |
| 4 | 1 layer: TPMI=4 |  |  |
| 5 | 1 layer: TPMI=5 |  |  |
| 6-7 | reserved |  |  |

...

Table 7.3.1.1.2-33: VRB-to-PRB mapping

| Bit field mapped to index | VRB-to-PRB mapping |
| --- | --- |
| 0 | Non-interleaved |
| 1 | Interleaved |

[TS 38.214, clause 6.1.2.1]

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report on PUSCH by a DCI, the Time domain resource assignment field value md of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report by a CSI request field on a DCI, the Time-domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the applied resource allocation table is defined in sub-clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and K2 is determined based on the corresponding list entries of the higher layer parameter reportSlotConfig in CSI-ReportConfig for the triggered CSI Reporting Settings. The ith codepoint of K2 s determined as  where  is the ith codepoint of .

-	The slot where the UE shall transmit the PUSCH is determined by K2 as  where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively, and

-	The starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	The PUSCH mapping type is set to Type A or Type B as defined in Subclause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} | {0,…,12} | {1,…,12} | {1,…,12} |

When the UE is configured with aggregationFactorUL > 1, the same symbol allocation is applied across the aggregationFactorUL consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the aggregationFactorUL consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB is determined according to table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version when aggregationFactorUL > 1

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

If the UE procedure for determining slot configuration, as defined in subclause 11.1 of [6, TS 38.213], determines symbols of a slot allocated for PUSCH as downlink symbols, the transmission on that slot is omitted for multi-slot PUSCH transmission.

[38.214 clause 6.1.2.2]

The UE shall determine the resource block assignment in frequency domain using the resource allocation field in the detected PDCCH DCI. Two uplink resource allocation schemes type 0 and type 1 are supported. Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. Uplink resource allocation scheme type 1 is supported for PUSCH for both cases when transform precoding is enabled or disabled.

If the scheduling DCI is configured to indicate the uplink resource allocation type as part of the Frequency domain resource assignment field by setting a higher layer parameter resourceAllocation in pusch-Config to ‘dynamicswitch’, the UE shall use uplink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the uplink frequency resource allocation type as defined by the higher layer parameter resourceAllocation.

The UE shall assume that when the scheduling PDCCH is received with DCI format 0_0, then uplink resource allocation type 1 is used.

If a bandwidth part indicator field is not configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI, the RB indexing for uplink type 0 and type 1 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI, except for the case when DCI format 0_0 is decoded in any PDCCH common search space in CORESET 0 in which case the initial bandwidth part shall be used. The UE shall upon detection of PDCCH intended for the UE determine first the uplink bandwidth part and then the resource allocation within the bandwidth part.

[38.214 clause 6.1.2.2.1]

In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Sizeconfigured for PUSCH and the size of the carrier bandwidth part as defined in Table 6.1.2.2.1-1.

Table 6.1.2.2.1-1: Nominal RBG size P

| Carrier Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

The total number of RBGs () for a uplink carrier bandwidth part i of sizePRBs is given by  where

-	the size of the first RBG is ,

-	the size of the last RBG is if and P otherwise.

-	the size of all other RBG is P.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency of the carrier bandwidth part and starting at the lowest frequency. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB of the bitmap. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

[38.214 clause 6.1.2.2.2]

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active carrier bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in the Type0-PDCCH common search space in CORESET 0 in which case the initial bandwidth part of size  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

[TS 38.214, clause 6.1.4.1]

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, or SP-CSI-RNTI, the transform precoding is enabled if transformPrecoder in PUSCH-Config is set to 'enabled', or if transformPrecoder in PUSCH-Config is not configured and msg3-transformPrecoding in rach-ConfigCommon is set to 'enabled'; otherwise the transform precoding is disabled.

For the PUSCH assigned by a DCI format 0_0/0_1 with CRC scrambled by CS-RNTI, or the PUSCH with configured grant using CS-RNTI, the transform precoding is enabled if transformPrecoder in ConfiguredGrantConfig is set to 'enabled'; otherwise the transform precoding is disabled.

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, TC-RNTI, or CS-RNTI, or SP-CSI-RNTI, or for a PUSCH with configured grant using CS-RNTI,

if transformPrecoder is disabled for this PUSCH transmission

-	if mcs-Table in PUSCH-Config is set to 'qam256', and PUSCH is scheduled with C-RNTI or SP-CSI-RNTI, and PUSCH is assigned by DCI format 0_1,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with new-RNTI, mcs-Table in PUSCH-Config is set to 'qam64LowSE', the PUSCH is scheduled with C-RNTI, or SP-CSI-RNTI, and the PUSCH is assigned by a PDCCH in a UE-specific search space,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with new-RNTI, and the PUSCH is scheduled with new-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in ConfiguredGrantConfig is set to 'qam256', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in ConfiguredGrantConfig is set to 'qam64LowSE', and PUSCH is scheduled with CS-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

[TS 38.214, clause 5.1.3.1]

Table 5.1.3.1-2: MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

[TS 38.214, clause 6.1.4.2]

For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by a DCI format 0_0/0_1 with CRC scrambled by C-RNTI, new-RNTI, TC-RNTI, CS-RNTI, or SP-CSI-RNTI.

if

-	and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  is the number of symbols of the PUSCH allocation within the slot,  is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 0_1 or as described for DCI format 0_0 in Subclause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 0, 6, 12, or 18), the  is assumed to be 0. For MSG3 transmission the  is always set to 0..

-	A UE determines the total number of REs allocated for PUSCH  by where  is the total number of allocated PRBs for the UE.

-	Next, proceed with steps 2-4 as defined in Subclause 5.1.3.2

else if

-	 and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant, the TBS shall be determined from the most recent configured scheduling PDCCH.

[TS 38.214, clause 5.1.3.2]

2)	Intermediate number of information bits (Ninfo) is obtained by .

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-2 find the closest TBS that is not less than .

Table 5.1.3.2-2: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

7.1.1.4.2.4.3	Test description

7.1.1.4.2.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except set the NR Cell bandwidth and applicable BWP to maximum for the NR Band under test as specified in Table 5.3.5-1 in TS 38.101-1 [16] / TS 38.101-2 [17] (to enable testing of nPRB up to maximum value).

Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

Note: if pc_supportOfRedCap_r17=true, the FR1 test channel bandwidth is MIN {20MHz, common highest mandatory UL and DL channel bandwidth in 38.508-1[4] clause 4.3.1}, and the FR2 test channel bandwidth is MIN{100MHz, common highest mandatory UL and DL channel bandwidth in 38.508-1[4] clause 4.3.1}

7.1.1.4.2.4.3.2	Test procedure sequence

Table 7.1.1.4.2.4.3.2-1: Maximum TBS for different UE categories

| UE Category | Maximum number of bits of a UL-SCH transport block received within a TTI |
| --- | --- |
| TS 38.306 [23] clause 4.1.2 require UE without ue-CategoryDL and ue-CategoryUL, to support Max TBS achievable based on max bandwidth of the Band under test. |  |

Table 7.1.1.4.2.4.3.2-2: Number of downlink PDCP SDUs and PDCP SDU size used as test data for non-RedCap UE

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 136 ≤ TBS ≤12128 note 2 | 1 | 8*FLOOR((TBS – 128)/8) |
| 12129 ≤ TBS ≤24200 | 2 | 8*FLOOR((TBS – 200)/16) |
| 24201 ≤ TBS ≤ 36272 | 3 | 8*FLOOR((TBS – 272)/24) |
| 36273 ≤ TBS ≤48344 | 4 | 8*FLOOR((TBS – 344)/32) |
| 48345≤ TBS ≤60416 | 5 | 8*FLOOR((TBS – 416)/40) |
| 60417 ≤ TBS ≤ 72488 | 6 | 8*FLOOR((TBS –488)/48) |
| 72489 ≤ TBS ≤84560 | 7 | 8*FLOOR((TBS – 560)/56) |
| 84561 ≤ TBS ≤96632 | 8 | 8*FLOOR((TBS –632)/64) |
| 96633< TBS ≤108704 | 9 | 8*FLOOR((TBS –704)/72) |
| 108705 ≤ TBS ≤120776 | 10 | 8*FLOOR((TBS – 776)/80) |
| 120777≤ TBS ≤132848 | 11 | 8*FLOOR((TBS –848)/88) |
| 132849 ≤ TBS ≤ 144920 | 12 | 8*FLOOR((TBS – 920)/96) |
| TBS> 144920 | 13 | 8*FLOOR((TBS – 992)/104) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 24 bits for the RLC AM and 18-bit SN case; AMD PDU header size is 24 bits with 18 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*24- N*24 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 136 bits. |  |  |

Table 7.1.1.4.2.4.3.2-2AA: Number of downlink PDCP SDUs and PDCP SDU size used as test data for RedCap UE

| TBS [bits] | Number of PDCP SDUs | PDCP SDU size [bits] (Note 1) |
| --- | --- | --- |
| 120 ≤ TBS ≤12112 note 2 | 1 | 8*FLOOR((TBS – 112)/8) |
| 12113 ≤ TBS ≤24168 | 2 | 8*FLOOR((TBS – 168)/16) |
| 24169 ≤ TBS ≤ 36224 | 3 | 8*FLOOR((TBS – 224)/24) |
| 36225 ≤ TBS ≤ 48280 | 4 | 8*FLOOR((TBS – 280)/32) |
| 48281≤ TBS ≤ 60336 | 5 | 8*FLOOR((TBS – 336)/40) |
| 60337 ≤ TBS ≤ 72392 | 6 | 8*FLOOR((TBS – 392)/48) |
| 72393 ≤ TBS ≤ 84448 | 7 | 8*FLOOR((TBS – 448)/56) |
| 84449 ≤ TBS ≤ 96504 | 8 | 8*FLOOR((TBS – 504)/64) |
| 96505 < TBS ≤ 108560 | 9 | 8*FLOOR((TBS – 560)/72) |
| 108561 ≤ TBS ≤ 120616 | 10 | 8*FLOOR((TBS – 616)/80) |
| 120617 ≤ TBS ≤ 132672 | 11 | 8*FLOOR((TBS – 672)/88) |
| 132673 ≤ TBS ≤ 144728 | 12 | 8*FLOOR((TBS – 728)/96) |
| TBS> 144728 | 13 | 8*FLOOR((TBS – 784)/104) |
| Note 1: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 [21] clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is PDCP SDU size = (TBS – N*PDCP header size – N*AMD PDU header size - N*MAC header size – Size of Timing Advance – RLC Status PDU size- MAC header for RLC Status PDU) / N, where PDCP header size is 16 bits for the RLC AM and 12-bit SN case; AMD PDU header size is 16 bits with 12 bit SN; MAC header size for AMD PDU = 16 or 24 bits depending on L=8 or 16 bits. Worst case 24 is taken. Size of Timing Advance MAC CE with header is 16 bits (if no Timing Advance and/or RLC status needs to be sent, padding will occur instead). RLC Status PDU size = 24 bits with 1 ACK_SN, With a MAC header of 16 bits. This gives: PDCP SDU size = 8*FLOOR((TBS – N*16- N*16 – N*24 -56 )/(8*N)) bits. Note 2: According to the final PDCP SDU size formula in Note 1, the smallest TBS that can be tested is 120 bits. |  |  |

Table 7.1.1.4.2.4.3.2-2A: Bandwidth part Dependent Parameters for Resource allocation 0 with start of BWP assumed as 0

| = | Nominal RBG size P (Configuration1) | Size of last RBG | Allowed Values |
| --- | --- | --- | --- |
| 11 | 2 | 1 | All 1…11 |
| 18 | 2 | 2 | 2,4,6,8,10,12,16,18 |
| 24 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24 |
| 25 | 2 | 1 | All 1…25 |
| 31 | 2 | 1 | All 1…31 |
| 32 | 2 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32 |
| 38 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38 |
| 51 | 4 | 3 | 3,4,7,8,11,12,15,16,19,20,23,24,27,28,31,32,35,36,39,40,43,44,47,48,51 |
| 52 | 4 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52 |
| 65 | 4 | 1 | 1,4,5,8,9,12,13,16,17,20,21,24,25,28,29,32,33,36,37,40,41,44,45,48,49, 52,53,56,57,60,61,64,65 |
| 66 | 4 | 2 | 2,4,6,8,10,12,16,18,20,22,24,26,28,30,32,34,36,38,40,42,44,46,48,50,52, 54,56,58,60,62,64,66 |
| 79 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79 |
| 106 | 8 | 2 | 2,8,10,16,18,24,26,32,34,40,42,48,50,56,58,64,66,72,74,80,82,88,90,96, 92,104,106 |
| 107 | 8 | 3 | 3,8,11,16,19,24,27,32,35,40,43,48,51,56,59,64,67,72,75,80,83,88,91,96, 99,104,107 |
| 132 | 8 | 4 | 4,8,12,16,20,24,28,32,36,40,44,48,52,56,60,64,68,72,76,80,84,88,92,96, 100,104, 108,112,116,120,124,128,132 |
| 133 | 8 | 5 | 5,8,13,16,21,24,29,32,37,40,45,48,53,56,61,64,69,72,77,80,85,88,93,96, 101,104, 109,112,117,120,125,128,133 |
| 135 | 8 | 7 | 7,8,15,16,23,24,31,32,39,40,47,48,55,56,63,64,71,72,79,80,87,88,95,96, 103,104, 111,112,119,120,127,128,135 |
| 160 | 16 | 16 | 16,32,48,64,80,96,112,128,144,160 |
| 216 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,152,160,168, 176,184,192,200,208,216 |
| 217 | 16 | 9 | 9,16,25,32,41,48,57,64,73,80,89,96,105,112,121,128,137,144,153,160,169, 176,185,192,201,208,217 |
| 264 | 16 | 8 | 8,16,24,32,40,48,56,64,72,80,88,96,104,112,120,128,136,144,160,168, 176,184,192,200,208,216,224,232,240,248,256,264 |
| 270 | 16 | 14 | 14,16,30,32,46,44,62,64,78,80,94,96,110,112, 126,128,142,144,158,160, 174, 176,190,192, 206,208,222,224,238,240, 254,256,270 |
| 273 | 16 | 1 | 1,16,17,32,33,48,49,64,65,80,81,96,97,112,113,128,129,144,145,160, 161,176,171, 192,193, 208,209, 224,225,240,241,256,257,272,273 |

Table 7.1.1.4.2.4.3.2-3: Specific Parameter

| Parameter | Value | Comment | Condition |
| --- | --- | --- | --- |
| number of layers (ʋ) | 1 |  |  |
| mcs-Table | qam256 |  |  |
| resourceAllocation | dynamicSwitch |  | pc_dynamicSwitchRA_Type0_1_PUSCH |
|  | resourceAllocationType1 |  | NOT pc_dynamicSwitchRA_Type0_1_PUSCH AND Steps 1-5 |
|  | resourceAllocationType0 |  | NOT pc_dynamicSwitchRA_Type0_1_PUSCH AND pc_ra_Type0_PUSCH AND Steps 6-10 |
| rbg-Size | Not present | configuration 1 applicable |  |
| NstartBWP | 0 |  |  |

Table 7.1.1.4.2.4.3.2-4: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 to 5 are repeated for allowed values of 1 to in BWP, time domain resource as per Table 7.1.1.4.2.0-1 and from 0 to 27. | - | - | - | - |
| 1 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 2 to 5 are performed if TBS is less than or equal to UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.4.3.2-1 and larger than or equal to X bits as specified in Table 7.1.1.4.2.4.3.2-2/2AA.(Note 4) Skip the execution of steps 2 to 5 for which the TBS size equal to 3824 or 3840. (Note 3) | - | - | - | - |
| 2 | SS creates one or more PDCP SDUs depending on TBS in accordance with Table 7.1.1.4.2.4.3.2-2/2AA. (Note 5) | - | - | - | - |
| 3 | After 300ms, the SS transmits all PDCP SDUs (NSDUs) as created in step 2 in a MAC PDU. | <-- | MAC PDU (NxPDCP SDUs) | - | - |
| 4 | After 60ms of step 3 SS transmits UL Grant DCI 0_1, and values of S, L,and nPRB.. | <-- | (UL Grant) (DCI: (DCI Format 0_1, S, L,and nPRB.) | - | - |
| 5 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 3 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 4? | --> | (NxPDCP SDUs) | 2 | P |
| - | EXCEPTION : Steps 5Aa1 to 10 are executed if pc_ra_Type0_PUSCH | - | - | - | - |
| - | EXCEPTION : Steps 5Aa1 to 5Aa2 are executed if NOT pc_dynamicSwitchRA_Type0_1_PUSCH | - | - | - | - |
| 5Aa1 | The SS transmits a NR RRCReconfiguration message including PUSCH-Config with IE resourceAllocation set to resourceAllocationType1 (Note 1) | <-- | RRCReconfiguration | - | - |
| 5Aa2 | The UE transmit a NR RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 6 to 10 are repeated for allowed values of as per Table 7.1.1.4.2.4.3.2-2A in BWP, time domain resource length L 3 to 14-S and from 0 to 27. | - | - | - | - |
| 6 | SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L,and nPRB. | - | - | - | - |
| - | EXCEPTION: Steps 7 to 10 are performed if TBS is less than or equal to UE capability "Maximum number of UL-SCH transport block bits received within a TTI" as specified in Table 7.1.1.4.2.4.3.2-1 and larger than or equal to X bits as specified in Table 7.1.1.4.2.4.3.2-2/2AA. (Note 4) Skip the execution of steps 7 to 10 for which the TBS size equal to 3824 or 3840. (Note 3) | - | - | - | - |
| 7 | SS creates one or more PDCP SDUs depending on TBS in accordance with Table 7.1.1.4.2.4.3.2-2/2AA. (Note 5) | - | - | - | - |
| 8 | After 300ms, the SS transmits all PDCP SDUs (NSDUs) as created in step 7 in a MAC PDU. | <-- | MAC PDU (NxPDCP SDUs) | - | - |
| 9 | After 60ms of step 8 SS transmits UL Grant DCI 0_1, and values of S, L,and nPRB.. | <-- | (UL Grant) (DCI: (DCI Format 0_1, S, L,and nPRB.) | - | - |
| 10 | CHECK: Does UE return the same number of PDCP SDUs with same content as transmitted by the SS in step 8 using Time, frequency Resources and modulation and coding scheme as configured by the SS in step 4? | --> | (NxPDCP SDUs) | 1 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: There is ambiguity of TBS calculation when 3824.0 < Ninfo < 3825.0 in clause 5.1.3.2 of TS 38.214 [15]. Note 4: For pc_supportOfRedCap_r17=false, Table 7.1.1.4.2.3.3.2-2 is used and X =136. For pc_supportOfRedCap_r17=true, Table 7.1.1.4.2.3.3.2-2AA is used and X=120. Note 5: For pc_supportOfRedCap_r17=false, Table 7.1.1.4.2.3.3.2-2 is used. For pc_supportOfRedCap_r17=true, Table 7.1.1.4.2.3.3.2-2AA is used. |  |  |  |  |  |

7.1.1.4.2.4.3.3	Specific message contents

[None].

<End of modified section 1>