3GPP TSG-RAN5 Meeting #97	R5-226578

# Toulouse, France, 14th - 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-2 | CR | 0994 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to applicability for 8.7.1.1_H.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TTA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The applicability condition for TC 8.7.1.1_H.5 is incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated applicability condition for TC 8.7.1.1_H.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case couldn’t be excuted correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

4.1	RF conformance test cases

NOTE:	To determine applicability of a test case, FGI support in combined or fdd-Add-UE-EUTRA-Capabilities or tdd-Add-UE-EUTRA-Capabilities is taken into account.

Table 4.1-1: Applicability of RF conformance test cases, ref. TS 36.521-1 [1]

| Clause |  | Title |  | Release |  |  | Applicability |  |  |  | Tested Bands / CA-Configurations Selection |  |  | Branch |  | Additional Information |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Condition |  | Comments |  |  |  |  |  |  |  |  |
| Transmitter Characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2.2 |  | UE Maximum Output Power |  | Rel-8 |  |  | C186 |  | UE supporting E-UTRA Power Class 3 |  | D01 |  |  | FDD, TDD |  | Not required to be tested in Band 41 for Power Class 2 UE |  |
| 6.2.2_1 |  | UE Maximum Output Power for HPUE |  | Rel-10 |  |  | C39a |  | UE supporting E-UTRA Power Class 1 |  | D16 |  |  |  |  |  |  |
|  |  |  |  |  |  |  | C39b |  | UE supporting E-UTRA Power Class 2 |  | D17 |  |  |  |  |  |  |
| 6.2.2A.1 |  | UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) |  | Rel-10 |  |  | C19 |  | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA |  | E01 |  |  | FDD, TDD |  | Not required to be tested in Band 41 for Power Class 2 UE |  |
| Many unchanged rows skipped |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.1.1 |  | FDD PHICH Single-antenna Port Performance |  | Rel-8 |  |  | C01 |  | UE supporting E-UTRA FDD |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.1.2 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.1.2.1 |  | FDD PHICH Transmit Diversity 2x2 |  | Rel-8 only |  |  | C09 |  | UE supporting E-UTRA FDD and operating bands supporting 1,4 MHz Bandwidth |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.1.2.1_1 |  | FDD PHICH Transmit Diversity 2x2 (Release 9 and forward) |  | Rel-9 |  |  | C01 |  | UE supporting E-UTRA FDD |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.1.2.2 |  | FDD PHICH Transmit Diversity 4x2 |  | Rel-8 only |  |  | C01 |  | UE supporting E-UTRA FDD |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.1.2.2_1 |  | FDD PHICH Transmit Diversity 4x2 (Release 9 and forward) |  | Rel-9 |  |  | C01 |  | UE supporting E-UTRA FDD |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.1.2.3_C.1 |  | FDD PHICH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) |  | Rel-10 |  |  | C29 |  | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.1.2.3_E.1 |  | FDD PHICH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) |  | Rel-11 |  |  | C77 |  | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.1.2.5 |  | FDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH- 2 Tx Antenna Ports under Asynchronous Network |  | Rel-13 |  |  | C285 |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |
| 8.5.1.2.6 |  | FDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer |  | Rel-13 |  |  | C285 |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |
| 8.5.1.2.7 |  | FDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer |  | Rel-13 |  |  | C286 |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |
| 8.5.1.2.8 |  | FDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer |  | Rel-13 |  |  | C286 |  | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |
| 8.5.2.1 |  | TDD PHICH Single-antenna Port Performance |  | Rel-8 |  |  | C02 |  | UE supporting E-UTRA TDD |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.2.2 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.5.2.2.1 |  | TDD PHICH Transmit Diversity 2x2 |  | Rel-8 only |  |  | C10 |  | UE supporting E-UTRA TDD and operating bands supporting 1,4 MHz Bandwidth |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.2.2.1_1 |  | TDD PHICH Transmit Diversity 2x2 (Release 9 and forward) |  | Rel-9 |  |  | C02 |  | UE supporting E-UTRA TDD |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.2.2.2 |  | TDD PHICH Transmit Diversity 4x2 |  | Rel-8 only |  |  | C02 |  | UE supporting E-UTRA TDD |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.2.2.2_1 |  | TDD PHICH Transmit Diversity 4x2 (Release 9 and forward) |  | Rel-9 |  |  | C02 |  | UE supporting E-UTRA TDD |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.2.2.3_C.1 |  | TDD PHICH Transmit Diversity 2x2 for eICIC (non-MBSFN ABS) |  | Rel-10 |  |  | C30 |  | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.2.2.3_E.1 |  | TDD PHICH Transmit Diversity 2x2 for feICIC (non-MBSFN ABS) |  | Rel-11 |  |  | C78 |  | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.5.2.2.5 |  | TDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH- 2 Tx Antenna Ports with Colliding CRS Dominant Interfere |  | Rel-13 |  |  | C287 |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |
| 8.5.2.2.6 |  | TDD Enhanced Downlink Control Channel Performance Requirement Type A for PHICH - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer |  | Rel-13 |  |  | C287 |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |  |  |  |
| 8.5.2.2.7 |  | TDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH- 2 Tx Antenna Ports with Colliding CRS Dominant Interferer |  | Rel-13 |  |  | C288 |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |
| 8.5.2.2.8 |  | TDD Enhanced Downlink Control Channel Performance Requirement Type B for PHICH- 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer |  | Rel-13 |  |  | C288 |  | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type B |  |  |  |  |  |  |  |  |
| 8.7.1.1 |  | FDD sustained data rate performance (Rel-9 and forward) |  | Rel-9 |  |  | C76 |  | UE supporting E-UTRA FDD and not supporting 256QAM in DL (UE categories from1 to 4) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.1.1_A.1 or 8.7.3.1 is executed. |  |
| 8.7.1.1_1 |  | FDD sustained data rate performance (Rel-10 and forward) |  | Rel-10 |  |  | C42 |  | UE supporting E-UTRA FDD and not supporting 256QAM in DL (UE categories 6, 7) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.1.1_A.1or 8.7.3.1 is executed. |  |
| 8.7.1.1_2 |  | FDD sustained data rate performance for UE category 1bis |  | Rel-13 |  |  | C145d |  | UE supporting E-UTRA FDD (UE category 1bis) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.7.1.1_A.1 |  | FDD Sustained data rate performance for CA (2 DL CA ) |  | Rel-10 |  |  | C107 |  | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  | Refer to 36.521-1 8.1.2.3 |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.2 is executed. |  |
|  |  |  |  | Rel-11 |  |  | C93 |  | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  |  |  |  |  | Note 7 |  |
| 8.7.1.1_A.2 |  | FDD Sustained data rate performance for CA (3DL CA) |  | Rel-10 |  |  | C126 |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  | Refer to 36.521-1 8.1.2.3 |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.4 is executed |  |
|  |  |  |  | Rel-10 |  |  | C126a |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and supporting at most 40MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 6 and 7) |  |  | Refer to 36.521-1 8.1.2.3 |  |  |  | Note 7 |  |
|  |  |  |  | Rel-11 |  |  | C127 |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  | TBD |  |  |  |  |  |
|  |  |  |  | Rel-11 |  |  | C127a |  | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and supporting at most 40MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 6 and 7) |  |  | TBD |  |  |  |  |  |
| 8.7.1.1_A.4 |  | FDD Sustained data rate performance for CA (4DL CA) |  | Rel-11 |  |  | C189 |  | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and not supporting 256QAM in DL (UE category 11 and 12) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.5 is executed. Note 7 |  |
|  |  |  |  | Rel-11 |  |  | C189a |  | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and supporting at most 60MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 9 and 10) |  |  |  |  |  |  |  |  |
| 8.7.1.1_A.5 |  | FDD Sustained data rate performance for CA (5DL CA) |  | Rel-11 |  |  | C266 |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 and not supporting 256QAM in DL (UE DL category 11,12) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.6 is executed. Note 7 |  |
|  |  |  |  | Rel-12 |  |  | C267 |  | UE supporting E-UTRA FDD and 5DL CA configurations in Table 4.1-5 and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  |  |  |  |  |  |  |
| 8.7.1.1_A.6 |  | FDD Sustained data rate performance for CA (6DL CA) |  | Rel-11 |  |  | C330 |  | UE supporting E-UTRA FDD and 6DL with CA configurations in Table 4.1-6 and not supporting 256QAM in DL (UE DL category 11,12) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.7 is executed. Note 7 |  |
|  |  |  |  | Rel-12 |  |  | C331 |  | UE supporting E-UTRA FDD and 6DL CA configurations in Table 4.1-6 and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  |  |  |  |  |  |  |
| 8.7.1.1_A.7 |  | FDD Sustained data rate performance for CA (7DL CA) |  | Rel-15 |  |  | C343 |  | UE supporting E-UTRA FDD and 7DL CA and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  |  |  | 2Rx, 4Rx |  | Note 7 |  |
| -8.7.1.1_H.1 |  | FDD sustained data rate performance (Single Carrier) for 256QAM in DL |  | Rel-12 |  |  | C42h |  | UE supporting E-UTRA FDD and 256QAM and UE DL category 13 |  |  |  |  |  |  | Test execution not necessary if 8.7.1.1_H.2 is executed |  |
| 8.7.1.1_H.2 |  | FDD Sustained data rate performance for CA (2DL CA) for 256QAM in DL |  | Rel-12 |  |  | C107h |  | UE supporting E-UTRA FDD and 2DL CA and 256QAM in DL (UE DL category 11, 12 and 13) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_H.3 is executed Note 7 |  |
| 8.7.1.1_H.3 |  | FDD Sustained data rate performance for CA (3DL CA) for 256QAM in DL |  | Rel-12 |  |  | C126h |  | UE supporting E-UTRA FDD and 3DL CA ,and supporting 256QAM in DL (UE DL category 11, 12 and 15) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_H.4 is executed Note 7 |  |
|  |  |  |  | Rel-12 |  |  | C126ha |  | UE supporting E-UTRA FDD and 3DL CA ,and supporting 256QAM in DL and supporting at most 40MHz aggregated bandwidth (UE DL category 13) |  |  |  |  |  |  | Test execution not necessary if 8.7.1.1_H.4 is executed |  |
| 8.7.1.1_H.4 |  | FDD Sustained data rate performance for CA (4DL CA) for 256QAM in DL |  | Rel-13 |  |  | C189h |  | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and supporting 256QAM in DL |  |  |  |  |  |  | Test execution not necessary if 8.7.1.1_H.5 is executed. |  |
| 8.7.1.1_H.5 |  | FDD Sustained data rate performance for CA (5DL CA) for 256QAM in DL |  | Rel-13 |  |  | C267h |  | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 and supporting 256QAM in DL |  |  |  |  |  |  |  |  |
| 8.7.1.1_H.6 |  | FDD Sustained data rate performance for CA (6DL CA) for 256QAM in DL |  | Rel-13 |  |  | C331h |  | UE supporting E-UTRA FDD and 6DL with CA configurations in Table 4.1-6 and supporting 256QAM in DL |  |  |  |  |  |  |  |  |
| 8.7.1.1_H.7 |  | FDD Sustained data rate performance for CA (7DL CA) for 256QAM in DL |  | Rel-15 |  |  | C344 |  | UE supporting E-UTRA FDD and 7DL and supporting 256QAM in DL |  |  |  |  |  |  | Test execution not necessary if 8.7.1.1_H.8 is executed |  |
| 8.7.2.1 |  | TDD sustained data rate performance (Rel-9 and forward) |  | Rel-9 |  |  | C111 |  | UE supporting E-UTRA TDD and not supporting 256QAM in DL (UE categories from 1 to 4) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.2.1_A.1 or 8.7.4.1 is executed. |  |
| 8.7.2.1_1 |  | TDD sustained data rate performance (Rel-10 and forward) |  | Rel-10 |  |  | C73 |  | UE supporting E-UTRA TDD and not supporting 256QAM in DL (UE category 6 and 7) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.2.1_A.1or 8.7.4.1 is executed. |  |
| 8.7.2.1_2 |  | TDD sustained data rate performance for UE category 1bis |  | Rel-13 |  |  | C156f |  | UE supporting E-UTRA TDD (UE category 1bis) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.7.2.1_A.1 |  | TDD sustained data rate performance for CA (2DL CA) |  | Rel-10 |  |  | C74 |  | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA and not supporting 256QAM in DL (UE category 6, 7, 9 and 10) |  |  | Refer to 36.521-1 8.1.2.3 |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.2 is executed. |  |
|  |  |  |  | Rel-11 |  |  | C75 |  | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA and not supporting 256QAM in DL (UE category 6, 7, 9 and 10) |  |  |  |  |  |  | Note 7 |  |
| 8.7.2.1_A.2 |  | TDD Sustained data rate performance for CA (3DL CA) |  | Rel-10 |  |  | C130 |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  | Refer to 36.521-1 8.1.2.3 |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.3 is executed. |  |
|  |  |  |  | Rel-11 |  |  | C131 |  | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  |  |  |  |  | Note 7 |  |
| 8.7.2.1_A.3 |  | TDD Sustained data rate performance for CA (4DL CA) |  | Rel-11 |  |  | C213 |  | UE supporting E-UTRA TDD and 4DL CA configurations in Table 4.1-3 (UE DL category 11, 12 and 15) |  |  | Refer to 36.521-1 8.1.2.3 |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.4 is executed. Note 7 |  |
| 8.7.2.1_A.5 |  | TDD Sustained data rate performance for CA (5DL CA) |  | Rel-11 |  |  | C360 |  | UE supporting E-UTRA TDD and 5DL CA configurations in Table 4.1-3 (UE DL category 11, 12 and 15) |  |  | Refer to 36.521-1 8.1.2.3 |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.6 is executed. Note 7 |  |
| 8.7.2.1_A.6 |  | TDD Sustained data rate performance for CA (6DL CA) |  | Rel-15 |  |  | C331a |  | UE supporting E-UTRA TDD and 6DL CA configurations in Table 4.1-3 (UE DL category 11, 12 and 15) |  |  | Refer to 36.521-1 8.1.2.3 |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.7 is executed. Note 7 |  |
| 8.7.2.1_A.7 |  | TDD Sustained data rate performance for CA (7DL CA) |  | Rel-15 |  |  | C345 |  | UE supporting E-UTRA TDD and 7DL CA (UE DL category 11, 12 and 15) |  |  | Refer to 36.521-1 8.1.2.3 |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.8 is executed. Note 7 |  |
| 8.7.2.1_H.1 |  | TDD sustained data rate performance (Single Carrier) for 256QAM in DL |  | Rel-12 |  |  | C73h |  | UE supporting E-UTRA TDD and 256QAM in DL and UE DL category 13 |  |  |  |  |  |  | Test execution not necessary if 8.7.2.1_H.2 is executed. |  |
| 8.7.2.1_H.2 |  | TDD sustained data rate performance for CA (2DL CA) for 256QAM in DL |  | Rel-12 |  |  | C74h |  | UE supporting E-UTRA TDD and 2DL CA, and supporting 256QAM in DL (UE DL category 11, 12 and 13) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.3 is executed. Note 7 |  |
| 8.7.2.1_H.3 |  | TDD Sustained data rate performance for CA (3DL CA) for 256QAM in DL |  | Rel-12 |  |  | C130h |  | UE supporting E-UTRA TDD and 3DL CA and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.4 is executed. Note 7 |  |
| 8.7.2.1_H.4 |  | TDD Sustained data rate performance for CA (4DL CA) for 256QAM in DL |  | Rel-13 |  |  | C213h |  | UE supporting E-UTRA TDD and 4DL CA and supporting 256QAM in DL (UE DL Category 11, 12,15 and 16) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.5 is executed. Note 7 |  |
| 8.7.2.1_H.5 |  | TDD Sustained data rate performance for CA (5DL CA) for 256QAM in DL |  | Rel-14 |  |  | C332 |  | UE supporting E-UTRA TDD and 5DL CA and supporting 256QAM in DL (UE DL Category 16) |  |  |  |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.2.1_H.6 |  | TDD Sustained data rate performance for CA (6DL CA) for 256QAM in DL |  | Rel-15 |  |  | C331ha |  | UE supporting E-UTRA TDD and 6DL CA and supporting 256QAM in DL (UE DL Category 16) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.7 is executed. Note 7 |  |
| 8.7.2.1_H.7 |  | TDD Sustained data rate performance for CA (7DL CA) for 256QAM in DL |  | Rel-15 |  |  | C346 |  | UE supporting E-UTRA TDD and 7DL CA and supporting 256QAM in DL (UE DL Category 16) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.8 is executed. Note 7 |  |
| 8.7.3.1 |  | FDD sustained data rate performance for EPDCCH scheduling |  | Rel-11 |  |  | C55 |  | UE supporting E-UTRA FDD and EPDCCH |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.7.4.1 |  | TDD sustained data rate performance for EPDCCH scheduling |  | Rel-11 |  |  | C56 |  | UE supporting E-UTRA TDD and EPDCCH |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 8.7.5.1.1 |  | TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) |  | Rel-12 |  |  | C138 |  | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  | TBD |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.2 is executed. Note 7 |  |
| 8.7.5.1.2 |  | TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) |  | Rel-12 |  |  | C139 |  | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  | TBD |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.3 is executed. Note 7 |  |
| 8.7.5.1.3 |  | TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) |  | Rel-12 |  |  | C139a |  | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 11 and 12) |  |  | TBD |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.4 is executed. Note 7 |  |
| 8.7.5.1.4 |  | TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) |  | Rel-12 |  |  | C139b |  | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell and not supporting 256QAM in DL (UE DL category 15) |  |  | TBD |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.5.1.5 |  | TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) |  | Rel-12 |  |  | C139c |  | UE supporting E-UTRA FDD and TDD and 6DL CA with FDD as PCell and not supporting 256QAM in DL (UE DL category 15) |  |  | TBD |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.5.1.6 |  | TDD FDD CA Sustained data rate performance for FDD PCell (7DL CA) |  | Rel-12 |  |  | C369 |  | UE supporting E-UTRA FDD and TDD and 7DL CA with FDD as PCell and not supporting 256QAM in DL (UE DL category 15) |  |  | TBD |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.5.1_H.1 |  | TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) for 256QAM in DL |  | Rel-12 |  |  | C138h |  | UE supporting E-UTRA FDD and TDD and 2DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL (UE DL category 11, 12 and 13) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1_H.2 is executed. Note 7 |  |
| 8.7.5.1_H.2 |  | TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) for 256QAM in DL |  | Rel-12 |  |  | C139h |  | UE supporting E-UTRA FDD and TDD and 3DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1_H.3 is executed. Note 7 |  |
| 8.7.5.1_H.3 |  | TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) for 256QAM in DL |  | Rel-12 |  |  | C139ha |  | UE supporting E-UTRA FDD and TDD and 4DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  |  |  | Test execution not necessary if 8.7.5.1_H.4 is executed. |  |
| 8.7.5.1_H.4 |  | TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) for 256QAM in DL |  | Rel-12 |  |  | C139hb |  | UE supporting E-UTRA FDD and TDD and 5DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  |  |  |  |  |
| 8.7.5.1_H.5 |  | TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) for 256QAM in DL |  | Rel-12 |  |  | C139hc |  | UE supporting E-UTRA FDD and TDD and 6DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  |  |  |  |  |
| 8.7.5.2.1 |  | TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) |  | Rel-12 |  |  | C140 |  | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  | TBD |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.2 is executed. Note 7 |  |
| 8.7.5.2.2 |  | TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) |  | Rel-12 |  |  | C141 |  | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  | TBD |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.3 is executed. Note 7 |  |
| 8.7.5.2.3 |  | TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) |  | Rel-12 |  |  | C141a |  | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 11 and 12) |  |  | TBD |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.4 is executed. Note 7 |  |
| 8.7.5.2.4 |  | TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) |  | Rel-12 |  |  | C141b |  | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 15) |  |  | TBD |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.5.2.5 |  | TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) |  | Rel-12 |  |  | C141c |  | UE supporting E-UTRA FDD and TDD and 6DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 15) |  |  | TBD |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.5.2.6 |  | TDD FDD CA Sustained data rate performance for TDD PCell (7DL CA) |  | Rel-12 |  |  | C370 |  | UE supporting E-UTRA FDD and TDD and 7DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 15) |  |  | TBD |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.5.2_H.1 |  | TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) for 256QAM in DL |  | Rel-12 |  |  | C140h |  | UE supporting E-UTRA FDD and TDD and 2DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 13) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2_H.2 is executed. Note 7 |  |
| 8.7.5.2_H.2 |  | TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) for 256QAM in DL |  | Rel-12 |  |  | C141h |  | UE supporting E-UTRA FDD and TDD and 3DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2_H.3 is executed. Note 7 |  |
| 8.7.5.2_H.4 |  | TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) for 256QAM in DL |  | Rel-12 |  |  | C141hb |  | UE supporting E-UTRA FDD and TDD and 5DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 16) |  |  |  |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.5.2_H.5 |  | TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) for 256QAM in DL |  | Rel-12 |  |  | C142h |  | UE supporting E-UTRA FDD and TDD and 6DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 16) |  |  |  |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.6.1 |  | FDD sustained data rate performance for Dual Connectivity 64QAM |  | Rel-12 |  |  | C171 |  | UE supporting E-UTRA FDD and Dual Connectivity and not supporting 256QAM in DL (UE Category 3, 4, 6, 7, 9, and 10) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.6.2 |  | FDD sustained data rate performance for Dual Connectivity 256QAM |  | Rel-12 |  |  | C173 |  | UE supporting E-UTRA FDD and Dual Connectivity and supporting 256QAM in DL |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.7.1 |  | TDD sustained data rate performance for Dual Connectivity 64QAM |  | Rel-12 |  |  | C172 |  | UE supporting E-UTRA TDD and Dual Connectivity and not supporting 256QAM in DL (UE Category 6, 7, 9, and 10) |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.7.2 |  | TDD sustained data rate performance for Dual Connectivity 256QAM |  | Rel-12 |  |  | C174 |  | UE supporting E-UTRA TDD and Dual Connectivity and supporting 256QAM in DL |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  | 2Rx, 4Rx |  | Note 7 |  |
| 8.7.9.2 |  | FDD sustained data rate performance for 4 layer MIMO (single carrier) |  | Rel-10 |  |  | C226 |  | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 6 and 7 and UE DL category 13) |  |  | One "Test Number" to be performed. The selected band shall lead to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.9.3 |  | FDD sustained data rate performance for 4 layer MIMO (2DL CA) |  | Rel-10 |  |  | C227 |  | UE supporting 2DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band contiguous DL CA or inter-band DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  | One "Test Number" to be performed. The selected CA configuration shall lead to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  | Rel-11 |  |  | C228 |  | UE supporting 2DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band non-contiguous DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  |  |  |  |  |  |
| 8.7.9.4 |  | FDD sustained data rate performance for 4 layer MIMO (3DL CA) |  | Rel-10 |  |  | C229 |  | UE supporting 3DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  | Rel-11 |  |  | C230 |  | UE supporting 3DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 and 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  |  |  |  |  |  |
| 8.7.9.5 |  | FDD sustained data rate performance for 4 layer MIMO (4DL CA) |  | Rel-11 |  |  | C236 |  | UE supporting 4DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 4DL with CA configurations in Table 4.1-4 (UE DL category 15, 16, 18 and 19) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.9.6 |  | FDD sustained data rate performance for 4 layer MIMO (5DL CA) |  | Rel-11 |  |  | C237 |  | UE supporting 5DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 15, 16, 18 and 19) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  | Rel-12 |  |  | C238 |  | UE supporting 5DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 15, 16, 18 and 19) |  |  |  |  |  |  |  |  |
| 8.7.10.2 |  | TDD sustained data rate performance for 4 layer MIMO (single carrier) |  | Rel-10 |  |  | C239 |  | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 6 and 7 and UE DL category 13) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.10.3 |  | TDD sustained data rate performance for 4 layer MIMO (2DL CA) |  | Rel-10 |  |  | C240 |  | UE supporting 2DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band contiguous DL CA or inter-band DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  | Rel-11 |  |  | C241 |  | UE supporting 2DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band non-contiguous DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  |  |  |  |  |  |
| 8.7.10.4 |  | TDD sustained data rate performance for 4 layer MIMO (3DL CA) |  | Rel-10 |  |  | C242 |  | UE supporting 3DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  | Rel-11 |  |  | C243 |  | UE supporting 3DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  |  |  |  |  |  |
| 8.7.10.5 |  | TDD sustained data rate performance for 4 layer MIMO (4DL CA) |  | Rel-11 |  |  | C244 |  | UE supporting 4DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 4DL with CA configurations in Table 4.1-4 (UE DL category 16, 18 and 19) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.10.6 |  | TDD sustained data rate performance for 4 layer MIMO (5DL CA) |  | Rel-11 |  |  | C245 |  | UE supporting 5DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 18 and 19) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
|  |  |  |  | Rel-12 |  |  | C246 |  | UE supporting 5DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 18 and 19) |  |  |  |  |  |  |  |  |
| 8.7.11.2 |  | TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) |  | Rel-12 |  |  | C247 |  | UE supporting 2DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.11.3 |  | TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) |  | Rel-12 |  |  | C248 |  | UE supporting 3DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.11.4 |  | TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) |  | Rel-12 |  |  | C249 |  | UE supporting 4DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE DL category 15, 16, 18 and 19) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.11.5 |  | TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) |  | Rel-12 |  |  | C250 |  | UE supporting 5DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE DL category 16, 18 and 19) |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |  |
| 8.7.12.1.2.1 |  | LAA sustained data rate performance with FDD PCell with 2DL CA (2Rx) |  | Rel-13 |  |  | C209 |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |  |  |
| 8.7.12.1.2.2 |  | LAA sustained data rate performance with FDD PCell for 4 Layer MIMO (2DL CA) |  | Rel-13 |  |  | C328 |  | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell abd 4-layer spatial multiplexing |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |  |  |
| 8.7.12.2.2.1 |  | LAA sustained data rate performance with TDD PCell with 2DL CA (2Rx) |  | Rel-13 |  |  | C210 |  | UE supporting E-UTRA TDD and downlink LAA |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |  |  |
| 8.7.12.2.2.2 |  | LAA sustained data rate performance with TDD PCell for 4 Layer MIMO (2DL CA) |  | Rel-13 |  |  | C329 |  | UE supporting E-UTRA TDD and downlink LAA with FDD as Pcell abd 4-layer spatial multiplexing |  |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |  |  |
| Many unchanged rows skipped |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MBMS Performance Testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 |  | FDD MBMS performance (Fixed Reference Channel) |  | Rel-9 |  | C03 |  |  | UE supporting E-UTRA FDD and MBMS |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 10.1_1 |  | FDD MBMS performance (Fixed Reference Channel) (Release 13 and forward) |  | Rel-13 |  | C03 |  |  | UE supporting E-UTRA FDD and MBMS |  |  | Performed once |  |  |  |  |  |
| 10.2 |  | TDD MBMS performance (Fixed Reference Channel) |  | Rel-9 |  | C04 |  |  | UE supporting E-UTRA TDD and MBMS |  |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |  |  |
| 10.2_1 |  | TDD MBMS performance (Fixed Reference Channel) (Release 13 and forward) |  | Rel-13 |  | C04 |  |  | UE supporting E-UTRA TDD and MBMS |  |  | Performed once |  |  |  |  |  |
| V2X Sidelink Performance Testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14.2 |  | Demodulation of PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Rel-14 |  | C313 |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  |  |  |  |
| 14.3 |  | Demodulation of PSCCH / Non-concurrent with E-UTRA uplink transmissions |  | Rel-14 |  | C313 |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  |  |  |  |
| 14.4 |  | Power imbalance performance with two links / Non-concurrent with E-UTRA uplink transmissions |  | Rel-14 |  | C313 |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  |  |  |  |
| 14.6 |  | Demodulation of PSSCH with eNB based synchronization |  | Rel-14 |  | C320 |  |  | UE supporting E-UTRA and V2X Sidelink communication |  |  | E16 |  | FDD,TDD |  |  |  |
| 14.7 |  | Soft buffer test |  | Rel-14 |  | C313 |  |  | UE supporting V2X Sidelink communication |  |  | D14 |  | TDD |  |  |  |
| 14.8 |  | PSCCH/PSSCH decoding capability test / Non-concurrent with E-UTRA uplink transmissions |  | Rel-14 |  | C321 |  |  | UE supporting V2X Sidelink communication |  |  |  |  |  |  |  |  |
| 14.9 |  | Sustained downlink data rate with active sidelink |  | Rel-14 |  | C320 |  |  | UE supporting E-UTRA and V2X Sidelink communication |  |  | E16 |  | FDD,TDD |  |  |  |
| Note 1: Due to UE capability signalling for UL 64QAM is introduced from Rel-12, this test case can optionally be executed with a Rel-12 UE. Note 2: For a transition period until RAN5#72, this condition in version 13.0.0 of 36.521-2 shall be used. This is to ensure no test coverage is lost before the UL 64QAM test case becomes available. Note 3: Equivalent aggregated bandwidth is defined as: . Where is number of CCs, and is MIMO layer and bandwidth of CC . The number of MIMO layer for CC in each CA configuration is according to Table A.4.6.1-3, Table A.4.6.2-3, Table A.4.6.3-3, Table A.4.6.3-4 or Table A.4.6.3-5. Note 4: For a transition period until RAN5#83, this condition in version 15.3.1 of 36.521-2 shall be used. This is to ensure no test coverage is lost before the UL 256QAM test cases become available. Note 5: Void. Note 6: The categories for which FGI 103/104 are mandated are (UE category >=11 OR UE DL Category = 11 OR UE DL Category = 12 OR UE DL Category >= 14) according to TS 36.331. The UE DL Categories higher than 14 are not explicitly listed in the corresponding expression in Table 4.1-1a because in this version of core specification UE DL Category > 14 requires supporting of UE Category 11 or 12. Note 7: Skipping 2RX testing if UE is verified with four RX antenna ports in operating bands where it is equipped with four RX antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>