3GPP TSG-RAN5 Meeting #97	R5-226406

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5135 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 9.2.3.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step 8, UE moves from Cell B (PLMN1) to Cell C (PLMN2) and at Step 11, SS transmits TAU Accept message including PLMN1 in the list of equivalent PLMNs. Hence UE would not perform IMS reregistration post Step 12A and Step 12Aa1 will be redundant. Whereas at step 16, UE moves from Cell C (PLMN2) to Cell D (PLMN3) and at Step 19, SS transmits TAU Accept message without the list of equivalent PLMNs. Hence Cell D would be considered as VPLMN and UE would like to perform IMS reregistration post Step 20A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | IMS Reregistration procedure moved from Step12Aa1 to Step20Aa1 (new step after Step20A) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with R5s221216 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Modified Section>

##### 9.2.3.1.4	Normal tracking area update / List of equivalent PLMNs in the TRACKING AREA UPDATE ACCEPT message

9.2.3.1.4.1	Test Purpose (TP)

(1)

with { UE in EMM-TRACKING-AREA-UPDATING-INITIATED state }

ensure that {

when { the UE receives TRACKING AREA UPDATE ACCEPT message including a list of equivalent PLMNs }

then { the UE stores correctly the list and considers a forbidden PLMN if the forbidden PLMN is included in the equivalent list }

}

(2)

with { UE in EMM-TRACKING-AREA-UPDATING-INITIATED state }

ensure that {

when { the UE receives TRACKING AREA UPDATE ACCEPT message without a list of equivalent PLMNs }

then { the UE deletes the stored list and applies a normal PLMN selection process }

}

9.2.3.1.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.3.2.4.

[TS 24.301, clause 5.5.3.2.4]

The MME may also include of list of equivalent PLMNs in the TRACKING AREA UPDATE ACCEPT message. Each entry in the list contains a PLMN code (MCC+MNC). The UE shall store the list as provided by the network, after having removed from the list any PLMN code that is already in the list of forbidden PLMNs. In addition, the UE shall add to the stored list the PLMN code of the registered PLMN that sent the list. The UE shall replace the stored list on each receipt of the TRACKING AREA UPDATE ACCEPT message. If the TRACKING AREA UPDATE ACCEPT message does not contain a list, then the UE shall delete the stored list.

9.2.3.1.4.3	Test description

9.2.3.1.4.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, PLMN1);

-	cell B (belongs to TAI-2, PLMN1);

-	cell C (belongs to TAI-3, PLMN2);

-	cell D (belongs to TAI-4, PLMN3);

- system information combination 2 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cell B and C.

UE:

-	the UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE last attempted to register on cell D and received reject cause "forbidden PLMN" (so the "forbidden PLMN list" contains PLMN3).

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508[18].

9.2.3.1.4.3.2	Test procedure sequence

Table 9.2.3.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Set the cell type of cell A to the ''Non-Suitable cell''. Set the cell type of cell B to the ''Serving cell''. Set the cell type of cell C to the '' Non-Suitable off cell''. Set the cell type of cell D to the '' Non-Suitable off cell''. | - | - | - | - |
| 2 | The UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 3 | Void | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | SS responds with a TRACKING AREA UPDATE ACCEPT message including PLMN2 and PLMN3 in the list of equivalent PLMNs. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 6 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 6A | SS Releases the RRC Connection. | - | - | - | - |
| 7 | Set the cell type of cell A to the ''Non-Suitable off cell''. Set the cell type of cell B to the ''Non-Suitable cell''. Set the cell type of cell C to the '' Suitable cell''. | - | - | - | - |
| 8 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on cell C (PLMN2)? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 9 | Void | - | - | - | - |
| 10 | Void | - | - | - | - |
| 11 | The SS transmits a TRACKING AREA UPDATE ACCEPT message including PLMN1 and PLMN3 in the list of equivalent PLMNs. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 12 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 12A | SS Releases the RRC Connection. | - | - | - | - |
| 12Aa1 | Void | - | - | - | - |
| 13 | Set the cell type of cell C to the ''Non-Suitable cell''. Set the cell type of cell D to the '' Serving cell''. Note: Cell D (PLMN3) belongs to the forbidden PLMN. | - | - | - | - |
| 14 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on cell D (PLMN3) in next 30 seconds? | --> | TRACKING AREA UPDATE REQUEST | 1 | F |
| 15 | The UE is switched to manual PLMN selection mode and is made to select PLMN3 in order to remove PLMN3 in the forbidden PLMN list in the UE. | - | - | - | - |
| 16 | The UE transmits a TRACKING AREA UPDATE REQUEST message on cell D (PLMN3). | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 17 | Void | - | - | - | - |
| 18 | Void | - | - | - | - |
| 19 | The SS transmits a TRACKING AREA UPDATE ACCEPT message without the list of equivalent PLMNs. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 20 | Check: Does the UE transmit a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | 1 | P |
| 20A | SS releases the RRC connection | - | - | - | - |
|  | EXCEPTION: Step 20Aa1 describes optional behaviour that depends on the UE capability. | - | - | - | - |
| 20Aa1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS VPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| 21 | The UE is switched back to automatic PLMN selection mode. | - | - | - | - |
| 21A | SS waits for 5 minutes to allow UE to complete automatic PLMN search |  |  |  |  |
| 22 | Set the cell type of cell D to ''Non-Suitable cell''. Set the cell type of cell B to ”Serving cell”. Set the cell type of cell C to ”Suitable neighbour intra-frequency cell”. | - | - | - | - |
| 23 | Void | - | - | - | - |
| 24 | Void | - | - | - | - |
| 24A | Void | - | - | - | - |
| 25 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message on Cell B (PLMN1)? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 26 | Void | - | - | - | - |
| 27 | SS responds with TRACKING AREA UPDATE ACCEPT message | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 28 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 29 | SS Releases the RRC Connection. | - | - | - | - |
|  | EXCEPTION: Step 29a1 describes optional behaviour that depends on the UE capability. | - | - | - | - |
| 29a1 | The UE may attempt the generic test procedure for IMS Re-registration on EPS HPLMN as specified in TS 36.508 [18] subclause 4.5A.30. | - | - | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA Idle ( E1) according to TS 36.508. | - | - | - | - |

9.2.3.1.4.3.3	Specific message contents

Table 9.2.3.1.4.3.3-1: Void

Table 9.2.3.1.4.3.3-2: Void

Table 9.2.3.1.4.3.3-3: Message TRACKING AREA UPDATE ACCEPT (step 5, Table 9.2.3.1.4.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | - | Includes MCC and MNC digits for PLMN2 and PLMN3. |  |

Table 9.2.3.1.4.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 11, Table 9.2.3.1.4.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | - | Includes MCC and MNC digits for PLMN1 and PLMN3. |  |

Table 9.2.3.1.4.3.3-5: Void

Table 9.2.3.1.4.3.3-6: Void

<End of Modified Section>