3GPP TSG-RAN5 Meeting #97	R5-226023

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3212 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NAS MICO test case 9.1.5.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To meet the condition “UE is in 5GMM-DEREGISTERED state and is switched off” In TP1, UE state in preamble should be 0N-B. The steps for REGISTRATION COMPLETE message and PDU session establishment are missing. According to below requirement in TS 38.304, when MICO mode is activated and UE enters RRC_Idle state after step 18, UE will not pefrom idle mode tasks, such as cell or PLMN reselection. Therefore, the initial condition of TP3 and TP4 will not be performed. When the UE is in RRC_IDLE state, upper layers may deactivate AS layer when MICO mode is activated as specified in TS 24.501 [14]. When MICO mode is activated, the AS configuration (e.g. priorities provided by dedicated signalling) is kept and all running timers continue to run but the UE need not perform any idle mode tasks. If a timer expires while MICO mode is activated it is up to the UE implementation whether it performs the corresponding action immediately or the latest when MICO mode is deactivated. When MICO mode is deactivated, the UE shall perform all idle mode tasks. To achive this, MICO mode needs to be deactivated by sending mobile originated signalling or user data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | State 0-A was replaced by 0N-B in preamble. Step 17A & 17B were added. Step 21A was added to trigger MICO mode deactivation. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

9.1.5.1.4	Initial registration / 5GS services / MICO mode / TAI list handling

9.1.5.1.4.1	Test Purpose (TP)

(1)

with { The UE is in 5GMM-DEREGISTERED state and is switched off }

ensure that {

when { the UE supports MICO mode and requests the use of MICO mode }

then { the UE includes the MICO indication IE in the REGISTRATION REQUEST message }

}

(2)

with { The UE has received REGISTRATION ACCEPT message }

ensure that {

when { the REGISTRATION ACCEPT message included MICO indication IE indicating “all PLMN registration area allocated” }

then { the UE treats all TAIs in the current PLMN as a registration area and deletes its old TAI list }

}

(3)

with { The UE detecting a better NG cell in same PLMN }

ensure that {

when { the UE treats all TAIs in the current PLMN as a registration area and has deleted its old TAI list as a result of REGISTRATION ACCEPT message included MICO indication IE indicating “all PLMN registration area allocated” }

then { the UE does not perform the REGISTRATION procedure for mobility }

}

(4)

with { The UE detecting a better NG cell in a different PLMN }

ensure that {

when { the UE transmits the REGISTRATION REQUEST }

then { the UE sets the IE 5GS registration type to “mobility registration updating” and performs a REGISTRATION procedure for Mobility }

}

9.1.5.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 5.5.1.2.2 and 5.5.1.2.4. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.2]

…

If the UE supports MICO mode and requests the use of MICO mode, then the UE shall include the MICO indication IE in the REGISTRATION REQUEST message.

…

[TS 24.501, clause 5.5.1.2.4]

…

The AMF shall include the MICO indication IE in the REGISTRATION ACCEPT message only if the MICO indication IE was included in the REGISTRATION REQUEST message, the AMF supports and accepts the use of MICO mode. If the AMF supports and accepts the use of MICO mode, the AMF may indicate "all PLMN registration area allocated" in the MICO indication IE in the REGISTRATION ACCEPT message. If "all PLMN registration area allocated" is indicated in the MICO indication IE, the AMF shall not assign and include the TAI list in the REGISTRATION ACCEPT message. If the REGISTRATION ACCEPT message included an MICO indication IE indicating "all PLMN registration area allocated", the UE shall treat all TAIs in the current PLMN as a registration area and delete its old TAI list.

…

9.1.5.1.4.3	Test description

9.1.5.1.4.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A, NGC Cell C and NGC Cell E are configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in TS 38.508-1 [4].

UE:

None.

Preamble:

-	The UE is in state Switched OFF (State 0N-B) as per TS 38.508-1 [4] Table 4.4A.2-0.

9.1.5.1.4.3.2	Test procedure sequence

Table 9.1.5.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS configures: - NGC Cell A as the "Non-suitable “Off” cell. - NGC Cell C as the "Non-suitable "Off" cell". - NGC Cell E as the "Non-suitable "Off" cell". | - | - | - | - |
| 1 | The UE is switched ON. | - | - | - | - |
| 2 | The user requests enabling of MICO mode by MMI or AT command. | - | - | - | - |
| 3 | The SS configures: - NGC Cell A as the "Serving cell". - NGC Cell C as the "Non-suitable "Off" cell". - NGC Cell E as the "Non-suitable "Off" cell". | - | - | - | - |
| 4-6 | The UE establishes an RRC connection by executing steps 2–4 of Table 4.5.2.2-2 in TS38.508-1 [4]. | - |  | - | - |
| 7 | Check: Does the UE transmit a REGISTRATION REQUEST message including IE MICO indication. | --> | REGISTRATION REQUEST | 1 | P |
| 8-16 | Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38508-1 [4] are performed. | - |  | - | - |
| 17 | SS transmits a REGISTRATION ACCEPT message that includes IE MICO indication. | <-- | REGISTRATION ACCEPT | - | - |
| 17A | The UE transmits a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| 17B | Steps 3-6a1 of Table 4.5A.2.2.2-1 in TS 38.508-1 [4] are performed. | - | - | - | - |
| 18 | The SS releases the RRC Connection. | - |  | - | - |
| 19 | The SS configures: - NGC Cell A as the "Non-suitable “Off” cell". - NGC Cell C as the "Serving cell". - NGC Cell E as the "Non-suitable “Off” cell". | - |  | - | - |
| 20 | Check: Does the UE transmit a RRCSetupRequest on NGC Cell C? This is checked for 60s | --> | NR RRC: RRCSetupRequest | 2, 3 | F |
| 21 | The SS configures: - NGC Cell A as the "Non-suitable “Off” cell". - NGC Cell C as the "Non-suitable “Off” cell". - NGC Cell E as the "Serving cell". | - | - | - | - |
| - | The following messages are to be observed on NGC Cell E unless explicitly stated otherwise. | - | - | - | - |
| 21A | Cause the UE to request sending of uplink data. (Note 1) | - | - | - | - |
| 22-24 | The UE establishes an RRC connection by executing steps 2–4 of Table 4.5.2.2-2 in TS38.508-1 [4]. | - | - | - | - |
| 25 | Check: Does the UE transmit a REGISTRATION REQUEST message with IE 5GS registration type set to “mobility registration updating”? | --> | REGISTRATION REQUEST | 4 | P |
| 26-27 | Steps 4–5 of Table 4.9.5.2.2-1 in TS38.508-1 [4] are performed. | - | - | - | - |
| Note 1: The request of sending uplink data may be performed by MMI or AT command. |  |  |  |  |  |

9.1.5.1.4.3.3	Specific message contents

Table 9.1.5.1.4.3.3-1: REGISTRATION REQUEST (step 6 Table 9.1.5.1.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | ‘001’B | Initial registration | INITIAL |
| MICO indication |  | ‘0000’B |  |  |

Table 9.1.5.1.4.3.3-2: REGISTRATION ACCEPT (step 17 Table 9.1.5.1.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MICO Indication |  | ‘0001’B | All PLMN registration area allocated |  |
| TAI list |  | Not present |  |  |

Table 9.1.5.1.4.3.3-3: REGISTRATION REQUEST (step 24 Table 9.1.5.1.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | ‘010’B | Mobility registration updating | MOBILITY |

[End of change]