3GPP TSG-RAN5 Meeting #97	R5-226310

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0480 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS testcases 10.7 and 10.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN5#96e, according to action point AP#96.01 NR5GC testcases 11.4.3 and 11.4.5 were to be updated to include the conformance requirements for IMS testcases 10.7 and 10.8 respectively. Also the IMS testcases 10.7 and 10.8 were to be voided. This needs to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Voided testcases 10.7 and 10.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Action point AP#96.01 will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.7, 10.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.6	Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS

10.6.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to start an emergency call }

then { UE performs IMS registration and sets up emergency call }

}

(2)

with { Emergency call ongoing }

ensure that {

when { Network sends BYE }

then { UE sends 200 OK for BYE }

}

(3)

with { Emergency call being terminated }

ensure that {

when { UE is made to initiate a non UE detectable emergency call }

then { UE sends correctly composed INVITE request for normal voice call }

}

(4)

with { UE having sent INVITE for normal voice call }

ensure that {

when { UE receives 380 Alternative Service response }

then { UE acknowledges and initiates an emergency call }

}

(5)

with { Emergency call ongoing }

ensure that {

when { Network sends BYE }

then { UE sends 200 OK for BYE }

}

10.6.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.3.1]:

In the event the UE receives a 380 (Alternative Service) response to an initial INVITE request the response containing a P-Asserted-Identity header field with a value equal to the value of the last entry of the Path header field value received during registration and the response containing a 3GPP IM CN subsystem XML body that includes an <ims-3gpp> element, including a version attribute, with an <alternative-service> child element with the <type> child element set to "emergency" (see table 7.6.2), the UE shall select a domain in accordance with the conventions and rules specified in 3GPP TS 22.101 [1A] and 3GPP TS 23.167 [4B], and:

-	if the CS domain is selected, the UE behaviour is defined in subclause 7.1.2 of 3GPP TS 23.167 [4B] and, where appropriate, in the access technology specific annex; and

-	if the IM CN subsystem is selected, the UE shall apply the procedures in subclause 5.1.6 with the exception of selecting a domain for the emergency call attempt.

NOTE 10:	The last entry on the Path header field value received during registration is the value of the SIP URI of the P-CSCF. If there are multiple registration flows associated with the registration, then the UE has received from the P-CSCF during registration multiple sets of Path header field values. The last entry of the Path header field value corresponding to the flow on which the 380 (Alternative Service) response was received is checked.

[TS 24.229 clause 5.1.6.2A]:

The UE shall perform a new initial emergency registration, as specified in subclause 5.1.6.2, if the UE determines that:

-	it has previously performed an emergency registration which has not yet expired; and

-	it has obtained an IP address from the serving IP-CAN, as specified in subclause 9.2.1, different than the IP address used for the emergency registration.

10.6.3	Test description

10.6.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

10.6.3.2	Test procedure sequence

Table 10.6.3.2-1: Main Behaviour

| St |  | Procedure |  | Message Sequence |  |  |  | TP |  | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S |  | Message |  |  |  |  |  |
| 1 |  | UE is made to make an emergency call |  | - |  | - |  | - |  | - |  |
| 2-11 |  | Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. |  | - |  | - |  | - |  | - |  |
| - |  | EXCEPTION: In parallel to the events described in steps 12-16 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. |  | - |  | - |  | - |  | - |  |
| 12-15 |  | Steps 1-4 of Annex A.6 happens |  | - |  | - |  | - |  | - |  |
| 16 |  | Step 5 of Annex A.6 happens Check: Does the UE send an ACK? |  | --> |  | ACK |  | 1 |  | P |  |
| 17 |  | Step 1 of Annex A.8 happens |  | <-- |  | BYE |  | - |  | - |  |
| 18 |  | Step 2 of Annex A.8 happens Check: Does the UE send 200 OK for the BYE request and ends the call? |  | --> |  | 200 OK |  | 2 |  | P |  |
| 18A-18C |  | Steps 3b1-3b34 of generic procedure specified in Table 4.9.12B.2.2-1 of TS 38.508-1 [21] are performed to release emergency speech bearer and Keep emergency PDU session. |  | - |  | - |  | - |  | - |  |
| 19 |  | UE is made to attempt an IMS voice call |  | - |  | - |  | - |  | - |  |
| 20 |  | UE sends INVITE with the first SDP offer Check: Does the UE send correctly composed INVITE request for normal voice call? |  | --> |  | INVITE |  | 3 |  | P |  |
| 21 |  | SS responds with 380 Alternative Service |  | <-- |  | 380 Alternative Service |  | - |  | - |  |
| 22 |  | UE acknowledges the receipt of 380 Alternative Service response |  | --> |  | ACK |  | - |  | - |  |
| - |  | EXCEPTION: In parallel to the events described in steps 23-27 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. |  | - |  | - |  | - |  | - |  |
| 23 |  | Step 1 of Annex A.6 happens Check: Does UE send INVITE message for emergency call? |  | --> |  | INVITE |  | 4 |  | P |  |
| 24-27 |  | Steps 2-5 of Annex A.6 happens |  | - |  | - |  | - |  | - |  |
| 28 |  | Step 1 of Annex A.8 happens |  | <-- |  | BYE |  | - |  | - |  |
| 29 |  | Step 2 of Annex A.8 happens Check: Does the UE send 200 OK for the BYE request and ends the call? |  | --> |  | 200 OK |  | 5 |  | P |  |

10.6.3.3	Specific message contents

Table 10.6.3.3-1: INVITE for non UE detectable emergency call (step 20, table 10.6.3.2-1)

| Derivation path: TS 34.229-1 [2], Annex A.2.1, Conditions A1, A3, A4, A28, A29, and A30 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | The following SDP types and values. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] Time description: t= (start-time) (stop-time) Media description: m=audio (transport port) [Note 2] c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: EVS codec shall be present in the media attributes, optionally including channel number "/1". |  | TS 24.229 [7] |

Table 10.6.3.3-2: 380 Alternative Service (step 21, table 10.6.3.2-1)

| Derivation path: TS 34.229-1 [2], Annex A.4.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Message-body |  | <?xml version="1.0" encoding="UTF-8"?> <ims-3gpp version="1"> <alternative-service> <type>emergency</type> <reason/> <action>emergency-registration</action> </alternative-service> </ims-3gpp> |  | TS 24.229 [7] |

Table 10.6.3.3-3: ACK (step 22, table 10.6.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.7, Conditions A1 and A4

## 10.7	 Void

## 10.8	 Void

## 10.9	Emergency call without emergency registration / UE credentials are not accepted / 5GS

10.9.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and being made to initiate an emergency call }

ensure that {

when { UE attempts IMS emergency registration and network declines by sending 403 Forbidden }

then { UE initiates and completes IMS emergency calls on non-protected ports }

}

10.9.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 33.203 clause 7.1]

The P-CSCF is allowed to receive only REGISTER messages, messages relating to emergency services in accordance with TS 23.167 [31] and TS 24.229 [8], and error messages related to unprotected messages on unprotected ports. All other messages not arriving on a protected port shall be either discarded or rejected by the P-CSCF.

4.	The UE is allowed to receive only the following messages on an unprotected port:

-	responses to unprotected REGISTER messages;

-	messages relating to emergency services in accordance with TS 23.167 [31] and TS 24.229 [8];

-	error messages related to unprotected messages.

All other messages not arriving on a protected port shall be rejected or silently discarded by the UE.

[TS 24.229 clause 5.1.6.1]

If the IM CN subsystem is selected and the UE is currently attached to its home operator's network (e.g. HPLMN) and the UE is currently registered and the IP-CAN defines emergency bearers and the core network has indicated that it supports emergency bearers, the UE shall:

1)	perform an initial emergency registration, as described in subclause 5.1.6.2; and

2)	attempt an emergency call as described in subclause 5.1.6.8.3.

10.9.3	Test description

10.9.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS.

10.9.3.2	Test procedure sequence

Table 10.9.3.2-1: Main Behaviour

| St |  | Procedure |  | Message Sequence |  |  |  | TP |  | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S |  | Message |  |  |  |  |  |
| 1 |  | Make the UE to initiate an emergency call. |  | - |  | - |  | - |  | - |  |
| 2-9 |  | Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1[21] are performed. |  | - |  | - |  | - |  | - |  |
|  |  | EXCEPTION: In parallel to the events described in steps 10-11 below the events specified in steps 1a1 of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place. |  | - |  | - |  | - |  | - |  |
| 10-11 |  | Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. |  | - |  | - |  | - |  | - |  |
| 12 |  | UE sends REGISTER (Step 1 of Annex A.3) |  | --> |  | REGISTER |  | - |  | - |  |
| 13 |  | SS sends 401 Unauthorized (Step 2 of Annex A.3) |  | <-- |  | 401 Unauthorized |  | - |  | - |  |
| 14 |  | UE sends REGISTER (Step 3 of Annex A.3) |  | --> |  | REGISTER |  | - |  | - |  |
| 15 |  | SS sends 403 Forbidden The following messages are exchanged on non protected port. |  | <-- |  | 403 Forbidden |  |  |  |  |  |
| - |  | EXCEPTION: In parallel to steps 16-20 below, steps 16-18 of generic procedure specified in Table 4.9.12.2.2-1 of TS 38.508-1 [21] takes place. |  | - |  | - |  | - |  | - |  |
| 16 |  | Check: Does the UE send a correctly composed INVITE request? (Step 1 of annex A.6) |  | --> |  | INVITE |  | 1 |  | P |  |
| 17 |  | SS sends 100 Trying. (Step 2 of annex A.6) |  | <-- |  | 100 Trying |  | - |  | - |  |
| 18 |  | SS sends 180 Ringing. (Step 3 of annex A.6) |  | <-- |  | 180 Ringing |  | - |  | - |  |
| 19 |  | SS sends 200 OK. (Step 4 of annex A.6) |  | <-- |  | 200 OK |  | - |  | - |  |
| 20 |  | Check: Does the UE send ACK? (Step 5 of annex A.6) |  | --> |  | ACK |  | 1 |  | P |  |
| 21 |  | SS sends BYE. (Step 1 of annex A.8) |  | <-- |  | BYE |  | - |  | - |  |
| 22 |  | UE sends 200 OK. (Step 1 of annex A.8) |  | --> |  | 200 OK |  | - |  | - |  |

10.9.3.3	Specific message contents

Table 10.9.3.3-1: 403 Forbidden (step 15, table 10.9.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.3.2, Conditions A1.

Table 10.9.3.3-2: INVITE (step 16, table 10.9.3.2-1)

Derivation Path: Annex A.6, Step 1, Conditions A6, A28 of TS 34.229-1 [2] cl A.2.1

Table 10.9.3.3-3: 180 Ringing (step 18, table 10.9.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.6, Conditions A4, A7, A14.

Table 10.9.3.3-4: 200 OK (step 19, table 10.9.3.2-1)

Derivation Path: Annex A.6, Step 4, Conditions A7, A10, A22 of TS 34.229-1 [2] cl A.3.1.

Table 10.9.3.3-5: BYE (step 21, table 10.9.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8, Conditions A3, A6.