3GPP TSG-RAN5 Meeting #97	R5-226534

# Toulouse, France, 14th – 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0438 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of MU for PC1 Demod |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MU for Demod with PC1 is not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The MU for Demod with PC1 has been documented. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Demod with PC1 cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated discussion paper in R5-226533 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

Annex D: Acceptable uncertainty of test system for test cases defined in TS 38.521-4 for radiative testing

Editor’s note: The MU tables in D-1, D-2, and D-3 serve as sample, consolidated baseline tables for demodulation test cases and can be removed once the MU tables customized for each TS 38.521-4 test case have been finalized.

This annex contains suggested uncertainties for each test case in TS 38.521-4.

The baseline MU table for Mode 1 (conditions with external noise) is shown in Table D-1 for baseband-combining implementation and in Table D-2 for external-combining implementation.

Table D-1: Uncertainty Contributions for Mode 1 Demodulation Test Cases for PC1 and PC3 (Baseband-Combining Implementation)

| UID | Uncertainty source | Uncertainty value |  | Distribution of the probability | Divisor |  | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Signal-to-noise ratio uncertainty |  |  |  |  |  |  |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |
| 1 | Positioning misalignment |  |  | N/A | N/A |  |  |
| 2 | Measure distance uncertainty |  |  | N/A | N/A |  |  |
| 3 | Quality of Quiet Zone |  |  | N/A | N/A |  |  |
| 4 | Mismatch |  |  | N/A | N/A |  |  |
| 5 | Standing wave between the DUT and measurement antenna |  |  | N/A | N/A |  |  |
| 6 | gNB emulator SNR uncertainty | 0.3 |  | Note 3 | 1.96 |  | 0.153 |
| 7 | Phase curvature |  |  | N/A | N/A |  |  |
| 8 | Amplifier uncertainties |  |  | N/A | N/A |  |  |
| 9 | Random uncertainty |  |  | N/A | N/A |  |  |
| 10 | Influence of the XPD |  |  | N/A | N/A |  |  |
| 11 | Insertion Loss Variation |  |  | N/A | N/A |  |  |
| 12 | RF leakage (from measurement antenna to the receiver/transmitter) |  |  | N/A | N/A |  |  |
| 13 | Multiple measurement antenna uncertainty |  |  | N/A | N/A |  |  |
| 14 | DUT repositioning |  |  | N/A | N/A |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |
| 15 | Mismatch |  |  | N/A | N/A |  |  |
| 16 | Amplifier Uncertainties |  |  | N/A | N/A |  |  |
| 17 | Misalignment of positioning System |  |  | N/A | N/A |  |  |
| 18 | Uncertainty of the Network Analyzer |  |  | N/A | N/A |  |  |
| 19 | Uncertainty of the absolute gain of the calibration antenna |  |  | N/A | N/A |  |  |
| 20 | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  |  | N/A | N/A |  |  |
| 21 | Phase centre offset of calibration antenna |  |  | N/A | N/A |  |  |
| 22 | Quality of quiet zone for calibration process |  |  | N/A | N/A |  |  |
| 23 | Standing wave between reference calibration antenna and measurement antenna |  |  | N/A | N/A |  |  |
| 24 | Influence of the calibration antenna feed cable |  |  | N/A | N/A |  |  |
| 25 | Insertion Loss Variation |  |  | N/A | N/A |  |  |
| Total Signal-to-Noise ratio uncertainty |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 0.153 |
| Other contributors affecting test result |  |  |  |  |  |  |  |
| 27 | gNB emulator fading model impairments |  | 0.5 for 1Tx 0.7 for 2Tx | Note 3 | 1.96 |  | 0.255 for 1Tx 0.357 for 2Tx |
| 28 | AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig (Note 4) |  | 3.6 | Note 3 | 1.96 |  | 1.837 |
| 29 | SNR uncertainty due to finite test time |  | 0.3 for PDSCH and Doppler < 100 Hz 0.0 for PDSCH and Doppler ≥ 100 Hz 0.4 for PDCCH | Note 3 | 1.96 |  | 0.153 for PDSCH and Doppler < 100 Hz 0.0 for PDSCH and Doppler ≥ 100 Hz 0.204 for PDCCH |
|  | Systematic uncertainties |  |  |  |  | Value |  |
| 26 | Impact on non-ideal isolation between branches for the wireless cable mode |  |  |  |  | 0.45 (Note 1) 0.60 (Note 2) |  |
| Overall system uncertainty |  |  |  |  |  | Value |  |
|  |  |  |  |  |  | Note 5 |  |
| Note 1: applies to Rank 2 test cases for FR2a, FR2b, and FR2c Note 2: applies to Rank 1 test cases for FR2a, FR2b, and FR2c Note 3: Divisor of 1.96 is applied as the uncertainty value is based on 95% confidence level k=1.96. Note 4: AWGN flatness and signal flatness has x 0.25 effect on the required SNR. This sensitivity factor shall be considered in the calculation of the test case specific uncertainty. Note 5: Example calculation for fading conditions: Overall system uncertainty for fading conditions comprises five quantities: 1. Total Signal-to-noise ratio uncertainty 2. gNB emulator fading model impairments 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time 5. Impact on non-ideal isolation between branches for the wireless cable mode Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared. Item 5 is systematic and is added: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Overall system uncertainty = 1.96 x SQRT (Total Signal-to-noise ratio uncertainty 2 + gNB emulator fading model impairments2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time2) + Impact on non-ideal isolation between branches for the wireless cable model |  |  |  |  |  |  |  |

Table D-1a: Overall system uncertainty for Mode 1 Demodulation Test Cases for PC1 and PC3 (Baseband-Combining Implementation)

| Overall system uncertainty | Value |
| --- | --- |
| PDSCH 1Tx with Doppler < 100 Hz | 1.71 |
| PDSCH 2Tx with Doppler < 100 Hz, rank 1 | 1.82 |
| PDSCH 2Tx with Doppler < 100 Hz, rank 2 | 1.67 |
| PDSCH 1Tx with Doppler ≥ 100 Hz | 1.67 |
| PDSCH 2Tx with Doppler ≥ 100 Hz, rank 1 | 1.78 |
| PDSCH 2Tx with Doppler ≥ 100 Hz, rank 2 | 1.63 |
| PDCCH 1Tx, rank 1 | 1.74 |
| PDCCH 2Tx, rank 1 | 1.84 |

< End of changes >