3GPP TSG-RAN5 Meeting #97	R5-227572

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3220 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates for EN-DC RRC test case 8.2.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160, Bureau Veritas, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the upgrade to baseline definitions in TS 38.331 version 17.2.0, a set of IEs remain not checked or missing. There’s NOTE 12 defined in n77 that the band has defined dedicate frequency range to be used in USA and Canada according to the spectrum defined by FCC certification and ISED authorization. n7712; 3300 MHz – 4200 MHz; 3300 MHz – 4200 MHz; TDD. NOTE 12: In the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. In Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. Currently, the UE capability test cases is not check these element and need to be updated accordingly. |
| Summary of change: |  | Added check of features depending on existing parameters pc_dl_SchedulingOffset_PDSCH_TypeA pc_NR_dl_DedicatedMessageSegmentation pc_twoPUCCH_group pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 Created Tables 8.2.1.1.1.3.3-6a and 8.2.1.1.1.3.3-6b from Table 8.2.1.1.1.3.3-6 to improve readability. Fixed unbalanced branches in Table 8.2.1.1.1.3.3-6. Updated extendedBand-n77-r16 to “Checked” point in Table 8.2.1.1.1.3.3-6: UE-NR-Capability (Table 8.2.1.1.1.3.3-5). Added check point to extendedBand-n77-2-r17 in Table 8.2.1.1.1.3.3-6: UE-NR-Capability (Table 8.2.1.1.1.3.3-5). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Several capability IEs will be missed in Audit test case 8.2.1.1.1 UE supporting restricted frequency range in n77 will keep missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact. See R5s221179 Other related WICs TEI15_Test, 5GS_NR_LTE-UEConTest Associated new ICS added in TS38.508-2 CR 0401 (R5-226757) and CR 0402 (R5-226759) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-224068. |  |  |  |  |  |  |  |  |

##### 8.2.1.1.1	UE capability transfer / Success / EN-DC

8.2.1.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) only }

ensure that {

when { UE receives an UECapabilityEnquiry message that includes eutra }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

(2)

with { UE in RRC_CONNECTED state with EN-DC, and, MCG(s) (E-UTRA PDCP) only }

ensure that {

when { UE receives an UECapabilityEnquiry message that includes eutra-nr and nr }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

8.2.1.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.3.3, 5.6.22.2 and 5.6.22.3.

[TS 36.331, clause 5.6.3.3]

The UE shall:

1>	for NB-IoT, set the contents of UECapabilityInformation message as follows:

2>	include the UE Radio Access Capability Parameters within the ue-Capability-Container;

2>	include ue-RadioPagingInfo;

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

1>	else, set the contents of UECapabilityInformation message as follows:

2>	if the ue-CapabilityRequest includes eutra:

3>	include the UE-EUTRA-Capability within a ue-CapabilityRAT-Container and with the rat-Type set to eutra;

3>	if the UE supports FDD and TDD:

4>	set all fields of UECapabilityInformation, except field fdd-Add-UE-EUTRA-Capabilities and tdd-Add-UE-EUTRA-Capabilities (including their sub-fields), to include the values applicable for both FDD and TDD (i.e. functionality supported by both modes);

4>	if (some of) the UE capability fields have a different value for FDD and TDD:

5>	if for FDD, the UE supports additional functionality compared to what is indicated by the previous fields of UECapabilityInformation:

6>	include field fdd-Add-UE-EUTRA-Capabilities and set it to include fields reflecting the additional functionality applicable for FDD;

5>	if for TDD, the UE supports additional functionality compared to what is indicated by the previous fields of UECapabilityInformation:

6>	include field tdd-Add-UE-EUTRA-Capabilities and set it to include fields reflecting the additional functionality applicable for TDD;

NOTE 1: The UE includes fields of XDD-Add-UE-EUTRA-Capabilities in accordance with the following:

-	The field is included only if one or more of its sub-fields (or bits in the feature group indicators string) has a value that is different compared to the value signalled elsewhere within UE-EUTRA-Capability;

(this value signalled elsewhere is also referred to as the Common value that is supported for both XDD modes)

-	For the fields that are included in XDD-Add-UE-EUTRA-Capabilities, the UE sets:

-	the sub-fields (or bits in the feature group indicators string) that are not allowed to be different to the same value as the Common value;

-	the sub-fields (or bits in the feature group indicators string) that are allowed to be different to a value indicating at least the same functionality as indicated by the Common value;

3>	else (UE supports single xDD mode):

4>	set all fields of UECapabilityInformation, except field fdd-Add-UE-EUTRA-Capabilities and tdd-Add-UE-EUTRA-Capabilities (including their sub-fields), to include the values applicable for the xDD mode supported by the UE;

3>	compile a list of band combinations, candidate for inclusion in the UECapabilityInformation message, comprising of band combinations supported by the UE according to the following priority order (i.e. listed in order of decreasing priority):

4>		include all non-CA bands, regardless of whether UE supports carrier aggregation, only:

-	if the UE includes ue-Category-v1020 (i.e. indicating category 6 to 8); or

-	if for at least one of the non-CA bands, the UE supports more MIMO layers with TM9 and TM10 than implied by the UE category; or

-	if the UE supports TM10 with one or more CSI processes;

-	if the UE supports 1024QAM in DL;

4>	if the UECapabilityEnquiry message includes requestedFrequencyBands and UE supports requestedFrequencyBands:

5>	include all 2DL+1UL CA band combinations, only consisting of bands included in requestedFrequencyBands;

5>	include all other CA band combinations, only consisting of bands included in requestedFrequencyBands, and prioritized in the order of requestedFrequencyBands, (i.e. first include remaining band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on);

4>	else (no requested frequency bands):

5>	include all 2DL+1UL CA band combinations;

5>	include all other CA band combinations;

4>	if UE supports maximumCCsRetrieval and if the UECapabilityEnquiry message includes the requestedMaxCCsDL and the requestedMaxCCsUL (i.e. both UL and DL maximums are given):

5>	remove from the list of candidates the band combinations for which the number of CCs in DL exceeds the value indicated in the requestedMaxCCsDL or for which the number of CCs in UL exceeds the value indicated in the requestedMaxCCsUL;

5>	indicate in requestedCCsUL the same value as received in requestedMaxCCsUL;

5>	indicate in requestedCCsDL the same value as received in requestedMaxCCsDL;

4>	else if UE supports maximumCCsRetrieval and if the UECapabilityEnquiry message includes the requestedMaxCCsDL (i.e. only DL maximum limit is given):

5>	remove from the list of candidates the band combinations for which the number of CCs in DL exceeds the value indicated in the requestedMaxCCsDL;

5>	indicate value in requestedCCsDL the same value as received in requestedMaxCCsDL;

4>	else if UE supports maximumCCsRetrieval and if the UECapabilityEnquiry message includes the requestedMaxCCsUL (i.e. only UL maximum limit is given):

5>	remove from the list of candidates the band combinations for which the number of CCs in UL exceeds the value indicated in the requestedMaxCCsUL;

5>	indicate in requestedCCsUL the same value as received in requestedMaxCCsUL;

4>	if the UE supports reducedIntNonContComb and the UECapabilityEnquiry message includes requestReducedIntNonContComb:

5>	set reducedIntNonContCombRequested to true;

5>	remove from the list of candidates the intra-band non-contiguous CA band combinations which support is implied by another intra-band non-contiguous CA band combination included in the list of candidates as specified in TS 36.306 [5, 4.3.5.21]:

4>	if the UE supports requestReducedFormat and UE supports skipFallbackCombinations and UECapabilityEnquiry message includes requestSkipFallbackComb:

5>	set skipFallbackCombRequested to true;

5>	for each band combination included in the list of candidates (including 2DL+1UL CA band combinations), starting with the ones with the lowest number of DL and UL carriers, that concerns a fallback band combination of another band combination included in the list of candidates as specified in TS 36.306 [5]:

6>	remove the band combination from the list of candidates;

6>	include differentFallbackSupported in the band combination included in the list of candidates whose fallback concerns the removed band combination, if its capabilities differ from the removed band combination;

4>	if the UE supports requestReducedFormat and diffFallbackCombReport, and UECapabilityEnquiry message includes requestDiffFallbackCombList:

5>	if the UE does not support skipFallbackCombinations or UECapabilityEnquiry message does not include requestSkipFallbackComb:

6>	remove all band combination from the list of candidates;

5>	for each CA band combination indicated in requestDiffFallbackCombList:

6>	include the CA band combination, if not already in the list of candidates;

6>	include the fallback combinations for which the supported UE capabilities are different from the capability of the CA band combination;

5>	include CA band combinations indicated in requestDiffFallbackCombList into requestedDiffFallbackCombList;

3>	if the UECapabilityEnquiry message includes requestReducedFormat and UE supports requestReducedFormat:

4>	include in supportedBandCombinationReduced as many as possible of the band combinations included in the list of candidates, including the non-CA combinations, determined according to the rules and priority order defined above;

3>	else

4>	if the UECapabilityEnquiry message includes requestedFrequencyBands and UE supports requestedFrequencyBands:

5>	include in supportedBandCombination as many as possible of the band combinations included in the list of candidates, including the non-CA combinations and up to 5DL+5UL CA band combinations, determined according to the rules and priority order defined above;

5>	include in supportedBandCombinationAdd as many as possible of the remaining band combinations included in the list of candidates, (i.e. the candidates not included in supportedBandCombination), up to 5DL+5UL CA band combinations, determined according to the rules and priority order defined above;

4>	else

5>	include in supportedBandCombination as many as possible of the band combinations included in the list of candidates, including the non-CA combinations and up to 5DL+5UL CA band combinations, determined according to the rules defined above;

5>	if it is not possible to include in supportedBandCombination all the band combinations to be included according to the above, selection of the subset of band combinations to be included is left up to UE implementation;

3>	indicate in requestedBands the same bands and in the same order as included in requestedFrequencyBands, if received;

3>	if the UE is a category 0, M1 or M2 UE, or supports any UE capability information in ue-RadioPagingInfo, according to TS 36.306 [5]:

4>	include ue-RadioPagingInfo and set the fields according to TS 36.306 [5];

3>	if the UE supports EN-DC and if requestedFreqBandsNR-MRDC is included in the request:

4>	include into featureSetsEUTRA the feature sets that are applicable for the received requestedFreqBandsNR-MRDC as specified in TS 38.331 [82], clause 5.6.1.4.

NOTE:	Even if the network requests (only) capabilities for eutra, it may include NR band numbers in the requestedFreqBandsNR-MRDC in order to ensure that the UE includes all necessary feature sets (i.e. E-UTRA and NR) needed for subsequently requested eutra-nr capabilities.

3>	if the UECapabilityEnquiry message includes request-sTTI-sPT-Capability and if the UE supports short TTI and/or SPT:

4> for each band combination the UE included in a field of the UECapabilityInformation message in accordance with the previous:

4>	if the UE supports short TTI, include the short TTI capabilities for each of the band combinations using the STTI-SPT-BandCombinationParameters;

4>	if the UE supports SPT, include the SPT capabilities for each of the band combinations using the STTI-SPT-BandCombinationParameters;

NOTE:	The UE may have to add/repeat the band combinations to the list of band combinations included earlier, to include short TTI capabilities and/or SPT capabilities.

2>	if the UE supports short TTI and/or SPT:

3>	include in the UE radio access capabilities the IE sTTI-SPT-Supported and set to supported;

2>	if the ue-CapabilityRequest includes geran-cs and if the UE supports GERAN CS domain:

3>	include the UE radio access capabilities for GERAN CS within a ue-CapabilityRAT-Container and with the rat-Type set to geran-cs;

2>	if the ue-CapabilityRequest includes geran-ps and if the UE supports GERAN PS domain:

3>	include the UE radio access capabilities for GERAN PS within a ue-CapabilityRAT-Container and with the rat-Type set to geran-ps;

2>	if the ue-CapabilityRequest includes utra and if the UE supports UTRA:

3>	include the UE radio access capabilities for UTRA within a ue-CapabilityRAT-Container and with the rat-Type set to utra;

2>	if the ue-CapabilityRequest includes cdma2000-1XRTT and if the UE supports CDMA2000 1xRTT:

3>	include the UE radio access capabilities for CDMA2000 within a ue-CapabilityRAT-Container and with the rat-Type set to cdma2000-1XRTT;

2>	if the ue-CapabilityRequest includes nr and if the UE supports NR:

3>	include the UE radio access capabilities for NR within a ue-CapabilityRAT-Container, with the rat-Type set to nr;

3>	include band combinations and feature sets as specified in TS 38.331 [82], clause 5.6.1.4, considering the FreqBandList included in requestedFreqBandsNR-MRDC and the eutra-nr-only flag (if present);

2>	if the ue-CapabilityRequest includes eutra-nr and if the UE supports EN-DC:

3>	include the UE radio access capabilities for EUTRA-NR within a ue-CapabilityRAT-Container, with the rat-Type set to eutra-nr

3>	include band combinations and feature sets as specified in TS 38.331 [82], clause 5.6.1.4, considering the FreqBandList included in requestedFreqBandsNR-MRDC;

1>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

[TS 36.331, clause 5.6.22.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8];

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the ULDedicatedMessageSegment message as specified in 5.6.22.3;

[TS 36.331, clause 5.6.22.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 36.323 [8]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

8.2.1.1.1.3	Test description

8.2.1.1.1.3.1	Pre-test conditions

System Simulator:

-	EUTRA Cell 1 is the PCell and NR Cell 1 is the PS Cell.

UE:

-	None

Preamble:

-	The UE is in state RRC_CONNECTED in EN-DC mode according to TS 38.508-1 [4] clause 4.5.4.2 with MCG(s) and SCG.

8.2.1.1.1.3.2	Test procedure sequence

Table 8.2.1.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message including eutra with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 2a1 and 2a2 will be performed else step 2b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? | --> | ULDedicatedMessageSegment | - | - |
| 2a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 1 | P |
| 2b1 | Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? | --> | UECapabilityInformation | 1 | P |
| 3 | The SS transmits a UECapabilityEnquiry message including eutra-nr and nr with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 4a1 and 4a2 will be performed else step 4b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 4a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 4a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? | --> | ULDedicatedMessageSegment | - | - |
| 4a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 2 | P |
| 4b1 | Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? | --> | UECapabilityInformation | 2 | P |

8.2.1.1.1.3.3	Specific message contents

Table 8.2.1.1.1.3.3-1: UECapabilityEnquiry (step 1, Table 8.2.1.1.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF RAT TYPE { |  |  |  |
| RAT-Type [1] | eutra |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedFrequencyBands-r11 SEQUENCE {} | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestReducedFormat-r13 | Not present |  |  |
| requestSkipFallbackComb-r13 | Not present |  |  |
| requestedMaxCCsDL-r13 | Not present |  |  |
| requestedMaxCCsUL-r13 | Not present |  |  |
| requestReducedIntNonContComb-r13 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestDiffFallbackCombList-r14 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedFreqBandsNR-MRDC-r15 | Not present |  |  |
| nonCriticalExtension SEQUENCE { | Not present |  |  |
| requestSTTI-SPT-Capability-r15 | true |  |  |
| eutra-nr-only-r15 | true |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedCapabilityNR-r15 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedCapabilityCommon-r15 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| criticalExtensionsFuture SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-2: UECapabilityInformation (steps 2a2 and 2b1, Table 8.2.1.1.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF UE-CapabilityRAT-Container { | 1 entry |  |  |
| UE-CapabilityRAT-Container[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | eutra |  | pc_EUTRA |
| ueCapabilityRAT-Container | UE-EUTRA-Capability | Encoded as per TS 36.331 [11] clause 6.3.6 | pc_EUTRA |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-3: UE-EUTRA-Capability (Table 8.2.1.1.1.3.3-2)

| Derivation Path: TS 36.331 [11], clause 6.3.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  |  | Value/remark |  |  |  | Comment |  |  |  | Condition |  |  |  |
| UE-EUTRA-Capability :: = SEQUENCE { |  |  |  |  |  |  |  | As per TS 36.523-1 [13] Table 8.5.4.1.3.3-2; details not checked in teh scope of the present NR RRC TC 8.2.1.1.1. |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| irat-ParametersNR-r15 |  |  |  | IRAT-ParametersNR-r15 |  |  |  |  |  |  |  |  |  |  |  |
| featureSetsEUTRA-r15 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| pdcp-ParametersNR-r15 |  |  |  | PDCP-ParametersNR-r15 |  |  |  |  |  |  |  |  |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1510 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| pdcp-ParametersNR-r15 |  |  |  | PDCP-ParametersNR-r15 |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1510 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| pdcp-ParametersNR-r15 |  |  |  | PDCP-ParametersNR-r15 |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| measParameters-v1520 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| measParameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| otherParameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| neighCellSI-AcquisitionParameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| mac-Parameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| phyLayerParameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| rf-Parameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| pdcp-Parameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ue-CategoryDL-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ue-BasedNetwPerfMeasParameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| rlc-Parameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| sl-Parameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| extendedNumberOfDRBs-r15 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| reducedCP-Latency-r15 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| laa-Parameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ue-CategoryUL-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1530 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| neighCellSI-AcquisitionParameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| reducedCP-Latency-r15 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1530 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| neighCellSI-AcquisitionParameters-v1530 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| reducedCP-Latency-r15 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| phyLayerParameters-v1540 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| otherParameters-v1540 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1540 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| eutra-5GC-Parameters-r15 |  |  |  | EUTRA-5GC-Parameters-r15 |  |  |  |  |  |  |  |  |  |  |  |
| irat-ParametersNR-v1540 |  |  |  | IRAT-ParametersNR-v1540 |  |  |  |  |  |  |  | See note 1 |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1540 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| eutra-5GC-Parameters-r15 |  |  |  | EUTRA-5GC-Parameters-r15 |  |  |  |  |  |  |  |  |  |  |  |
| irat-ParametersNR-v1540 |  |  |  | IRAT-ParametersNR-v1540 |  |  |  |  |  |  |  | See note 1 |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sl-Parameters-v1540 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| irat-ParametersNR-v1540 |  |  |  | IRAT-ParametersNR-v1540 |  |  |  |  |  |  |  | See note 1 |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| neighCellSI-AcquisitionParameters-v1550 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| phyLayerParameters-v1550 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| mac-Parameters-v1550 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1550 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1550 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| pdcp-ParametersNR-v1560 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| irat-ParametersNR-v1560 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| appliedCapabilityFilterCommon-r15 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1560 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1560 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rf-Parameters-v1570 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| irat-ParametersNR-v1570 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| neighCellSI-AcquisitionParameters-v15a0 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| eutra-5GC-Parameters-r15 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v15a0 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v15a0 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| highSpeedEnhParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| neighCellSI-AcquisitionParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| mbms-Parameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| pdcp-Parameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| mac-Parameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| phyLayerParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| measParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| pur-Parameters-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| eutra-5GC-Parameters-v1610 |  |  |  | EUTRA-5GC-Parameters-v1610 |  |  |  |  |  |  |  |  |  |  |  |
| otherParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| dl-DedicatedMessageSegmentation-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| mmtel-Parameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| irat-ParametersNR-v1610 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nr-HO-ToEN-DC-r16 |  |  |  | Checked |  |  |  |  |  |  |  | pc_interRAT_NR_ToENDC(see note 2) |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR1-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR1-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| rf-Parameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| mobilityParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ue-BasedNetwPerfMeasParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| sl-Parameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1610 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| phyLayerParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| pur-Parameters-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| measParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| eutra-5GC-Parameters-v1610 |  |  |  | EUTRA-5GC-Parameters-v1610 |  |  |  |  |  |  |  |  |  |  |  |
| irat-ParametersNR-v1610 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nr-HO-ToEN-DC-r16 |  |  |  | Checked |  |  |  |  |  |  |  | pc_interRAT_NR_ToENDC(see note 2) |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR1-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR1-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| neighCellSI-AcquisitionParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| mobilityParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1610 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| phyLayerParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  | >=Rel16 |  |  |  |
| pur-Parameters-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| measParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| eutra-5GC-Parameters-v1610 |  |  |  | EUTRA-5GC-Parameters-v1610 |  |  |  |  |  |  |  |  |  |  |  |
| irat-ParametersNR-v1610 SEQUENCE { |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nr-HO-ToEN-DC-r16 |  |  |  | Checked |  |  |  |  |  |  |  | pc_interRAT_NR_ToENDC(see note 2) |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR1-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR1-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-FDD-FR2-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| ce-EUTRA-5GC-HO-ToNR-TDD-FR2-r16 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| neighCellSI-AcquisitionParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| mobilityParameters-v1610 |  |  |  | Not checked |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| nonCriticalExtension {} |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| >=Rel-16 | For Rel-16 or later Releases |
| Note 1: If the UE is Single Mode (FDD or TDD) the value of SupportedBandListNR-r15 in IRAT-ParametersNR-v1540 IE will be signaled in the common IE. If the UE is Dual Mode (FDD & TDD) and the value of SupportedBandListNR-r15 in IRAT-ParametersNR-v1540 IE is different in both modes then it will be signaled under X-Add-UE-EUTRA-Capabilities-v1540 IE (X – fdd and/or tdd respectively), and it will not be present in the common IE. It will be signalled in the common IE if the value is the same on both modes. The IE may be signaled independently for either FDD or TDD or both. Note 2: If the UE is Single Mode (FDD or TDD) the support of nr-HO-ToEN-DC-r16 will be signaled in the common IE. If the UE is Dual Mode (FDD & TDD) and the support of nr-HO-ToEN-DC-r16 is different in both modes then it will be signaled under X-Add-UE-EUTRA-Capabilities-v1610 IE (X – fdd and/or tdd respectively), and it will not be present in the common IE. It will be signalled in the common IE if the value is the same on both modes (i.e., “supported” on both FDD and TDD). The IE may be signaled independently for either FDD or TDD or both. |  |

Table 8.2.1.1.1.3.3-3A: IRAT-ParametersNR-r15 (Table 8.2.1.1.1.3.3-3)

| Derivation Path: TS 36.331 [11], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IRAT-ParametersNR-r15 SEQUENCE { |  |  |  |
| en-DC-r15 | Not checked |  |  |
| eventB2-r15 | Not checked |  |  |
| supportedBandListEN-DC-r15 | SupportedBandListNR-r15 |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-3B: PDCP-ParametersNR-r15 (Table 8.2.1.1.1.3.3-3)

| Derivation Path: TS 36.331 [11], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-ParametersNR-r15 | Not checked |  |  |

Table 8.2.1.1.1.3.3-3C: EUTRA-5GC-Parameters-r15 (Table 8.2.1.1.1.3.3-3)

| Derivation Path: TS 36.331 [11], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-5GC-Parameters-r15 SEQUENCE { |  |  |  |
| eutra-5GC-r15 | Not checked |  |  |
| eutra-EPC-HO-EUTRA-5GC-r15 | Not checked |  |  |
| ho-EUTRA-5GC-FDD-TDD-r15 | Not checked |  |  |
| ho-InterfreqEUTRA-5GC-r15 | Not checked |  |  |
| ims-VoiceOverMCG-BearerEUTRA-5GC-r15 | Not checked |  |  |
| inactiveState-r15 | Not checked |  |  |
| reflectiveQoS-r15 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-3D: IRAT-ParametersNR-v1540 (Table 8.2.1.1.1.3.3-3)

| Derivation Path: TS 36.331 [11], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| IRAT-ParametersNR-v1540 SEQUENCE { |  |  |  |
| eutra-5GC-HO-ToNR-FDD-FR1-r15 | Not checked |  |  |
| eutra-5GC-HO-ToNR-TDD-FR1-r15 | Not checked |  |  |
| eutra-5GC-HO-ToNR-FDD-FR2-r15 | Not checked |  |  |
| eutra-5GC-HO-ToNR-TDD-FR2-r15 | Not checked |  |  |
| eutra-EPC-HO-ToNR-FDD-FR1-r15 | Checked |  | pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 |
| eutra-EPC-HO-ToNR-TDD-FR1-r15 | Checked |  | pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 |
| eutra-EPC-HO-ToNR-FDD-FR2-r15 | Not checked |  |  |
| eutra-EPC-HO-ToNR-TDD-FR2-r15 | Not checked |  |  |
| ims-VoiceOverNR-FR1-r15 | Not checked |  |  |
| ims-VoiceOverNR-FR2-r15 | Not checked |  |  |
| sa-NR-r15 | Not checked |  |  |
| supportedBandListNR-SA-r15 | SupportedBandListNR-r15 |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-3E: SupportedBandListNR-r15 (Table 8.2.1.1.1.3.3-3A and Table 8.2.1.1.1.3.3-3D)

| Derivation Path: TS 36.331 [11], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SupportedBandListNR-r15 SEQUENCE (SIZE (1..maxBandsNR-r15)) OF SupportedBandNR-r15 { |  |  |  |
| bandNR-r15 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-3F: EUTRA-5GC-Parameters-v1610 (Table 8.2.1.1.1.3.3-3)

| Derivation Path: TS 36.331 [11], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-5GC-Parameters-v1610 SEQUENCE { |  |  |  |
| ce-InactiveState-r16 | Not checked |  |  |
| ce-EUTRA-5GC-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-4: UECapabilityEnquiry (step 3, Table 8.2.1.1.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF RAT-Type { | 2 entries |  |  |
| RAT-Type[1] | nr | entry 1 |  |
| RAT-Type[2] | eutra-nr | entry 2 |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedFreqBandsNR-MRDC-r15 | OCTET STRING including the FreqBandList IE according to TS 38.508-1 [4] table [4.6.4-21] |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestSTTI-SPT-Capability-r15 | Not present |  |  |
| eutra-nr-only-r15 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedCapabilityNR-r15 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedCapabilityCommon-r15 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-5: UECapabilityInformation (steps 4a2 and 4b1, Table 8.2.1.1.1.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF UE-CapabilityRAT-Container { | 2 entries |  |  |
| UE-CapabilityRAT-Container[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  | pc_NR_FDD or pc_NR_TDD |
| ueCapabilityRAT-Container | UE-NR-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 | pc_NR_FDD or pc_NR_TDD |
| } |  |  |  |
| UE-CapabilityRAT-Container[2] SEQUENCE { |  | entry 2 |  |
| rat-Type | eutra-nr |  | pc_EN_DC |
| ueCapabilityRAT-Container | UE-MRDC-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 | pc_EN_DC |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-6: UE-NR-Capability (Table 8.2.1.1.1.3.3-5)

| Derivation Path: TS 38.331 [12], clause 6.3.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE-NR-Capability ::= SEQUENCE { |  |  |  |  |
| accessStratumRelease |  | Checked | "rel-15" or higher |  |
| pdcp-Parameters SEQUENCE { |  |  |  |  |
| supportedROHC-Profiles SEQUENCE { |  |  |  |  |
| profile0x0000 |  | Not checked |  |  |
| profile0x0001 |  | Not checked |  |  |
| profile0x0002 |  | Not checked |  |  |
| profile0x0003 |  | Not checked |  |  |
| profile0x0004 |  | Not checked |  |  |
| profile0x0006 |  | Not checked |  |  |
| profile0x0101 |  | Not checked |  |  |
| profile0x0102 |  | Not checked |  |  |
| profile0x0103 |  | Not checked |  |  |
| profile0x0104 |  | Not checked |  |  |
| } |  |  |  |  |
| maxNumberROHC-ContextSessions |  | Not checked |  |  |
| uplinkOnlyROHC-Profiles |  | Not checked |  |  |
| continueROHC-Context |  | Not checked |  |  |
| outOfOrderDelivery |  | Checked |  | pc_outOfOrderDelivery |
| shortSN |  | Checked |  | pc_shortSN |
| pdcp-DuplicationSRB |  | Checked |  | pc_pdcp_DuplicationSplitSRB |
| pdcp-DuplicationMCG-OrSCG-DRB |  | Checked |  | pc_pdcp_DuplicationMCG_OrSCG_DRB |
| drb-IAB-r16 |  | Not checked |  |  |
| non-DRB-IAB-r16 |  | Not checked |  |  |
| extendedDiscardTimer-r16 |  | Not checked |  |  |
| continueEHC-Context-r16 |  | Not checked |  |  |
| ehc-r16 |  | Checked |  | pc_NR_ehc_r16 |
| maxNumberEHC-Contexts-r16 |  | Not checked |  |  |
| jointEHC-ROHC-Config-r16 |  | Not checked |  |  |
| pdcp-DuplicationMoreThanTwoRLC-r16 |  | Checked |  | pc_pdcp_DuplicationMoreThanTwoRLC_r16 |
| longSN-RedCap-r17 |  | Not checked |  |  |
| udc-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| rlc-Parameters SEQUENCE { |  |  |  |  |
| am-WithShortSN |  | Checked |  | pc_am_WithShortSN |
| um-WithShortSN |  | Checked |  | pc_um_WithShortSN |
| um-WithLongSN |  | Checked |  | pc_um_WIthLongSN |
| extendedT-PollRetransmit-r16 |  | Not checked |  |  |
| extendedT-StatusProhibit-r16 |  | Not checked |  |  |
| am-WithLongSN-RedCap-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| mac-Parameters SEQUENCE { |  |  |  |  |
| mac-ParametersCommon SEQUENCE { |  |  |  |  |
| lcp-Restriction |  | Checked |  | pc_lcp_Restriction |
| dummy |  | Not checked |  |  |
| lch-ToSCellRestriction |  | Checked |  |  |
| recommendedBitRate |  | Checked |  | pc_recommendedBitRate |
| recommendedBitRateQuery |  | Checked |  | pc_recommendedBitRateQuery |
| recommendedBitRateMultiplier-r16 |  | Not checked |  |  |
| preEmptiveBSR-r16 |  | Not checked |  |  |
| autonomousTransmission-r16 |  | Not checked |  |  |
| lch-PriorityBasedPrioritization-r16 |  | Checked |  | pc_lch_PriorityBasedPrioritization_r16 |
| lch-ToConfiguredGrantMapping-r16 |  | Not checked |  |  |
| lch-ToGrantPriorityRestriction-r16 |  | Checked |  | pc_lch_ToGrantPriorityRestriction_r16 |
| singlePHR-P-r16 |  | Not checked |  |  |
| ul-LBT-FailureDetectionRecovery-r16 |  | Not checked |  |  |
| tdd-MPE-P-MPR-Reporting-r16 |  | Not checked |  |  |
| lcid-ExtensionIAB-r16 |  | Not checked |  |  |
| spCell-BFR-CBRA-r16 |  | Not checked |  |  |
| srs-ResourceId-Ext-r16 |  | Not checked |  |  |
| enhancedUuDRX-forSidelink-r17 |  | Not checked |  |  |
| mg-ActivationRequestPRS-Meas-r17 |  | Not checked |  |  |
| mg-ActivationCommPRS-Meas-r17 |  | Not checked |  |  |
| intraCG-Prioritization-r17 |  | Not checked |  |  |
| jointPrioritizationCG-Retx-Timer-r17 |  | Not checked |  |  |
| survivalTime-r17 |  | Not checked |  |  |
| lcg-ExtensionIAB-r17 |  | Not checked |  |  |
| harq-FeedbackDisabled-r17 |  | Not checked |  |  |
| uplink-Harq-ModeB-r17 |  | Not checked |  |  |
| sr-TriggeredBy-TA-Report-r17 |  | Not checked |  |  |
| extendedDRX-CycleInactive-r17 |  | Not checked |  |  |
| simultaneousSR-PUSCH-DiffPUCCH-groups-r17 |  | Not checked |  |  |
| lastTransmissionUL-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |  |
| skipUplinkTxDynamic |  | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer |  | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle |  | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle |  | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations |  | Not checked |  |  |
| multipleConfiguredGrants |  | Checked (NOTE 12) |  | pc_multipleConfiguredGrants_r16 |
| secondaryDRX-Group-r16 |  | Not checked |  |  |
| enhancedSkipUplinkTxDynamic-r16 |  | Not checked |  |  |
| enhancedSkipUplinkTxConfigured-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| phy-Parameters SEQUENCE { |  |  |  |  |
| phy-ParametersCommon SEQUENCE { |  |  |  |  |
| csi-RS-CFRA-ForHO |  | Checked |  | pc_csi_RS_CFRA_ForHO |
| dynamicPRB-BundlingDL |  | Not checked |  |  |
| sp-CSI-ReportPUCCH |  | Not checked |  |  |
| sp-CSI-ReportPUSCH |  | Not checked |  |  |
| nzp-CSI-RS-IntefMgmt |  | Not checked |  |  |
| type2-SP-CSI-Feedback-LongPUCCH |  | Not checked |  |  |
| precoderGranularityCORESET |  | Not checked |  |  |
| dynamicHARQ-ACK-Codebook |  | supported |  |  |
| semiStaticHARQ-ACK-Codebook |  | Not checked |  |  |
| spatialBundlingHARQ-ACK |  | Checked |  | pc_spatialBundlingHARQ_ACK |
| dynamicBetaOffsetInd-HARQ-ACK-CSI |  | Not checked |  |  |
| pucch-Repetition-F1-3-4 |  | Not checked |  |  |
| ra-Type0-PUSCH |  | Checked |  | pc_ra_Type0_PUSCH |
| dynamicSwitchRA-Type0-1-PDSCH |  | Checked |  | pc_dynamicSwitchRA_Type0_1_PDSCH |
| dynamicSwitchRA-Type0-1-PUSCH |  | Checked |  | pc_dynamicSwitchRA_Type0_1_PUSCH |
| pdsch-MappingTypeA |  | supported |  | pc_pdsch_MappingTypeA |
| pdsch-MappingTypeB |  | Checked |  | pc_pdsch_MappingTypeB |
| interleavingVRB-ToPRB-PDSCH |  | Checked |  | pc_interleavingVRB_ToPRB_PDSCH |
| interSlotFreqHopping-PUSCH |  | Not checked |  |  |
| type1-PUSCH-RepetitionMultiSlots |  | Not checked |  |  |
| type2-PUSCH-RepetitionMultiSlots |  | Not checked |  |  |
| pusch-RepetitionMultiSlots |  | Checked |  | pc_pusch_RepetitionMultiSlots |
| pdsch-RepetitionMultiSlots |  | Checked |  | pc_pdsch_RepetitionMultiSlots |
| downlinkSPS |  | Checked |  | pc_downlinkSPS |
| configuredUL-GrantType1 |  | Checked |  | pc_configuredUL_GrantType1 |
| configuredUL-GrantType2 |  | Checked |  | pc_configuredUL_GrantType2 |
| pre-EmptIndication-DL |  | Not checked |  |  |
| cbg-TransIndication-DL |  | Not checked |  |  |
| cbg-TransIndication-UL |  | Not checked |  |  |
| cbg-FlushIndication-DL |  | Not checked |  |  |
| dynamicHARQ-ACK-CodeB-CBG-Retx-DL |  | Not checked |  |  |
| rateMatchingResrcSetSemi-Static |  | Not checked |  |  |
| rateMatchingResrcSetDynamic |  | Not checked |  |  |
| bwp-SwitchingDelay |  | Checked | 38.306, 4.2.7.10: It is mandatory to report type 1 or type 2. This capability is not applicable to IAB-MT | pc_bwp_SwitchingDelay_Type1 or pc_bwp_SwitchingDelay_Type2 |
| dummy |  | Not checked |  |  |
| maxNumberSearchSpaces |  | Checked |  | pc_maxNumberSearchSpaces |
| rateMatchingCtrlResrsSetDynamic |  | Not checked |  |  |
| maxLayersMIMO-Indication |  | Not checked |  |  |
| spCellPlacement |  | Not checked |  |  |
| twoStepRACH-r16 |  | Not checked |  |  |
| dci-Format1-2And0-2-r16 |  | Checked |  | pc_dci_Format1_2And0_2_r16 |
| monitoringDCI-SameSearchSpace-r16 |  | Not checked |  |  |
| type2-CG-ReleaseDCI-0-1-r16 |  | Not checked |  |  |
| type2-CG-ReleaseDCI-0-2-r16 |  | Not checked |  |  |
| sps-ReleaseDCI-1-1-r16 |  | Not checked |  |  |
| sps-ReleaseDCI-1-2-r16 |  | Not checked |  |  |
| csi-TriggerStateNon-ActiveBWP-r16 |  | Not checked |  |  |
| separateSMTC-InterIAB-Support-r16 |  | Not checked |  |  |
| separateRACH-IAB-Support-r16 |  | Not checked |  |  |
| ul-flexibleDL-SlotFormatSemiStatic-IAB-r16 |  | Not checked |  |  |
| ul-flexibleDL-SlotFormatDynamics-IAB-r16 |  | Not checked |  |  |
| dft-S-OFDM-WaveformUL-IAB-r16 |  | Not checked |  |  |
| dci-25-AI-RNTI-Support-IAB-r16 |  | Not checked |  |  |
| t-DeltaReceptionSupport-IAB-r16 |  | Not checked |  |  |
| guardSymbolReportReception-IAB-r16 |  | Not checked |  |  |
| harqACK-CB-SpatialBundlingPUCCH-Group-r16 |  | Not checked |  |  |
| crossSlotScheduling-r16 |  | Not checked |  |  |
| maxNumberSRS-PosPathLossEstimateAllServingCells-r16 |  | Not checked |  |  |
| extendedCG-Periodicities-r16 |  | Not checked |  |  |
| extendedSPS-Periodicities-r16 |  | Not checked |  |  |
| codebookVariantsList-r16 |  | Not checked |  |  |
| pusch-RepetitionTypeA-r16 |  | Not checked |  |  |
| dci-DL-PriorityIndicator-r16 |  | Checked |  | pc_dci_DL_PriorityIndicator_r16 |
| dci-UL-PriorityIndicator-r16 |  | Checked |  | pc_dci_UL_PriorityIndicator_r16 |
| maxNumberPathlossRS-Update-r16 |  | Not checked |  |  |
| type2-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |
| maxTotalResourcesForAcrossFreqRanges-r16 |  | Not checked |  |  |
| harqACK-separateMultiDCI-MultiTRP-r16 |  | Not checked |  |  |
| harqACK-jointMultiDCI-MultiTRP-r16 |  | Not checked |  |  |
| bwp-SwitchingMultiCCs-r16 |  | Not checked |  |  |
| targetSMTC-SCG-r16 |  | Not checked |  |  |
| supportRepetitionZeroOffsetRV-r16 |  | Not checked |  |  |
| cbg-TransInOrderPUSCH-UL-r16 |  | Not checked |  |  |
| bwp-SwitchingMultiDormancyCCs-r16 |  | Not checked |  |  |
| supportRetx-Diff-CoresetPool-Multi-DCI-TRP-r16 |  | Not checked |  |  |
| pdcch-MonitoringAnyOccasionsWithSpanGapCrossCarrierSch-r16 |  | Not checked |  |  |
| newBeamIdentifications2PortCSI-RS-r16 |  | Not checked |  |  |
| pathlossEstimation2PortCSI-RS-r16 |  | Not checked |  |  |
| mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16 |  | Not checked |  |  |
| guardSymbolReportReception-IAB-r17 |  | Not checked |  |  |
| restricted-IAB-DU-BeamReception-r17 |  | Not checked |  |  |
| recommended-IAB-MT-BeamTransmission-r17 |  | Not checked |  |  |
| case6-TimingAlignmentReception-IAB-r17 |  | Not checked |  |  |
| case7-TimingAlignmentReception-IAB-r17 |  | Not checked |  |  |
| dl-tx-PowerAdjustment-IAB-r17 |  | Not checked |  |  |
| desired-ul-tx-PowerAdjustment-r17 |  | Not checked |  |  |
| fdm-SoftResourceAvailability-DynamicIndication-r17 |  | Not checked |  |  |
| updated-T-DeltaRangeRecption-r17 |  | Not checked |  |  |
| slotBasedDynamicPUCCH-Rep-r17 |  | Not checked |  |  |
| sps-HARQ-ACK-Deferral-r17 |  | Not checked |  |  |
| unifiedJointTCI-commonUpdate-r17 |  | Not checked |  |  |
| mTRP-PDCCH-singleSpan-r17 |  | Not checked |  |  |
| supportedActivatedPRS-ProcessingWindow-r17 |  | Not checked |  |  |
| cg-TimeDomainAllocationExtension-r17 |  | Not checked |  |  |
| ta-BasedPDC-TN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |
| directionalCollisionDC-IAB-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 17) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |
| } |  |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |
| dummy1 |  | Not checked |  |  |
| twoFL-DMRS |  | Not checked |  |  |
| dummy2 |  | Not checked |  |  |
| dummy3 |  | Not checked |  |  |
| supportedDMRS-TypeDL |  | Not checked |  |  |
| supportedDMRS-TypeUL |  | Not checked |  |  |
| semiOpenLoopCSI |  | Not checked |  |  |
| csi-ReportWithoutPMI |  | Not checked |  |  |
| csi-ReportWithoutCQI |  | Not checked |  |  |
| onePortsPTRS |  | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |
| pucch-F2-WithFH |  | supported |  |  |
| pucch-F3-WithFH |  | supported |  |  |
| pucch-F4-WithFH |  | Not checked |  |  |
| freqHoppingPUCCH-F0-2 |  | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 |  | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot |  | Not checked |  |  |
| uci-CodeBlockSegmentation |  | Not checked |  |  |
| onePUCCH-LongAndShortFormat |  | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot |  | Not checked |  |  |
| intraSlotFreqHopping-PUSCH |  | Not checked |  |  |
| pusch-LBRM |  | Not checked |  |  |
| pdcch-BlindDetectionCA |  | Not checked |  |  |
| tpc-PUSCH-RNTI |  | Not checked |  |  |
| tpc-PUCCH-RNTI |  | Not checked |  |  |
| tpc-SRS-RNTI |  | Not checked |  |  |
| absoluteTPC-Command |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |
| pusch-HalfPi-BPSK |  | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK |  | Not checked |  |  |
| almostContiguousCP-OFDM-UL |  | Not checked |  |  |
| sp-CSI-RS |  | Not checked |  |  |
| sp-CSI-IM |  | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot |  | Not checked |  |  |
| multipleCORESET |  | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback |  | Not checked | CSI-RS-IM-ReceptionForFeedback |  |
| csi-RS-ProcFrameworkForSRS |  | Not checked | CSI-RS-ProcFrameworkForSRS |  |
| csi-ReportFramework |  | Not checked | CSI-ReportFramework |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |  |
| sameSymbol |  | Not checked |  |  |
| diffSymbol |  | Not checked |  |  |
| } |  |  |  |  |
| mux-SR-HARQ-ACK-PUCCH |  | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap |  | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |
| dl-64QAM-MCS-TableAlt |  | Not checked |  |  |
| ul-64QAM-MCS-TableAlt |  | Not checked |  |  |
| cqi-TableAlt |  | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL |  | Not checked |  |  |
| pdcch-BlindDetectionNRDC |  | Not checked |  |  |
| mux-HARQ-ACK-PUSCH-DiffSymbol |  | Not checked |  |  |
| type1-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |
| enhancedPowerControl-r16 |  | Not checked |  |  |
| simultaneousTCI-ActMultipleCC-r16 |  | Not checked |  |  |
| simultaneousSpatialRelationMultipleCC-r16 |  | Not checked |  |  |
| cli-RSSI-FDM-DL-r16 |  | Not checked |  |  |
| cli-SRS-RSRP-FDM-DL-r16 |  | Not checked |  |  |
| maxLayersMIMO-Adaptation-r16 |  | Not checked |  |  |
| aggregationFactorSPS-DL-r16 |  | Not checked |  |  |
| maxTotalResourcesForOneFreqRange-r16 |  | Not checked |  |  |
| csi-ReportFrameworkExt-r16 |  | Not checked |  |  |
| twoTCI-Act-servingCellInCC-List-r16 |  | Not checked |  |  |
| cri-RI-CQI-WithoutNon-PMI-PortInd-r16 |  | Not checked |  |  |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| phy-ParametersFR1 SEQUENCE { |  |  |  |  |
| pdcchMonitoringSingleOccasion |  | Not checked |  |  |
| scs-60kHz |  | Not checked |  |  |
| pdsch-256QAM-FR1 |  | Checked |  | pc_pdsch_256QAM_FR1 |
| pdsch-RE-MappingFR1-PerSymbol |  | Not checked |  |  |
| pdsch-RE-MappingFR1-PerSlot |  | Not checked |  |  |
| pdcch-MonitoringSingleSpanFirst4Sym-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| phy-ParametersFR2 SEQUENCE { |  |  |  |  |
| dummy |  | Not checked |  |  |
| pdsch-RE-MappingFR2-PerSymbol |  | Not checked |  |  |
| pCell-FR2 |  | Not checked |  |  |
| pdsch-RE-MappingFR2-PerSlot |  | Not checked |  |  |
| defaultSpatialRelationPathlossRS-r16 |  | Not checked |  |  |
| spatialRelationUpdateAP-SRS-r16 |  | Not checked |  |  |
| maxNumberSRS-PosSpatialRelationsAllServingCells-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| rf-Parameters SEQUENCE { |  |  |  |  |
| supportedBandListNR SEQUENCE (SIZE (1..maxBands)) OF BandNR { |  |  |  |  |
| BandNR[i] SEQUENCE { |  |  |  |  |
| bandNR |  | Checked | Checked for 'maxBands' entries of FreqBandIndicatorNR[i] | pc_nrBandx ('x' being the band number/type related PICS listed in TS 38.508-2) |
| modifiedMPR-Behaviour |  | Not checked |  |  |
| mimo-ParametersPerBand |  | Not checked | MIMO-ParametersPerBand[i] |  |
| extendedCP |  | Not checked |  |  |
| multipleTCI |  | supported |  |  |
| bwp-WithoutRestriction |  | Checked |  | pc_bwp_WithoutRestriction |
| bwp-SameNumerology |  | Not checked |  |  |
| bwp-DiffNumerology |  | Not checked |  |  |
| crossCarrierScheduling-SameSCS |  | Not checked |  |  |
| pdsch-256QAM-FR2 |  | Checked | Note 8 | pc_pdsch_256QAM_FR2 |
| pusch-256QAM |  | Checked | Note 9 | pc_pusch_256QAM_FR1OR pc_pusch_256QAM_FR2 |
| ue-PowerClass |  | Not checked |  |  |
| rateMatchingLTE-CRS |  | Not checked |  |  |
| channelBWs-DL-v1530 CHOICE { |  |  |  |  |
| fr1 SEQUENCE { |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |
| scs-30kHz |  | Not checked |  |  |
| scs-60kHz |  | Not checked |  |  |
| } |  |  |  |  |
| fr2 SEQUENCE { |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |
| scs-120kHz |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| channelBWs-UL-v1530 CHOICE { |  |  |  |  |
| fr1 SEQUENCE { |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |
| scs-30kHz |  | Not checked |  |  |
| scs-60kHz |  | Not checked |  |  |
| } |  |  |  |  |
| fr2 SEQUENCE { |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |
| scs-120kHz |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |  |
| maxUplinkDutyCycle-PC2-FR1 |  | Not checked |  |  |
| pucch-SpatialRelInfoMAC-CE |  | Not checked |  |  |
| powerBoosting-pi2BPSK |  | Not checked |  |  |
| maxUplinkDutyCycle-FR2 |  | Not checked |  |  |
| channelBWs-DL-v1590 |  | Not checked |  |  |
| channelBWs-UL-v1590 |  | Not checked |  |  |
| asymmetricBandwidthCombinationSet |  | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-r16 |  | Not checked |  |  |
| cancelOverlappingPUSCH-r16 |  | Not checked |  |  |
| multipleRateMatchingEUTRA-CRS-r16 |  | Not checked |  |  |
| overlapRateMatchingEUTRA-CRS-r16 |  | Not checked |  |  |
| pdsch-MappingTypeB-Alt-r16 |  | Not checked |  |  |
| oneSlotPeriodicTRS-r16 |  | Not checked |  |  |
| olpc-SRS-Pos-r16 |  | Not checked |  |  |
| spatialRelationsSRS-Pos-r16 |  | Not checked |  |  |
| simulSRS-MIMO-TransWithinBand-r16 |  | Not checked |  |  |
| channelBW-DL-IAB-r16 |  | Not checked |  |  |
| channelBW-UL-IAB-r16 |  | Not checked |  |  |
| rasterShift7dot5-IAB-r16 |  | Not checked |  |  |
| ue-PowerClass-v1610 |  | Not checked |  |  |
| condHandover-r16 |  | Checked |  | pc_condHandover_r16 |
| condHandoverFailure-r16 |  | Checked |  | pc_condHandoverFailure_r16 |
| condHandoverTwoTriggerEvents-r16 |  | Checked |  | pc_condHandoverTwoTriggerEvents_r16 |
| condPSCellChange-r16 |  | Checked |  | pc_condPSCellChange_r16 |
| condPSCellChangeTwoTriggerEvents-r16 |  | Not checked |  |  |
| mpr-PowerBoost-FR2-r16 |  | Not checked |  |  |
| activeConfiguredGrant-r16 |  | Not checked |  |  |
| jointReleaseConfiguredGrantType2-r16 |  | Not checked |  |  |
| sps-r16 |  | Not checked |  |  |
| jointReleaseSPS-r16 |  | Not checked |  |  |
| simulSRS-TransWithinBand-r16 |  | Not checked |  |  |
| trs-AdditionalBandwidth-r16 |  | Not checked |  |  |
| handoverIntraF-IAB-r16 |  | Not checked |  |  |
| simulTX-SRS-AntSwitchingIntraBandUL-CA-r16 |  | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-v1630 |  | Not checked |  |  |
| handoverUTRA-FDD-r16 |  | Not checked |  |  |
| enhancedUL-TransientPeriod-r16 |  | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-v1640 |  | Not checked |  |  |
| type1-PUSCH-RepetitionMultiSlots-v1650 |  | Not checked |  |  |
| type2-PUSCH-RepetitionMultiSlots-v1650 |  | Not checked |  |  |
| pusch-RepetitionMultiSlots-v1650 |  | Not checked |  |  |
| configuredUL-GrantType1-v1650 |  | Not checked |  |  |
| configuredUL-GrantType2-v1650 |  | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-v1650 |  | Not checked |  |  |
| enhancedSkipUplinkTxConfigured-v1660 |  | Not checked |  |  |
| enhancedSkipUplinkTxDynamic-v1660 |  | Not checked |  |  |
| maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 |  | Checked |  | pc_nrBandx_maxUplinkDutyCycle_PC1dot5_MPE_FR1_r16 ('x' being the band number related PICS listed in Table A.4.3.1-4h of TS 38.508-2) |
| txDiversity-r16 |  | Not checked |  |  |
| pdsch-1024QAM-FR1-r17 |  | Not checked |  |  |
| ue-PowerClass-v1700 |  | Not checked |  |  |
| fr2-2-AccessParamsPerBand-r17 |  | Not checked |  |  |
| rlm-Relaxation-r17 |  | Not checked |  |  |
| bfd-Relaxation-r17 |  | Not checked |  |  |
| cg-SDT-r17 |  | Not checked |  |  |
| locationBasedCondHandover-r17 |  | Not checked |  |  |
| timeBasedCondHandover-r17 |  | Not checked |  |  |
| eventA4BasedCondHandover-r17 |  | Not checked |  |  |
| mn-InitiatedCondPSCellChangeNRDC-r17 |  | Not checked |  |  |
| sn-InitiatedCondPSCellChangeNRDC-r17 |  | Not checked |  |  |
| pdcch-SkippingWithoutSSSG-r17 |  | Not checked |  |  |
| sssg-Switching-1BitInd-r17 |  | Not checked |  |  |
| sssg-Switching-2BitInd-r17 |  | Not checked |  |  |
| pdcch-SkippingWithSSSG-r17 |  | Not checked |  |  |
| searchSpaceSetGrp-switchCap2-r17 |  | Not checked |  |  |
| uplinkPreCompensation-r17 |  | Not checked |  |  |
| uplink-TA-Reporting-r17 |  | Not checked |  |  |
| max-HARQ-ProcessNumber-r17 |  | Not checked |  |  |
| type2-HARQ-Codebook-r17 |  | Not checked |  |  |
| type1-HARQ-Codebook-r17 |  | Not checked |  |  |
| type3-HARQ-Codebook-r17 |  | Not checked |  |  |
| ue-specific-K-Offset-r17 |  | Not checked |  |  |
| multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 |  | Not checked |  |  |
| multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 |  | Not checked |  |  |
| parallelPRS-MeasRRC-Inactive-r17 |  | Not checked |  |  |
| nr-UE-TxTEG-ID-MaxSupport-r17 |  | Not checked |  |  |
| prs-ProcessingRRC-Inactive-r17 |  | Not checked |  |  |
| prs-ProcessingWindowType1A-r17 |  | Not checked |  |  |
| prs-ProcessingWindowType1B-r17 |  | Not checked |  |  |
| prs-ProcessingWindowType2-r17 |  | Not checked |  |  |
| srs-AllPosResourcesRRC-Inactive-r17 |  | Not checked |  |  |
| olpc-SRS-PosRRC-Inactive-r17 |  | Not checked |  |  |
| spatialRelationsSRS-PosRRC-Inactive-r17 |  | Not checked |  |  |
| maxNumberPUSCH-TypeA-Repetition-r17 |  | Not checked |  |  |
| puschTypeA-RepetitionsAvailSlot-r17 |  | Not checked |  |  |
| tb-ProcessingMultiSlotPUSCH-r17 |  | Not checked |  |  |
| tb-ProcessingRepMultiSlotPUSCH-r17 |  | Not checked |  |  |
| maxDurationDMRS-Bundling-r17 |  | Not checked |  |  |
| pusch-RepetitionCRC-r17 |  | Not checked |  |  |
| sharedSpectrumChAccessParamsPerBand-v1710 |  | Not checked |  |  |
| parallelMeasurementWithoutRestriction-r17 |  | Not checked |  |  |
| maxNumber-NGSO-SatellitesWithinOneSMTC-r17 |  | Not checked |  |  |
| k1-RangeExtension-r17 |  | Not checked |  |  |
| aperiodicCSI-RS-FastScellActivation-r17 |  | Not checked |  |  |
| aperiodicCSI-RS-AdditionalBandwidth-r17 |  | Not checked |  |  |
| bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17 |  | Not checked |  |  |
| halfDuplexFDD-TypeA-RedCap-r17 |  | Not checked |  |  |
| posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 |  | Not checked |  |  |
| channelBWs-DL-SCS-480kHz-FR2-2-r17 |  | Not checked |  |  |
| channelBWs-UL-SCS-480kHz-FR2-2-r17 |  | Not checked |  |  |
| channelBWs-DL-SCS-960kHz-FR2-2-r17 |  | Not checked |  |  |
| channelBWs-UL-SCS-960kHz-FR2-2-r17 |  | Not checked |  |  |
| ul-GapFR2-r17 |  | Not checked |  |  |
| oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 |  | Not checked |  |  |
| oneShotHARQ-feedbackPhy-Priority-r17 |  | Not checked |  |  |
| enhancedType3-HARQ-CodebookFeedback-r17 |  | Not checked |  |  |
| triggeredHARQ-CodebookRetx-r17 |  | Not checked |  |  |
| ue-OneShotUL-TimingAdj-r17 |  | Not checked |  |  |
| pucch-Repetition-F0-2-r17 |  | Not checked |  |  |
| cqi-4-BitsSubbandNTN-SharedSpectrumChAccess-r17 |  | Not checked |  |  |
| mux-HARQ-ACK-DiffPriorities-r17 |  | Not checked |  |  |
| ta-BasedPDC-NTN-SharedSpectrumChAccess-r17 |  | Not checked |  |  |
| ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17 |  | Not checked |  |  |
| maxNumberG-RNTI-r17 |  | Not checked |  |  |
| dynamicMulticastDCI-Format4-2-r17 |  | Not checked |  |  |
| maxModulationOrderForMulticast-r17 |  | Not checked |  |  |
| dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |
| dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17 |  | Not checked |  |  |
| nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17 |  | Not checked |  |  |
| ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17 |  | Not checked |  |  |
| maxNumberG-CS-RNTI-r17 |  | Not checked |  |  |
| re-LevelRateMatchingForMulticast-r17 |  | Not checked |  |  |
| pdsch-1024QAM-2MIMO-FR1-r17 |  | Not checked |  |  |
| prs-MeasurementWithoutMG-r17 |  | Not checked |  |  |
| maxNumber-LEO-SatellitesPerCarrier-r17 |  | Not checked |  |  |
| prs-ProcessingCapabilityOutsideMGinPPW-r17 |  | Not checked |  |  |
| srs-SemiPersistent-PosResourcesRRC-Inactive-r17 |  | Not checked |  |  |
| channelBWs-DL-SCS-120kHz-FR2-2-r17 |  | Not checked |  |  |
| channelBWs-UL-SCS-120kHz-FR2-2-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| supportedBandCombinationList SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination { |  |  |  |  |
| BandCombination SEQUENCE { |  |  |  |  |
| bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters { |  |  |  |  |
| BandParameters CHOICE { |  |  |  |  |
| eutra SEQUENCE { |  |  | EUTRA bands |  |
| bandEUTRA |  | Not checked | FreqBandIndicatorEUTRA |  |
| ca-BandwidthClassDL-EUTRA |  | Not checked | CA-BandwidthClassEUTRA |  |
| ca-BandwidthClassUL-EUTRA |  | Not checked | CA-BandwidthClassEUTRA |  |
| } |  |  |  |  |
| nr SEQUENCE { |  |  | NR bands |  |
| bandNR |  | Not checked | FreqBandIndicatorNR |  |
| ca-BandwidthClassDL-NR |  | Not checked | CA-BandwidthClassNR |  |
| ca-BandwidthClassUL-NR |  | Not checked | CA-BandwidthClassNR |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureSetCombination |  | Not checked | FeatureSetCombinationId |  |
| ca-ParametersEUTRA |  | Not checked | CA-ParametersEUTRA |  |
| ca-ParametersNR |  | Not checked | CA-ParametersNR |  |
| mrdc-Parameters |  | Not checked | MRDC-Parameters |  |
| supportedBandwidthCombinationSet |  | Not checked |  |  |
| powerClass-v1530 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| appliedFreqBandListFilter |  | Not checked | FreqBandList |  |
| supportedBandCombinationList-v1540 SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1540 { |  |  | BandCombinationList-v1540 |  |
| BandCombination-v1540 SEQUENCE { |  |  |  |  |
| bandList-v1540 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1540 { |  | Not checked | BandParameters-v1540 |  |
| BandParameters-v1540 SEQUENCE { |  |  |  |  |
| srs-CarrierSwitch CHOICE { |  |  |  |  |
| nr SEQUENCE { |  |  | NR bands |  |
| srs-SwitchingTimesListNR SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR |  | Not checked |  |  |
| } |  |  |  |  |
| eutra SEQUENCE { |  |  | EUTRA bands |  |
| srs-SwitchingTimesListEUTRA SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| srs-TxSwitch-v1540 SEQUENCE { |  |  |  |  |
| supportedSRS-TxPortSwitch |  | Not checked |  |  |
| txSwitchImpactToRx |  | Not checked |  |  |
| txSwitchWithAnotherBand |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| ca-ParametersNR-v1540 |  | Not checked | CA-ParametersNR-v1540 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| srs-SwitchingTimeRequested |  | Not checked |  |  |
| supportedBandCombinationList-v1550 |  | Not checked | BandCombinationList-v1550 |  |
| supportedBandCombinationList-v1560 |  | Not checked | BandCombinationList-v1560 |  |
| supportedBandCombinationList-v1570 |  | Not checked | BandCombinationList-v1570 |  |
| supportedBandCombinationList-v1580 |  | Not checked | BandCombinationList-v1580 |  |
| supportedBandCombinationList-v1590 |  | Not checked | BandCombinationList-v1590 |  |
| supportedBandCombinationList-v15a0 |  | Not checked | BandCombinationList-v15a0 |  |
| supportedBandCombinationList-v1610 SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1610 { |  |  | BandCombinationList-v1610 |  |
| BandCombination-v1610 SEQUENCE { |  |  |  |  |
| bandList-v1610 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1610 { |  |  |  |  |
| ca-ParametersNR-v1610 |  | Checked | CA-ParametersNR-v1610 (Table 8.2.1.1.1.3.3-11) |  |
| ca-ParametersNRDC-v1610 |  | Not checked | CA-ParametersNRDC-v1610 |  |
| powerClass-v1610 |  | Not checked |  |  |
| powerClassNRPart-r16 |  | Not checked |  |  |
| featureSetCombinationDAPS-r16 |  | Not checked | FeatureSetCombinationId |  |
| mrdc-Parameters-v1620 |  | Not checked | MRDC-Parameters-v1620 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| supportedBandCombinationListSidelinkEUTRA-NR-r16 |  | Not checked | BandCombinationListSidelinkEUTRA-NR-r16 |  |
| supportedBandCombinationList-UplinkTxSwitch-r16 |  | Not checked | BandCombinationList-UplinkTxSwitch-r16 |  |
| supportedBandCombinationList-v1630 |  | Not checked | BandCombinationList-v1630 |  |
| supportedBandCombinationListSidelinkEUTRA-NR-v1630 |  | Not checked | BandCombinationListSidelinkEUTRA-NR-v1630 |  |
| supportedBandCombinationList-UplinkTxSwitch-v1630 |  | Not checked | BandCombinationList-UplinkTxSwitch-v1630 |  |
| supportedBandCombinationList-v1640 |  | Not checked | BandCombinationList-v1640 |  |
| supportedBandCombinationList-UplinkTxSwitch-v1640 |  | Not checked | BandCombinationList-UplinkTxSwitch-v1640 |  |
| supportedBandCombinationList-v1650 |  | Not checked | BandCombinationList-v1650 |  |
| supportedBandCombinationList-UplinkTxSwitch-v1650 |  | Not checked | BandCombinationList-UplinkTxSwitch-v1650 |  |
| extendedBand-n77-r16 |  | Checked |  | pc_extendedBand_n77_r16 |
| supportedBandCombinationList-UplinkTxSwitch-v1670 |  | Not checked | BandCombinationList-UplinkTxSwitch-v1670 |  |
| supportedBandCombinationList-v1680 |  | Not checked | BandCombinationList-v1680 |  |
| supportedBandCombinationList-v1690 |  | Not checked | BandCombinationList-v1690 |  |
| supportedBandCombinationList-UplinkTxSwitch-v1690 |  | Not checked | BandCombinationList-UplinkTxSwitch-v1690 |  |
| supportedBandCombinationList-v1700 |  | Not checked | BandCombinationList-v1700 |  |
| supportedBandCombinationList-UplinkTxSwitch-v1700 |  | Not checked | BandCombinationList-UplinkTxSwitch-v1700 |  |
| supportedBandCombinationListSL-RelayDiscovery-r17 |  | Not checked |  |  |
| supportedBandCombinationListSL-NonRelayDiscovery-r17 |  | Not checked |  |  |
| supportedBandCombinationListSidelinkEUTRA-NR-v1710 |  | Not checked | BandCombinationListSidelinkEUTRA-NR-v1710 |  |
| sidelinkRequested-r17 |  | Not checked |  |  |
| extendedBand-n77-2-r17 |  | Checked |  | pc_extendedBand_n77_2_r17 |
| supportedBandCombinationList-v1720 |  | Not checked | BandCombinationList-v1720 |  |
| supportedBandCombinationList-UplinkTxSwitch-v1720 |  | Not checked | BandCombinationList-UplinkTxSwitch-v1720 |  |
| } |  |  |  |  |
| measAndMobParameters SEQUENCE |  |  |  |  |
| measAndMobParametersCommon SEQUENCE { |  |  |  |  |
| supportedGapPattern |  | Not checked |  |  |
| ssb-RLM |  | supported |  |  |
| ssb-AndCSI-RS-RLM |  | Not checked |  |  |
| eventB-MeasAndReport |  | Checked |  | pc_eventB_MeasAndReport |
| handoverFDD-TDD |  | Not checked |  |  |
| eutra-CGI-Reporting |  | Checked |  | pc_eutra_CGI_Reporting |
| nr-CGI-Reporting |  | Checked |  | pc_nr_CGI_Reporting |
| independentGapConfig |  | Checked |  | pc_independentGapConfig |
| periodicEUTRA-MeasAndReport |  | Checked |  | pc_periodicEUTRA_MeasAndReport |
| handoverFR1-FR2 |  | Not checked |  |  |
| maxNumberCSI-RS-RRM-RS-SINR |  | Not checked |  |  |
| nr-CGI-Reporting-ENDC |  | Not checked |  |  |
| eutra-CGI-Reporting-NEDC |  | Not checked |  |  |
| eutra-CGI-Reporting-NRDC |  | Not checked |  |  |
| nr-CGI-Reporting-NEDC |  | Not checked |  |  |
| nr-CGI-Reporting-NRDC |  | Not checked |  |  |
| reportAddNeighMeasForPeriodic-r16 |  | Not checked |  |  |
| condHandoverParametersCommon-r16 |  | Not checked |  |  |
| nr-NeedForGap-Reporting-r16 |  | Not checked |  |  |
| supportedGapPattern-NRonly-r16 |  | Not checked |  |  |
| supportedGapPattern-NRonly-NEDC-r16 |  | Not checked |  |  |
| maxNumberCLI-RSSI-r16 |  | Not checked |  |  |
| maxNumberCLI-SRS-RSRP-r16 |  | Not checked |  |  |
| maxNumberPerSlotCLI-SRS-RSRP-r16 |  | Not checked |  |  |
| mfbi-IAB-r16 |  | Not checked |  |  |
| dummy |  | Not checked |  |  |
| nr-CGI-Reporting-NPN-r16 |  | Checked |  | pc_nr_CGI_Reporting_NPN_r16 |
| idleInactiveEUTRA-MeasReport-r16 |  | Not checked |  |  |
| idleInactive-ValidityArea-r16 |  | Not checked |  |  |
| eutra-AutonomousGaps-r16 |  | Not checked |  |  |
| eutra-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |
| eutra-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |
| pcellT312-r16 |  | Not checked |  |  |
| supportedGapPattern-r16 |  | Not checked |  |  |
| concurrentMeasGap-r17 | Not checked |  |  |  |
| nr-NeedForGapNCSG-Reporting-r17 | Not checked |  |  |  |
| eutra-NeedForGapNCSG-Reporting-r17 | Not checked |  |  |  |
| ncsg-MeasGapPerFR-r17 | Not checked |  |  |  |
| ncsg-MeasGapPatterns-r17 | Not checked |  |  |  |
| ncsg-MeasGapNR-Patterns-r17 | Not checked |  |  |  |
| preconfiguredUE-AutonomousMeasGap-r17 | Not checked |  |  |  |
| preconfiguredNW-ControlledMeasGap-r17 | Not checked |  |  |  |
| handoverFR1-FR2-2-r17 | Not checked |  |  |  |
| handoverFR2-1-FR2-2-r17 | Not checked |  |  |  |
| independentGapConfigPRS-r17 | Not checked |  |  |  |
| rrm-RelaxationRRC-ConnectedRedCap-r17 | Not checked |  |  |  |
| parallelMeasurementGap-r17 | Not checked |  |  |  |
| condHandoverWithSCG-NRDC-r17 | Not checked |  |  |  |
| gNB-ID-LengthReporting-r17 | Not checked |  |  |  |
| gNB-ID-LengthReporting-ENDC-r17 | Not checked |  |  |  |
| gNB-ID-LengthReporting-NEDC-r17 | Not checked |  |  |  |
| gNB-ID-LengthReporting-NRDC-r17 | Not checked |  |  |  |
| gNB-ID-LengthReporting-NPN-r17 | Not checked |  |  |  |
| parallelSMTC-r17 | Not checked |  |  |  |
| concurrentMeasGapEUTRA-r17 | Not checked |  |  |  |
| serviceLinkPropDelayDiffReporting-r17 | Not checked |  |  |  |
| ncsg-SymbolLevelScheduleRestrictionInter-r17 | Not checked |  |  |  |
| } |  |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |  |
| intraAndInterF-MeasAndReport |  | Checked (NOTE 1) |  | pc_intraAndInterF-MeasAndReport |
| eventA-MeasAndReport |  | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF |  | Checked (NOTE 15) |  | pc_ handoverInterF |
| handoverLTE |  | Not checked |  |  |
| handover-eLTE |  | Not checked |  |  |
| sftd-MeasNR-Neigh |  | Checked (NOTE 13) |  | pc_SFTD_MeasNR_Neigh_FDD and/or pc_SFTD_MeasNR_Neigh_TDD |
| sftd-MeasNR-Neigh-DRX |  | Not checked |  |  |
| dummy |  | Not checked |  |  |
| } |  |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |  |
| ss-SINR-Meas |  | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB |  | Checked (NOTE 3) |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB |  | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas |  | Not checked |  |  |
| csi-RS-RLM |  | Not checked |  |  |
| handoverInterF |  | Checked (NOTE 14) |  | pc_ handoverInterF |
| handoverLTE |  | Not checked |  |  |
| handover-eLTE |  | Not checked |  |  |
| maxNumberResource-CSI-RS-RLM |  | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology |  | Not checked |  |  |
| nr-AutonomousGaps-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |
| dummy |  | Not checked |  |  |
| cli-RSSI-Meas-r16 |  | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 |  | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 |  | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 |  | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 |  | Not checked |  |  |
| idleInactiveNR-MeasBeamReport-r16 |  | Not checked |  |  |
| increasedNumberofCSIRSPerMO-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| fdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 17) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |
| } |  |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |  |
| skipUplinkTxDynamic |  | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer |  | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle |  | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle |  | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations |  | Not checked |  |  |
| multipleConfiguredGrants |  | Checked (NOTE 12) |  | pc_multipleConfiguredGrants_r16 |
| } |  |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |  |
| intraAndInterF-MeasAndReport |  | Checked (NOTE 1) |  | pc_intraAndInterF-MeasAndReport |
| eventA-MeasAndReport |  | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF |  | Checked (NOTE 15) |  | pc_ handoverInterF |
| handoverLTE |  | Not checked |  |  |
| handover-eLTE |  | Not checked |  |  |
| sftd-MeasNR-Neigh |  | Checked (NOTE 13) |  | pc_SFTD_MeasNR_Neigh_FDD and/or pc_SFTD_MeasNR_Neigh_TDD |
| sftd-MeasNR-Neigh-DRX |  | Not checked |  |  |
| dummy |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| tdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 17) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |
| } |  |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |  |
| skipUplinkTxDynamic |  | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer |  | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle |  | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle |  | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations |  | Not checked |  |  |
| multipleConfiguredGrants |  | Checked (NOTE 15) |  | pc_multipleConfiguredGrants_r16 |
| } |  |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |  |
| intraAndInterF-MeasAndReport |  | Checked (NOTE 1) |  | pc_intraAndInterF-MeasAndReport |
| eventA-MeasAndReport |  | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF |  | Checked (NOTE 15) |  | pc_ handoverInterF |
| handoverLTE-EPC |  | Not checked |  |  |
| handover-eLTE-5GC |  | Not checked |  |  |
| sftd-MeasNR-Neigh |  | Checked (NOTE 13) |  | pc_SFTD_MeasNR_Neigh_FDD and/or pc_SFTD_MeasNR_Neigh_TDD |
| sftd-MeasNR-Neigh-DRX |  | Not checked |  |  |
| dummy |  | Not checked |  |  |
| } |  |  |  |  |
| fr1-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |
| dummy1 |  | Not checked |  |  |
| twoFL-DMRS |  | Not checked |  |  |
| dummy2 |  | Not checked |  |  |
| dummy3 |  | Not checked |  |  |
| supportedDMRS-TypeDL |  | Not checked |  |  |
| supportedDMRS-TypeUL |  | Not checked |  |  |
| semiOpenLoopCSI |  | Not checked |  |  |
| csi-ReportWithoutPMI |  | Not checked |  |  |
| csi-ReportWithoutCQI |  | Not checked |  |  |
| onePortsPTRS |  | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |
| pucch-F2-WithFH |  | Not checked |  |  |
| pucch-F3-WithFH |  | Not checked |  |  |
| pucch-F4-WithFH |  | Not checked |  |  |
| freqHoppingPUCCH-F0-2 |  | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 |  | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot |  | Not checked |  |  |
| uci-CodeBlockSegmentation |  | Not checked |  |  |
| onePUCCH-LongAndShortFormat |  | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot |  | Not checked |  |  |
| intraSlotFreqHopping-PUSCH |  | Not checked |  |  |
| pusch-LBRM |  | Not checked |  |  |
| pdcch-BlindDetectionCA |  | Not checked |  |  |
| tpc-PUSCH-RNTI |  | Not checked |  |  |
| tpc-PUCCH-RNTI |  | Not checked |  |  |
| tpc-SRS-RNTI |  | Not checked |  |  |
| absoluteTPC-Command |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |
| pusch-HalfPi-BPSK |  | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK |  | Not checked |  |  |
| almostContiguousCP-OFDM-UL |  | Not checked |  |  |
| sp-CSI-RS |  | Not checked |  |  |
| sp-CSI-IM |  | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot |  | Not checked |  |  |
| multipleCORESET |  | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback |  | Not checked |  |  |
| csi-RS-ProcFrameworkForSRS |  | Not checked |  |  |
| csi-ReportFramework |  | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |  |
| sameSymbol |  | Not checked |  |  |
| diffSymbol |  | Not checked |  |  |
| } |  |  |  |  |
| mux-SR-HARQ-ACK-PUCCH |  | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap |  | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |
| dl-64QAM-MCS-TableAlt |  | Not checked |  |  |
| ul-64QAM-MCS-TableAlt |  | Not checked |  |  |
| cqi-TableAlt |  | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL |  | Not checked |  |  |
| pdcch-BlindDetectionNRDC |  | Not checked |  |  |
| mux-HARQ-ACK-PUSCH-DiffSymbol |  | Not checked |  |  |
| type1-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |
| enhancedPowerControl-r16 |  | Not checked |  |  |
| simultaneousTCI-ActMultipleCC-r16 |  | Not checked |  |  |
| simultaneousSpatialRelationMultipleCC-r16 |  | Not checked |  |  |
| cli-RSSI-FDM-DL-r16 |  | Not checked |  |  |
| cli-SRS-RSRP-FDM-DL-r16 |  | Not checked |  |  |
| maxLayersMIMO-Adaptation-r16 |  | Not checked |  |  |
| aggregationFactorSPS-DL-r16 |  | Not checked |  |  |
| maxTotalResourcesForOneFreqRange-r16 |  | Not checked |  |  |
| csi-ReportFrameworkExt-r16 |  | Not checked |  |  |
| twoTCI-Act-servingCellInCC-List-r16 |  | Not checked |  |  |
| cri-RI-CQI-WithoutNon-PMI-PortInd-r16 |  | Not checked |  |  |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |  |
| ss-SINR-Meas |  | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB |  | Checked (NOTE 3) |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB |  | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas |  | Not checked |  |  |
| csi-RS-RLM |  | Not checked |  |  |
| handoverInterF |  | Checked (NOTE 14) |  |  |
| handoverLTE |  | Not checked |  |  |
| handover-eLTE |  | Not checked |  |  |
| maxNumberResource-CSI-RS-RLM |  | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology |  | Not checked |  |  |
| nr-AutonomousGaps-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |
| dummy |  | Not checked |  |  |
| cli-RSSI-Meas-r16 |  | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 |  | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 |  | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 |  | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 |  | Not checked |  |  |
| idleInactiveNR-MeasBeamReport-r16 |  | Not checked |  |  |
| increasedNumberofCSIRSPerMO-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  | Not checked |  |  |
| dynamicSFI |  | Not checked |  |  |
| dummy1 |  | Not checked |  |  |
| twoFL-DMRS |  | Not checked |  |  |
| dummy2 |  | Not checked |  |  |
| dummy3 |  | Not checked |  |  |
| supportedDMRS-TypeDL |  | Not checked |  |  |
| supportedDMRS-TypeUL |  | Not checked |  |  |
| semiOpenLoopCSI |  | Not checked |  |  |
| csi-ReportWithoutPMI |  | Not checked |  |  |
| csi-ReportWithoutCQI |  | Not checked |  |  |
| onePortsPTRS |  | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |
| pucch-F2-WithFH |  | Not checked |  |  |
| pucch-F3-WithFH |  | Not checked |  |  |
| pucch-F4-WithFH |  | Not checked |  |  |
| freqHoppingPUCCH-F0-2 |  | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 |  | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot |  | Not checked |  |  |
| uci-CodeBlockSegmentation |  | Not checked |  |  |
| onePUCCH-LongAndShortFormat |  | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot |  | Not checked |  |  |
| intraSlotFreqHopping-PUSCH |  | Not checked |  |  |
| pusch-LBRM |  | Not checked |  |  |
| pdcch-BlindDetectionCA |  | Not checked |  |  |
| tpc-PUSCH-RNTI |  | Not checked |  |  |
| tpc-PUCCH-RNTI |  | Not checked |  |  |
| tpc-SRS-RNTI |  | Not checked |  |  |
| absoluteTPC-Command |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |
| pusch-HalfPi-BPSK |  | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK |  | Not checked |  |  |
| almostContiguousCP-OFDM-UL |  | Not checked |  |  |
| sp-CSI-RS |  | Not checked |  |  |
| sp-CSI-IM |  | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot |  | Not checked |  |  |
| multipleCORESET |  | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback |  | Not checked |  |  |
| csi-RS-ProcFrameworkForSRS |  | Not checked |  |  |
| csi-ReportFramework |  | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |  |
| sameSymbol |  | Not checked |  |  |
| diffSymbol |  | Not checked |  |  |
| } |  |  |  |  |
| mux-SR-HARQ-ACK-PUCCH |  | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap |  | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |
| dl-64QAM-MCS-TableAlt |  | Not checked |  |  |
| ul-64QAM-MCS-TableAlt |  | Not checked |  |  |
| cqi-TableAlt |  | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL |  | Not checked |  |  |
| pdcch-BlindDetectionNRDC |  | Not checked |  |  |
| mux-HARQ-ACK-PUSCH-DiffSymbol |  | Not checked |  |  |
| type1-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |
| enhancedPowerControl-r16 |  | Not checked |  |  |
| simultaneousTCI-ActMultipleCC-r16 |  | Not checked |  |  |
| simultaneousSpatialRelationMultipleCC-r16 |  | Not checked |  |  |
| cli-RSSI-FDM-DL-r16 |  | Not checked |  |  |
| cli-SRS-RSRP-FDM-DL-r16 |  | Not checked |  |  |
| maxLayersMIMO-Adaptation-r16 |  | Not checked |  |  |
| aggregationFactorSPS-DL-r16 |  | Not checked |  |  |
| maxTotalResourcesForOneFreqRange-r16 |  | Not checked |  |  |
| csi-ReportFrameworkExt-r16 |  | Not checked |  |  |
| twoTCI-Act-servingCellInCC-List-r16 |  | Not checked |  |  |
| cri-RI-CQI-WithoutNon-PMI-PortInd-r16 |  | Not checked |  |  |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |  |
| ss-SINR-Meas |  | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB |  | Checked (NOTE 3) |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB |  | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas |  | Not checked |  |  |
| csi-RS-RLM |  | Not checked |  |  |
| handoverInterF |  | Checked (NOTE 14) |  | pc_ handoverInterF |
| handoverLTE-EPC |  | Not checked |  |  |
| handover-eLTE-5GC |  | Not checked |  |  |
| maxNumberResource-CSI-RS-RLM |  | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology |  | Not checked |  |  |
| nr-AutonomousGaps-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |
| dummy |  | Not checked |  |  |
| cli-RSSI-Meas-r16 |  | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 |  | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 |  | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 |  | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 |  | Not checked |  |  |
| idleInactiveNR-MeasBeamReport-r16 |  | Not checked |  |  |
| increasedNumberofCSIRSPerMO-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureSets SEQUENCE { |  |  |  |  |
| featureSetsDownlink SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink { |  |  |  |  |
| FeatureSetDownlink SEQUENCE { |  | Not checked |  |  |
| featureSetListPerDownlinkCC |  | Not checked |  |  |
| intraBandFreqSeparationDL |  | Not checked |  |  |
| scalingFactor |  | Not checked |  |  |
| crossCarrierScheduling-OtherSCS |  | Not checked |  |  |
| scellWithoutSSB |  | Not checked |  |  |
| csi-RS-MeasSCellWithoutSSB |  | Not checked |  |  |
| dummy1 |  | Not checked |  |  |
| type1-3-CSS |  | Not checked |  |  |
| pdcch-MonitoringAnyOccasions |  | Not checked |  |  |
| dummy2 |  | Not checked |  |  |
| ue-SpecificUL-DL-Assignment |  | Not checked |  |  |
| searchSpaceSharingCA-DL |  | Not checked |  |  |
| timeDurationForQCL SEQUENCE { |  |  |  |  |
| scs-60kHz |  | Not checked |  |  |
| scs-120kHz |  | Not checked |  |  |
| } |  |  |  |  |
| pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |
| scs-30kHz |  | Not checked |  |  |
| scs-60kHz |  | Not checked |  |  |
| scs-120kHz |  | Not checked |  |  |
| } |  |  |  |  |
| dummy3 |  | Not checked |  |  |
| dummy4 |  | Not checked |  |  |
| dummy5 |  | Not checked |  |  |
| dummy6 |  | Not checked |  |  |
| dummy7 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureSetsDownlinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC { |  |  |  |  |
| FeatureSetDownlinkPerCCSEQUENCE { |  |  |  |  |
| supportedSubcarreirSpacingDL |  | Not checked | SubcarrierSpacing |  |
| supportedBandwidthDL |  | Not checked | SupportedBandwidth |  |
| channelBW-90mhz |  | Not checked |  |  |
| maxNumberMIMO-LayersPDSCH |  | Not checked | MIMO-LayersDL |  |
| supportedModulationOrderDL |  | Not checked | ModulationOrder |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureSetsUplink SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink { |  |  |  |  |
| FeatureSetUplink SEQUENCE { |  |  |  |  |
| featureSetListPerUplinkCC |  | Not checked |  |  |
| scalingFactor |  | Not checked |  |  |
| crossCarrierScheduling-OtherSCS |  | Not checked |  |  |
| intraBandFreqSeparationUL |  | Not checked |  |  |
| searchSpaceSharingCA-UL |  | Not checked |  |  |
| dummy1 |  | Not checked |  |  |
| supportedSRS-Resources |  | Not checked | SRS-Resources |  |
| twoPUCCH-Group |  | Checked |  | pc_twoPUCCH_group |
| dynamicSwitchSUL |  | Checked |  | pc_dynamicSwitch_SUL |
| simultaneousTxSUL-NonSUL |  | Not checked |  |  |
| pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |
| scs-30kHz |  | Not checked |  |  |
| scs-60kHz |  | Not checked |  |  |
| scs-120kHz |  | Not checked |  |  |
| } |  |  |  |  |
| dummy2 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureSetsUplinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC { |  |  |  |  |
| FeatureSetUplinkPerCC SEQUENCE { |  |  |  |  |
| supportedSubcarrierSpacingUL |  | Not checked | SubcarrierSpacing |  |
| supportedBandwidthUL |  | Not checked | SupportedBandwidth |  |
| channelBW-90mHz |  | Not checked |  |  |
| mimo-CB-PUSCH SEQUENCE { |  |  |  |  |
| maxNumberMIMO-LayersCB-PUSCH |  | Not checked |  |  |
| maxNumberSRS-ResourcePerSet |  | Not checked |  |  |
| } |  |  |  |  |
| maxNumberMIMO-LayersNonCB-PUSCH |  | Not checked |  |  |
| supportedModulationOrderUL |  | Not checked | ModulationOrder |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureSetsDownlink-v1540 SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1540 { |  |  |  |  |
| FeatureSetDownlink-v1540SEQUENCE { |  |  |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-DL |  | Not checked |  |  |
| additionalDMRS-DL-Alt |  | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-DL |  | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-DL |  | Not checked |  |  |
| pdcch-MonitoringAnyOccasionsWithSpanGap SEQUENCE { |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |
| scs-30kHz |  | Not checked |  |  |
| scs-60kHz |  | Not checked |  |  |
| scs-120kHz |  | Not checked |  |  |
| } |  |  |  |  |
| pdsch-SeparationWithGap |  | Not checked |  |  |
| pdsch-ProcessingType2 SEQUENCE { |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |
| scs-30kHz |  | Not checked |  |  |
| scs-60kHz |  | Not checked |  |  |
| } |  |  |  |  |
| pdsch-ProcessingType2-Limited SEQUENCE { |  |  |  |  |
| differentTB-PerSlot-SCS-30kHz |  | Not checked |  |  |
| } |  |  |  |  |
| dl-MCS-TableAlt-DynamicIndication |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureSetsUplink-v1540 SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink-v1540 { |  |  |  |  |
| FeatureSetUplink-v1540 SEQUENCE { |  |  |  |  |
| zeroSlotOffsetAperiodicSRS |  | Not checked |  |  |
| pa-PhaseDiscontinuityImpacts |  | Not checked |  |  |
| pusch-SeparationWithGap |  | Not checked |  |  |
| pusch-ProcessingType2 SEQUENCE { |  |  |  |  |
| scs-15kHz |  | Not checked |  |  |
| scs-30kHz |  | Not checked |  |  |
| scs-60kHz |  | Not checked |  |  |
| } |  |  |  |  |
| ul-MCS-TableAlt-DynamicIndication |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureSetsUplinkPerCC-v1540 SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1540 { |  | i entries |  |  |
| FeatureSetUplinkPerCC-v1540SEQUENCE { |  |  |  |  |
| mimo-NonCB-PUSCH SEQUENCE { |  |  |  |  |
| maxNumberSRS-ResourcePerSet |  | Not checked |  |  |
| maxNumberSimultaneousSRS-ResourceTx |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureSetsDownlink-v15a0 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v15a0 |  | Not checked |  |  |
| featureSetsDownlink-v1610 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1610 |  | Checked | FeatureSetDownlink-v1610 (Table 8.2.1.1.1.3.3-9) |  |
| featureSetsUplink-v1610 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1610 |  | Checked | FeatureSetUplink-v1610 (Table 8.2.1.1.1.3.3-10) |  |
| featureSetDownlinkPerCC-v1620 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1620 |  | Not checked |  |  |
| featureSetsUplink-v1630 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1630 |  | Not checked |  |  |
| featureSetsUplink-v1640 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1640 |  | Not checked |  |  |
| featureSetsDownlink-v1700 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1700 |  | Not checked |  |  |
| featureSetsDownlinkPerCC-v1700 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1700 |  | Not checked |  |  |
| featureSetsUplink-v1710 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1710 |  | Not checked |  |  |
| featureSetsUplinkPerCC-v1700 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1700 |  | Not checked |  |  |
| featureSetsDownlink-v1720 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1720 |  | Not checked |  |  |
| featureSetsDownlinkPerCC-v1720 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1720 |  | Not checked |  |  |
| featureSetsUplink-v1720 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1720 |  | Not checked |  |  |
| } |  |  |  |  |
| featureSetCombinations SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination { |  |  |  |  |
| FeatureSetCombination SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand { |  |  |  |  |
| FeatureSetsPerBand SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet { |  |  |  |  |
| FeatureSetCHOICE { |  |  |  |  |
| eutra SEQUENCE { |  |  |  |  |
| downlinkSetEUTRA |  | Not checked | FeatureSetEUTRA-DownlinkId |  |
| uplinkSetEUTRA |  | Not checked | FeatureSetEUTRA-UplinkId |  |
| } |  |  |  |  |
| nr SEQUENCE { |  |  |  |  |
| downlinkSetNR |  | Not checked | FeatureSetDownlinkId |  |
| uplinkSetNR |  | Not checked | FeatureSetUpinkId |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| fdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |  |
| rsrqMeasWidebandEUTRA |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| tdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |  |
| rsrqMeasWidebandEUTRA |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| dummy |  | Not checked |  |  |
| interRAT-Parameters SEQUENCE { |  |  |  |  |
| eutra SEQUENCE { |  |  |  |  |
| supportedBandListEUTRA SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA |  | Not checked |  |  |
| eutra-ParametersCommon SEQUENCE { |  |  |  |  |
| mfbi-EUTRA |  | Not checked |  |  |
| modifiedMRP-BehaviorEUTRA |  | Not checked |  |  |
| multiNS-Pmax-EUTRA |  | Not checked |  |  |
| rs-SINR-MeasEUTRA |  | C hecked |  | pc_RS_SINR_MeasEUTRA |
| ne-DC |  | Not checked |  |  |
| nr-HO-ToEN-DC-r16 |  | Checked |  | pc_interRAT_NR_ToENDC |
| } |  |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |  |
| rsrqMeasWidebandEUTRA |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| inactiveState |  | Checked |  | pc_inactiveState |
| delayBudgetReporting |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| sdap-Parameters SEQUENCE { |  |  |  |  |
| as-ReflectiveQoS |  | Checked |  | pc_as_ReflectiveQoS |
| } |  |  |  |  |
| overheatingInd |  | Not checked |  |  |
| ims-Parameters SEQUENCE { |  |  |  |  |
| ims-ParametersCommon SEQUENCE { |  |  |  |  |
| voiceOverEUTRA-5GC |  | Not checked |  |  |
| voiceOverSCG-BearerEUTRA-5GC |  | Not checked |  |  |
| voiceFallbackIndicationEPS-r16 |  | Checked |  | pc_voiceFallbackIndication |
| } |  |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |  |
| voiceOverNR |  | Checked (NOTE 16) |  | pc_voiceOverNR |
| } |  |  |  |  |
| } |  |  |  |  |
| fr1-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |  |
| voiceOverNR |  | Checked (NOTE 16) |  | pc_voiceOverNR |
| } |  |  |  |  |
| } |  |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |  |
| voiceOverNR |  | Checked (NOTE 16) |  | pc_voiceOverNR |
| } |  |  |  |  |
| } |  |  |  |  |
| fr1-fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |  |
| dynamicSFI |  | Not checked |  |  |
| dummy1 |  | Not checked |  |  |
| twoFL-DMRS |  | Not checked |  |  |
| dummy2 |  | Not checked |  |  |
| dummy3 |  | Not checked |  |  |
| supportedDMRS-TypeDL |  | Not checked |  |  |
| supportedDMRS-TypeUL |  | Not checked |  |  |
| semiOpenLoopCSI |  | Not checked |  |  |
| csi-ReportWithoutPMI |  | Not checked |  |  |
| csi-ReportWithoutCQI |  | Not checked |  |  |
| onePortsPTRS |  | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols |  | Not checked |  |  |
| pucch-F2-WithFH |  | Not checked |  |  |
| pucch-F3-WithFH |  | Not checked |  |  |
| pucch-F4-WithFH |  | Not checked |  |  |
| freqHoppingPUCCH-F0-2 |  | Not checked |  |  |
| freqHoppingPUCCH-F1-3-4 |  | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot |  | Not checked |  |  |
| uci-CodeBlockSegmentation |  | Not checked |  |  |
| onePUCCH-LongAndShortFormat |  | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot |  | Not checked |  |  |
| intraSlotFreqHopping-PUSCH |  | Not checked |  |  |
| pusch-LBRM |  | Not checked |  |  |
| pdcch-BlindDetectionCA |  | Not checked |  |  |
| tpc-PUSCH-RNTI |  | Not checked |  |  |
| tpc-PUCCH-RNTI |  | Not checked |  |  |
| tpc-SRS-RNTI |  | Not checked |  |  |
| absoluteTPC-Command |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH |  | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH |  | Not checked |  |  |
| pusch-HalfPi-BPSK |  | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK |  | Not checked |  |  |
| almostContiguousCP-OFDM-UL |  | Not checked |  |  |
| sp-CSI-RS |  | Not checked |  |  |
| sp-CSI-IM |  | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot |  | Not checked |  |  |
| multipleCORESET |  | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback |  | Not checked | CSI-RS-IM-eceptionForFeedback |  |
| csi-RS-ProcFrameworkForSRS |  | Not checked | CSI-RS-ProcFrameworkForSRS |  |
| csi-ReportFramework |  | Not checked | CSI-ReportFramework |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |  |
| sameSymbol |  | Not checked |  |  |
| diffSymbol |  | Not checked |  |  |
| } |  |  |  |  |
| mux-SR-HARQ-ACK-PUCCH |  | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap |  | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA |  | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB |  | Not checked |  |  |
| ul-SchedulingOffset |  | Not checked |  |  |
| dl-64QAM-MCS-TableAlt |  | Not checked |  |  |
| ul-64QAM-MCS-TableAlt |  | Not checked |  |  |
| cqi-TableAlt |  | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL |  | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL |  | Not checked |  |  |
| pdcch-BlindDetectionNRDC |  | Not checked |  |  |
| mux-HARQ-ACK-PUSCH-DiffSymbol |  | Not checked |  |  |
| type1-HARQ-ACK-Codebook-r16 |  | Not checked |  |  |
| enhancedPowerControl-r16 |  | Not checked |  |  |
| simultaneousTCI-ActMultipleCC-r16 |  | Not checked |  |  |
| simultaneousSpatialRelationMultipleCC-r16 |  | Not checked |  |  |
| cli-RSSI-FDM-DL-r16 |  | Not checked |  |  |
| cli-SRS-RSRP-FDM-DL-r16 |  | Not checked |  |  |
| maxLayersMIMO-Adaptation-r16 |  | Not checked |  |  |
| aggregationFactorSPS-DL-r16 |  | Not checked |  |  |
| maxTotalResourcesForOneFreqRange-r16 |  | Not checked |  |  |
| csi-ReportFrameworkExt-r16 |  | Not checked |  |  |
| twoTCI-Act-servingCellInCC-List-r16 |  | Not checked |  |  |
| cri-RI-CQI-WithoutNon-PMI-PortInd-r16 |  | Not checked |  |  |
| cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 |  | Not checked |  |  |
| } |  |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |  |
| ss-SINR-Meas |  | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB |  | Checked (NOTE 3) |  | pc_csi-RSRP-AndRSRQ-MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB |  | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas |  | Not checked |  |  |
| csi-RS-RLM |  | Not checked |  |  |
| handoverInterF |  | Checked (NOTE 14) |  | pc_ handoverInterF |
| handoverLTE-EPC |  | Not checked |  |  |
| handover-eLTE-5GC |  | Not checked |  |  |
| maxNumberResource-CSI-RS-RLM |  | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology |  | Not checked |  |  |
| nr-AutonomousGaps-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 |  | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 |  | Not checked |  |  |
| dummy |  | Not checked |  |  |
| cli-RSSI-Meas-r16 |  | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 |  | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 |  | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 |  | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 |  | Not checked |  |  |
| idleInactiveNR-MeasBeamReport-r16 |  | Not checked |  |  |
| increasedNumberofCSIRSPerMO-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UE-NR-Capability-v1550 |  |
| reducedCP-Latency |  | Checked |  | pc_reducedCP_Latency |
| nonCriticalExtension SEQUENCE { |  |  | UE-NR-Capability-v1560 |  |
| nrdc-Parameters |  | Not checked | NRDC-Parameters |  |
| receivedFilters |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UE-NR-Capability-v1570 |  |
| nrdc-Parameters-v1570 |  | Not checked | NRDC-Parameters-v1570 |  |
| nonCriticalExtension SEQUENCE { |  |  | UE-NR-Capability-v1610 |  |
| inDeviceCoexInd-r16 |  | Not checked |  |  |
| dl-DedicatedMessageSegmentation-r16 |  | Checked |  | pc_NR_dl_DedicatedMessageSegmentation |
| nrdc-Parameters-v1610 |  | Not checked |  |  |
| powSav-Parameters-r16 |  | Checked | PowSav-Parameters-r16 (Table 8.2.1.1.1.3.3-12) |  |
| fr1-Add-UE-NR-Capabilities-v1610 |  | Not checked |  |  |
| fr2-Add-UE-NR-Capabilities-v1610 |  | Not checked |  |  |
| bh-RLF-Indication-r16 |  | Not checked |  |  |
| directSN-AdditionFirstRRC-IAB-r16 |  | Not checked |  |  |
| bap-Parameters-r16 |  | Not checked |  |  |
| referenceTimeProvision-r16 |  | Not checked |  |  |
| sidelinkParameters-r16 |  | Not checked |  |  |
| highSpeedParameters-r16 |  | Not checked |  |  |
| mac-Parameters-v1610 |  | Not checked |  |  |
| mcgRLF-RecoveryViaSCG-r16 |  | Not checked |  |  |
| resumeWithStoredMCG-SCells-r16 |  | Not checked |  |  |
| resumeWithStoredSCG-r16 |  | Not checked |  |  |
| resumeWithSCG-Config-r16 |  | Not checked |  |  |
| ue-BasedPerfMeas-Parameters-r16 |  | Checked | UE-BasedPerfMeas-Parameters-r16 (Table 8.2.1.1.1.3.3-13) |  |
| son-Parameters-r16 { |  |  |  |  |
| rach-Report-r16 |  | Checked |  | pc_rachReport_r16 |
| } |  |  |  |  |
| onDemandSIB-Connected-r16 |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked | UE-NR-Capability-v1640 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |  |
| } |  |  |  |  |
| Note 1: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and intraAndInterF-MeasAndReport is supported in both modes, then support of intraAndInterF-MeasAndReport will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and intraAndInterF-MeasAndReport is only supported in one mode, then support of intraAndInterF-MeasAndReport will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate.) Note 2: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and eventA-MeasAndReport is supported in both modes, then support of eventA-MeasAndReport will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and eventA-MeasAndReport is only supported in one mode, then support of eventA-MeasAndReport will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 3: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is supported in both frequency ranges, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signaled in measAndMobParameters/MeasAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is only supported in one frequency range, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate.) Note 4: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and skipUplinkTxDynamic is supported in both modes, then support of skipUplinkTxDynamic will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and skipUplinkTxDynamic is only supported in one mode, then skipUplinkTxDynamic will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 5: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and logicalChannelSR-DelayTimer is supported in both modes, then support of logicalChannelSR-DelayTimer will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and logicalChannelSR-DelayTimer is only supported in one mode, then support of logicalChannelSR-DelayTimer will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 6: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and longDRX_Cycle is supported in both modes, then support of longDRX_Cycle will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and longDRX_Cycle is only supported in one mode, then support of longDRX_Cycle will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 7: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and shortDRX_Cycle is supported in both modes, then support of shortDRX_Cycle will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and shortDRX_Cycle is only supported in one mode, then support of shortDRX_Cycle will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 8: If the UE supports FR2 and pc_pdsch_256QAM_FR2 is supported, then support of pdsch_256QAM_FR2 will be signaled in at least one entry of supportedBandListNR as appropriate. Note 9: If pc_pusch_256QAM_FR1 or pc_pusch_256QAM_FR2 is supported, then support of pusch-256QAM will be signaled in at least one entry of supportedBandListNR as appropriate. Note 10: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and ss-SINR-Meas is supported in both frequency ranges, then support of ss-SINR-Meas will be signaled in measAndMobParameters/MeasAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and ss-SINR-Meas is only supported in one frequency range, then support of ss-SINR-Meas will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate.) Note 11: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and csi-RSRP-AndRSRQ-MeasWithoutSSB is supported in both frequency ranges, then support of csi-RSRP-AndRSRQ-MeasWithoutSSB will be signaled in measAndMobParameters/MeasAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and csi-RSRP-AndRSRQ-MeasWithoutSSB is only supported in one frequency range, then support of csi-RSRP-AndRSRQ-MeasWithoutSSB will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate.) Note 12: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and multipleConfiguredGrants is supported in both modes, then support of multipleConfiguredGrants will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and multipleConfiguredGrants is only supported in one mode, then support of multipleConfiguredGrants will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 13: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and sftd-MeasNR-Neigh is supported in both modes, then support of sftd-MeasNR-Neigh will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and sftd-MeasNR-Neigh is only supported in one mode, then support of sftd-MeasNR-Neigh will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate.) Note 14: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and handoverInterF is supported in both frequency ranges, then support of handoverInterF will be signaled in measAndMobParameters/MeasAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and handoverInterF is only supported in one frequency range, then support of handoverInterF will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate.) Note 15: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and handoverInterF is supported in both modes, then support of handoverInterF will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and handoverInterF is only supported in one mode, then support of handoverInterF will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 16: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and voiceOverNR is supported in both frequency ranges, then support of voiceOverNR will be signaled in ims-Parameters/ims-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and voiceOverNR is only supported in one frequency range, then support of voiceOverNR will be signaled in one of fr1-Add-UE-NR-Capabilities-v1540/ims-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities-v1540/ims-ParametersFRX-Diff as appropriate.) Note 17: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and dl-SchedulingOffset-PDSCH-TypeA is supported in both modes, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in phy-Parameters/phy-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and dl-SchedulingOffset-PDSCH-TypeA is only supported in one mode, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in one of fdd-Add-UE-NR-Capabilities/ phy-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/phy-ParametersXDD-Diff as appropriate.) Note 18: If the UE supports single frequency range (FR1 or FR2) the value of dl-SchedulingOffset-PDSCH-TypeA will be signaled in phy-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 and FR2) and dl-SchedulingOffset-PDSCH-TypeA is supported in both frequency ranges, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in fr1-fr2-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff and it will not be present in the common IE. If the UE supports both frequency ranges (FR1 + FR2) and dl-SchedulingOffset-PDSCH-TypeA is only supported in one frequency range, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in one of fr1-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff as appropriate. |  |  |  |  |

Table 8.2.1.1.1.3.3-7: UE-MRDC-Capability (Table 8.2.1.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.4-48 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-MRDC-Capability ::= SEQUENCE { |  |  |  |
| measAndMobParametersMRDC SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-Common SEQUENCE { |  |  |  |
| independentGapConfig | Checked |  | pc_independentGapConfig |
| } |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Checked (NOTE 5) |  | pc_SFTD_MeasPSCell_MRDC_FDD and/or pc_SFTD_MeasPSCell_MRDC_T DD |
| sftd-MeasNR-Cell | Checked (NOTE 6) |  | pc_SFTD_MeasNR_Cell_FDD and/or pc_SFTD_MeasNR_Cell_TDD |
| } |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| phy-ParametersMRDC-v1530 SEQUENCE { |  |  |  |
| naics-Capability-List SEQUENCE (SIZE (1..maxNrofNAICS-Entries)) OF NAICS-Capability-Entry { | Not checked |  |  |
| NAICS-Capability-Entry SEQUENCE { |  |  |  |
| numberOfNAICS-CapableCC | Not checked |  |  |
| numberOfAggregatedPRB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| rf-ParametersMRDC SEQUENCE { |  |  |  |
| supportedBandCombinationList SEQUENCE { (SIZE (1..maxBandComb)) OF BandCombination { |  |  |  |
| BandCombination SEQUENCE { |  |  |  |
| bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters { | Not checked |  |  |
| BandParameters CHOICE |  |  |  |
| eutra SEQUENCE { |  | EUTRA bands |  |
| bandEUTRA | Not checked | FreqBandIndicatorEUTRA |  |
| ca-BandwidthClassDL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| ca-BandwidthClassUL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| } |  |  |  |
| nr SEQUENCE { |  | NR bands |  |
| bandNR | Not checked | FreqBandIndicatorNR |  |
| ca-BandwidthClassDL-NR | Not checked | CA-BandwidthClassNR |  |
| ca-BandwidthClassUL-NR | Not checked | CA-BandwidthClassNR |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombination | Not checked | FeatureSetCombinationId |  |
| ca-ParametersEUTRA | Not checked | CA-ParametersEUTRA |  |
| ca-ParametersNR | Not checked | CA-ParametersNR |  |
| mrdc-Parameters SEQUENCE { |  |  |  |
| singleUL-Transmission | Not checked |  |  |
| dynamicPowerSharingENDC | Checked | Note 4 | pc_dynamicPowerSharing |
| tdm-Pattern | Not checked |  |  |
| ul-SharingEUTRA-NR | Not checked |  |  |
| ul-SwitchingTimeEUTRA-NR | Not checked |  |  |
| simultaneousRxTxInterBandENDC | Not checked |  |  |
| asyncIntraBandENDC | Not checked |  |  |
| dualPA-Architecture | Not checked |  |  |
| intraBandENDC-Support-v1540 | Not checked |  |  |
| ul-TimingAlignmentEUTRA-NR | Not checked |  |  |
| } |  |  |  |
| supportedBandwidthCombinationSet | Not checked |  |  |
| powerClass-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| appliedFreqBandListFilter | Not checked | FreqBandList |  |
| srs-SwitchingTimeRequested | Not checked |  |  |
| supportedBandCombinationList-v1540 SEQUENCE (SIZE (1..maxBandComb)) OF OF BandCombination-v1540 { |  | BandCombinationList-v1540 |  |
| BandCombination-v1540 SEQUENCE { |  |  |  |
| bandList-v1540 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF OF BandParameters-v1540 { | Not checked | BandParameters-v1540 |  |
| BandParameters-v1540 SEQUENCE { |  |  |  |
| srs-CarrierSwitch CHOICE { |  |  |  |
| nr SEQUENCE { |  | NR bands |  |
| srs-SwitchingTimesListNR SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeNR | Not checked |  |  |
| } |  |  |  |
| eutra SEQUENCE { |  | EUTRA bands |  |
| srs-SwitchingTimesListEUTRA SEQUENCE (SIZE (1..maxSimultaneousBands)) OF SRS-SwitchingTimeEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-TxSwitch-v1540 SEQUENCE { |  |  |  |
| supportedSRS-TxPortSwitch | Not checked |  |  |
| txSwitchImpactToRx | Not checked |  |  |
| txSwitchWithAnotherBand | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ca-ParametersNR-v1540 SEQUENCE { |  |  |  |
| simultaneousSRS-AssocCSI-RS-AllCC | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedbackPerBandComb SEQUENCE { |  |  |  |
| maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC | Not checked |  |  |
| totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC | Not checked |  |  |
| } |  |  |  |
| simultaneousCSI-ReportsAllCC | Not checked |  |  |
| dualPA-Architecture | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| generalParametersMRDC SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Checked (NOTE 1) |  | pc_splitSRB_WithOneUL_Path |
| splitDRB-withUL-Both-MCG-SCG | Checked (NOTE 2) |  | pc_splitDRB_withUL_Both_MCG_SCG |
| srb3 | Checked (NOTE 1) |  | pc_srb3 |
| v2x-EUTRA | Not checked |  |  |
| } |  |  |  |
| fdd-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Checked (NOTE 5) |  | pc_SFTD_MeasPSCell_MRDC_FDD and/or pc_SFTD_MeasPSCell_MRDC_T DD |
| sftd-MeasNR-Cell | Checked (NOTE 6) |  | pc_SFTD_MeasNR_Cell_FDD and/or pc_SFTD_MeasNR_Cell_TDD |
| } |  |  |  |
| generalParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Checked (NOTE 1) |  | pc_splitSRB_WithOneUL_Path |
| splitDRB-withUL-Both-MCG-SCG | Checked (NOTE 2) |  | pc_splitDRB_withUL_Both_MCG_SCG |
| srb3 | Checked (NOTE 3) |  | pc_srb3 |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| sftd-MeasPSCell | Checked (NOTE 5) |  | pc_SFTD_MeasPSCell_MRDC_FDD and/or pc_SFTD_MeasPSCell_MRDC_T DD |
| sftd-MeasNR-Cell | Checked (NOTE 6) |  | pc_SFTD_MeasNR_Cell_FDD and/or pc_SFTD_MeasNR_Cell_TDD |
| } |  |  |  |
| generalParametersMRDC-XDD-Diff SEQUENCE { |  |  |  |
| splitSRB-WithOneUL-Path | Checked (NOTE 1) |  | pc_splitSRB_WithOneUL_Path |
| splitDRB-withUL-Both-MCG-SCG | Checked (NOTE 2) |  | pc_splitDRB_withUL_Both_MCG_SCG |
| srb3 | Checked (NOTE 3) |  | pc_srb3 |
| v2x-EUTRA-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-MRDC-Capabilities SEQUENCE { |  |  |  |
| measAndMobParametersMRDC-FRX-Diff SEQUENCE { |  |  |  |
| simultaneousRxDataSSB-DiffNumerology | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombinations SEQUENCE (SIZE (1.. maxFeatureSetCombinations)) OF FeatureSetCombination { | Not checked | FeatureSetCombination |  |
| FeatureSetCombination SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand { |  |  |  |
| FeatureSetsPerBand SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet { |  |  |  |
| FeatureSet CHOICE { |  |  |  |
| eutra SEQUENCE { |  | EUTRA bands |  |
| downlinkSetEUTRA | Not checked |  |  |
| uplinkSetEUTRA | Not checked |  |  |
| } |  |  |  |
| nr SEQUENCE { |  | NR bands |  |
| downlinkSetNR | Not checked |  |  |
| uplinkSetNR | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pdcp-ParametersMRDC-v1530 SEQUENCE { |  |  |  |
| pdcp-DuplicationSplitSRB | Checked |  | pc_pdcp_DuplicationSplitSRB |
| pdcp-DuplicationSplitDRB | Checked |  | pc_pdcp_DuplicationSplitDRB |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| Note 1: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and splitSRB-WithOneUL-Path is supported in both modes, then support of splitSRB-WithOneUL-Path will be signaled in generalParametersMRDC/generalParametersMRDC-XDD-Diff. If the UE is dual mode (FDD + TDD) and splitSRB-WithOneUL-Path is only supported in one mode, then support of splitSRB-WithOneUL-Path will be signaled in one of fdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff as appropriate. Note 2: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and splitDRB-withUL-Both-MCG-SCG is supported in both modes, then support of splitDRB-withUL-Both-MCG-SCG will be signaled in generalParametersMRDC/generalParametersMRDC-XDD-Diff. If the UE is dual mode (FDD + TDD) and splitDRB-withUL-Both-MCG-SCG is only supported in one mode, then support of splitDRB-withUL-Both-MCG-SCG will be signaled in one of fdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff as appropriate. Note 3: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and srb3 is supported in both modes, then support of srb3 will be signaled in generalParametersMRDC/generalParametersMRDC-XDD-Diff. If the UE is dual mode (FDD + TDD) and srb3 is only supported in one mode, then support of srb3 will be signaled in one of fdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/generalParametersMRDC-XDD-Diff as appropriate. Note 4: If pc_dynamicPowerSharing is supported, then support of dynamicPowerSharingENDC will be signaled in at least one entry of supportedBandCombinationList as appropriate. As per 38.306 cl 4.2.7.9 this is applicable to FR1 only. Note 5: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and sftd-MeasPSCell is supported in both modes, then support of sftd-MeasPSCell will be signalled in measAndMobParametersMRDC/measAndMobParametersMRDC-XDD-Diff If the UE is dual mode (FDD + TDD) and sftd-MeasPSCell is only supported in one mode, then support of sftd-MeasPSCell will be signalled in one of fdd-Add-UE-MRDC-Capabilities/measAndMobParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/ measAndMobParametersMRDC-XDD-Diff as appropriate. Note 6: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and sftd-MeasNR-Cell is supported in both modes, then support of sftd-MeasNR-Cell will be signalled in measAndMobParametersMRDC/measAndMobParametersMRDC-XDD-Diff If the UE is dual mode (FDD + TDD) and sftd-MeasNR-Cell is only supported in one mode, then support of sftd-MeasNR-Cell will be signalled in one of fdd-Add-UE-MRDC-Capabilities/measAndMobParametersMRDC-XDD-Diff or tdd-Add-UE-MRDC-Capabilities/ measAndMobParametersMRDC-XDD-Diff as appropriate. |  |  |  |

Table 8.2.1.1.1.3.3-8: ULDedicatedMessageSegment (steps 2a1 and 4a1 Table 8.2.1.1.1.3.2-1)

| Derivation Path: 36.508 [7], Table 4.6.1-22A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| segmentNumber-r16 | 0 for first segment, and incremented by 1 for each subsequent segment |  |  |
| rrc-MessageSegmentContainer-r16 | Not Checked | OCTET STRING including segmented UECapabilityInformation message |  |
| rrc-MessageSegmentType-r16 | notLastSegment for all segments except the last segment. lastSegment for the last segment |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-9: FeatureSetDownlink-v1610 (Table 8.2.1.1.1.3.3-6)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetDownlink-v1610 ::= SEQUENCE { |  |  |  |
| cbgPDSCH-ProcessingType1-DifferentTB-PerSlot-r16 | Not checked |  |  |
| cbgPDSCH-ProcessingType2-DifferentTB-PerSlot-r16 | Not checked |  |  |
| intraFreqDAPS-r16 | Checked |  | pc_intraFreqDAPS_r16 |
| intraBandFreqSeparationDL-v1620 | Not checked |  |  |
| intraBandFreqSeparationDL-Only-r16 | Not checked |  |  |
| pdcch-Monitoring-r16 | Not checked |  |  |
| pdcch-MonitoringMixed-r16 | Not checked |  |  |
| crossCarrierSchedulingProcessing-DiffSCS-r16 | Not checked |  |  |
| singleDCI-SDM-scheme-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-10: FeatureSetUplink-v1610 (Table 8.2.1.1.1.3.3-6)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetUplink-v1610 ::= SEQUENCE { |  |  |  |
| pusch-RepetitionTypeB-r16 | Checked |  | pc_pusch_RepetitionTypeB_r16 |
| ul-CancellationSelfCarrier-r16 | Not checked |  |  |
| ul-CancellationCrossCarrier-r16 | Not checked |  |  |
| ul-FullPwrMode2-MaxSRS-ResInSet-r16 | Not checked |  |  |
| cbgPUSCH-ProcessingType1-DifferentTB-PerSlot-r16 | Not checked |  |  |
| cbgPUSCH-ProcessingType2-DifferentTB-PerSlot-r16 | Not checked |  |  |
| supportedSRS-PosResources-r16 | Not checked |  |  |
| intraFreqDAPS-UL-r16 | Not checked |  |  |
| intraBandFreqSeparationUL-v1620 | Not checked |  |  |
| multiPUCCH-r16 | Not checked |  |  |
| twoPUCCH-Type1-r16 | Not checked |  |  |
| twoPUCCH-Type2-r16 | Not checked |  |  |
| twoPUCCH-Type3-r16 | Not checked |  |  |
| twoPUCCH-Type4-r16 | Not checked |  |  |
| mux-SR-HARQ-ACK-r16 | Not checked |  |  |
| dummy1 | Not checked |  |  |
| dummy2 | Not checked |  |  |
| twoPUCCH-Type5-r16 | Not checked |  |  |
| twoPUCCH-Type6-r16 | Not checked |  |  |
| twoPUCCH-Type7-r16 | Not checked |  |  |
| twoPUCCH-Type8-r16 | Not checked |  |  |
| twoPUCCH-Type9-r16 | Not checked |  |  |
| twoPUCCH-Type10-r16 | Not checked |  |  |
| twoPUCCH-Type11-r16 | Not checked |  |  |
| ul-IntraUE-Mux-r16 | Not checked |  |  |
| ul-FullPwrMode-r16 | Not checked |  |  |
| crossCarrierSchedulingProcessing-DiffSCS-r16 | Not checked |  |  |
| ul-FullPwrMode1-r16 | Not checked |  |  |
| ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 | Not checked |  |  |
| ul-FullPwrMode2-TPMIGroup-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-11: CA-ParametersNR-v1610 (Table 8.2.1.1.1.3.3-6)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| CA-ParametersNR-v1610 ::= SEQUENCE { |  |  |  |
| parallelTxMsgA-SRS-PUCCH-PUSCH-r16 | Not checked |  |  |
| msgA-SUL-r16 | Not checked |  |  |
| jointSearchSpaceSwitchAcrossCells-r16 | Not checked |  |  |
| half-DuplexTDD-CA-SameSCS-r16 | Not checked |  |  |
| scellDormancyWithinActiveTime-r16 | Not checked |  |  |
| scellDormancyOutsideActiveTime-r16 | Checked |  | pc_scellDormancyOutsideActiveTime_r16 |
| crossCarrierA-CSI-trigDiffSCS-r16 | Not checked |  |  |
| defaultQCL-CrossCarrierA-CSI-Trig-r16 | Not checked |  |  |
| interCA-NonAlignedFrame-r16 | Not checked |  |  |
| simul-SRS-Trans-BC-r16 | Not checked |  |  |
| interFreqDAPS-r16 | Checked |  | pc_interFreqDAPS_r16 |
| codebookParametersPerBC-r16 | Not checked |  |  |
| blindDetectFactor-r16 | Not checked |  |  |
| pdcch-MonitoringCA-r16 | Not checked |  |  |
| pdcch-BlindDetectionCA-Mixed-r16 | Not checked |  |  |
| pdcch-BlindDetectionMCG-UE-r16 | Not checked |  |  |
| pdcch-BlindDetectionSCG-UE-r16 | Not checked |  |  |
| pdcch-BlindDetectionMCG-UE-Mixed-r16 | Not checked |  |  |
| pdcch-BlindDetectionSCG-UE-Mixed-r16 | Not checked |  |  |
| crossCarrierSchedulingDL-DiffSCS-r16 | Not checked |  |  |
| crossCarrierSchedulingDefaultQCL-r16 | Not checked |  |  |
| crossCarrierSchedulingUL-DiffSCS-r16 | Not checked |  |  |
| simul-SRS-MIMO-Trans-BC-r16 | Not checked |  |  |
| codebookParametersAdditionPerBC-r16 | Not checked |  |  |
| codebookComboParametersAdditionPerBC-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-12: PowSav-Parameters-r16 (Table 8.2.1.1.1.3.3-6)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| PowSav-Parameters-r16 ::= SEQUENCE { |  |  |  |
| powSav-ParametersCommon-r16 SEQUENCE { |  |  |  |
| drx-Preference-r16 | Not checked |  |  |
| maxCC-Preference-r16 | Not checked |  |  |
| releasePreference-r16 | Checked |  | pc_releasePreference_r16 |
| minSchedulingOffsetPreference-r16 | Not checked |  |  |
| } |  |  |  |
| powSav-ParametersFRX-Diff-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.2.1.1.1.3.3-13: UE-BasedPerfMeas-Parameters-r16 (Table 8.2.1.1.1.3.3-6)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UE-BasedPerfMeas-Parameters-r16 ::= SEQUENCE { |  |  |  |
| barometerMeasReport-r16 | Checked |  | pc_barometer_r16 |
| immMeasBT-r16 | Not checked |  |  |
| immMeasWLAN-r16 | Checked |  | pc_immMeasWLAN_r16 |
| loggedMeasBT-r16 | Not checked |  |  |
| loggedMeasurements-r16 | Checked |  | pc_loggedMeasurements_r16 |
| loggedMeasWLAN-r16 | Checked |  | pc_loggedMeasWLAN_r16 |
| orientationMeasReport-r16 | Checked |  | pc_orientation_r16 |
| speedMeasReport-r16 | Checked |  | pc_speed_r16 |
| gnss-Location-r16 | Checked |  | pc_GNSS_location_r16 |
| ulPDCP-Delay-r16 | Checked |  | pc_PDCP_Delay_r16 |
| } |  |  |  |