3GPP TSG RAN Meeting #98-e									RP-222836

Electronic Meeting, December 12-16, 2022

## Status Report to TSG

Agenda item:			9.3.1.4

| WI / SI Name | NR MIMO evolution for downlink and uplink |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_evo_DL_UL |  |  |  |  |
| Unique ID | 940096 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213598 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 12/2023 | Performance part: 06/2024 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 40% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur (primary) | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |
| Rapporteur | Name | Yubo Yang |
|  | Company | Huawei |
|  | Email | yangyubo1@huawei.com |
| Rapporteur | Name | Riki Okawa |
|  | Company | NTT DOCOMO |
|  | Email | riki.ookawa.rp@nttdocomo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

In RAN1#110bis-e, the following agreements were made.

Multi-TRP enhancement

Conclusion

On unified TCI framework extension in Rel-18, there is no consensus to support simultaneous configuration of both joint and separate DL/UL TCI modes in a serving cell

Conclusion

On unified TCI framework extension in Rel-18, there is no consensus to support separate RRC-configured TCI state list(s) for each of TRPs

Agreement

On unified TCI framework extension for M-DCI based MTRP:

- The existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) associated with one coresetPoolIndex value can indicate the joint/DL/UL TCI state(s) specific to the same coresetPoolIndex value

  - FFS: The UE shall apply the indicated joint/DL/UL TCI state(s) specific to a coresetPoolIndex value to channel(s)/signal(s) that have explicit or implicit association with the same coresetPoolIndex value

- A coresetPoolIndex value field is included in TCI state activation command (MAC-CE) to indicate that the mapping between the activated TCI state(s) and the TCI codepoint(s) is specific to which coresetPoolIndex value

Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following:

- Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported)

Agreement

On unified TCI framework extension for M-DCI based MTRP:

- For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

- For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

Agreement

On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2:

- Alt1: Use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

- Alt2: PUSCH transmission scheduled/activated by the DCI format 0_1/0_2 follows the spatial domain transmission filter(s) used for the SRS resource(s) indicated by the DCI format 0_1/0_2

  - FFS : PL-RS(s), and UL PC parameter setting(s) (including P0, alpha, and closed loop index) for the PUSCH

Agreement

On unified TCI framework extension for S-DCI based MTRP, down-select one alternative from the followings in RAN1#111 for PUCCH transmission:

- Alt1: Use RRC configuration to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/ group

- Alt2: Use RRC configuration to inform the association between a CORESET group and a PUCCH resource/group, and the indicated joint/UL TCI state(s) associated with the CORESET group applies to the PUCCH resource/group associated with the same CORESET group

- Alt3: Use MAC-CE to inform the association between the indicated joint/UL TCI state(s) and a PUCCH resource/group

- Note: the association indicates whether the UE shall apply the first one, the second one, or both of the joint/UL TCI states indicated by DCI/MAC-CE to a PUCCH resource/group

Agreement

On unified TCI framework extension, up to 2 joint TCI states can be indicated by MAC-CE/DCI and applied to CJT-based PDSCH reception (PDSCH-CJT) in a BWP/CC configured with joint DL/UL TCI mode

- Support of 1 or 2 indicated joint TCI states for PDSCH-CJT is up to UE capability

- FFS: QCL type(s)/assumption(s) of the indicated joint TCI state(s) applied to PDSCH-CJT

- Note: On how to inform UE to apply which indicated joint TCI state(s) to target channel(s)/signal(s) in the BWP/CC, it is discussed individually in AI 9.1.1.1

Agreement

On unified TCI framework extension for M-DCI based MTRP:

- The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET that is associated with the same coresetPoolIndex value

- The UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDSCH scheduled/activated by PDCCH on a CORESET that is associated with the same coresetPoolIndex value

- FFS: Other channel(s)/signal(s) that has explicit or implicit association with a coresetPoolIndex value

- FFS: Other channel(s)/signal(s) that doesn’t have association with a coresetPoolIndex value

Above are applicable to the CORESET(s) that is configured/allowed to follow the indicated joint/DL TCI state

FFS: The configuration/rule to configure/allow CORESET(s) to follow the indicated joint/DL TCI state, including the option to reuse the same configuration/rule as in Rel-17 unified TCI framework

Agreement

On unified TCI framework extension, study the following enhancements for TRP-specific BFR:

- Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP

- Enhancement to beam update after NW response to TRP-specific BFR request

Agreement

On unified TCI framework extension for S-DCI based MTRP , down-select one alternative from the followings in RAN1#111:

- Alt1: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) for one of the two TRPs or both TRPs in a CC/BWP or a set of CCs/BWPs in a CC list

- Alt2: In one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL /UL TCI state(s) only specific to one of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list

  - Note: According to the agreement in RAN1#109-e, support of one additional TCI field or a field associating the TCI field to the TRP(s) is not precluded

Note: It has been agreed to use the existing TCI field for TCI state indication for S-DCI based MTRP in RAN1#109e

Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

FFS: The behavior if the UE receives a beam indication DCI that indicates joint/DL/UL TCI state(s) for one TRP

Agreement

For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG

- baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length

- as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length

  - FFS: the maximum Rx timing difference (could be up to RAN4)

  - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

Agreement

Multi-DCI multi-TRP operation with two TAs is supported for Rel-15/16/17 TCI frameworks and unified TCI framework extension discussed in 9.1.1.1 as well as UL beam indication via spatial relation.

Agreement

For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support one of the alternatives (down selection to be done in RAN1#111):

- Alt 1: PDCCH scheduling RAR will always be received from serving cell => there is no need for additional type 1 CSS configuration per additional PCI

- Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported  additional type 1 CSS configuration per additional PCI needs to be supported

Agreement

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support PRACH configuration associated with additional configured PCIs different from the PCI of the serving cell.

- FFS: details

Agreement

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support a mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order

- FFS:  Explicit indication or implicit indication through PDCCH order

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support one of the following alternatives in RAN1#111:

- Alt 1: PDCCH order sent by one TRP triggers RACH procedure towards the same TRP

  - note: with Alt 1, PDCCH order sent by one TRP triggering RACH procedure towards another TRP is not allowed

- Alt 2: PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP

  - This does not preclude PDCCH order triggering two RACH procedures for two TRPs

Agreement

For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the four options agreed in RAN1#110bis-e are refined as below (down-selection of one or a combination of the options to be performed in RAN1#111):

- Option 1: Associate TAG to TCI-state/spatial relation

  - Configure TAG ID as part of UL/joint TCI state or spatial relation

  - for UL transmission, the TAG ID associated with the UL/joint TCI state or spatial relation is utilized

- Option 2: Associate TAG to CORESETPoolIndex

  - for dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission

  - for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured.

  - FFS:   Other signals/channels:  AP-SRS, and dynamic HARQ-ACK

- Option 3: Associate TAG to SSB group (if such an association is agreed in agenda 9.1.1.2). For a UL transmission, UE adopts the TAG associated with the SSB group such that

  - if the PL RS is an SSB, then the UE adopts the TAG associated with the SSB group which the PL RS of the UL transmission belongs to

  - if the PL RS is a CSI-RS, then the UE adopts the TAG associated with the SSB group which the QCL source SSB of the PL RS belongs to

- Option 4:  TAG association performed as follows:

  - for dynamically scheduled/activated channels/signals, TAG associated with the CORESET pool index of the CORESET carrying the scheduling PDCCH is utilized for UL transmission

  - for P/SP UL channels / signals (not scheduled or activated by DCI), TAG ID is RRC-configured.

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command:

- FFS: whether the indication is implicit or explicit

- Detailed indication schemes are FFS

- This does not preclude indication of two TAG IDs (if supported)

- Note: This applies at least to MSGB in case of C-RNTI

Conclusion 1

For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs.

- Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region

Agreement

For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111):

- Alt 1:  indicate TAG ID as part of TA command in RAR

- Alt 2:  indicate TAG ID as part of PDCCH order

- Alt 3:  divide SSBs into two groups, one for each TRP.    If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

- Alt 4:  divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

- Alt 5:  divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP

- Alt 6:   TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order

- Alt 7:  Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order

Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported)

CSI enhancement

Agreement

On the Type-II codebook refinement for CJT mTRP, following legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II), for a given CSI-RS resource:

- SD basis selection is layer-common and polarization-common, with N1, N2, O1, O2 defined per Rel-16 specification for refinement based on Rel-16 regular eType-II, and per Rel-17 specification for refinement based on Rel-17 PS FeType-II

- FD basis selection is

  - For refinement based on Rel-16 regular eType-II: per-layer with Mv, pv, N3, and R defined per Rel-16 specification

  - For refinement based on Rel-17 PS FeType-II: layer-common with M, N3, and R defined per Rel-17 specification

  - FFS: Details on FD basis selection window

Note: The supported value(s) for each of the defined parameters are to be discussed separately (e.g. possibilities of adding new or removing existing value(s) in addition to those supported by legacy specification).

Agreement

On the SD basis selection for Type-II codebook refinement for CJT mTRP, following legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II), SD basis selection is per CSI-RS-resource.

- Down select from the following alternatives (RAN1#110bis-e) on the L parameter:

  - Alt1. Per-CSI-RS-resource Ln parameter

    - TBD: Whether {Ln, n=1, ..., N} are higher-layer configured by gNB, or the total n=1NLn is higher-layer configured by gNB while {Ln, n=1, ..., N} are reported by the UE

  - Alt2. gNB configures a common L parameter for all N CSI-RS resources via higher-layer signaling

FFS: Study on additional optimization for collocated multi-panel scenario

Agreement

On the Type-II codebook refinement for CJT mTRP, following legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II), regarding the location of non-zero coefficients (NZCs) indicated by bitmap (following legacy mechanism), for each layer, support separate bitmap per each CSI-RS resource

- Total size = n=1NBn where Bn is the bitmap size for CSI-RS resource n

  - TBD: Whether Bn=2LnMv,n (Mv,n=Mv for mode 2) analogous to legacy, or further reduction of bitmap size is supported.

  - FFS: Depending on the outcome of other issues, whether Mv,n=Mv or  Ln=L

- FFS: Per-CSI-RS-resource NNZC (number of NZCs) constraint vs. joint NNZC constraint across N CSI-RS-resources

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, the constraint on the maximum number of non-zero coefficients (NZCs) per-layer (K0) is defined jointly across all N CSI-RS resources

- TBD: the constraint on the total number of NZCs across all layers

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP,

- Only CSI reporting over PUSCH is supported

  - FFS: Whether AP only, or both AP and SP (following legacy), is supported

- An associated Resource Setting includes a CMR comprising K≥1 NZP CSI-RS resources from one CSI-RS resource set

  - Periodic, semi-persistent, and aperiodic NZP CSI-RS are supported

  - The supported CSI-RS resource parameter settings follow the legacy specification (without additional enhancement)

  - FFS: Whether or not the K NZP CSI-RS resources are constrained to be in the same slot

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, for each layer:

- One (common) SCI applies across all N CSI-RS resources

- Further down-select one from the following alternatives by RAN1#110bis-e:

  - Alt1. One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2)

    - FFS: Amplitude quantization table considering transmission power difference between multiple TRPs

    - For each of the amplitude groups (other than the group associated with the SCI), the reference amplitude is reported

  - Alt3. One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N)

    - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported

FFS: The need for “strongest” TRP/TRP-group indicator in addition to the SCI

Agreement

For the Rel-18 Type-II codebook for CJT mTRP, the switching between mode-1 and mode-2 is gNB-initiated via RRC signalling

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, support RI={1,2,3,4}.

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, support UE “predicting” channel/CSI after slot l where the location of slot l is configured (from multiple candidate values) by gNB via higher-layer signalling

- Candidates of slot l location include the legacy CSI reference resource location (n – nCSI,ref ) and slot (n+δ) where δ ≥ 0

- FFS: Possible value(s) of δ and possible value(s) of WCSI

Note: Per legacy behavior, the legacy CSI reference resource, i.e., (n – nCSI,ref ), is reused for locating the last CSI-RS occasion used for a CSI report

For a UE that supports UE-side prediction, the support of l = (n – nCSI,ref ) is UE optional.

Agreement

For the Type-II codebook refinement for high/medium velocities, down-select from the following alternatives:

- Alt1. Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimensional bitmap corresponds to qth selected DD basis vector

  - The number of selected DD basis vectors is denoted as Q

  - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors.

- Alt2. A DD-basis-common per-layer 2-dimensional bitmap for indicating the location of NZCs used in Rel-16/17 Type-II is used

  - This implies that for each layer, the location of NZCs in SD-FD is common across all the Q selected DD basis vectors

FFS: Further overhead reduction on bitmap(s)

FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, support the following codebook structure where N4 is gNB-configured via higher-layer signaling:

- For N4=1, Doppler-domain basis is the identity (no Doppler-domain compression) reusing the legacy W1, W2, and Wf, e.g. W1W2(Wf)H

- For N4>1, Doppler-domain orthogonal DFT basis commonly selected for all SD/FD bases reusing the legacy W1 and Wf, e.g. W1W2Wf⨂WdH

  - Only Q (denoting the number of selected DD basis vectors) >1 is allowed

  - TBD (by RAN1#110bis): whether rotation is used or not

  - FFS: identical or different rotation factors for different SD components

  - FFS: Whether Q is RRC-configured or reported by the UE

Note: Detailed designs for SD/FD bases including the associated UCI parameters follow the legacy specification

FFS: Whether one CSI reporting instance includes multiple W2 and a single W1 and Wf report.

Agreement

On the SD basis selection for Type-II codebook refinement for CJT mTRP, support the following on the L parameter:

- Per-CSI-RS-resource Ln parameter

  - TBD: Whether {Ln, n=1, ..., N} are higher-layer configured by gNB, or the total n=1NLn is higher-layer configured by gNB while {Ln, n=1, ..., N} are reported by the UE, one L configured and {Ln} determined from configured L

  - FFS: The value of Ln is taken from a pre-defined set

Conclusion

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group and Strongest Coefficient Indicator (SCI) design, there is no consensus on supporting “strongest” CSI-RS resource indicator in addition to the agreed SCI.

- Note: This doesn’t preclude any (future) proposal on reference CSI-RS resource(s) for other purpose(s)

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, following legacy, support both aperiodic and semi-persistent CSI reporting on PUSCH.

Conclusion

On the usage of CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, there is no consensus in supporting any specification enhancement for the following assumptions:

- Legacy UE procedure for CSI measurement/calculation (equivalent to the combination of l = (n – nCSI,ref ) and WCSI=1)

- gNB-side prediction

  - Note: This doesn’t preclude any gNB implementation

Agreement

For the Type-II codebook refinement for high/medium velocities, only CSI reporting over PUSCH is supported

- Following legacy, support both aperiodic and semi-persistent CSI reporting on PUSCH.

Agreement

For the Type-II codebook refinement for high/medium velocities, the selection of DD basis vectors is layer-specific

- The number of selected DD basis vector (denoted as Q) is layer-common

Agreement

For the Rel-18 TRS-based TDCP reporting, down select one of the following alternatives by RAN1#110bis-e:

- AltA. Based on Doppler profile

  - E.g., Doppler spread derived from the 2nd moment of Doppler power spectrum, average Doppler shifts, Doppler shift per resource, maximum Doppler shift, relative Doppler shift, etc

- AltB. Based on quantized amplitude of time-domain correlation profile

  - E.g. Correlation within one TRS resource, correlation across multiple TRS resources

  - Note: The correlation over one or more lags of TRS resource may be considered.  The lags may be within one TRS burst or different TRS bursts

Note: Different alternatives may or may not apply to different use cases

FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high

FFS: TDCP parameter(s) signaled with respect to each alternative

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR:

- Time-domain behaviour for NZP CSI-RS resource: periodic (P), semi-persistent (SP), aperiodic (AP)

  - FFS: Whether to introduce constraints on allowed configuration

- Down select from the following:

  - Alt1. Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) -CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s):

  - Alt2. Support one NZP CSI-RS resource in a CSI-RS resource set, where K>1 occasions are received via a single triggering instance, for aperiodic (AP)-CSI-RS-based channel measurement where the separation between 2 consecutive AP-CSI-RS resources is m slot(s).

  - For any of the alternatives:

    - No CRI is reported

    - FFS: Details, e.g., supported value(s) of K, m, other use cases for the AP-CSI-RS resources (e.g., for training filter coefficients, prediction or performance monitoring)

- Support only one NZP CSI-RS resource for P or SP-CSI-RS-based channel measurement

Agreement

On the Type-II codebook refinement for CJT mTRP, the selection of N CSI-RS resources is performed by UE and reported as a part of CSI report where N∈{1,..., NTRP}

- N is the number of cooperating CSI-RS resources, while NTRP is the maximum number of cooperating CSI-RS resources configured by gNB via higher-layer signaling

- The selection of N out of NTRP CSI-RS resources is reported via NTRP-bit bitmap in CSI part 1

  - Note: The value of N is inferred from the selection

- A restricted configuration (gNB-configured via higher-layer signaling) where N=NTRP is supported

  - NTRP-bit bitmap is not reported when the restriction is configured

  - FFS: Whether other RRC-configured TRP selection restriction including configuring the value of N is supported

- This feature is UE optional

Note: This agreement does not impact the decision on Ln being configured by gNB or selected by UE

Note: per WID and previous agreement, the candidate values for NTRP of are 1, 2, 3, and 4.

Note: only one transmission hypothesis is reported. UE is not mandated to calculate CSI for multiple transmission hypotheses.

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, when N4>1, if multiple candidates of Q value are supported, the value of Q is gNB-configured via higher-layer (RRC) signalling

Conclusion

For the Rel-18 TRS-based TDCP reporting, there is no consensus in supporting periodic, semi-persistent, and event-triggered/UE-initiated TDCP reporting.

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer:

- Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2)

  - FFS: Amplitude quantization table enhancement

  - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

- Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111)

  - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N)

    - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported

- If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP, also support a constraint on the total number of non-zero coefficients (NZCs) summed across all layers:

- Following the legacy specification, the maximum total number is 2K0

Agreement

On the SD basis selection for Type-II codebook refinement for CJT mTRP, on the L parameter, down select from the following alternatives (by RAN1#111):

- Alt1. Each of the {Ln, n=1, ..., N} is gNB-configured via higher-layer (RRC) signaling

  - FFS: The candidate values for Ln, e.g. follow the legacy specification

- Alt2. Ltot=n=1NLn where Ltot is gNB-configured via higher-layer (RRC) signaling and the relative value(s) of {Ln, n=1, ..., N} are reported by the UE

  - TBD: Whether for a given configured value of Ltot, the possible combinations of {Ln, n=1, ..., N} are fixed/pre-determined or gNB-configured via higher-layer (RRC) signaling

  - TBD: Whether the value(s) of {Ln, n=1, ..., N} are reported implicitly or explicitly, and whether some value(s) don’t need to be reported

  - FFS: The candidate values for Ln

- Alt3. An L parameter is gNB-configured via higher-layer (RRC) signaling and {Ln, n=1, ..., N} are determined from the value of L

  - TBD: How to determine {Ln, n=1, ..., N} from L, e.g. L1=L and other Ln = L/2

  - FFS: The candidate values for L

- Alt4. Lmax is gNB-configured via higher-layer (RRC) signaling and the relative value(s) of {Ln, n=1, ..., N} are reported by the UE

  - The relative value(s) of {Ln, n=1, ..., N} are reported by the UE, such that n=1NLn≤Lmax

  - TBD: Whether the value(s) of {Ln, n=1, ..., N} are reported implicitly or explicitly, and whether some value(s) don’t need to be reported

  - FFS: The candidate values for Ln

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, support the following CSI-RS resource types/structures for CMR, support the following:

- (Alt1) Support K>1 NZP CSI-RS resources, received via a single triggering instance, for aperiodic (AP) CSI-RS-based channel measurement in a same CSI-RS resource set where the separation between 2 consecutive AP-CSI-RS resources is m slot(s)

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameters, for a given CSI-RS resource, the supported value(s) of the following parameters follow the legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II) specification:

- N1, N2, N3, O1, O2

- M (only for design based on Rel-17 PS FeType-II)

For the following parameters, decide in RAN1#111 whether the supported value(s) follow the legacy (Rel-16 regular eType-II and Rel-17 PS FeType-II) specification or further refinement is needed:

- R: including, e.g. supporting only R=1, or supporting larger R values

- Mv/pv (Rel-16 regular eType-II): including, e.g. supporting smaller pv values such as {1/8, 1/4, 1/2} for v=1,2 and/or removing larger legacy value(s)

- : including, e.g. supporting smaller values such as {1/16, 1/8, 3/8}

Note: The outcome of Parameter Combination discussion will further restrict the supported combinations of parameter value(s)

FFS: For N>1, whether the maximum 2N1N2 (identical to the number of CSI-RS ports used for CMR) is limited to 32 just as in legacy specification

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the bitmap(s) for indicating the locations of NZCs, down-select from the following alternatives for the size of the bitmap for CSI-RS resource n (Bn) (by RAN1#111):

- Alt1. Analogous to legacy, Bn=2LnMv,n (Mv,n=Mv for mode 2)

- Alt2. Non-rectangular bitmap, i.e., NZC bitmap allowing different lengths for different SD/FD basis vectors.

  - TBD: How to determine the lengths for different SD/FD basis vectors

Agreement

For the Rel-18 Type-II codebook for CJT mTRP, for mode-1, the number of FD basis vectors (Mv related to pv for Rel-16, M for Rel-17) is common across all N CSI-RS resources

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, when N4>1, down-select from the following alternatives (by RAN1#111) for the orthogonal DFT DD basis:

- Alt1. No rotation factor

- Alt2. A rotation factor is selected for each SD basis vector

  - FFS: Supported values of rotation factor

Note: At least two companies opine that Alt2 is not aligned either with the agreement in RAN1#110bis-e or WID objective #1

Agreement

For the Type-II codebook refinement for high/medium velocities, for N4>1, study the supported values for Q from (but not limited to) the following candidates, in conjunction with the supported values of N4 and DD units:

- Alt1. Q is determined as a function of N4, e.g., Q=2 for N4=2, and Q=ceil(N4/2) for N4>2

- Alt2. Q is selected from multiple candidate values, e.g., {2, 3, 4, …,} (or a subset thereof, e.g. {2, 3}), the maximum value is FFS

- Alt3. Only single value is supported, e.g. Q=2 only or Q=4 only

Agreement

On the CSI reporting and measurement for the Rel-18 Type-II codebook refinement for high/medium velocities, when UE-side prediction is assumed, study the supported value(s) for δ and WCSI from (but not limited to) the following candidates, in conjunction with the supported values of N4 and DD units:

- δ (slots): {0, 1, 2, 3, 4, 6, 8}, or a subset thereof with at least two values including 0, or a single fixed value (e.g. 0 or 1)

- WCSI (slots): 1, N4, following periodicity of P/SP-CSI-RS or SP-CSI (e.g., 4, 5, 8, 10, 16, 20, 40), dN4 (d=DD unit size in slots, N4 is unit-less)

FFS: Dependence on sub-carrier spacing should also be studied

Conclusion

For the Rel-18 TRS-based TDCP reporting, the description in the 2nd and 3rd columns of Table 1 in R1-2210523 (“what to report” and “how to calculate”, respectively) will be used as a reference for further evaluation and down selection in RAN1#111, with the following edit (underlined and yellow highlighted):

- Scheme B column 2: “Amplitude At,τ vs. delay value τ, e.g. Non-zero quantized version of amplitude At,τ for a number of delay values t (quantized amplitude vs delay) ….”

Reference signal enhancement

Conclusion

- For discussion purpose, definition of Rel.15 DMRS ports and Rel-18 DMRS ports are:

  - Rel.15 Type 1/Type 2 DMRS ports: DMRS ports with FD-OCC length =2.

  - Rel.18 eType 1/eType 2 DMRS ports: DMRS ports with FD-OCC length >2.

- Following figure as an example shows difference between Rel.15 Type 1 DMRS ports and Rel.18 eType 1 DMRS ports.

Agreement

For more than 4 layers SU-MIMO PUSCH, support

- Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports.

  - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports.

    - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed.

  - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.

Agreement

For enhanced FD-OCC length for DMRS of PDSCH/PUSCH for Rel.18 eType 1 DMRS, support

- Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group

Agreement

Confirm the working assumption in RAN1#110 with the following update:

To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)).

- FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2.

- FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options).

Agreement

For FD-OCC length 4 for DMRS of PDSCH/PUSCH for Rel.18 eType 1/eType 2 DMRS, support one from the following FD-OCCs (to be selected in RAN1#111):

- Opt.1-1: Walsh matrix (Hadamard code):

| FD-OCC index | wf(0) | wf(1) | wf(2) | wf(3) |
| --- | --- | --- | --- | --- |
| 0 | +1 | +1 | +1 | +1 |
| 1 | +1 | -1 | +1 | -1 |
| 2 | +1 | +1 | -1 | -1 |
| 3 | +1 | -1 | -1 | +1 |

- Opt.1-2: Cyclic shift with {0, π, π/2, 3π/2}:

| FD-OCC index | wf(0) | wf(1) | wf(2) | wf(3) |
| --- | --- | --- | --- | --- |
| 0 | +1 | +1 | +1 | +1 |
| 1 | +1 | -1 | +1 | -1 |
| 2 | +1 | +j | -1 | -j |
| 3 | +1 | -j | -1 | +j |

Agreement

For Rel.18 eType 1/eType 2 DMRS ports of PDSCH/PUSCH with FD-OCC length 4, association between DMRS port indexes, CDM group index, FD-OCC index, and TD-OCC index (across consecutive DMRS symbols, if any) are determined by the following Table 1 and Table 2.

- The p in Table 1 and Table 2 corresponds to DMRS port index for PUSCH.

- DMRS port index for PDSCH is determined by p +1000 in Table 1 and Table 2.

Table 1. Rel.18 eType 1 DMRS ports for PUSCH

| p | CDM group index | FD-OCC index | TD-OCC index |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 2 | 1 | 0 | 0 |
| 3 | 1 | 1 | 0 |
| 4 | 0 | 0 | 1 |
| 5 | 0 | 1 | 1 |
| 6 | 1 | 0 | 1 |
| 7 | 1 | 1 | 1 |
| 8 | 0 | 2 | 0 |
| 9 | 0 | 3 | 0 |
| 10 | 1 | 2 | 0 |
| 11 | 1 | 3 | 0 |
| 12 | 0 | 2 | 1 |
| 13 | 0 | 3 | 1 |
| 14 | 1 | 2 | 1 |
| 15 | 1 | 3 | 1 |

Table 2. Rel.18 eType 2 DMRS ports for PUSCH

| p | CDM group index | FD-OCC index | TD-OCC index |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 2 | 1 | 0 | 0 |
| 3 | 1 | 1 | 0 |
| 4 | 2 | 0 | 0 |
| 5 | 2 | 1 | 0 |
| 6 | 0 | 0 | 1 |
| 7 | 0 | 1 | 1 |
| 8 | 1 | 0 | 1 |
| 9 | 1 | 1 | 1 |
| 10 | 2 | 0 | 1 |
| 11 | 2 | 1 | 1 |
| 12 | 0 | 2 | 0 |
| 13 | 0 | 3 | 0 |
| 14 | 1 | 2 | 0 |
| 15 | 1 | 3 | 0 |
| 16 | 2 | 2 | 0 |
| 17 | 2 | 3 | 0 |
| 18 | 0 | 2 | 1 |
| 19 | 0 | 3 | 1 |
| 20 | 1 | 2 | 1 |
| 21 | 1 | 3 | 1 |
| 22 | 2 | 2 | 1 |
| 23 | 2 | 3 | 1 |

Agreement

For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following:

- Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS.

  - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS.

- The scheduling restriction above means satisfying all of the following at least for other than M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme.

  - 1) The number of consecutively scheduled PRBs for PDSCH is even.

  - 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even.

  - 3) FFS: Restriction on scheduling of different UEs in case of MU-MIMO.

  - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme.

- Note1: Up to UE how to implement DMRS channel estimation.

- Note2: No further RAN1 specification enhancement is introduced to handle the orphan REs (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) for UE that is scheduled PDSCH without the scheduling restriction.

- Note 3: Other scheduling restrictions, if identified in future meetings, are not precluded.

Agreement

For FD-OCC length 4 in Rel.18 eType 1 DMRS for PUSCH,

- No spec. enhancement is needed to handle orphan RE issue (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs), because gNB (receiver) can decide whether the scheduling restriction is needed or not.

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Agreement

For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following:

- The hopping pattern (e.g., the pseudo-random sequence, time-domain granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset / cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if CS hopping and/or comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

Other details are not excluded

Agreement

For SRS TD OCC for SRS enhancements for TDD CJT, study:

- Comparison against SRS on 1 OFDM symbol

- Comparison against SRS repeated on multiple OFDM symbols

- Study the following aspects: evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

Conclusion

The discussion of resource mapping for SRS transmission based on network-provided parameters or system parameters is merged into the discussions of other SRS enhancements for TDD CJT.

Agreement

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2:

  - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

Agreement

For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

Agreement

For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- n can be 8

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

Conclusion

- No further discussion of increasing the maximum number of cyclic shifts for CJT SRS.

- No further discussion of partial frequency sounding extensions for CJT SRS.

Enhanced uplink transmission

Agreement

- Reuse the DCI field ‘Antenna Ports’ in DCI format 0_1 and 0_2 to indicate DMRS ports for SDM scheme of single-DCI based STxMP PUSCH:

  - The total numbers of layers, L, indicated by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH is used to determine the DMRS port indication table.

  - L1 of the indicated DMRS ports are associated with the L1 PUSCH layers which are indicated by the first TPMI field for CB PUSCH or the first SRI field for NCB PUSCH and the rest L- L1 of the indicated DMRS ports are associated with the L-L1 PUSCH layers which are indicated by the second TPMI field for CB PUSCH or the second SRI field for the NCB PUSCH.

    - FFS: how to partition the indicated DMRS ports among the PUSCH layers.

- Down-select one from the following two Alts for SDM scheme in RAN1#111:

  - Alt1: the DMRS ports associated with two TPMI/SRI fields must be in different CDM groups.

  - Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups.

Agreement

Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18.

- Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP.

- The total number of layers of these two PUSCHs is up to 4.

- FFS: whether the number of layers of each of these two PUSCHs is up to 2.

Agreement

For SDM scheme of single-DCI based STxMP PUSCH

- Configure two SRS resource sets for CB or NCB .

  - FFS : These two SRS resource sets can have different number of SRS resources for codebook -based or non-codebook based.

- For codebook -based PUSCH , DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set.

- For non-codebook based PUSCH and codebook -based PUSCH , DCI indicates two SRI fields and each field indicates SRS resource(s)  for each SRS resource set separately.

  - FFS : For codebook -based PUSCH , the two SRS resources indicated by the two SRI fields can have different number of SRS ports

Agreement

Regarding the TPMI/SRI indication for multi-DCI based STxMP PUSCH+PUSCH:

- Configure two SRS resource sets for CB or NCB.

  - FFS: Whether/how to associate coresetPoolIndex with SRS resource set implicitly or explicitly.

  - FFS: the maximal number of configured/indicated SRS resources in each set for NCB/CB

  - FFS: the maximal number of SRS ports in each set for CB.

- FFS: Separate codebooks and separate maxRanks are configured for different SRS resource sets.

- For type 1 CG-PUSCH (if supported), FFS how to associate the PUSCH with one TRP

  - e.g., configure a coresetPoolIndex value in a type 1 CG-PUSCH

  - e.g., use a single CG to configure two type 1 CG PUSCHs for STxMP PUSCH+PUSCH

Agreement

Support dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission

- FFS the indication of dynamic switching

- FFS: max number of layers when switching to sTRP transmission

Agreement

Support SFN-based transmission scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18

Agreement

Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission:

- For 2 PTRS ports, study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM scheme

Agreement

For the switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme, Alt2 is supported. FFS: Whether Alt1 is supported in addition to Alt2.

- Alt1: Support dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme

  - FFS: how to support dynamic switching, e.g., using the indicated PUSCH repetition number

  - Note: It is up to gNB implementation to configure SDM scheme of single-DCI based STxMP PUSCH or Rel-17 mTRP PUSCH TDM scheme or both of them in RRC. Dynamic switching between them is only when both schemes are configured in RRC.

- Alt2: Support RRC-based switching between SDM scheme of single-DCI based STxMP PUSCH and Rel-17 mTRP PUSCH TDM scheme

Agreement

The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain.

- FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements).

- Note: No symbol-level power adjustment within a PUSCH transmission occassion in the case of fully/partially overlapping in time domain

Agreement

Multi-DCI based STxMP PUSCH+PUSCH transmission at least supports the following PUSCH combinations:

- DG-PUSCH + DG-PUSCH

- CG-PUSCH + DG-PUSCH

Agreement

Support the following cases for codebook design for 8TX precoders

- Full coherent precoders with Ng=1

  - FFS: Full coherent precoders with Ng=2, Ng=4

- Partial coherent precoders with Ng=2 and Ng=4

  - This does not imply any relation with the number of TPMI indications for 8TX precoder

- Non-coherent precoders

Agreement

For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource

- For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7}

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

  - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7}

- For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

  - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7}

  - Alt3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5, 7}

- Note: Other alternatives which are not foreseen are not precluded

Agreement

For SRI and/or transmitter precoder matrix indication for codebook-based uplink transmission by an 8TX UE, study

- Whether/how to indicate one or multiple TPMI/SRI, according to the number of antenna groups, coherence capability, codebooksubset configuration, etc.

- Whether/how to extend Rel-17 framework, e.g., TPMI/SRI indication in MTRP PUSCH

- Whether/how to separate/joint indication of rank and precoding information.

- Whether/how to indicate n (<=Ng) selected antenna group(s) separately from TPMI/TRI indication

Agreement

In Rel-18, on support of full power operation by a partial/non-coherent 8TX UE configured with codebook-based transmission,

- Identify and agree on at least one potential PA architecture by RAN1 meeting #111

Agreement

For 8TX UE codebook-based uplink transmission,

- For partially/non-coherent precoding, support NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook

Agreement

For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is

- Alt1: A single SRS resource set configured with up to 8 single-port SRS resources

- FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively.

Agreement

For SRS configuration supporting codebook -based UL transmission for an 8TX UE ,

- Support configuration of 1 SRS resource set containing up to X 8-port SRS resource(s), where X = 2

  - FFS : Other values for X, if needed

- FFS : Configuration of at least one SRS resource set, configured with more than one SRS resources where each SRS resource may have the same or different number of SRS ports, e.g., for support full power operation, if supported

- FFS : Configuration of at least one SRS resource set, configured with 8/M of M-port SRS resources, for example,

  - Configuration of an SRS resource set, configured with at least 4 of 2-port SRS resources

  - Configuration of an SRS resource set, configured with at least 2 of 4-port SRS resources

Working Assumption

For uplink transmission with rank>4, support dual CW transmission.

Agreement

If dual CW is supported for uplink transmission with Rank>4 by an 8TX UE, reuse DL Rel-15 codeword to layer mapping for both codebook-based and non-codebook-based transmission.

In RAN1#111, the following agreements were made.

Multi-TRP enhancement

Agreement

On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception

- The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception

  - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception

- FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field)

- FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold

FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0.

Above applies for the case where PDSCHs scheduled by the same DCI.

Agreement

On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group

- Note: Detail of the RRC configuration is left to RAN2 design

Agreement

On unified TCI framework extension for S-DCI based MTRP, in one beam indication instance, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate joint/DL/UL TCI state(s) for one or both of the two TRPs in a CC/BWP or a set of CCs/BWPs in a CC list

- FFS: Increase on the size of the TCI field

- Note: The term TRP is used only for discussion purpose in RAN1 and whether/how to capture this is FFS

Agreement

On unified TCI framework extension, PDSCH-CJT is supported as a S-DCI based MTRP scheme

Note: Above does not preclude discussions specific to PDSCH-CJT design in the unified TCI framework

Agreement

On unified TCI framework extension, down-select at least one of the following alternatives for PDSCH-CJT applying both indicated joint TCI states (if the UE supports two indicated joint/DL states for PDSCH-CJT):

- Alt1: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA

- Alt2: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint TCI state

- Alt3: PDSCH DMRS port(s) is QCLed with the DL RS of the first indicated joint TCI state with respect to QCL-TypeA and QCLed with the DL RS of the second indicated joint TCI state with respect to QCL-TypeB

Agreement

On unified TCI framework extension for S-DCI based MTRP, use an indicator field (could be reusing an existing DCI field or introducing a new DCI field) in the DCI format 0_1/0_2 to inform which joint/UL TCI state(s) indicated by MAC-CE/DCI the UE shall apply to PUSCH transmission scheduled/activated by the DCI format 0_1/0_2

Agreement

On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH.

Agreement

On unified TCI framework extension for M-DCI based MTRP, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to PUSCH transmission scheduled/activated by PDCCH (including DG-PUSCH and Type2 CG-PUSCH) on a CORESET that is associated with the same coresetPoolIndex value

Agreement

On unified TCI framework extension for S-DCI based MTRP, a new indicator field is supported as the DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception

- FFS: Detail design of the new indicator field

Working Assumption

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI

- the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases.

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI.

- FFS: for intra-cell Multi-DCI

- FFS: whether there are any restrictions needed

- FFS: if cross TRP RACH triggering is an optional feature

Conclusion

For multi-DCI based Multi-TRP operation with two TA enhancement, there is no consensus to support enhancements for CBRA triggered by PDCCH order.

Conclusion

For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure.

Agreement: Send LS to RAN2 regarding the above conclusion. Final LS in R1-2213004.

CSI enhancement

Agreement

For the Type-II codebook refinement for CJT mTRP, the values of the following codebook parameters are gNB-configured via higher-layer (RRC) signaling and common across all N CSI-RS resources:

- N1, N2

  - Note: Since (O1, O2) are fixed for a given (N1, N2), (O1, O2) are also common across all N CSI-RS resources

- R, 

  - Note: Since N3 is inferred from the number of CQI subbands and R, N3 is also common across all N CSI-RS resources

  - Note: Since per-layer K0 is defined jointly across all N CSI-RS resources, so is .

Conclusion

On the Type-II codebook refinement for CJT mTRP, for mode-1 and mode-2, there is no consensus on introducing additional/explicit per-CSI-RS-resource amplitude scaling and/or co-phase (with separate alphabet set(s)) as additional PMI component(s).

- Note: This conclusion has no impact on the Working Assumption reached in RAN1#110bis-e regarding W2 quantization group

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter R, the supported value(s) from the legacy specification are reused.

- FFS: whether additional value 4 can also be added

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter , introduce as a candidate value  = 1/8 in addition to the supported value(s) from the legacy specification.

- FFS (by RAN1#111): whether additional value 1 can also be added

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter pv, in addition to the supported value(s) from the legacy specification for Rel-16 regular eType-II codebook, introduce as a candidate value

- pv = 1/8 for v=1,2 (hence 1/16 for v=3,4)

FFS (by RAN1#111): whether additional value pv = 1/2 for v=1,2,3,4 can also be added

Agreement

On the Type-II codebook refinement for CJT mTRP:

- The maximum value of 2NN1N2 is 128

  - The support of 2NN1N2 >32 is UE optional for UEs supporting Rel-18 CJT CSI enhancement

- Note: Following the legacy specification on the maximum number of NZP CSI-RS ports per CSI-RS resource, the maximum value of 2N1N2 is 32.

Agreement

On the Type-II codebook refinement for CJT mTRP, on the L parameter, down select from the following alternatives (by RAN1#111):

- Alt1. Each of the {Ln, n=1, ..., N} is gNB-configured via higher-layer (RRC) signaling

- Alt4. Lmax is gNB-configured via higher-layer (RRC) signaling and the relative value(s) of {Ln, n=1, ..., N} are reported by the UE

  - The relative value(s) of {Ln, n=1, ..., N} are reported by the UE, such that n=1NLn≤Lmax

  - TBD: Whether the value(s) of {Ln, n=1, ..., N} or the total value of n=1NLn are reported implicitly or explicitly

FFS (by RAN1#111):

- Whether the supported candidate values for Ln follow the legacy candidate values for L, or some additional value(s) are also supported

- If Alt4 is supported, whether the candidate values for Ln are gNB-configured via higher-layer signaling

Agreement

On the Type-II codebook refinement for CJT mTRP, for mode-1, study and down select (no later than RAN1#112) only one from the following schemes:

- Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

  - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources

- Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset)

- Alt3. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

  - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf,n where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf,n is independently selected across N CSI-RS resources

For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis.

Note: Per previous agreements, the number of selected FS basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

Conclusion

For the Rel-18 Type-II codebook refinement for high/medium velocities, there is no consensus on applying DD unit for CQI. Therefore, DD unit (of size d slots) is applied only to PMI.

- Note: This conclusion has no impact on the number of CQIs included in one CSI reporting instance (a separate issue to be decided separately)

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities,

- For PMI, DD unit duration of d (in slots) is the duration associated with each of the N4 W2 matrices (combining coefficients before DD compression at the UE, or after DD de-compression at the gNB).

- TBD (by RAN1#111): The time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), and the number of CQI(s) X included in a CSI report

Agreement

For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding the parameter Q, at least Q=2 is supported.

- FFS: Whether Q=3 and/or Q=4 are also supported as other candidate value(s), as well as the supported Parameter Combination(s)

Agreement

For the Type-II codebook refinement for high/medium velocities, the parameter WCSI (in slots) is determined as follows: WCSI = dN4

Agreement

For the Type-II codebook refinement for high/medium velocities, the parameter N4 (length of DFT vector, unit-less) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {1, 2, 4}

- FFS: If additional candidate value(s) of N4 are supported, e.g. 3, 5, 6, 8, 10, 16, 32, as well as the supported Parameter Combination(s)

Agreement

For the Type-II codebook refinement for high/medium velocities, the parameter K (the number of AP-CSI-RS resources for the CMR) is gNB-configured via higher-layer (RRC) signalling at least from the following set of candidate values: {4, 8}

- FFS: If additional candidate value(s) of K are supported, e.g. 5, 12, 16, also taking into account other use cases (e.g. for training filter coefficients, prediction or performance monitoring) and TDD

Agreement

For the Type-II codebook refinement for high/medium velocities, the parameter m (offset between two AP-CSI-RS resources for the CMR, in slots) is gNB-configured via higher-layer (RRC) signalling from the following set of candidate values: {1, 2}

- FFS: Whether 4, 5, 8, 12, and/or 16 are also supported as other candidate value(s)

Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, support the following:

- Q different 2-dimensional bitmaps are introduced for indicating the location of the NZCs, where the qth (q=1,…., Q) 2-dimensional bitmap corresponds to qth selected DD basis vector

  - The number of selected DD basis vectors is denoted as Q

  - This implies that for each layer, the location of NZCs in SD-FD can be different for different selected DD basis vectors.

FFS: Further overhead reduction on bitmap(s)

FFS: Whether the number of NZCs is upper bounded across all DD basis vectors or per DD basis vector

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities,

- The definition and supported values for each of the SD/FD codebook parameters follow the legacy specification.

  - FFS: The supported parameter combinations considering SD, FD, and DD codebook parameters

- For N4=1, the legacy quantization is fully reused.

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, for N4=1, one CSI reporting instance includes a single W2 per layer, a single W1, and a single Wf

Conclusion

For the Rel-18 Type-II codebook refinement for high/medium velocities, when N4>1, there is no consensus in introducing rotation factor (selected for each SD basis vector) as an additional PMI component

Agreement

For the Type-II codebook refinement for high/medium velocities, in one CSI reporting instance, for a given CQI sub-band, at least support including one CQI

- FFS: The association of the CQI with PMI(s) and/or slot(s) within one duration of CSI reporting window WCSI

- FFS: The support for including more than one CQIs

Agreement

For the Type-II codebook refinement for high/medium velocities, the parameter δ (in slots) is gNB-configured via higher-layer (RRC) signalling from a set of the following candidate values:

- First candidate value: δ=0,

- 2 additional non-zero values of parameter δ are supported

  - FFS: the non-zero value(s), to be selected from 1, 2, 3, 4, 5, 6, and 8

Conclusion

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter , there is no consensus in supporting the additional value of 1.

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter pv, support the additional value of pv=1/2 for v=1,2,3,4 with the following condition:

Only to be used in combination with other parameter value(s) to limit the increase in PMI overhead comparable to the maximum overhead of the legacy Rel-16/17 Type-II codebooks (exact parameter combination(s) FFS)

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the bitmap(s) for indicating the locations of NZCs, reuse the legacy design. This implies that the size of the bitmap for selected CSI-RS resource n (Bn) is, Bn=2LnMv

- FFS: additional mechanism to reduce bitmap overhead for larger N values, e.g. including via Parameter Combination

Agreement

On the W2 coefficient quantization scheme for the Type-II codebook refinement for CJT mTRP, for N=3 and N=4, just as N=2, reuse the following components of the legacy Rel-16/17 per-coefficient quantization scheme:

- Alphabets for amplitude and phase

- Quantization of phase and quantization of differential amplitude relative to a reference, reference amplitude (with SCI determining the location of one reference amplitude), where the reference is defined for each layer and each “group” of coefficients

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), assuming 1 CQI in one sub-band and one CSI reporting instance, down-select (by RAN1#112) one from the following alternatives:

- Alt1. The CQI is associated with the entire duration of the CSI reporting window and all the N4 W2 matrices

- Alt2A. The CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices

- Alt2B.  The CQI is associated with the first/earliest d slots of the CSI reporting window and the first/earliest one of the N4 W2 matrices

Note: The N4 W2 matrices represent the combining coefficients before DD compression at the UE, or after DD de-compression at the gNB

Agreement

For the Rel-18 Type-II codebook refinement for high/medium velocities, decide by RAN1#112 whether including X>1 CQIs in one sub-band and one CSI reporting instance are supported

- If supported, also decide the value(s) of X and the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots)

Agreement

For the Type-II codebook refinement for high/medium velocities, for N4>1, regarding parameter Q, decide in RAN1#112 whether to support the additional values of 3 and/or 4

Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the parameter δ (in slots), support the additional value of 2

- FFS (by RAN1#112): For the last supported additional value, down select between 1, 3, 4, and 5

Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the parameter N4 (length of DFT vector, unit-less), support 8 as an additional candidate value

- FFS (by RAN1#112): Whether any of the following additional candidate values are supported: 3, 5, 16, 32

- The candidate values supported by UE are reported via UE capability (details can be discussed in UE feature).

Agreement

For the Type-II codebook refinement for high/medium velocities, regarding the parameter d (in slots),

- Support at least the following candidate value:

  - If the configured CMR is AP-CSI-RS, this candidate value is the configured value of m parameter

  - FFS: If the configured CMR is P/SP-CSI-RS

  - FFS: Whether in the above two cases, the number of slots between the last CSI-RS occasion no later than the legacy reference resource and the starting of WCSI window shall be integer multiples of d slots.

- FFS: Whether additional candidate value(s) of d are supported, e.g. d<m, d>m, d=1

If more than one candidate values of d are supported, the value of d is gNB-configured via higher-layer (RRC) signalling

Conclusion

For the Type-II codebook refinement for high/medium velocities, regarding the parameter m (offset between two AP-CSI-RS resources for the CMR, there is no consensus in supporting additional candidate value(s).

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the SD basis selection, for a configured value of NTRP, a set of NL combinations of values for {L1, ..., LNTRP} is gNB-configured via higher-layer (RRC) signaling

- When NL>1, the selected combination of values for {L1, ..., LNTRP} is reported in CSI part 1 using an indicator, selected from the NL configured combinations

  - NL =1 is one of the supported candidate values

  - FFS: Other supported value(s) of NL, and its respective UE capability

  - FFS: The supported combinations of values for {L1, ..., LNTRP}

- Following the legacy design, the SD basis selection for the n-th (n=1,...,N) selected CSI-RS resource is indicated in CSI part 2 using a combinatorial indicator selected from a set of  PCSI-RS/2Ln codepoints where, for Rel-16-based refinement PCSI-RS = 2*N1N2.

- The supported candidate values for each of the Ln parameters include the legacy candidate values, i.e. {2,4,6} for Rel-16-based refinement, and

  - for Rel-17-based refinement, the gNB configures a set of N_L combinations for {alpha1, ..., alphaNTRP}  Ln=αnPCSI-RS/2 where αn=12,34, 1

FFS: Whether the set of NL combinations of values for {L1, ..., LNTRP} can be implicitly derived

Following the legacy design, for all the selected N CSI-RS resources, the SD basis oversampling group for each CSI-RS resource is indicated in CSI part 2 using an indicator selected from a set of O1O2 codepoints.

Agreement

For the Rel-18 TRS-based TDCP reporting, down select only one of the following alternatives by RAN1#112:

- AltA.1 (Doppler spread) as described in R1-2210523

- AltA.2 (Doppler shift): A UE is configured to report the Doppler shifts corresponding to the M strongest peaks of the wideband Doppler spectrum, for each of the N≥1 configured TRS resources

  - A TDCP report can be configured with N periodic TRS resources (e.g., N=2 with one TRS resource per TRP)

  - Parameter M is RRC configured with candidate values TBD, e.g. M=1,2,3,…

  - Wideband Doppler spectrum is calculated from the wideband time correlation function, given, as an example, by At,τ=ct,τct,0 , where  ct,τ=n=0N-1hnt+τ∙hn*t and hn is the channel for subcarrier n.

- AltB (TD correlation profile) as described in R1-2210523

Down-selection is to done based on, at least, the (single-)user throughput (LLS) performance comparison among the alternatives assuming:

- Three special cases of an agreed use case (companies can select only one or more): aiding gNB to determine switching between Type-I and Rel-16 eType-II codebooks, or to determine SRS periodicity in the UL-SRS reciprocity-based precoding scheme; or aiding the gNB implementation in CSI prediction for TDD

  - In their simulations on switching between Type-I and Rel-16 eType-II codebooks, companies should state how to calculate the metric for the determination and how to set the threshold, and what the UE reports.

  - In their simulations on UL-SRS reciprocity-based precoding scheme, companies should state how to set the SRS periodicity based on the reported metrics, and what the UE reports; and the results should be displayed in terms of user throughput vs SRS overhead

  - In their simulations on CSI prediction for TDD, the results should be the correlation between real channel and predicted channel, and what the UE reports; aided by the reported metric.

  - Other scenarios of the agreed use cases can optionally be simulated

- Based on the agreed EVM for sTRP and mTRP

Note: Different alternatives may or may not apply to different use cases

FFS: The need for a measure of confidence level in the TDCP report, and/or UE behaviour when the quality of TDCP measurement is not sufficiently high

FFS: TDCP parameter(s) signalled with respect to each alternative

Reference signal enhancement

Agreement

For FD-OCC length 4 for PDSCH/PUSCH, select the following:

- Opt.1-1 (Walsh matrix) for PDSCH

- Opt.1-2 (Cyclic shift) for PUSCH

Agreement

- For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, all of the following port combinations can be indicated:

  - Cat. 1) Legacy port indexes (eType 1: p=0~7, eType 2: p=0~11)

  - Cat. 2) New port indexes (eType 1: p=8~15, eType 2: p=12~23)

  - Cat. 3) Legacy port indexes and New port indexes at least within a CDM group at least for maxLength=1 (eType 1: up to 4 ports from {0, 1, 8, 9} and/or up to 4 ports from {2, 3, 10, 11}, eType 2: up to 4 ports from {0, 1, 12, 13} and/or up to 4 ports from {2, 3, 14, 15} and/or up to 4 ports from {4, 5, 16, 17}) at least for S-TRP case,

    - For up to 4 ranks, only one CDM group is used per UE. For larger than 4 ranks, more than one CDM groups can be used per UE.

- FFS: Whether to increase the size of antenna ports field in DCI format 1_1/1_2, or introduce new DCI field for antenna ports indication, or not.

- FFS: Whether the new antenna port(s) table is specified or not.

- FFS: MU restrictions for certain entries. e.g., DMRS ports = {0,2}, or {8,10}, etc.

- FFS: Cat.3 for M-TRP case.

- Note: DMRS port index for PDSCH is determined by p +1000

Agreement

For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS.

- For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following:

  - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook.

- For Rel.18 eType1/eType2 DMRS ports,

  - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details).

  - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked.

- FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group.

- FFS: One or more than one DMRS port combination(s) for each rank and TPMI

- Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17.

- FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports.

Agreement

For length 2 TD-OCC (across consecutive DMRS symbols, if any) for DMRS of PDSCH/PUSCH for Rel.18 eType 1/2 DMRS, support

| TD-OCC index | Wt(0) | Wt(1) |
| --- | --- | --- |
| 0 | +1 | +1 |
| 1 | +1 | -1 |

Agreement

- For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured.

- Down select from the following in RAN1#112:

  - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2.

    - FFS: Association with the CW with the higher MCS.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW with the higher MCS |
| 1 | 2nd scheduled DMRS port the CW with the higher MCS |
| 2 | 3rd scheduled DMRS port the CW with the higher MCS |
| 3 | 4th scheduled DMRS port the CW with the higher MCS |

  - Alt.2: The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port |
| 4 | 5th scheduled DMRS port |
| 5 | 6th scheduled DMRS port |
| 6 | 7th scheduled DMRS port |
| 7 | 8th scheduled DMRS port |

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case.

- Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes |
| 0 | [1] | [0] | Cat. 1 | [0] | [2] | [0,1,2,3,8] | [Rank 5-8 with one DMRS symbol] |
| 1 | [1] | [1] |  | [1] | [2] | [0,1,2,3,8,10] |  |
| 2 | [1] | [0,1] |  | [2] | [2] | [0,1,2,3,8,9,10] |  |
| 3 | 2 | 0 |  | [3] | [2] | [0,1,2,3,8,9,10,11] |  |
| 4 | 2 | 1 |  |  |  |  |  |
| 5 | 2 | 2 |  |  |  |  |  |
| 6 | 2 | 3 |  |  |  |  |  |
| 7 | 2 | 0,1 |  |  |  |  |  |
| 8 | 2 | 2,3 |  |  |  |  |  |
| 9 | [2] | [0-2] |  |  |  |  |  |
| 10 | [2] | [0-3] |  |  |  |  |  |
| 11 | [2] | [0,2] |  |  |  |  |  |
| 12 | [1] | [8] | Cat.2 |  |  |  |  |
| 13 | [1] | [9] |  |  |  |  |  |
| 14 | [1] | [8,9] |  |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |  |
| 21 | [2] | [8-10] |  |  |  |  |  |
| 22 | [2] | [8-11] |  |  |  |  |  |
| 23 | [2] | [8, 10], [9, 11] |  |  |  |  |  |
| 24 | [1] | [0,1,8] | Cat.3 |  |  |  |  |
| 25 | [1] | [0,1,8,9] |  |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |  |

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

- FFS: Hopping pattern

- Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

- Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

  - FFS: Other types of SRS

- FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

- FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support:

- For comb 2, support 1 and 2 comb offsets

- For comb 4, support 2 and [4] comb offset

- For comb 8, support 4 comb offsets

Agreement

For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols

- Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM)

- FFS: m can be legacy values, i.e., 2,4,[8,10,12,14].

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with a network-configured ID.

- FFS: The ID could be cell ID nIDcell, SRS sequence identity nIDSRS, C-RNTI, or a new ID

- FFS: The relation between the legacy group / sequence hopping and the new hopping

Agreement

For SRS interference randomization, support one from the following options (to be decided in RAN1#112):

- Opt. 1: Cyclic shift hopping

- Opt. 2: Comb offset hopping

- Opt. 3: Both cyclic shift hopping and comb offset hopping

  - FFS: details including whether to support separate and/or combined hopping

  - FFS: details on UE capability and signaling

Conclusion

No consensus on enhanced signaling for flexible SRS transmission in Rel-18

Enhanced uplink transmission

Agreement

For the DMRS port indication for SDM scheme single-DCI based STxMP transmission, support Alt2:

- Alt2: the DMRS ports associated with two TPMI/SRI fields can be in same or different CDM groups.

Agreement

Support CG PUSCH + CG PUSCH in multi-DCI based STxMP PUSCH+PUSCH transmission.

Agreement

Support the SFN scheme for single-DCI based STxMP PUCCH transmission

Conclusion

There is no consensus on the support STxMP PUCCH+PUCCH in multi-DCI based mTRP system

Agreement

- For multi-DCI based STxMP, to schedule a PUSCH for STxMP PUSCH+PUSCH transmission,

  - Alt1: The first SRS resource set is associated with coresetPoolIndex value 0 and the other SRS resource set is associated with coresetPoolIndex value 1

    - The PUSCH is associated with SRS resource set with the same value of coresetPoolIndex

    - FFS: Which is the first SRS resource set, e.g., the set with lower set ID.

- Regarding how to interpret the SRI/TPMI field in DCI:

  - For DG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where scheduling DCI format 0_1 or 0_2 is received

  - For Type 2 CG-PUSCH, the indicated SRI/TPMI field corresponds to the SRS resource set associated with same coresetPoolIndex value of the CORESET where activation DCI is received.

- For Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in RRC in ConfiguredGrantConfig and the srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set

Agreement

- Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission

  - The DCI field “SRS resource set indicator” is used to indicate the switching between SFN scheme and sTRP transmission.

- (Conclusion) There is no consensus to support DCI-based dynamic switching between SFN scheme and SDM scheme

Agreement

For the SFN scheme of single-DCI based STxMP PUSCH:

- Configure two SRS resource sets for CB or NCB.

  - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource

- The DCI indicates two SRI fields and TPMI fields for SFN transmission,

- On the indication of number of layers for CB and NCB PUSCH:

  - Alt1: Similar to rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH)

Agreement

For dynamic switching between SDM scheme of single-DCI based STxMP PUSCH and sTRP transmission:

- Use the 2-bit “SRS resource set indicator” DCI field to dynamically indicate the sTRP or SDM transmission.

- FFS: how to interpret each codepoint of “SRS resource set indicator”.

- For the maximal number of layers for sTRP transmission, down-select:

  - Option1: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) in current spec

  - Option2:  Configuring one additional maximal numbers of layers for sTRP transmission, in addition to maxRank in current spec.

- Down-select one from the following for maximal number of layers of SDM transmission:

  - Alt1: Configure one single maximal number of layers (separate from the maximal number(s) of layers for sTRP), that is applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt1a: The maxRank (or Lmax) in current spec is also applied to the first SRS resource set and the second SRS resource set, separately.

  - Alt2: Configure separate maximal numbers of layers for the first SRS resource set and the second SRS resource set

  - Alt3: no dedicated configuration for SDM. The maximal number(s) of layers of sTRP and the UE capability reporting for SDM are used to determine the maximal number of layers of SDM transmission.

  - Alt4: The maximal number(s) of layers in above Option1/2 also applied to the first SRS resource set and the second SRS resource set, separately.

- FFS:  To ensure the same size of DCI for sTRP and SDM cases, how to use/interpret the TPMI/SRI field(s) and whether to do reserved bit or zero padding.

Agreement

For SDM scheme single-DCI based STxMP transmission, when L1 and L2 layers are indicated/determined by two TPMI fields of CB PUSCH or two SRI fields of NCB PUSCH respectively:

- The first L1 indicated DMRS ports correspond to the L1 layers indicated by the first TPMI or SRI field

- The remaining L2 indicated DMRS ports correspond to the L2 layers indicated by the second TMPI or SRI field

- Support at least one of the following options for indication of layer combination {1+2}:

  - Option 1: new entry is added to DMRS table, e.g., {0, 2, 3}, {2, 0, 1}.

  - Option 2: use DCI field (e.g., SRS resource set indicator) to indicate that for layer combination {1+2}, the first two indicated DMRS ports correspond to the 2 layers indicated by the second TPMI or SRI field and the rest one indicated DMRS port correspond to the layer indicated by the first TPMI or SRI field.

  - Option 3: For layer combination of {1+2}, the DMRS port in the CDM group with only one port is mapped to the SRI/TPMI field indicating one layer, and the DMRS ports in the CDM group with 2 ports are mapped to the SRI/TPMI field indicating 2 layers

  - Other options are not precluded

Conclusion

There is no consensus to support layer combinations {1+3} and {3+1} in SDM scheme of single-DCI based STxMP PUSCH.

Agreement

For a fully coherent uplink precoding by an 8TX UE,

- Support NR Rel-15 single panel DL Type I codebook as the starting point for design of the codebook

  - FFS: For a constructed codebook with size M based on above method, unless M=2N; otherwise, round up the codebook size to the smallest integer 2N>M by adding 2N-M precoders generated via Alt 2a.

- No LS to RAN4 will be needed

Agreement

For PUSCH transmission with rank>4 by an 8TX UE, to support dual CW transmission,

- specify MCS, NDI, RV indication for the second CW

- specify PUSCH Scrambling for the second CW

- specify UCI multiplexing on PUSCH for dual CW transmission

- study whether/how Enabling/Disabling the second CW

FFS: Optimization of DCI to indicate the above

Note: Strive to reuse Rel-15 NR DL schemes where possible.

Agreement

For PUSCH transmission with rank>4 by an 8TX UE, to support UCI multiplexing on PUSCH, down-select at least one of the following options in RAN1#112,

- Option1: UCI is always multiplexed on one of the CWs

- Option2: UCI is multiplexed on both CWs

- Option3: Based on UCI (e.g., type, payload size, etc.) UCI is multiplexed on one or both CWs

- Option4: UCI is multiplexed only when single CW is enabled

- Option5: UCI is repeated across the two CWs

- Other options are not precluded

Agreement

For CB-based 8TX PUSCH transmission, for rank indication, down-select among the following

- Separate indication of TRI and TPMI

- Joint indication of TRI and TPMI

Agreement

Study full TX power uplink codebook-based transmission by a partially/non-coherent 8TX precoder,

- Reuse Rel-16 UE capability definitions for discussion purpose, i.e., UE Capability 1, 2 and 3

- For full TX power transmission by UE Capability 2/3, at least, following exemplary PA architectures can be considered

  - Other cases of interest are not precluded, down-select preferred potential architecture for the purpose of 8TX full power study in RAN#112.

  - This can be used for other UE Power Classes as well.

| 8TX UE, Power class 3 (23 dBm) Pi= Nominal power rating of each PA |  |  |
| --- | --- | --- |
|  | Regular UE | P1=P2= …=P8=14 dBm (Full power supported by Mode1) |
|  | Full-power capable UE | Full power capability with any PA comb. (CAP1) Example: P1=P2= …=P8= 23 dBm |
|  |  | Full power capability with 1 PA (CAP3) Example: P1=P2= …=P7= 14 dBm P8= 23 dBm |
|  |  | (lower priority) Full power capability with 2 PAs (CAP2) Example 2a: P1=P2= …=P6= 14 dBm, P7=P8 ≥ 20 dBm Example 2b: P1=P2= …= P8= 20 dBm |
|  |  | (lower priority) Full power capability with 4 PAs (CAP2) Example 3a: P1=P2= …=P4= 14 dBm, P5=P6= …=P8 ≥ 17 dBm Example 3b: P1=P2= …= P8 = 17 dBm |
|  |  | (lower priority) Full power capability with 6 PAs (CAP2) Example 4a: P1=P2= 14 dBm, P3=P4= …=P8 ≥ 15.3 dBm Example 4b: P1=P2= … = P8≥ 15.3 dBm |

Agreement

For an 8TX partial/non-coherent precoder, for study on full power codebook-based PUSCH transmissions, use Rel-16 full power modes as the starting point for the design.

Note: This does not mandate support of all Rel-16 modes.

#### 2.1.2	Remaining Open issues

Multi-TRP enhancement

- Remaining specification details on extension of Rel-17 unified TCI framework, including:

  - Indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework

  - UL beam indication for, if specified, PUCCH/PUSCH to facilitate simultaneous multi-panel UL transmission, where unified TCI framework extension is assumed, considering single DCI and multi-DCI based multi-TRP operation

    - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

- Remaining specification details on power control for UL single DCI for multi-TRP operation where unified TCI framework extension is assumed

- Remaining specification details on Two TAs for UL multi-DCI for multi-TRP operation

CSI enhancement

- Remaining specification details on CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, including:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

- Remaining specification details on Rel-16/17 Type-II codebook refinement for CJT mTRP and its associated CSI reporting, targeting FDD FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs

Reference signal enhancement

- Remaining specification details on DMRS enhancement to increase the number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM.

- Remaining specification details on DMRS enhancements for support of more than 4 layers SU-MIMO PUSCH.

- Remaining specification details of SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization

- Remaining specification details on SRS enhancements to enable 8 Tx UL operation

Enhanced uplink transmission

- Remaining specification details on UL precoding indication for PUSCH to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable):

  - No new codebook is introduced for multi-panel simultaneous transmission

  - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

- Remaining specification details on UL transmission for 8TX UE, including;

  - SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Consideration of coherence assumption, full/non-full power modes

  - Potential additional specification support to enable two codewords for rank > 4

## 2.2	RAN2

#### 2.2.1	Agreements

Not yet started

#### 2.2.2	Remaining Open issues

Not yet started

## 2.3	RAN3

n/a

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Not yet started

#### 2.4.2	Remaining Open issues

Not yet started

## 2.5	RAN5

n/a

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

n/a

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template