3GPP TSG RAN Meeting #98-e						RP-222838

Electronic Meeting, December 12-16, 2022

______________________________________________________________________Agenda item: 9.3.1.5

Source: LG Electronics

Title: Discussion on the scope of Rel-18 sidelink evolution

Document for: Discussion

- Introduction

Rel-18 WI on NR sidelink evolution has two objectives on sidelink carrier aggregation and sidelink FR2 on which a decision in RAN#98-e is necessary as follows [1]:

1. Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#98-e) - Support only LTE sidelink CA features for NR (i.e., SL carrier (re-)selection, synchronization of aggregated carriers, handling the limited capability, power control for simultaneous sidelink TX, packet duplication) - The work is limited to FR1 licensed spectrum and ITS band in FR1. - No specific enhancements of Rel-17 sidelink features with sidelink CA support. - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI) […] 3. Study and specify enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2, RAN4] (Determine in RAN#98-e whether to continue the study or study + specification work for FR2 until the end of R18) - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Work is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

This contribution discusses how to proceed with these objectives in Rel-18.

- Discussion

While the WG progress of this WI has been reasonable so far, we still envision that WGs will need considerable discussion time to resolve several essential topics for the objectives of sidelink unlicensed band and LTE/NR sidelink co-channel coexistence. For SL CA and SL FR2 objectives, RAN1 and RAN2 work that might become relatively manageable as discussed in [2], but we note that both objectives also require RAN4 work, for example, new RF requirements for sidelink CA and FR2, RRM requirements for FR2 beam management, etc. Such RAN4 work can be started after RAN1 and RAN2 make some progress but, now that these WGs did not have a chance to discuss the technical solutions, it seems challenging to complete the normative work of the two objectives, sidelink CA and FR2.

Based on this consideration, we propose to defer the normative work of sidelink CA and sidelink FR2 to Rel-19.

- Conclusion

This document discussed how to proceed with the objective on the sidelink CA and sidelink FR2 in Rel-18. We propose RAN to take the following proposal.

Proposal: The normative work for sidelink CA and sidelink FR2 is deferred to Rel-19.

- References

- RP-221938, “WID revision: NR sidelink evolution,” OPPO.

- RP-222016, “Discussion on the scope of Rel-18 sidelink evolution,” LG Electronics.