3GPP TSG-RAN WG1 Meeting #111	R1-2212912

Toulouse, France, November 14th-18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0385 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Priority rules for CSI reports in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the Priority rules for CSI reports in 5.2.5 of 38.214, PUSCH with data always overrides PUSCH with SP-CSI. Since no physical channel priorities are further specified in 38.214, this implies that LP PUSCH with data overrides HP PUSCH with SP-CSI regardless of their priorities. This is not the intention and should be corrected. This issue has been indirectly adressed in RAN1#110 where it was included as one case out of in total 5 cases related to overlap with SP-CSI. For all other cases, agreements could be achieved, wheras for the particular case brought up here, i.e. Case 2, only following conclusion could be made: Conclusion For Case 2, overlap between HP PUSCH with SP-CSI without DCI and LP PUSCH with DCI is an error case. Since other cases will be mentioned in the specification, but if not Case 2, then it may still cause an ambiguity between the gNB and the UE about the expected behaviour. It is not sufficient to use a conclusion to exclude the overlap between HP PUSCH with SP-CSI without DCI and LP PUSCH with DCI, since it is highly likely that for implementation not all companies will refer to the conclusion from the RAN1#110 meeting to identify the handling of this specific scenario, especially when other cases are explictely included in the specification. A simple solution to avoid this potential misunderstanding is to clarify that the priority rules for CSI reports defined in 38.214 are only for channels with the same PHY priority. In RAN1#110bis-e meeting, whether to capture the conclusion for case 2 is discussed in R1-2210712 and most companies agreed to capture the clarification in 38.214 to make the spec clearer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that the Priority rules for CSI reports in 5.2.5 is only for channels with the same priority in physical layer, if the UE is configured with two priority levels in physical layer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has isolated impact on applying the Priority rules for CSI reports if the UE is configured with two priority levels in physical layer. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.2.5	Priority rules for CSI reports

For two overlapping PUSCHs, the priority rules in this clause are applied for physical channels with same priority index according to clause 9 in [6, TS 38.213].

CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where

-	 for aperiodic CSI reports to be carried on PUSCH  for semi-persistent CSI reports to be carried on PUSCH,  for semi-persistent CSI reports to be carried on PUCCH and  for periodic CSI reports to be carried on PUCCH;

-	 for CSI reports carrying L1-RSRP or L1-SINR and  for CSI reports not carrying L1-RSRP or L1-SINR;

-	c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells;

-	s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations.

A first CSI report is said to have priority over second CSI report if the associated  value is lower for the first report than for the second report.

< Unchanged parts are omitted >