3GPP TSG RAN WG1 Meeting #111 R1-2212789 Toulouse, France, November 14th – 18th, 2022 CR-Form-v12.0. CHANGE REQUEST. 38.214; CR; 0376; rev; -; Current version:; 17.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; CR on beam application time for unified TCI in TS 38.214. Source to WG:; Moderator (ZTE), ZTE, Google, Huawei, Nokia, Samsung. Source to TSG:; R1. Work item code:; NR_FeMIMO-Core; Date:; 2022-11-17. Category:; F; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17). Reason for change:; The agreement was reached in RAN1#110bis-e as follows regarding beam application time for unified TCI. Since now, the agreement has not been captured in the spec. Agreement On beam application time for unified TCI framework, the active BWP is determined based on the active BWP with the smallest SCS among the active BWP(s) from the applying CCs at the end of PUCCH/PUSCH carrying the HARQ-ACK for the TCI indication.. Summary of change:; Clarifying that time point of determining active BWPs with smallest SCS among applying CCs is based on the end of PUCCH/PUSCH carrying the HARQ-ACK for the TCI indication.. Consequences if not approved:; It is unclear on time point to determine the active BWPs with smallest SCS among applying CCs.. Clauses affected:; 5.1.5. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: The CR aims to clarify the UE behavior on when to determine active BWPs with smallest SCS among applying CCs. The proposed clarification is essential for unified TCI state operation. It is expected UEs and networks have same understanding for implementation on this function.. This CR's revision history:; This is the first version for this CR..

### 5.1.5	Antenna ports quasi co-location

<Unchanged part omitted>

When the UE would transmit a PUCCH with HARQ-ACK information or a PUSCH with HARQ-ACK information corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State is different from the previously indicated one, the indicated DLorJointTCIState or UL-TCIstate should be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH. The first slot and the  beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State or TCI-UL-State that are active at the end of the PUCCH or the PUSCH carrying the HARQ-ACK information.

<Unchanged part omitted>