3GPP TSG RAN WG1 #111			R1-2212992

Toulouse France, November 14th – 18th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0391 | Rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the re-evaluation and pre-emption checking for periodic and aperiodic transmissions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (OPPO), Huawei, HiSilicon, Samsung, LG Electronics, Ericsson, ZTE, Sanechips, vivo, Lenovo, NTT DOCOMO, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Re-evaluation in sidelink mode 2 resource allocation, it is triggered for pre-selected but not yet reserved resources and pre-emption checking is triggered for reserved resources. When these procedures are triggered, they may be triggered individually or at the same time. Hence this needs to be corrected in the specification. Due to an error in the RAN1 agreement on the re-evaluation and pre-emption checking for periodic transmission (Prsvp_TX≠0 ), tyiSL is defined as the first candidate slot after slot n+T3, where the resource provided by higher layer for re-evaluation and pre-emption checking is in slot T3. This means, the first candidate slot does not include the resource provided in T3 for re-evaluation and pre-emption checking. And therefore, it should be corrected so that the resource provided in slot T3 is included as part of re-evaluation and pre-emption checking. When the UE is triggered to perform re-evaluation and/or pre-emption checking for aperiodic transmission, it is unclear what type of partial sensing scheme should be applied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that UE only can be triggered to perform re-evaluation and/or pre-emption checking. It is corrected that t'yiSL is the first candidate slot starting from slot n+T3 in re-evaluation and pre-emption checking procedures for both aperiodic and periodic transmissions. It is clarified what when the UE is triggered to perform re-evaluation and/or pre-emption checking for aperiodic transmission, it is clarified that the UE should perform at least contiguous partial sensing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains incorrect in its description whether the UE can be triggered to re-evaluation or pre-emption checking, or both. The specification remains to be unclear and ambiguous on the type of partial sensing the UE should performed when re-evaluation and pre-emption checking is triggered. The resource under the re-evaluation and/or pre-emption checking in n+T3 will never be part of the candidate resource set and subsequently the UE will always need to perform re-selection of resource under the evaluation and/or pre-emption checking. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

When the UE performs periodic-based partial sensing and contiguous partial sensing, and when the UE is triggered to perform re-evaluation and/or pre-emption checking, and if Prsvp_TX≠0,

-	During the qth reservation period (q=0,1,2,…, Cresel-1), candidate resource set (SA) is initialized to the remaining Y candidate slots starting from slot t'yiSL and ending at the last slot of the Y candidate slots, where the slot indices of the remaining Y candidate slots are equal to t'y+q×Prsvp_TX'SL, where t'ySL is a slot index of Y candidate slots used in the initial resource (re)selection.

-	t'yiSL is the first candidate slot starting from slot n+T3.

-	The UE performs PBPS for the remaining Y candidate slots according to ty'-k×Preserve''SL, wheret'y'SL is a slot belonging to the remaining Y candidate slots, and k and Preserve are the same as resource (re)selection, where the values of k correspond to the most recent sensing occasion earlier than t'yiSL-(Tproc,0SL+Tproc,1SL ) if sl-Additional-PBPS-Occasion is not (pre-)configured, and additionally includes the value of k corresponding to the last periodic sensing occasion prior to the most recent one if sl-Additional-PBPS-Occasion is (pre-)configured.

-	The UE performs CPS starting from M logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL.

-	By default, M is 31 unless (pre-)configured with another value. by sl-CPS-WindowPeriodic.

When the UE is triggered to perform re-evaluation and/or pre-emption checking, performs at least contiguous partial sensing, and if Prsvp_TX=0,

-	Candidate resource set (SA) is initialized to the remaining Y' candidate slots starting from slot t'yiSL and ending at the last slot of the Y' candidate slots, where t'yiSL is the first candidate slot starting from slot n+T3.

-	It is up to UE implementation that UE may perform PBPS for periodic sensing occasions after the resource (re)selection when higher layer parameter sl-MultiReserveResource is enabled

-	UE performs CPS starting from at least M consecutive logical slots earlier than t'yiSL to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL

-	For minimum size M of the CPS monitoring window [n+TA, n+TB], by default, M is 31 unless (pre-)configured with another value, by sl-CPS-WindowAperiodic.

When the minimum M slots for CPS cannot be guaranteed, UE senses in all available slots starting from the resource (re)selection trigger slot of the same TB to Tproc,0SL+Tproc,1SL slots earlier than t'yiSL. The UE re-evaluation and pre-emption checking is based on all available sensing results after n-T0.

<Unchanged parts omitted>