3GPP TSG-RAN WG1 Meeting #111	R1-2212995

# Toulouse, France, November 14th-18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0134 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specification is ambigous to whether the rightmost bit in the RRC parameter csi-ReportingBand is interpreted as subband 0 or whether the rightmost bit with value “1” is interpreted as subband 0. The CR clarifies this ambiguity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The change clarifies that the rightmost bit with value “1” in the RRC parameter csi-ReportingBand is interpreted as subband 0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description in 38.212 is unclear and leads to ambiguity. When network and UE have different understanding of the interpretation the CSI report may not be decodable correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.3.1.1.2	CSI only

If cqi-BitsPerSubband is configured, this Clause 6.3.1.1.2 applies by taking Subband CQI as Subband differential CQI and replacing the corresponding number of bits 2 by 4.

The bitwidth for PMI of codebookType=typeI-SinglePanel with 2 CSI-RS ports is 2 for Rank=1 and 1 for Rank=2, according to Clause 5.2.2.2.1 in [6, TS 38.214].

The bitwidth for PMI of codebookType=typeI-SinglePanel with more than 2 CSI-RS ports is provided in Tables 6.3.1.1.2-1, where the values of and  are given by Clause 5.2.2.2.1 in [6, TS 38.214].

Table 6.3.1.1.2-1: PMI of codebookType=typeI-SinglePanel

|  | Information field for wideband PMI |  |  | Information field for wideband PMI or per subband PMI |  |
| --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |
|  | codebookMode=1 | codebookMode=2 |  | codebookMode=1 | codebookMode=2 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (,) | N/A | 2 | 4 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (, 0) | N/A | 2 | 4 |
| Rank=2 with 4 CSI-RS ports, | (,) | (, 0) | 1 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (,) | 2 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (, 0) | 2 | 1 | 3 |
| Rank=3 or 4, with 4 CSI-RS ports | (,) |  | 0 | 1 |  |
| Rank=3 or 4, with 8 or 12 CSI-RS ports | (,) |  | 2 | 1 |  |
| Rank=3 or 4 , with >=16 CSI-RS ports | (, ) |  | 2 | 1 |  |
| Rank=5 or 6 | (,) |  | N/A | 1 |  |
| Rank=7 or 8, | (, ) |  | N/A | 1 |  |
| Rank=7 or 8, | (,) |  | N/A | 1 |  |
| Rank=7 or 8, with or or | (,) |  | N/A | 1 |  |

The bitwidth for PMI of codebookType= typeI-MultiPanel is provided in Tables 6.3.1.1.2-2, where the values of and  are given by Clause 5.2.2.2.2 in [6, TS 38.214].

Table 6.3.1.1.2-2: PMI of codebookType= typeI-MultiPanel

|  | Information fields for wideband |  |  |  |  | Information fields for wideband or per subband |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |  |  |  |  |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | N/A | N/A | 2 | N/A | N/A | N/A |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | 2 | 2 | 2 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=1 with codebookMode=2 | (,) | N/A | 2 | 2 | N/A | N/A | 2 | 1 | 1 |
| Rank=2 with , codebookMode=2 | (,) | 1 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=3 or 4 with , codebookMode=2 | (,) | 0 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=2 or 3 or 4 with , codebookMode=2 | (,) | 2 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |

The bitwidth for PMI with 1 CSI-RS port is 0.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel or reportQuantity set to 'cri-RI-CQI' is provided in Tables 6.3.1.1.2-3.

Table 6.3.1.1.2-3: RI, LI, CQI, and CRI of codebookType=typeI-SinglePanel, or reportQuantity set to 'cri-RI-CQI'

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator when codebookType=typeI-SinglePanel | 0 |  |  |  |  |
| Rank Indicator when reportQuantity set to 'cri-RI-CQI' | 0 | 1 | 2 | 3 | 3 |
| Layer Indicator | 0 |  |  |  |  |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 | 2 | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 0 | 2 |
| CRI |  |  |  |  |  |

in Table 6.3.1.1.2-3 is the number of allowed rank indicator values according to Clause 5.2.2.2.1 [6, TS 38.214].  is the value of the rank. The value of  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. For higher layer parameter reportQuantity set to 'cri-RI-CQI', the values of the rank indicator field are mapped to rank indicator values with increasing order, where '0' is mapped to rank-1.

Table 6.3.1.1.2-3A: RI, LI, CQI, and CRI associated with one CSI-RS resource pair and csi-ReportMode= Mode 1 or Mode 2

| Field | Bitwidth |  |
| --- | --- | --- |
|  | 1 antenna port per Resource | >1 antenna ports per Resource |
| Rank Combination Indicator | 0 | min(2,log2 nRI,NCJT) |
| The first Layer Indicator | 0 | log2v1 |
| The second Layer Indicator | 0 | log2v2 |
| Wide-band CQI for the first TB | 4 | 4 |
| Subband differential CQI for the first TB | 2 | 2 |
| CRI if csi-ReportMode= Mode 1 | log2N | log2N |
| CRI if csi-ReportMode= Mode 2 | log2M1+M2+N | log2M1+M2+N |

Table 6.3.1.1.2-3B: RI, LI, CQI, and CRI associated with one CSI-RS resource and csi-ReportMode= Mode 1 or Mode 2

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator | 0 | min(1,log2 nRI,sTRP) | min(2,log2 nRI,sTRP) | log2 nRI,sTRP | log2 nRI,sTRP |
| Layer Indicator | 0 | log2v | min⁡(2, log2v) | min⁡(2, log2v) | min⁡(2, log2v) |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 | 2 | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 0 | 2 |
| CRI if csi-ReportMode= Mode 1 and numberOfSingleTRP-CSI-Mode1 = 1 | log2M1+M2 | log2M1+M2 | log2M1+M2 | log2M1+M2 | log2M1+M2 |
| CRI if csi-ReportMode= Mode 1 and numberOfSingleTRP-CSI-Mode1 = 2 | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI | log2M1 for the first CRI; log2M2 for the second CRI |
| CRI if csi-ReportMode= Mode 2 | log2M1+M2+N | log2M1+M2+N | log2M1+M2+N | log2M1+M2+N | log2M1+M2+N |

nRI,NCJT in Table 6.3.1.1.2-3A is the number of allowed rank combination indicator values associated with one CSI-RS resource pair according to Clause 5.2.2.2.1X [6, TS 38.214]. The values of the rank combination indicator field are mapped to allowed rank combinations in the following order: {1,1}, {1,2}, {2,1},{2,2}, where '0' is mapped to {1,1}. v1 and v2 are the values of the first and the second rank associated with two CSI-RS resources of the CSI-RS resource pair respectively.

nRI, sTRP in Table 6.3.1.1.2-3B is the number of allowed rank indicator values associated with one CSI-RS resource according to Clause 5.2.2.2.1X [6, TS 38.214]. v is the value of the rank associated with the CSI-RS resource. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The value of N in Table 6.3.1.1.2-3A and Table 6.3.1.1.2-3B is the number of CSI-RS resource pairs configured within a CSI-RS resource set. The values of M1 and M2 in Table 6.3.1.1.2-3A and Table 6.3.1.1.2-3B are given by

-	If sharedCMR = "Enabled", M1 = K1 and M2 = K2

-	If sharedCMR is absent and N = 1, M1 = K1 - 1 and M2 = K2 – 1

-	If sharedCMR is absent and N = 2,

-	M1 = K1 - 2 and M2 = K2 – 2, if the two resource pairs do not share any CSI-RS resource

-	M1 = K1 - 1 and M2 = K2 – 2, if the two resource pairs share the same CSI-RS resource from the first CSI-RS resource group

-	M1 = K1 - 2 and M2 = K2 – 1, if the two resource pairs share the same CSI-RS resource from the second CSI-RS resource group

where the values of K1 and K2 are the numbers of CSI-RS resources in the first and second CSI-RS resource groups within the CSI-RS resource set respectively.

The bitwidth for RI/LI/CQI/CRI of codebookType= typeI-MultiPanel is provided in Table 6.3.1.1.2-4.

Table 6.3.1.1.2-4: RI, LI, CQI, and CRI of codebookType=typeI-MultiPanel

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| CRI |  |

where  is the number of allowed rank indicator values according to Clause 5.2.2.2.2 [6, TS 38.214],  is the value of the rank, and  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The bitwidth for RI/LI/CQI of codebookType= typeII or codebookType=typeII-PortSelection is provided in Table 6.3.1.1.2-5.

Table 6.3.1.1.2-5: RI, LI, and CQI of codebookType=typeII or typeII-PortSelection

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the number of non-zero wideband amplitude coefficients for layer |  |

where  is the number of allowed rank indicator values according to Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and  is the value of the rank. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The bitwidth for CRI, SSBRI, RSRP, differential RSRP, and CapabilityIndex are provided in Table 6.3.1.1.2-6.

Table 6.3.1.1.2-6: CRI, SSBRI, RSRP, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI |  |
| SSBRI |  |
| RSRP | 7 |
| Differential RSRP | 4 |
| CapabilityIndex | 2 |

where  is the number of CSI-RS resources in the corresponding resource set, and  is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP'.

The bitwidth for CRI, SSBRI, SINR, differential SINR, and CapabilityIndex are provided in Table 6.3.1.1.2-6A.

Table 6.3.1.1.2-6A: CRI, SSBRI, SINR, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI | log2KsCSI-RS |
| SSBRI | log2KsSSB |
| SINR | 7 |
| Differential SINR | 4 |
| CapabilityIndex | 2 |

where KsCSI-RS  is the number of CSI-RS resources in the corresponding resource set, and KsSSB is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-SINR'.

Table 6.3.1.1.2-7: Mapping order of CSI fields of one CSI report, pmi-FormatIndicator=widebandPMI and cqi-FormatIndicator=widebandCQI or reportQuantity set to 'cri-RI-CQI' and cqi-FormatIndicator=widebandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4, if reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4, if reported |
|  | Zero padding bits , if needed |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4, if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4, if reported |

The number of zero padding bits  in Table 6.3.1.1.2-7 is 0 for 1 CSI-RS port and  for more than 1 CSI-RS port, where

-	 and  is the set of rank values  that are allowed to be reported;

-	, where  is the reported rank;

-	For 2 CSI-RS ports, ;

-	For more than 2 CSI-RS ports, ;

-	if PMI is reported,  and ; otherwise, ;

-	if PMI  is reported,  is obtained according to Tables 6.3.1.1.2-1/2; otherwise, ;

-	if PMI  is reported,  is obtained according to Tables 6.3.1.1.2-1/2; otherwise, ;

-	if CQI is reported,  is obtained according to Tables 6.3.1.1.2-3/4; otherwise, ;

-	if LI is reported,  is obtained according to Tables 6.3.1.1.2-3/4; otherwise, .

Table 6.3.1.1.2-7A: Mapping order of CSI fields of one CSI report, pmi-FormatIndicator=widebandPMI, cqi-FormatIndicator=widebandCQI, csi-ReportMode= Mode 1 and numberOfSingleTRP-CSI-Mode1=0

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI as in Tables 6.3.1.1.2-3A, if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if reported |
|  | Two Layer Indicators as in Table 6.3.1.1.2-3A, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; |
|  | Zero padding bits OP, if needed |
|  | PMI wideband information fields X1, from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields X2, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields X1, from left to right as in Tables 6.3.1.1.2-1 associated with the second resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields X2, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if reported |

The number of zero padding bits OP in Table 6.3.1.1.2-7A is 0 for 1 CSI-RS port and OP=Nmax-Nreported for more than 1 CSI-RS port, where

-	Nmax=maxr∈SRankB(r)  and SRank is the set of rank combination values of r={r1,r2} that are allowed to be reported;

-	Nreported=B(R) where R is the reported rank combination;

-	For 2 CSI-RS ports, Br=NPMI(r1)+NPMI(r2)+NCQIr+NLI(r1)+NLI(r2);

-	For more than 2 CSI-RS ports, Br=NPMI,i1(r1)+NPMI,i1(r2)+NPMI,i2(r1)+NPMI,i2(r2)+NCQIr+NLI(r1)+NLI(r2) ;

-	if PMI is reported, NPMI1=2 and  NPMI2=1; otherwise, NPMI=0;

-	if PMI i1 is reported, NPMI,i1(r1) and NPMI,i1(r2) are obtained according to Tables 6.3.1.1.2-1; otherwise, NPMI,i1=0;

-	if PMI  i2 is reported, NPMI,i2(r1) and NPMI,i2(r2) are obtained according to Tables 6.3.1.1.2-1; otherwise,  NPMI,i2=0;

-	if CQI is reported, NCQIr is obtained according to Tables 6.3.1.1.2-3A; otherwise, NCQIr=0;

-	if LI is reported, NLI(r1) and NLI(r2) are obtained according to Tables 6.3.1.1.2-3A; otherwise , NLI=0.

Table 6.3.1.1.2-8: Mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP or CRI/RSRP/CapabilityIndex or SSBRI/RSRP/CapabilityIndex reporting, or mapping order of CSI fields of one report for inter-cell SSBRI/RSRP reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #3 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #4 as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #3 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #4 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #1 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #2 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #3 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #4 as in Table 6.3.1.1.2-6, if reported |

Table 6.3.1.1.2-8A: Mapping order of CSI fields of one report for CRI/SINR or SSBRI/SINR or CRI/SINR/CapabilityIndex or SSBRI/SINR/CapabilityIndex reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6A, if reported |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6A, if reported |
|  | CRI or SSBRI #3 as in Table 6.3.1.1.2-6A, if reported |
|  | CRI or SSBRI #4 as in Table 6.3.1.1.2-6A, if reported |
|  | SINR #1 as in Table 6.3.1.1.2-6A, if reported |
|  | Differential SINR #2 as in Table 6.3.1.1.2-6A, if reported |
|  | Differential SINR #3 as in Table 6.3.1.1.2-6A, if reported |
|  | Differential SINR #4 as in Table 6.3.1.1.2-6A, if reported |
|  | CapabilityIndex #1 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #2 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #3 as in Table 6.3.1.1.2-6, if reported |
|  | CapabilityIndex #4 as in Table 6.3.1.1.2-6, if reported |

Table 6.3.1.1.2-8B: Mapping order of CSI fields of one report for group-based CRI/RSRP or SSBRI/RSRP reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | Resource set indicator |
|  | CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported |
|  | RSRP of CRI or SSBRI #1 of 1st resource group as in Table 6.3.1.1.2-6 |
|  | Differential RSRP of CRI or SSBRI #2 of 1st resource group as in Table 6.3.1.1.2-6 |
|  | Differential RSRP of CRI or SSBRI #1 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #2 of 2nd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #1 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #2 of 3rd resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #1 of 4th resource group as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP of CRI or SSBRI #2 of 4th resource group as in Table 6.3.1.1.2-6, if reported |

where the 1-bit resource set indicator, with value of 0 or 1, indicates the 1st or the 2nd channel measurement resource set respectively, from which CRI or SSBRI #1 of 1st resource group is reported from; and all remaining resource groups, if reported, follow the same mapping order as the 1st resource group where CRI or SSBRI #1 of all remaining resource groups is reported from the indicated channel measurement resource set. For all reported resource groups, CRI or SSBRI #1 and CRI or SSBRI #2 are reported from different channel measurement resource sets.

Table 6.3.1.1.2-9: Mapping order of CSI fields of one CSI report, CSI part 1, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to ‘1’ are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to ‘1’ as subband 0. |  |

Table 6.3.1.1.2-9A: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3A, if reported |
|  | CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; First CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Rank Indicator associated with the first CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI associated with the first CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 1 if reported; Subband differential CQI associated with the first CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Second CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Rank Indicator associated with the second CRI as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI associated with the second CRI for the first TB as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI associated with the second CRI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to ‘1’ are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to ‘1’ as subband 0. |  |

Table 6.3.1.1.2-9B: Mapping order of CSI fields of one CSI report, CSI part 1, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; CRI as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Rank Combination Indicator as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Rank Indicator as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported; Zero padding bits OP, if needed |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Wideband CQI for the first TB as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3A, if associated with one CSI-RS resource pair and if reported; Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3B, if associated with one CSI-RS resource and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to ‘1’ are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to ‘1’ as subband 0. |  |

The number of zero padding bits OP in Table 6.3.1.1.2-9B is 0 for 1 CSI-RS port and OP=Nmax-Nreported(R) for more than 1 CSI-RS port, where

-	Nmax=maxr∈SRankN(r) . SRank is the set of rank and rank combination values r that are allowed to be reported. Nr is obtained according to Tables 6.3.1.1.2-3A/3B for rank combination indicator and rank indicator respectively.

-	Nreported R is obtained according to Tables 6.3.1.1.2-3A for rank combination indicator and R is the reported rank combination.

-	Nreported (R) is obtained according to Tables 6.3.1.1.2-3B for rank indicator and R is the reported rank.

Table 6.3.1.1.2-10: Mapping order of CSI fields of one CSI report, CSI part 2 wideband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.1.1.2-10A: Mapping order of CSI fields of one CSI report, CSI part 2 wideband, csi-ReportMode= Mode 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Two Layer Indicators as in Table 6.3.1.1.2-3A, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second resource within the CSI-RS resource pair, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the first CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the first CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Layer Indicator as in Table 6.3.1.1.2-3B, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if associated with the second CRI in CSI part 1, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if associated with the second CRI in CSI part 1, pmi-FormatIndicator= widebandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.1.1.2-10B: Mapping order of CSI fields of one CSI report, CSI part 2 wideband, csi-ReportMode= Mode 2

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | Two Layer Indicators as in Table 6.3.1.1.2-3A, if reported part 1 is associated with one CSI-RS resource pair, where the first Layer Indicator and the second Layer Indicator are associated with the first resource and the second resource within the resource pair respectively and if reported; Layer Indicator as in Table 6.3.1.1.2-3B, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the first resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the first CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1 associated with the second CSI-RS resource within the CSI-RS resource pair, if reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] associated with the second CSI-RS resource within the CSI-RS resource pair, if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, if reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

Table 6.3.1.1.2-11: Mapping order of CSI fields of one CSI report, CSI part 2 subband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |

Note:	Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand with value set to ‘1’ are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand with value set to ‘1’ as subband 0.

Table 6.3.1.1.2-11A: Mapping order of CSI fields of one CSI report, CSI part 2 subband, csi-ReportMode= Mode 1

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the first CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 1 and if reported; PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with the second CRI in CSI part 1, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second CRI in CSI part 1 according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI, numberOfSingleTRP-CSI-Mode1 = 2 and if reported |

Table 6.3.1.1.2-11B: Mapping order of CSI fields of one CSI report, CSI part 2 subband, csi-ReportMode= Mode 2

| CSI report #n Part 2 subband | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the first resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with the second resource within the CSI-RS resource pair, according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource pair and if reported |
|  | Subband differential CQI for the second TB of all even subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number associated with one CSI-RS resource, as in Tables 6.3.1.1.2-3B, if cqi-FormatIndicator=subbandCQI and reported part 1 is associated with one CSI-RS resource and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports associated with one CSI-RS resource according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and reported part 1 is associated with one CSI-RS resource and if reported |

If none of the CSI reports for transmission on a PUCCH is of two parts, the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to.

Table 6.3.1.1.2-12: Mapping order of CSI reports to UCI bit sequence , without two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1 as in Table 6.3.1.1.2-7/7A/8/8B |
|  | CSI report #2 as in Table 6.3.1.1.2-7/7A/8/8B |
|  | … |
|  | CSI report #n as in Table 6.3.1.1.2-7/7A/8/8B |

If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated,  and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-13, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to. The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-14, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to . If the length of UCI bit sequence  is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3.

Table 6.3.1.1.2-13: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1 if CSI report #1 is not of two parts, or CSI report #1, CSI part 1, if CSI report #1 is of two parts, as in Table 6.3.1.1.2-7/7A/8/8B/9/9A/9B |
|  | CSI report #2 if CSI report #2 is not of two parts, or CSI report #2, CSI part 1, if CSI report #2 is of two parts, as in Table 6.3.1.1.2-7/7A/8/8B/9/9A/9B |
|  | … |
|  | CSI report #n if CSI report #n is not of two parts, or CSI report #n, CSI part 1, if CSI report #n is of two parts, as in Table 6.3.1.1.2-7/7A/8/8B/9/9A/9B |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.1.1.2-13 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

Table 6.3.1.1.2-14: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1, CSI part 2 wideband, as in Table 6.3.1.1.2-10/10A/10B if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 wideband, as in Table 6.3.1.1.2-10/10A/10B if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 wideband, as in Table 6.3.1.1.2-10/10A/10B if CSI part 2 exists for CSI report #n |
|  | CSI report #1, CSI part 2 subband, as in Table 6.3.1.1.2-11/11A/11B if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 subband, as in Table 6.3.1.1.2-11/11A/11B if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 subband, as in Table 6.3.1.1.2-11/11A/11B if CSI part 2 exists for CSI report #n |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.1.1.2-14 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].