3GPP TSG RAN WG1 #111	R1-2212901

Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | 0383 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on UL Tx switching for PUCCH with HARQ-ACK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Apple), ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_RF_FR1_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| ` |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | In NR, for UL Tx switching, scheduling PDCCH can trigger the UL Tx switching for the PUCCH with HARQ-ACK corresponding to the scheduled PDSCH according to clause 6.1.6 in TS 38.214 If UE has reported switching period (Tswitch) to perform UL Tx switching, then the the PDSCH processing timeline (Tproc,1) timeline is not long enough for UE to perform UL Tx switching in some of the supported scheduling cases because Tswitch is not added to Tproc,1 according to clause 5.3 in TS 38.214. Therefore, UE is not expected to be scheduled in these cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify, that UE is not expected to be scheduled to transmit PUCCH with HARQ-ACK if UL Tx switching is triggered by scheduling PDCCH for PDSCH and there is not sufficient gap from the end of PDCCH to the start of PUCCH to accommodate the reported switching gap |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE is not able to perform PUCCH transmission with UL Tx switching, triggered by the scheduling PDCCH in some of the scheduling cases that are supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR has isolated impact. The CR clarifies the scheduling restriction if UL Tx switching is triggered for PUCCH with HARQ-ACK |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* Unchanged part omitted *

## 5.3	UE PDSCH processing procedure time

* Unchanged part omitted *

-	If this PUCCH resource is overlapping with another PUCCH or PUSCH resource, then HARQ-ACK is multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the HARQ-ACK message is transmitted on PUCCH.

- UE is not expected to be scheduled to transmit PUCCH carrying the HARQ-ACK information for PDSCH scheduled by a PDCCH if uplink switching gap is triggered for the PUCCH as defined in clause 6.1.6 and the first uplink symbol of the PUCCH starts earlier than the duration of {Tswitch + Tproc,1} from the last symbol of the PDCCH, where Tswitch  equals to the switching gap duration.

Otherwise the UE may not provide a valid HARQ-ACK corresponding to the scheduled PDSCH. The value of Tproc,1 is used both in the case of normal and extended cyclic prefix.

For a PDSCH that consists of two PDSCH transmission occasions in time domain in one slot, d1,1 is calculated based on the first PDSCH transmission occasion in the slot, and as described above.

* Unchanged part omitted *