3GPP TSG RAN Meeting #98-e								        	       RP-222874

Electronic Meeting, December 12-16, 2022

Status Report to TSG

Agenda item:			9.3.1.6

| WI / SI Name | NR network-controlled repeaters |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_netcon_repeater |  |  |  |  |
| Unique ID | 970079 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222673 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: N/A | Core part: 80% RAN1(85%) RAN2(70%) RAN3(80%) RAN4(15%) | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Nan Zhang |
|  | Company | ZTE |
|  | Email | Zhang.nan152@zte.com.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#110-bis-e

Agreement:

The following information can be used to characterize the physical beam(s) supported by NCR-Fwd for access link:

- Number of beams supported for access link

  - FFS: How to define the detailed value (e.g., per beam type)

  - FFS: Whether the number of beam can be derived by beam layout

- Spatial relationship between different beams

- FFS: Beam types defined by the beam width (e.g. two types as wide beam and narrow beam type)

- FFS: Beam direction defined by the boresight of beam

- FFS: Whether/How to deliver this information to gNB

- FFS: Coverage area for each beam type

- FFS: Beam ID (via explicit or implicit means)

Agreement:

The following methods are supported for access link beam indication:

- Single beam index per indication is supported to indicate one beam.

- The indication of multiple beams in one indication is supported

  - Note: The multiple beams are applied in TDMed over same frequency resource

Agreement

Confirm the WA that in access link, a DL beam and a UL beam which are correspondent with each other have the same beam index.

Agreement

The following aspects should be NCR capability:

- Simultaneous UL transmission of C-link and backhaul link

- Adaptive beam for C-link/backhaul-link

- Note-1: Fixed beam for C-link/backhaul link is default capability

- Note-2: TDMed UL transmission of C-link and backhaul link is default capability.

- FFS: How to define the capability for adaptive beam for C-link/backhaul-link

Conclusion

An NCR is not expected to perform forwarding in “OFF” state.

Agreement

For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the default behavior of the NCR-Fwd is expected to be OFF or not forwarding over these symbols

- FFS: The behavior over these symbol if dynamic DL/UL operation is supported by NCR-MT and/or NCR-Fwd.

Agreement

Following parameters should be supported to define the time resource:

- For a periodic and/or semi-persistent configured time resource, starting time, duration per beam(s) and periodicity is needed.

- For aperiodic indication of time resource, starting time and duration per beam(s) is needed.

- FFS: The SCS for starting time, duration, and periodicity

- FFS: How to define the duration, e.g., via the length of time resource or resource index(es)

- FFS: Whether indication of starting time can be implicit (e.g., the first slot after the time to apply the received beam indication and the first OFDM symbol in the slot)

Agreement

If adaptive beams are adopted for C-link and backhaul link, new signaling is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link.

- Predefined rule is used to define the beam in case there is no indication via the new signalling

  - FFS: Details of the predefined rule

  - FFS: Application of predefined rule for other cases

- Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link.

- The new signalling, if needed, is an optional NCR capability

Agreement

For the ON/OFF information indication, at least one of following options is supported to indicate the ON state of NCR

- Alt-1: Explicit indication with dedicated field to indicate ON state

  - Note: At least it’s supported when the beam indication is not applicable

- Alt-2: Implicit indication via the beam indication

- Alt-3: Indication via the time domain resource indication (i.e., the NCR is assumed to be ON over the indicated time domain resource)

Agreement:

For NCR-MT which can support adaptive beams in C link,

- Rel-15 beam indication framework can be reused.

- Rel-17 beam indication framework (i.e., the unified TCI) can be reused as well, if the NCR supports. The gNB can configure the unified TCI for the NCR-MT, if the NCR-MT supports.

Agreement:

To support CSI measurement/reporting mechanisms for NCR-MT in C-link

- The necessary legacy mechanism for receiving CSI-RS is reused for NCR-MT.

- The necessary legacy mechanism for reporting CSI is reused for NCR-MT.

- FFS: The details of the necessary mechanisms will be further discussed and decided.

- Note: this does not mean all the legacy procedures for receiving CSI-RS and reporting CSI will be supported.

Agreement:

HARQ-ACK feedback for PDSCH carrying the side control information from higher layer (e.g., MAC-CE, RRC) is supported. The legacy HARQ-ACK feedback mechanism is reused.

- FFS: Whether HARQ-ACK feedback for PDCCH carrying side control information is supported

- Note: This does not mean all legacy HARQ-ACK feedback mechanism will be supported.

Agreement:

PUCCH and PUSCH are supported for NCR-MT.

Agreement:

The TA adjustment mechanism of legacy UEs is supported for NCR-MT in C link.

Agreement:

To support the sounding procedure for NCR-MT in C link, the necessary mechanism of legacy UE sounding procedure is supported.

- FFS: The details of the necessary mechanism of legacy UE sounding procedure.

- Note: This does not mean all legacy UE sounding procedure will be supported.

RAN1#111

The response to RAN4 in R1-2212887 is endorsed. Final LS is approved in R1-2212910.

Agreement

The following is supported to deliver the information to characterize the supported physical beam of NCR-Fwd for access link:

Option-2: The information is informed to gNB and NCR via OAM

- Note-1: In this option, how to characterize the beam information is based on implementation (e.g., declaration from NCR vendor).

- Note-2: In this option, the beam(s) used by NCR-Fwd for access link is configured for gNB and NCR by OAM based on implementation.

  - The beam index in SCI corresponds to the configured beam(s) sequentially.

Agreement

For FR2, the “ON” state of NCR-Fwd is indicated:

- Alt-2: Implicit indication via the beam indication (i.e., if there is beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s))

Agreement

The semi-static beam indication for backhaul link is supported as:

- If the beam indication framework in Rel-15 is used for NCR-MT

- The DL beam is indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link

  - The UL beam is indicated by SRI on C-link via MAC CE.

- If the beam indication framework in Rel-17 is used for NCR-MT

  - The DL and UL beam are indicated by MAC CE to select one of TCI state ID from the RRC-configured list of beams for C-link

Agreement

The following pre-defined rules are applied to determine the beam for backhaul link:

- In the time domain resource with simultaneous downlink reception or uplink transmission in C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link regardless whether there is beam indicated by the dedicated signal for backhaul link.

- In the time domain resource without simultaneous downlink reception or uplink transmission in C-link and backhaul link, if the NCR does not support capability with the new signalling for backhaul beam indication or if no beam is indicated for backhaul link by the dedicated signal,

  - When Rel-15/16 beam indication framework is used for C-link,

    - The beam determined by QCL assumption for CORESET with the lowest ID and spatial relationship for PUCCH with lowest PUCCH resource ID in the C-link is applied for the DL and UL of backhaul link, respectively.

  - When Rel-17 beam indication framework (i.e., unified TCI framework) is used for C-link, the indicated unified TCI for C-link DL and UL is applied for the DL and UL of backhaul link, respectively.

  - Otherwise, the beam indicated by the dedicated signalling is applied for backhaul link.

Agreement

For FR1, the “ON” state of NCR-Fwd is indicated:

- Alt-2: Indication via the beam indication (i.e., if there is beam indication, the NCR is assumed to be ON over the indicated time domain resource associated with corresponding beam(s))

  - When there is only one beam, the sole purpose of the beam indication is for indicating “ON” state of NCR-Fwd

Conclusion

For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the dynamic DL/UL operation of NCR-MT and NCR-Fwd is not supported in Rel-18.

Agreement

For each periodic beam indication for access link, one RRC signalling is used with the information defined by the following:

Option-2:

- A list of X(1≤X≤Xmax) forwarding resource, each is defined as {Beam index, time resource}

- FFS: The value of Xmax

Each time resource is defined by {Starting slot defined as the slot offset in one period, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field.

- The periodicity is configured as part of the RRC signaling for periodic beam indication

  - The same periodicity is assumed for all time resource(s) in one periodic beam indication.

- The reference SCS is configured as part of the RRC signaling for periodic beam indication

  - The same reference SCS is assumed for all time resource(s) in one periodic beam indication.

Agreement

For each aperiodic beam indication for access link, one DCI is used with the information defined by

Option-1:

- Lmax fields are used to indicate the beam information and each field refers to one beam index ;

  - Note: The bitwidth of this field is determined by the number of beams used for access link.

- Tmax fields to indicate the time resource;

  - Note: A list of time resource is pre-defined by RRC signalling. The bitwidth of this field for time resource indication is determined by the length of list.

- FFS: The value of Tmax

  - Down-select between Tmax=1 or Tmax=Lmax.

- FFS: How to define the association between time indication and beam indication

Each time resource is defined by {Starting slot defined as the slot offset, starting symbol defined by symbol offset within the slot, duration defined by the number of symbols} with dedicated field.

Agreement

As optional functionalities for the NCR-MT, at least Rel-15 legacy BFD/BFR/RLM mechanisms are supported

- FFS: The behavior of NCR-Fwd when BFR/RLF happen in C link.

Agreement

- Legacy Rel-15 initial/random access procedure is supported for NCR-MTs in C link.

  - Note: No additional enhancement is necessary from RAN1 point of view.

- The CRC bits of the PDCCHs carrying side control information are scrambled by a new dedicated RNTI

  - Applicable only for NCR-MT

2.1.2	Remaining Open issues

Remaining details on signalling design for beam indication

RRC/MAC CE parameter lists.

Other issues including power control (pending on the decision on RAN#98-e).

2.2	RAN2

2.2.1	Agreements

RAN2#119-bis-e

Agreement:

RAN2 confirms to use RRC signalling to configure NCR-MT to receive side control information. How the side control information itself is transmitted (i.e. via RRC or DCI or MAC CE) is up to RAN1 (RAN2 may discussion the initial RAN1 decision and revisit if needed).

Agreement:

NCR-MT supports RRC_CONNECTED and RRC_IDLE states, FFS on RRC_INACTIVE state (e.g. optional support or not support).

Agreement

NCR-MT supports SRB0/1/2 and DRB is optional. FFS on maximum number of DRBs.

Agreements

RRM functions supported by NCR-MR:

- Cell selection is mandatory

- Cell reselection, RLM, BFD, BFR are FFS

RAN2#120

Agreements

Introduce an NCR-support indication in SIB1 per PLMN; whether it is also per NPN is FFS

RRC_INACTIVE is optionally supported without any specific enhancements

Agreements

gNB cell that NCR-Fwd is forwarding is the same cell the NCT-MT is connected to. Whether the NCR-Fwd can forward other cells is up to implementation

NCR-MT indicates the maximum number of supported DRB in UE capability, values {1, 16}. If absent, the NCR-MT does not support DRB.

SRB2 is mandatory feature for NCR-MT.

On NCR-Fwd ON/OFF:

- When NCR-MT is in RRC_CONNECTED mode, the NCR-Fwd can be ON or OFF following the side control information received from the gNB.

- After NCR-MT enters RRC_INACTIVE mode, the NCR-Fwd can be ON or OFF following the last configuration received from the gNB.

- Release to RRC-IDLE is FFS.

On NCR-MT RLF:

- After RLF is declared by NCR-MT, NCR-MT performs cell selection and trigger RRC re-establishment;

- If NCR-MT enters RRC_IDLE due to no suitable cell is find, NCR-Fwd is OFF;

- During RRC re-establishment procedure, NCR-Fwd is OFF.

NCR-MT mandatorily support cell reselection and RRM measurements in RRC_IDLE and RRC_INACTIVE.

In Rel-18, NCR-MT does not support handover and RRM measurements in RRC_CONNECTED.

For reporting the capabilities of NCR-MT, the existing UECapabilityEnquiry and UECapabilityInformation messages are

reused.

In NCR-MT capability discussion, to focus on mandatory features that are required for NCR-MT.

All existing optional features are considered as applicable to NCR-MT unless explicitly excluded (Same as IAB-MT). FFS on

taking IAB specified features as a baseline for future discussion.

NPN capable NCR-MT should consider cellReservedForOtherUse for determination of an NPN-only cell.

2.2.2	Remaining Open issues

NCR-MT capability and feature list

RRC/MAC CE design according to the progress, e.g., RAN1’s inputs

Potential signalling design for NCR management

Whether NCR_MT can be released to RRC_IDLE and the NCR_Fwd state when NCR_MT is in RRC_IDLE.

2.3	RAN3

2.3.1	Agreements

RAN3#117-bis-e

The NCR authorization indicator is provided from AMF to gNB explicitly over the NG interface.

It can be applied to any solution selected.

The discussion on RAN impact on validation function is pending to SA3 reply LS.

gNB-CU knows whether the connected gNB-DU supports NCR based on OAM configuration.

Down selection on all solutions which takes the feedback from SA3 and SA5 into account can be discussed in next RAN3

meeting.

The NCR-OAM connectivity requirement should be supported, further details can be discussed.

RAN3#118

Exclude the solution 2.

OAM-NCR connectivity can be provided via PDU session.

Rev in R3-226926 Endorsed as BL CR

gNB-DU needs to know the authorization status of NCR.

Take Solution 3 as the basis for NCR management. FFS on any additional aspects.

The NCR may be configured with a list of allowed and/or forbidden cells.

2.3.2	Remaining Open issues

Additional aspects for NCR management.

The NCR may be configured with a list of allowed and/or forbidden cells.

Discussion on further stage 2 related aspects agreed by RAN1.

Whether the needs for gNB-CU or gNB-DU to configure which cell(s) can be used for NCR device accessing.

2.4	RAN4

2.4.1	Agreements

RAN4#105:

RF and system parameters

Issue 1-1:  Beamforming capability for FR1 NCR

- Agreement:

  - Beamforming capability can be supported for FR1 NCR from RAN4 perspective

Issue 1-2:  System parameter for NCR-MT

  - Proposal 1 agreed

Issue 1-3:  NCR-MT feature list

- Agreement: Postpone the discussion on NCR-MT feature list into future RAN4 meetings.

Issue 2-2-1  NCR-MT UL assumption

- Agreement:

  - NCR-MT uplink transmission is supported

Issue 2-2-2  RF requirements for NCR-MT transmitter requirements

- Agreement: For NCR-MT transmitter:

  - Baseline assumption: Legacy uplink transmitter requirements from Rel-17 RF repeater still be applicable

    - Further discuss the power control requirements for NCR-MT

    - Update on other requirements not precluded

  - For NCR Tx direction requirements, reuse the concept of OTA coverage range which also include OTA peak directions sets.

Issue 2-3  BC for NCR

- Agreement:

  - Taking IAB-MT approach as starting point, and further discuss how to address different BC capabilities.

Issue 2-4  Rx requirements for NCR-MT

- Agreement: Pro

  - Proposal 1: For NCR-MT Rx direction requirements, it’s suggested to specify following OTA Rx direction declarations. These declarations apply for both input signal and interference signal.

- A declared receiver target redirection range. This range is not needed if NCR don’t support redirection.

- [five] declared sensitivity RoAoA comprising the conformance testing directions. If don’t support redirection, only one RoAoA is suggested

- Receiver target reference direction

- Min EIS may only need for MT part

  - Proposal 3: for the RF requirement of NCR-MT receiver, to define the refense requirement for NCR-MT receiver.

RRM part:

Issue 1-1: Cell selection requirement for NCR-MT

- Agreements

  - Do not define RRM requirement for cell selection of NCR-MT

Issue 2-1: transmit timing requirement for NCR-MT

- Agreement

  - Further discuss and define, if necessary, NCR-MT timing requirements based on further RAN1/RAN2 progress

  - Candidate requirements include

    - NCR transmit timing (Te and gradual timing adjustment)

    - NCR timer accuracy

    - Timing advance

Issue 3-1: adaptive beamforming for NCR-MT

- Agreements

  - Further discuss and define, if necessary, NCR-MT beam management related requirements based on further RAN1/RAN2 progress.

  - Candidate requirements include

    - Active TCI state switching delay

    - Uplink spatial relation switch delay

    - Active downlink TCI switching delay for unified TCI

    - Active uplink TCI switching delay for unified TCI

    - L1-RSRP measurement and reporting

  - FFS on requirements applicability to different NCR classes

Issue 4-1: cell phase synchronization for NCR

- Agreements

  - Do not define RRM requirements on cell phase synchronization accuracy for NCR nodes

RAN4#104bis-e

Issue 1-1: work plan for NCR in Rel-18

Agreements:

- Endorse the work plan for RF part;

- To remove the demod part from the work plan at the current stage.

| WG | Meeting Number | TU | Task |
| --- | --- | --- | --- |
| RAN4 | #104bis | Core RF 0.25 | - Identify and specify the RF requirements for NCR |
|  |  | Core RD 0.5 |  |
| RAN4 | #105 | Core RF 0.25 | - Specify the RF requirements for NCR |
|  |  | Core RD 0.5 |  |
| RAN4 | #106 | Core RF 0.25 | - Specify the RF requirements for NCR - Discussion on the work split to draft the NCR RF requirements |
|  |  | Core RD 0.5 |  |
| RAN4 | #106bis | Core RF 0.25 | - Specify RF requirements for NCR and endorse the CR if possible. - Identify and specify the EMC core requirements for NCR |
|  |  | Perf RF 0.25 | - Initialize the discussion for the conformance testing of RF core requirements of NCR |
|  |  | CoreRD 0.25 |  |
|  |  | Perf RD 0.25 |  |
| RAN4 | #107 | Core RF 0.25 | - Specify RF requirements for NCR and endorse the CR if possible. - The maintenance of RF requirements for NCR if necessary - Specify the EMC core requirement for NCR |
|  |  | Perf RF 0.25 | - Discuss the conformance testing of RF core requirements of NCR |
|  |  | CoreRD 0.25 |  |
|  |  | Perf RD 0.25 |  |
| RAN4 | #108 | Core RF 0.25 | - Specify RF requirements for NCR and endorse the CR if possible - The maintenance of RF requirements for NCR if necessary - Specify EMC core requirements for NCR and discuss the work split to draft the NCR RF requirements |
|  |  | Perf RF 0.25 | - Discuss the conformance testing of RF core requirements of NCR |
|  |  | CoreRD 0.5 |  |
|  |  | Perf RD 0.25 |  |
| RAN4 | #108bis | Core RF 0.25 | - Specify RF requirements for NCR and endorse the CR if possible - The maintenance of RF requirements for NCR if necessary - Specify EMC requirements for NCR and endorse the CR if possible |
|  |  | Perf RF 0.25 | - Discuss the conformance testing of RF core requirements of NCR |
|  |  | CoreRD 0.5 |  |
|  |  | Perf RD 0.5 |  |
| RAN4 | #109 | Core RF 0.25 | - Specify RF requirements for NCR and approve the big CR for NCR - The maintenance of EMC core requirements for NCR if necessary |
|  |  | Perf RF 0.25 | - Discuss the conformance testing of RF core requirement of NCR - Initialize the discussion for the EMC performance requirements of NCR |
|  |  | CoreRD 0.5 |  |
|  |  | Perf RD 0.5 |  |
| RAN4 | #110 | Pref RF 0.25 | - Discuss the conformance testing of RF core requirement of NCR - Prepare Draft CR for NCR conformance testing - Continue to discuss the EMC performance requirements of NCR |
|  |  | Pref RD 0.5 |  |
| RAN4 | #110bis | Pref RF 0.25 | - Discuss the conformance testing of RF core requirement of NCR - Endorse Draft CR for NCR conformance testing - Continue to discuss the EMC performance requirements of NCR |
|  |  | Pref RD 0.5 |  |
| RAN4 | #111 | Pref RF 0.25 | - Discuss the conformance testing of RF core requirement of NCR - Approve the big CR for NCR conformance testing. - Approve the CR for NCR EMC performance requirements |
|  |  | Pref RD 0.5 |  |

Issue 1-2-1:  spec drafting for NCR RF and EMC requirements

Agreements:

- Capture RF core requirement into TS 38.106 and capture the RF perf requirement and EMC core/perf requirements into TS38.114.

RF part:

Issue 2-1: RF architecture for NCR in Rel-18

- Agreements: To have generic diagram for NCR if necessary;

Issue 2-2 NCR type and NCR class:

- Aagreements:

- For NCR-fwd link in Rel-18, at least reuse the Rel-17 repeater type/Class i.e.,

  - Repeater Type: 1-C and 2-O;

  - Repeater class: wide-area and local area (DL, UL), medium range for DL

- FFS for repeater type 1-H and 1-O for FR1 repeater given the beamforming capability assumption;

- The baseline assumption that same type/class applied for NCR-fwd link and NCR-MT

Issue 2-3 NCR-MT UL assumption

- Agreement: To check with RAN1 on NCR-MT UL transmission to progress the further work in RAN4

Issue 2-4-1 Dynamic beamforming.

- Agreement:

  - no new requirement on top of Rel-17 needed as starting point.

  - Agree on directions (as indicated by CMCC R4-2215488) and declaration framework (as indicated by ZTE R4-2216553) as starting point, further details could be discussed further.

Issue 2-4-3  ON-OFF information:

- Agreement:

  - For repeater operating in TDD bands, to reuse the existing ON-OFF transition time and ON-OFF power requirement;

  - For repeater operating in FDD bands, to reuse the existing ON-OFF transition time and ON-OFF power requirement;

  - FFS for drafting the related ON/OFF signalling in the RAN4 spec if necessary.

Issue 2-4-4  BC for NCR-Fwd

- Agreement

  - To send the LS to check with RAN1 on the BC capability for NCR;

Issue 2-4-5  multi-band operation for NCR-fwd

- Agreement: no additional requirements are needed on top of existing Rel-17 repeater requirements for multi-band NCR-fwd.

Issue 2-5-1  Rx requirements for NCR-MT

- Agreement: Proposal 1 agreed as starting point pending on the further analysis of the test feasibility

Issue 2-5-2 Tx requirements for NCR-MT

- Agreement: Postpone the discussion on Tx requirements for NCR-MT until RAN1 response received.

RRM part:

Topic #1: General principle for NCR-MT RRM work

Issue 1-1: general principle for NCR-MT RRM work

Agreements:

Agreement:

- The RAN4 study on RRM requirement for RRM procedure should start after that procedure has been agreed by RAN1 and/or RAN2.

- RAN4 starts study on the requirements which are not depending on RAN1/2 conclusions.

Issue 1-2: How to draft the spec for NCR-MT if necessary

Agreement:

- Unanimous agreement on that 38.106 would be the container of RRM requirements for NCT if identified.

Topic #2: work plan for NCR-MT RRM work

To follow the following high level principle,

Agreement:

- The RAN4 study on RRM requirement for RRM procedure should start after that procedure has been agreed by RAN1 and/or RAN2.

- RAN4 starts study on the requirements which are not depending on RAN1/2 conclusions.

For requirements which are not depending RAN1/RAN2 conclusion, RAN4 could follow the work plan as below to continue the work.

For RRM requirement which depends on RAN1/RAN2 RRM procedure, this could be start the RRM requirement after its conclusion in other WG.

| WG | Meeting Number | TU | Task |
| --- | --- | --- | --- |
| RAN4 | #104bis |  |  |
|  |  | Core RD 0.5 | - Identify the necessity of the RRM requirements for NCR-MT and its scope of RRM requirements for NCR-MT if necessary. |
| RAN4 | #105 |  |  |
|  |  | Core RD 0.5 | - If an RRM requirement is considered necessary based on the study then specify the corresponding RRM requirement for NCR-MT. |
| RAN4 | #106 | Core RF 0.25 |  |
|  |  | Core RD 0.5 | - If an RRM requirement is considered necessary based on the study then specify the corresponding RRM requirement for NCR-MT. |
| RAN4 | #106bis | Core RF 0.25 |  |
|  |  | Perf RF 0.25 |  |
|  |  | CoreRD 0.25 | - If an RRM requirement is considered necessary based on the study then specify the corresponding RRM requirement for NCR-MT. |
|  |  | Perf RD 0.25 |  |
| RAN4 | #107 | Core RF 0.25 |  |
|  |  | Perf RF 0.25 |  |
|  |  | CoreRD 0.25 | - If an RRM requirement is considered necessary based on the study then specify the corresponding RRM requirement for NCR-MT. - Endorse the CR if the RRM requirement for NCR-MT is to be specified, if possible. |
|  |  | Perf RD 0.25 |  |
| RAN4 | #108 | Core RF 0.25 |  |
|  |  | Perf RF 0.25 |  |
|  |  | CoreRD 0.5 | - If an RRM requirement is considered necessary based on the study then specify the corresponding RRM requirement for NCR-MT. Endorse the CR if the RRM requirement for NCR-MT is to be specified. |
|  |  | Perf RD 0.25 |  |
| RAN4 | #108bis | Core RF 0.25 |  |
|  |  | Perf RF 0.25 |  |
|  |  | CoreRD 0.5 | - If an RRM requirement is considered necessary based on the study then specify the corresponding RRM requirement for NCR-MT. - Endorse the CR if the RRM requirement for NCR-MT is to be specified. |
|  |  | Perf RD 0.5 |  |
| RAN4 | #109 | Core RF 0.25 |  |
|  |  | Perf RF 0.25 |  |
|  |  | CoreRD 0.5 | - Endorse the CR if the RRM requirement for NCR-MT is to be specified - Agree on the big CR if the RRM requirement for NCR-MT is to be specified. |
|  |  | Perf RD 0.5 |  |
| RAN4 | #110 | Pref RF 0.25 |  |
|  |  | Pref RD 0.5 |  |
| RAN4 | #110bis | Pref RF 0.25 |  |
|  |  | Pref RD 0.5 |  |
| RAN4 | #111 | Pref RF 0.25 |  |
|  |  | Pref RD 0.5 |  |

2.4.2	Remaining Open issues

NCR-MT feature list

The details of system parameters for NCR-MT

NCR RF requirements e.g. NCR-MT Tx, Rx requirement, Beam correspondence, beamforming for FR1 NCR etc

NCR RRM requirement except for cell selection and cell phase synchronization

NCR-MT feature list

The details of system parameters for NCR-MT

NCR RF requirements e.g. NCR-MT Tx, Rx requirement, Beam correspondence, beamforming for FR1 NCR etc

NCR RRM requirement except for cell selection and cell phase synchronization

Performance part.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

R1-2208314 	TR 38.867 v0.2.0 for Study on NR network-controlled repeaters

R1-2208690	Work plan for NR network-controlled repeaters	ZTE

R1-2208422	Side control information and behaviors for network-controlled repeaters	Huawei, HiSilicon

R1-2208563	Discussion on side control information and NCR behavior	Spreadtrum Communications

R1-2208656	Discussion on side control information and NCR behavior	vivo

R1-2208684	Discussions on side control information for NR network-controlled repeaters	InterDigital, Inc.

R1-2208691	Discussion on side control information for NCR	ZTE

R1-2208778	Discussion on side control information and NCR behavior	China Telecom

R1-2208989	Discussion on side control information and NCR behavior	CATT

R1-2209018	Discussion on side control information and NCR behavior	Fujitsu

R1-2209065	Discussion on Side control information and NCR behaviour	Intel Corporation

R1-2209086	Discussion on side control information and NCR behavior	Nokia, Nokia Shanghai Bell

R1-2209089	Signalling support for NR network-controlled repeaters	AT&T, FirstNet

R1-2209110	Considerations on side control information and NCR behaviour	Sony

R1-2209154	Discussion on side control information to enable NR network-controlled repeaters	NEC

R1-2209238	Discussion on side control information and NCR behavior	CAICT

R1-2209297	Discussion on side control information and NCR behavior	xiaomi

R1-2209350	Discussion on side control information and NCR behavior	CMCC

R1-2209378	Discussion on side control information and NCR behavior	Panasonic

R1-2209408	Discussion on side control information for network-controlled repeater	ETRI

R1-2209511	Side control information and NCR behavior	MediaTek Inc.

R1-2209593	On side control information and NCR behavior	Apple

R1-2209664	Side control information and configuration for NCR behavior	Sharp

R1-2209744	Discussion on side control information and NCR behavior	Samsung

R1-2209763	Discussion on UL-DL timing alignment in NCR-Fwd	Rakuten Mobile, Inc

R1-2209811	Discussion on side control information and NCR behavior	LG Electronics

R1-2209915	Discussion on Side control information and NCR behavior	NTT DOCOMO, INC.

R1-2209998	On side control information and NCR behavior	Qualcomm Incorporated

R1-2210022	Discussion on side control information and NCR behaviour	Lenovo

R1-2210068	Discussion on side control information and NCR behavior	KDDI Corporation

R1-2210114	Discussion on signaling side control information and NCR behavior	CEWiT, IITK

R1-2210231	Side-control information and behavior in NCR	Ericsson

R1-2210343	Summary#2 of discussion on side control information	Moderator (ZTE)

R1-2210483	Summary#3 of discussion on side control information	Moderator (ZTE)

R1-2210484	Summary#4 of discussion on side control information	Moderator (ZTE)

R1-2208423	Configuration for side control information signalling of network-controlled repeaters	Huawei, HiSilicon

R1-2208657	Discussion on other aspects of NCR	vivo

R1-2208685	L1/L2 signaling for side control information of network-controlled repeaters	InterDigital, Inc.

R1-2208692	Discussion on other aspects for NCR	ZTE

R1-2208990	Discussion on control plane signalling and procedures for NR network-controlled repeaters	CATT

R1-2209019	Discussion on control plane signaling and procedures relevant to RAN1	Fujitsu

R1-2209066	Discussions on L1/L2 signaling and configuration for side control information	Intel Corporation

R1-2209087	"Discussion on other aspects including control plane signalling and procedures relevant to RAN1"	Nokia, Nokia Shanghai Bell

R1-2209155	Discussion on other aspects for network-controlled repeaters	NEC

R1-2209298	Discussion on other aspects of NCR	xiaomi

R1-2209351	Discussion on other aspects including control plane signalling and procedures relevant to RAN1	CMCC

R1-2209409	Discussion on NCR signaling and procedures	ETRI

R1-2209594	On control plane signaling and procedures for NCR	Apple

R1-2209745	On signaling and procedures for network-controlled repeaters	Samsung

R1-2209812	Discussion on other aspects for NCR	LG Electronics

R1-2209916	Other aspects including control plane signalling and procedures relevant to RAN1	NTT DOCOMO, INC.

R1-2209999	On other NCR aspects	Qualcomm Incorporated

R1-2210036	Discussion on signalling of side control information for network-controlled repeaters	Lenovo

R1-2210232	Signaling, procedures and other aspects in NCR	Ericsson

R1-2210282	Summary#1 on other aspects	Moderator (Fujitsu)

R1-2210463	Summary#2 on other aspects	Moderator (Fujitsu)

R1-2210818	LS to RAN1 on NCR-MT transmission and Beam correspondence	RAN4, CMCC

R1-2212886	Summary#1 of discussion on the reply LS to RAN4	Moderator (CMCC)

R1-2212887	[Draft] Reply LS to RAN4 on NCR-MT transmission and Beam correspondence	Moderator (CMCC)

R1-2210882	Side control information and behaviors for network-controlled repeaters	Huawei, HiSilicon

R1-2211020	Discussion on side control information to enable NR network-controlled repeaters	vivo

R1-2211080	Discussion on side control information and NCR behavior	Fujitsu

R1-2211107	Discussion on side control information for NCR	ZTE

R1-2211211	Discussion on side control information and NCR behavior	CATT

R1-2211243	Discussion on side control information and NCR behavior	Spreadtrum Communications

R1-2211303	Discussion on side control information and NCR behavior	Nokia, Nokia Shanghai Bell

R1-2211317	Discussions on side control information and NCR behavior	InterDigital, Inc.

R1-2211374	Discussion on side control information and NCR behavior	xiaomi

R1-2211412	Discussion on Side control information to enable NR network-controlled repeater	Intel Corporation

R1-2211533	Discussion on side control information and NCR behaviour	China Telecom

R1-2211564	Discussion on side control information for network-controlled repeater	ETRI

R1-2211602	Discussion on side control information and NCR behavior	Panasonic

R1-2211621	Aspects of side control information and NCR behavior	Sony

R1-2211693	Discussion on side control information and NCR behavior	CMCC

R1-2211822	On side control information and NCR behavior	Apple

R1-2211877	Side information on NCR beams and forwarding behaviors	Sharp

R1-2211882	Discussion on side control information and NCR behaviour	Lenovo

R1-2211927	Discussion on side control information and NCR behavior	LG Electronics

R1-2211935	Signalling support for NR network-controlled repeaters	AT&T

R1-2211996	Discussion on Side control information and NCR behavior	NTT DOCOMO, INC.

R1-2212058	Discussion on side control information and NCR behavior	Samsung

R1-2212130	On side control information and NCR behavior	Qualcomm Incorporated

R1-2212244	Side control information and NCR behavior	MediaTek Inc.

R1-2212286	Discussion on side control information and NCR behavior	KDDI Corporation

R1-2212291	Discussion on side control information and NCR behaviour	KT Corp.

R1-2212314	Discussion on side control information for NCR	Rakuten Mobile, Inc

R1-2212323	Discussion on side control information and NCR behavior	NICT

R1-2212344	Side-control information and NCR behavior	Ericsson

R1-2212369	Discussion on side control information and NCR behaviour	NEC

R1-2212430	Discussion on signaling side control information and NCR behavior	CEWiT, IITK

R1-2212590	Summary#1 of discussion on side control information	Moderator (ZTE)

R1-2212605	Summary#2 of discussion on side control information	Moderator (ZTE)

R1-2212785	Summary#3 of discussion on side control information	Moderator (ZTE)

R1-2210883	Other aspects of network-controlled repeaters	Huawei, HiSilicon

R1-2211021	Discussion on signaling for side control information	vivo

R1-2211081	Discussion on control plane signaling and procedures relevant to RAN1	Fujitsu

R1-2211108	Discussion on other aspects for NCR	ZTE

R1-2211212	Discussion on  control plane signalling and procedures for NR network-controlled repeaters	CATT

R1-2211304	Discussion on other aspects including control plane signalling and procedures relevant to RAN1	Nokia, Nokia Shanghai Bell

R1-2211318	Discussion on control plane signaling and procedures for network-controlled repeaters	InterDigital, Inc.

R1-2211375	Discussion on other aspects of NCR	xiaomi

R1-2211413	Discussions on L1/L2 signaling and configuration for side control information	Intel Corporation

R1-2211511	Discussion on other aspects including control plane signalling and procedures for NCR	IIT-K, CEWiT

R1-2211565	Discussion on NCR signaling and procedures	ETRI

R1-2211622	Aspects of signaling and control procedures relevant to RAN1	Sony

R1-2211632	Discussion on NCR control plane signalling and procedures	Panasonic

R1-2211694	Discussion on other aspects including control plane signalling and procedures relevant to RAN1	CMCC

R1-2211763	Discussion on NCR control information signalling and procedures	Lenovo

R1-2211823	On control plane signaling and procedures for NCR	Apple

R1-2211918	Discussion on other aspects of NCR	CAICT

R1-2211928	Discussion on other aspects for NCR	LG Electronics

R1-2211997	Other aspects including control plane signalling and procedures relevant to RAN1	NTT DOCOMO, INC.

R1-2212059	On signaling and procedures for network-controlled repeaters	Samsung

R1-2212131	On other NCR aspects	Qualcomm Incorporated

R1-2212319	Discussion on other aspects for NCR	Rakuten Mobile, Inc

R1-2212345	Other aspects including control signaling and procedures in NCR	Ericsson

R1-2212370	Discussion on other aspects for network-controlled repeaters	NEC

R1-2212552	Summary#1 on other aspects	Moderator (Fujitsu)

R1-2212705	Summary#2 on other aspects	Moderator (Fujitsu)

R2-2209328	LS on NCR Solutions (R3-225253; contact: ZTE)	RAN3	LS in	Rel-18	FS_NR_netcon_repeater	To:SA3, SA5	Cc:RAN2, SA2

R2-2209329	Progress on NCR identification and authorization (R3-225254; contact: ZTE)	RAN3	LS in	Rel-18	FS_NR_netcon_repeater	To:RAN1	Cc:RAN2

R2-2210294	Work plan for Network-controlled repeaters	ZTE Corporation, Sanechips	Work Plan	Rel-18	FS_NR_netcon_repeater

R2-2210920	Report of [AT119bis-e][NCR] NCR open issues (ZTE)

R2-2210155	Discussion on signalling for side control information	CMCC	discussion	Rel-18	NR_netcon_repeater

R2-2210279	Signalling for NCR side control information	MediaTek Inc.	discussion	Rel-18

R2-2210334	Discussion on RAN2 topics for NCR	Ericsson	discussion	Rel-18	FS_NR_netcon_repeater

R2-2210295	Consideration on NCR signalling and RRM functions	ZTE Corporation, Sanechips	discussion	Rel-18	FS_NR_netcon_repeater

R2-2210454	Discussion on NCR capability framework	Philips International B.V.	discussion	Rel-18	FS_NR_netcon_repeater

R2-2209367	Signaling for side control information and RRM functions	CATT	discussion	Rel-18	FS_NR_netcon_repeater

R2-2209630	Discussion on C-plane aspects for NCR-MT	Fujitsu	discussion	Rel-18	NR_netcon_repeater

R2-2209639	Signalling of side control information for NCR	Intel Corporation	discussion	Rel-18	NR_netcon_repeater

R2-2209667	Discussion on NCR configuration signaling and RRM functions	Huawei, HiSilicon	discussion	Rel-18	NR_netcon_repeater

R2-2209680	NCR side control signalling and other RRC and RRM aspects	Nokia, Nokia Shanghai Bell	discussion	NR_netcon_repeater

R2-2209697	Signalling for side control information to support NR network-controlled repeaters	AT&T, FirstNet	discussion

R2-2209705	Configuration of signaling for side control information	Qualcomm Inc.	discussion	Rel-18	FS_NR_netcon_repeater

R2-2209773	Discussion on Signaling for Side Control Information	Apple	discussion	Rel-18	DUMMY

R2-2209933	Discussion on Signaling and procedures for side control information	Lenovo	discussion	Rel-18

R2-2210135	Control plane signaling and procedures of network-controlled repeater	NEC Corporation	discussion

R2-2210155	Discussion on signalling for side control information	CMCC	discussion	Rel-18	NR_netcon_repeater

R2-2210200	Network-controlled repeaters - key issues	Samsung R&D Institute UK	discussion

R2-2210207	Considerations on NCR fwd link config	Sony	discussion	Rel-18	NR_netcon_repeater

R2-2210386	Discussion on NCR Related Procedures	vivo	discussion	Rel-18

R2-2210431	Consideration of network-controlled repeaters 	Kyocera 	discussion	Rel-18	R2-2208293

R2-2210563	Discussion on RAN2 issues for NCR	LG Electronics	discussion	Rel-18	NR_netcon_repeater

R2-2210572	On RAN2 impact of Network-Controlled Repeaters	China Telecom Corporation Ltd.	discussion

R2-2209706	Management of Network-Controlled Repeater	Qualcomm Inc.	discussion	Rel-18	FS_NR_netcon_repeater

R2-2211173	Reply LS on NCR Solutions (S3-223080; contact: ZTE)	SA3	LS in	Rel-18	FS_NR_netcon_repeater	To:RAN3	Cc:RAN2, SA2, SA5

R2-2211198	Further discussion on the necessary aspects to support NCR	Huawei, HiSilicon	discussion	Rel-18	NR_netcon_repeater

R2-2211376	Discussion on NCR Functionality and UE Capability	Intel Corporation	discussion	Rel-18	NR_netcon_repeater

R2-2211474	Configuration of Network-controlled Repeater	Qualcomm Inc.	discussion	Rel-18	NR_netcon_repeater

R2-2211521	NCR-MT RRM functions	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_netcon_repeater

R2-2211695	Discussion on Signaling for side control information	Apple	discussion	DUMMY

R2-2211802	Discussion on Signaling for Side Control Information	vivo	discussion	Rel-18

R2-2211857	Discussion on state transition for NCR-MT	Fujitsu	discussion	Rel-18	NR_netcon_repeater

R2-2211908	Consideration on NCR open issues	ZTE Corporation, Sanechips	discussion	Rel-18	NR_netcon_repeater	Late

R2-2211915	Considerations on side control information	Sony	discussion	Rel-18	NR_netcon_repeater

R2-2211976	On NCR Features supported	Samsung R&D Institute UK	discussion	Rel-18	NR_netcon_repeater

R2-2212017	Discussion on open issues for NCR-MT	Lenovo	discussion	Rel-18

R2-2212143	Signaling for side control information and RRM functions	CATT	discussion	Rel-18	FS_NR_netcon_repeater

R2-2212309	Signalling for NCR side control information	MediaTek Inc.	discussion	Rel-18

R2-2212492	Discussion on signalling aspects for NCR	Ericsson	discussion	Rel-18	NR_netcon_repeater

R2-2212498	Considerations on NCR remaining issues	NEC Corporation	discussion

R2-2212525	Further consideration of network-controlled repeaters 	Kyocera 	discussion	Rel-18

R2-2212621	Discussion on signaling for side control information	CMCC	discussion	Rel-18	FS_NR_netcon_repeater

R2-2212731	RLM/RRM support for NR network-controlled repeaters	AT&T	discussion

R2-2212791	Discussion on signalling for side control information	China Telecom 	discussion

R2-2212920	Resolving open issues for NCR.	LG Electronics	discussion	Rel-18

R2-2211199	Way forwad for NCR management	Huawei, HiSilicon	discussion	Rel-18	NR_netcon_repeater

R2-2211377	Authorization and verification of NCR: RAN2 impact	Intel Corporation	discussion	Rel-18	NR_netcon_repeater

R2-2211475	Management of Network-controlled Repeater	Qualcomm Inc.	discussion	Rel-18	NR_netcon_repeater

R2-2211522	Down-selection of NCR management solutions	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_netcon_repeater

R2-2211696	Discussion on NCR repeater management	Apple	discussion	DUMMY

R2-2211803	Discussion on NCR Management	vivo	discussion	Rel-18

R2-2211858	Discussion on NCR management	Fujitsu	discussion	Rel-18	NR_netcon_repeater

R2-2211881	Repeater management	Samsung R&D Institute UK	discussion

R2-2211909	Consideration on NCR management	ZTE Corporation, Sanechips	discussion	Rel-18	NR_netcon_repeater

R2-2211916	Clarifications about NCR management solutions based on SA3 reply	Sony	discussion	Rel-18	NR_netcon_repeater

R2-2212018	Discussion on repeater management for NCR-MT	Lenovo	discussion	Rel-18

R2-2212144	Management of Network-Controlled Repeater	CATT	discussion	Rel-18	FS_NR_netcon_repeater

R2-2212493	Discussion on capabilities and remaining issues for NCR	Ericsson	discussion	Rel-18	NR_netcon_repeater

R2-2212497	Down-selection of NCR management solutions	NEC Corporation	discussion

R2-2212499	Management of Network-controlled repeater	Philips International B.V.	discussion	Rel-18

R2-2212609	Discussion on NCR management	Rakuten Mobile, Inc	discussion	Rel-18

R2-2212622	Discussion on the network-controlled repeater management	CMCC	discussion	Rel-18	FS_NR_netcon_repeater

R2-2212793	Discussion on identification and authorization of Network-Controlled Repeaters	China Telecom	discussion

R2-2212853	NCR management	MediaTek Inc.	discussion	Rel-18

R3-225869 Work plan for NR network-controlled repeaters (ZTE)

R3-225362 RAN3 Aspects of Network-Controlled Repeater (Qualcomm Inc.)

R3-225463 Discussion on Identification and authorization of Network Controlled Repeater (Nokia, Nokia Shanghai Bell)

R3-225577 Authorization for Network-Controlled Repeaters (Ericsson LM)

R3-225578 Network-Controlled Repeaters Authorization - Solution 4 (Ericsson LM)

R3-225626 Discussion on NCR management (CATT)

R3-225627 CR for TS 38.413 Support NCR authorization (CATT)

R3-225651 Authorization for Network Controlled Repeater (Samsung)

R3-225666 (TPs to TS 38.300, TS 38.413 BL CRs) Support of Network-Controlled Repeater (Huawei)

R3-225667 (TPs to TS 38.401, TS 38.473 BL CRs) Support of Network-Controlled Repeater in disaggregated architecture (Huawei)

R3-225759 Authorization for NR network-controlled repeaters (AT&T)

R3-226174 Reply LS on NCR Solutions (SA3)

R3-226749 Discussion on NCR management (ZTE)

R3-226751 [Draft] Reply LS on clarification for NCR validation (ZTE)

R3-226684 Why RAN validation is necessary (Intel Corporation)

R3-226685 [Draft] Reply LS on NCR validation (Intel Corporation)

R3-226701 Discussion on the network-controlled repeater management (CMCC)

R3-226571 Discussion on NCR management (CATT)

R3-226572 CR for TS 38.413 Support NCR authorization (CATT)

R3-226405 Network-Controlled Repeater Authorization and Management (Ericsson LM)

R3-226406 Network-Controlled Repeaters Authorization - Solution 4 (Ericsson LM)

R3-226319 Discussion on Identification and authorization of Network Controlled Repeater (Nokia, Nokia Shanghai Bell)

R3-226320 Stage-2 TP for NCR (Nokia, Nokia Shanghai Bell)

R3-226270 Management of Network-Controlled Repeater (Qualcomm Inc.)

R3-226375 Down-selection of NCR management solutions (NEC)

R3-226574 Supporting NCR Device-OAM connectivity (Samsung)

R3-226648 Authorization for NR network-controlled repeaters (AT&T)

R3-226687 Discussion on the Potential Solutions for Repeater Management (China Telecom)

R3-226750 Discussion on network architecture for NCR (ZTE)

R3-226325 (TPs to TS 38.300, TS 38.413, TS 38.401 BL CRs) Support of Network-Controlled Repeater (Huawei)

R3-226326 Support of Network-Controlled Repeater (Huawei)

R3-226573 Supporting Authorization for Network Controlled Repeater (Samsung)

R4-2219374	Discussion on RAN4 feature list for NCR-MT ZTE Corporation

R4-2219805	Discussion on NR Network-controlled repeaters Nokia, Nokia Shanghai Bell

R4-2218639	discussion on NCR RF requirements CMCC

R4-2219375	Further discussion on RF requirements for NCR ZTE Corporation

R4-2219641	On NCR RF requirements Ericsson

R4-2219806	Study of RF core requirements of NR Network-controlled repeaters Nokia, Nokia Shanghai Bell

R4-2220255	WF for NCR system parameters and RF requirements ZTE,CMCC

R4-2219256	Discussion on RRM impacts for NR network-controlled repeaters Huawei, HiSilicon

R4-2219376	Further discussion on RRM requirements for NCR-MT ZTE Corporation

R4-2219718	On NCR RRM Requirements Nokia, Nokia Shanghai Bell

R4-2219845	Analysis of RRM requirements for network controlled repeater Ericsson

R4-2220140	Summary for [105][314] NR_netcon_repeater Moderator (ZTE)

R4-2220708	WF on NR Network-controlled Repeaters RRM requirements  ZTE

R4-2220076	Topic summary for [105][230] NR_netcon_repeater_RRM Moderator (ZTE)

R4-2216198	Discussion on NR Network-controlled repeaters Nokia, Nokia Shanghai Bell

R4-2216552	Discussion on work plan and spec drafting for NCR in Rel-18 ZTE Corporation

R4-2215488	discussion on NCR RF requirements  CMCC

R4-2216199	On RF core requirements of NR Network-controlled repeaters Nokia, Nokia Shanghai Bell

R4-2216553	Discussion on RF requirements for NCR in Rel-18 ZTE Corporation

R4-2216789	On RF and EMC requirements for network controlled repeaters Ericsson France S.A.S

R4-2216793	Network-controlled repeater specification impact in 38.106 Qualcomm France

R4-2217506	LS to RAN1 on NCR-MT transmission and Beam correspondence CMCC

R4-2216289	Initial discussion on RRM impacts for NR network-controlled repeaters Huawei, HiSilicon

R4-2216554	Discussion on RRM requirements for NCR-MT in Rel-18 ZTE Corporation

R4-2216862	Impact of RRM on network controlled repeater Ericsson.

R4-2217263	WF on RRM requirements for NCR ZTE Corporation

R4-2216898	Email discussion summary for [104-bis-e][314] NR_netcon_repeater Moderator (ZTE)

R4-2216936	Email discussion summary for [104-bis-e][225] NR_netcon_repeater_RRM Moderator (ZTE)

R4-2217158	Email discussion summary for [104-bis-e][225] NR_netcon_repeater_RRM Moderator (ZTE)

R4-2217470	Work plan for NCR RF requirements in Rel-18 ZTE

R4-2217471	WF on NCR RF requirements in Rel-18 ZTE

R4-2217512	Email discussion summary for [104-bis-e][314] NR_netcon_repeater Moderator (ZTE)

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template