3GPP TSG-RAN WG3 Meeting #118	R3-226136

# Toulouse, France, 14-18 November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.470 | CR | 0107 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 38.470 on Positioning System Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Ericsson, Nokia, Nokia Shanghai Bell, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2022-11-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS38.331, RRCSystemInfoRequest message can be used by UE to request SI messages configured by SI-SchedulingInfo IE or posSI-SchedulingInfo IE in SIB1. Upon receiving the request, the gNB-CU shall send the System Information Delivery message over F1AP to command the gNB-DU to broadcast the requested system information. For example, in this message, the SIType List IE can be used by the gNB-CU to provide the SI list of other SI to be broadcasted. The issue is that requesting the gNB-DU to broadcast the requested posSI configured in posSI-SchedulingInfo is not supported yet on F1AP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new Positioning System Information Delivery function and procedure. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has a limited impact on the protocol. This CR has functionality impact. This CR can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 spec is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.11, 6.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 38.473 CR1060 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#1, resubmission to #118 meeting. |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

5.2.11	Positioning function

This function allows to transfer location management messages between gNB-CU and gNB-DU. With this function, gNB-CU request TRP information from gNB-DU, and gNB-DU response to gNB-CU with the TRP information. With this function, gNB-CU request positioning measurements from gNB-DU, and gNB-DU response to gNB-CU with the positioning measurements.

The function allows to transfer the positioning assistance data from gNB-CU to gNB-DU. The gNB-DU is responsible for broadcasting the positioning assistance data according to the scheduling parameters available.

The function allows the gNB-CU to request the gNB-DU to configure PRS transmissions.

The function allows the gNB-CU to request the gNB-DU to configure measurement gap or PRS processing window.

The function allows the gNB-CU to request the gNB-DU to activate or deactivate preconfigured measurement gap or PRS processing window.

The function allows the gNB-CU to request the gNB-DU to broadcast positioning system information.

<<<<<<<<<<<<<<<<<<<<Next Change>>>>>>>>>>>>>>>>>>>>

6.1.11	Positioning procedures

The F1 Positioning procedures are listed below:

-	Positioning Assistance Information Control procedure;

-	Positioning Assistance Information Feedback procedure;

-	Positioning Measurement procedure;

-	Positioning Measurement Report procedure;

-	Positioning Measurement Abort procedure;

-	Positioning Measurement Failure Indication procedure;

-	Positioning Measurement Update procedure;

-	TRP Information Exchange procedure;

-	Positioning Information Update;

-	Positioning Information Exchange procedure;

-	Positioning Activation procedure;

-	Positioning Deactivation procedure;

-	E-CID Measurement Initiation;

-	E-CID Measurement Failure Indication procedure;

-	E-CID Measurement Report procedure;

-	E-CID Measurement Termination procedure.

-	PRS Configuration Exchange procedure;

-	Measurement Preconfiguration;

-	Measurement Activation;

-    Positioning System Information Delivery procedure.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>