3GPP TSG RAN Meeting #98-e    	            RP-222923

E-meeting, December 12th – 16th, 2022

Source:	Qualcomm Incorporated

Title:	Support of BWP without restriction

Agenda Item:	9.11

Document for: 	Discussion and Decision

# Background

At the RAN#97 meeting, RAN asked RAN4 to do a high-level analysis of options for BWP without restriction. RAN has received a LS from RAN4 regarding support of BWP without restriction [1], which includes comprehensive high-level analyses for the following options (other options were deprioritized in RAN4). The analyses are copied in the Annex of this contribution from the LS for convenience.

- Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP - Option B) Perform BM/RLM/BFD based on SSB outside active BWP - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps - Option B-2-2) Dedicated MG or NCSG for RLM/BFD/BM measurements - Option C) NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

In this contribution, we provide our views on how RAN plenary should proceed on this issue.

# Discussion

The high-level analyses from RAN4 indicate that different options have different impacts on specification/workload, mobility performance, throughput, and UE power consumption/complexity. In order to make the feature attractive for operators, it is important to make sure that the feature has no data/throughput performance degradation. From network point of view, measurement gap for intra-frequency L3 measurement or new signal transmissions should not be preferred. Further, it is the best if the BWP without restriction can be realized by gNB implementation for BWP with restriction (FG 6-1). For 3GPP, minimum workload is preferred considering the Rel-18 timeline.

Having said that, we believe Option B-1-1 is the best choice and RAN should task WGs to specify it. Following is from the LS [1]:

- Option B-1-1 does not cause data/throughput degradation.

- Option B-1-1 does not need measurement gap for intra-frequency L3 measurement.

- Option B-1-1 does not require specific gNB implementation.

- Option B-1-1 can be realized with the minimum 3GPP workload.

Proposal:

- Task the relevant Working Groups (RAN1, 2, 4) to specify Option B-1-1 (Using larger BW covering SSB outside active BWP without interruptions).

# Conclusion

In this contribution, we provided our views on BWP without restriction. Considering the following benefits of Option B-1-1, we propose to adopt Option B-1-1 and task WGs to specify it.

- Option B-1-1 does not cause data/throughput degradation.

- Option B-1-1 does not need measurement gap for intra-frequency L3 measurement.

- Option B-1-1 does not require specific gNB implementation.

- Option B-1-1 can be realized with the minimum 3GPP workload.

Proposal:

- Task the relevant Working Groups (RAN1, 2, 4) to specify Option B-1-1 (Using larger BW covering SSB outside active BWP without interruptions).

# Reference

- RP-222725, LS on BWP operation without restriction (R4-2220437; to: RAN; cc: RAN2, RAN1; contact: vivo)

# Annex: High-level analyses of options agreed in RAN4

Table 1: High-level analysis of candidate options on RRM requirements impact/workload in RAN4

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | - CSI-RS based RLM/BFD/BM requirements are already specified. - Further study is needed to decide on whether timing requirements may need to be updated | Low |
| Option B-1-1) | - Existing SSB based RLM/BFD/BM measurement requirements will apply. - The applicability rule of existing requirements is to be updated. - Depending on RF BW and BB BW (FFT BW) assumption, at least intra-frequency measurement with gap need to be revisited | Low |
| Option B-1-2) | - Same as Option B-1-1). Additionally, - Interruption requirements need to be developed additionally to allow UE for switching. | Low or Medium |
| Option B-2-2) | - Existing requirements need to be updated for SSB based RLM/BFD/BM with dedicated NCSG. - Existing requirements need to be updated for SSB based RLM/BFD/BM with dedicated MG. | NCSG: Low or Medium MG: Medium |
| Option C) | - Existing requirements for SSB based RLM/BFD/BM can be re-used. - Clarification on the requirement applicability might be needed. | Low |

Note: “Low”/ “Low to Medium”/ “Medium” correspond to the low/low to medium/medium RRM requirements impact.

Table 2: High-level analysis of candidate options on UE power consumption / UE complexity

| Options | Technical analysis | Summary |  |
| --- | --- | --- | --- |
|  |  | Power consumption | UE complexity |
| Option A) | - UE works in active BWP. - No additional implementation complexity is needed | Low | Low |
| Option B-1-1) | - Depending on RF BW and BB BW (FFT BW) assumption, UE can consume much higher power than other options - If BB BW is assumed adapted to BWP BW, it should be FFS how interruption can be avoided and its impact on UE complexity | High | Low to medium |
| Option B-1-2) | - UE works in larger RF BW than active BWP periodically and UE may consume larger power than other options except Option B-1-1. | Medium | Low to Medium |
| Option B-2-2) | - UE works in active BWP. | Low | Medium |
| Option C) | - UE works in active BWP | Low | Low |

Note: “Low”/ “Low to Medium”/ “Medium” correspond to the low/low to medium/medium negative impact on UE power consumption and UE complexity comparing to the legacy solution.

Table 3: High-level analysis of candidate options on Mobility performance impact

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | - Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-1-1) | - No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement - Can also help L3 intra-frequency measurement | Low |
| Option B-1-2) | - No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-2-2) | - Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option C) | - Measurement gap may or may not be needed for intra-frequency L3 measurement depending on whether or not NCD-SSBs from all neighbor cells are present within UE active BWP. - Measurement gap is needed for inter-frequency L3 measurement. | Low |

Note: “Low” - means a limited impact on mobility performance comparing to the legacy solution.

Table 4: High-level analysis of candidate options on Throughput impact (Data interruption)

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |
| Option B-1-1) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |
| Option B-1-2) | Interruption is needed for BM/RLM/BFD measurements | Low to Medium |
| Option B-2-2) | NCSG or MG is needed for BM/RLM/BFD measurements | NCSG: Low to Medium MG: Medium |
| Option C) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |

Note 1: Throughput impact due to L3 measurements is not considered.

Note 2: “Low”/ “Medium” correspond to the low/medium negative impact on throughput comparing to the legacy solution.