3GPP TSG RAN Meeting #98-e								   RP-222928

Online meeting, December. 12-16, 2022

Status Report to TSG

Agenda item:			9.5.1.2

| WI / SI Name | Further Enhancements on MIMO for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_FeMIMO |  |  |  |  |
| Unique ID | 860040 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-212535 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2022 | Performance part: 12/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 100% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | NO |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

RAN4 #104-bis-e (Oct. 2022, Electronic)

Performance Requirement

- RRM Performance Requirement

A summary of the electronic meeting email discussion on NR feMIMO RRM performance requirement was provided in [R4-2217139]. In last meeting, the big CR for all test cases was agreed. Some remaining details were discussed and the following WF was approved in RAN4#104-bis-e:

- WF on FeMIMO RRM Performance Requirement and Test Case was approved in R4-2217208

The following draft CRs were endorsed and the big-CR to 38.133 for FeMIMO RRM performance requirements was agreed in [R4-2217591]

- CR on maintenance of UL TCI state switching of FR2 PCell was endorsed in R4-2216822

- Draft CR on test case for DL TCI state switching for Cell with different PCI in FR2 NR-SA was endorsed in R4-2217209

- Draft CR on TC for joint unified TCI state switching in FR2 NR SA was endorsed in R4-2217210

- Draft CR on test case for L1-RSRP measurement procedure in FR1 NR-SA was endorsed in R4-2216366

- Draft CR on TC of L1-RSRP measurement on cells with different PCI was endorsed in R4-2217211

- maintenance CR on test cases for TRP specific BFD and LR was endorsed in R4-2217212

- DraftCR on maintaining TRP specific BFR test cases was endorsed in R4-2217213

The following agreements were made on RRM performance requirements, captured in approved WF [R4-2217208]:

Unified TCI state switching

- Issue 1-1-1: Pathloss RS configuration in joint TCI test cases

- Agreements:

  - Configure CSI-RS as the PLRS explicitly for joint TCI state switching and UL TCI state test cases.

- Issue 1-3-1: The rate of correct events for unified TCI state switching test case

- Agreements:

  - Specify “The rate of correct events observed during repeated tests shall be at least [90]%” in the unified TCI state switching test case.

TRP specific BFD and LR

- Issue 3-1-2: Beam recovery method configured in the test case

- Agreements:

  - For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases

  - For BFR on SpCells, CFRA based BFR is configured.

- Issue 3-1-3/Issue 3-1-4

- Agreements:

  - For both intra- and inter-cell TRP specific BFR test cases, if SSB is configured as BFD-RS for TRP specific BFR test case, whether SSBs are overlapped or not? How long are the time periods of T1~T5?

    - [No matter SSBs are from the same cell, or they are from serving cell and a cell with additionalPCI], If SSBs is configured as BFD-RS, they are not overlapped and the duration time will not be extended.

    - T1~T5 are:

| FR | RS | DRX | T1(s) | T2(s) | T3(s) | T4(s) | T5(s) | D1(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | SSB | Non-DRX | 1 | 2.61 | 1.64 | 0 | 1.01 | 0.97 |

- Issue 3-1-5: If CSI-RS is configurated as BFD-RS for TRP specific BFR test case, whether CSI-RSs are overlapped or not?

- Agreement:

  - If CSI-RS is configured as BFD-RS for TRP specific BFR test cases, CSI-RSs are considered as overlapped.

- Issue 3-1-6: If CSI-RS is configurated as BFD-RS for TRP specific BFR test case, how long are the time periods of T1~T5?

- Agreement:

| FR | RS | DRX | T1(s) | T2(s) | T3(s) | T4(s) | T5(s) | D1(s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 | CSI-RS | DRX | 1 | 10.81 | 10.28 | 0 | 0.57 | 0.53 |

- Demodulation and CSI requirement

A summary of the electronic meeting email discussion on NR feMIMO demodulation and CSI requirement was provided in [R4-2217486]. Some remaining details were discussed and the following WF was approved in RAN4#104-bis-e:

- WF on UE demodulation and CSI requirement for FeMIMO was approved in R4-2217440

The following draft CRs were endorsed and the big-CR to 38.101-4 for FeMIMO demodulation and CSI requirement was agreed in [R4-2217433]

- Draft CR on PDSCH requirement for HST-SFN scheme B was endorsed in R4-2217435

- Draft CR for 38_101-4 FeMIMO Applicability of Requirements was endorsed in R4-2217434

- Draft CR on PDSCH requirement for HST-SFN scheme B was endorsed in R4-2217436

- Draft CR on PMI requirement for multi-TRP was endorsed in R4-2217437

- Draft CR to 38.101-4: PDCCH requirement with inter-slot repetition was endorsed in R4-2217438

- Draft CR on reference measurement channels for multi-TRP PDCCH performance was endorsed in R4-2217439

The following agreements were made on UE demodulation and CSI requirements, captured in approved WF [R4-2217440]:

- additional margin requirement for multi-TRP repetition transmission

- Agreement

  - Specify additional margin with [average span -2.5dB] = [0.8] dB for FDD 2Rx/4Rx, and TDD 4Rx cases, based on simulation results collection declared by companies for requirement derived

  - Specify additional margin with [span -2.5dB] = [1.28] dB for TDD 2Rx case

    - 	The value of additional margin can be updated pending on the simulation results

  - Companies are encouraged to further check the simulation results and updated in the next meeting

- SNR value for PDCCH requirement for multi-TRP repetition

- Agreement

  - Specify the SNR requirement = average impairment results+ extra margin +additional margin based on simulation summary R4-2215728

    - Extra margin =0.5 dB for QPSK

    - additional margin = 0.8 dB for FDD 2Rx/4Rx, and TDD 4Rx cases

    - additional margin = 1.28 dB for TDD 2Rx case

- MCS for HST-SFN scheme A

- Agreement

  - Introduce HST-SFN scheme A with MCS 13 for both FDD and TDD

- MCS for HST-SFN scheme B

- Agreement

  - Introduce HST-SFN scheme B with MCS 13 for both FDD and TDD

- Test applicability rule for scheme A and scheme B

- Agreement

  - Introduce HST-SFN scheme B with MCS 13 for both FDD and TDD If UE support HST-SFN scheme A and pass the test of HST SFN scheme A, it can skip the test of HST SFN scheme B

- SNR value for PDSCH requirement with HST-SFN scheme A and B

- Agreement

  - Specify the SNR requirement = average impairment results+ extra margin based on simulation summary R4-2215728

    - Extra margin =0.5 dB for QPSK and 16QAM

  - For the test case with larger span than [2.5] dB, further check the details of simulation assumption and further discuss extra margin on a case-by-case basis, leave TBD for SNR requirement in this meeting if no further updated.

  - Companies are encouraged to further check the simulation results and updated in the next meeting

- MCS for PMI requirement for multi-TRP

- Agreement

  - MCS as 13

- Gamma value

- Agreement

  - Keep the gamma value with [1.6] into draft CR.

RAN4 #105 (Nov. 2022, Toulouse)

Performance Requirement

- RRM Performance Requirement

A summary of the electronic meeting email discussion on NR feMIMO RRM performance requirement was provided in [R4-2220055]. In last meeting, the big CR for all test cases was agreed. Some remaining details were discussed and the following WF was approved in RAN4#104-bis-e:

- WF on Rel-17 FeMIMO RRM performance requirements was approved in R4-2220311

The following draft CRs were endorsed and the big-CR to 38.133 for FeMIMO RRM performance requirements was agreed in [R4-2219032]

- Draft CR on general configurations in R17 feMIMO test cases was endorsed in R4-2220436

- Draft CR on TC for joint unified TCI state switching in FR2 NR SA was endorsed in R4-2220428

- CR on test cases of Unified TCI state switch delay was endorsed in R4-2220308

- draft CR on test cases for R17 unified TCI was endorsed in R4-2220309

- Draft CR for correcting TRP specific BFR test cases was endorsed in R4-2220310

- CR on maintaining CSI-RS configuration used for TRP specific BFR test cases was endorsed in R4-2219249

The following agreements were made on RRM performance requirements, captured in approved WF [R4-2220311]:

- Issues 1-1-1: Whether PL-RS is maintained or not maintained in joint TCI test cases and how to config?

- Agreement:

  - Configure PL-RS to be maintained with the same CSI-RSs configured as PL-RS for target TCI and source TCI.

- Issue 1-1-2: Whether to configure PL-RS of target TCI is QCL-D with source RS?

- Agreement

  - PL-RS of the target TCI are configured to be QCL type D with the source RS

- Issue 2-1-1: For TRP specific BFR test cases, whether intra-cell TRP or inter-cell TRP are designed?

- Agreement

  - CSI-RS based TCs are designed as intra-cell TRP. 2 SSB based TCs are designed as inter-cell TRP.

- Demodulation and CSI requirement

A summary on NR feMIMO demodulation and CSI requirement was provided in [R4-2220149].

The following draft CRs were endorsed and the big-CR to 38.101-4 for FeMIMO demodulation and CSI requirement was agreed in [R4-2219024]

- Draft CR on PDSCH requirement for HST-SFN scheme A was endorsed in R4-2220194

- Draft CR on PDSCH requirement for HST-SFN scheme B was endorsed in R4-2220195

- draft CR to 38.101-4: PDCCH requirement for enhancement on multi-TRP was endorsed in R4-2219277

2.4.2	Remaining Open issues

Performance Requirement

- RRM Performance Requirement

- None

- Demodulation and CSI requirement

- None

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #104-bis-e (Oct. 2022, Electronic)

- R4-2217138	Email discussion summary for 104-bis-e][205] NR_feMIMO_RRM_1				Moderator (Intel)

- R4-2217139	Email discussion summary for [104-bis-e][206] NR_feMIMO_RRM_2 			Moderator (Samsung)

- R4-2217486	Email discussion summary for [104-bis-e][322] NR_FeMIMO_Demod 			Moderator (Samsung)

- R4-2217203	WF on FeMIMO Unified TCI state								Intel

- R4-2217204	WF on FeMIMO RRM requirements for inter-cell beam management				Huawei

- R4-2217208	WF on FeMIMO RRM Performance Requirement and Test Case				Samsung

- R4-2217440	WF on UE demodulation and CSI requirement for FeMIMO					Samsung

- R4-2217591	Big CR for NR feMIMO RRM performance requirements					MCC, Samsung

- R4-2215353	Discussion on remaining issue about Unified TCI state in FeMIMO				Intel Corporation

- R4-2215591	On remaining issues for unified TCI requirements						Apple

- R4-2215743	Discussion on remaining issues of FeMIMO RRM core requirements for unified TCI state	Samsung

- R4-2215764	Discussion on unified TCI for DL and UL							MediaTek Inc.

- R4-2216280	Discussion on RRM remaining issues for R17 unified TCI framework				Huawei, HiSilicon

- R4-2216281	CR on maintaining TCI state switching requirements for R17 unified TCI			Huawei, HiSilicon

- R4-2216360	Discussion on remaining issues in unified TCI in R17 feMIMO				vivo

- R4-2216361	CR on unified TCI in R17 feMIMO								vivo

- R4-2216486	Discussion on Unified TCI for DL and UL							ZTE Corporation

- R4-2216596	Remaining issues for UL TCI state switch delay						Nokia, Nokia Shanghai Bell

- R4-2216817	Discussion on remaining issues on Unified TCI for DL and UL				Ericsson

- R4-2216818	CR on maintenance of unified TCI state switching requirements				Ericsson

- R4-2217205	CR for unified TCI										Apple, Vivo

- R4-2215354	Discussion on remaining issue about inter-cell beam management in FeMIMO			Intel Corporation

- R4-2215593	On remaining issues for inter-cell beam management						Apple

- R4-2215744	Discussion on remaining issues of FeMIMO RRM core requirements for inter-cell beam management																Samsung

- R4-2215765	Discussion on inter cell beam management							MediaTek Inc.

- R4-2215767	CR on applicability of R17 inter cell beam management for FR2-2				MediaTek Inc.

- R4-2216282	Discussion on RRM remaining issues for R17 inter-cell beam managements			Huawei, HiSilicon

- R4-2216283	CR on maintaining L1-RSRP measurement requirements for R17 inter-cell BM		Huawei, HiSilicon

- R4-2216362	Discussion on remaining issues in inter-cell beam managements in R17 feMIMO		vivo

- R4-2216363	CR on inter-cell beam managements in R17 feMIMO						vivo

- R4-2216485	Discussion on remaining RRM requirements for inter-cell beam management			ZTE Corporation

- R4-2216819	Discussion on remaining issues of Inter-cell beam management				Ericsson

- R4-2217206	CR for inter-cell beam management								Apple

- R4-2217207	Maintenance CR on inter-cell BM								Ericsson

- R4-2215747	Correction on requirements for TRP specific link recovery procedures				Samsung

- R4-2217583	CR on SFN based RLM and LRP								ZTE Corporation

- R4-2216364	Discussion on R17 feMIMO test case configurations						vivo

- R4-2216821	Discussion on test cases for TRP specific BFD and LR						Ericsson

- R4-2215745	Discussion on remaining issues of test cases for unified TCI state				Samsung

- R4-2216822	CR on maintenance of UL TCI state switching of FR2 PCell					Ericsson

- R4-2217209	Draft CR on test case for DL TCI state switching for Cell with different PCI in FR2 NR-SA	vivo

- R4-2217210	Draft CR on TC for joint unified TCI state switching in FR2 NR SA				MediaTek Inc.

- R4-2216366	Draft CR on test case for L1-RSRP measurement procedure in FR1 NR-SA			vivo

- R4-2217211	Draft CR on TC of L1-RSRP measurement on cells with different PCI				Samsung

- R4-2215358	Discussion on TRP specific Beam Failure Detection and Link Recovery Test case		Intel Corporation

- R4-2215746	Discussion on remaining issues of test cases for TRP specific BFD and LR			Samsung

- R4-2217212	maintenance CR on test cases for TRP specific BFD and LR					Ericsson

- R4-2217213	DraftCR on maintaining TRP specific BFR test cases						Huawei, HiSilicon

- R4-2215728     Simulation results summary for FeMIMO demodulation and CSI requirement                           Samsung

- R4-2217433     Big draft CR on UE demodulation and CSI requirement for FeMIMO                                        Samsung

- R4-2217434     Draft CR for 38_101-4 FeMIMO Applicability of Requirements                                                 Samsung

- R4-2215595     On demod requirements for HST Scheme-A                                                                                 Apple

- R4-2217435     Draft CR on PDSCH requirement for HST-SFN scheme A                                                          Apple

- R4-2215713     Updated simulation results for HST-SFN                                                                                      CMCC

- R4-2215726     Discussion and simulation results on demodulation requirement for Enhancement on HST-SFN scenario

Samsung

- R4-2215882     Views on HST-SFN Tests                                                                                                             Qualcomm

- R4-2217284     Simulation Results on HST-SFN Scheme A                                                                                 Qualcomm

- R4-2217285     Simulation Results on HST-SFN Scheme B                                                                                 Qualcomm

- R4-2215938     Discussion on demodulation requirement for Enhancement on HST-SFN deployment               MTK

- R4-2216002     Discussion on UE FeMIMO demod HST-SFN                                                                             Huawei

- R4-2216003     Simulation results on UE FeMIMO demod HST-SFN                                                                 Huawei

- R4-2217436     Draft CR on PDSCH requirement for HST-SFN scheme B                                                         Huawei

- R4-2216387     Discussion on the PDSCH requirement for HST-SFN                                                                 Ericsson

- R4-2216388     Simulation restuls for PDSCH requirement for HST-SFN                                                           Ericsson

- R4-2215597     On PDCCH demod requirements for multi-TRP                                                                          Apple

- R4-2217437     Draft CR on PMI requirement for multi-TRP                                                                               Samsung

- R4-2217286     Simulation Results on Multi-TRP PDCCH Tests                                                                         Qualcomm

- R4-2215876     Views on Performance Requirements for Multi-TRP PDCCH Tests                                           Qualcomm

- R4-2215939     Discussion on demodulation performance requirements definition for Rel17 multi-TRP           MTK

- R4-2216005     Discussion on UE FeMIMO demod mTRP                                                                                  Huawei

- R4-2216006     Simulation results on UE FeMIMO demod mTRP                                                                       Huawei

- R4-2216391     Discussion on the requirement of PDCCH enhancement for multi-TRP                                     Ericsson

- R4-2216392     Simulation results for PDCCH enhancement for multi-TRP                                                        Ericsson

- R4-2217438     draft CR to 38.101-4: PDCCH requirement with inter-slot repetition                                          Ericsson

- R4-2217439     Draft CR on reference measurement channels for multi-TRP PDCCH performance                  Qualcomm\

- R4-2216007     Discussion on UE FeMIMO CSI Mtrp                                                                                         Huawei

- R4-2216008     Simulation results on UE FeMIMO CSI Mtrp                                                                             Huawei

- R4-2216389     Discussion on the gamma value for PMI reporting tests for multi-TRP                                      Ericsson

- R4-2216390     Simulation results for PMI reporting tests for multi-TRP                                                            Ericsson

- R4-2220054	Topic summary for [105][208] NR_feMIMO_RRM_1						Moderator (Intel)

- R4-2220055	Topic summary for [105][209] NR_feMIMO_RRM_2						Moderator (Samsung)

- R4-2220713	WF on Rel-17 FeMIMO RRM core requirement maintenance					Intel Corporation

- R4-2220311	WF on Rel-17 FeMIMO RRM performance requirements					Samsung

- R4-2219032	Big CR for NR feMIMO RRM performance requirements					MCC, Samsung

- R4-2218184	On Remaining Issues for FeMIMO RRM Core Requirements					Apple

- R4-2218331	Discussion on remaining issue in Rel-17 FeMIMO						Intel

- R4-2218744	Discussion on R17 feMIMO for RRM core requirement maintenance				MediaTek Inc.

- R4-2218935	Remaining issues for UL TCI state switch							Nokia, Nokia Shanghai Bell

- R4-2219027	Discussion on remaining issues of FeMIMO RRM core requirements				Samsung

- R4-2219172	Discussion on remaining issues in R17 feMIMO RRM requirements				vivo

- R4-2219245	Discussion on RRM core remaining issues for R17 FeMIMO					Huawei, HiSilicon

- R4-2219437	Discussion on remaining RRM requirements for FeMIMO					ZTE Corporation

- R4-2219943	Discussion on remanining open issue of FeMIMO						Ericsson

- R4-2220405	CR for Unified TCI State switching requirements						Apple

- R4-2220406	CR for Inter-cell Beam Management								Apple

- R4-2220712	CR on R17 feMIMO RRM requirements							vivo

- R4-2220408	CR on maintaining RRM requirements for R17 FeMIMO					Huawei, HiSilicon

- R4-2220409	CR on conditions for PL-RS maintenance							Nokia, Nokia Shanghai Bell

- R4-2219944	CR on maintenance of FeMIMO								Ericsson

- R4-2218187	On Remaining Issues for FeMIMO Performance Requirements				Apple

- R4-2219247	Discussion on remaining issues for R17 FeMIMO test cases					Huawei, HiSilicon

- R4-2219174	Discussion on general aspects for R17 feMIMO Test cases					vivo

- R4-2219945	on general configuration									Ericsson

- R4-2219028	Discussion on remaining issues of test cases for unified TCI state				Samsung

- R4-2220436	Draft CR on general configurations in R17 feMIMO test cases					vivo

- R4-2220428	Draft CR on TC for joint unified TCI state switching in FR2 NR SA				MediaTek Inc.

- R4-2220308	CR on test cases of Unified TCI state switch delay						Samsung

- R4-2220309	draft CR on test cases for R17 unified TCI							vivo

- R4-2219248	CR on maintaining unified TCI state switching test cases					Huawei, HiSilicon

- R4-2219029	Discussion on remaining issues of test cases for TRP specific BFD and LR			Samsung

- R4-2220310	Draft CR for correcting TRP specific BFR test cases						Apple, Samsung

- R4-2219031	CR on test cases of TRP specific BFR								Samsung

- R4-2219249	CR on maintaining CSI-RS configuration used for TRP specific BFR test cases		Huawei, HiSilicon

- R4-2220149     Summary for [105][323] NR_FeMIMO_Demod                                                                           Moderator (Samsung)

- R4-2219024     Big CR on UE demodulation and CSI requirement for FeMIMO                                                 Samsung

- R4-2218931     Simulation results summary for FeMIMO demodulation and CSI requirement                           Samsung

- R4-2218189     Simulation results for demod requirements with HST-SFN Scheme-A                                        Apple

- R4-2219658     Simulation results on UE FeMIMO demod HST-SFN                                                                  Huawei

- R4-2219276     Update simulation restuls for PDSCH requirement for HST-SFN                                                Ericsson

- R4-2220194     Draft CR on PDSCH requirement for HST-SFN scheme A                                                          Apple

- R4-2220195     Draft CR on PDSCH requirement for HST-SFN scheme B                                                          Huawei

- R4-2219659     Simulation results on UE FeMIMO demod mTRP                                                                        Huawei

- R4-2219277     draft CR to 38.101-4: PDCCH requirement for enhancement on multi-TRP                                Ericsson

- R4-2218371     Simulation Results on PMI Reporting Requirement for m-TRP CSI                                            Qualcomm