3GPP TSG RAN Meeting #98-e	RP-222956

Electronic Meeting, December 12-16, 2022

Status Report to TSG

Agenda item:			9.3.4.10

| WI / SI Name | # Support of intra-band non-collocated EN-DC/NR-CA deployment |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NonCol_intraB_ENDC_NR_CA |  |  |  |  |
| Unique ID | 950081 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222309 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 09/2023 | Performance part: 12/2023 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 20% (The WI has been started from Q3 2022) | Performance Part: 0% (The WI will be started from Q2 2023) | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Yasuki Suzuki |
|  | Company | KDDI |
|  | Email | ui-suzuki@kddi.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#104-e
<UE RF>

- Work plan in [3] was approved.

- WF in [2] captured the following agreements:

  - Revise the current wording “Intra-band non-collocated EN-DC” of the WID as follows in next RAN plenary.

    - Inter-band non-collocated EN-DC

      - inter-band EN-DC with overlapping

    - Intra-band non-collocated NR-CA

  - Prohibit Simultaneous Rx/Tx behavior for UE for this study

  - Total four RF antenna is assumed.

  - Reuse UE RF architecture of inter-band non-contiguous DC_42_n77/78 EN-DC Type-2 (i.e. 2 layer/2 Rx Chain per CC, total 4 Rx Chain) as the baseline.

  - 1 CC on each sub-block. This means to be a total of 2 CC, i.e. n78(2A)/n77(2A) for CA as baseline

    - 3 CC, i.e. n77(3A)/n78(3A), is not precluded. The work on 3CC should be done after work for 2CC is finalized and it is clear how many non-collocated base stations are assumed.

  - 25dB power imbalance, 1dB REFSENS relaxation. RAN4 may revisit if there is technical concern.

  - Agree to consider sync NR-CA scenario.

    - No Tx-Rx simultaneous operation is assumed to be supported for sync NR-CA scenario.

  - FFS async NR-CA scenario.

    - Encourage operators to give their preference of async scenario.

  - Type-1 is default/legacy already specified and Type-2 should be specified for non-collocated NR-CA.

  - Focus on NR-CA Type-2 UE (2 Rx Chains per CC, total 4 Rx Chains) in this meeting and begin discussions on UE RF architecture of “New Type UE” from next meeting

RAN4#105
<UE RF>

- WF in [38] captured the following agreements:

  - Discuss both MRTD and TAE together in join session for face-to-face meeting and treat them in one email thread for e-meeting.

  - Not to define frequency separation limitation for DL adding to existence specification, and no associated new signalling needed

  - The assumption for FS is ≤ 600MHz under this WI (work is limited to CA/EN-DC for bands 42, n77/n78)

  - Type 2 UE is the reference.

<RRM>

- WF in [36] captured the following agreements:

  - MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE

    - MRTD=33us

    - MTTD=34.6us

  - MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 2 UE

    - MRTD=33us

    - MTTD=35.21us

2.4.2	Remaining Open issues

The core part of the work item includes:

- Phase I:

  - Study the feasibility to support non-co-located scenarios for both FR1 inter-band EN-DC with overlapping and FR1 intra-band NR-CA, except for 2-layer case of EN-DC with supporting the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 already specified in Rel-16 and Rel-17.

    - Investigate the tolerable power imbalance between carriers

    - Investigate the required arrival time difference between CCs

    - Investigate the additional impacts of contiguous case, if time units are available.

    - Evaluate the UE performance under the power imbalance and arrival time difference

      - Discuss and decide reference UE architecture considering the UE capability of interBandMRDC-WithOverlapDL-Bands-r16 for 2-layer MIMO case for NR-CA, and 4-layer MIMO case for both EN-DC/NR-CA

        - NOTE 1: For UE capable of supporting 2-layer MIMO, only assuming power imbalance 25dB

        - NOTE 2: For UE capable of supporting 4-layer MIMO, 2-layer MIMO power imbalance assumption can be considered as a base line but another value or the same power imbalance with different throughput performance requirement is not precluded.

        - NOTE 3: RAN4 is recommended to start the work on 2-layer first and after that start 4-layer work based on the conclusion of 2-layer work.

    - Work is limited to CA/EN-DC for EN-DC/NR-CA for bands 42, n77/n78

    - Investigate whether the power imbalance should be explicitly (e.g. as an RF requirement) or implicitly specified (e.g. through a demodulation performance test). Specify the power imbalance based on the outcome of the investigation.

    - If any change in RAN1 or RAN2 spec is needed, it will be triggered by RAN4 LS

- Phase II:

  - Phase II work will get started after the feasibility in phase I is confirmed

  - Specify MRTD and MTTD requirements in non-collocated deployment

  - Discuss and decide if the different requirements will be specified based on UE capability of interBandMRDC-WIthOverlapDL-Bands-r16.

The performance part of the work item includes:

- Phase II:

  - Phase II work will get started after the feasibility in phase I is confirmed

  - Specify PDSCH demodulation requirements for non-collocated scenarios for intra-band non-contiguous EN-DC and NR-CA

    - Define PDSCH demodulation performance requirement based on the applicable MRTD and power imbalance values.

    - Power imbalance between the carriers is limited

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R4-2214246	Email Discussion Summary for [104-e][135] NonCol_intraB

- R4-2214458	WF on NonCol_intraB_ENDC_NR_CA

- R4-2211795	Work Plan for NonCol_intraB_ENDC_NR_CA

- R4-2211752	Discussion on the power imbalance requirement for intra-band non-contiguous CA with non-collocated scenario

- R4-2211796	Discussion on non-collocated EN-DC,NR-CA Type2

- R4-2211914	On intra-band non-colocation EN-DC/CA

- R4-2212011	On the UE RF aspects for non-collocated EN-DC,NR-CA

- R4-2212098	Architecture impact and options for non-collocated DC_42_n77

- R4-2212147	Issues for Non-collocated Deployments

- R4-2212717	Discussion on non-collocated EN-DC and NR-CA

- R4-2212792	Discussion on intra-band NC CA under non-collocated deployment

- R4-2212904	On required arrival time difference between CCs

- R4-2213133	Discussion on DL transmission EN-DC /NR CA non-collocated scenario supporting 2x2 MIMO

- R4-2213760	Discussion on support of intra-band non-collocated EN-DC/NR-CA deployment

- R4-2218192 WID scope update

- R4-2218193 discussion on noncollocated intraB_v1

- R4-2218231 Discussion on RRM requirements for non-co-located EN-DC and CA

- R4-2218281 Intra-band non-collocated EN-DC NR-CA type 4 UE

- R4-2218417 Type 3 Capability for Non-collocated Intra-band.doc

- R4-2218526 Type 3 architecture aspects for 4Rx non-collocated overlapping bands

- R4-2218682 CA requirement for intraB

- R4-2218683	Draft CR on introducing MRTD/MTTD requirement

- R4-2218687	Discussion on RRM requirements for non-collocated FR1 intra-band EN-DC/NR-CA

- R4-2218688	MRTD/MTTD requirement for intra-band non-collocated CA

- R4-2218757 Further consideration on Fs assumption for non-collocated EN-DC, NR-CA deployment

- R4-2218758 LS to RAN2 on intraBandFreqSeparationUL-AggBW-GapBW-r16

- R4-2218770 Issues for Non-collocated Deployments with 4Layers per CC

- R4-2218831 discussion on intra-band non-co-located

- R4-2218875 Discussion on the new type UE to support non-collocated deployment

- R4-2219255	Discussion on RRM impacts due to support intra-band non-collocated CA/DC

- R4-2219286	Discussion on RRM requirements for intra-band non-collocated EN-DC/NR-CA deployment

- R4-2219334	MRTD and MTTD requirements

- R4-2219526	Discussion on RRM requirements of Type 2 UE for intra-band non-collocated NR-CA deployment

- R4-2220069 Topic summary for [105][222] NonCol_intraB_ENDC_NR_CA_RRM

- R4-2220113 Topic summary for [105][133] NonCol_intraB

- R4-2220398 WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA

- R4-2220534 LS to RAN2 on intraBandFreqSeparationUL-AggBW-GapBW-r16

- R4-2220537 WF on the requirement for intra-band non-collocated CA,EN-DC