3GPP TSG RAN meeting #98										RP-222959

E-meeting, December 12th – 16th 2022

## Status Report to TSG

Agenda item:			9.3.2.6

| WI / SI Name | WI on NR Support for UAV (Uncrewed Aerial Vehicles) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_UAV |  |  |  |  |
| Unique ID | 941105 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-213600 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 20% | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Jedrzej Stanczak |
|  | Company | Nokia, Nokia Shanghai Bell |
|  | Email | jedrzej.stanczak@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#110bis:

Study extending application of FR2-only beam management parameters e.g., spatial relation, beam correspondence, etc. to FR1 for UAV UEs - FFS: Other parameters - FFS: Impacts to legacy beam management for FR1 - FFS: Application of beam correspondence in FDM bands Note: Identification of relevant UAV UE capabilities does not require commitment to support a specific TCI framework, and relevant parameters may change depending on the framework supported Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks) Study indication of beam characteristics, e.g., number of beams, beamwidth, beam center, radiated EIRP, etc. as UAV UE capability - FFS: Feasibility/benefit of indicating orientation of beams including height dependence - FFS: Necessary parameters, ranges of suitable values, and method of indication - FFS: Height-dependence on relevant parameters - FFS: Indication of beams as either ‘fixed’ or ‘adaptive’ Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks) Study indication of minimum beam application latency as UAV UE capability - If unifiedJointTCI-r17 is supported, suitable range of values for minBeamApplicationTime-r17 - If unifiedJointTCI-r17 is not supported, enhancements to timedurationforQCL may be considered - FFS: additional parameters, e.g., beamSwitchTiming Note: further consideration does not require commitment to support a specific TCI framework. Note: Whether or not to specify above parameters should depend on the identification of the target scenarios and the potential issues faced by UAV, the identification of which existing capabilities and/or mechanisms and/or frameworks can be treated as candidate for addressing above issues, and the necessity of specifying above parameters (i.e., whether the potential issues faced by UAV can already be solved with the existing capabilities and/or mechanisms and/or frameworks)

#### 2.1.2	Remaining Open issues

Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling:

- FR1 with directional antenna at UE side

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#119bis:

Agreements: - The time information reported as part of flight path plan is optional. UE includes time info, if configured by the network and available at the UE. FFS on flight path details (waypoints and what is time information). - Allow the flight path to be updated. FFS on the details. - FFS on reporting format and initial flight path reporting (i.e. what information to report and how) – next meeting - Continue to study height-depending scaling, triggering and combinations - As in LTE, as a baseline, events A3, A4 and A5 can be configured with the configured number of cells (numberofTriggeringCells)

RAN2#120:

Agreements: - A waypoint is a planned location for the UE along the flight path and is described via the existing parameter type LocationCoordinates defined in TS 37.355. - A timestamp provides the UTC time associated with estimated time of arrival to a waypoint as baseline. FFS on granularity - No requirements are placed on spatial distribution of waypoints - A UE indicates whether flight plan information is available within the RRCReconfigurationComplete, RRCReestablishmentComplete, RRCResumeComplete, or RRCSetupComplete message. Flight path reporting uses at the UE Information request/response procedure as baseline. - UE indicates to the network a new flight path is available in the UE (whether it is initial or update). Then, reuse the normal request/response procedure of flight path report. - UAI message can also be used to indicate the UE has flight path availability. - FFS whether and what triggering conditions are specified for flight update. FFS The maximum number of waypoints within flight path plan is left FFS.

#### 2.2.2	Remaining Open issues

Specify the following enhancements on measurement reports

- UE-triggered measurement report based on configured height thresholds

- Reporting of height, location and speed in measurement report

- Flight path reporting

- Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously

Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling:

- FR1 with directional antenna at UE side

Study and specify, if needed, enhancements for UAV identification broadcast.

Specify the signaling to support subscription-based aerial-UE identification

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3#117bis:

- Agreed to introduce Aerial UE Subscription Information IE over NGAP in INITIAL CONTEXT SETUP, UE CONTEXT MODIFICATION, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE messages. - Whether to introduce Aerial UE Subscription Information IE over XnAP in HANDOVER REQUEST message - Whether to introduce Aerial UE Subscription Information IE over XnAP RETRIEVE UE CONTEXT RESPONSE message is FFS - The Aerial UE Subscription Information IE is based on LTE format with codepoints ENUMERATED (allowed, not allowed, …) - Whether any other codepoints needed are pending to RAN2 progress. - Inter-RAT or DC scenarios is subject to RAN2 progress and FFS.

RAN3#118:

- Introduce Aerial UE Subscription Information IE over XnAP. - Whether additional codepoints are needed for Aerial UAV Subscription Information IE is subject to SA2 and RAN2 discussions. - RAN3 will not initiate the discussion on Inter-RAT and NR-DC support. - R3-226840 Endorsed as BL CR - R3-226841 Endorsed as BL CR - R3-226823 Endorsed as BL CR

#### 2.3.2	Remaining Open issues

Finalize the signaling to support subscription-based aerial-UE identification.

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

TDocs submitted to RAN1:

- R1-2209762        Discussion on the benefits of UE beamforming capability reporting                           Samsung

- R1-2209631        UE capability and RRC signaling for UAV beamforming                                                   Nokia, Nokia Shanghai Bell

- R1-2208674        Discussion on UE capability and RRC signaling for UAV beamforming                        vivo

- R1-2208405        UE capability and RRC signaling for UAV beamforming                                                   Huawei, HiSilicon

- R1-2208697        Discussion on  UE capability for UAV beamforming                                                         ZTE

- R1-2208749        Discussion on UE capability and RRC signaling for UAV beamforming                        Lenovo

- R1-2209274        Discussion on UE capability and RRC signaling for UAV beamforming                        xiaomi

- R1-2209928        Discussion on UE capability and RRC signaling for UAV beamforming                        NTT DOCOMO, INC.

- R1-2210016        Discussion of UE capability and RRC signaling for UAV beamforming                         Qualcomm Incorporated

- R1-2210092        On UAV beamforming capabilities                                                                                        Ericsson

- R1-2210697        Session notes for 9.15 (NR support for Uncrewed Aerial Vehicles)                              Ad-Hoc Chair (Huawei)

- R1-2209085        Work plan for Rel-18 Uncrewed Aerial Vehicles                                                               Rapporteur (Nokia)

- R1-2210310        Summary#1 of discussion on UAV capability and RRC signaling for UAV                   Moderator (Nokia)

- R1-2210311        Summary#2 of discussion on UAV capability and RRC signaling for UAV                   Moderator (Nokia)

- R1-2210312        Summary#3 of discussion on UAV capability and RRC signaling for UAV                   Moderator (Nokia)

TDocs submitted to RAN2:

- R2-2209307	LS response to 3GPP RAN on Location Services for Drones (LI(21)P61035r1; contact: ETSI)	ETSI TC LI	LS in	To:RAN, RAN2	Cc:SA3LI

- R2-2210354	Uncrewed Aerial Vehicles in Rel-18 - updated workplan	Nokia, Nokia Shanghai Bell	Work Plan	Rel-18	NR_UAV-Core

- R2-2209368	Conditional HO in NR UAV	CATT	discussion	Rel-18	NR_UAV-Core

- R2-2209418	Measurement Enhancement for UAV	OPPO	discussion	Rel-18

- R2-2209446	Measurement and reporting enhancements	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

- R2-2209532	Measurement reports   	Ericsson	discussion	Rel-18

- R2-2209582	UAV support for NR	Intel Corporation	discussion	Rel-18	NR_UAV-Core

- R2-2209754	Considerations on Measurement Reports Enhancements	NEC Europe Ltd	discussion	Rel-18	NR_UAV-Core

- R2-2209795	User consent on UAV location reporting	Apple	discussion	Rel-18	NR_UAV

- R2-2209934	Measurement enhancement for NR UAV	Lenovo	discussion	Rel-18

- R2-2210161	Measurement Reporting for NR UAV	CMCC	discussion	Rel-18	NR_UAV-Core

- R2-2210175	On measurement reporting enhancements for NR UAV	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

- R2-2210219	Considerations about UAV mobility and user consent	Sony	discussion	Rel-18	NR_UAV

- R2-2210355	On measurements and measurement reporting enhancements for Rel-18 UAVs	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2210356	On measurement reporting based on a configured number of cells triggering – evaluation results	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2210435	Discussion on measurement reporting for NR UAV	Sharp	discussion

- R2-2210441	Measurement reporting for UAV	InterDigital	discussion	Rel-18	NR_UAV-Core

- R2-2210489	Discussion on measurement reporting for NR UAV	Xiaomi	discussion	Rel-18	NR_UAV-Core

- R2-2210504	Potential issues and enhancements for UAV measurements	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

- R2-2210535	Consideration on flight path reporting of UAV for NR 	DENSO CORPORATION	discussion	NR_UAV-Core

- R2-2210601	Discussion on measurement reporting enhancement for NR UAV	vivo	discussion	Rel-18	NR_UAV

- R2-2210602	Discussion on flight path reporting for NR UAV	vivo	discussion	Rel-18	NR_UAV

- R2-2210623	Further discussion on NR support for UAV	NTT DOCOMO, INC.	discussion	Rel-18

- R2-2210648	Measurement Report Enhancement	LG Electronics Finland	discussion	Rel-18

- R2-2210652	Flight path information report enhancement	LG Electronics Finland	discussion	Rel-18

- R2-2210675	Draft LS on Scaling the RRM Parameters for UAV UE	CMCC	LS out	Rel-18	NR_UAV-Core	To:RAN4,RAN1

- R2-2210753	Discussion on flight path reporting and user consent for location reporting	Samsung	discussion	Rel-18	NR_UAV-Core

- R2-2209369	Subscription-based Aerial-UE Identification for NR	CATT	discussion	Rel-18	NR_UAV-Core

- R2-2209419	Subscription-based aerial-UE identification	OPPO	discussion	Rel-18	R2-2207234

- R2-2209447	Enhancements for subscription-based aerial-UE identification	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

- R2-2209755	Considerations on Subscription-based Identification for NR UAV	NEC Europe Ltd	discussion	Rel-18	NR_UAV-Core

- R2-2210162	Subscription-based aerial-UE identification for NR UAV	CMCC	discussion	Rel-18	NR_UAV-Core

- R2-2210176	Discussion on subscription based identification for NR UAV	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

- R2-2210505	Consideration on subscription-based UAV identification	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

- R2-2210739	Discussion on subscription-based aerial-UE identification for NR UAV	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_UAV-Core

- R2-2210781	OG0022_LS-MITRE-Engenuity Open Generation DAA input_PC5_DAA_RID_PRS OG0022 (contact: vivo)                MITRE Engenuity Open Generation 5G Consortium

- R2-2209531	On broadcasting UAV identification	Ericsson	discussion	Rel-18

- R2-2209923	UAV Identity broadcast and Identification	Beijing Xiaomi Mobile Software	discussion	Rel-18	NR_UAV-Core

- R2-2209935	Discussion on broadcasting remote id for UAV	Lenovo	discussion	Rel-18

- R2-2210220	UAV identification broadcast	Sony	discussion	Rel-18	NR_UAV

- R2-2212266	Uncrewed Aerial Vehicles in Rel-18 - Updated Workplan	Nokia, Nokia Shanghai Bell	Work Plan	Rel-18	NR_UAV-Core

- R2-2212267	SA2 Status for Uncrewed Aerial Vehicles in Rel-18	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2211190	Measurement Enhancement for UAV	OPPO	discussion	Rel-18

- R2-2211305	Measurement and reporting enhancements	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

- R2-2211404	Discussion on reducing measurement reporting and flight path update for UAV	Intel Corporation	discussion	Rel-18	NR_UAV-Core

- R2-2211452	Considerations on Measurement Reports Enhancements	NEC Europe Ltd	discussion	Rel-18	NR_UAV-Core

- R2-2211738	Measurement reporting enhancement in UAV	Apple	discussion	Rel-18	NR_UAV

- R2-2211739	User consent on UAV	Apple	discussion	Rel-18	NR_UAV

- R2-2211766	On measurement reporting enhancements in NR UAV	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_UAV-Core

- R2-2211798	Further consideration on measurement reporting for NR UAV	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

- R2-2211819	Discussion on flight path reporting for NR UAV	vivo	discussion	Rel-18	NR_UAV

- R2-2211820	Discussion on measurement reporting enhancement for NR UAV	vivo	discussion	Rel-18	NR_UAV

- R2-2211931	Considerations about UAV mobility and user consent	Sony	discussion	Rel-18	NR_UAV

- R2-2211996	Further discussion on NR support for UAV	NTT DOCOMO, INC.	discussion	Rel-18

- R2-2212019	Measurement enhancement for NR UAV	Lenovo	discussion	Rel-18

- R2-2212145	Measurement Reporting for NR UAV	CATT	discussion	Rel-18	NR_UAV-Core

- R2-2212268	On Measurement Related Aspects for UAV UEs	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2212269	On Flight Path Plan (FPP) for UAVs – Role, Content and Reporting Aspects	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2212340	Flight path reporting for UAV	InterDigital	discussion	Rel-18	NR_UAV-Core

- R2-2212616	Measurement Reporting for NR UAV	CMCC	discussion	Rel-18	NR_UAV-Core

- R2-2212638	Further discussion on UAV measurement enhancements	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

- R2-2212657	Discussion on measurement reporting for NR UAV	Xiaomi	discussion	Rel-18	NR_UAV-Core

- R2-2212669	Discussion on measurement reporting for NR UAV	Sharp	discussion

- R2-2212736	Consideration on flight path reporting of NR support for UAV	DENSO CORPORATION	discussion	NR_UAV-Core

- R2-2212800	Discussion on flight path reporting for NR UAV	China Telecom	discussion

- R2-2212824	Measurement Report Enhancement	LG Electronics Finland	discussion	Rel-18

- R2-2212846	Flight path information report Enhancement	LG Electronics Finland	discussion	Rel-18

- R2-2212900	Flight path reporting and UAV measurement reports 	Ericsson	discussion	Rel-18	NR_UAV-Core

- R2-2212933	Further discussion on NR support for UAV	NTT DOCOMO, INC.	discussion	Rel-18

- R2-2211191	Subscription-based aerial-UE identification	OPPO	discussion	Rel-18	R2-2209419

- R2-2211306	Enhancements for subscription-based aerial-UE identification	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core	R2-2209447

- R2-2211453	Considerations on Subscription-based Identification for NR UAV	NEC Europe Ltd	discussion	Rel-18	NR_UAV-Core

- R2-2211651	Discussion on subscription-based aerial-UE identification for NR UAV	Samsung Electronics Co., Ltd	discussion	Rel-18	NR_UAV-Core	R2-2210739

- R2-2211799	On subscription based identification for NR UAV	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

- R2-2212146	Subscription-based Aerial-UE Identification for NR	CATT	discussion	Rel-18	NR_UAV-Core

- R2-2212513	UAV Sub.UE Identification and identity broadcast	Beijing Xiaomi Mobile Software	discussion	Rel-18	NR_UAV-Core

- R2-2212617	Subscription-based aerial-UE identification for NR UAV	CMCC	discussion	Rel-18	NR_UAV-Core

- R2-2212639	Discussion on subscription-based UAV identification	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

- R2-2212898	Subscription-based aerial UEs identification 	Ericsson	discussion	Rel-18	NR_UAV-Core

- R2-2211125	OG0022_LS-MITRE-Engenuity Open Generation DAA input_PC5_DAA_RID_PRS OG0022 (contact: vivo)	MITRE Engenuity Open Generation 5G Consortium	LS in	NR_UAV-Core	To:SA2	Cc:RAN2

- R2-2211932	UAV identification broadcast	Sony	discussion	Rel-18	NR_UAV

- R2-2212020	Discussion on broadcasting remote id for UAV	Lenovo	discussion	Rel-18

TDocs submitted to RAN3:

- R3-225374	UAV Support over NG-RAN interface (NEC)	Discussion

- R3-225572	On subscription-based UAV UE identification (Nokia, Nokia Shanghai Bell)	Discussion

- R3-225618	Subscription-based Aerial-UE Identification for NR (CATT)	Other

- R3-225694	Initial discussion on NR support for UAV (Huawei)	Discussion

- R3-225804	Subscription-based aerial-UE identification for NR UAV (CMCC)	Discussion

- R3-225855	NR support for UAV WI (Ericsson)	Discussion

- R3-225856	Introduction of Aerial authorization information (Ericsson, AT&T, NTT DOCOMO, INC, Qualcomm Incorporated, Intel Corporation)

- R3-225875 	Discussion on NR support for UAV (ZTE)

- R3-225570	Uncrewed Aerial Vehicles in Rel-18 - workplan (Nokia, Nokia Shanghai Bell)

- R3-225933	Summary of Offline Discussion on CB: # UAV_Identification (Nokia – moderator)

- R3-226198	On subscription-based UAV UE identification (Nokia, Nokia Shanghai Bell)

- R3-226374	UAV Support over NG-RAN interface (NEC)

- R3-226458	Introduction of Aerial authorization information (Ericsson, AT&T, NTT DOCOMO, INC, Qualcomm Incorporated, Intel Corporation)

- R3-226459	NR support for UAV WI (Ericsson)

- R3-226595	Discussion on open issue for NR support for UAV (Samsung)

- R3-226616	Further discussion on NR support for UAV (Huawei)

- R3-226617	NR support for UAV over NG (Huawei, China Unicom, China Telecom, CATT)

- R3-226618	NR support for UAV over Xn (Huawei, China Unicom, China Telecom, CATT)

- R3-226633	Subscription based aerial UE identification in NGAP and XnAP (CATT)

- R3-226671	Discussions on UAV identifications (Intel Corporation)

- R3-226752	Discussion on NR support for UAV (ZTE)

- R3-226404	Uncrewed Aerial Vehicles in Rel-18 - updated workplan (Nokia, Nokia Shanghai Bell)

- R3-226791	Summary of Offline Discussion on CB: # 12_R18UAV (Nokia – moderator)

- R3-226840	Introduction of Aerial authorization information (Ericsson, AT&T, NTT DOCOMO INC, Qualcomm Incorporated, Intel Corporation, Samsung, NEC, Nokia, Nokia Shanghai Bell, Huawei, ZTE, CATT)

- R3-226841	NR support for UAV over Xn (Huawei, China Unicom, China Telecom, CATT, Intel Corporation, ZTE, Ericsson, Qualcomm, Nokia, Nokia Shanghai Bell, Samsung)

- R3-226823	Draft CR to 38.300 on NR support for UAV Nokia, Nokia Shanghai Bell, Huawei, Intel Corporation, ZTE, Ericsson, Samsung, CATT, Qualcomm, Deutsche Telekom, NEC) New CR to 38.300

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template