3GPP TSG RAN meeting #98e									          RP-223008

Electronic Meeting, December 12 - 16, 2022

## Status Report to TSG

Agenda item:			9.3.3.3

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_mobile_IAB |  |  |  |  |
| Unique ID | 941009 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222671 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: 06/2024 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 20% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

R2#119bis-e

Organizational:

Approved LS out to SA2, Cc: RAN, RAN3, RAN4, on FS_VMR solutions in R2-2211062.

Mobility enhancements:

Agreements on mobile-IAB-node to network indication:

- UE capability signalling is the baseline to let CU know that the MT is a “mobile-IAB” type. FFS early mobile-IAB indication, e.g. in Msg5.

- Regarding moving status/mode indication, R2 observes that legacy reporting of mobility state (e.g. mobilityState-r16) could be reused, and maybe also current location reporting from the UE. FFS whether any of this need to be enhanced or complemented, e.g. for the potential purpose of predictive mobility.

Agreements on network to mobile-IAB-node indication:

- FFS if to Introduce that stationary network broadcasts indication of “supporting mobile-IAB” (into intended for the Mobile IAB MT)

Agreements on mobile-IAB-Node to UE indications and UE mobility enhancements:

- RAN2 confirms that Mobile IAB need to work with legacy UEs.

- RAN2 observes that a UE could potentially consider itself on-board of a mobile-IAB cell, if the UE camps on/connects to a mobile IAB cell during a long period (i.e. the UE then need to know that this is such a cell). FFS the time. FFS if this is needed.

- RAN2 assume below for the UEs working in the mobile IAB cell (may be obvious):

  - Assumption 1: From the NW perspective of mobile-IAB cell, the principle of setting the legacy parameters (including cell (re)selection, cell reservations and access restrictions) does not change, compared to the legacy IAB cell.

  - Assumption 2: No spec impact to legacy UEs behaviors.

  - Assumption 3: Any R18 newly broadcasted info of mobile-IAB cell (if agreed) does not forbid/control the access of legacy UEs.

  - Assumption 4: Non-enhanced UEs (including legacy UEs and R18 UEs not supporting the enhancement) just ignore the R18 newly broadcasted info of mobile-IAB cell (if agreed).

- RAN2 assumption: For the mobile IAB cell broadcasting info:

  - 1 bit mobile-IAB cell type indication is introduced, to assist mobility in Idle/Inactive mode for Rel-18 UEs (FFS if to assist UE to know it is onboard, if this need to be known)

  - FFS how this is used (might be implementation specific).

- RAN2 has from the Mobile IAB WI perspective not identified any modifications to prevent the surrounding UE from accessing the mobile IAB-node, but believes that SA2 may be working on Rel-18 solutions that may be applicable (wait for SA2)

Agreements on Group Handover

- RAN2 assumes that O1 and O3 above could work, and FFS if O2 above (new trigger etc) is needed.

Others:

Agreements on UE handover during full migration

- RAN2 focuses on the scenario where, during full migration, the UE sees the two logical DU cells as different physical cells (e.g. with different PCI if same carrier), and where the two logical DU cells use separate physical resources (i.e., different carriers, or orthogonal time and frequency resources of the same carrier, as supported by legacy L1).

- No LS is needed

R2#120

Mobility enhancements:

Agreements on mobile-IAB-node to network indication:

- R2 assumes that it is up to RAN3 or SA2 to decide whether to support early mobile IAB indication in Msg5 because it depends whether donor CU needs to select an AMF supporting mobile IAB.

- R2 assumes that Donor CU can determine mobile IAB node's moving status via legacy reporting (e.g. mobility state and UE location / velocity specified in SON/MDT), i.e. R2 assumes enhanced / new reporting is not needed.

Agreements on network to mobile-IAB-node indication:

- A mobile IAB node may camp on and connect to legacy Rel-16/Rel-17 IAB capable cell.

- R2 assumes "supporting mobile-IAB" indication is provided by Rel-18 Mobile IAB capable parent cell.

Agreements on mobile-IAB-Node to UE indications and UE mobility enhancements:

- Regarding the assumed mobile-IAB cell type indication, RAN2 assumes it may be specified if some related UE behaviour is specified.

Other:

Agreements on PCI collision

- RAN2 assumes that PCI collision can be avoided, by reconfigurations, and this may be handled by RAN3. If RAN3 finds issues that RAN2 should work on then RAN2 can work. e.g. based on LS.

Agreements on RACH configuration collisions

- RAN2 understands that RACH interference and collisions may be avoided by RACH configuration, and RACH configurations can e.g. be exchanged by Xn, so RACH interference and collisions better be handled between RAN3 and RAN1, if needed.

#### 2.2.2	Remaining Open issues

25% of the items defined in the RAN2 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.3	RAN3

#### 2.3.1	Agreements

R2#117bis-e

General

Agreed LS out to SA2, Cc: RAN, RAN2, RAN4, on FS_VMR solutions in R3-2216048.

Agreements on IAB-node mobility

- mIAB-DU migration and mIAB-MT handover can be executed independently from each other. Details on the scenarios need to be further discussed

- For partial migration of mIAB-node, the inter-donor HO of mIAB-MT is decided and triggered by the donor CU serving the mIAB-MT.

- The donor CU serving the mIAB-DU decides whether to execute mIAB-DU migration or inter-donor F1 transport migration for the mIAB-DU.

- For inter-donor partial migration, the donor CU serving the mIAB-DU is informed about the mIAB-MT HO. FFS on signalling details concerning the indication.

- WA: The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the:

  - gNB ID of the target donor CU for the mIAB-MT HO.

  - ID(s) of the mIAB-MT. How the mIAB-MT ID is maintained across migrations needs to be further discussed

  - FFS: the TNL address of the target donor CU for the mIAB-MT HO.

- For partial migration of mIAB-node, the donor CU serving the mIAB-DU and the target donor CU for the mIAB-MT HO can directly exchange Xn IAB Transport Migration messages, in case direct Xn connectivity exists (or is established) between the two donor CUs.

- Focus first on the scenarios where Xn and IP connectivity are available between the source and target donors for IAB-MT HO and mIAB-DU migration.

- RAN3 to discuss support of mIAB-node mobility over NGAP. Which type of migration needs to be further discussed

- To hand over the UEs between the logical mIAB-DUs, the source donor CU for mIAB-DU migration should be notified about the cell IDs served by the second (target) logical mIAB-DU.

- The source donor CU for mIAB-DU migration should be informed that the second logical mIAB-DU has successfully established an F1 connection towards the target CU. Details are FFS.

Agreements on Mobility Enhancements

- After baseline procedures have been established, RAN3 to discuss the benefit and whether to support signaling of information related to multiple UE contexts in a single message for UE handover preparation, path switch, and context release procedures.

- As the baseline, F1 establishment and configuration of the new logical DU follows legacy procedures.

- RAN3 to discuss whether and which information can be shared between two logical DUs in case of IAB-DU migration.

- RAN3 to discuss which of the OAM-configured and network-configured parameters may be pre-configured at a mobile IAB-node, after a baseline procedure for IAB-DU migration is developed.

- Source donor CU of mobile IAB-MT informs the target donor CU of mobile IAB-MT that the migrating node is a mobile IAB-node, via explicit indication in XnAP HO Request message.

- RAN3 to discuss whether source donor should know whether the target cell belongs to a mIAB-Node.

- The NCGI of the mobile IAB-DU cell is changed when the F1-terminating donor CU of the mobile IAB-DU is changed.

- RAN3 to further discuss the following options for TAC/RANAC issue:

  - Option 1: The TAC/RANAC for the mobile IAB cell can be changed in order to reflect the physical location when the mobile IAB-node moves.

  - Option 2: Using static TAC/RANAC for mobile IAB when it moves. Involvement of SA2 may be needed

Agreements on Mitigation of interference

- PCI Space Partitioning is performed by OAM and up to implementation.

- As baseline, to avoid PCI collision, F1-terminating IAB-donor can reconfigure PCI for the cell of mobile IAB-DU via existing F1AP message.

- PCI-change on the IAB-node can be supported via handover of connected UEs between cells using old and new PCI, respectively.

- PCI collision can be detected by the F1-terminating IAB-donor of the mobile IAB-node.

R2#118

Agreements on IAB-node mobility

- WA: The mIAB-MT and its co-located mIAB-DU can be handed over/migrated to different donor CUs. This WA is subject to validation that the impact involved is affordable.

- The source donor CU for the mIAB-MT HO provides to the donor CU serving the mIAB-DU at least the:

  - gNB ID of the target donor CU for the mIAB-MT HO.

  - ID(s) of the mIAB-MT.

- In case the donor of the mIAB-DU decides the F1AP setup for DU migration, the donor of the mIAB-DU triggers via F1 signalling the IAB node to perform the F1 Setup procedure for the DU migration. An OAM based solution is not excluded.

- For the establishment of Xn, the mIAB-DU’s donor CU can obtain the TNL address of the mIAB-MT’s target donor CU via legacy mechanisms.

- The info sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU does not include the target donor CU’s TNL address.

- The mIAB-MT’s source donor CU can send the info on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO.

- The mIAB-MT ID sent by the mIAB-MT’s source donor CU to the mIAB-DU’s donor CU is the XnAP UE ID.

- The trigger for F1 setup between the mobile IAB-node’s second logical DU and its donor CU may be based on OAM or pre-configuration.

Agreements on Mobility Enhancements

- Static TAC solution is not pursued.

- RAN3 assumes that dynamic TAC solution should be supported.

- RAN3 to continue discussions on impacts (if any) of dynamic TAC solutions on RAN3 specs

- Send an LS to RAN2 (include SA2 in To) informing RAN2 of the decisions taken by RAN3

Agreed: LS out to SA2 and RAN2 on TAC handling for mobile IAB in R3-226831

#### 2.3.2	Remaining Open issues

25% of the items defined in the RAN3 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.4	RAN4

#### 2.4.1	Agreements

None. Work has not started.

#### 2.4.2	Remaining Open issues

0% of items defined in the RAN4 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#119bis-e:

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2209350 | LS on FS_VMR solutions review (S2-2207070; contact: Qualcomm) | SA2 |
| R2-2209522 | Mobile IAB mobility enhancement | Huawei, HiSilicon |
| R2-2209523 | Full migration, interference mitigation and SA2 LS related issues | Huawei, HiSilicon |
| R2-2209615 | Discussion on LS on VMR solutions from SA2 | ZTE, Sanechips |
| R2-2209616 | Discussion on mobility enhancement for mobile IAB | ZTE, Sanechips |
| R2-2209640 | Mobility Enhancement of mobile IAB-node and served UEs | Intel Corporation |
| R2-2209641 | Discussion on Migration and PCI handling of mobile IAB-node | Intel Corporation |
| R2-2209699 | Mobility enhancements for group mobility | AT&T |
| R2-2209702 | Workplan for Rel-18 mobile IAB | Qualcomm Inc. (Rapporteur) |
| R2-2209703 | Enhancements for IAB-node mobility | Qualcomm Inc. |
| R2-2209704 | Other enhancements for mobile IAB | Qualcomm Inc. |
| R2-2209763 | Mobility enhancement in mobile IAB | Apple |
| R2-2209764 | Inter-donor full migration and mitigation of interference in mobile IAB | Apple |
| R2-2209953 | Mobility enhancements for mobile IAB-node and its served UE | Lenovo |
| R2-2209954 | Discussion on inter-donor full migration of mobile IAB | Lenovo |
| R2-2209997 | Discussion on mobility enhancements for mobile IAB | CANON Research Centre France |
| R2-2210049 | mIAB - other key issues | Samsung R&D Institute UK |
| R2-2210109 | Discussion on UE handover during IAB-node mobility | Fujitsu |
| R2-2210208 | Mobility enhancement for mobile IAB | Sony |
| R2-2210209 | PCI collision in mobile IAB | Sony |
| R2-2210272 | RAN impacts due to IAB-node mobility | Nokia, Nokia Shanghai Bell |
| R2-2210273 | Interference mitigation | Nokia, Nokia Shanghai Bell |
| R2-2210327 | Mobility enhancements for mIAB node | Ericsson |
| R2-2210328 | General aspects on mobile IAB support | Ericsson |
| R2-2210387 | Discussion on mobile IAB open issues | vivo |
| R2-2210404 | Consideration on PCI collisions for Mobile IAB | Sharp |
| R2-2210429 | Mobility enhancements for mobile IAB | Kyocera |
| R2-2210430 | PCI and RACH collisions on mobile IAB | Kyocera |
| R2-2210447 | Scenarios for consideration in mIAB cell selection and reselection | Beijing Xiaomi Mobile Software |
| R2-2210522 | Discussion on the enhancement of IAB node mobility | Samsung R&D Institute UK |
| R2-2210548 | IAB node mobility state and UE measurements | InterDigital, Inc. |
| R2-2210562 | Handover and cell reselection enhancements for on-board UE mobility | LG Electronics |
| R2-2210577 | Dynamic PCI change for mobile IAB | InterDigital, Inc. |
| R2-2210591 | Consideration on full migration, PCI and RACH configuration collision | LG Electronics Inc. |
| R2-2210778 | RAN impacts due to IAB-node mobility | Nokia, Nokia Shanghai Bell |
| R2-2211021 | Report of [AT119bis-e][021][eIAB] Enhancements for Idle Inactive UE | Huawei, HiSilicon |
| R2-2211022 | [DRAFT] Reply LS on FS_VMR solutions review | Qualcomm |
| R2-2211054 | [AT119bis-e][022][eIAB] Dual Cells LS (AT&T) | AT&T (Rapporteur of the offline) |
| R2-2211056 | [AT119bis-e][020][eIAB] Reply LS on FS_VMR solutions review | Qualcomm (Rapporteur) |
| R2-2211062 | Reply LS on FS_VMR solutions review | RAN2 |

RAN2#120:

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2211163 | Reply LS on FS_VMR solutions review (R3-226048; contact: Qualcomm) | RAN3 |
| R2-2211374 | Mobility enhancement of mobile IAB-node and served UEs | Intel Corporation |
| R2-2211375 | PCI collision and TAC/RANAC update of mobile IAB | Intel Corporation |
| R2-2211472 | Workplan for Rel-18 mobile IAB | Qualcomm Inc. (Rapporteur) |
| R2-2211473 | Enhancements for IAB-node mobility | Qualcomm Inc. |
| R2-2211551 | IAB node migration with same physical cell resources at logical DUs | Nokia, Nokia Shanghai Bell |
| R2-2211621 | mIAB mobility enhancement aspects | Samsung Electronics Romania |
| R2-2211686 | Further discussion on mobility enhancement in mobile IAB | Apple |
| R2-2211687 | Further discussion on interference mitigation in mobile IAB | Apple |
| R2-2211804 | Discussion on mobile IAB Issues | vivo |
| R2-2211812 | Discussion on mobility enhancement | ZTE, Sanechips |
| R2-2211813 | Discussion on topology adaptation in mobile IAB scenario | ZTE, Sanechips |
| R2-2211879 | mIAB - other key issues | Samsung R&D Institute UK |
| R2-2211937 | PCI collision in mobile IAB | Sony |
| R2-2211938 | Handling of UE´s in RRC-Inactive mode | Sony |
| R2-2212015 | Mobile IAB mobility enhancement | Huawei, HiSilicon |
| R2-2212016 | BAP impact in DU migration and PCI/TAC/RANAC issues for mobile IAB | Huawei, HiSilicon |
| R2-2212030 | Mobility enhancements for mobile IAB-node and its served UE | Lenovo |
| R2-2212031 | Discussion on inter-donor full migration of mobile IAB | Lenovo |
| R2-2212187 | Enhancements for IAB node mobility | Nokia, Nokia Shanghai Bell |
| R2-2212431 | Discussion on mobility enhancements for mIAB node | Ericsson |
| R2-2212432 | General aspects on interference mitigation and migration for mobile IAB | Ericsson |
| R2-2212450 | PCI collisions for Mobile IAB | SHARP Corporation |
| R2-2212504 | Use of mobile IAB indication for UE cell selection and reselection | Beijing Xiaomi Mobile Software |
| R2-2212523 | Mobility enhancements for mobile IAB | Kyocera |
| R2-2212524 | PCI and RACH collisions on mobile IAB | Kyocera |
| R2-2212542 | On IAB node mobility state and associated UE behavior | InterDigital, Inc. |
| R2-2212651 | PCI allocation of mobile IAB cells | InterDigital, Inc. |
| R2-2212732 | Mobility enhancements for group mobility | AT&T |
| R2-2212756 | Consideration on full migration and interference mitigation | LG Electronics Inc. |
| R2-2212916 | Discussion on reselection and miscellaneous issues | LG Electronics |
| R2-2212917 | Connected mode mobility for onboard Ues | LG Electronics |
| R2-2212956 | Discussion on topology adaptation in mobile IAB scenario | ZTE Corporation, Sanechips |
| R2-2212970 | Enhancements for IAB node mobility | Nokia, Nokia Shanghai Bell |

RAN3#117bis-e:

| TDoc | Title | Source |
| --- | --- | --- |
| R3-225317 | LS on FS_VMR solutions review | SA2 |
| R3-225344 | Discussion of SA2 FS_VMR Solutions | Ericsson |
| R3-225345 | The Migration Procedure for Mobile IAB-Nodes | Ericsson |
| R3-225346 | Enhancements for IAB-Node Mobility | Ericsson |
| R3-225347 | paper | Ericsson |
| R3-225348 | PCI Collision Avoidance for Mobile IAB-Nodes | Ericsson |
| R3-225357 | Workplan for Rel-18 mobile IAB | Qualcomm Inc. (Rapporteur) |
| R3-225358 | [Draft] Reply LS on FS_VMR solutions review | Qualcomm Inc. |
| R3-225359 | Topology adaptation for mobile IAB | Qualcomm Inc. |
| R3-225360 | Enhancements for mobility of IAB-node and its served UEs | Qualcomm Inc. |
| R3-225361 | PCI collision avoidance for mobile IAB | Qualcomm Inc. |
| R3-225434 | Discussion on IAB-node mobility | Fujitsu |
| R3-225435 | Discussion on IAB full migration | Fujitsu |
| R3-225436 | Discussion on IAB-node mobility together with served UEs | Fujitsu |
| R3-225437 | Avoidance of resource collisions due to IAB-node mobility | Fujitsu |
| R3-225438 | Discussion on LS on VMR solutions from SA2 | ZTE |
| R3-225439 | Discussion on inter-donor migration in mobile IAB scenario | ZTE |
| R3-225440 | Discussion on enhancements to UE migration in mobile IAB scenario | ZTE |
| R3-225441 | Discussion on mobility enhancement for mobile IAB | ZTE |
| R3-225442 | Discussion on enhancement for PCI collision avoidance for mobile IAB | ZTE |
| R3-225452 | Discussion on the questions from SA2 LS on VMR | Nokia, Nokia Shanghai Bell |
| R3-225453 | [draft] Reply LS on FS_VMR solutions review | Nokia, Nokia Shanghai Bell |
| R3-225454 | IAB-DU migration based on dual-DU | Nokia, Nokia Shanghai Bell |
| R3-225455 | Discussion on mobility enhancements | Nokia, Nokia Shanghai Bell |
| R3-225456 | Mobile IAB interference mitigation | Nokia, Nokia Shanghai Bell |
| R3-225488 | Discussion on mobile IAB-node inter-donor topology adaptation | Lenovo |
| R3-225489 | Inter-donor full migration procedure of mobile IAB-node | Lenovo |
| R3-225490 | Mobility enhancements for mobile IAB-node and its served UE | Lenovo |
| R3-225491 | (TP to TS 38.401) Support for group mobility of mobile IAB-node | Lenovo |
| R3-225492 | Interference mitigation of mobile IAB-node mobility | Lenovo |
| R3-225531 | [Draft] Reply to LS on FS_VMR Solutions Review | Ericsson |
| R3-225549 | PCI Collision Avoidance with Mobile IAB | CANON Research Centre France |
| R3-225680 | Disscussion on partial migration for mobile IAB | Huawei |
| R3-225681 | Disscussion on full migration for mobile IAB | Huawei |
| R3-225682 | Group mobility for mobile IAB | Huawei |
| R3-225683 | Preconfiguration and information sharing for mobile IAB | Huawei |
| R3-225684 | Discussion on the PCI collision and TAC/RANAC issue for mobile IAB | Huawei |
| R3-225714 | Discussion on IAB-node mobility | Samsung |
| R3-225715 | Discussion on full migration procedure | Samsung |
| R3-225716 | Discussion on mobility enhancements | Samsung |
| R3-225717 | Discussion on mitigation of interference | Samsung |
| R3-225751 | Discussion on LS of FS_VMR solutions review | Xiaomi |
| R3-225752 | Discussion on IAB-node mobility | Xiaomi |
| R3-225753 | Discussion on mobility enhancement | Xiaomi |
| R3-225754 | Discussion on mitigation of interference | Xiaomi |
| R3-225936 | CB: # IAB1_General - Summary of email discussion | Qualcomm - moderator |
| R3-225937 | CB: # IAB2_Mobility - Summary of email discussion | Ericsson - moderator |
| R3-225938 | CB: # IAB3_MobEnh - Summary of email discussion | Huawei - moderator |
| R3-225939 | CB: # IAB4_IntMit - Summary of email discussion | Lenovo - moderator |
| R3-225980 | CB: # IAB3_MobEnh - Summary of email discussion | Huawei - moderator |
| R3-225981 | CB: # IAB4_IntMit - Summary of email discussion | Lenovo - moderator |
| R3-226009 | [Draft] Reply LS on FS_VMR solutions review | Qualcomm Inc. |
| R3-226016 | (TP for NR_mobile_IAB BL CR for TS 38.423): mobile IAB indication in HO preparation | Huawei |
| R3-226048 | Reply LS on FS_VMR solutions review | Qualcomm Inc. |

RAN3#118:

| TDoc | Title | Source |
| --- | --- | --- |
| R3-226186 | Reply LS on FS_VMR solutions review | RAN2 |
| R3-226267 | Workplan for Rel-18 mobile IAB | Qualcomm Inc. (Rapporteur) |
| R3-226268 | Topology adaptation for mobile IAB | Qualcomm Inc. |
| R3-226269 | Enhancements for mobility of IAB-node and its served UEs | Qualcomm Inc. |
| R3-226271 | Discussion on the DU Migration of Mobile IAB-node | CANON Research Centre France |
| R3-226309 | Discussion on IAB-DU consecutive partial migration and full migration | Nokia, Nokia Shanghai Bell |
| R3-226310 | Support for mobile IAB | Nokia, Nokia Shanghai Bell |
| R3-226311 | Discussion on mobility enhancements | Nokia, Nokia Shanghai Bell |
| R3-226312 | Mobile IAB interference mitigation | Nokia, Nokia Shanghai Bell |
| R3-226327 | Discussion on IAB-node consecutive partial migrations | Fujitsu |
| R3-226328 | Discussion on IAB full migration | Fujitsu |
| R3-226329 | Discussion on IAB-node mobility together with served UEs | Fujitsu |
| R3-226350 | (TP for NR_mobile_IAB BL CR for TS 38.401):Partial migration for mobile IAB | Huawei |
| R3-226351 | DU migration for mobile IAB | Huawei |
| R3-226352 | (TP for NR_mobile_IAB BL CR for TS 38.423): Mobility enhancement for mobile IAB | Huawei |
| R3-226353 | PCI collision for mobile IAB | Huawei |
| R3-226382 | Discussion on inter-donor migration in mobile IAB scenario | ZTE |
| R3-226383 | Discussion on enhancements to UE migration in mobile IAB scenario | ZTE |
| R3-226384 | Discussion on mobility enhancement for mobile IAB | ZTE |
| R3-226385 | Discussion on PCI collision avoidance for mobile IAB | ZTE |
| R3-226407 | Discussion on mobility enhancements for mobile-IAB | KT Corp. |
| R3-226414 | Migration Procedure for Mobile IAB-Nodes | Ericsson |
| R3-226415 | Enhancements of IAB-Node Mobility | Ericsson |
| R3-226416 | PCI Collision Avoidance for Mobile IAB-Nodes | Ericsson |
| R3-226435 | Discussion on mobile IAB-node inter-donor topology adaptation | Lenovo |
| R3-226436 | Inter-donor full migration procedure of mobile IAB | Lenovo |
| R3-226437 | Mobility enhancements for mobile IAB-node and its served UE | Lenovo |
| R3-226438 | Interference mitigation of mobile IAB-node mobility | Lenovo |
| R3-226603 | Discussion on IAB-node mobility | Samsung |
| R3-226604 | Discussion on mobility enhancements | Samsung |
| R3-226605 | Discussion on mitigation of interference | Samsung |
| R3-226730 | Discussion on IAB-node mobility | Xiaomi |
| R3-226731 | Discussion on mobility enhancement | Xiaomi |
| R3-226732 | Discussion on mitigation of interference | Xiaomi |
| R3-226826 | CB: # 35_IAB1_Mobility- Summary of email discussion | Qualcomm - moderator |
| R3-226831 | LS on static and dynamic TAC solutions for mobile IAB node | ZTE |

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template