3GPP TSG RAN meeting #98e										RP-223097

Electronic Meeting, December 12-16, 2022

Status Report to TSG

Agenda item:			9.2.9

| WI / SI Name | Study on low-power Wake-up Signal and Receiver for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_LPWUS |  |  |  |  |
| Unique ID | 940085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222644 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2023 |  |  |  |  |
| Overall Completion level | Study Item: 20 % |  |  |  |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 1 |
| --- | --- | --- |
| Rapporteur | Name | Xiaodong Shen |
|  | Company | vivo Communication Technology |
|  | Email | shenxiaodong@vivo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1 #110b-e

Evaluation on low power WUS

For future meetings on LP WUS:

Use the following terminology for future discussion,

- Main radio (MR): the Tx/Rx module operating for NR signals/channels apart from signals/channel related to low-power wake-up

- LP-WUR (LR): The Rx module operating for receiving/processing signals/channel related to low-power wake-up.

Agreement

For evaluation, 1 Rx chain for LP-WUS receiver is baseline.

Agreement

Both RRC IDLE/INACTIVE and CONNECTED modes are to be studied as part of the LP-WUS/WUR SI.

- FFS: Further prioritization if needed during the study item.

Agreement

Take the following power model for main radio for evaluation in LP-WUS/WUR SI,

- For IoT and wearable cases, reuse TR38.875 power model as baseline.

- For eMBB and other cases, reuse TR38.840 power model as baseline.

- Introduce ‘Ultra-deep sleep’ power state for main radio of UEs with LP-WUS receiver

  - FFS: The details of ‘Ultra-deep sleep’ power state

R1-2210512	FL summary#2 of evaluation on low power WUS	Moderator (vivo)

Agreement

- The following power models are used ‘Ultra-deep sleep’ power state for main radio for evaluation

| Power State | Relative Power (unit) | Ramp-up and down transition energy (Note1): (unit multiplied by ms) | Ramp-up time | Time for sync/re-sync |
| --- | --- | --- | --- | --- |
| Ultra-deep sleep | [0.015] | [2000 ~ 40000] - Study to converge on candidate numbers to use for evaluation - FFS: other values and reported by companies. - FFS: down-selection of the values, - companies are encouraged to provide details for down-selection | [400ms], FFS: 100ms | X |

Note1:

  - Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.],

  - Time for sync/re-sync consists of the procedure for [main radio to re-synchronization with the serving gNB etc.],

    - FFS: X and whether/how to have different values depending on other factors, e.g., signal-to-noise ratio

    - Companies can report the assumption of X in the initial evaluation.

  - Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated.

- The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync.

  - FFS whether/how to define ramp-down time, whether to separately describe the ramp-down energy consumption

Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state.

Note 3: The values inside of ‘[ ]’ are to be used as starting point of future study on LP-WUS

Agreement

The following power model for LP-WUR/WUS evaluation is considered,

  - Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring:

    - [0.001]

  - Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring:

    - [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4]

    - Other values are not precluded to be evaluated.

    - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation

  - No additional transition energy and transition time between ‘on’ and ‘off’ state as start point, FFS any transition energy and transition time if needed.

Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver.

Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion.

Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture)

FFS: LP-WUR power consumption values for FR2.

Agreement

For R18 LP-WUS/WUR power evaluation in RRC connected mode, the following can be considered,

- XR traffic model with evaluation methodologies and assumptions captured in TR 38.838.

- eMBB traffic model with evaluation methodologies and assumptions captured in TR 38.840

- Heartbeat traffic models in 3GPP TR 38.875.

- Other models are not precluded.

Company to further provide the followings,

- Parameters (e.g., frame rate, data rate, jitter range, DRX configurations and etc if needed.)

- How to use LP-WUS, e.g., LP-WUS to trigger/adapt PDCCH monitoring

- Other details if any

Agreement

- For LP-WUS coverage evaluation, the noise figure of LP-WUR is

  - Options : [9, 12, 15, 18, 21, 24], Other values can be reported by companies

- FFS: how to determine the NF option.

- The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion.

Agreement

For the performance evaluations of LP-WUS candidate designs, it is assumed that

- The miss-detection rate (MDR) of LP-WUS [1%],

- The false-alarm rate (FAR) of LP-WUS

  - [0.1%, 1%, 10%]

  - Other values are not precluded for studying reported by companies

- Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly

- The above values applied in both RRC CONNECTED and IDLE/INACTIVE mode.

- FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead

- FFS: Note: FAR should be evaluated both in the absence of gNB transmissions and in the presence of transmissions from gNB. Proponent to provide the details.

Agreement

For system impact analysis, the following performance metrics are considered to be provided,

| Performance Metric | Note |
| --- | --- |
| System overhead | expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations. |
| FFS: Capacity impact | [Evaluate the system capacity impact due to introducing of LP-WUS] |
| FFS: NW power consumption / Energy Efficiency | [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.] |

For power and latency evaluation of the LP-WUS, the following performance metrics are considered to be provided.

| Performance Metric | Note |
| --- | --- |
| Power consumption | Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme). |
| Latency | For IDLE/INACTIVE state, the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - FFS: if UE is not required to monitor a PO after wake-up, e.g., latency is the time interval between the data arrival time at the gNB and the time UE transmits the PRACH after LP-WUS detection. - sync/re-sync for main radio is included For CONNECTED state, TBD |
| FFS: UPT | FFS Note: it is for connected mode purpose. |

Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI

Companies to report the power consumption / power saving gain considering the FAR impact , latency considering MDR impact

Other performance metrics (e.g., mobility) can be reported by companies (if any)

Agreement

The following is assumed for RRC IDLE/INACTIVE evaluation,

| Parameters | Value |
| --- | --- |
| i-DRX cycle length | 1.28s and other values not precluded and reported by companies, consider both with PEI/ without PEI |
| e-DRX cycle length | 20.48s, 61.44s and other values not precluded, company to report which value(s) are used. Note: ‘ultra-deep sleep’ state can be assumed for eDRX whenever necessary for baseline UE |
| Number of POs in Paging Frame | 1 |
| Number of DRXs per PTW | 4 |
| Number of SSB before PO / PEI | 1, 2 or 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) company to report which value(s) are used Note: the assumptions is for MR wakes from ‘Deep sleep’ |
| Sync/re-sync after ultra-deep sleep | companies to report the timeline of sync/re-sync and X value, X is the time for sync/re-sync |
| RRM Measurement | Company to report whether and how the RRM measurement is assumed, e.g., whether RRM performed by main radio or LP-WUR, whether RRM is relaxed or not. |
| LP-WUS monitoring | Option 1: continuously monitoring Option 2: discontinuously monitoring, with [T] ms as the period for complete an on-and-off cycle, and [D] ms as the active time for monitoring LP-WUS every cycle. |
| Traffic | Option 1 (baseline): Per UE paging rate (R_E)= ([1%]) or ([0.1%]) or ([0.01%]) or ([0.001%]) within duration Y, [FFS Y is an i-DRX cycle length or an absolute time duration length] - R_G denotes as the group paging rate and R_E denotes as UE paging rate, and 1-R_G=(1-R_E)^N, where N is the number of UEs in the group, and N is [TBD] - FFS: how (R_G, R_E) for e-DRX derived from FFS: Option 2 (optional): Reusing TR 38.875 heart beat traffic model Model; FTP3. Packet size; 100 Bytes. Mean inter-arrival time; 60s (per UE paging rate≈2%). Model RRC connection phase power consumption as follows, RRC connection duration; [30ms]. Relative energy consumption of RRC connection block (Relative power x ms); [=3000]. Other options are not precluded can be reported by companies. |
| Others | Reported by companies |

Agreement

For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline.

- MIL is used as the metric for LP-WUS coverage evaluation

- urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded.

Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830.

Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss

Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

Low power WUS receiver architectures

Conclusion

RAN1 does not intend to mandate the implementation of any specific type(s) of LP WUR architecture at the UE.

- Note: this does not prevent RAN4 from defining requirements for LP WUR in the normative phase.

Agreement:

Study at least the following three types of receiver architectures for LP-WUR:

- Architecture with RF envelope detection

- Heterodyne architecture with IF envelope detection

- Homodyne/zero-IF architecture with baseband envelope detection

- Note: The details of each type of receiver architecture are discussed separately.

- Note: Above receiver architectures are considered suitable for OOK modulation. Some of the architectures can be applicable for other modulations such as FSK.

Agreement

Study the architecture with RF envelope detection based on at least the following diagram for LP-WUR.

- The RF signal is converted into baseband signal directly via an RF envelope detector.

- There is no Local Oscillator (LO) and no Phase-Locked Loop (PLL).

- 1-bit or multi-bit ADC is applied.

- Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied.

- High-Q matching network and/or RF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- FFS the support of band and/or carrier tuning

Agreement

Study the heterodyne architecture with IF envelope detection based on at least the following diagram for LP-WUR.

- The RF signal is down converted into IF signal via an RF mixer with a LO. The IF signal is converted into baseband signal via an IF envelope detection.

  - There may be one or multiple IF stages depending on design.

- The choice of the LO is one of the major factors that determines the power consumption.

  - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation.

  - FLL (frequency locked loop) may replace PLL for non-coherent detection.

- 1-bit or multi-bit ADC is applied.

- High-Q matching network and/or RF BPF and/or IF BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Some component(s), e.g., RF LNA and/or IF AMP and/or BB AMP, can be optionally applied.

- Image rejection filter or an image rejection mixer is required.

- FFS the support of band and/or carrier tuning

- FFS the choice of IF frequency range

Agreement

Study the homodyne/zero-IF architecture with baseband envelope detection based on at least the following diagram for LP-WUR.

- The RF signal is directly down converted into baseband signal via an RF mixer with a LO.

- Baseband envelope detection can be done either in analog domain or in digital domain depending on design, which is not explicitly shown in the diagram.

- The choice of the LO is one of the major factors that determines the power consumption.

  - Lower power consumption can be achieved by relaxing the accuracy and stability requirements of the LO. However, such increased frequency offset and phase noise should be taken into account in the design and evaluation.

  - FLL (frequency locked loop) may replace PLL for non-coherent detection.

- 1-bit or multi-bit ADC is applied.

- High-Q matching network and/or RF BPF and/or BB BPF [and/or BB LPF] can be used to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- No image rejection filter is required.

- Some component(s), e.g., RF LNA and/or BB AMP, can be optionally applied.

- FFS the support of band and/or carrier tuning

Agreement

Further study the receiver architectures for FSK, with two examples shown below:

- Example 1: parallel OOK receivers and a comparator circuit, e.g.,

  - Each path can be implemented using either of [the architecture with RF envelope detection,] heterodyne architecture with IF envelope detection, or homodyne/zero-IF architecture with baseband envelope detection.

- Example 2: using an FM-to-AM detector [or an FM detector]

  - Alt 1: Use an analog FM-to-AM detector with a similar architecture as for OOK (e.g. heterodyne or zero-IF architecture), except that the envelope detector is replaced by a FM-to-AM detector.

    - Analog FM-to-AM detector can be implemented at least in BB or low-IF.

  - Alt 2: Use a FM-to-AM detector [or an FM detector] implemented in digital domain after ADC, with a heterodyne or zero-IF architecture.

    - Digital FM-to-AM detector implementation can be considered as part of digital baseband processing.

    - Here is an example of using zero-IF architecture:

  - The FM-AM detector can be implemented using a frequency discriminator, which converts frequency variations into amplitude changes. It can be implemented in either analog domain (as in Alt 1) or digital domain (as in Alt 2).

    - One example, as shown in the figure below, is a conventional quadrature FM discriminator. It multiplies received frequency modulated signal with a phase shifted version, followed by a low pass filter. The amplitude of the output signal is proportional to the frequency of the input signal.

- Note: Other architectures are not precluded.

Agreement

For the analysis of a receiver architecture, companies are encouraged to provide at least the following (when applicable):

- Details of the receiver

  - Receiver architecture type

  - Assumed modulation/waveform/coding

  - Presence of a RF LNA / IF AMP / BB AMP, and the corresponding gain, if any

  - Local oscillator

    - Type of oscillator and the corresponding frequency accuracy/drifting

  - Handling of time/frequency impairments

  - Presence of PLL or FLL

  - ADC: sampling rate, bit-width

  - Assumed signal bandwidth and guard band, and frequency location within a carrier (including whether it is fixed or can be flexible)

  - RF/IF/BB filter characteristics (e.g. type of filter, order, cut-off frequency/frequencies), if any

  - Baseband processing (e.g., sequence correlation detection / decoding, other signal processing, if any)

  - Assumed frequency band(s) and the support of band and/or carrier tuning

  - Duty cycle handling of WUS and other signals (if any)

  - Interference rejection capability (including both adjacent-channel interference and interference from adjacent subcarriers occupied by legacy NR signals or other LP WUS)

  - Handling of inter-cell interference

  - Whether there is any mobility support function, e.g. measurement capability

- Performance metrics

  - Power consumption during active monitoring/reception and during off state (and breakdown if possible)

  - Noise figure

  - Sensitivity/coverage

  - Data rate

  - FFS: other performance metrics for, e.g., cost/complexity, interference rejection capability and inter-cell interference handling

- Note: The performance and design of receiver architecture is expected to be dependent on WUS design. This list can be updated later when the discussion on WUS signal/procedure design (AI 9.13.3) starts.

RAN1 #111

Evaluation on low power WUS

Agreement

For system impact analysis, the following performance metrics are considered to be provided,

| Performance Metric | Note |
| --- | --- |
| System overhead | expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations. |
| Capacity impact | Evaluate the system capacity impact due to introducing of LP-WUS Note: it is for UEs which are in connected mode. Definition is the same as in XR TR. |
| FFS: NW power consumption / Energy Efficiency | [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.] |

For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study

| Performance Metric | Note |
| --- | --- |
| Power consumption | Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme). |
| Latency | For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can monitor the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency . In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included |
| UPT | The definition is the same as in [TR38.840] Note: it is for connected mode purpose. |

Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI

Companies to report the power consumption / power saving gain considering the FAR impact, latency considering MDR impact

Other performance metrics (e.g., mobility) can be reported by companies (if any)

Agreement

Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2,

- The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where

  - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s

  - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group

    - FFS: Value of N

- For LP-WUS

  - Both per group and UE paging can be assumed.

Note：

- For i-DRX with i-DRX cycle duration Y second,

  - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF

  - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group

- For e-DRX with K i-DRX cycles duration, L PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second

  - Per UE paging probability is

    - RE = 1 – (1 – RE, REF )(K-L)Y/YREF for the first i-DRX cycle within the PTW

    - RE = 1 – (1 – RE, REF )LY/YREF for each of the remaining L-1 i-DRX cycles within the PTW

  - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group

  - L=4 (as agreed in RAN1#110bis)

Agreement

For MR, at least for FR1 evaluation,

- Number of SSBs for sync/re-sync for MR is up to 10

  - Companies to report timeline and energy consumption

- Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112

Agreement

The following power model for LP-WUR is used for evaluation for FR1,

| Power State | Relative Power (unit) | Transition energy: (unit multiplied by ms) | Ramp-up time TLR, ramp-up (ms) |
| --- | --- | --- | --- |
| Off | 0.001 | [TLR, ramp-up *(PON+POFF)/2] | TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up |
| On | 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed |  |  |

FFS: whether further categorization/sub-categorization is needed and how.

Low power WUS receiver architectures

Agreement

Include the following in the LS to RAN4:

RAN1 kindly asks RAN4 to take RAN1 agreements into account, study at least the LP WUR architectures that RAN1 identifies and provide feedback, potentially considering the aspects including but not limited to:

- The reasonable assumption on adjacent channel selectivity (ACS) assumption for the study and the impact on the LP WUR architectures and signal design

- The impact of adjacent subcarrier interference suppression/rejection on the LP WUR architectures if LP WUS is multiplexed with other signals/channels in frequency, including e.g.

  - The necessity of guard band (if needed, the minimum guard band) between LP WUS subcarriers and adjacent subcarriers

  - Whether it is feasible to have LP WUS location flexible within the carrier

- The feasible noise figure(s) for each type of LP WUR architectures

- Impact, if any, LP-WUS transmission on existing gNB emissions/compliance requirements

- The potential RF impairments to be considered include e.g. timing error, frequency error, image impact, LO leakage (DC offset) and flicker (1/f) noise

- Whether certain LP WUR architectures can support multi-band capability

- Note: RAN1 may or may not identify further architecture(s) for the study.

Include all agreements on 9.13.2. Mention that other agreements have been made in other AIs. Final LS is in R1-2212999.

Agreement

The following observation to be captured in TR38.869:

For the architecture with RF envelope detection,

- It can achieve relatively low power consumption due to the removal of LO/PLL.

- Interference suppression for adjacent channel interference requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- Interference suppression for interference from legacy NR signals and/or other LP WUS on adjacent subcarriers, if performed in RF, requires very high-Q matching network and/or RF BPF, which is challenging due to the high Q values and may require off-chip components.

- The support of multiple bands and/or carriers may require multiple high-Q matching networks and/or RF BPFs or multiple off-chip components.

- RF LNA can be applied to improve sensitivity, with the cost of additional power consumption.

- The noise figure can be relatively high.

Agreement

The following observation to be captured in TR38.869:

For homodyne/zero-IF architecture with baseband envelope detection,

- For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency.

- The matching network and RF BPF for LP WUR may or may not reuse those of the main radio.

- It is more effective and less complex to use BB BPF/LPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Using FLL instead of PLL consumes less power, but it may result in larger frequency error.

- It can suffer from LO leakage (DC offset) and flicker (1/f) noise. The impact may be alleviated by using BB BPF in some cases.

- RF LNA can be applied to improve sensitivity, with the cost of additional power consumption.

- The baseband envelope detection can be done in either analog domain (before ADC) or digital domain (after ADC).

Agreement

The following observation to be captured in TR38.869:

For heterodyne architecture with IF envelope detection,

- For the support of band and/or carrier tuning, the band and/or carrier tuning can be achieved via tuning the LO frequency.

- The matching network and RF BPF for LP WUR may or may not reuse those of the main radio.

- It is more effective and less complex to use IF BPF instead of high-Q matching network and/or RF BPF to suppress adjacent channel interference or interference from legacy NR signals and/or other LP WUS on adjacent subcarriers.

- Using FLL instead of PLL consumes less power, but it may result in larger frequency error.

- The IF frequency can be properly selected to avoid LO leakage (DC offset) and flicker (1/f) noise.

- Image rejection can be done via either image rejection filter or image rejection mixer.

  - Image rejection filter can be done in either RF or IF, which may require high-Q filter.

  - Image rejection mixer requires two-branch (I/Q) mixing with good matching in gain and phase, which consumes additional power.

- RF LNA and/or IF AMP can be applied to improve sensitivity, with the cost of additional power consumption.

L1 signal design and procedure for low power WUS

Agreement

- Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform

  - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS.

    - Note that above does not preclude DFT-S-OFDMA

- Study generation and link performance of multi-carrier (MC)-FSK waveforms

  - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS.

- Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS)

  - Other signal/channel structures are not precluded

- For next meeting, companies to provide input on aspects to consider that might impact link performance

Agreement

For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further

- whether BW of LP-WUS is configurable (implicitly or explicitly)

- size of guard band [FFS: within or outside of BW X], if any

- whether there is different X for Idle, Connected, Inactive modes

FFS: Whether FR2 is included in the scope of LP-WUS SI

Agreement

For a UE support LP-WUR in IDLE/INACTIVE mode,

- Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support,

  - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR

    - FFS: measurement metric

    - FFS: whether and how to identify cell/ tracking area

    - FFS: need for neighbouring cells

    - FFS: need for relaxation of existing RRM measurement requirements (for UE)

2.1.2	Remaining Open issues

- Remaining issues for evaluation methodology (including the use cases) & KPIs [RAN1]

- Remaining issues for low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1].

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms and their coverage availability, as well as latency impact. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

The following open issues need to be addressed:

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

The following open issues need to be addressed:

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #110b-e:

- R1-2208378	Evaluation of Low Power WUS and initial performance results	FUTUREWEI

- R1-2208379	Low Power WUS Receiver Architectures Considerations and Modeling	FUTUREWEI

- R1-2208380	Low Power WUS Design	FUTUREWEI

- R1-2208417	Evaluation methodology for LP-WUS	Huawei, HiSilicon

- R1-2208418	On architectures of LP-WUS receiver	Huawei, HiSilicon

- R1-2208419	Signal design and procedure for LP-WUS	Huawei, HiSilicon

- R1-2208481	Low Power WUS receiver architecture	TCL Communication Ltd.

- R1-2208482	L1 signal design and procedure for low power WUS	TCL Communication Ltd.

- R1-2208572	Discussion on evaluation on low power WUS	Spreadtrum Communications

- R1-2208573	Discussion on low power WUS receiver architectures	Spreadtrum Communications

- R1-2208574	Discussion on L1 signal design and procedure for low power WUS	Spreadtrum Communications

- R1-2208666	TR 38.869 skeleton for study on low-power wake up signal and receiver for NR	vivo

- R1-2208667	Work plan for low-power wake up signal and receiver for NR	vivo

- R1-2208668	Evaluation methodologies for R18 LP-WUS/WUR	vivo

- R1-2208669	Discussion on low power wake-up receiver architecture	vivo

- R1-2208670	Discussion on physical signal and procedure for low power WUS	vivo

- R1-2208686	Discussion on evaluation on LP-WUS	InterDigital, Inc.

- R1-2208687	Discussion on LP-WUS receiver architectures	InterDigital, Inc.

- R1-2208688	Discussion on L1 signal design and procedure for LP-WUS	InterDigital, Inc.

- R1-2208698	Low power WUS Evaluation Methodology	Nokia, Nokia Shanghai Bell

- R1-2208699	Low Power WUS receiver architectures	Nokia, Nokia Shanghai Bell

- R1-2208700	L1 signal design and procedure for low power WUS	Nokia, Nokia Shanghai Bell

- R1-2208843	Evaluation discussion on lower power wake-up signal	OPPO

- R1-2208844	Discussion on low power WUS receiver	OPPO

- R1-2208845	L1 signal design consideration on lower power wake-up signal	OPPO

- R1-2208960	Deployment scenarios and evaluation methodologies for low-power WUS	CATT

- R1-2208961	Design consideration of Low-Power WUS receiver	CATT

- R1-2208962	Physical layer signals and procedures for Low-Power WUS	CATT

- R1-2209075	Discussion on evaluations on LP WUS	Intel Corporation

- R1-2209076	Discussion on LP-WUS receiver architecture	Intel Corporation

- R1-2209077	Discussions on L1 signal design and procedure for low power WUS	Intel Corporation

- R1-2209158	Discussion on L1 signal design and procedure for LP-WUS	NEC

- R1-2209199	Evaluation on LP-WUS	ZTE, Sanechips

- R1-2209200	LP-WUS receiver architectures	ZTE, Sanechips

- R1-2209201	LP-WUS design and related procedure	ZTE, Sanechips

- R1-2209270	Evaluation on low power WUS	xiaomi

- R1-2209271	Discussions on L1 signal design and procedure for low power WUS	xiaomi

- R1-2209361	Discussion on evaluation methodology and applicable scenarios for low power WUR	CMCC

- R1-2209362	Discussion on L1 signal design for low power WUS	CMCC

- R1-2209502	Evaluation on low power WUS	MediaTek Inc.

- R1-2209503	Low power WUS receiver architectures	MediaTek Inc.

- R1-2209504	L1 signal design and procedure for low power WUS	MediaTek Inc.

- R1-2209605	On performance evaluation for low power wake-up signal	Apple

- R1-2209606	On low power wake-up receiver architectures	Apple

- R1-2209607	On the L1 signal design and procedures for low power wake-up signal	Apple

- R1-2209621	Discussion on low power WUS evaluation	Rakuten Symphony

- R1-2209622	Receiver architectures for low power WUS	Rakuten Symphony

- R1-2209634	Discussion on Low power WUS receiver architectures	Panasonic

- R1-2209635	Discussion on low power wake up signal design	Panasonic

- R1-2209665	Discussion on the evaluation methodology for low power WUS	Lenovo

- R1-2209666	Discussion on the receiver architecture for low power WUS	Lenovo

- R1-2209667	Discussion on the L1 signal design and procedure for low power WUS	Lenovo

- R1-2209685	Discussion on evaluation for low power WUS	Sharp

- R1-2209686	L1 signal design and procedure for low power WUS	Sharp

- R1-2209756	Evaluation on LP-WUS/WUR	Samsung

- R1-2209757	Receiver architecture for LP-WUS	Samsung

- R1-2209758	Signal design and procedure for LP-WUS	Samsung

- R1-2209766	Initial view on evaluation of low-power WUS	Rakuten Mobile, Inc

- R1-2209862	Evaluation framework for low power WUS	Ericsson

- R1-2209863	Low power WUS receiver architectures	Ericsson

- R1-2209864	L1 signal design and procedure for low power WUS	Ericsson

- R1-2210010	Evaluation methodology for LP-WUS	Qualcomm Incorporated

- R1-2210011	Receiver architecture for LP-WUS	Qualcomm Incorporated

- R1-2210012	L1 signal design and procedures for LP-WUR	Qualcomm Incorporated

- R1-2210051	Discussion on Evaluation on Low power WUS	EURECOM

- R1-2210052	Discussion on Low power WUS receiver architectures	EURECOM

- R1-2210054	Discussion on L1 signal design and procedure for low power WUS	EURECOM

- R1-2210169	Discussion on evaluation methodology for low power WUS	NTT DOCOMO, INC.

- R1-2210171	Discussion on L1 signal design and procedure for low power WUS	NTT DOCOMO, INC.

- R1-2210197	On LP-WUS evaluation	Nordic Semiconductor ASA

- R1-2210198	On LP-WUS architecture	Nordic Semiconductor ASA

- R1-2210199	On LP-WUS signal design	Nordic Semiconductor ASA

- R1-2210222	Evaluation for low power WUS	Sony

- R1-2210223	On LP-WUS receiver architectures	Sony

- R1-2210224	LP-WUS L1 signal design and procedures	Sony

- R1-2210238	Signal design and procedure for LP-WUS	Samsung

- R1-2210430	TR 38.869 skeleton for study on low-power wake up signal and receiver for NR	vivo

- R1-2210437	FL summary#1 of evaluation on low power WUS	Moderator (vivo)

- R1-2210479	Summary #1 on [110bis-e-R18-LP_WUS-02] LP WUR architecture	Moderator (Apple)

- R1-2210480	Summary #2 on [110bis-e-R18-LP_WUS-02] LP WUR architecture	Moderator (Apple)

- R1-2210512	FL summary#2 of evaluation on low power WUS	Moderator (vivo)

- R1-2210666	Summary #3 on [110bis-e-R18-LP_WUS-02] LP WUR architecture	Moderator (Apple)

- R1-2210668	FL summary#3 of evaluation on low power WUS	Moderator (vivo)

RAN1 #111:

- R1-2210850	Evaluation of Low Power WUS and Performance Results	FUTUREWEI

- R1-2210851	Low Power WUS Design	FUTUREWEI

- R1-2210908	Evaluations for LP-WUS	Huawei, HiSilicon

- R1-2210909	Discussion on architecture of LP-WUS receiver	Huawei, HiSilicon

- R1-2210910	Signal design and procedure for LP-WUS	Huawei, HiSilicon

- R1-2211030	Evaluation methodologies for R18 LP-WUS/WUR	vivo

- R1-2211031	Discussion on low power wake-up receiver architecture	vivo

- R1-2211032	Discussion on physical signal and procedure for low power WUS	vivo

- R1-2211067	Low Power WUS receiver architecture	TCL Communication Ltd.

- R1-2211068	L1 signal design and procedure for low power WUS	TCL Communication Ltd.

- R1-2211182	Remaining issues of Deployment scenarios and evaluation methodologies for low-power wakeup receiver	CATT

- R1-2211183	Low-Power WUS receiver Architectures and its performance	CATT

- R1-2211184	Physical layer signals and procedures for Low-Power WUS	CATT

- R1-2211252	Discussion on evaluation on low power WUS	Spreadtrum Communications

- R1-2211253	Discussion on low power WUS receiver architectures	Spreadtrum Communications, H3C

- R1-2211269	Low power WUS Evaluation Methodology	Nokia, Nokia Shanghai Bell

- R1-2211270	Low Power WUS Receiver Architectures	Nokia, Nokia Shanghai Bell

- R1-2211271	L1 signal design and procedures for low power WUS	Nokia, Nokia Shanghai Bell

- R1-2211319	Discussion on evaluation on LP-WUS	InterDigital, Inc.

- R1-2211320	Discussion on LP-WUS receiver architectures	InterDigital, Inc.

- R1-2211321	Discussion on L1 signal design and procedure for LP-WUS	InterDigital, Inc.

- R1-2211325	Power requirement for WUS	Everactive

- R1-2211326	FSK Architectures for WUR	Everactive

- R1-2211327	L1 WUS Considerations	Everactive

- R1-2211348	Evaluation on low power WUS	xiaomi

- R1-2211349	Discussions on L1 signal design and procedure for low power WUS	xiaomi

- R1-2211420	Discussion on evaluations on LP WUS	Intel Corporation

- R1-2211421	Discussion on LP-WUS receiver architecture	Intel Corporation

- R1-2211422	Discussions on L1 signal design and procedure for low power WUS	Intel Corporation

- R1-2211471	Evaluation for lower power wake-up signal	OPPO

- R1-2211472	Discussion on low power WUS receiver	OPPO

- R1-2211473	L1 signal design consideration on lower power wake-up signal	OPPO

- R1-2211546	Discussion on L1 signal design and procedure for low power WUS	Spreadtrum Communications

- R1-2211599	Discussion on low power wake up receiver architectures	Panasonic

- R1-2211600	Discussion on low power wake up signal design	Panasonic

- R1-2211628	Evaluation of low power WUS	Sony

- R1-2211629	L1 signal design and procedure for LP-WUS	Sony

- R1-2211704	Discussion on L1 signal design and procedure for LP-WUS	CMCC

- R1-2211834	On performance evaluation for low power wake-up signal	Apple

- R1-2211835	On low power wake-up receiver architectures	Apple

- R1-2211836	On the L1 signal design and procedures for low power wake-up signal	Apple

- R1-2211857	L1 signal design and procedure for low power WUS	Sharp

- R1-2211907	Evaluation on LP-WUS	ZTE, Sanechips

- R1-2211908	LP-WUS receiver architectures	ZTE, Sanechips

- R1-2211909	LP-WUS design and related procedure	ZTE, Sanechips

- R1-2212006	Discussion on evaluation methodology for low power WUS	NTT DOCOMO, INC.

- R1-2212007	Discussion on low power WUS receiver architectures	NTT DOCOMO, INC.

- R1-2212008	Discussion on L1 signal design and procedure for low power WUS	NTT DOCOMO, INC.

- R1-2212070	Evaluation on LP-WUS/WUR	Samsung

- R1-2212071	Receiver architecture for LP-WUS	Samsung

- R1-2212072	Signal design and procedure for LP-WUS	Samsung

- R1-2212142	Evaluation methodology for LP-WUS	Qualcomm Incorporated

- R1-2212143	Receiver architecture for LP-WUS	Qualcomm Incorporated

- R1-2212144	L1 signal design and procedures for LP-WUR	Qualcomm Incorporated

- R1-2212158	Evaluation framework for low power WUS	Ericsson

- R1-2212159	Low power WUS receiver architectures	Ericsson

- R1-2212160	L1 signal design and procedure for low power WUS	Ericsson

- R1-2212261	Evaluation on low power WUS	MediaTek Inc.

- R1-2212262	Low power WUS receiver architectures	MediaTek Inc.

- R1-2212263	L1 signal design and procedure for low power WUS	MediaTek Inc.

- R1-2212316	Evaluation of low power WUS	Rakuten Symphony

- R1-2212317	Receiver architectures for low power WUS	Rakuten Symphony

- R1-2212318	L1 signal design for low power WUS	Rakuten Symphony

- R1-2212366	Discussion on L1 signal design and procedure for LP-WUS	NEC

- R1-2212384	Discussion on Evaluation on Low power WUS	EURECOM

- R1-2212385	Discussion on L1 signal design and procedure for low power WUS	EURECOM

- R1-2212410	Discussion on the evaluation for low power WUS	Lenovo

- R1-2212411	Design consideration of Low-Power WUS receiver	Lenovo

- R1-2212412	Discussion on the L1 signal design and procedure for low power WUS	Lenovo

- R1-2212417	On LP-WUS evaluation	Nordic Semiconductor ASA

- R1-2212418	On LP-WUS architecture	Nordic Semiconductor ASA

- R1-2212419	On LP-WUS signal design	Nordic Semiconductor ASA

- R1-2212674	Summary #1 on LP WUR architecture	Moderator (Apple)

- R1-2212675	Summary #2 on LP WUR architecture	Moderator (Apple)

- R1-2212676	Summary #3 on LP WUR architecture	Moderator (Apple)

- R1-2212693	void	MCC ETSI

- R1-2212749	Summary #1 of discussions on L1 signal design and procedure for low power WUS	Moderator (Nordic Semiconductor ASA)

- R1-2212768	FL summary#1 of evaluation on low power WUS	Moderator (vivo)

- R1-2212866	Summary #2 of discussions on L1 signal design and procedure for low power WUS	Moderator (Nordic Semiconductor ASA)

- R1-2212899	FL summary#2 of evaluation on low power WUS	Moderator (vivo)

- R1-2212953	Draft LS to RAN4 on low-power wake-up receiver architectures	Moderator (Apple)

- R1-2212978	Final summary  of discussions on L1 signal design and procedure for low power WUS	Moderator (Nordic Semiconductor ASA)

- R1-2212999	LS to RAN4 on low-power wake-up receiver architectures	RAN1, Apple, vivo

- R1-2213005	Final FL summary of evaluation on low power WUS	Moderator (vivo)

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template