3GPP TSG RAN meeting #98e										       RP-223126

Electronic Meeting, December 12 – 16, 2022

## Status Report to TSG

Agenda item:			9.5.1.1

| WI / SI Name | Extending current NR operation to 71GHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_ext_to_71GHz |  |  |  |  |
| Unique ID | 860041 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222656 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2022 | Performance part: 03/2023 (changed from 12/2022) | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 85% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Jing Sun (RAN1); Aida Vera Lopez (RAN4) |
|  | Company | Qualcomm; Intel Corporation |
|  | Email | jingsun@qti.qualcomm.com; aida.l.vera.lopez@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

### 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

RAN1 completed the work. Remaining details will be completed in maintenance phase.

### 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

RAN2 completed the work. Remaining details will be completed in maintenance phase.

### 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

### 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	BS RF conformance testing

RAN4 #104Bis-e

MU contributors for FR2-2

- Agreements:

  - In principle, FR2-2 will follow the MU evaluation approach described in sub-clause 5.2 of TR 37.941

  - Whether additional calibration procedures can be considered is subject to companies’ further input. RAN4 shall focus on the MU values based on companies’ individual input.

  - MU values of the following components (described in TR 37.941, annexes A to C) needs to be updated:

    - A2-5: Mismatch of receiver chain (i.e., between receiving antenna and measurement receiver (a) / low power receiver (b))

    - A2-19: Uncertainty of the LNA (FR2 only)

    - A2-20: Uncertainty of the mixer, up/down converter (FR2 only)

    - B2-5: Mismatch of transmit chain (i.e., between transmitting measurement antenna and BS)

    - C1-1: Uncertainty of the RF power measurement equipment (e.g., spectrum analyzer, power meter, power sensor)

    - C1-2: Uncertainty of the RF signal generator

    - C1-3: Uncertainty of the network analyzer

    - C1-7: RF power measurement equipment (e.g., spectrum analyzer, power meter) - low power (UEM, absolute ACLR)

    - C1-8: RF power measurement equipment (e.g., spectrum analyzer, power meter) - relative (ACLR)

    - C1-9: RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2

    - MUOOBint = additional uncertainty for the OOB interferer signal

    - MUPA = uncertainty due to use of PA

  - MU values of other components can be maintained (i.e., no need to update 37 < f < 43.5 GHz for FR2-2)

Test equipment MU

- Agreements:

  - Table 2.4-1 from TR 37.941 will be used as baseline to collect MU values from companies for common test equipment

    - Including parameters VNA, SA, PM, SG and/or RF power measurement

  - FFS whether mismatch term can be considered and how to be reflected into MU budget

OTA chambers applicability

- Agreements:

  - MU evaluation approach described in TS 37.941 will be adopted for FR2-2, using the applicable tables in TR 37.941 as baseline to further discuss the applicable test methods

    - Technical analysis shall be provided and confirmed for the applicable test methods per test requirement

    - Confirmed test methods and corresponding MU evaluation will be included in TR 37.941 for FR2-2 per test requirement

  - Compact Antenna Test Range (CATR) is applicable for FR2-2 for the following tests:

    - EIRP for directional transmitter requirements

    - EIS for directional receiver requirements. Use of power amplifier needs to be assumed

    - Unwanted emission requirements

  - Define [142] GHz as the maximum test frequency for spurious emission measurements

Work-split

- Agreement: the work-split detailed in the table below for the RF related work of the FR2-2 NR BS conformance testing has been approved

| Section number | Section name in TS 38.141-2 | Assigned company |
| --- | --- | --- |
| 1 | Scope | Spec editor (Huawei) |
| 2 | References |  |
| 3 | Definitions, symbols, and abbreviations |  |
| 4 (except 4.1.2.2 and 4.1.2.3) | General radiated test conditions and declarations | Ericsson |
| 4.1.2.2 4.1.2.3 | Measurement of transmitter Measurement of receiver |  |
| 5 | Operating bands and channel arrangement | n/a |
| 6.2 | Radiated transmit power (EIRP) | Ericsson |
| 6.3 | OTA base station output power (TRP) | Nokia |
| 6.4 | OTA output power dynamics | Nokia |
| 6.5 | OTA transmit ON/OFF power | Nokia |
| 6.6 Annex L | OTA transmitted signal quality Annex L (normative): In-channel TX tests | Keysight |
| 6.7.1 6.7.2 6.7.3 | OTA unwanted emissions: General OTA occupied bandwidth OTA Adjacent Channel Leakage Power Ratio (ACLR) | Huawei |
| 6.7.4 | OTA operating band unwanted emissions | Huawei |
| 6.7.5 6.8 | OTA transmitter spurious emissions OTA transmitter intermodulation | Ericsson |
| 7.1 7.2 7.3 | Radiated receiver characteristics: General OTA sensitivity OTA reference sensitivity level | CATT |
| 7.4 7.5 | OTA dynamic range OTA in-band selectivity and blocking | CATT |
| 7.6 | OTA out-of-band blocking | Huawei |
| 7.7 7.8 | OTA receiver spurious emissions OTA receiver intermodulation | ZTE |
| 7.9 | OTA in-channel selectivity | ZTE |
| 8 | Radiated performance requirements | n/a |
| Annex A | Reference measurement channels | ZTE |
| Annex B, G, H, I, J, K |  | n/a |
| Annex C | Test tolerances and derivation of test requirements | All |
| Annex D | Calibration | TBD |
| Annex E | OTA measurement system set-up | TBD |
| Annex M | General rules for statistical testing | n/a |
| NOTE 1: Companies responsible for specific Tx/Rx requirement in section 6 and 7, are expected to additionally update related requirement’s specific MU and TT values in sections 4.1.2.2, 4.1.2.3, as well as in Annex C.1/C.2 – single tdoc is advised. NOTE 2: A single draft CR per company is advised. NOTE 3: Companies responsible for specific Tx/Rx requirement in section 6 and 7, are expected to additionally update respective MU derivation section in TR 37.941. |  |  |

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2217482 | Email discussion summary for [104-bis-e][306] NR_exto71GHz_BSRF | Moderator (Huawei) | Noted |
| R4-2217362 | WF on the list of MU contributors for FR2-2 | Nokia | Approved |
| R4-2217363 | WF on the test equipment MU | Keysight | Approved |
| R4-2217364 | WF on the OTA chambers applicability for FR2-2 | Ericsson | Approved |
| R4-2217365 | WF for work-split | Huawei | Approved |

RAN4 #105

Composite test equipment and additional calibration

- Agreement:

  - Consider both approaches case by case (per requirement) to verify when the power meter is usable and focus on the final MU introduction for each requirement

OTA chambers for the FR2-2 OTA test procedures

- Agreement:

  - Work on MU budget for both CATR and RC (other chambers are not precluded)

Occupied bandwidth

- Agreement:

  - Span for OBW measurement shall be twice the channel bandwidth

  - Minimum number of OBW measurement points for 100MHz and 400MHz CHBW shall be: 2×BWChannel200 kHz

  - Minimum number of OBW measurement points for 800MHz, 1600MHz, and 2000MHz CHBW shall be: 2×BWChannel800 kHz

EVM

- Agreement:

  - Increase EVM MU for FR2-2 by 0.1% to propose [1.1%]

Spurious emissions

- Agreement:

  - For MU of spurious emissions within 380MHz - 26GHz measured in RC, reuse the FR2-1 MU evaluation for FR2-2

Frequency sub-ranges for MU

- Agreement:

  - For spurious measurement,110GHz is the breaking point

OBW MU

- Agreement:

  - With measurement points resolution increased to 800 kHz, OBW MU to be increased to [2400 kHz] for those CHBWs

Test equipment MU

- Agreement:

  - The following ranges are tentatively agreed and will be further discussed to reach final MU values

Table 1: TE MU values for in-band measurements

| UID | Uncertainty source | Standard uncertainty values [dB] 52.6 < f ≤ 71 GHz |
| --- | --- | --- |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g., spectrum analyzer, power meter) - high power (EIRP, TRP) | [0.3 - 2.36] |
| C1-7 | RF power measurement equipment (e.g., spectrum analyzer, power meter) - low power (UEM, absolute ACLR) | [0.7 - 2.36] |
| C1-8 | RF power measurement equipment (e.g., spectrum analyzer, power meter) - relative (ACLR) | [0.7 - 2.36] |
| C1-9 | RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2 | [1.65 - 2.61] |
| C1-2 | RF signal generator | [0.3 - 2.37] |
| C1-3 | Uncertainty of the network analyzer | [0.4 - 0.87] |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.30 |

Table 2: TE MU values for out-of-band measurements

| UID | Uncertainty source | Standard uncertainty values [dB] 71 < f ≤ 142 GHz |
| --- | --- | --- |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g., spectrum analyzer, power meter) - low power (UEM, absolute ACLR) | [0.6 -2.49] |
| C1-3 | Uncertainty of the network analyzer | [0.3 - 1] |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.30 |

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2220132 | Summary for [105][306] NR_exto71GHz_BSRF | Moderator (Huawei) | Noted |
| R4-2220280 | WF for FR2-2 MU calculation | Huawei | Approved |
| R4-2219970 | draft CR to TS 38.141-2 on FR2-2 implementation (sections 1,2,3,6.7.1, 6.7.2, 6.7.3, 6.7.4, 7.6) | Huawei, HiSilicon | Endorsed |
| R4-2220200 | draftCR to 38.141-2: OTA OBW requirements for FR2-2 | NEC | Endorsed |
| R4-2220201 | Draft CR to TS 38.141-2 in clauses 6.3 to 6.5 for extending current NR operation to 71GHz | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2220204 | draftCR to 38.141-2: 71 GHz Extension BS EVM Clause 6.6 and Annex L (6.6, L) | Keysight Technologies UK Ltd | Endorsed |
| R4-2220208 | Draft CR for 38.141-2, On FR2-2 BS RF conformance testings in clauses 7.1-7.5 and respective MU and TT | CATT | Endorsed |
| R4-2220211 | draft CR to TS38.141-2 Clause 7.7, 7.8 | ZTE Corporation | Endorsed |
| R4-2220212 | draft CR to TS38.141-2 Clause 7.9 | ZTE Corporation | Endorsed |
| R4-2220287 | Draft CR to TS 38.141-2: Addition of FR2-2 aspects in clause 4 | Ericsson | Endorsed |
| R4-2220307 | big draft CR to TS38.141-2 FR2-2 RF conformance introduction | Huawei | Reserved |

##### 2.4.1.2	Demodulation performance

RAN4 #104Bis-e

General aspects

SCS and CBW for demodulation requirements

- Agreement: RAN 4 will define 120kHz and 480kHz SCS and not consider 960kHz SCS for FR2-2 PUSCH, PUCCH and PRACH requirements

SNR limit

- Agreement: Use 20dB SNR limit in discussions; limit might be revised if there are further conclusions in the RF session

CR work split agreement

# CR split for TS 38.104

| TS 38.104 - BigCR | Intel |
| --- | --- |
| PUSCH requirements | Intel |
| PUCCH requirements | Huawei |
| PRACH requirements | Nokia |
| PUSCH FRC | Ericsson |
| PRACH Test preambles, propagation conditions | Samsung |

# CR split for TS 38.141-2

| TS 38.141-2 - BigCR | Nokia |
| --- | --- |
| PUSCH requirements | Huawei |
| PUCCH requirements | Intel |
| PRACH requirements | Nokia |
| PUSCH FRC | Ericsson |
| PRACH Test preambles, propagation conditions | Samsung |

PUSCH demodulation

Channel bandwidth for PUSCH requirement with 120kHz SCS

- Agreement: Define PUSCH BS demodulation requirements for 120kHz SCS with 100MHz and 400MHz

Channel bandwidth for PUSCH requirement with 480kHz SCS

- Agreement: Define PUSCH BS demodulation requirements for 480kHz SCS with 400MHz

PTRS configuration

- Agreement: Disable PTRS for QPSK for both CP-OFDM and DFT-S-OFDM

Parameters for PUSCH requirements

List of test cases for FR2-2 PUSCH

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD30-200Note 1 | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 20 Note 2 | TDLD30-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 400 | 4 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 400 | 16 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD10-200Note 1 | 2x2 Low | 70% of maximum TP |
| 120 | 400 | 20 Note 2 | TDLD30-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD10-200Note 1 | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 20 Note 2 | TDLD10-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| NOTE 1: Confirmation on the channel model for MCS16 with 2x2 is pending NOTE 2: Confirmation on the use of MCS18 of MCS20 with 2x2 is pending |  |  |  |  |  |

PUCCH demodulation

SCS and channel bandwidth for PUCCH requirements

- Agreement: 120kHz/100MHz and 480kHz/400MHz

PUCCH configuration tables

Test configurations for PUCCH format 0

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 1 | 120 | 100 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |
|  |  | TDLA10-650 Low | 1 | 480 | 400 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% |

Test configurations for PUCCH format 1

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 2 | 120 | 100 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<0.1% |
|  |  | TDLA10-650 Low | 2 | 480 | 400 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<1% |

Test configurations for PUCCH format 2

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 120 | 100 | 1 | 4 | 4 | N/A | Prob(DTX to ACK)<1% and Prob(ACK miss)<1% |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | UCI BLER < 1% |
|  |  | TDLA10-650 Low | 480 | 400 | 1 | 4 | 4 | N/A | Prob(DTX to ACK)<1% and Prob(ACK miss)<1% |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | UCI BLER < 1% |

DMRs configuration for PUCCH format 3

- Agreement: RAN4 to align DRMS configuration of PUCH format 3 in FR2-2 with the existing configuration for FR2-1

  - Adopt DRMS configuration 1+1 and 1+0 for PUCCH format 3 with 14 OFDM symbols

  - Adopt DRMS configuration 1+0 for PUCCH format 3 with 4 OFDM symbols

Test configurations for PUCCH format 3

| Number of TX antennas | Number of demodulation branches | Modulation order | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA30-650 Low | 120 | 100 | 14 | 1 | 16 | Enabled | 1+0 and 1+1 | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% |
|  |  |  | TDLA10-650 Low | 480 | 400 | 14 | 1 | 16 | Enabled | 1+0 and 1+1 | Prob(UCI BLER)<1% |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% |

Test configurations for PUCCH format 4

| Number of TX antennas | Number of demodulation Branches | Modulation order | Propagation conditions and correlation matrix | Length of the orthogonal cover code | Index of the orthogonal cover code | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | QPSK | TDLA30-650 Low | n2 | n0 | 120 | 100 | 14 | 1 | 22 | Enabled | 1+0 and 1+1 | Prob(UCI BLER)<1% |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | 1+0 and 1+1 | Prob(UCI BLER)<1% |
|  |  |  | TDLA10-650 Low | n2 | n0 | 480 | 400 | 14 | 1 | 22 | Enabled | 1+0 and 1+1 | Prob(UCI BLER)<1% |
|  |  |  |  |  |  |  |  | 14 | 16 | 22 | Enabled | 1+0 and 1+1 | Prob(UCI BLER)<1% |

PRACH demodulation

SCS for PRACH requirements

- Agreement: focus on 120kHz and 480kHz SCSs to introduce FR2-2 BS demodulation requirements in Rel-17

Channel model

- Agreement: RAN4 will define PRACH demodulation requirements using AWGN and TDL-A 30-650 channel model for 120kHz SCS and using AWGN and TDL-A 10-650 channel model for 480kHz SCS

Ncs, logical sequence index combinations

- Agreement: RAN4 will define PRACH demodulation performance requirements using following Ncs parameters

  - For requirements with LRA = 139, use NCS = 69

  - For requirements with LRA = 571, use NCS = 285

  - For requirements with LRA = 1151, use NCS = 575

PRACH time error tolerance general aspects

- Agreements:

  - Not to define time error tolerance that is smaller than minimum possible step for the timing advance

  - Reuse calculation of PRACH time error tolerance for fading channel from Rel-15 and Rel-16

PRACH time error tolerance for 120kHz SCS

- Agreement: Reuse calculation of PRACH time error tolerance for 120kHz SCS requirements with AWGN and TDLA30

PRACH time error tolerance for 480kHz SCS

- Agreement: Use the configuration below as time error tolerance

| SCS | Channel model | Time error tolerance (ns) |
| --- | --- | --- |
| 480 | AWGN | 18 |
|  | TDLA10-650 | 68 |

UE demodulation

Maximum testable SNR

- Updated version of Table 7.2.3-2 in TR 38.884 (v18.2.0), which contains the Maximum DL testable SNR

Table 7.2.3-2: Maximum DL testable SNR preliminary extension for band n263 CBW (MHz); Test method. IFF. Single band UE; 100; [9.8]. 400; [2.6]. 800; [-2.3]. 1600; < -20 (NOTE 1). 2000; < -20 (NOTE 1). Multi band UE; 100; TBD. 400; TBD. 800; TBD. 1600; TBD. 2000; TBD. NOTE 1: Result does not converge.

Requirements with 800MHz CBW for 480kHz SCS

- Agreement: Requirements will not be introduced

Requirements with 960kHz SCS

- Agreement: RAN4 focus on 120kHz and 480kHz SCS for introducing UE demodulation requirements in FR2-2. It’s not precluded to discuss and introduce requirements for 960kHz SCS in future releases

SDR test for FR2-2 in Rel-17

- Agreement: SDR test for FR2-2 will not be defined in Rel-17. Further discussion in future releases is not precluded.

PDSCH requirements with 30% of peak throughput

- Agreement: Add one test case for 120kHz case only with 30% peak throughput

Requirements with 650Hz Doppler Frequency

- Agreement: Requirements will be introduced

Design margin for PN models in TR 38.808 Set 1

- Agreement: According to the PN for UE, Example 1 in 38.808, use: BS Margin = 0dB; UE margin 5dB

PDSCH requirements

MCS Choice for 120kHz/100MHz for 70% requirements

- Agreement: MCS 4, 13, 17 are agreed; Further discuss for MCS 20

Requirements with MCS17 in FR2-2 for 480kHz/400MHz

- Agreement: PDSCH requirements will not be introduced

MCS Choice for 120kHz/100MHz for 70% requirements

- Agreement: MCS 4, 13 are agreed

RB allocation for 120kHz/100MHz for MCS 4, 13 for 70% requirements

- Agreement: For 120kHz/100MHz, agree on 66RBs (full) allocation for MCS at least up to 13

RB allocation for 480kHz/400MHz for MCS 4 for 70% requirements

- Agreement: 66 RBs

RB allocation for 480kHz/400MHz for MCS 17 for 70% requirements

- Agreement: Do not include MCS 17 for 480kHz

Propagation channels for PDSCH requirements for 120kHz/100MHz

- TDLA30-650 for MCS 4 and MCS 13 (30%)

- TDLA30-200 for MCS 13 (70%)

- TDLD30-200 for MCS 17 and above

Propagation channels for PDSCH requirements for 480kHz/400MHz

- For PDSCH requirements for 480kHz/400Mhz: following channel model adopted:

  - TDLA10-200 for MCS 4

  - TDLD10-200 for MCS 13 and above

MCS and RB allocation choice for 120kHz/100MHz for 30% Requirements

- Recommended WF: MCS 13, 66RBs

PDSCH requirements table

| SCS (KHz)/ CBW (MHz) | MCS | Propagation Channel | Antenna Conf. | Throughput | Num PRB |
| --- | --- | --- | --- | --- | --- |
| 120kHz/100MHz | MCS4 | TDLA30-650 | 2x2 Low | 70% | 66 |
|  | MCS13 | TDLA30-200 | 2x2 Low | 70% | 66 |
|  | MCS13 | TDLA30-650 | 2x2 Low | 30% | 66 |
|  | MCS17 | TDLD30-200 | 2x2 Low | 70% | [66, 32] |
|  | MCS20 | TDLD30-200 | 2x2 Low | 70% | [32] |
| [120kHz/400MHz] | [TBD] | [TBD] | 2x2 Low | [TBD] | [TBD] |
| 480kHz/400MHz | MCS4 | TDLA10-200 | 2x2 Low | 70% | 66 |
|  | MCS13 | TDLD10-200 | 2x2 Low | 70% | [16,20,32] |

PDSCH requirements for CA

Applicability rule for FR1+FR2-2 CA requirements

- Agreement: Copy all cases of single CC to CA and define applicability rules as follows:

  - If UE supports standalone operation, UE can only be tested cases with single CC

  - If UE does not support standalone operation, UE can only be tested cases with CA

Test cases for FR1+FR2-2 CA requirements

- Agreement: Reuse all test cases of single CC to CA requirements

How to define FR1+FR2-2 CA requirements

- Agreement: Only define the requirements for FR2-2 CC

  - FFS whether all single CC requirements will apply to CA requirements

PDSCH simulation assumptions

- Agreements:

  - TE will not transmit OCNG on PDSCH slots during PDSCH testing

  - Partial bandwidth allocation position in the CBW: center

  - Choice of Noh PRB for TBS calculations: NOHPRB = 6

70% peak throughput

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [kHz] | 120 kHz, 480 kHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (10 ns and 30 ns delay spread) TDLD (10 ns and 30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): Up to MCS 20 (64QAM) |
| NOH | 6 |
| Number of HARQ Processes | 8 for 120kHz and 16 for 480kHz SCS |

30% peak throughput

| Assumptions | Value |
| --- | --- |
| Carrier Frequency [GHz] | 70 GHz |
| Subcarrier Spacing [kHz] | 120 kHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) |
| Antenna configuration | 2x2 ULA Low |
| Velocity | 3 km/h, 10 km/h |
| PA Model | None |
| gNB PN Model | None |
| UE PN Model | Up to UE implementation |
| Pre-loaded Tx EVM | 6% |
| Additive Rx EVM | 0% |
| I-Q Imbalance | None |
| Frequency Offset | 0 ppm |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 1 |
| DMRS Configuration | 2 DMRS symbols at (2,11) symbol index |
| PTRS Configuration | For CP-OFDM: (K = 2, L = 1) |
| MCS/TBS | From MCS Table 1 (TS38.214): MCS 13 |
| NOH | 6 |
| Number of HARQ Processes | 16 |

CSI Reporting requirements

- Agreement: CQI requirements including 16QAM will be defined

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2217483 | Email discussion summary for [104-bis-e][319] NR_exto71GHz_Demod_Part1 | Moderator (Nokia) | Noted |
| R4-2217484 | Email discussion summary for [104-bis-e][320] NR_exto71GHz_Demod_Part2 | Moderator (Qualcomm) | Noted |
| R4-2217371 | WF on general aspects for FR2-2 demodulation requirements | Huawei | Approved |
| R4-2217372 | WF on PUSCH demodulation requirements for FR2-2 | Nokia | Approved |
| R4-2217373 | WF on PUCCH demodulation requirements for FR2-2 | Ericsson | Approved |
| R4-2217374 | WF on PRACH demodulation requirements for FR2-2 | Samsung | Approved |
| R4-2217391 | WF for FR2-2 UE demodulation and CSI requirements | Qualcomm Inc. | Approved |
| R4-2215585 | Draft CR for Introducing Propagation channel models for requirements in FR2-2 | Apple Inc. | Endorsed |
| R4-2215586 | Draft CR for Introducing FRCs for requirements in FR2-2 | Apple Inc. | Endorsed |
| R4-2216011 | BigCR: Introduction of FR2-2 UE demodulation and CSI requirements in 38.101-4 | Huawei, HiSilicon | Endorsed |
| R4-2216182 | Draft CR to 38.101-4 for FR2-2 Demod - PBCH Requirements | Qualcomm Inc. | Endorsed |
| R4-2217376 | Draft CR 38.104: PUSCH requirements for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Endorsed |
| R4-2217377 | Draft CR 38.141-2: PUCCH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2217378 | draftCR for TS38.104 introduce FRC tables for FR2-2 PUSCH requirements | Ericsson | Endorsed |
| R4-2217379 | draftCR for TS38.141-2 introduce FRC tables for FR2-2 PUSCH requirements | Ericsson | Endorsed |
| R4-2217380 | Draft CR: Introduction of FR2-2 PUSCH radiated conformance testing requirements in TS 38.141-2 | Huawei, HiSilicon | Endorsed |
| R4-2217384 | Draft CR Introduction of FR2-2 PUCCH performance requirements in TS 38.104 | Huawei, HiSilicon | Endorsed |
| R4-2217387 | Draft CR 38.104: PRACH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2217388 | Draft CR 38.141-2: PRACH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2217389 | Draft CR on annex for PRACH requirement for TS 38.104 | Samsung | Endorsed |
| R4-2217390 | Draft CR on annex for PRACH requirement for TS 38.141-2 | Samsung | Endorsed |
| R4-2217394 | Draft CR to 38.101-4 for FR2-2 Demod - General section | Qualcomm Inc. | Endorsed |
| R4-2217395 | draft CR on PDSCH requirements for 52.6 - 71 GHz band | Ericsson | Endorsed |
| R4-2217398 | Draft CR: Introduction of FR1+FR2-2 CA PDSCH performance requirements in TS 38.101-4 | Huawei, HiSilicon | Endorsed |
| R4-2217399 | Nokia_DraftCR_38101-4_PDCCH | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2217403 | Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz | Apple Inc. | Endorsed |
| R4-2217516 | Big draftCR to 38.104: demodulation requirements introduction for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Endorsed |
| R4-2217517 | Big draftCR to 38.141-2: demodulation requirements introduction for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Endorsed |

RAN4 #105

UE demodulation

Extended maximum Testable SNR for reduced allocation

- Agreement:

  - The table below, containing the maximum DL testable SNR for band n263 used for the definition of the requirements, is agreed

|  | CBW (MHz) | Num RBs / SCS (kHz) | Test method |
| --- | --- | --- | --- |
|  |  |  | IFF |
| Single band UE | 100 | 66 / 120 | [9.8] |
|  |  | 32 / 120 | [13.2] |
|  | 400 | 66 / 480 | [2.6] |
|  |  | 32 / 480 | [6.6] |
|  |  | 20 / 480 | [8.9] |

PDSCH Requirements for FR2-2 Single Carrier

- Agreements:

  - RB allocation for 480kHz/400MHz for MCS 13 for 70% requirements is 20RBs

  - Reduce PDCCH CORESET BW if it is larger than PDSCH RB allocation

PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=32RBs

- Agreement:

  - If PDSCH requirements with PDSCH allocation = 32RBs are introduced, Option 1 (below) shall be the starting point of further discussions

    - Option 1: CORESET BW = 30PRB, Duration = 1Sym, AL = 4

PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=20RBs

- Agreement:

  - If PDSCH requirements with PDSCH allocation = 20RBs are introduced, Option 1 (below) shall be the starting point of further discussions

    - Option 1: CORESET BW = 18PRB, Duration = 2Sym, AL = 4

RF impairments for PDSCH Demodulation requirements

- Agreement:

  - Margin of impairments is up to UE implementation

PDSCH Requirements with FR1+ FR2-2 CA

- Agreements:

  - CA requirements for FR1 + FR2-2 will reuse single carrier requirements

    - Whether to introduce CA requirements for cases not covered by single carrier requirements is FFS

  - FR1 Carrier configuration for CA requirements will follow the table below

Table 2-4: Proposed test parameters for Pcell CC

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| TDD pattern |  |  | 7D1S2U S=6D:4S:4U |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | See FR1.30-1 as defined in Annex A.1.2 |

Scheduling and feedback pattern for FR1+FR2-2 CA requirements

- Agreement:

  - For FR1+FR2-2 with 120kHz SCS, PDSCH is scheduled in slot 0,1,2,3,5,6,7,8 in every 40 slots of FR2-2 considering 8 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1

  - For FR1+FR2-2 with 480kHz SCS, PDSCH is scheduled in slot 0~14 and slot 20 in every 160 slots of FR2-2 considering 16 HARQ processes is assumed. HARQ-ACK is transmitted in slot 8 every 10 slots of FR1

Channel Model for FR2-2 PDCCH requirements Test #5 (480kHz SCS, AL4)

- Agreement: Remove test #5 for PDCCH due to testable SNR limitation

Channel Model for FR2-2 PBCH requirements Test 1-1 (120kHz SCS)

- Agreement: TDLA30-650

Rank to use for CQI reporting requirements

- Agreement: RANK 1

SNR Test point for high modulation order

- Agreement: [7/8] dB

SNR Test point for low modulation order

- Agreement: 0/1 dB

BS demodulation

Channel model for PUSCH requirements

- Agreement: Make the changes captured below to the delay profile description in the specification

## G.2.1 Delay profiles The delay profiles are simplified from the TR 38.901 [16] TDL models. The simplification steps are shown below for information. These steps are only used when new delay profiles are created. Otherwise, the delay profiles specified in G.2.1.1 and G.2.1.2 can be used as such. Step 1: Use the original TDL model from TR 38.901 [16]. Step 2: Re-order the taps in ascending delays. Step 3: Perform delay scaling according to the procedure described in clause 7.7.3 in TR 38.901 [16]. Step 4: Apply the quantization to the delay resolution 5 ns or 2 ns, as described in Table G.2.1.2-1. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution. Step 5: If multiple Rayleigh taps are rounded to the same delay bin, merge them by calculating their linear power sum. Step 6: If there are more than 12 taps for 5ns resolution or 16 taps for 2ns resolution ns as described in Table G.2.1.2-1 in the quantized model, merge the Rayleigh taps as follows <Unchanged part is skipped> • Otherwise, the location of the merged tap is based on the average delay of the weakest tap and selected tap. If the average delay is on the sampling grid, the location of the merged tap is the average delay. Otherwise, the location of the merged tap is rounded towards the direction of the selected tap (e.g. For 5ns resolution, 10 ns & 20 ns 15 ns, 10 ns & 25 ns 20 ns, if 25 ns had higher or equal power; 15 ns, if 10 ns had higher power). The weakest tap and the selected tap are removed. - Repeat step 6 until the final number of taps is 12 or 16. Step 7: Round the amplitudes of taps to one decimal (e.g. -8.78 dB -8.8 dB) Step 8: If the delay spread has slightly changed due to the tap merge, adjust the final delay spread by increasing or decreasing the power of the last tap so that the delay spread is corrected. Step 9: Re-normalize the highest Rayleigh tap to 0 dB if there is no LOS path in the model.

AWGN power levels

- Agreement: Update AWGN power level considering the following cases

  - For 120 kHz SCS, and 400 MHz: EISREFSENS_50M + ΔFR2_REFSENS + 24 dBm / 380.16 MHz

  - For 480 kHz SCS, and 400 MHz: EISREFSENS_50M + ΔFR2_REFSENS + 24 dBm / 380.16 MHz

Phase noise consideration

- Agreement: For PUSCH demodulation requirements, the performance loss due to phase noise should be included in impairment results and the ideal simulation alignment should be based on results without phase noise modelled

PUSCH demodulation requirements

Channel model for 16 QAM for PUSCH requirements

- Agreement: Update channel model for MCS 16 with 120 kHz SCS and 480kHz SCS

  - For 1x2 Low, use TDLA30-650 for 100MHz CBW, TDLA10-650 for 400MHz CBW

  - For 2x2 Low, use TDLD30-200 for 100MHz CBW, and TDLD10-200 for 400MHz CBW

MCS and number Tx/Rx branches for PUSCH requirements

- Agreement: Update AWGN power level considering the following cases

  - MCS 20 for 1T2R Low

  - Whether to introduce MCS 18 with 2T2R Low is FFS

PTRS configuration for PUSCH requirements

- Agreement: Adopt 1 PTRS port for PUSCH demodulation requirements

Applicability rules for PUSCH

- Agreement: Reuse current applicability rule for different channel bandwidths for FR2-2 120kHz SCS PUSCH requirements

PRACH demodulation requirements

Applicability rules for PRACH

- Agreement: Update D.103 with the following

| D.103 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [20], i.e., format: 0, A1, A2, A3, B4, C0, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [20], i.e.: - For BS type 1-O: 15 kHz, 30 kHz or both. - For BS type 2-O in FR2-1: 60 kHz, 120 kHz or both. - For BS type 2O in FR2-2: 120 kHz, 480 kHz or both. |
| --- | --- | --- |

- Agreement: Update D.113 as captured below

| D.113 | PRACH format with LRA = 1151 for 15 kHz SCS, LRA = 571 for 30 kHz SCS, LRA = 1151 and LRA = 571 for 120 kHz SCS and LRA = 571 for 480 kHz SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [17], i.e., format: A2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format as specified in TS 38.211 [17], i.e.: -For BS type 1-O: 15 kHz, 30 kHz or both. -For BS type 2-O in FR2-2: 120 kHz, 480 kHz or both. |
| --- | --- | --- |

Simulation alignment

- PUSCH and PUCCH: requirements are captured as TDB and will be revised based on updated results during RAN4#106

- PUCCH: requirements are captured as TDB and will be revised based on updated results during RAN4#106

- PRACH: draft CRs include requirements without square backets

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2220146 | Summary for [105][320] NR_exto71GHz_Demod _Part1 | Moderator (Nokia) | Noted |
| R4-2220147 | Summary for [105][321] NR_exto71GHz_Demod _Part2 | Moderator (Qualcomm) | Noted |
| R4-2220182 | WF for FR2-2 BS demodulation requirements | Nokia | Approved |
| R4-2220187 | WF for FR2-2 UE demodulation part | Qualcomm Inc. | Approved |
| R4-2219514 | Big CR on FR2-2 UE demodulation performance requirements in TS 38.101-4 | Huawei, HiSilicon | Reserved |
| R4-2219747 | Big CR to 38.104: demodulation requirements introduction for FR2-2 | Nokia, Nokia Shanghai Bell, Intel | Reserved |
| R4-2220174 | draft CR for TS38.104 FRC table for FR2-2 PUSCH demodulation | Ericsson | Endorsed |
| R4-2220175 | draft CR for TS38.141-2 FRC table for FR2-2 PUSCH demodulation | Ericsson | Endorsed |
| R4-2220177 | Draft CR: Introduction on FR2-2 PUSCH requirements in TS 38.141-2 | Huawei, HiSilicon | Endorsed |
| R4-2220179 | Draft CR: Introduction on FR2-2 PUCCH requirements in TS 38.104 | Huawei, HiSilicon | Endorsed |
| R4-2220180 | Draft CR 38.104: PRACH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2220181 | Draft CR 38.141-2: PRACH requirements for FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2220183 | Draft CR for PDSCH FRCs for requirements in FR2-2 | Apple Inc. | Endorsed |
| R4-2220184 | draft CR on PDSCH requirements for 52.6 - 71 GHz band | Ericsson | Endorsed |
| R4-2220185 | DraftCR on PDCCH requirements for ext71GHz | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2220186 | Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz | Apple Inc. | Endorsed |
| R4-2220283 | Draft CR 38.104: PUSCH requirements for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Endorsed |
| R4-2220284 | Draft CR 38.141-2: PUCCH requirements for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Endorsed |
| R4-2220286 | Draft CR: General test applicability of FR2-2 UE demodulation requirements introduction | Qualcomm Inc. | Endorsed |
| R4-2220288 | Draft CR: Introduction on FR1+FR2-2 CA PDSCH requirements | Huawei, HiSilicon | Endorsed |

##### 2.4.1.3	RRM performance

RAN4 #104Bis-e

Test configurations

- Agreement: For UL transmit timing and timing advance accuracy test cases

  - UE is required to be tested with the largest supported SCS

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2217136 | Email discussion summary for [104-bis-e][203] NR_ext_to_71GHz_RRM_1 | Moderator (Huawei) | Noted |
| R4-2217137 | Email discussion summary for [104-bis-e][204] NR_ext_to_71GHz_RRM_2 | Moderator (Qualcomm) | Noted |
| R4-2217194 | WF on NR extension to 71 GHz – RRM - 1 | Qualcomm Inc. | Approved |
| R4-2217196 | Draft CR on test cases for SA RRC Re-establishment for extending NR operation to 71GHz | CATT | Endorsed |
| R4-2217197 | Draft CR on test cases for Beam failure detection and link recovery for extending NR operation to 71GHz | CATT | Endorsed |
| R4-2217198 | Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2 | vivo | Endorsed |
| R4-2217199 | Draft CR random access test cases in FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2217200 | Draft CR introducing BFD and TCI state switch test cases in FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2217201 | CR on test cases for HO for FR2-2 | Huawei, HiSilicon | Endorsed |
| R4-2217202 | draft CR on Test Cases on RLM for SCell activation to 71GHz | Ericsson | Endorsed |

RAN4 #105

CCA modelling in test cases

- Agreement: For DL CCA model in FR2-2 test cases:

  - TE generates a uniform random variable p from the range [0,1]

  - If p > PCCA_DL,

    - TE picks one SSB/SMTC occasion out of a group of each 12 consecutive SSB/SMTC occasions based on a fixed pattern, where one SSB/SMTC occasion is equivalent to one SSB burst Set.

    - TE models CCA failure in this SSB/SMTC occasion. Note that other 11 SSB/SMTC occasions shall be transmitted by the TE

    - Whole SSB occasion group is considered as unavailable to the UE

  - If p < PCCA_DL,

    - TE transmit 12 consecutive SSB/SMTC occasions

    - Whole SSB occasion group is considered as available to the UE

PCCA_DL

- Agreement:

  - PCCA_DL = [93] %

Rate of correct tests

- Agreement:

  - The rate of correct tests observed during repeated tests shall be at least 90%

Test cases for RRM performance requirements without CCA

| Group of requirements | Test cases | Sub-test | 120 kHz SCS | 480/960 kHz SCS | Company | Status |
| --- | --- | --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 | intra-frequency | Yes | Yes | Ericsson | Submit to A.7 |
|  |  | inter-frequency | Yes | yes | Ericsson | Submit to A.7 |
| HO (delay and interruptions) | FR2-2->FR2-2 | intra-frequency, unknown target cell | Yes | Yes | Huawei | Available |
|  |  | inter-frequency, unknown target cell | Yes | Low priority | Huawei | Available |
|  | FR1->FR2-2 | unknown target cell | Yes | Yes | Huawei | Available |
| RRC Re-establishment | FR2-2->FR2-2 | intra frequency | Yes | Low priority | CATT | Endorsed |
|  |  | inter frequency | Yes | Yes | CATT | Endorsed |
|  |  | intra frequency without serving cell timing | Yes | Yes | CATT | Endorsed |
| Random access | 4-step RACH | Contention based | Reuse FR2-1 | Yes | Nokia | Endorsed |
|  |  | Non-contention based | Reuse FR2-1 | Yes | Nokia | Endorsed |
| Timing | UE Transmit Timing |  | Yes FR2 configuration uses 240kHz SSB, so new test is needed for 120kHz SCS | Yes | Huawei | Available |
|  | Timing advance adjustment accuracy |  | Yes FR2 configuration uses 240kHz SSB, so new test is needed for 120kHz SCS | Yes | Huawei | Available |
| Active BWP switch | DCI-based and timer-based active BWP switch | FR2-2->FR2-2 | Reuse FR2-1 | Yes | Huawei | Available |
|  | RRC-based active BWP switch |  | Reuse FR2-1 | Yes | Huawei | Available |
|  | RRC-based Active BWP Switch on multiple CCs |  | Reuse FR2-1 | Yes | Huawei | Available |
| Radio link monitoring | FR2-2 PCell, SSB-based | Out-of-sync, in non-DRX | Yes | Yes | Ericsson | Endorsed |
|  |  | In-sync, in non-DRX mode | Yes | Low priority | Ericsson | Endorsed |
|  |  | Out-of-sync, in DRX mode | Yes | Low priority | Ericsson | Endorsed |
|  |  | In-sync, in DRX mode | Yes | Yes | Ericsson | Endorsed |
|  | FR2-2 PCell, CSI-RS-based | Out-of-sync, in non-DRX | Yes | Low priority | Ericsson | Endorsed |
|  |  | In-sync, in non-DRX mode | Yes | Yes | Ericsson | Endorsed |
|  |  | Out-of-sync, in DRX mode | Yes | Yes | Ericsson | Endorsed |
|  |  | In-sync, in DRX mode | Yes | Low priority | Ericsson | Endorsed |
|  | Scheduling restrictions |  | Yes | Yes | Ericsson | Endorsed |
| Beam failure detection and link recovery | FR2-2 PCell | SSB-based in non-DRX mode | Yes | Yes | Nokia | Endorsed |
|  |  | SSB-based in DRX mode | Yes | Low priority | Nokia | Endorsed |
|  |  | CSI-RS-based in non-DRX mode | Yes | Low priority | CATT | Endorsed |
|  |  | CSI-RS -based in non-DRX mode | Yes | Yes | CATT | Endorsed |
|  | Scheduling restrictions | SSB-based in non-DRX mode | Yes | Yes | CATT | Endorsed |
| SCell activation/deactivation delay | SCell in FR2 | intra-band in non-DRX | Yes | Yes | Ericsson | Submit to A.7 |
|  | FR1+FR2 inter-band with target SCell in FR2 |  | Yes | Yes | Ericsson | Submit to A.7 |
| PSCell addition and release delay | Known PSCell |  | Yes FR2 configuration uses 240kHz SSB, so new test is needed for 120kHz SCS | No | Qualcomm | Submitted |
|  | Unknown PSCell |  | Yes FR2 configuration uses 240kHz SSB, so new test is needed for 120kHz SCS | Yes | Qualcomm | Submitted |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 | Reuse FR2-1 | Yes | Nokia | Endorsed |
|  | RRC based | PCell in FR2-2 | Reuse FR2-1 | Low priority | Nokia | Endorsed |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes | Yes | vivo | Endorsed |
|  |  | under DRX | Yes | Low priority | vivo | Endorsed |
|  | SA event triggered reporting test with per-UE gap | under non-DRX | Yes | Low priority | vivo | Endorsed |
|  |  | under DRX | Yes | Yes | vivo | Endorsed |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes | Yes | vivo | Available |
|  |  | PCell in FR2-2, DRX is used | Yes | Low Priority | vivo | Available |
|  |  | PCell in FR1, DRX is not used | Yes | Low Priority | vivo | Endorsed |
|  |  | PCell in FR1, DRX is used | Yes | Yes | vivo | Endorsed |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is not used | Yes | Low Priority | vivo | Available |
|  |  | PCell in FR2-2, DRX is used | Yes | Yes | vivo | Available |
|  |  | PCell in FR1, DRX is not used | Yes | Yes | vivo | Endorsed |
|  |  | PCell in FR1, DRX is used | Yes | Low Priority | vivo | Endorsed |
| L1-RSRP measurement for beam reporting | SSB based | DRX is used | Yes | Yes | Qualcomm |  |
|  | CSI-RS based | DRX not used | Yes | Yes | Qualcomm |  |

Test cases for RRM performance requirements with CCA

| Group of requirements | Test cases | Sub-test | Needed / Not needed | Company | Status |
| --- | --- | --- | --- | --- | --- |
| RRC_IDLE, cell re-selection | FR2-2 -> FR2-2 (CCA) | intra-frequency | Yes, at least with 120kHz and 480kHz | Ericsson |  |
|  |  | inter-frequency | Yes, at least with 120kHz and 480kHz | Ericsson |  |
| HO (delay and interruptions) | FR2-2->FR2-2 (CCA) | intra-frequency, unknown target cell | Yes | Huawei | Submitted |
|  | FR1->FR2-2 (CCA) | unknown target cell | Yes | Huawei | Submitted |
| RRC Re-establishment | FR2-2->FR2-2 (CCA) | inter frequency | Yes | Nokia | Submitted |
|  |  | intra frequency without serving cell timing | Yes | Nokia | Submitted |
| Random access | 4-step RACH | Contention based | Yes | Nokia | Submitted |
|  |  | Non-contention based | Yes | Nokia | Submitted |
| Radio link monitoring | FR2-2 Pcell (CCA), SSB-based | Out-of-sync, in non-DRX | Yes | Ericsson | Submitted |
|  |  | In-sync, in DRX mode | Yes | Ericsson | Submitted |
| Beam failure detection and link recovery | FR2-2 PCell (CCA) | SSB-based in non-DRX mode | Yes | CATT | Submitted |
|  |  | CSI-RS -based in non-DRX mode | Yes | CATT | Submitted |
| SCell activation/deactivation delay | SCell in FR2 (CCA) | intra-band in non-DRX | Yes | CATT | Submitted |
|  |  | inter-band in non-DRX | Low priority | CATT | Submitted |
|  | FR1+FR2 (CCA)inter-band with target SCell in FR2 |  | Yes | CATT | Submitted |
| PSCell addition and release delay | Unknown PSCell |  | Yes | Huawei | Submitted |
| Active TCI state switching delay | MAC-CE based | PCell in FR2-2 (CCA) | Yes | Huawei | Submitted |
| Intra-frequency measurement procedure | SA event triggered reporting test without gap | under non-DRX | Yes | vivo | Submitted |
|  |  | under DRX | Yes | vivo | Submitted |
| Inter-frequency measurement procedure | SA event triggered reporting tests For FR2 without SSB time index detection | PCell in FR2-2, DRX is not used | Yes | vivo | Submitted |
|  |  | PCell in FR1, DRX is used | Yes | vivo | Submitted |
|  | SA event triggered reporting tests For FR2 with SSB time index detection | PCell in FR2-2, DRX is used | Yes | vivo | Submitted |
|  |  | PCell in FR1, DRX is not used | Yes | vivo | Submitted |

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2220052 | Topic summary for [105][206] NR_ext_to_71GHz_ RRM_1 | Moderator (Huawei) | Noted |
| R4-2220053 | Topic summary for [105][207] NR_ext_to_71GHz_ RRM_2 | Moderator (Qualcomm) | Noted |
| R4-2220318 | WF on Rel-17 NR extension to 71GHz RRM Core requirements maintenance | Huawei | Approved |
| R4-2220321 | WF on Rel-17 NR extension to 71GHz RRM Performance requirements | Huawei | Approved |
| R4-2218774 | Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2 | vivo | Endorsed |
| R4-2218933 | Draft CR on RRC connected mobility test cases in FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2219226 | DraftCR on HO test cases for FR2-2 with CCA | Huawei, HiSilicon | Endorsed |
| R4-2219228 | DraftCR on timing test cases for FR2-2 | Huawei, HiSilicon | Merged |
| R4-2220043 | Draft CR on TC - PSCell addition and release delay | Qualcomm Inc. | Endorsed |
| R4-2220045 | Big CR for NR operation to 71GHz RRM performance requirements | Qualcomm Inc. | Reserved |
| R4-2220445 | Draft CR on test cases for beam failure detection and link recovery with CCA for FR2-2 | CATT | Endorsed |
| R4-2220447 | Draft CR on test cases for SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 with CCA for FR2-2 | CATT | Endorsed |
| R4-2220448 | Draft CR on introduction of intra-frequency and inter-frequency measurement test cases with CCA for FR2-2 | vivo | Endorsed |
| R4-2220449 | CR on corrections to timing test cases in FR2-2 | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2220450 | RLM test cases for 71GHz with CCA | Ericsson | Endorsed |
| R4-2220451 | DraftCR on signaling characteristics test cases for FR2-2 | Huawei, HiSilicon | Endorsed |

#### 2.4.2	Remaining Open issues

Remaining issues for Performance part are captured below:

- BS RF conformance testing

  - Finalize MU evaluation

- Demodulation performance

  - BS demodulation performance requirements

  - UE demodulation performance requirements

### 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

### 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g., a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #104Bis-e

- R4-2215416, “Discussion of remaining issues on RRM core requirements for extension to 71GHz,” CATT

- R4-2215417, “Further discussion on general RRM performance requirements for NR extension to 71 GHz,” CATT

- R4-2215418, “Draft CR on test cases for SA RRC Re-establishment for extending NR operation to 71GHz,” CATT

- R4-2215419, “Draft CR on test cases for Beam failure detection and link recovery for extending NR operation to 71GHz,” CATT

- R4-2215532, “On PDSCH Requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2215533, “PDSCH simulation results for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2215534, “On PDCCH and PBCH Requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2215535, “PDCCH and PBCH simulation results for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2215536, “Nokia_DraftCR_38101-4_PDCCH,” Nokia, Nokia Shanghai Bell

- R4-2215537, “On CSI Reporting Requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2215571, “CR to TS 38.104 on reference to FRCs,” Nokia, Nokia Shanghai Bell

- R4-2215572, “Proposal on suitability of OTA measurement systems on BS conformance testing for extending current NR operation to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2215573, “Proposal on measurement uncertainty of BS OTA transmitter requirements for extending current NR operation to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2215574, “Proposal on measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2215585, “Draft CR for Introducing Propagation channel models for requirements in FR2-2,” Apple Inc.

- R4-2215586, “Draft CR for Introducing FRCs for requirements in FR2-2,” Apple Inc.

- R4-2215587, “On PDSCH demod requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2215588, “On PDCCH and PBCH demod requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2215589, “On CQI reporting requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2215590, “Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2215617, “Remaining general aspects for NR operation in 52.6GHz - 71GHz,” Apple Inc.

- R4-2215618, “LBT impacts on RRM requirements for NR operation in 52.6GHz - 71GHz,” Apple Inc.

- R4-2215659, “Beam Correspondence for band n263,” Apple Inc.

- R4-2215690, “Discussion on general and PUSCH issue for FR2-2 BS demodulation,” Ericsson

- R4-2215691, “Simulation results for FR2-2 PUSCH,” Ericsson

- R4-2215692, “Simulation results for FR2-2 PUCCH,” Ericsson

- R4-2215693, “Simulation results for FR2-2 PRACH,” Ericsson

- R4-2215694, “draftCR for TS38.104 introduce FRC tables for FR2-2 PUSCH requirements,” Ericsson

- R4-2215695, “draftCR for TS38.141-2 introduce FRC tables for FR2-2 PUSCH requirements,” Ericsson

- R4-2215799, “Discussion on TCI assumption for RSSI measurement for FR2-2,” LG Electronics Inc.

- R4-2215800, “CR on QCL-ed assumption for inter-frequency RSSI measurement in FR2-2,” LG Electronics Inc.

- R4-2215828, “Draft CR to TS 38.141-2: Addition of FR2-2 transmitter support in clause 6,” Ericsson

- R4-2215829, “Draft CR to TS 38.141-2: Addition of FR2-2 receiver support in clause 7,” Ericsson

- R4-2215830, “Draft CR to TR 37.941: Addition of aspects related to EIRP measurement in CATR relevant for FR2-2 in sub-clause 7.3, 8.3, 9.2.3 and 9.2.7,” Ericsson

- R4-2215831, “On further general aspects relevant for FR2-2 conformance testing,” Ericsson

- R4-2215832, “CR to TS 38.104: Correction of guardband for FR2-2 in sub-clause 5.3.3,” Ericsson

- R4-2215833, “On further aspects related to FR2-2 transmitter conformance testing,” Ericsson

- R4-2215834, “On further aspects related to FR2-2 receiver conformance testing,” Ericsson

- R4-2215836, “Draft CR to TS 38.141-2: Addition of FR2-2 aspects in clause 4,” Ericsson

- R4-2215863, “Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2,” vivo

- R4-2215910, “The remaining issues of the PDSCH requirements in 52.6 – 71 GHz band,” Ericsson

- R4-2215911, “Simulation results for PDSCH demodulation in 52.6 – 71 GHz band,” Ericsson

- R4-2215912, “The remaining issues for PDCCH and PBCH requirements in FR2-2,” Ericsson

- R4-2215913, “Simulation results for PDCCH and PBCH demodulation in FR2-2,” Ericsson

- R4-2215914, “SDR requirements in 52.6 – 71 GHz band,” Ericsson

- R4-2215915, “Simulation results for SDR requirements in 52.6 – 71 GHz band,” Ericsson

- R4-2215916, “CSI reporting requirements in 52.6 – 71 GHz band,” Ericsson

- R4-2215917, “Simulation results for CSI reporting requirements in FR2-2,” Ericsson

- R4-2215918, “draft CR on PDSCH requirements for 52.6 - 71 GHz band,” Ericsson

- R4-2215919, “SDR requirements for 52.6 GHz – 71 GHz band,” Ericsson

- R4-2216010, “Discussion on general issues for FR2-2 demodulation requirements,” Huawei, HiSilicon

- R4-2216011, “BigCR: Introduction of FR2-2 UE demodulation and CSI requirements in 38.101-4,” Huawei, HiSilicon

- R4-2216012, “Discussions on FR2-2 PDSCH demodulation requirements,” Huawei, HiSilicon

- R4-2216013, “Simulation results on FR2-2 PDSCH demodulation requirements,” Huawei, HiSilicon

- R4-2216014, “Draft CR: Introduction of FR1+FR2-2 CA PDSCH performance requirements in TS 38.101-4,” Huawei, HiSilicon

- R4-2216015, “Simulation results on FR2-2 PDCCHPBCH requirements,” Huawei, HiSilicon

- R4-2216016, “Discussions on FR2-2 SDR requirements,” Huawei, HiSilicon

- R4-2216017, “Simulation results FR2-2 SDR requirements,” Huawei, HiSilicon

- R4-2216018, “Discussion on remaining issues on FR2-2 CQI requirements,” Huawei, HiSilicon

- R4-2216019, “Simulation results on FR2-2 CQI requirements,” Huawei, HiSilicon

- R4-2216020, “Draft CR: Introduction of FR2-2 PUSCH radiated conformance testing requirements in TS 38.141-2,” Huawei, HiSilicon

- R4-2216021, “Discussions on FR2-2 PUSCH demodulation requirements,” Huawei, HiSilicon

- R4-2216022, “Simulation results on FR2-2 PUSCH demodulation requirements,” Huawei, HiSilicon

- R4-2216023, “Discussions and simulation results on FR2-2 PUCCH demodulation requirements,” Huawei, HiSilicon

- R4-2216024, “Draft CR Introduction of FR2-2 PUCCH performance requirements in TS 38.104,” Huawei, HiSilicon

- R4-2216025, “Discussions on FR2-2 PRACH demodulation requirements,” Huawei, HiSilicon

- R4-2216026, “Simulation results on FR2-2 PRACH demodulation requirements,” Huawei, HiSilicon

- R4-2216178, “On Demod Requirements for FR2-2 - General,” Qualcomm Inc.

- R4-2216179, “Draft CR to 38.101-4 for FR2-2 Demod - General section,” Qualcomm Inc.

- R4-2216180, “Simulation Results for FR2-2 UE Demodulation PDSCH,” Qualcomm Inc.

- R4-2216181, “Simulation Results for FR2-2 UE Demodulation PDCCH/PBCH,” Qualcomm Inc.

- R4-2216182, “Draft CR to 38.101-4 for FR2-2 Demod - PBCH Requirements,” Qualcomm Inc.

- R4-2216183, “Discussion on FR2-2 CQI Requirements,” Qualcomm Inc.

- R4-2216256, “CR on applicability of RRM requirements with CCA in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2216257, “Discussion on RRM requirements with CCA in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2216258, “Draft CR random access test cases in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2216259, “Discussion on RRM performance timing requirements in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2216260, “Draft CR introducing BFD and TCI state switch test cases in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2216261, “Discussion on general requirements on FR2-2,” Huawei, HiSilicon

- R4-2216262, “CR on RLM requirements for FR2-2,” Huawei, HiSilicon

- R4-2216263, “CR on SCell activation requirements of FR2-2,” Huawei, HiSilicon

- R4-2216264, “Discussion on LBT impact on requirements for FR2-2,” Huawei, HiSilicon

- R4-2216265, “CR on LBT assumption for FR2-2,” Huawei, HiSilicon

- R4-2216266, “CR on RSSI measurement for FR2-2,” Huawei, HiSilicon

- R4-2216267, “Discussion on performance requirements for FR2-2,” Huawei, HiSilicon

- R4-2216268, “CR on test cases for HO for FR2-2,” Huawei, HiSilicon

- R4-2216496, “FR2-2 BS conformance test consideration for Tx testing,” Keysight Technologies UK Ltd

- R4-2216497, “FR2-2 BS conformance test consideration for Rx testing,” Keysight Technologies UK Ltd

- R4-2216498, “FR2-2 Test Model details and TP,” Keysight Technologies UK Ltd

- R4-2216499, “FR2-2 EVM measurement detail and TP,” Keysight Technologies UK Ltd

- R4-2216501, “draft CR on Test Cases on RLM for SCell activation to 71GHz,” Ericsson

- R4-2216560, “Further discussion on BS conformance testing for 52.6-71GHz,” ZTE Corporation

- R4-2216570, “Discussion on PUSCH demodulation requirements for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2216571, “PUSCH simulation results for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2216572, “Discussion on PUCCH demodulation requirements for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2216573, “PUCCH simulation results for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2216574, “Discussion on PRACH demodulation requirements for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2216575, “PRACH simulation results for demodulation requirements for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2216576, “Draft CR 38.104: PRACH requirements for FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2216577, “Draft CR 38.141-2: PRACH requirements for FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2216606, “Reply LS on signalling of CCA configurations of neighbour cells in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2216683, “FR2-2 maintenance aspects for UE Tx,” Intel Corporation

- R4-2216684, “System parameters maintenance for NR ext. to 71GHz,” Intel Corporation

- R4-2216691, “View on BS demodulation requirement for NR extended to 71GHz,” Samsung

- R4-2216692, “Draft CR on annex for PRACH requirement for TS 38.104,” Samsung

- R4-2216693, “Draft CR on annex for PRACH requirement for TS 38.141-2,” Samsung

- R4-2216694, “Initial simulation results on PUSCH demodulation requirement for Rel-17 71GHz,” Samsung

- R4-2216695, “Initial simulation results on PUCCH demodulation requirement for Rel-17 71GHz,” Samsung

- R4-2216696, “Initial simulation results on PRACH demodulation requirement for Rel-17 71GHz,” Samsung

- R4-2216707, “CSI Simulation Results for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2216795, “60 GHz UE PRACH and PTRS capabilities,” Qualcomm France

- R4-2216796, “CR to 38.101-2 on band n263 UE Rx aspects,” Qualcomm France

- R4-2216797, “CR to 38.101-2 on band n263 UE Tx aspects,” Qualcomm France

- R4-2216882, “Draft CR on Measurement Procedures,” Qualcomm Inc.

- R4-2216890, “Email discussion summary for [104-bis-e][306] NR_exto71GHz_BSRF,” Moderator (Huawei)

- R4-2216903, “Email discussion summary for [104-bis-e][319] NR_exto71GHz_Demod_Part1,” Moderator (Nokia)

- R4-2216904, “Email discussion summary for 1[04-bis-e][320] NR_exto71GHz_Demod_Part2,” Moderator (Qualcomm)

- R4-2216914, “Email discussion summary for [104-bis-e][203] NR_ext_to_71GHz_RRM_1,” Moderator (Huawei)

- R4-2216915, “Email discussion summary for [104-bis-e][204] NR_ext_to_71GHz_RRM_2,” Moderator (Qualcomm)

- R4-2216940, “Email discussion summary for [104-bis-e][102] NR_ext_to_71GHz,” Moderator (Qualcomm)

- R4-2216983, “Email discussion summary for [104-bis-e][102] NR_ext_to_71GHz,” Moderator (Qualcomm)

- R4-2217056, “CR to 38.101-2 on band n263 UE Tx aspects,” Qualcomm France

- R4-2217090, “WF on 60 GHz Operation bands and system parameter and UE RF requirement maintenance,” Qualcomm Inc.

- R4-2217133, “Email discussion summary for [104-bis-e][204] NR_ext_to_71GHz_RRM_2,” Moderator (Qualcomm)

- R4-2217136, “Email discussion summary for [104-bis-e][203] NR_ext_to_71GHz_RRM_1,” Moderator (Huawei)

- R4-2217137, “Email discussion summary for [104-bis-e][204] NR_ext_to_71GHz_RRM_2,” Moderator (Qualcomm)

- R4-2217186, “WF on core requirements maintenance for FR2-2,” Huawei, HiSilicon

- R4-2217187, “LS on capability of relaxed cell detection requirements for FR2-2,” Huawei, HiSilicon

- R4-2217188, “CR on QCL-ed assumption for inter-frequency RSSI measurement in FR2-2	LG Electronics Inc.

- R4-2217189, “CR on SCell activation requirements of FR2-2,” Huawei, HiSilicon

- R4-2217190, “Draft CR on Measurement Procedures,” Qualcomm Inc.

- R4-2217191, “CR on LBT assumption for FR2-2,” Huawei, HiSilicon

- R4-2217192, “CR on RSSI measurement for FR2-2,” Huawei, HiSilicon

- R4-2217193, “Reply LS on signalling of CCA configurations of neighbour cells in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2217194, “WF on NR extension to 71 GHz – RRM - 1,” Qualcomm Inc.

- R4-2217196, “Draft CR on test cases for SA RRC Re-establishment for extending NR operation to 71GHz,” CATT

- R4-2217197, “Draft CR on test cases for Beam failure detection and link recovery for extending NR operation to 71GHz,” CATT

- R4-2217198, “Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2,” vivo

- R4-2217199, “Draft CR random access test cases in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2217200, “Draft CR introducing BFD and TCI state switch test cases in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2217201, “CR on test cases for HO for FR2-2,” Huawei, HiSilicon

- R4-2217202, “draft CR on Test Cases on RLM for SCell activation to 71GHz,” Ericsson

- R4-2217362, “WF on the list of MU contributors for FR2-2,” Nokia

- R4-2217363, “WF on the test equipment MU,” Keysight

- R4-2217364, “WF on the OTA chambers applicability for FR2-2,” Ericsson

- R4-2217365, “WF for work-split,” Huawei

- R4-2217366, “Draft CR to TR 37.941: Addition of aspects related to EIRP measurement in CATR relevant for FR2-2 in sub-clause 7.3, 8.3, 9.2.3 and 9.2.7,” Ericsson

- R4-2217367, “Draft CR to TS 38.141-2: Addition of FR2-2 aspects in clause 4,” Ericsson

- R4-2217368, “Draft CR to TS 38.141-2: Addition of FR2-2 transmitter support in clause 6,” Ericsson

- R4-2217369, “FR2-2 EVM measurement detail and TP,” Keysight Technologies UK Ltd

- R4-2217370, “Draft CR to TS 38.141-2: Addition of FR2-2 receiver support in clause 7,” Ericsson

- R4-2217371, “WF on general aspects for FR2-2 demodulation requirements,” Huawei

- R4-2217372, “WF on PUSCH demodulation requirements for FR2-2,” Nokia

- R4-2217373, “WF on PUCCH demodulation requirements for FR2-2,” Ericsson

- R4-2217374, “WF on PRACH demodulation requirements for FR2-2,” Samsung

- R4-2217375, “Simulation results collection for Fr2-2 BS demod,” Huawei

- R4-2217376, “Draft CR 38.104: PUSCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel

- R4-2217377, “Draft CR 38.141-2: PUCCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2217378, “draftCR for TS38.104 introduce FRC tables for FR2-2 PUSCH requirements,” Ericsson

- R4-2217379, “draftCR for TS38.141-2 introduce FRC tables for FR2-2 PUSCH requirements,” Ericsson

- R4-2217380, “Draft CR: Introduction of FR2-2 PUSCH radiated conformance testing requirements in TS 38.141-2,” Huawei, HiSilicon

- R4-2217381, “Discussions on FR2-2 PUSCH demodulation requirements,” Huawei, HiSilicon

- R4-2217382, “Simulation results on FR2-2 PUSCH demodulation requirements,” Huawei, HiSilicon

- R4-2217383, “Discussions and simulation results on FR2-2 PUCCH demodulation requirements,” Huawei, HiSilicon

- R4-2217384, “Draft CR Introduction of FR2-2 PUCCH performance requirements in TS 38.104,” Huawei, HiSilicon

- R4-2217385, “Discussions on FR2-2 PRACH demodulation requirements,” Huawei, HiSilicon

- R4-2217386, “Simulation results on FR2-2 PRACH demodulation requirements,” Huawei, HiSilicon

- R4-2217387, “Draft CR 38.104: PRACH requirements for FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2217388, “Draft CR 38.141-2: PRACH requirements for FR2-2	Nokia, Nokia Shanghai Bell

- R4-2217389, “Draft CR on annex for PRACH requirement for TS 38.104,” Samsung

- R4-2217390, “Draft CR on annex for PRACH requirement for TS 38.141-2,” Samsung

- R4-2217391, “WF for FR2-2 UE demodulation and CSI requirements,” Qualcomm Inc.

- R4-2217392, “Simulation results summary for UE demod in FR2-2,” Ericsson

- R4-2217393, “Discussion on general issues for FR2-2 demodulation requirements,” Huawei, HiSilicon

- R4-2217394, “Draft CR to 38.101-4 for FR2-2 Demod - General section,” Qualcomm Inc.

- R4-2217395, “draft CR on PDSCH requirements for 52.6 - 71 GHz band,” Ericsson

- R4-2217396, “Discussions on FR2-2 PDSCH demodulation requirements,” Huawei, HiSilicon

- R4-2217397, “Simulation results on FR2-2 PDSCH demodulation requirements,” Huawei, HiSilicon

- R4-2217398, “Draft CR: Introduction of FR1+FR2-2 CA PDSCH performance requirements in TS 38.101-4,” Huawei, HiSilicon

- R4-2217399, “Nokia_DraftCR_38101-4_PDCCH,” Nokia, Nokia Shanghai Bell

- R4-2217400, “Simulation results on FR2-2 PDCCHPBCH requirements,” Huawei, HiSilicon

- R4-2217401, “Discussions on FR2-2 SDR requirements,” Huawei, HiSilicon

- R4-2217402, “Simulation results FR2-2 SDR requirements,” Huawei, HiSilicon

- R4-2217403, “Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2217404, “Discussion on remaining issues on FR2-2 CQI requirements,” Huawei, HiSilicon

- R4-2217405, “Simulation results on FR2-2 CQI requirements,” Huawei, HiSilicon

- R4-2217482, “Email discussion summary for [104-bis-e][306] NR_exto71GHz_BSRF,” Moderator (Huawei)

- R4-2217483, “Email discussion summary for [104-bis-e][319] NR_exto71GHz_Demod_Part1,” Moderator (Nokia)

- R4-2217484, “Email discussion summary for [104-bis-e][320] NR_exto71GHz_Demod_Part2,” Moderator (Qualcomm)

- R4-2217516, “Big draftCR to 38.104: demodulation requirements introduction for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2217517, “Big draftCR to 38.141-2: demodulation requirements introduction for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2217590, “Big CR for NR operation to 71GHz RRM performance requirements,” MCC, Qualcomm Inc.

- R4-2217762, “Email discussion summary for [104-bis-e][102] NR_ext_to_71GHz,” Moderator (Qualcomm)

RAN4 #105

- R4-2218162, “RRM core requirement maintenance for NR operation in 52.6GHz - 71GHz,” Apple Inc.

- R4-2218163, “CR on TCI assumption for RSSI measurement for FR2-2,” Apple Inc.

- R4-2218179, “On PDSCH demod requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2218180, “Draft CR for PDSCH FRCs for requirements in FR2-2,” Apple Inc.

- R4-2218181, “On PDCCH and PBCH demod requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2218182, “On CQI reporting requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2218183, “Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2218229, “Discussion on RRM requirements for FR2-2,” MediaTek inc.

- R4-2218230, “Discussion on test for RRM requirements in FR2-2 with CCA,” MediaTek inc.

- R4-2218306, “Collection of Simulation Results for UE demodulation in FR2-2,” Ericsson

- R4-2218307, “The remaining issues of the PDSCH requirements in 52.6 – 71 GHz band,” Ericsson

- R4-2218308, “Simulation results for PDSCH demodulation in 52.6 – 71 GHz band,” Ericsson

- R4-2218309, “The remaining issues for PDCCH and PBCH requirements in FR2-2,” Ericsson

- R4-2218310, “Simulation results for PDCCH and PBCH demodulation in FR2-2,” Ericsson

- R4-2218311, “CSI reporting requirements in 52.6 – 71 GHz band,” Ericsson

- R4-2218312, “Simulation results for CSI reporting requirements in FR2-2,” Ericsson

- R4-2218313, “draft CR on PDSCH requirements for 52.6 - 71 GHz band,” Ericsson

- R4-2218319, “CR to TS 38.104 on tables for BS channel bandwidths,” Nokia, Nokia Shanghai Bell, NTT Docomo

- R4-2218320, “Draft CR to TS 38.141-2 in clauses 6.3 to 6.5 for extending current NR operation to 71GHz,” Nokia, Nokia Shanghai Bell

- R4-2218321, “Proposals on measurement uncertainty of BS OTA transmitter requirements for extending current NR operation to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2218322, “Proposals on measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz	,” Nokia, Nokia Shanghai Bell

- R4-2218463, “Draft CR for 38.141-2, On FR2-2 BS RF conformance testings in clauses 7.1-7.5 and respective MU and TT,” CATT

- R4-2218464, “Draft CR for 37.941, On FR2-2 RX directional requirements in clauses 10.1-10.5 and respective MU and TT summary,” CATT

- R4-2218470, “Further discussion on general RRM performance requirements for FR2-2,” CATT

- R4-2218471, “Draft CR on test cases for beam failure detection and link recovery with CCA for FR2-2,” CATT

- R4-2218472, “Draft CR on test cases for SCell Activation and deactivation for SCell in FR2 in non-DRX with CCA for FR2-2,” CATT

- R4-2218473, “Draft CR on test cases for SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 with CCA for FR2-2,” CATT

- R4-2218591, “Discussion on QCL assumption for Inter-frequency RSSI measurement,” LG Electronics Inc.

- R4-2218592, “CR on QCL-ed assumption for inter-frequency RSSI measurement in FR2-2,” LG Electronics Inc.

- R4-2218701, “CR for TS38.104 editorial correction on CBW and SCS per operating band,” NTT DOCOMO, INC.

- R4-2218703, “Discussion on general and PUSCH demodulation for NR FR2-2,” Ericsson

- R4-2218704, “Simulation results for NR FR2-2 PUSCH demodulation results,” Ericsson

- R4-2218705, “draft CR for TS38.104 FRC table for FR2-2 PUSCH demodulation,” Ericsson

- R4-2218706, “draft CR for TS38.141-2 FRC table for FR2-2 PUSCH demodulation,” Ericsson

- R4-2218773, “Draft CR on introduction of intra-frequency and inter-frequency measurement test cases with CCA for FR2-2,” vivo

- R4-2218774, “Draft CR on introduction of intra-frequency and inter-frequency measurement test cases without CCA for FR2-2,” vivo

- R4-2218833, “Draft CR to TR 37.941: Addition of aspects related to EIRP measurement in CATR relevant for FR2-2 in sub-clause 7.3, 8.3, 9.2.3 and 9.2.7,” Ericsson

- R4-2218834, “On general aspects relevant for FR2-2 conformance testing,” Ericsson

- R4-2218835, “On aspects related to FR2-2 transmitter conformance testing,” Ericsson

- R4-2218836, “On aspects related to FR2-2 receiver conformance testing,” Ericsson

- R4-2218837, “Draft CR to TS 38.141-2: Addition of FR2-2 aspects in clause 4,” Ericsson

- R4-2218838, “Draft CR to TR 37.941: Addition of aspects related to EIS measurement in CATR relevant for FR2-2 in sub-clause 10.2.3 and sub-clause 10.2.7,” Ericsson

- R4-2218908, “Discussion on OBW measurements for FR2-2,” NEC

- R4-2218909, “CR to 38.141-2: OTA OBW requirements for FR2-2,” NEC

- R4-2218932, “Discussion on RRM performance requirements with CCA in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2218933, “Draft CR on RRC connected mobility test cases in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2218934, “CR on corrections to timing test cases in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2219014, “Discussion and simulation results on PUSCH demodulation requirement for Rel-17 71GHz,” Samsung

- R4-2219015, “Simulation results on PUCCH demodulation requirement for Rel-17 71GHz,” Samsung

- R4-2219016, “Simulation results on PRACH demodulation requirement for Rel-17 71GHz,” Samsung

- R4-2219056, “On PDSCH Requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2219057, “PDSCH simulation results for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2219058, “On PDCCH and PBCH requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2219059, “PDCCH and PBCH simulation results for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2219060, “DraftCR on PDCCH requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2219061, “On CSI reporting requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2219062, “CSI Simulation Results for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2219063, “RLM test cases for 71GHz with CCA,” Ericsson

- R4-2219064, “LBT impacts on RRM requirements for extending NR operation to 71GHz,” Ericsson

- R4-2219222, “CR on cell reselection requirements for FR2-2,” Huawei, HiSilicon

- R4-2219223, “CR on QCL assumptions of inter-frequency measurement for FR2-2,” Huawei, HiSilicon

- R4-2219224, “Discussion on general requirements on FR2-2,” Huawei, HiSilicon

- R4-2219225, “Discussion on performance requirements for FR2-2,” Huawei, HiSilicon

- R4-2219226, “DraftCR on HO test cases for FR2-2 with CCA,” Huawei, HiSilicon

- R4-2219227, “DraftCR on timing test cases for FR2-2 with CCA,” Huawei, HiSilicon

- R4-2219228, “DraftCR on timing test cases for FR2-2,” Huawei, HiSilicon

- R4-2219229, “DraftCR on signaling characteristics test cases for FR2-2,” Huawei, HiSilicon

- R4-2219345, “draft CR to TS38.141-2 Clause 7.7, 7.8,” ZTE Corporation

- R4-2219346, “draft CR to TS38.141-2 Clause 7.9,” ZTE Corporation

- R4-2219449, “FR2-2 Power measurement equipment MU with Mixer for BS conformance testing,” Keysight Technologies UK Ltd

- R4-2219450, “FR2-2 Test equipment MU for BS conformance testing,” Keysight Technologies UK Ltd

- R4-2219451, “DraftCR to 37.941: 71 GHz Extension TE MU update,” Keysight Technologies UK Ltd

- R4-2219452, “draftCR to 38.141-2: 71 GHz Extension BS EVM Clause 6.6 and Annex L (6.6, L),” Keysight Technologies UK Ltd

- R4-2219502, “Discussions on open issues on FR2-2 PDSCH requirements,” Huawei, HiSilicon

- R4-2219503, “Simulation results on FR2-2 PDSCH requirements,” Huawei, HiSilicon

- R4-2219504, “Simulation results on FR2-2 PDCCH requirements,” Huawei, HiSilicon

- R4-2219505, “Discussions on open issues on FR2-2 CQI requirements,” Huawei, HiSilicon

- R4-2219506, “Simulation results on FR2-2 CQI requirements,” Huawei, HiSilicon

- R4-2219507, “Discussions on open issues on FR2-2 PUSCH demodulation requirements,” Huawei, HiSilicon

- R4-2219508, “Simulation results on FR2-2 PUSCH demodulation requirements,” Huawei, HiSilicon

- R4-2219509, “Simulation results on FR2-2 PUCCH demodulation requirements,” Huawei, HiSilicon

- R4-2219510, “Simulation results on FR2-2 PRACH performance requirements,” Huawei, HiSilicon

- R4-2219511, “Draft CR: Introduction on FR1+FR2-2 CA PDSCH requirements,” Huawei, HiSilicon

- R4-2219512, “Draft CR: Introduction on FR2-2 PUSCH requirements in TS 38.141-2,” Huawei, HiSilicon

- R4-2219513, “Draft CR: Introduction on FR2-2 PUCCH requirements in TS 38.104,” Huawei, HiSilicon

- R4-2219514, “Big CR on FR2-2 UE demodulation performance requirements in TS 38.101-4,” Huawei, HiSilicon

- R4-2219729, “Discussion on PUSCH demodulation requirements for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2219730, “PUSCH simulation results for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2219731, “Draft CR 38.104: PUSCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel

- R4-2219732, “PUCCH simulation results for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2219733, “Draft CR 38.141-2: PUCCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel

- R4-2219734, “Remaining issues for PRACH demodulation requirements in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2219735, “PRACH simulation results for demodulation requirements for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2219736, “Draft CR 38.104: PRACH requirements for FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2219737, “Draft CR 38.141-2: PRACH requirements for FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2219747, “Big CR to 38.104: demodulation requirements introduction for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2219748, “Big CR to 38.141-2: demodulation requirements introduction for FR2-2,”	Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2219814, “Adding DL RMC for band n263,” Huawei, HiSilicon

- R4-2219838, “Discussion on General FR2-2 UE Demodulation,” Qualcomm Inc.

- R4-2219839, “Updated Simulation Results for FR2-2 UE Demodulation PDSCH,” Qualcomm Inc.

- R4-2219840, “Updated Simulation Results for FR2-2 UE Demodulation PDCCH/PBCH,” Qualcomm Inc.

- R4-2219842, “Discussion on FR2-2 CQI Requirements,” Qualcomm Incorporated

- R4-2219843, “FR2-2 BS conformance test consideration,” Keysight Technologies UK Ltd

- R4-2219896, “60 GHz UE PTRS capabilities,” Qualcomm Inc.

- R4-2219899, “CR to 38.101-2 on 60GHz EVM PTRS capability,” Qualcomm Inc.

- R4-2219970, “draft CR to TS 38.141-2 on FR2-2 implementation (sections 1,2,3,6.7.1, 6.7.2, 6.7.3, 6.7.4, 7.6),” Huawei, HiSilicon

- R4-2220026, “System parameters maintenance for FR2-2,” Intel Corporation

- R4-2220030, “Remaining issues for FR2-2 RRM,” Qualcomm Inc.

- R4-2220042, “CR on Measurement Procedures,” Qualcomm Inc.

- R4-2220043, “Draft CR on TC - PSCell addition and release delay,” Qualcomm Inc.

- R4-2220044, “Draft CR on TC - RRC Connection Release with re-direction,” Qualcomm Inc.

- R4-2220045, “Big CR for NR operation to 71GHz RRM performance requirements,” Qualcomm Inc.

- R4-2220052, “Topic summary for [105][206] NR_ext_to_71GHz_RRM_1,” Moderator (Huawei)

- R4-2220053, “Topic summary for [105][207] NR_ext_to_71GHz_RRM_2,” Moderator (Qualcomm)

- R4-2220083, “Topic summary for [105][103] NR_ext_to_71GHz,” Moderator (Qualcomm)

- R4-2220132, “Summary for [105][306] NR_exto71GHz_BSR,”	Moderator (Huawei)

- R4-2220146, “Summary for [105][320] NR_exto71GHz_Demod_Part1,” Moderator (Nokia)

- R4-2220147, “Summary for [105][321] NR_exto71GHz_Demod_Part2,” Moderator (Qualcomm)

- R4-2220174, “draft CR for TS38.104 FRC table for FR2-2 PUSCH demodulation,” Ericsson

- R4-2220175, “draft CR for TS38.141-2 FRC table for FR2-2 PUSCH demodulation,” Ericsson

- R4-2220176, “Draft CR 38.104: PUSCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2220177, “Draft CR: Introduction on FR2-2 PUSCH requirements in TS 38.141-2,” Huawei, HiSilicon

- R4-2220178, “Draft CR 38.141-2: PUCCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2220179, “Draft CR: Introduction on FR2-2 PUCCH requirements in TS 38.104,” Huawei, HiSilicon

- R4-2220180, “Draft CR 38.104: PRACH requirements for FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2220181, “Draft CR 38.141-2: PRACH requirements for FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2220182, “WF for FR2-2 BS demodulation requirements,” Nokia

- R4-2220183, “Draft CR for PDSCH FRCs for requirements in FR2-2,” Apple Inc.

- R4-2220184, “draft CR on PDSCH requirements for 52.6 - 71 GHz band,” Ericsson

- R4-2220185, “DraftCR on PDCCH requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2220186, “Draft CR for Introducing CSI reporting requirements for 52.6 - 71 GH,” Apple Inc.

- R4-2220187, “WF for FR2-2 UE demodulation part	Qualcomm Inc.

- R4-2220200, “draftCR to 38.141-2: OTA OBW requirements for FR2-2,” NEC

- R4-2220201, “Draft CR to TS 38.141-2 in clauses 6.3 to 6.5 for extending current NR operation to 71GHz,” Nokia, Nokia Shanghai Bell

- R4-2220202, “Draft CR to TS 38.141-2: Addition of FR2-2 aspects in clause 4,” Ericsson

- R4-2220203, “DraftCR to 37.941: 71 GHz Extension TE MU update,” Keysight Technologies UK Ltd

- R4-2220204, “draftCR to 38.141-2: 71 GHz Extension BS EVM Clause 6.6 and Annex L (6.6, L),” Keysight Technologies UK Ltd

- R4-2220205, “FR2-2 Test equipment MU for BS conformance testing,” Keysight Technologies UK Ltd

- R4-2220206, “draft CR to TS 38.141-2 on FR2-2 implementation (sections 1,2,3,6.7.1, 6.7.2, 6.7.3, 6.7.4, 7.6) ,” Huawei, HiSilicon

- R4-2220207, “Draft CR to TR 37.941: Addition of aspects related to EIRP measurement in CATR relevant for FR2-2 in sub-clause 7.3, 8.3, 9.2.3 and 9.2.7,” Ericsson

- R4-2220208, “Draft CR for 38.141-2, On FR2-2 BS RF conformance testings in clauses 7.1-7.5 and respective MU and TT,” CATT

- R4-2220209, “Draft CR for 37.941, On FR2-2 RX directional requirements in clauses 10.1-10.5 and respective MU and TT summary,” CATT

- R4-2220210, “Draft CR to TR 37.941: Addition of aspects related to EIS measurement in CATR relevant for FR2-2 in sub-clause 10.2.3 and sub-clause 10.2.7,” Ericsson

- R4-2220211, “draft CR to TS38.141-2 Clause 7.7, 7.8,” ZTE Corporation

- R4-2220212, “draft CR to TS38.141-2 Clause 7.9,” ZTE Corporation

- R4-2220280, “WF for FR2-2 MU calculation,” Huawei

- R4-2220283, “Draft CR 38.104: PUSCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2220284, “Draft CR 38.141-2: PUCCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2220286, “Draft CR: General test applicability of FR2-2 UE demodulation requirements introduction,” Qualcomm Inc.

- R4-2220287, “Draft CR to TS 38.141-2: Addition of FR2-2 aspects in clause 4,” Ericsson

- R4-2220288, “Draft CR: Introduction on FR1+FR2-2 CA PDSCH requirements,” Huawei, HiSilicon

- R4-2220307, “big draft CR to TS38.141-2 FR2-2 RF conformance introduction,” Huawei

- R4-2220318, “WF on Rel-17 NR extension to 71GHz RRM Core requirements maintenance,” Huawei

- R4-2220319, “CR on QCL-ed assumption for inter-frequency RSSI measurement in FR2-2,” LG Electronics Inc.

- R4-2220320, “CR on Measurement Procedures,” Qualcomm Inc.

- R4-2220321, “WF on Rel-17 NR extension to 71GHz RRM Performance requirements,” Huawei

- R4-2220445, “Draft CR on test cases for beam failure detection and link recovery with CCA for FR2-2,” CATT

- R4-2220446, “Draft CR on test cases for SCell Activation and deactivation for SCell in FR2 in non-DRX with CCA for FR2-2,” CATT

- R4-2220447, “Draft CR on test cases for SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 with CCA for FR2-2,” CATT

- R4-2220448, “Draft CR on introduction of intra-frequency and inter-frequency measurement test cases with CCA for FR2-2,” vivo

- R4-2220449, “CR on corrections to timing test cases in FR2-2,” Nokia, Nokia Shanghai Bell

- R4-2220450, “RLM test cases for 71GHz with CCA,” Ericsson

- R4-2220451, “DraftCR on signaling characteristics test cases for FR2-2,” Huawei, HiSilicon

- R4-2220576, “Adding DL RMC for band n263,” Huawei, HiSilicon

- R4-2220821, “Adding DL RMC for band n263,” Huawei, HiSilicon

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template