3GPP TSG RAN meeting #98-e									       RP-223145

Electronic Meeting, December 12-16, 2022

Status Report to TSG

Agenda item:			9.2.14

| WI / SI Name | Enhancement for 700/800/900MHz band combinations |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_700800900_combo_enh |  |  |  |  |
| Unique ID | 950070 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222554 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2023 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: 80% | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Huiping Shan |
|  | Company | CATT |
|  | Email | shanhuiping@catt.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

The followings are the agreements in RAN4#104be:

TR and TPs

- R4-2217712	Skeleton of TR 38.872

- R4-2217711	TR 38.872 v0.2.0

- R4-2217713	TP and discussion on CA_n5-n28

- R4-2217714	TP and discussion on CA_n8-n20-n28

- R4-2217715	TP and discussion on CA_n5-n8

For CA_n5-n8, the followings are agreed:

Issue 2-1: Whether 2 antenna architecture is down selected for the requirements study

Issue 2-2: Which architecture is based for the UE RF requirement if two architectures are allowed?

- Both 2 and 3 antenna architectures will be analysed in the study item

  - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements.

Issue 2-3: RF parameters for requirements analysis

- Proposals

The following parameters are needed for the feasibility study:

    - n5 Tx filter attenuation at n8 Rx frequency range

    - n5 Rx filter attenuation level at n8 Tx frequency range

    - n8 Tx filter attenuation at n5 Rx frequency range

    - n8 Rx filter attenuation level at n5 Tx frequency range

    - antenna ISO

    - RF front end loss

- Agreements

    - Companies are encouraged to provide the RF parameters when the feasibility study is conducted.

    - The parameters are not limited to above.

Issue 2-4: Whether the filter can be dedicated, based on the restricted frequency range.

- Proposals

  - Option 1: Yes

  - Option 2: No

- Agreements

    - Both single band filter and dedicated filter can be used in the study.

Issue 2-5: Possible solutions for CA_n5-n8

- Proposals

  - Option 1: n8 Tx restricted RBs

  - Option 2: n8 TX power reduction

  - Option 3: non-simultaneous Rx/Tx between n5 DL and n8 UL

  - Option 4: restrict to n5 UL only for 1UL/2DL NR CA_n5-n8

- Agreements

    - All of the solutions can be candidates at current stage. The detail solution for the corresponding RF architecture can be analyzed and discussed in future meetings.

    - The above solutions are for full filters of n5/n8.

Issue 2-6: IMD for 2UL CA_n5-n8

- Proposals

  - Option 1: No need to define the IMD MSD requirement for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8.

- Agreements

    - No need to define the IMD MSD requirement for UL CA_n5-n8 considering the frequency ranges restriction of n5 and n8.

    - MSD due to n5 DL and n8 UL overlapping is further studied in both 1UL and 2UL cases.

    - This does not prevent from agreeing on any solutions given in Issue 2-5.

For CA_n5-n28, the followings are agreed:

Issue 3-1: Whether the requirements for 1UL CA_n5-n28 can be reused for 2UL CA_n5-n28

Agreements: Revisit the 2UL cross band MSD compared with 1UL CA in next meeting.

Issue 3-2: Whether both 2 antenna and 3 antenna should be allowed for the requirement analysis

Agreement:

- Both 2 and 3 antenna architectures will be analysed in the study item

  - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements.

Issue 3-3: RF parameters for requirements analysis

Agreements: Companies are encouraged to provide the RF parameters when the feasibility study is conducted.

Issue 3-4: Other observations and proposals

Agreements: Companies are encouraged to provide the MSD analysis for CA_n5-n28 considering the already specified 1UL CA_n5-n28 and DC_28-n5.

- Architecture assumption and multiplexer isolation performance needs to be clarified

For CA_n8-n20-n28, the followings are agreed:

- Use 3 antenna UE architecture as the baseline for evaluation.

  - Do not preclude 2 and 4 antennas in the study item

Three antenna architecture(s)

- Assuming it is the worst case, analysis is prioritized for 3 antenna architecture based on:

  - n28UL/n20+n28DL/n20UL triplexer on antenna 1

  - n8UL/n8DL duplexer on antenna 2

  - n20+n28DL/n8DL duplexer on antenna 3

  - For comparison purpose, analysis of alternate two or three antenna solutions are not precluded, and if a worse case is found it can be discussed whether it should be used for the MSD feasibility.

- 10db antenna isolation is used for MSD calculations

- The normal Tx/Rx 50dB isolation is used between UL and DL of the same FDD bands for duplexer/triplexer/quadplexer. If higher insertion loss is needed to meet the 50dB it can be accounted in DeltaT/R

- For different FDD bands within triplexer/quadplexer companies are encouraged to provide feasible isolation between ULs and DLs

- For diversity path, the DL filter provide at least 40dB of Rejection of the UL of the same band and companies are encouraged to provide feasible rejection for UL of other bands.

- Companies are encouraged to provide input on the feasibility of such values

MSD studies

Way forward on IMD3 of CA_n20-n8 UL in n28 DL:

- 5MHz n8 UL at 887.5MHz with 25RB0

- 5MHz n20 UL at 834.5MHz with 25RB0

- 5MHz n28 DL at 781.5MHz

- IMD3 MSD is evaluated by calculation, simulation, or measurement

Way forward on IMD3 of CA_n20-n28 UL in n8 DL:

- 5MHz n20 UL at 834.5MHz with 25RB0

- 5MHz n20 UL at 715.5MHz with 25RB0

- 5MHz n8 DL at 952.5MHz

- IMD3 MSD is evaluated by calculation, simulation, or measurement

The followings are the agreements in RAN4#105:

TR and TPs

- R4-2220504  TP for TR 38.872 on CA_n5-n8 architectures

- R4-2220505  TP on CA_n5-n28

- R4-2220506  TP for TR 38.872 to capture the RF architecture assumption and IMD evaluation for CA_n8-n20-n28

- R4-2218418  TR 38.872 v0.3.0

For CA_n5-n8, the followings are agreed:

RF architectures

Baseline RF architecture

Way forward:

Full n5 and n8 RF filters are agreed as a baseline.

Note: This agreement does not preclude analysis using dedicated RF filters to be captured in the TR. This agreement does not preclude considering dedicated RF filters as discussed in section 7.

Information to be captured in TR with each RF architecture

Way forward:

Each RF architecture captured in TR should have an assessment on MSD, potential antenna challenges and RF filter challenges

Number of antennas to be used for deriving requirements

It was agreed in previous meeting that the decision on if 2 or 3 antenna RF architecture is used to specify requirements would be made only in WI phase. While this agreement certainly offers the flexibility, the work is not converging. It is suggested to make further agreements here.

Way forward:

Two-antenna architecture is agreed to be used as baseline.

Three-antenna architecture can be also analysed.

Overlap of n5 DL and n8 UL with baseline RF architecture

There have been multiple contributions and a lot of discussion on how to manage the overlap between n5 DL and n8 UL when full band n5 and n8 RF filters are used. This topic is essential to be solved to finalise the SI. We need to converge the work, so concrete options are proposed as WF

Way forward:

Companies to propose which of the methods is used to manage the overlap of n5 DL and n8 UL

- Option 1: Restricting UL support to n5 UL only

- Option 2: 2UL but Non-concurrent operation between n5 DL and n8 UL

  - NOTE: Check if this kind of configuration is allowed according to current RAN2 specifications

Baseline RF architecture for deriving the requirements

The work seems to be converging towards either architectures supporting n5 UL only in CA_n5-n8 or architectures supporting 2UL but non-concurrent operation of n5 DL and n8 UL. For both of these high-level alternatives there are a few different RF architecture options.

The following architectures have been discussed:

Two antenna:

Three antenna:

Way forward:

Companies to propose which RF architecture is used derive requirements

NOTE: Requirements are not specified during the SI

MSD/ΔTIB/ΔRIB evaluations

While no requirements are to be specified during SI phase, it is still beneficial to collect companies evaluations on different architectures.

Way forward:

Companies to provide evaluations on the architectures illustrated in section 5 and in section 7:

- CA_n5-n8 with n5 UL only:

  - n8 Cross-band MSD

  - ΔTIB/ΔRIB

  - REFSENS impact due to the loading of the overlapping n8 UL and n5 DL, if applicable in the architecture

- CA_n5-n8 with 2UL and non-concurrent n8 UL and n5DL:

  - n8 Cross-band MSD for 1UL and 2UL

NOTE: For 2UL there is no IMD, but still if 1UL Cross-band MSD is >0dB, then also respective 2UL cross-band MSD may be >0dB

  - ΔTIB/ΔRIB

- CA_n5-n8 with concurrent 2UL/2DL using dedicated RF filtering

  - Cross-band MSD for 1UL and 2UL

NOTE: For 2UL there is no IMD, but still if 1UL Cross-band MSD is >0dB, then also respective 2UL cross-band MSD may be >0dB

  - ΔTIB/ΔRIB

Usage of dedicated RF filters

This topic is very important to be captured properly in the TR, with the purpose to support 2UL 2DL concurrent operation. A clear distinction between full band n5 and n8 RF filters and dedicated RF filters must be made, and the conditions under which requirements could be specified using dedicated RF filters must be clarified.

Information to be captured in TR

Way forward:

RF architectures using dedicated filters can be captured in TR, but they shall not be mixed with RF architectures using full band n5 and n8 RF filters

Each RF architecture captured in TR should have an assessment on MSD, potential antenna challenges and RF filter challenges

The following RF architectures have been discussed:

NOTE: nX and nY mean the restricted frequency ranges in table below.

|  | UL | DL |
| --- | --- | --- |
| Frequency x (800MHz) | 824MHz - 835MHz | 869MHz - 880MHz |
| Frequency y (900MHz) | 904MHz - 915MHz | 949MHz - 960MHz |

Two antenna:

Three antenna:

Conditions for using dedicated RF filters in specifying requirements

Way forward:

Study the conditions under which requirements could be specified using dedicated RF filters

- Option 1: New bands would be required

- Option 2: New bands would not be required

- Option 3: Other

For CA_n5-n28, the followings are agreed:

Reference architectures for CA_n5-n28

Issue 2-1: The two antenna reference architecture

- Proposals

  - Option 1:  Architecture a and b (Apple R4-2218368, Skyworks R4-2218387)

  - Option 2: Only architecture a (Xiaomi R4-2219260, Huawei R4-2219617)

  - Option 3: Only architecture b (Qualcomm R4-2219873 provided analysis for architecture b)

- Recommended WF

  - Option 1 for further feasibility study.

    - Need to check the n28 (dual duplexer) can be supported with Quadplexer in architecture A.

    - Need to check the n28 (dual duplexer) can be supported with Triplexer in architecture B.

Issue 2-2: The three antenna reference architecture

- Proposals

  - Option 1: The following architecture (Apple, Skyworks, Xiaomi, Huawei, Qualcomm)

- Recommended WF:

  - Option 1.

For n28 implementation, consider separate RF chain for band n28A (703 to 733 MHz / 758 to 788 MHz) and n28B (718 to 748 MHz / 773-803 MHz).

RF requirements impact for CA_n5-n28

Issue 2-3: Tib/Rib for three antenna reference architecture

- Proposals

  - Option 1: all are zeroes (ZTE)

Table 9: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

Table 10: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

- Recommended WF

- For CA_n5-n28 with 3 antenna implementation, the TIB,c and RIB,c values are shown below.

Table 9: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

Table 10: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

Issue 2-4: 1UL cross band MSD issue

- Proposals

  - Option 1: Apply single UL MSD specified in 38.101-1 for dual UL MSD for CA_n5-n28 (Murata R4-2218129)

  - Option 2: The MSD requirement for DC_28-n5 can be reused for CA_n5-n28 with 2UL. (Xiaomi R4-2219260)

  - Option 3: MSD proposal in R4-2219617 (Huawei)

Table 1 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |
| n5 | n28 | 834 | 20 | 15 | 20 (Rbstart=0) | 888 | 30 | 2.9 | ACLR2 |

- Recommended WF

- Issue 2-4: 1UL cross band MSD issue

  - Based on the agreed CR R4-2206134, one MSD test configuration (n28 30MHz) can be a reference considering the impacts from band n5 UL.

Table 5.2.2.5-1 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (Rbstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

Issue 2-5: 2UL cross band MSD issue

- Proposals

  - Option 1: Two UL n5 and n28 combined MSD for n28DL is further studied for different DL channel bandwidths and UL allocations, in order to define a specific 2UL cross-band MSD for n28 DL under the combined interference on n28 and n5UL. (Skyworks R4-2218387)

- Recommended WF

  - The following reference exception can be further studied.

Table 5.2.2.5-1 MSD due to two UL band cross band interference for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (Rbstart=0) | TBD | TBD | TBD | TBD |
| n28 |  | TBD | TBD | 15 | TBD |  |  |  |  |

For CA_n8-n20-n28, the followings are agreed:

Antenna architecture(s) discussed in RAN4#105.

Multiple companies provided input [2-10] to this band combinations with 3 antenna approaches which are summarized below. 2-antenna and 4 antenna baseline options that were discussed are also shown:

3-antenna Architecture baseline (n28ATX_n28A/n20RX_n20TX Triplexer on Antenna 1, n8TX/n8RX Duplexer on Antenna 2, and n28A/n20RX_n8RX Diplexer on Antenna 3,

2-antenna Architecture baseline (n28ATX_n28A/n20RX_n20TX_n8TX Quad-plexer on Antenna 1, and n28A/n20RX_n8RX_n8TX Triplexer on Antenna 2,

4-antenna Architecture baseline (n8TX/n8RX Duplexer on Antenna 1, n28A/n20RX_n28TX Duplexer on Antenna 2, n28A/n20RX_n20TX Duplexer on Antenna 3, and n8 RX filter on Antenna 4

- Note that in this band combinations, the n28 frequency range is reduced and can be accounted for in the architecture while still supporting full n28 range for other combinations.

3 antenna architecture shown seems like the best compromise with some optimization option for the support of the upper/lower 30MHz of n28 and should be analysed in priority. However, if proponents want to provide analysis for alternate solutions, it should not be precluded.

Similarly, although not prioritized, analysis of the delta for a two-antenna case may be useful for comparison.

Other architecture aspects:

Given that three antenna architecture is still the baseline:

Way forward:

- Provide MSD analysis for the TR (Technical Report)

- Assuming it is the worst case, analysis is prioritized for 3 antenna architecture based on:

  - n28UL/n20+n28DL/n20UL triplexer on antenna 1

  - n8UL/n8DL duplexer on antenna 2

  - n20+n28DL/n8DL duplexer on antenna 3

  - For comparison purpose, analysis of alternate two or three antenna solutions are not precluded, and if a worse case is found it can be discussed whether it should be used for the MSD feasibility.

- 10db antenna isolation is used for MSD calculations

- The normal Tx/Rx 50dB isolation is used between UL and DL of the same FDD bands for duplexer/triplexer/quadplexer. If higher insertion loss is needed to meet the 50dB it can be accounted in DeltaT/R

- For different FDD bands within triplexer/quadplexer companies are encouraged to provide feasible isolation between ULs and DLs

- For diversity path, the DL filter provide at least 40dB of Rejection of the UL of the same band and companies are encouraged to provide feasible rejection for UL of other bands.

- Companies are encouraged to provide input on the feasibility of such values.

MSD studies

No change in the agreement in the WF from RAN4#104-bis-e [11]. Companies [2, 8, 10] have already provided MSD analysis given the previous WF. The analysis/results of the baseline architecture shall be considered for the technical report.

2.4.2	Remaining Open issues

For CA_n5-n8:

Tib/Rib and MSD studies for 1UL and 2UL.

For CA_n5-n28:

MSD study for 2UL.

For CA_n8-n20-n28:

Tib/Rib and MSD study for CA_n8-n20-n28.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

RAN4#104be meeting contributions:

R4-2215445	CA_n5-n8 Feasibility and assumptions for UE-UE RF Requirements	Murata Manufacturing Co Ltd.

R4-2215662	On CA_n5-n8 architectures	Apple

R4-2215663	On CA_n5-n28 architectures	Apple

R4-2215664	On CA_n8-n20-n28 architectures	Apple

R4-2215896	Feasible on CA band combination of n5-n8	ZTE Corporation

R4-2215897	RF requirements on CA band combination of n5-n8	ZTE Corporation

R4-2215898	RF requirements on CA band combination of n5-n28	ZTE Corporation

R4-2216039	Discussion on feasibility study for CA_n5-n8	Xiaomi

R4-2216040	Discussion on feasibility study for CA_n5-n28	Xiaomi

R4-2216041	Discussion on feasibility study for CA_n8-n20-n28	Xiaomi

R4-2216136	Further discussion on feasibility aspects for 700800900MHz CA band combinations	vivo

R4-2216137	Further discussion on RF requirement impacts for 700800900MHz CA band combinations	vivo

R4-2216340	Considerations on CA_n5-n8	Qualcomm Finland RFFE Oy

R4-2216345	Considerations on CA_n5-n28	Qualcomm Finland RFFE Oy

R4-2216346	Considerations on CA_n8-n20-n28		Qualcomm Finland RFFE Oy

R4-2216679	CA_n5-n8 with restricted frequency range		Skyworks Solutions Inc.

R4-2216870	CA_n5-n28 MSD for 2UL case	Skyworks Solutions Inc.

R4-2216876	CA_n5-n20-n28 MSD for 2UL case	Skyworks Solutions Inc.

R4-2217708	WF on CA_n5-n8	Xiaomi

R4-2217709	"WF on CA_n5-n28"	ZTE

R4-2217710	WF on CA_n8-n20-n28	Skyworks

R4-2217711	TR 38.872 v0.2.0	CATT

R4-2217712	Skeleton of TR 38.872	CATT

R4-2217713	TP and discussion on CA_n5-n28	Huawei, HiSilicon

R4-2217714	TP and discussion on CA_n8-n20-n28	Huawei, HiSilicon

R4-2217715	TP and discussion on CA_n5-n8	Huawei, HiSilicon

R4-2217777	Email discussion summary for [104-bis-e][125] FS_NR_700800900	Moderator (CATT)

RAN4#105 meeting contributions:

R4-2218128	CA_n5-n8 feasibility discussion	Murata Manufacturing Co Ltd.

R4-2218129	CA_n5-n28 MSD due to 2 UL and feasibility	Murata Manufacturing Co Ltd.

R4-2218130	CA_n8-20-n28 Architecture and MSD due to IMD	Murata Manufacturing Co Ltd.

R4-2218294	Non-simultaneous Rx/Tx between n5 DL and n8 UL	Nokia, Nokia Shanghai Bell

R4-2218368	TP for TR 38.872 on CA_n5-n28 architectures	Apple

R4-2218369	TP for TR 38.872 on CA_n8-n20-n28 architectures	Apple

R4-2218387	MSD due to 2UL configuration for CA_n5-n28	Skyworks Solutions Inc.

R4-2218388	Blocking and de-sense issues with n8 UL for CA_n5-n8 in both 1UL and 2UL configurations	Skyworks Solutions Inc.

R4-2218418	TR 38.872 v0.3.0	CATT

R4-2218484	Discussion of the solutions for CA_n5-n8	CATT

R4-2218737	Amendment of the requirement on TX power management 	Sony, Ericsson

R4-2218865	Further discussion on feasibility aspects and RF requirements impact for CA_n5-n8	vivo

R4-2218866	Further discussion on feasibility aspects and RF requirements impact for CA_n5-n28	vivo

R4-2218867	Further discussion on feasibility aspects and RF requirements impact for CA_n8-n20-n28	vivo

R4-2219205	Feasible on CA band combination of n5-n8	ZTE Corporation

R4-2219206	TP to TR38.872: TIB,c and RIB,c values for n5-n28	ZTE Corporation

R4-2219259	Discussion on CA_n5-n8	Xiaomi

R4-2219260	Discussion on CA_n5-n28	Xiaomi

R4-2219261	Discussion on CA_n8-n20-n28	Xiaomi

R4-2219262	TP on CA_n5-n8	Xiaomi

R4-2219264	TP on CA_n8-n20-n28	Xiaomi

R4-2219617	Discussion on RF architecture and IMD evaluation for CA_n5-n28	Huawei, HiSilicon

R4-2219618	TP for TR 38.872 to capture the RF architecture assumption and IMD evaluation for CA_n5-n28	Huawei, HiSilicon

R4-2219619	Discussion on RF architecture and IMD evaluation for CA_n8-n20-n28	Huawei, HiSilicon

R4-2219621	Discussion on RF architecture and potential solutions for CA_n5-n8	Huawei, HiSilicon

R4-2219622	TP for TR 38.872 to capture the RF architecture assumption for CA_n5-n8	Huawei, HiSilicon

R4-2219870	Considerations on CA_n5-n8	Qualcomm Finland RFFE Oy

R4-2219873	Considerations on CA_n5-n28	Qualcomm Finland RFFE Oy

R4-2219874	Considerations on CA_n8-n20-n28	Qualcomm Finland RFFE Oy

R4-2220104	Topic summary for [105][124] FS_NR_700800900	Moderator (CATT)

R4-2220504	TP for TR 38.872 on CA_n5-n8 architectures	Apple

R4-2220505	TP on CA_n5-n28	Xiaomi

R4-2220506	TP for TR 38.872 to capture the RF architecture assumption and IMD evaluation for CA_n8-n20-n28	Huawei, HiSilicon

R4-2220507	WF on UE architecture and other aspects for CA_n5-n8	Qualcomm

R4-2220508	WF on UE architecture and other aspects for CA_n5-n28	Vivo

R4-2220809	WF on UE architecture and other aspects for CA_n8-n20-n28	Murata

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template