3GPP TSG RAN Meeting #98-e		                                                                 RP-223148

Electronic Meeting, December 12-16, 2022

Source:	CATT

Title:	Remaining open issues of Rel-18 multi-carrier enhancement for NR

Agenda Item:	9.3.1.2

Document for:	Discussion and Decision

Introduction

The WI of Rel-18 multi-carrier enhancement [1] was not complete from RAN1 perspective after RAN1#111 and additional TU in 1Q 2023 was requested to complete the open issues. In this contribution, we provide our views on the remaining open issues in RAN1.

Discussion

Multi-carrier scheduling with a single DCI

In RAN1#110b-e, it was agreed that HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable for multi-cell scheduling.

Agreement Confirm below working assumption: Working Assumption HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable when multi-cell PDSCH scheduling is configured.

The issues on reference PDSCH for HARQ-ACK feedback timing determination, last DCI for PUCCH resource determination and DAI counting for DCI format 1_X have been discussed in several RAN1 meetings without conclusion.,

In additional, since the PDSCHs in different co-scheduled cells may be in different slots, multi-cell scheduling may have impact on the procedure of Type-1 HARQ-ACK codebook generation. How to generate Type-1 HARQ-ACK codebook for DCI format 1_X requires further discussion.

In our view, the remaining open issues for HARQ enhancement include the following:

  - Reference PDSCH for HARQ-ACK feedback timing determination

  - Last DCI for PUCCH resource determination

  - DAI counting for DCI format 1_X

  - Type-1 HARQ-ACK codebook generation for DCI format 1_X

Observation 1: The remaining open issues on HARQ enhancements for multi-carrier scheduling with single DCI include:

- Reference PDSCH for HARQ-ACK feedback timing determination

- Last DCI for PUCCH resource determination

- DAI counting for DCI format 1_X

- Type-1 HARQ-ACK codebook generation for DCI format 1_X

In RAN1#110 meeting, it agreed to consider Type-1/2/3 field for discussing field design of DCI format 0_X/1_X as following. Particularly, Type-1B refers to use a single field to indicate each of co-scheduled cells via a joint indication, which may bring additional spec impact.

Agreement For discussing field design of DCI format 0_X/1_X which schedules more than one cell, reformulate the types of DCI fields as below: - Type-1 field: - Type-1A field: A single field indicating common information to all the co-scheduled cells - Type-1B field: A single field indicating separate information to each of co-scheduled cells via joint indication - Type-1C field: A single field indicating an information to only one of co-scheduled cells - Type-2 field: Separate field for each of the co-scheduled cells - Type-3 field: Common or separate to each of the co-scheduled cells, or separate to each sub-group, dependent on explicit configuration. - Note: One sub-group comprises a subset of co-scheduled cells where a single field is commonly applied to the co-scheduled cell(s) belonging to a same sub-group. - Note: Handling of any parameters applicable to multi-cell scheduling where corresponding fields are not included in DCI format 0_X/1_X (if any) will be separately discussed.

In the past four RAN1 meeting, the type of each field in DCI format 0_X/1_X was discussed with agreements in the appendix. However, the design details of fields in DCI format 0_X/1_X are still pending in RAN1. For example, several potential options on the indicator of co-scheduled cells field were discussed in RAN1 meeting without any conclusion.  Although FDRA field was agreed as Type-2 field, the details on FDRA field size compression still require further discussion. In addition, it is still controversial whether including the UL/SUL indicator field in DCI format 0_X. What’s more, the Type-1B field may require additional RRC parameters for DCI format 0_X/1_X, and will affect the discussion in RAN2. Thus, the remaining open issues on DCI format 0_X/1_X field design at least including following:

  - Details on indicator of co-scheduled cells

  - Details on FDRA field

  - UL/SUL indicator

  - Details on Type-1B fields

    - BWP indicator

    - Rate matching indicator

    - ZP CSI-RS trigger

    - TCI

    - SRS request

    - SRS offset indicator

Observation 2: The remaining open issues on DCI format 0_X/1_X field design include:

  - Details on indicator of co-scheduled cells

  - Details on FDRA field

  - UL/SUL indicator

  - Details on Type-1B fields

    - BWP indicator

    - Rate matching indicator

    - ZP CSI-RS trigger

    - TCI

    - SRS request

    - SRS offset indicator

Multi-carrier UL TX switching scheme

Most of the design aspects of multi-carrier UL TX switching have been completed by RAN1#111 meeting. There are two remaining open issues to be decided in RAN1.

RAN4 sent an LS [3] to RAN1 on switching period location determination and asked RAN1 to further discuss the details. The scheme for switching period location determination may have impact on the RRC parameters. Thus, it is better to handle this issue in the next RAN1 meeting.

Issue 4: Location of switching period - For single-TAG case, RAN4 agreed to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1. Meanwhile, RAN4 has not concluded on the switching period location for 2-TAG case, with further discussions ongoing.

In RAN1#111, an agreement on minimum separation time between two UL Tx switching was achieved to reduce complexity of UE as following. However, there is still a working assumption for further confirmation by companies. This issue should be handled in the next RAN1 meeting as well.

Agreement Following restrictions are applied for Rel-18 UL Tx switching across 3 or 4 bands. - The UE does not expect to perform more than one uplink switching within a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active UL bandwidth parts of the bands in the band combination - If there are two consecutive intra-band carriers in one band, µUL, 1 = max(µUL, 1-1, µUL, 1-2), where µUL, 1-1 and µUL, 1-2 are SCSs of active UL bandwidth parts of the carriers in the band - (working assumption) If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching. - X us is subject to UE capability with a value set of {0us, 500us}

Thus, the remaining opens on Multi-carrier UL TX switching scheme at least include following:

  - Switching period location determination

  - Minimum separation time between two UL Tx switching

Observation 3: The remaining open issues on Multi-carrier UL TX switching scheme include:

  - Switching period location determination

  - Minimum separation time between two UL Tx switching

In a summary, we propose to address the following open issues on Rel-18 multi-carrier enhancement for NR in RAN1 if additional TU is agreed:

- On HARQ enhancements issues

- Reference PDSCH for HARQ-ACK feedback timing determination

- Last DCI for PUCCH resource determination

- DAI counting for DCI format 1_X

- Type-1 HARQ-ACK codebook generation for DCI format 1_X

- On DCI format 0_X/1_X field design

  - Details on indicator of co-scheduled cells

  - Details on FDRA field

  - UL/SUL indicator

  - Details on Type-1B types fields

    - BWP indicator

    - Rate matching indicator

    - ZP CSI-RS trigger

    - TCI

    - SRS request

    - SRS offset indicator

- On Multi-carrier UL TX switching scheme

  - Switching period location determination

  - Minimum separation time between two UL Tx switching

Proposal 1: RAN1 is tasked to address the following remaining open issues on Rel-18 multi-carrier enhancement for NR if additional TU is agreed:

- On HARQ enhancements issues

- Reference PDSCH for HARQ-ACK feedback timing determination

- Last DCI for PUCCH resource determination

- DAI counting for DCI format 1_X

- Type-1 HARQ-ACK codebook generation for DCI format 1_X

- On DCI format 0_X/1_X field design

  - Details on indicator of co-scheduled cells

  - Details on FDRA field

  - UL/SUL indicator

  - Details on Type-1B types fields

    - BWP indicator

    - Rate matching indicator

    - ZP CSI-RS trigger

    - TCI

    - SRS request

    - SRS offset indicator

- On Multi-carrier UL TX switching scheme

  - Switching period location determination

  - Minimum separation time between two UL Tx switching

Conclusion

In this contribution, the remaining open issues on Rel-18 MC_enh are discussed. Particularly, we have following observations and proposal.

Observation 1: The remaining open issues on HARQ enhancements for multi-carrier scheduling with single DCI include:

- Reference PDSCH for HARQ-ACK feedback timing determination

- Last DCI for PUCCH resource determination

- DAI counting for DCI format 1_X

- Type-1 HARQ-ACK codebook generation for DCI format 1_X

Observation 2: The remaining open issues on DCI format 0_X/1_X field design include:

  - Details on indicator of co-scheduled cells

  - Details on FDRA field

  - UL/SUL indicator

  - Details on Type-1B fields

    - BWP indicator

    - Rate matching indicator

    - ZP CSI-RS trigger

    - TCI

    - SRS request

    - SRS offset indicator

Observation 3: The remaining open issues on Multi-carrier UL TX switching scheme include:

  - Switching period location determination

  - Minimum separation time between two UL Tx switching

Proposal 1: RAN1 is tasked to address the following remaining open issues on Rel-18 multi-carrier enhancement for NR if additional TU is agreed:

- On HARQ enhancements issues

- Reference PDSCH for HARQ-ACK feedback timing determination

- Last DCI for PUCCH resource determination

- DAI counting for DCI format 1_X

- Type-1 HARQ-ACK codebook generation for DCI format 1_X

- On DCI format 0_X/1_X field design

  - Details on indicator of co-scheduled cells

  - Details on FDRA field

  - UL/SUL indicator

  - Details on Type-1B types fields

    - BWP indicator

    - Rate matching indicator

    - ZP CSI-RS trigger

    - TCI

    - SRS request

    - SRS offset indicator

- On Multi-carrier UL TX switching scheme

  - Switching period location determination

  - Minimum separation time between two UL Tx switching

References

- RP-22251 “Revised WID on Multi-carrier enhancements”, RAN#97e, NTT DOCOMO,INC

- RP-223164 “Status report of WI: Multi-carrier enhancements for NR; rapporteur: NTT DOCOMO,INC”, RAN#98e, RAN1

- R4-2217741“LS on Rel-18 UL Tx switching”, RAN4#104-e-bis

Appendix

RAN1#109-e

Agreement

For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered:

- Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells.

  - The table is configured by RRC signaling.

  - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X

  - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not

- Other options are not precluded.

- Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

RAN1#110

Agreement

For DCI format 1_X/0_X which can schedule more than one cell,

- Type-1 fields at least include below:

  - Type-1A:

    - Identifier for DCI formats

    - Downlink assignment index

    - TPC for scheduled PUCCH

    - PUCCH resource indicator

    - PDSCH-to-HARQ timing indicator

    - One-shot HARQ-ACK request

- Type-2 fields at least include below:

  - New data indicator per TB

  - Redundancy version per TB

- FFS: Other fields to be included in DCI format 1_X/0_X and which type of the fields belongs to.

- FFS: size for each field

RAN1#110b-e

Agreement:

At least below fields are excluded from DCI format 1_X/0_X:

- CBGTI

- CBGFI

- PDSCH group index

- New feedback indicator

- Number of requested PDSCH group(s)

- Sidelink assignment index

- Second TPC command for scheduled PUSCH

- Second SRS resource indicator

- Second Precoding information

- Second PTRS-DMRS association

- Second TPC command for scheduled PUCCH

Agreement:

For DCI format 1_X/0_X, Type-1 fields at least include below:

- Priority indicator

- Indicator of co-scheduled cells

- beta offset indicator

- CSI request

- UL-SCH indicator

- FFS: ChannelAccess-CPext

Agreement

For a set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the set of cells are jointly indicated by a single TDRA field in the DCI format 0_X/1_X.

- Separate {SLIV, mapping type, scheduling offset K0 (or K2)} is indicated for each of co-scheduled PDSCHs/PUSCHs.

- FFS details of the TDRA table design

RAN1#111

Agreement

For DCI format 1_X/0_X,

- Type-1 fields at least include below:

  - ChannelAccess-Cpext

  - TDRA

- Below fields are agreed to be supported for DCI format 0_X/1_X. FFS: Whether the fields are type1, type2, type configurable, or omitted. FFS: details on the fields (e.g. length, which legacy configurations are applicable), other fields.

  - HARQ process number

  - MCS (FFS: potential compression scheme)

  - Bandwidth part indicator

  - Frequency domain resource assignment (FFS: potential compression scheme)

  - VRB-to-PRB mapping

  - PRB bundling size indicator

  - Rate matching indicator

  - ZP CSI-RS trigger

  - Antenna port(s)

  - Transmission configuration indication

  - DMRS sequence initialization

  - Frequency hopping flag

  - TPC command for scheduled PUSCH

  - Precoding information and number of layers

  - PTRS-DMRS association

  - SRS request

  - SRS resource indicator

  - SRS offset indicator

  - PTRS-DMRS association

  - Open-loop power control parameter set indication

  - UL/SUL indicator

Note: RAN1 strives to minimize the number of fields which are type configurable.

Agreement

The types for below fields in DCI format 1_X are listed (R1-2212924):

| Field | Type | Details (for information only) |
| --- | --- | --- |
| HARQ process number | Type 2 | Details in Section 7.1.1 |
| MCS | Alt 1: Type 2 (without compression) | Details in Section 7.1.2 |
| BWP indicator | Type 1A | Details in Section 7.1.3 |
| FDRA | Type 2 - Further consider larger RBG granularity than existing maximum specified or configured value for RA type 0 - Use large RBG-based RIV for RA type 1 based on R16 configurable granularities for DCI format 1_2 | Details in Section 7.1.4 |
| VRB-to-PRB mapping | Type 1A | Details in Section 7.1.5 |
| PRB bundling size indicator | Type 1A | Details in Section 7.1.6 |
| Rate matching indicator | Type 1B (up to 4 bits) | Details in Section 7.1.7 |
| ZP CSI-RS trigger | Type 1B (up to 3 bits) | Details in Section 7.1.8 |
| Antenna port(s) | Configurable between Type 1A and Type 2 | Details in Section 7.1.9 |
| TCI | Type 1B (up to 4 bits) | Details in Section 7.1.10 |
| DMRS sequence initialization | Type 1A | Details in Section 7.1.11 |
| SRS request | Type 1B (up to 4 bits) | Details in Section 7.1.12 |
| SRS offset indicator | Type 1B (up to 3 bits) | Details in Section 7.1.13 |

This does not imply that payload of DCI can be larger than what is supported for polar code in Rel-17.

FFS: Details

Agreement

- The types for below fields in DCI format 0_X are listed:

| Field | Type | Details (for information only) |
| --- | --- | --- |
| HARQ process number | Type 2 | Details in Section 7.2.1 |
| MCS | Alt 1: Type 2 (without compression) | Details in Section 7.2.2 |
| BWP indicator | Type 1A | Details in Section 7.2.3 |
| FDRA | Type 2 - Further consider larger RBG granularity than existing maximum specified or configured value for RA type 0 - Use large RBG-based RIV for RA type 1 based on R16 configurable granularities for DCI format 1_2 | Details in Section 7.2.4 |
| Frequency hopping flag | Type 1A | Details in Section 7.2.5 |
| TPC command for scheduled PUSCH | Type 2 | Details in Section 7.2.6 |
| Open-loop power control parameter set indication | Type 1A | Details in Section 7.2.7 |
| Antenna port(s) | Configurable between Type 1A and Type-2 | Details in Section 7.2.8 |
| Precoding information and number of layers | Configurable between Type 1A and Type-2 | Details in Section 7.2.9 |
| PTRS-DMRS association | Type 2 | Details in Section 7.2.10 |
| DMRS sequence initialization | Type 1A | Details in Section 7.2.11 |
| SRS request | Type 1B (up to 4 bits) | Details in Section 7.2.12 |
| SRS resource indicator | Configurable between Type 1A and Type-2 | Details in Section 7.2.13 |
| SRS offset indicator | Type 1B (up to 3 bits) | Details in Section 7.2.14 |
| UL/SUL indicator | FFS | Details in Section 7.2.15 |

This does not imply that payload of DCI can be larger than what is supported for polar code in Rel-17.

FFS: Details