3GPP TSG RAN Meeting #98-e						                                        RP-223149

Electronic Meeting, December 12 - 16, 2022

Agenda item:	9.11

Source: 	CATT

Title: 	Discussion on BWP operation without bandwidth restriction

Document for:	Discussion

1 Introduction

In the RAN#97 meeting, RAN discussed the solutions for the cases that UE supports the BWP operation without bandwidth restriction and is not configured with CSI-RS based on RAN1 and RAN4 LS. The conclusions are captured in the summary [1]. It is agreed that no new solutions shall be added Rel-17 and RAN4 is asked to do a high-level analysis of the options in RAN4’s LS RP-221911 and report it to RAN#98 for RAN decision.

RAN4 has done the high-level analysis in the two meetings in 2022 Q4 and provide the LS [2] to RAN#98. In this paper, we provide our views on the solutions based on RAN4 analysis and give our proposals.

2 Discussion

2.1 Results of the High level analysis

Based on the RAN4 LS [2], they gave the analysis on the following options which are more detailed and down selected from the options in RP-221911:

- Candidate options

  - Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP

  - Option B) Perform BM/RLM/BFD based on SSB outside active BWP

    - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD

      - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions

      - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions

    - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps

      - Option B-2-2) Dedicated MG or NCSG for RLM/BFD/BM measurements

  - Option C) NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

And the analysis for each option includes the impacts on the following four aspects:

  - RRM requirements impact (Spec impact) / workload in RAN4

  - Mobility performance impact

  - Throughput impact (Data interruption)

  - UE power consumption / UE complexity

From the analysis results provided by RAN4, we can see all the options have low impact on the mobility performance, so we can skip this aspect. For the other aspects (RRM requirements, power consumption, UE complexity and throughput), the impacts of each option are summarized as below:

- For option A) and option C), the impacts on all the four aspects are low (or none). (4 low)

- For option B-1-1), the impacts on RRM requirements and throughput are low (or none), but the impact on UE complexity is possibly medium and the impact on UE power consumption is high. (2 low, 1 medium and 1 high)

- For option B-1-2), the impacts on all the four aspects are possibly medium. (4 medium)

- For option B-2-2), the impacts on RRM requirements, UE complexity and throughput are possibly medium and the impact on UE power consumption is low. (1 low and 3 medium)

Based on the summaries above, we can put the solutions into three categories:

- CAT 1: option A) and option C), for which all the impact are very low;

- CAT 2: option B-1-1), for which the RRM impact and throughput impact are low but UE implementation related impact is medium;

- CAT 3: option B-1-2) and B-2-2), for which the RRM impact and throughput impact are medium and UE implementation related impact is also medium.

Observation 1: Based on RAN4 analysis, the solutions can be put into three categories:

- CAT 1: option A) and option C);

- CAT 2: option B-1-1);

- CAT 3: option B-1-2) and B-2-2).

2.2 Solutions to be specified in R18

Based on the report [2] from RAN4, we need to decide in RAN plenary which solutions to be specified in R18 and which WI would be used to specify them. When we decide the solutions, we think we can consider three aspects: the workload (RRM requirements and signalling needed), the performance that the solutions can reach (throughput), UE implementation (complexity and power consumption).

But before we make the decision for this issue, we need to keep it in mind that the TU allocation for RAN4 is already overloaded and it is better not to introduce too much new features in R18 at this stage. Based on the analysis, we understand most of the work would be within RAN4 scope and some solutions require RAN2 work e.g. signalling is clearly needed for option B-2-2). Considering the workload of different work groups, we think we don’t need to specify all the solutions above in R18 and some down selections are needed.

Proposal 1: There is no need to specify all the solutions, i.e. some down selections are needed.

Based on the analysis results from RAN4, option A) and option C) can be considered to be specified firstly in R18 due to low impact on all the aspects. We think these two solutions are sufficient and it’s better not to further specify other solutions.

If companies have strong demand for other options, the RRM impact and throughput impact for option B-1-1) are acceptable, and the impact of other aspects is due to different UE implementations which can be optional for UE. So this option can be a second alternative.

For option B-1-2) and B-2-2), the RRM impact is medium which means higher workload in RAN4 and the throughput impact is medium which means worse performance. Especially for option B-2-2), RAN2 work is also needed which requires more coordination between different groups. And also MG/NCSG for serving cell measurement (i.e. option B-2-2) is basically to introduce a new procedure which is not even typical. So we think it is not very necessary and these two options are not preferable.

Proposal 2: Option A) and option C) are recommended as baseline solutions for Rel-18.

For the WI to be used, since the work is generally to specify RRM requirements for existing procedure, we think R18 RRM enhancement is proper or we can introduce a small TEI WI for this BWP without restriction feature.

Proposal 3: The solutions can be specified in R18 further RRM enhancement WI or a new introduced TEI WI.

3 Summary

This contribution further discusses the options for BWP without restriction, and the following is proposed.

Proposal 1: There is no need to specify all the solutions, i.e. some down selections are needed.

Proposal 2: Option A) and option C) are recommended as baseline solutions for Rel-18.

Proposal 3: The solutions can be specified in R18 further RRM enhancement WI or a new introduced TEI WI.

4 References

[1] RP-222630 Moderator's summary for discussion [97e-05-BWP-WithoutRestriction]

[2] RP-222725 (R4-2220437) LS on BWP operation without bandwidth restriction

Annex RAN4 LS to RAN

The following is copied from RAN4 LS [2]:

1. Overall Description:

In the RAN#97 meeting, RAN asks RAN4 to do a high-level analysis of the options in RAN4’s answer to Q2 in RP-221911 and report it to RAN#98 for RAN decision. RAN4 has done the high-level analysis in the two RAN4 meetings and RAN4 would like to provide report as follows.

In the RAN4#104-bis-e meeting, RAN4 agreed to perform high-level analysis from the following 4 aspects/criteria.

- Agreements

- RAN4 works on the below aspects/criteria for highest-level analysis on options for UE performing RLM/BFD/BM when CD-SSB is outside active BWP

  - RRM requirements impact (Spec impact) / workload in RAN4

  - Mobility performance impact

  - Throughput impact (Data interruption)

  - UE power consumption / UE complexity

In the RAN4#105 meeting, RAN4 agreed to deprioritize some of the options and the final candidate options for high-level analysis and for RAN decision are as below.

- Agreement

- Candidate options

  - Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP

  - Option B) Perform BM/RLM/BFD based on SSB outside active BWP

    - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD

      - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions

      - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions

    - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps

      - Option B-2-2) Dedicated MG or NCSG for RLM/BFD/BM measurements

  - Option C) NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

RAN4 high-level analysis for candidate options for UE performing RLM/BFD/BM when CD-SSB is outside active BWP is summarized in Table 1, Table 2, Table 3, and Table 4, respectively.

Table 1: High-level analysis of candidate options on RRM requirements impact/workload in RAN4

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | - CSI-RS based RLM/BFD/BM requirements are already specified. - Further study is needed to decide on whether timing requirements may need to be updated | Low |
| Option B-1-1) | - Existing SSB based RLM/BFD/BM measurement requirements will apply. - The applicability rule of existing requirements is to be updated. - Depending on RF BW and BB BW (FFT BW) assumption, at least intra-frequency measurement with gap need to be revisited | Low |
| Option B-1-2) | - Same as Option B-1-1). Additionally, - Interruption requirements need to be developed additionally to allow UE for switching. | Low or Medium |
| Option B-2-2) | - Existing requirements need to be updated for SSB based RLM/BFD/BM with dedicated NCSG. - Existing requirements need to be updated for SSB based RLM/BFD/BM with dedicated MG. | NCSG: Low or Medium MG: Medium |
| Option C) | - Existing requirements for SSB based RLM/BFD/BM can be re-used. - Clarification on the requirement applicability might be needed. | Low |

Note: “Low”/ “Low to Medium”/ “Medium” correspond to the low/low to medium/medium RRM requirements impact.

Table 2: High-level analysis of candidate options on UE power consumption / UE complexity

| Options | Technical analysis | Summary |  |
| --- | --- | --- | --- |
|  |  | Power consumption | UE complexity |
| Option A) | - UE works in active BWP. - No additional implementation complexity is needed | Low | Low |
| Option B-1-1) | - Depending on RF BW and BB BW (FFT BW) assumption, UE can consume much higher power than other options - If BB BW is assumed adapted to BWP BW, it should be FFS how interruption can be avoided and its impact on UE complexity | High | Low to medium |
| Option B-1-2) | - UE works in larger RF BW than active BWP periodically and UE may consume larger power than other options except Option B-1-1. | Medium | Low to Medium |
| Option B-2-2) | - UE works in active BWP. | Low | Medium |
| Option C) | - UE works in active BWP | Low | Low |

Note: “Low”/ “Low to Medium”/ “Medium” correspond to the low/low to medium/medium negative impact on UE power consumption and UE complexity comparing to the legacy solution.

Table 3: High-level analysis of candidate options on Mobility performance impact

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | - Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-1-1) | - No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement - Can also help L3 intra-frequency measurement | Low |
| Option B-1-2) | - No measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements without gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option B-2-2) | - Measurement gap is needed for intra-frequency L3 measurement: i.e., legacy intra-frequency L3 measurements with gap requirements apply. - Measurement gap is needed for inter-frequency L3 measurement | Low |
| Option C) | - Measurement gap may or may not be needed for intra-frequency L3 measurement depending on whether or not NCD-SSBs from all neighbor cells are present within UE active BWP. - Measurement gap is needed for inter-frequency L3 measurement. | Low |

Note: “Low” - means a limited impact on mobility performance comparing to the legacy solution.

Table 4: High-level analysis of candidate options on Throughput impact (Data interruption)

| Options | Technical analysis | Summary |
| --- | --- | --- |
| Option A) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |
| Option B-1-1) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |
| Option B-1-2) | Interruption is needed for BM/RLM/BFD measurements | Low to Medium |
| Option B-2-2) | NCSG or MG is needed for BM/RLM/BFD measurements | NCSG: Low to Medium MG: Medium |
| Option C) | No NCSG/interruption or MG is needed for BM/RLM/BFD measurements | None |

Note 1: Throughput impact due to L3 measurements is not considered.

Note 2: “Low”/ “Medium” correspond to the low/medium negative impact on throughput comparing to the legacy solution.