3GPP TSG RAN meeting #98-e									RP-223164

Electronic meeting, December 12-16, 2022

Status Report to TSG

Agenda item:			9.3.1.2

| WI / SI Name | Multi-carrier enhancements for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MC_enh |  |  |  |  |
| Unique ID | 940094 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222251 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: 06/2024 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 70% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Hiroki Harada |
|  | Company | NTT DOCOMO, INC. |
|  | Email | hiroki.harada@docomo-lab.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The total amount of RAN1 TU was 3 TU in the agreed TU excel sheet, and RAN1 plan is to complete their work for this WI at RAN#98-e. However, although many agreements and progress were made at RAN1#110bis-e and RAN1#111 meetings, there are number of remaining RAN1 discussion points to complete this WI in RAN1. Therefore, it is requested in this status report to have an additional RAN1 TU for RAN1#112 meeting (1 TU).

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

9.10.1	Multi-cell PUSCH/PDSCH scheduling with a single DCI

RAN1#110bis-e

Agreement

Confirm the following working assumption reached in RAN1#110 meeting.

Working Assumption

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

- FFS: The maximum number of configurable cells for co-scheduling

Agreement

At least the following fields are excluded from DCI format 1_X/0_X:

- CBGTI

- CBGFI

- PDSCH group index

- New feedback indicator

- Number of requested PDSCH group(s)

- Sidelink assignment index

- Second TPC command for scheduled PUSCH

- Second SRS resource indicator

- Second Precoding information

- Second PTRS-DMRS association

- Second TPC command for scheduled PUCCH

Agreement

For DCI format 1_X/0_X, Type-1 fields at least include the following:

- Priority indicator

- Indicator of co-scheduled cells

- beta offset indicator

- CSI request

- UL-SCH indicator

- FFS: ChannelAccess-CPext

Agreement

Confirm below working assumption reached in RAN1#110 meeting with revision.

Working Assumption

- For any cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, RAN1 specification supports monitoring the DCI format 0_X/1_X and DCI format 0_0/1_0, 0_1/1_1, and/or 0_2/1_2 (if supported by the UE), if configured from a same scheduling cell.

- The DCI format 0_X/1_X and the DCI format 0_0/1_0/0_1/1_1/0_2/1_2 can be monitored simultaneously.

- Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell.

Agreement

For a set of cells co-scheduled by a DCI format 0_X/1_X, time domain resource allocations for the set of cells are jointly indicated by a single TDRA field in the DCI format 0_X/1_X.

- Separate {SLIV, mapping type, scheduling offset K0 (or K2)} is indicated for each of co-scheduled PDSCHs/PUSCHs.

- FFS details of the TDRA table design

Agreement

Confirm below working assumption:

Working Assumption

HARQ-ACK codebook types (Type-1, Rel-15 Type-2, Rel-16 Type-3, Rel-17 Type-3) are applicable when multi-cell PDSCH scheduling is configured.

Working Assumption

For a set of cells which is configured for multi-cell scheduling,

- Existing DCI size budget is maintained on each cell of the set of cells.

- DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - FFS which cell DCI size of the DCI format 0_X/1_X is counted on.

- BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - FFS which cell BD/CCE of the DCI format 0_X/1_X is counted on.

- Search space of DCI format 0_X/1_X is configured on one cell of the set of cells and associated with the search space of the scheduling cell with the same search space ID.

  - FFS which cell the SS of the DCI format 0_X/1_X is configured on.

- FFS: How to address Rel-17 BD/CCE limit for any given cell (operating the feature under Rel-17 BD/CCE limit)

- Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell.

Agreement

- UE does not expect to be configured both multi-PDSCH scheduling and multi-cell PDSCH scheduling on the same or different cells within a same PUCCH group.

Agreement

- For Type-2 HARQ-ACK codebook, if at least one cell of a set of cells which can be co-scheduled by DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell of the set of cells is equal to M, where M is the maximum number of TBs which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

Agreement

- For Type-2 HARQ-ACK codebook, a DCI format 1_X scheduling more than one cell is associated with the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

- If a UE is scheduled by a DCI format 1_X to receive PDSCH over multiple cells, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a cell from the multiple cells, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the cell is an uplink symbol, the UE does not receive the PDSCH in the cell.

- If a UE is scheduled by a DCI format 0_X to transmit PUSCH over multiple cells, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a cell from the multiple cells, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the cell is a downlink symbol, the UE does not transmit the PUSCH in the cell.

RAN1#111

Agreement

Confirm the RAN1#110bis-e working assumption with the following changes:

Working Assumption

For a set of cells which is configured for multi-cell scheduling,

- Existing DCI size budget is maintained on each cell of the set of cells.

- DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - DCI size of the DCI format 0_X/1_X is counted on the reference cell.

- BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells.

  - BD/CCE of the DCI format 0_X/1_X is counted on the reference cell.

- Same reference cell is used for both DCI format 0_X and DCI format 1_X.

- The reference cell is

  - the scheduling cell if the scheduling cell is included in the set of cells and search space of the DCI format 0_X/1_X is configured only on the scheduling cell;

  - one cell of the set of cells which search space of DCI format 0_X/1_X is configured on and associated with the search space of the scheduling cell with the same search space ID if search space of the DCI format 0_X/1_X is configured on the cell in addition to the scheduling cell.

    - It is up to gNB on which cell the SS of the DCI format 0_X/1_X is configured on.

- To address Rel-17 BD/CCE limit for any given cell (operating the feature under Rel-17 BD/CCE limit)

  - For the reference cell, a total number of configured BD/CCEs for both DCI formats 0_X/1_X and legacy DCI formats (if configured) does not exceed the Rel-17 limits.

  - For other cells in the sets of cells, Rel-17 limits for PDCCH/DCI monitoring and BD/CCE counting rules for legacy DCI formats (not including DCI formats 0_X/1_X) apply

- Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e.,  and ) for PDCCH candidates for each scheduled cell.

Agreement

For a set of cells which is configured for multi-cell scheduling, up to 4 cells within the set of cells are supported.

- A DCI format 0_X/1_X can schedule PUSCH(s)/PDSCH(s) on a combination of co-scheduled cells among the same set of cells.

Agreement

For DCI format 1_X/0_X,

- Type-1 fields at least include below:

  - ChannelAccess-Cpext

  - TDRA

- Below fields are agreed to be supported for DCI format 0_X/1_X. FFS: Whether the fields are type1, type2, type configurable, or omitted. FFS: details on the fields (e.g. length, which legacy configurations are applicable), other fields.

  - HARQ process number

  - MCS (FFS: potential compression scheme)

  - Bandwidth part indicator

  - Frequency domain resource assignment (FFS: potential compression scheme)

  - VRB-to-PRB mapping

  - PRB bundling size indicator

  - Rate matching indicator

  - ZP CSI-RS trigger

  - Antenna port(s)

  - Transmission configuration indication

  - DMRS sequence initialization

  - Frequency hopping flag

  - TPC command for scheduled PUSCH

  - Precoding information and number of layers

  - PTRS-DMRS association

  - SRS request

  - SRS resource indicator

  - SRS offset indicator

  - PTRS-DMRS association

  - Open-loop power control parameter set indication

  - UL/SUL indicator

Note: RAN1 strives to minimize the number of fields which are type configurable.

Agreement

For monitoring PDCCH candidates for a set of cells which is configured for multi-cell scheduling, the n_CI in the search space equation is determined by a value configured for the set of cells by RRC signaling.

Agreement

The types for below fields in DCI format 1_X are listed (R1-2212924):

| Field | Type | Details (for information only) |
| --- | --- | --- |
| HARQ process number | Type 2 | Details in Section 7.1.1 |
| MCS | Alt 1: Type 2 (without compression) | Details in Section 7.1.2 |
| BWP indicator | Type 1A | Details in Section 7.1.3 |
| FDRA | Type 2 - Further consider larger RBG granularity than existing maximum specified or configured value for RA type 0 - Use large RBG-based RIV for RA type 1 based on R16 configurable granularities for DCI format 1_2 | Details in Section 7.1.4 |
| VRB-to-PRB mapping | Type 1A | Details in Section 7.1.5 |
| PRB bundling size indicator | Type 1A | Details in Section 7.1.6 |
| Rate matching indicator | Type 1B (up to 4 bits) | Details in Section 7.1.7 |
| ZP CSI-RS trigger | Type 1B (up to 3 bits) | Details in Section 7.1.8 |
| Antenna port(s) | Configurable between Type 1A and Type 2 | Details in Section 7.1.9 |
| TCI | Type 1B (up to 4 bits) | Details in Section 7.1.10 |
| DMRS sequence initialization | Type 1A | Details in Section 7.1.11 |
| SRS request | Type 1B (up to 4 bits) | Details in Section 7.1.12 |
| SRS offset indicator | Type 1B (up to 3 bits) | Details in Section 7.1.13 |

This does not imply that payload of DCI can be larger than what is supported for polar code in Rel-17.

FFS: Details

Agreement

- The types for below fields in DCI format 0_X are listed:

| Field | Type | Details (for information only) |
| --- | --- | --- |
| HARQ process number | Type 2 | Details in Section 7.2.1 |
| MCS | Alt 1: Type 2 (without compression) | Details in Section 7.2.2 |
| BWP indicator | Type 1A | Details in Section 7.2.3 |
| FDRA | Type 2 - Further consider larger RBG granularity than existing maximum specified or configured value for RA type 0 - Use large RBG-based RIV for RA type 1 based on R16 configurable granularities for DCI format 1_2 | Details in Section 7.2.4 |
| Frequency hopping flag | Type 1A | Details in Section 7.2.5 |
| TPC command for scheduled PUSCH | Type 2 | Details in Section 7.2.6 |
| Open-loop power control parameter set indication | Type 1A | Details in Section 7.2.7 |
| Antenna port(s) | Configurable between Type 1A and Type-2 | Details in Section 7.2.8 |
| Precoding information and number of layers | Configurable between Type 1A and Type-2 | Details in Section 7.2.9 |
| PTRS-DMRS association | Type 2 | Details in Section 7.2.10 |
| DMRS sequence initialization | Type 1A | Details in Section 7.2.11 |
| SRS request | Type 1B (up to 4 bits) | Details in Section 7.2.12 |
| SRS resource indicator | Configurable between Type 1A and Type-2 | Details in Section 7.2.13 |
| SRS offset indicator | Type 1B (up to 3 bits) | Details in Section 7.2.14 |
| UL/SUL indicator | FFS | Details in Section 7.2.15 |

This does not imply that payload of DCI can be larger than what is supported for polar code in Rel-17.

FFS: Details

9.10.2	Multi-carrier UL Tx switching scheme

RAN1#110bis-e

Agreement

If Rel-18 UL Tx switching for 3 or 4 bands with dual UL is supported, UE is allowed to support only some of band pairs for concurrent UL transmission based on UE capability

- The supported band pair for concurrent transmission requires the support of UL CA on the corresponding band pair(s) by the UE

- Details on the UE capability such as how to report the support of dual UL and the supported band pair(s) for concurrent UL transmission are further discussed

- Details on the gNB configuration/indication such as how to indicate the band pair(s) UE should expect for concurrent UL transmission are further discussed

- Note: UE is also allowed to support all band pairs for concurrent transmission, and the design of Rel-18 UL Tx switching for 3 or 4 bands with dual UL does not impose any restriction

Agreement

If Rel-18 UL Tx switching for 3 or 4 bands is supported, UE is allowed to support only some of band(s) for up to 2 ports UL transmission based on UE capability

- Further down-select from the following alternatives

  - Alt.1: no restriction for both switched UL and dual UL and for both 3 bands and 4 bands

  - Alt.2: at least one band should support up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands

  - Alt.3: at least two bands should support up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands

- Details on the UE capability such as whether existing per-FS UL-MIMO capability can be reused or not are further discussed

- Details on the gNB configuration/indication such as whether/how to additionally indicate 2 ports UL transmission mode for a band/cell are further discussed

- Existing MIMO mechanism for MIMO mode indication should be reused

- Note: UE is also allowed to support all bands for up to 2 ports UL transmission, and the design of Rel-18 UL Tx switching for 3 or 4 bands does not impose any restriction

Agreement

If Rel-18 UL Tx switching for 3 or 4 bands is supported, following is considered as baseline.

- Existing conditions where the switching period is required can be reused for Rel-18 UL Tx switching with 3 or 4 bands when only two bands are involved in a switching

- New conditions where the switching period is required should be introduced for Rel-18 UL Tx switching with 3 or 4 bands when more than two bands are involved in a switching

  - For dual UL, following new conditions are considered

    - When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band (1st band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (2nd and 3rd band)

    - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 2T on a carrier on another band (3rd band)

    - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on one of the bands and another different band (1st or 2nd band, and 3rd band)

    - When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (3rd and 4th band)

  - FFS for switched UL and/or for the case with complexity reduction option 1 or 2

  - FFS the same or different switch period for existing conditions and new conditions

Conclusion

No consensus in RAN1 on complexity reduction option 3

Agreement

- Consider following alternatives for UE capability reporting about the supported UL Tx switching options

  - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination

  - Alt.2: report {switchedUL, dualUL, both} for the band combination and report supported band pair for concurrent transmission for the band combination

- Consider following alternatives for gNB configuration regarding dual UL

  - Alt.1: configure {switchedUL, dualUL} in CellGroupConfig

  - Alt.2: configure {switchedUL, dualUL} for each band pair (combination of serving cells?)

  - Alt.3: at least configuration of supported band pair (combination of serving cells) for concurrent transmission

  - Alt.4: No configuration of supported band pair (combination of serving cells) for concurrent transmission, i.e., UE just assumes as it reports

Working Assumption

Specify UL Tx switching schemes across up to 4 bands in Rel-18

Working Assumption

If Rel-18 UL Tx switching for 3 or 4 bands is supported, both Switched UL and Dual UL are supported

Agreement

Confirm the following working assumption made at the RAN1#110 meeting.

Working Assumption

If Rel-18 UL Tx switching is supported, following switching mechanism is considered as baseline for the Rel-18 UL Tx switching across 3 or 4 bands

- Alt.1: Dynamic Tx carrier switching can be across all the supported switching cases by the UE and based on the UL scheduling, i.e., via dynamic grant and/or RRC configuration for UL transmission

Working Assumption

At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases

- Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported

  - if twoT is indicated, both of two Tx chains are switched to band B

  - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A

- Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported

  - if twoT is indicated, both of two Tx chains are switched to band C

  - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS

    - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter

    - Alt.2: based on predefined rule

    - Other alternative is not precluded

- FFS for other potential cases

Agreement

Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options

  - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination

  - Alt.2: report {switchedUL, dualUL, both} for the band combination and report supported band pair for concurrent transmission for the band combination

    - Note：If there is no report on the supported band pair(s) for concurrent transmission while the UE reports “dualUL” or “both” for the band combination, gNB may assume that the UE supports concurrent transmission on all the band pairs within the band combination

  - Alt.3: report {dualUL} for each band pair in the band combination

    - Note: Within the band combination, the UE shall be capable of being operated in switched UL mode for all band pairs

Agreement

Ask RAN2 to consider following alternatives and specify gNB configuration

  - Alt.1: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination)

  - Alt.2: configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination)

  - Alt.3: configure {switchedUL, dualUL} for all serving cells (i.e., for the band combination), and configure combination(s) of serving cells (i.e., as supported serving cell pair(s) for each band pair in the band combination) for concurrent transmission

Working assumption

Study the following alternatives for the minimum separation time between two UL Tx switchings for Rel-18 UL Tx switching schemes across up to 3 or 4 bands, and decide in RAN1#111 whether/which of the following alternatives is needed

- Alt.1: define 14 symbols based on a SCS (FFS on SCS) as minimum separation time between two UL Tx switchings

- Alt.2: define that no more than one uplink Tx switching within a reference slot based on a SCS (FFS on SCS)

- Alt.3: define X slots as minimum separation time between two UL Tx switchings where 3 bands are involved in total, and define Y slots as minimum separation time between two UL Tx switchings where 4 bands are involved in total, where X and/or Y is no less than 1 (FFS on X,Y, FFS reference SCS for the slots in case of multiple SCSs across carriers or expressed in unit of micro second)

- Alt.4: report the minimum separation time for different switching cases

- Other alternative is not precluded

- FFS: Applicable cases for the restriction

- Note: Companies are encouraged to provide detailed numbers of minimum separation time

Agreement

Consider following alternatives on the supported switching cases (Tx chain states) for each scenario

- Scenario#1: For switched UL, if UE supports up to 2 ports UL transmission on all the bands in the band combination,

  - Alt.1-1: only switching cases (Tx chain states) with 2T are assumed

    - In case of 3 bands, 3 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}) are assumed

    - In case of 4 bands, 4 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}) are assumed

  - Alt.1-2: switching cases (Tx chain states) with 1T-1T can also be assumed

    - FFS: detailed switching cases to be assumed

- Scenario#2: For switched UL, if UE supports up to 2 ports UL transmission only on some of the bands,

  - Alt.2-1: for the band where 2 ports UL transmission is not supported, switching cases (Tx chain states) with 1T-1T can be assumed

    - FFS: detailed switching cases to be assumed with different number of bands supporting up to 2 ports UL transmission

  - Alt.2-2: only switching cases (Tx chain states) with 2T are assumed

    - Assumed switching cases are same as Scenario#1

  - Alt.2-3: switching cases (Tx chain states) with 1T-1T can also be assumed

    - FFS: detailed switching cases to be assumed

- FFS: Scenario#3: For dual UL, if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission on all the bands in the band combination,

  - Alt.3-1: corresponding switching case(s) with 1T-1T for the band pair(s) are not assumed

    - FFS: if UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission only on some of the bands

  - Alt.3-2: corresponding switching case(s) with 1T-1T for the band pair(s) are assumed

    - Assumed switching cases are same as the case where UE supports dual UL for all band pairs in the band combination

Agreement

LS on UE capability and gNB configuration for UL Tx switching across 3 or 4 bands in Rel-18 is endorsed. Final LS in R1-2210724.

RAN1#111

Agreement

For switched UL, if UE supports up to 2 ports UL transmission on all the bands in the band combination, only switching cases (Tx chain states) with 2T are assumed

- Conclusion: In case of 3 bands, 3 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}) are assumed

- Conclusion: In case of 4 bands, 4 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}) are assumed

- Based on the assumption, the switching gap is required for every UL transmission with changing transmitting band from preceding transmission in this scenario

Agreement

For switched UL, if UE supports up to 2 ports UL transmission only on some of the bands in the band combination, only switching cases (Tx chain states) with 2T are assumed

- Based on the assumption, the switching gap is required for every UL transmission with changing transmitting band from preceding transmission in this scenario

Agreement

For dual UL, if a UE does not support concurrent transmission on specific band pair(s) and supports up to 2 ports UL transmission on all the bands in the band combination, corresponding switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed

Agreement

For dual UL, if UE supports concurrent transmission on all band pairs and supports up to 2 ports UL transmission on all the bands in the band combination, all possible switching cases with 1T-1T and 2T are assumed

- In case of 3 bands, 6 switching cases ({2T,0T,0T}, {0T,2T,0T}, {0T,0T,2T}, {1T, 1T, 0T}, {1T, 0T, 1T}, {0T, 1T, 1T}) are assumed

- In case of 4 bands, 10 switching cases ({2T,0T,0T,0T}, {0T,2T,0T,0T}, {0T,0T,2T,0T}, {0T,0T,0T,2T}, {1T,1T,0T,0T}, {1T,0T,1T,0T}, {1T,0T,0T,1T}, {0T,1T,1T,0T}, {0T,1T,0T,1T}, {0T,0T,1T,1T}) are assumed

Agreement

For dual UL, if UE supports up to 2 ports UL transmission only on some of the bands in the band combination, corresponding switching case(s) with 2T for the band where up to 2 ports transmission is not supported are assumed

- If the UE does not support concurrent transmission on specific band pair(s) in the band combination, corresponding switching case(s) with 1T-1T for the band pair(s) where concurrent transmission is not supported are not assumed

Agreement

Following new conditions are applicable to dual UL only (i.e., not applicable to switched UL)

- When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band (1st band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (2nd and 3rd band)

- When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 2T on a carrier on another band (3rd band)

- When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on one of the bands and another different band (1st or 2nd band, and 3rd band)

- When the UE is to transmit a 1-port + 1-port transmission each on one uplink carrier on different bands (1st and 2nd band) and if Tx chain state at the preceding uplink transmission is 1T + 1T each on a carrier on other different bands (3rd and 4th band)

Agreement

Following working assumption is confirmed with updates.

Working Assumption

At least for dual UL, reuse existing RRC parameter {oneT, twoT} via uplinkTxSwitching-DualUL-TxState to solve the issue on ambiguous switching state at least for following cases

- Case#1 of the issue: two Tx chains are currently associated with band A, and next transmission is 1 port transmission on band B, but there are multiple possible switching cases where 1P on band B is supported

  - if twoT is indicated, both of two Tx chains are switched to band B

  - if oneT is indicated, one Tx chain is switched to band B while another Tx chain remains on band A

- Case#2 of the issue: two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, but there are multiple possible switching cases where 1P on band C is supported

  - if twoT is indicated, both of two Tx chains are switched to band C

  - if oneT is indicated, one Tx chain is switched to band C while how to determine the associated band for another Tx chain is FFS

    - Alt.1: based on gNB’s configuration/indication e.g., new RRC parameter

    - Alt.2: based on predefined rule

    - Other alternative is not precluded

- FFS for other potential cases

Agreement

In Case#2 where two Tx chains are currently associated with band A and B, and next transmission is 1 port transmission on band C, if oneT is indicated via uplinkTxSwitching-DualUL-TxState, one Tx chain is switched to band C and associated band for another Tx chain is determined by new RRC parameter which is down-selected from following alternatives.

- An associated band is configured for each band so that another Tx chain is associated with the configured band (as associated band for the transmitting band)

  - E.g., associated band for each transmitting band is configured as {B for A}, {A for B}, {A for C} and {C for D}.

    - When 1 port transmission on band C is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band B is switched to band C while another Tx chain associated with band A remains unchanged (because band A is associated band for band C)

    - When 1 port transmission on band D is scheduled and Tx chains are currently associated with band A and B, Tx chain associated with band A (or B) is switched to band D while another Tx chain associated with band B (or A) is switched to band C (because band C is associated band for band D)

If there is one band where concurrent transmission with any other band is not supported, NW does not configure an associated band for the band. In such case, even if oneT is configured, UE performs switching as twoT is configured when 1 port transmission on the band is scheduled

Agreement

There is no restriction on number of bands supporting up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands.

- It is up to UE capability to support 2 ports UL transmission on none/some/all of the 3 or 4 bands

- Note: UE with only 1 Tx chain is not expected to perform UL Tx switching (no spec impact)

Agreement

Confirm the following working assumption.

Working Assumption

Specify UL Tx switching schemes across up to 4 bands in Rel-18

Agreement

Following restrictions are applied for Rel-18 UL Tx switching across 3 or 4 bands.

- The UE does not expect to perform more than one uplink switching within a reference slot based on µUL = max(µUL, 1, µUL, 2, µUL, 3) in case of 3 bands, µUL = max(µUL, 1, µUL, 2, µUL, 3, µUL, 4) in case of 4 bands, where µUL, 1, µUL, 2, µUL, 3, µUL, 4 are SCSs of active UL bandwidth parts of the bands in the band combination

  - If there are two consecutive intra-band carriers in one band, µUL, 1 = max(µUL, 1-1, µUL, 1-2), where µUL, 1-1 and µUL, 1-2 are SCSs of active UL bandwidth parts of the carriers in the band

- (working assumption) If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time

  - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching.

  - X us is subject to UE capability with a value set of {0us, 500us}

2.1.2	Remaining Open issues

Objectives related to RAN1 from WID on Multi-carrier enhancements for NR as below

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1]

- Consider both intra-band and inter-band CA operation

- Consider both FR1 and FR2

- The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4]

- Some potential issues including switching period location

2.2	RAN2

2.2.1	Agreements

RAN2#119bis-e

Wait for RAN1/4 (No agreement)

RAN2#120

Multi-carrier UL Tx switching scheme

R2 assumes For UE capability to report applicability of DL interruption for Rel-18 UL Tx switching, RAN2 reuses uplinkTxSwitching-DL-Interruption-r16 (no spec impact).

R2 assumes to reuse the per band per BC capability, uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17, on UL-MIMO coherence for the 2Tx-capable UL band(s) for Rel-18 UL Tx switching (fallback description FFS).

2.2.2	Remaining Open issues

All objectives related to RAN2 from WID on Multi-carrier enhancements for NR as below

- Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4]

- UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4)

  - UE capability and RRC configuration related signalling (RAN2)

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#104bis-e

<R4-2217740>

Sub-topic 1-1: Switching period for Tx switching across 3/4 bands

Issue 1-1-1: Exact value of Tx switching period

Way forward:

For the exact value of Tx switching period for each band pair, further discuss the following options:

- Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18.

  - Note: with the understanding that the switching period in Rel-18 could be different for different band pairs, according to the granularity of per band pair per BC agreed in the last meeting.

- Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 band combination with 3/4 bands.

- Option 3: Option 1 for switchedUL, and option 2 for dualUL

Sub-topic 1-2: Impact from switching of one Tx chain on the other Tx chain

Issue 1-2: Impact on the band with the number of Tx chain unchanged due to switching (Case 2)

Way forward:

- For the impact on the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, further discuss the two options in the next meeting:

  - Option 1: Introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching.

    - Potential proposals on the granularity of the optional UE capability can also be discussed.

  - Option 2: Do not define other optional features to allow the other Tx chain to be used for transmission on the band with the number of Tx chain unchanged during the switching period.

Issue 1-2A: Issue of two Tx chains switched between two different band pairs

Way forward:

- When two Tx chains are switched between two different band pairs with different lengths of switching periods, as baseline UE assumption, neither of the two Tx chains is expected to be used for transmission during the larger one of the two switching periods.

- Whether advanced optional UE ability will be considered is related to the discussion in Issue 1-2.

Sub-topic 1-3: RAN4 RF requirements

GTW Agreement

  - Following Rel-16/17 approach, common switching time mask requirements for “switched UL” case and “dual UL” case are to be defined.

  - Further discuss whether to define additional requirements for CA option 2, by considering the switching cases to be introduced in RAN1.

Sub-topic 1-4: Location of switching period

GTW Agreement

  - For single-TAG case, RAN4 agrees to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1.

Sub-topic 1-5: Applicability of DL interruption

Way forward:

- For combinations of SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

- For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

  - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

Further discuss in the next meeting:

- For SUL+TDD+FDD combinations, for Tx switching between SUL and TDD bands, whether or not DL interruption on the SUL and TDD bands is required.

<R4-2217741>

LS from RAN4 to RAN1/2 (R4-2218841) was approved

<R4-2217742>

Sub-topic 2-1: UL switching time and outage time

Issue 2-1-1: UL switching time

WF UL switching time (Oct 12 GTW agreement):

the UL switching time is the same for single TAG and dual-TAG cases

Issue 2-1-1A: on the wording in the FFS bullet

WF on the wording in the FFS bullet:

continued discussion at the next meeting.

Issue 2-1-2: UL outage time and RAN4 CR text

Issue 2-1-2A: RAN4 CR text

WF RAN4 CR text:

continued discussion at the next meeting. Two options:

Option 1: modify the time mask for TX switching to include the case of dual TAG with different timing advance on the two TAGs.

Option 2: apply same approach as in R17 V2X that the time mask only contains the UE hardware requirement (switching period), and no TA difference included. The impact of Tx switching with multiple TAGs can be considered as scheduling restriction.

Issue 2-1-2B: UL outage time

WF UL outage time:

continued discussion at the next meeting.

Sub-topic 2-2: PUSCH preparation time

WF PUSCH preparation time (Oct 12 GTW agreement):

the timing difference effect and switching period have already been considered in the PUSCH preparation time in the current specification, and further specification update is not needed.

Sub-topic 2-3: Sceanrio

WF Scenario (Oct 12 GTW agreement):

The band combinations containing intra-band non-contiguous CA is not considered in the WI.

NR-DC with multi-TAG operation is not considered in the WI.

Sub-topic 2-4: Location of switching period

WF UL location of the switching period:

continued discussion at the next meeting. The location of the switching period must be specified.

<R4-2217258>

Topic #1:	DL interruption for Tx switching across 3/4 bands

<Agreement>: Issue 1-1: RRM impact due to Rel-18 Tx switching across 3 or 4 bands

- DL interruption requirements are to be specified

- No impact on the existing MTTD requirement due to the extension of UL TX switching for 2 bands from 1 TAG to 2 TAGs.

Issue 1-2: DL interruption length

<Agreement>:

For Rel-18 Tx switching across 3/4 bands with single TAG, reuse Rel-16/Rel-17 values for length of DL interruption.

< Way forward >:

For Rel-18 Tx switching across 3/4 bands with 2 TAGs,

- Option 1: reuse Rel-16/Rel-17 values for length of DL interruption.

- Option 2: interruption on other serving cells when UL Tx switching occurs across multiple bands shall be defined as:

Tinterrupt = ceil((switching period+2*TA adjustment uncertainty+2*MRTD-CP length)/symbol duration)+1

Where

- Switching period is being discussed in RF session and the outcome can be reused in interruption design.

- TA adjustment uncertainty remains same as legacy.

- MRTD=3us which is used to derive R16/R17 DL interruption length is not valid for non-collocated case, further discuss the concrete value of MTRD

<Agreement>: Issue 1-3: Starting point of the DL interruption

For R18 Tx switching across 3/4 bands with both single TAG and 2 TAGs, reuse the Rel-16/17 agreement on the starting symbol of DL interruption

<Agreement>: Issue 1-4: Interrupted cells (i.e., victim cells) where the DL interruption is allowed at UL Tx switching

- Option 1(Nokia, Apple, vivo): Wait for RAN1 /RF conclusion on the interrupted carriers at UL Tx switching across 3 or 4 bands.

- Option 2 (China Telecom, Huawei, Ericsson, Samsung): The capability uplinkTxSwitching-DL-Interruption-r16 is unaffected in the multiple TAG case.

- Option 2a (Samsung, Huawei): For Rel-18 TX switching across 3/4 bands, no reason to change the existing requirement structure and the signaling design for uplinkTxSwitching-DL-Interruption.

RRM can directly use the conclusion of RF session or other RAN group, when decisions are made. So no discussion in 2nd round discussion.

<Agreement>: Issue 1-5: DL interruption is applicable for ENDC

- Option 1: Same principle applies for ENDC, i.e., DL interruptions in TS 38.133 are not affected, even if the time masks in UE RF requirements might have to be different.

Conclusion: It is confirmed that RAN1 has the below agreement [RP-221436]. Following RAN1 agreement, the issue is closed.

RAN1#109-e Conclusion EN-DC cases are out of scope for Rel-18 UL Tx switching

< Way forward > Issue 1-6: Applicable scenario of DL interruption requirements

Candidate options:

- Option 1: reuse Rel-16/Rel-17 values for length of DL interruption. Define DL interruption requirements for Rel-18 Tx switching across 3 or 4 bands applicable to all scenarios including:

  - Inter-band UL-CA without SUL band

  - Inter-band UL CA for {SUL band + corresponding NUL band} + 1 or 2 other NUL band(s)

  - Intra-band two contiguous aggregated carriers within one non-SUL band out of 3 or 4 bands

  - {SUL band + corresponding NUL band} + {SUL band + corresponding NUL band}:

Topic #2: UL outage time for TX switching with 2 TAGs

Issue 2-1: Concept of UL outage time

Candidate options:

- Revised Option 1: The UL outage refers to the actual impacted UL OFDM symbols on serving carriers involved in the switching.

- New Option 2: The UL outage refers to the actual impacted UL OFDM symbols on other victim serving carriers NOT involved in the switching.

- Option 3: The UL outage refers to the possible range of interrupted UL OSs (like a potential interruption window)

Issue 2-2: Factors for UL outage time

To clarify, this issue is trigged by RF WF R4-2215163 (see below). RRM is just responsible to provide possible response to RF. Whether the UL outage time is captured in spec is still under discussion in RF session

In RF WF [R4-2215163] Issue 3-1-2: UL outage time Factors for UL outage time discussed in RF session, to be further checked in RRM session - UL switching time (UE capability) - The difference between the TA on the two TAGs, up to MTTD - Timing and measurement error

Candidate options:

  - Option 1(China Telecom): UL outage time for Tx switching with 2 TAGs includes:

    - UL switching time (UE capability)

    - Half of the difference between the actual TAs on the two TAGs (i.e., not directly use the MTTD defined in TS 38.133)

    - Timing and measurement error, including:

a) BS synchronization accuracy: 3us defined in clause 7.4.2 of TS 38.133 can be used;

b) UE transmit timing error: the requirement defined in clause 7.1 of TS 38.133 can be seen an upper bound, and the sum of maximum UE transmit timing error is 1.56 us for the carriers with 2 TAGs

c) TA quantization error: as defined in TS 38.213, it can be up to 0.52 us or 0.26us for 15 kHz SCS.

  - Option 2 (Huawei, Ericsson):

    - UL interrupted OFDM symbols shall consider the impact of MTTD, however directly using MTTD value to derive impacted UL OFDM symbols are not always correct

    - The third factor “Timing and measurement error” is already included in MTTD

Proposed: Impacted UL OFDM symbols= Ceiling (UL switching period/symbol length) +1 OS

  - Option 3 (Samsung): Confirm the three factors should be considered in RF session for UL outage time:

    - (1) UL switching time (UE capability),

    - (2) the difference between the TA on the two TAGs, up to MTTD,

    - (3) timing and measurement error: typical value is 27ns

Note: other options are not precluded.

RAN4#105

<R4-2220546>

Sub-topic 1-1: Exact value of switching period for Tx switching across 3/4 bands

Issue 1-1: Exact value of Tx switching period

Way forward:

For the exact value of Tx switching period for each band pair, select one of the two options in RAN4 #106:

- Option 1: For Rel-18 UE, for a band pair within a band combination supporting Tx switching among 3/4 bands, the switching period reported by UE for Rel-18 3/4-band Tx switching is same with the switching period for Rel-16/17 2-band switching operations.

  - Note: With the understanding that the switching period in Rel-18 could be different for different band pairs, according to the granularity of per band pair per BC agreed in RAN4 #104e.

- Option 2: For Rel-18 UE, for a band pair within a band combination supporting Tx switching among 3/4 bands, the switching period reported by UE for Rel-18 3/4-band Tx switching can be the same or different from the switching period for Rel-16/17 2-band switching operations.

  - Note: the set of candidate values is still the same, i.e., {35 us, 140 us, 210 us}, according to the agreement in RAN4 #104e.

Sub-topic 1-2: Impact from switching of one Tx chain on the other Tx chain

Issue 1-2-1: Impact on the band with the number of Tx chain unchanged due to switching

Way forward:

- For the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching.

  - Down-select and decide the granularity of the optional UE capability in RAN4 #106:

    - Option 1a: per band pair per BC

    - Option 1b: per band per band pair per BC

    - Other options are not precluded

  - Note: this optional advanced UE ability is not considered for the following case, as per the RAN4 #104e agreement

    - The switching is between band A and B, one of Tx chain is switched between band A and B, the other Tx chain keep unchanged with band B.

Issue 1-2-2: Issue of two Tx chains switched between two different band pairs

Way forward:

- When two Tx chains are switched between two different band pairs with different lengths of switching periods (denoted as Tswitch_1 and Tswitch_2 for the switching periods of Tx chain #1 and Tx chain #2 respectively, and Tswitch_1 < Tswitch_2), select one of the two options in RAN4 #106:

  - Option 1: In addition to the baseline UE assumption, introduce advanced optional UE ability to allow the Tx chain #1 to be used for transmission during the time duration of (Tswitch_2 - Tswitch_1)

    - Further discuss the granularity of the optional UE capability:

  - Option 1a: per band pair per BC

  - Other options are not precluded

  - Option 2: Do not introduce the advanced optional UE ability.

Issue 1-2-3: Ambiguity issue when two Tx chains are switched between two different band pairs

Way forward:

- For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}:

  - As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}.

    - Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively.

Sub-topic 1-3: RAN4 RF requirements

Issue 1-3-1: Requirements for “switched UL”

- Further discuss in the next meeting:

  - Option 1: Define the time mask requirements for bandpair, and the switching requirements should be satisfied for all band pairs capable of Tx switching in a band combination.

  - Other options are not precluded.

Issue 1-3-2: Clarification on the common requirements for “switched UL” and “dual UL” cases

- Further discuss the following options in the next meeting:

  - Option 1: Add the following clarification for switching time mask

    - “The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL.”

  - Option 2: Add the following clarification for switching time mask

    - “The time mask is applicable to uplink transmissions when configured with switchedUL or dualUL for uplinkTxSwtichingOption which is defined in [TS 38.331]. In case of simultaneous transmission in the two bands when configured with dualUL, the time mask are the same.”

  - Other options are not precluded.

Issue 1-3-3: Requirements for “dual UL” case

- In the next meeting, bring CRs for options for further discussion:

  - Option 1: For “dual UL”, define switching time mask requirements to cover the following cases with two Tx chains switched between two different band pairs, i.e.,

    - For Tx switching across 3 bands or Tx switching across 4 bands with concurrent UL transmission supported on only one band pair, the two Tx chains are in band A before switching, and are in band B and C respectively after switching.

    - For Tx switching across 4 bands with concurrent UL transmission supported on at least two band pairs, the two Tx chains are in band A and B respectively before switching, and are in band C and D respectively after switching.

    - Other cases are not precluded.

  - Option 2:

    - Transmission requirements for Tx switching between band pair part of switching and requirements for transmissions for bands with tx chains unchanged due to switching for baseline/advanced feature for scenarios described in issues 1-2-1, 1-2-2 and 1-2-3.

  - Other options are not precluded.

Issue 1-3-4: CR language

- Further discuss the language based on the CR in the next meeting:

  - Option 1:

    - UE requirements are written assuming known network behaviour i.e., not using “UE is not expected to ..” language.

    - Language to be used for specifying UE behaviour for concurrent switching periods of different lengths:

“UE shall be capable of starting transmission after the end of switching periods on all bands that are configured for uplink in tx switching. Transient periods are not included in the switching periods.”

  - FFS other options

Sub-topic 1-4: Applicability of DL interruption

Issue 1-4-1: DL interruption for SUL+TDD+FDD combinations

Way forward:

- For SUL+TDD+FDD combinations, when Tx switching is performed between SUL and TDD bands, whether or not to mandate no DL interruption on the SUL and TDD bands can be discussed in a case by case (i.e., per band combination basis) manner.

Issue 1-4-2: Issues of different band pairs having different capabilities of DL interruption

Way forward:

- Further discuss in the next meeting that:

  - RAN4 clarify applicability of DL interruption when different band pairs have different capability of DL interruption and Tx switching is performed for both band pairs simultaneously.

Issue 1-4-3: On previous agreements for combinations of SUL+TDD and TDD+TDD CA band combinations

Way forward:

- For synchronized CA between SUL configuration (with a SUL band and a TDD band) and TDD band(s) with the same UL-DL pattern across all TDD bands:

  - Option 1: DL interruption is not required

  - Option 2: whether to mandate no DL interruption can be discussed in a case by case (i.e., per band combination basis) manner.

- For synchronized CA among 3 or 4 TDD band(s) [‘with the same UL-DL pattern’ or ‘without mandatory simultaneous Rx/Tx requirement’] across all TDD bands, DL interruption is not required.

<R4-2220547>

Sub-topic 2-1: RF specification impact

Issue 2-1-1: Need of new time mask requirements for 2-TAG case

Option 1:

Time mask with an UL time difference covering all possible time delays (R4-2218827), example for 2 bands but also relevant for 3-4 bands for both SUL and CA configurations with dual TAG.

Option 2:

Do not modify the time mask for Tx switching for multiple TAGs. The impact of Tx switching with multiple TAGs can be considered as scheduling restriction. Example below (R4-2219410) for two bands:

General understanding: only full OS are “blanked” (not scheduled or configured for transmission)

WF:

TBD

Sub-topic 2-2: UL outage time

Issue 2-2-1: Discussion and specification for UL outage time

Wed agreement:

Agreement: outage time will be discussed in RF session.

WF:

TBD

Sub-topic 2-3: Location of switching period in relation to time T0

WF: the switching period is located immediately before time T0 (start of transmission on a carrier after the switch)]

- Other options no precluded

Sub-topic 2-4: Scenario

- Proposals:

  - Option 1: Clarifications on the multi-TAG scenario are needed for UL transmission on a carrier without corresponding DL carrier. RAN4’s CR should reflect all the possible multi-TAG scenarios. (ZTE)

    - There are two interpretations when there is a SUL band included in UL Tx switching across 3/4 bands. For example, there are four bands A+B+C+D, where D is SUL band, C is the corresponding NUL band, and A and B are the other two NUL bands

  - Interpretation 1: All the bands belong to one TAG.

  - Interpretation 2: bands C and D belong to one TAG, and bands A and B belong to another TAG.

- Recommended WF

  - Further discuss.

WF:

the SUL case included; the SUL has an associated NUL (the same uplink cell) in the same TAG, a NUL of another cell can be in the other TAG

<R4-2220548>

LS from RAN4 to RAN1/2 (R4-2220548) was approved

<R4-2220417>

Topic #1:	DL interruption for Tx switching across 3/4 bands with 2TAGs

<Agreement>: Issue 1-1: RTD for non-collocated inter-band scenario for Tx switching with 2 TAGs

Agreements on GTW:

  - Use RTD=6us side condition for non-collocated inter-band case to derive DL interruption length for 2TAGs case.

  - FFS if requirements for RTD > 6us is needed

  - Note: RTD value will be captured in the specification as an informational note in the table for DL interruption length for 2TAGs as an assumption to derive the values.

<Agreement>: Issue 1-2: The principle of deriving DL interruption length for 2TAGs

As R16/R17 the interruption length in symbols due to UL TX switching is define as,

Ceil((switching period+2*TA adjustment uncertainty+2*RTD-CP length)/symbol duration)+1

<Agreement>: Issue 1-3: DL interruption length for 2TAGs

Agreements on GTW:

- Use the following DL interruption length (in symbols) for R18 Tx switching across 3/4 bands with 2 TAGs for RTD = 6us

|  | NR Slot length (ms) | Uplink Tx switching period |  |  |
| --- | --- | --- | --- | --- |
|  |  | 35us | 140us | 210us |
| 0 | 1 | 2 | 3 | 4 |
| 1 | 0.5 | 3 | 6 | [7 or 8] |
| 2 | 0.25 | 4 | 10 | 14 |

<Agreement>: Issue 1-4: Applicable scenario of DL interruption requirements

It is recommended not to further discuss this issue in RRM session. RRM can directly use the conclusion of RANP or RF session.

Topic #2: UL outage time for TX switching with 2 TAGs

This topic is closed as the below agreement is achieved in RF session.

Agreement: outage time will be discussed in RF session.

2.4.2	Remaining Open issues

All objectives related to RAN4 from WID on Multi-carrier enhancements for NR as below

-Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4]

- UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4)

- Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4)

-Specify RRM test cases related to core requirements on UL Tx switching schemes across up to 3 or 4 bands

[RAN4]

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#110bis-e

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2208426 | Discussion on multi-cell scheduling with a single DCI | Huawei, HiSilicon |
| R1-2208486 | Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI | ZTE |
| R1-2208564 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | Spreadtrum Communications |
| R1-2208658 | Discussion on multi-cell scheduling | vivo |
| R1-2208779 | Discussion on multi-cell scheduling with a single DCI | China Telecom |
| R1-2208860 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | OPPO |
| R1-2208991 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | CATT |
| R1-2208997 | Discussions on multi-cell PUSCH/PDSCH scheduling with a single DCI | Langbo |
| R1-2209020 | Consideration on multi-cell PUSCH/PDSCH scheduling with a single DCI | Fujitsu |
| R1-2209067 | Discussions on multi-cell scheduling with a single DCI | Intel Corporation |
| R1-2209239 | Discussions on multi-cell PUSCH/PDSCH scheduling with a single DCI | CAICT |
| R1-2209299 | Discussion on the design of multi-cell scheduling with a single DCI | xiaomi |
| R1-2209305 | On multi-cell scheduling via a single DCI | Lenovo |
| R1-2209352 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | CMCC |
| R1-2209416 | Discussion on Multicarrier scheduling with a single DCI | FGI |
| R1-2209426 | Discussion on Multi-cell PXSCH scheduling with a single DCI | NEC |
| R1-2209454 | Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI | LG Electronics |
| R1-2209516 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | MediaTek Inc. |
| R1-2209595 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | Apple |
| R1-2209746 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | Samsung |
| R1-2209860 | Multi-cell PUSCH/PDSCH scheduling with a single DCI | Ericsson |
| R1-2209917 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | NTT DOCOMO, INC. |
| R1-2210000 | Multi-cell PUSCH/PDSCH scheduling with a single DCI | Qualcomm Incorporated |
| R1-2210032 | Discussion on multi-cell scheduling with a single DCI | ITRI |
| R1-2210034 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | Google Inc. |
| R1-2210151 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | Nokia, Nokia Shanghai Bell |
| R1-2210350 | Feature lead summary#1 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2210351 | Feature lead summary#2 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2210352 | Feature lead summary#3 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2210662 | Feature lead summary#4 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2208427 | Discussion on multi-carrier UL Tx switching | Huawei, HiSilicon |
| R1-2208487 | Discussion on Multi-carrier UL Tx switching scheme | ZTE |
| R1-2208565 | Discussion on multi-carrier UL Tx switching scheme | Spreadtrum Communications |
| R1-2208659 | Discussion on UL TX switching | vivo |
| R1-2208780 | Discussion on UL Tx switching across up to 3 or 4 bands | China Telecom |
| R1-2208861 | Discussion on multi-carrier UL Tx switching scheme | OPPO |
| R1-2208992 | Discussion on multi-carrier UL Tx switching scheme | CATT |
| R1-2209068 | Discussions on multi-carrier UL Tx switching scheme | Intel Corporation |
| R1-2209300 | Discussion on multi-carrier UL Tx switching scheme | xiaomi |
| R1-2209353 | Discussion on multi-carrier UL Tx switching scheme | CMCC |
| R1-2209455 | Discussion on Multi-carrier UL Tx switching scheme | LG Electronics |
| R1-2209596 | On multi-carrier UL Tx switching | Apple |
| R1-2209747 | On multi-carrier UL Tx switching | Samsung |
| R1-2209772 | On multi-carrier UL Tx switching scheme | MediaTek Inc. |
| R1-2209861 | Multi-carrier UL Tx switching | Ericsson |
| R1-2209918 | Discussion on Multi-carrier UL Tx switching scheme | NTT DOCOMO, INC. |
| R1-2210001 | Discussion on Rel-18 UL Tx switching | Qualcomm Incorporated |
| R1-2210035 | Discussion on multi-carrier UL Tx switching scheme | Google Inc. |
| R1-2210192 | On Multi-Carrier UL Tx Switching | Nokia, Nokia Shanghai Bell |
| R1-2210280 | Summary#1 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2210455 | Summary#2 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2210548 | Summary#3 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2210723 | Summary#4 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2210724 | LS on UE capability and gNB configuration for UL Tx switching across 3 or 4 bands in Rel-18 | RAN1, NTT DOCOMO |

RAN1#111

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2210856 | Discussion on multi-cell scheduling with a single DCI | Huawei, HiSilicon |
| R1-2210924 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | Nokia, Nokia Shanghai Bell |
| R1-2211022 | Discussion on multi-cell scheduling | vivo |
| R1-2211045 | Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI | ZTE |
| R1-2211063 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | Google Inc. |
| R1-2211082 | Consideration on multi-cell PUSCH/PDSCH scheduling with a single DCI | Fujitsu |
| R1-2211213 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | CATT |
| R1-2211244 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | Spreadtrum Communications |
| R1-2211376 | Discussion on the remaining issues for the multi-cell scheduling with a single DCI | xiaomi |
| R1-2211414 | Discussions on multi-cell scheduling with a single DCI | Intel Corporation |
| R1-2211488 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | OPPO |
| R1-2211534 | Discussion on multi-cell scheduling with a single DCI | China Telecom |
| R1-2211585 | On multi-cell scheduling via a single DCI | Lenovo |
| R1-2211695 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | CMCC |
| R1-2211824 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | Apple |
| R1-2211878 | Discussion on Multicarrier scheduling with a single DCI | FGI |
| R1-2211919 | Discussions on multi-cell PUSCH/PDSCH scheduling with a single DCI | CAICT |
| R1-2211998 | Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI | NTT DOCOMO, INC. |
| R1-2212060 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | Samsung |
| R1-2212132 | Multi-cell PUSCH/PDSCH scheduling with a single DCI | Qualcomm Incorporated |
| R1-2212156 | Multi-cell PUSCH/PDSCH scheduling with a single DCI | Ericsson |
| R1-2212190 | Discussions on multi-cell PUSCH/PDSCH scheduling with a single DCI | Langbo |
| R1-2212252 | On multi-cell PUSCH/PDSCH scheduling with a single DCI | MediaTek Inc. |
| R1-2212303 | Discussion on Multi-cell PUSCH/PDSCH scheduling with a single DCI | LG Electronics |
| R1-2212336 | Discussion on multi-cell scheduling with a single DCI | ITRI |
| R1-2212362 | Discussion on Multi-cell PXSCH scheduling with a single DCI | NEC |
| R1-2212392 | Feature lead summary#1 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2212393 | Feature lead summary#2 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2212394 | Feature lead summary#3 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2212923 | Feature lead summary#4 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2212924 | Feature lead summary#5 on multi-cell PUSCH/PDSCH scheduling with a single DCI | Moderator (Lenovo) |
| R1-2210857 | Discussion on multi-carrier UL Tx switching | Huawei, HiSilicon |
| R1-2211023 | Discussion on UL TX switching | vivo |
| R1-2211046 | Discussion on Multi-carrier UL Tx switching scheme | ZTE |
| R1-2211064 | Discussion on multi-carrier UL Tx switching scheme | Google Inc. |
| R1-2211214 | Discussion on multi-carrier UL Tx switching scheme | CATT |
| R1-2211245 | Discussion on multi-carrier UL Tx switching scheme | Spreadtrum Communications |
| R1-2211285 | On Multi-Carrier UL Tx Switching | Nokia, Nokia Shanghai Bell |
| R1-2211377 | Discussion on multi-carrier UL Tx switching scheme | xiaomi |
| R1-2211489 | Discussion on multi-carrier UL Tx switching scheme | OPPO |
| R1-2211535 | Discussion on UL Tx switching across up to 3 or 4 bands | China Telecom |
| R1-2211623 | Considerations on Multi-carrier UL Tx switching scheme | Sony |
| R1-2211696 | Discussion on multi-carrier UL Tx switching scheme | CMCC |
| R1-2211825 | On multi-carrier UL Tx switching | Apple |
| R1-2211999 | Discussion on Multi-carrier UL Tx switching scheme | NTT DOCOMO, INC. |
| R1-2212061 | On multi-carrier UL Tx switching | Samsung |
| R1-2212133 | Discussion on Rel-18 UL Tx switching | Qualcomm Incorporated |
| R1-2212157 | Multi-carrier UL Tx switching | Ericsson |
| R1-2212251 | On multi-carrier UL Tx switching scheme | MediaTek Inc. |
| R1-2212304 | Discussion on Multi-carrier UL Tx switching scheme | LG Electronics |
| R1-2212528 | Summary#1 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2212757 | Summary#2 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2212879 | Summary#3 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |
| R1-2212894 | Summary#4 of discussion on multi-carrier UL Tx switching scheme | Moderator (NTT DOCOMO, INC.) |

RAN2#119bis-e

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2210298 | Consideration on Rel-18 UL Tx switching capability and configuration | ZTE Corporation, Sanechips |
| R2-2210437 | Current status of Rel-18 UL Tx switching in RAN2 | NTT DOCOMO INC. |
| R2-2210490 | RAN2 impact to support Rel-18 UL Tx switching enhancements | Huawei, HiSilicon |
| R2-2210637 | On RAN2 aspects for UL TX switching Rel-18 | Ericsson |

RAN2#120

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2211221 | Discussion on R18 UL Tx Switching | OPPO |
| R2-2211454 | Discussion on Rel-18 UL Tx switching capability and configuration | CATT |
| R2-2211742 | Discussion on UL Tx switching | Apple |
| R2-2211755 | RAN2 impact to support Rel-18 UL Tx switching enhancements | Huawei, HiSilicon |
| R2-2212391 | On RAN2 aspects for UL TX switching Rel-18 | Ericsson |
| R2-2212500 | Discussion on Rel-18 UL Tx switching based on LS from RAN1 and RAN4 | NTT DOCOMO INC. |

RAN4#104bis-e

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2215613 | On R18 CA enhancement - RRM | Apple |
| R4-2215495 | Switching for UL Tx switching across 3 or 4 bands | CMCC |
| R4-2215569 | UL Outage time considerations for 2 TAG | Qualcomm Incorporated |
| R4-2215797 | UL Tx switching across 3/4 bands and Tx switching with 2 TAGs | China Telecom |
| R4-2215881 | Discussion on Tx switching across 3 or 4 bands | ZTE Wistron Telecom AB |
| R4-2215953 | Uplink TX switching with dual-TAG across two bands | Ericsson, Sony |
| R4-2215954 | Introduction of ON/OFF time mask for TX switching across two bands with dual-TAG | Ericsson |
| R4-2216042 | Discussion on RF requirements for multi-carrier enhancement | Xiaomi |
| R4-2216119 | Discussion on UE RF requirements for TxSwitching up to 3 or 4 bands | vivo |
| R4-2216160 | Continue discussion on UL Tx switching across 3 or 4 bands in Rel-18 | MediaTek Inc. |
| R4-2216433 | R18 Discussion on Tx switching | OPPO |
| R4-2216653 | Discussion on Multi-carrier enhancements with single TAG | Huawei, HiSilicon |
| R4-2216654 | Discussion on Multi-carrier enhancements with multiple TAG | Huawei, HiSilicon |
| R4-2216655 | Draft CR for 38.101-1 to clarify the time mask for switching with multiple TAGs | Huawei, HiSilicon |
| R4-2216714 | Further Discussion on Switching Time and other RF Aspects for UL Tx Switching Across 3 or 4 Bands | Samsung |
| R4-2217740 | WF on UL Tx switching across 3/4 bands with single TAG | China Telecom |
| R4-2217741 | LS on Rel-18 UL Tx switching | China Telecom |
| R4-2217742 | WF on UL Tx switching with multiple TAGs | Ericsson |
| R4-2215496 | RRM requirements for multi-carrier enhancements | CMCC |
| R4-2215788 | RRM core requirements for multi-carrier enhancements | Nokia, Nokia Shanghai Bell |
| R4-2215798 | DL interruption and UL outage time for Rel-18 Tx switching | China Telecom |
| R4-2215872 | Discussion on RRM impacts for mulit-carrier enhancement | vivo |
| R4-2216310 | Discussion on RRM core requirements for multi-carrier enhancements | Huawei, HiSilicon |
| R4-2216424 | RRM impact | Ericsson |
| R4-2216715 | Discussion on RRM requirements for UL Tx Switching Across 3 or 4 Bands | Samsung |
| R4-2217258 | WF on multi-carrier enhancements | Huawei, HiSilicon |
| R4-2216932 | Email discussion summary for [104-bis-e][221] NR_MC_enh_RRM | Moderator (Huawei) |
| R4-2216975 | Email discussion summary for [104-bis-e][139] NR_MC_enh_UERF | Moderator (China Telecom) |
| R4-2217018 | Email discussion summary for [104-bis-e][139] NR_MC_enh_UERF | Moderator (China Telecom) |
| R4-2217154 | Email discussion summary for [104-bis-e][221] NR_MC_enh_RRM | Moderator (Huawei) |
| R4-2217791 | Email discussion summary for [104-bis-e][139] NR_MC_enh_UERF | Moderator (China Telecom) |

RAN4#105

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2218403 | UL Tx switching across 3/4 bands with single TAG | China Telecom |
| R4-2218627 | UL Tx switching for single TAG | CMCC |
| R4-2218855 | Further discussion on UE RF requirements for TxSwitching up to 3 or 4 bands | vivo |
| R4-2218928 | Continue discussion on R18 Tx switching – Single TAG | MediaTek Inc. |
| R4-2219209 | Further discussion on Tx switching across 3 or 4 bands for single TAG | ZTE Corporation |
| R4-2219265 | Discussion on UL Tx switching with single TAG | Xiaomi |
| R4-2219408 | Discussion on Multi-carrier enhancements with single TAG | Huawei, HiSilicon |
| R4-2219590 | R18 Discussion on Tx switching with single TAG | OPPO |
| R4-2219876 | UE behaviour in case non-equal switching time | Qualcomm Incorporated |
| R4-2220018 | Views on Tx switching across 3 and 4 bands | NTT DOCOMO INC. |
| R4-2220546 | WF on UE Tx switching requirement for single TAG | China Telecom |
| R4-2220547 | WF on UE Tx swithing requirement for multiple TAG | Ericsson |
| R4-2220548 | LS on Rel-18 Tx switching | China Telecom |
| R4-2218404 | UL Tx switching with two TAGs | China Telecom |
| R4-2218628 | UL Tx switching with multiple TAGs | CMCC |
| R4-2218826 | Time masks and switching time location for uplink TX switching with dual-TAG | Ericsson, Sony |
| R4-2218827 | Introduction of ON/OFF time mask for TX switching across two bands with dual-TAG | Ericsson |
| R4-2218929 | Continue discussion on R18 Tx switching for multi-TAG | MediaTek Inc. |
| R4-2219210 | Further discussion on Tx switching across 3 or 4 bands for multiple TAG | ZTE Corporation |
| R4-2219266 | Discussion on UL Tx switching with multiple TAG | Xiaomi |
| R4-2219409 | Discussion on Multi-carrier enhancements with multiple TAG | Huawei, HiSilicon |
| R4-2219410 | Draft CR for 38.101-1 to clarify the time mask for switching with multiple TAGs | Huawei, HiSilicon |
| R4-2219589 | R18 Discussion on Tx switching with multi TAG | OPPO |
| R4-2219877 | UE requirements during overlaps due to NZ-MTTD | Qualcomm Incorporated |
| R4-2218601 | Discussion on RRM requirements for multi-carrier enhancements | ZTE Corporation |
| R4-2220417 | WF on RRM requirements for NR Multi-carrier enhancements | Huawei |
| R4-2218152 | On DL interruption for Tx switching across 3/4 bands | Apple |
| R4-2218405 | DL interruption for Tx switching across 3/4 bands | China Telecom |
| R4-2218629 | DL interruption for Tx switching | CMCC |
| R4-2218781 | Further discussion on DL interruption for Tx switching across 3 to 4 bands | vivo |
| R4-2218926 | DL interruption for Tx switching across 3/4 bands | Nokia, Nokia Shanghai Bell |
| R4-2219331 | On timing requirements DL | Ericsson |
| R4-2219401 | Discussion on DL interruption for Tx switching across 3/4 bands | Huawei, HiSilicon |
| R4-2218153 | On UL outage time for TX switching with 2 TAGs | Apple |
| R4-2218406 | UL outage time for TX switching with 2 TAGs | China Telecom |
| R4-2218630 | UL outage time for Tx switching with 2TAGs | CMCC |
| R4-2218782 | On UL outage time for TX switching with 2 TAGs | vivo |
| R4-2218927 | UL outage time for TX switching with 2 TAGs | Nokia, Nokia Shanghai Bell |
| R4-2219332 | On timing requirements DL | Ericsson |
| R4-2219402 | UL outage time for TX switching with 2 TAGs | Huawei, HiSilicon |
| R4-2220072 | Topic summary for [105][226] NR_MC_enh_RRM | Moderator (Huawei) |
| R4-2220118 | Topic summary for [105][138] NR_MC_enh_UERF | Moderator (China Telecom) |

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template