3GPP TSG RAN meeting #98-e									RP-223238

Electronic Meeting, December 12-16, 2022

## Status Report to TSG

Agenda item:			9.3.4.7

| WI / SI Name | Further enhancements on NR and MR-DC measurement gaps and measurements without gaps |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MG_enh2 |  |  |  |  |
| Unique ID | 950182 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222337 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall: 35% RAN4: 35% | Performance Part: Overall: 0% RAN4: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Ato (Tsang-Wei) YU, Rui HUANG |
|  | Company | MediaTek Inc., Intel Corporation |
|  | Email | ato.yu@mediatek.com, rui.huang@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #104bis-e (Oct, 2022, Electronic)

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

  - The following agreements were captured in the WF [14]

  - Scope and general issues

    - Deprioritize MR-DC scenarios in objective #1 of this WI

    - Type-1 MG: Gap(s) configured via GapConfig without suffix

    - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17

  - Case 1 (Pre-configured MG and concurrent MG)

    - Take the following as the baseline

      - For UE autonomous mechanism, only the measurements associated to the concerned pre-MG are used for the rule checking

      - For Network-controlled mechanism, only the bits corresponding to the concerned pre-MG are used for determining the status

    - RAN4 reuses the explicit association from Rel-17 MGE for concurrent gap to Rel-18.

      - FFS any further enhancement

      - FFS how to interpret the gap association to an intra-frequency measurement that does not need MG/NCSG

    - The priority of a Pre-MG which concurrent with other gaps should be up to network assignment. For the priority of a Pre-MG, once it is configured, it should be same until it is reconfigured by RRC signaling

      - FFS whether to introduce priority based on associated MO(s)

  - Case 2 (NCSG and concurrent MG)

    - For case 2, how to determine the priority is reused from Rel-17

- RRM core requirements for measurements without gaps

  - Measurement without gaps for UEs reporting NeedForGapsInfoNR

    - The following agreements were captured in the WF [15]

    - Reuse requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) for the reporting delay requirements for intra-frequency measurement without gap and no interruption allowed

  - Inter-RAT measurement without gap

    - The following agreements were captured in the WF [16]

    - Two scenarios for inter-RAT NR measurements without gap in Rel18 includes

      - Confirmed to support Case a-1: UE performing the measurements without gap in NR carriers as there is vacant RF chains for UE measurements

        - On top of “interRAT-NeedForGapsNR-r16” UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1)

        - FFS on top of NCSG to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1)

      - FFS on supporting Case a-2: NR reference signal to be measured are fully contained within UE’s LTE channel bandwidth

    - Two scenarios for inter-RAT LTE measurements without gap in Rel18 includes

      - FFS on supporting Case b-1: UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements

      - Confirmed to support Case b-2: LTE CRS are fully contained within UE’s active BWP

        - Reuse the existing capability ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN for the inter-RAT LTE measurement

        - FFS on: UE indicates via NeedForGap.

    - For inter-RAT measurement without MG, including both inter-RAT NR measurement and inter-RAT LTE measurement, the mixed numerology needs to be supported.

      - FFS on whether the additional UE capability is needed

RAN4 #105 (Nov, 2022, Toulouse, France)

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

  - The following agreements were captured in the WF [88]

  - Scope and general issues

    - Start with the requirement definition based on Type-2 MG. FFS whether and how to include Type-1 MG

  - Case 1 (Pre-configured MG and concurrent MG)

    - On whether to consider Pre-MG + Pre-MG in an FR: Narrow down options to Option 1 and 1a.

      - Option 1: Yes

      - Option 1a: Yes, with UE capability

    - Do not increase the max number of configured gaps for Case 1 (Pre-configured MG and multiple concurrent MGs) comparing to Rel-17 concurrent MG design.

    - Support of gap combinations including pre-configured MGs (Case 1) that cause dynamic collisions will be subject to new UE capability(ies).

    - Gap sharing rules shall not be considered when the two gaps are with different priority.

      - FFS whether RAN4 to consider the gap sharing rule when two gaps configured with equal priority.

    - The measurement requirements with concurrent MGs defined in Rel-17 can be reused except that only activated gaps are considered when defining CSSF, Kp and Kgap and P factor

      - A scaling factor Kgap needs to be account for collisions with other measurement gaps

      - CSSF for each component gap is defined separately

      - FFS whether to consider additional applicability to the requirements.

  - Case 2 (NCSG and concurrent MG)

    - On whether to consider NCSG + NCSG in an FR: Narrow down options to Option 1 and 3.

      - Option 1: Yes. FFS whether the same RF chain is assumed for the two NCSG patterns.

      - Option 3: Yes, with UE capability

    - Do not increase the max number of gaps for Case 2 (NCSG and multiple concurrent MGs) comparing to Rel-17 concurrent MG design.

    - Reuse Rel-17 proximity conditions for Case 2 (NCSG and Concurrent MG).

    - Gap sharing rules shall not be considered when the two gaps are with different priority.

      - FFS whether RAN4 to consider the gap sharing rule when two gaps configured with equal priority.

    - For NR SSB-based measurements performed within NCSG, the principle of defining CSSF, Kp and Kgap in Rel-17 concurrent MGs can be applied with the following updates

      - A scaling factor Kgap needs to be added to account for collisions with other measurement gaps

      - CSSF for each component gap is defined separately

    - No need to restrict the network configuration on the type of component gaps within the concurrent MGs.

- RRM core requirements for measurements without gaps

  - The following agreements were captured in the WF [89]

  - Measurement without gaps for UEs reporting NeedForGapsInfoNR

    - Introduce additional Rel-18 UE signalling to differentiate UE supporting no gap with interruption (Case 2)

      - Signalling details are FFS..

    - For requirement for inter-freq measurement without gap when no interruption, take requirements in Section 9.3.9 of TS38.133 (inter-freq w/o gap) as a starting point

    - The requirement (e.g. the new UE signalling to indicate whether the interruption allowed in Rel18]) shall apply from R18 UE

  - Inter-RAT measurement without gap

    - De-prioritize Case a-2 inter-RAT NR target scenario

    - Case b-1(UE performing the measurements without gap in LTE carriers as there is vacant RF chains for UE measurements) shall be considered

    - Deprioritize the inter-RAT measurements without gap (both case a-2 and b-2) in MR-DC

    - Only on top of ‘interRAT-NeedForGapsNR-r16’ capability to support Case a-1. The exact value to be reported can be FFS. E.g. after RAN4 concludes NeedForGapsNR requirements design

#### 2.4.2	Remaining Open issues

- RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG

  - Scope and general issues

    - Whether to consider Type 1 MG

  - Case 1 (Pre-configured MG and concurrent MG)

    - Detail in UE capability signaling

    - Supporting of simultaneous or non-simultaneous multiple Pre-MGs activation/deactivation

    - Detail measurement gap combinations to be supported

    - FFS the UE behavior when the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion

    - Detail rule for dynamic Pre-MG collision

    - Any change to activation/deactivation delay

    - FFS explicit and implicit gap association and corresponding clarifications to MO which does not need gap

    - Measurement delay requirement

  - Case 2 (NCSG and concurrent MG)

    - Whether to support parallel measurements upon gap collision and corresponding UE behavior

    - Detail measurement gap combinations to be supported

    - Potential changes for de-activated Scell

    - Measurement delay requirement

    - Gap interruption

- RRM core requirements for measurements without gaps

  - Measurement without gaps for UEs reporting NeedForGapsInfoNR

    - Interruption length, location and ratio

    - Measurement delay requirements

    - Whether to introduce a mapping between NeedForGap and NCSG capabilities when UE supports both

    - FFS UE behaviour mismatch between UE and NW

    - Detail in scheduling availability (UE capability, network indication, default SMTC pattern)

  - Inter-RAT measurement without gap

    - On top of which UE capability to support the inter-RAT LTE measurement requirements when UE has vacant RF chain available (Case b-1)

    - On top of which UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2)

    - Whether to introduce additional capability to support inter-RAT measurement without gap with mixed numerology

    - Measurement requirements (framework, general principle, scheduling restriction, searcher limitation, CSSF, effective measurement window)

    - Interruption requirements

    - Whether to introduce the inter-RAT LTE measurements without gap case b-2 as release independent from Rel-17

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

RAN4 #104bis-e (Oct, 2022, Electronic)

- R4-2215367	Discussion on RRM requirements for pre-configured MGs, multiple concurrent MGs and NCSG	Intel Corporation

- R4-2215426	Discussion on RRM requirements for combination of pre-MG, concurrent MGs and NCSG	CATT

- R4-2215457	RRM requirement for the combination of concurrent gaps, pre-MG and NCSG	Xiaomi

- R4-2215610	On R18 gap enhancement - joint configuration of Pre-MG, NCSG and concurrent gaps	Apple

- R4-2215714	Discussion on combination of pre-configured MGs, multiple concurrent MGs and NCSG	CMCC

- R4-2215821	Discussion on joint requirements for PreMG, concurrent MGs and NCSG	OPPO

- R4-2215966	Considerations on pre-configured MGs, multiple concurrent MGs and NCSG	vivo

- R4-2216336	Discussion on joint working of eMG features	Huawei, HiSilicon

- R4-2216460	Discussion on PreMG, ConMG, NCSG	Ericsson

- R4-2216482	Discussion on RRM requirements for joint considerations between pre-MG, concurrent MG and NCSG for NR and MR-DC	ZTE Corporation

- R4-2216582	Discussion on requirements for concurrent measurement gaps, pre-configured gaps and NCSG	Nokia, Nokia Shanghai Bell

- R4-2216723	On joint requirements for Rel-17 measurement gap enhancements	Qualcomm Incorporated

- R4-2216737	RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG	MediaTek inc.

- R4-2217251	WF on further enhancements on measurement gaps and measurements without gaps	MediaTek inc.

- R4-2217252	WF on Measurement without gaps for UEs reporting NeedForGapsInfoNR	Intel

- R4-2217253	WF on inter-RAT measurement without gap	Intel

- R4-2216746	Discussion on RRM requirements for measurement without gap	Qualcomm Incorporated

- R4-2215368	Discussion on measurements without gaps when UE reporting NFG	Intel Corporation

- R4-2215427	Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR	CATT

- R4-2215467	Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR	Xiaomi

- R4-2215611	On R18 gap enhancement - NeedForGap	Apple

- R4-2215715	Discussion on measurements without gaps for UEs reporting NeedForGapsInfoNR	CMCC

- R4-2215822	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	OPPO

- R4-2215967	Considerations on measurement without gaps for UEs reporting NeedForGapsInfoNR	vivo

- R4-2216337	Discussion on requirements for NeedForGaps	Huawei, HiSilicon

- R4-2216461	Discussion on NeedForGaps measurement	Ericsson

- R4-2216484	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	ZTE Corporation

- R4-2216583	Discussion on RRM requirements without gaps for MG_enh2	Nokia, Nokia Shanghai Bell

- R4-2216738	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	MediaTek inc.

- R4-2215369	Discussion on inter-RAT measurement without gaps	Intel Corporation

- R4-2215428	Discussion on RRM requirements for Inter-RAT measurement without gap	CATT

- R4-2215468	Discussion on RRM requirements for inter-RAT measurement without gap	Xiaomi

- R4-2215612	On R18 gap enhancement - inter-RAT measurement with gap	Apple

- R4-2215716	Discussion on inter-RAT measurements without gaps	CMCC

- R4-2215823	Discussion on RRM requirements for interRAT measurements without gaps	OPPO

- R4-2215968	Considerations on inter-RAT measurement without gap	vivo

- R4-2216338	Discussion on inter-RAT MG-less measurement in feMG	Huawei, HiSilicon

- R4-2216462	Discussion on Inter-RAT measurement without gap	Ericsson

- R4-2216483	Discussion on RRM requirements for inter-RAT measurement without gap	ZTE Corporation

- R4-2216739	Discussion on inter-rat measurements	MediaTek inc.

- R4-2216927	Email discussion summary for 1[04-bis-e][216] NR_MG_enh2_part1	Moderator (MediaTek)

- R4-2216928	Email discussion summary for [104-bis-e][217] NR_MG_enh2_part2	Moderator (Intel)

- R4-2217149	Email discussion summary for 1[04-bis-e][216] NR_MG_enh2_part1	Moderator (MediaTek)

- R4-2217150	Email discussion summary for [104-bis-e][217] NR_MG_enh2_part2	Moderator (Intel)

RAN4 #105 (Nov, 2022, Toulouse, France)

- R4-2218515	On joint requirements for Rel-17 measurement gap enhancements	Qualcomm Incorporated

- R4-2218147	On R18 gap enhancement scope	Apple

- R4-2218344	Discussion on using scenarios for pre-configured MGs, multiple concurrent MGs and NCSG	Intel Corporation

- R4-2218799	On scope and general issues for pre-configured MGs, multiple concurrent MGs and NCSG.	vivo

- R4-2219314	Discussion on the general issues of PreMG, ConMGs, NCSG	Ericsson

- R4-2219744	Discussion on general issues for preconfigured and NCSG	Nokia, Nokia Shanghai Bell

- R4-2219930	Scope and general issues	MediaTek inc.

- R4-2218148	On R18 gap enhancement case 1 requirements	Apple

- R4-2218345	Discussion on RRM requirements for combinations of pre-configured MGs and multiple concurrent MGs	Intel Corporation

- R4-2218392	Discussion on Pre-configured MG and concurrent MG	CMCC

- R4-2218435	Discussion on case 1 requirements for combination of pre-MG and concurrent MGs	CATT

- R4-2218567	RRM requirements for the combination of Pre-MG and concurrent MG	Xiaomi

- R4-2218800	On case 1 requirement for pre-configured MGs, multiple concurrent MGs and NCSG	vivo

- R4-2218992	Discussion on case 1 requirements preMG and conMG	OPPO

- R4-2219315	Discussion on PreMG and ConMGs	Ericsson

- R4-2219427	Discussion on Case 1 RRM requirements	ZTE Corporation

- R4-2219548	Discussion on joint working of pre-MG and con-MG	Huawei, HiSilicon

- R4-2219913	Discussion on Case 1 requirements	Nokia, Nokia Shanghai Bell

- R4-2219931	Discussion on case 1 requirements (Pre-configured MG and concurrent MG)	MediaTek inc.

- R4-2218149	On R18 gap enhancement case 2 requirements	Apple

- R4-2218346	Discussion on RRM requirements for combinations of NCSG and multiple concurrent MGs	Intel Corporation

- R4-2218391	Discussion on NCSG and concurrent MG	CMCC

- R4-2218436	Discussion on case 2 requirements for combination of NCSG and concurrent MGs	CATT

- R4-2218568	RRM requirements for the combination of NCSG and concurrent MG	Xiaomi

- R4-2218801	On case 2 requirements for pre-configured MGs, multiple concurrent MGs and NCSG	vivo

- R4-2218993	Discussion on case 2 requirements NCSG and conMG	OPPO

- R4-2219316	Discussion on NCSG and ConMGs	Ericsson

- R4-2219428	Discussion on Case 2 RRM requirements	ZTE Corporation

- R4-2219549	Discussion on joint working of NCSG and con-MG	Huawei, HiSilicon

- R4-2219914	Discussion on Case 2 requirements	Nokia, Nokia Shanghai Bell

- R4-2219932	Discussion on case 2 requirements (NCSG and concurrent MG)	MediaTek inc.

- R4-2218150	On measurement without gaps for UEs reporting NeedForGapsInfoNR	Apple

- R4-2218347	Discussion on measurements without gaps when UE reporting NFG	Intel Corporation

- R4-2218401	Discussion on measurements without gaps for UEs reporting NeedForGapsInfoNR	CMCC

- R4-2218437	Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR	CATT

- R4-2218543	Remaining issues on RRM requirements about Needforgap	Qualcomm Incorporated

- R4-2218578	Discussion on RRM requirements for measurement without gaps for UEs reporting NeedForGapsInfoNR	Xiaomi

- R4-2218802	On Measurement without gaps for UEs reporting NeedForGapsInfoNR	vivo

- R4-2218994	On Measurement without gaps for UEs reporting NeedForGapsInfoNR	OPPO

- R4-2219317	Discussion on NeedForGaps measurement	Ericsson

- R4-2219550	Discussion on requirements for NeedForGaps	Huawei, HiSilicon

- R4-2219745	Discussion on measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2219933	Discussion on measurement without gaps for UEs reporting NeedForGapsInfoNR	MediaTek inc.

- R4-2220359	WF on NR_MG_enh2 Part 1	MediaTek

- R4-2220360	WF on NR_MG_enh2 Part 2	Intel Corporation

- R4-2218151	On R18 inter-RAT measurement without gap	Apple

- R4-2218348	Discussion on inter-RAT measurement without gaps	Intel Corporation

- R4-2218398	Discussion on inter-RAT measurements without gaps	CMCC

- R4-2218438	Discussion on RRM requirements for Inter-RAT measurement without gap	CATT

- R4-2218544	Remaining issues on RRM requirements abou Inter-RAT measurements without gap	Qualcomm Incorporated

- R4-2218579	Discussion on RRM requirements for inter-RAT measurement without gap	Xiaomi

- R4-2218803	"On 	Inter-RAT measurement without gap"	vivo

- R4-2218995	Discussion on RRM requirements for Inter-RAT measurement without gap	OPPO

- R4-2219318	Discussion on Inter-RAT measurement without gap	Ericsson

- R4-2219551	Discussion on inter-RAT MG-less measurement	Huawei, HiSilicon

- R4-2219746	Discussion on inter-RAT measurements without gaps	Nokia, Nokia Shanghai Bell

- R4-2219934	Discussion on inter-RAT measurements	MediaTek inc.

- R4-2220066	Topic summary for [105][220] NR_MG_enh2_part1	Moderator (MediaTek)

- R4-2220067	Topic summary for [105][221] NR_MG_enh2_part2	Moderator (Intel)

- R4-2220329	Ad-hoc meeting minutes: Rel-18 Further enhancements on NR and MR-DC measurement gaps and measurements without gaps WI	MediaTek

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template