3GPP TSG-RAN4 Meeting #105 	R4-2220858

# Toulouse, France, 14th November 2022 - 18th November 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 1268 | rev | 1 | Current version: | 15.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TDD RMC for Intra-band EN-DC - 38.101-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are currently no default UL TDD RMC definitions compatible for intra-band EN-DC in TS 38.101-1 or TS 38.101-3, this has an impact on all the test cases where the TDD configuration is necessary along with the intra-band EN-DC setting. The most widely used implementation to circumvent that issue is to use a time offset between the NR configuration pattern relative to the E-UTRA UL-DL configuration. From core specification perspective, it is necessary to proceed to two changes to fix that issue: - In TS 38.101-1: Modification of Table A.3.3.1-1 of 38.101-1, adding “NOTE 4: A -2ms or +3ms time offset to the NR configuration pattern relative to the E-UTRA UL-DL configuration must be apply in the TDD intra-band EN-DC.”. - In TS 38.101-3: Modification of point e) of section 4.2 of 38.101-3, adding that “same configurations” between E-UTRA and NR have to be understood when compared time-wise including with the use of a time offset between the two RATs configurations to have such matching. The joint CR corresponding to the changes in TS 38-101-3 is R4-2219783. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modification of Table A.3.3.1-1 of 38.101-1, adding “NOTE 4: A -2ms or +3ms time offset to the NR configuration pattern relative to the E-UTRA UL-DL configuration must be apply in the TDD intra-band EN-DC.”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This may lead to different implementations of the test cases for intra-band EN-DC case due to different interpretation of the TS 38.101-1 [1] and other related specifications. Core specification is in disagreement with test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | An associated discussion paper (R4-2212534) was submitted at RAN4 #104-e. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

<<Start of change>>

## A.3.3	DL reference measurement channels for TDD

### A.3.3.1	General

Table A.3.3.1-1 Additional reference channels parameters for TDD

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | SCS 15 kHz (µ=0) | SCS 30 kHz (µ=1) | SCS 60 kHz (µ=2) |
| TDD Slot Configuration pattern (Note 1) |  | DDDSU | 7DS2U | 14DS1S24U |
| Special Slot Configuration (Note 2) |  | 10D+2G+2U | 6D+4G+4U | S1=12D+2G, S2=6G+8U |
| referenceSubcarrierSpacing |  | 15 kHz | 30 kHz | 60 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 5 ms | 5 ms | 5 ms |
|  | nrofDownlinkSlots | 3 | 7 | 14 |
|  | nrofDownlinkSymbols | 10 | 6 | 12 |
|  | nrofUplinkSlot | 1 | 2 | 4 |
|  | nrofUplinkSymbols | 2 | 4 | 8 |
| Number of HARQ Processes |  | 8 | 8 | 16 |
| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} | K1 = 8 if mod(i,10) = 0 K1 = 7 if mod(i,10) = 1 K1 = 6 if mod(i,10) = 2 K1 = 5 if mod(i,10) = 3 K1 = 4 if mod(i,10) = 4 K1 = 3 if mod(i,10) = 5 K1 = 2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} | K1 = 13 if mod(i,20) = 2 K1 = 12 if mod(i,20) = 3 K1 = 11 if mod(i,20) = 4 K1 = 10 if mod(i,20) = 5 K1 = 9 if mod(i,20) = 6 K1 = 8 if mod(i,20) = 7 K1 = 7 if mod(i,20) = 8 K1 = 6 if mod(i,20) = 9 K1 = 6 if mod(i,20) = 10 K1 = 6 if mod(i,20) = 11 K1 = 6 if mod(i,20) = 12 K1 = 6 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. NOTE 3: i is the slot index per frame. NOTE 4: A -2ms or +3ms time offset to the NR configuration pattern relative to the E-UTRA UL-DL configuration must be apply in the TDD intra-band EN-DC. |  |  |  |  |

<<End of change>>