3GPP TSG-RAN4 Meeting # 105	R4-2219163

Toulouse, France, November 14 – November 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0432 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.141-2: Additional BS conformance to other standards |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2022-11-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the final draft of EN 301 908-24, additional BS conformance to other standards was defined. The conformance with the requirements in ETSI EN 301 908-24 can also equally be demonstrated through MSR standards such as ETSI EN 301 908-18 for BS type 1-C, ETSI EN 301 908-23 for BS type 1-H and 1-O. But in current 38.141-1 and 38.141-2 such equal demonstration is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new sub-clause to include the additional BS conformance. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The additional equal conformance is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.141-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.104: "NR Base Station (BS) radio transmission and reception"

[3]	3GPP TS 38.141-1: "NR, Base Station (BS) conformance testing, Part 1: Conducted conformance testing"

[4]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[6]	Void

[7]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[8]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Clause 4: Stationary use at non-weather protected locations"

[9]	IEC 60 721: "Classification of environmental conditions"

[10]	IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[11]	IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[12]	IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[13]	Recommendation ITU-R M.328: "Spectra and bandwidth of emissions"

[14]	FCC publication number 662911: "Emissions Testing of Transmitters with Multiple Outputs in the Same Band".

[15]	ECC/DEC/(17)06: "The harmonised use of the frequency bands 1427-1452 MHz and 1492-1518 MHz for Mobile/Fixed Communications Networks Supplemental Downlink (MFCN SDL)"

[16]	Void

[17]	Void

[18]	3GPP TS 36.104: "E-UTRA; Base Station (BS) radio transmission and reception"

[19]	3GPP TS 38.212: "NR; Multiplexing and channel coding"

[20]	3GPP TS 38.211: "NR; Physical channels and modulation"

[21]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[22]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[23]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

[24]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[25]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[26]	ERC Recommendation 74-01: "Unwanted emissions in the spurious domain"

[27]	3GPP TR 25.942: "RF system scenarios".

[28]	ITU-T Recommendation O.150, "Equipment for the measurement of digital and analogue/digital parameters"

[29]	3GPP TR 37.941: "Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements"

[30]	3GPP TS 37.145-2: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: radiated conformance testing".

<End of Change 1>

<Start of Change 2>

### 4.8.3	Applicability of test configurations for multi-band RIB

For a multi-band RIB, the test configuration in table 4.8.3-1 shall be used for testing.

Unless otherwise stated, single carrier configuration (SC) tests shall be performed using signal with narrowest supported BS channel bandwidth with the smallest supported subcarrier spacing declared per operating band (D.7).

NOTE:	The applicability of test configurations in table 4.8.3-1 are not applicable to BS type 2-O.

Table 4.8.3-1: Test configuration for a multi-band RIB

| BS test case | Test configuration |
| --- | --- |
| Radiated transmit power | NRTC1/3 (Note 1), NRTC4 |
| OTA base station maximum output power | NRTC1/3 (Note 1), NRTC4 |
| OTA RE power control dynamic range | Tested with Error Vector Magnitude |
| OTA total power dynamic range | SC |
| OTA transmit ON/OFF power (only applied for NR TDD BS) | NRTC4 |
| OTA frequency error | Tested with Error Vector Magnitude |
| OTA Error Vector Magnitude | NRTC1/3 (Note 1), NRTC4 |
| OTA time alignment error | NRTC1/3 (Note 1), NRTC5 (Note 2) |
| OTA occupied bandwidth | SC, NRTC2 (Note 3) |
| OTA ACLR | NRTC1/3 (Note 1), NRTC5 (Note 4) |
| OTA CACLR | NRTC3 (Note 1), NRTC5 (Note 4) |
| OTA operating band unwanted emissions | NRTC1/3 (Note 1), NRTC5, SC (Note 5) |
| OTA transmitter spurious emissions | NRTC1/3 (Note 1), NRTC5 |
| OTA transmitter intermodulation | NRTC1/3 (Note 1) |
| OTA sensitivity | SC |
| OTA reference sensitivity level | SC |
| OTA dynamic range | SC |
| OTA adjacent channel selectivity | NRTC5 |
| In-band blocking | NRTC5 |
| OTA out-of-band blocking | NRTC5 |
| OTA receiver spurious emissions | NRTC1/3 (Note 1), NRTC5 |
| OTA receiver intermodulation | NRTC5 |
| OTA in-channel selectivity | SC |
| Note 1: NRTC1 and/or NRTC3 shall be applied in each supported operating band. Note 2: NRTC5 is only applicable when inter-band CA is supported. Note 3: NRTC2 is only applicable when contiguous CA is supported. Note 4: NRTC5 may be applied for Inter RF Bandwidth gap only. Note 5: OBUE SC shall be tested using the widest supported channel bandwidth and the highest supported sub-carrier spacing. |  |

4.8.4	Additional conformance

For a BS type 1-H additionally conforming to TS 37.145-2 [30], conformance to some of the RF requirements in the present document can be demonstrated through the corresponding requirements in TS 37.145-2 [30] as listed in Table 4.8.4-1.

Table 4.8.4-1: Alternative RF test requirements for a BS additionally conforming to TS 37.145-2 [30] for BS type 1-H

| RF requirement | Clause in the present document | Alternative clause in TS 37.145-2 [30] |
| --- | --- | --- |
| Radiated transmit power | 6.2.5 | 6.2.5 |
| OTA sensitivity | 7.2.5.2 | 7.2.5.5 |

For a BS type 1-O additionally conforming to TS 37.145-2 [30], conformance to some of the RF requirements in the present document can be demonstrated through the corresponding requirements in TS 37.145-2 [30] as listed in Table 4.8.4-2.

Table 4.8.4-2: Alternative RF test requirements for a BS additionally conforming to TS 37.145-2 [30] for BS type 1-O

| RF requirement | Clause in the present document | Alternative clause in TS 37.145-2 [30] |
| --- | --- | --- |
| Radiated transmit power | 6.2.5 | 6.2.5 |
| OTA base station output power | 6.3.5.1 | 6.3.2.5 |
| OTA transmit ON/OFF power | 6.5 | 6.5 |
| OTA Unwanted emissions |  |  |
| OTA transmitter spurious emission | 6.7.5.2.5, 6.7.5.3.5, 6.7.5.4.5, 6.7.5.5.5 | 6.7.6.2.5, 6.7.6.3.5, 6.7.6.4.5, 6.7.6.5.5 |
| OTA Operating band unwanted emissions | 6.7.4.5.1 | 6.7.5.5 (except for 6.7.5.5.5) |
| OTA Transmitter intermodulation | 6.8.5 | 6.8.5.1 |
| OTA sensitivity | 7.2.5.2 | 7.2.5.5 |
| OTA reference sensitivity level | 7.3.5.2 | 7.3.5.4 |
| OTA Narrowband blocking | 7.5.2.5.2 | 7.5.5.1.2 |
| OTA Blocking | 7.5.2.5.2 | 7.5.5.1.1 |
| OTA Out-of-band blocking | 7.6.5.1.1 | 7.6.2.5.1 |
| Co-location with other base stations | 7.6.5.1.2 | 7.6.3.5.1 |
| OTA Receiver spurious emissions | 7.7.5.1 | 7.7.5 |
| OTA Intermodulation | 7.8.5.1 | 7.8.5.1.1 |
| OTA Narrowband intermodulation | 7.8.5.1 | 7.8.5.1.2 |

<End of Change 2>