3GPP TSG-RAN WG4 Meeting # 105	R4-2219522

# Toulouse, France, November 14 – November 18, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0325 | rev |  | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on parameters of V2X HARQ buffer test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Perf |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The variable of time gap between initial transmission and retransmission in Table 11.1.7.1.1-1 is wrong and it is still with brackets |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the variable from “n” to “k” and remove the brackets |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The parameter will be confusing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

11.1.7	HARQ buffer soft combining test

11.1.7.1	2Rx requirement

11.1.7.1.1	Minimum requirement

The purpose of this test is to verify the maximum number of HARQ processes per TTI supported by the V2X UE.

The minimum requirement is specified in Table 11.1.7.1.1-2 with the test parameters specified in Table 11.1.7.1.1-1.

Table 11.1.7.1.1-1: Test Parameters

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| Active Sidelink UE(s) |  |  | Sidelink UE i, 0 ≤ i < n (Note 1,2) |
| Sidelink UE i, 0 ≤ i < n | Sidelink Transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern |  | {2} |
|  | Time gap between initial transmission and retransmission | Slots | k (Note 3) |
|  | Timing offset (Note 4) | s | 0 |
|  | Frequency offset (Note 5) | Hz | 0 |
|  | Synchronization source |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
|  | Redundancy version coding sequence |  | {0,2} |
| PSFCH resource period |  | Slots | 1 |
| Note 1: n is the number of HARQ process UE can support (based on IE harq-RxProcessSidelink) Note 2: When n = 16 or 24, sidelink UEs transmit one by one circularly for every slot; When n=32, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, and the 32nd UE transmits signal in the first slot but in the second subchannel; When n=48, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 17 UEs transmit signal in the same slot as the first 17 UEs but in the second subchannel; When n=64, first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel, the next 31 UEs transmit signal one by one circularly for every slot and in the second subchannel, the last 2 UEs transmit signal in the same slot as the first 2 UEs in the third subchannel Note 3: k = n if n < 32, otherwise k = 31 Note 4: Time offset of transmitted Sidelink UE signal is with respect to GNSS reference timing. Note 5: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |

Table 11.1.7.1.1-2: Minimum performance

| Test num. | Bandwidth (MHz) / Subcarrier spacing(kHz) | PSSCH Reference channel | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | PSSCH BLER (%) | SNR (dB) of PSSCH |
| 1 | 20 / 30 | R.PSSCH.2-1.5 | AWGN | 5 | 10.9 |