3GPP TSG-RAN4 Meeting #104-bis-e	R4-2217162

# Electronic Meeting, Oct 10 - Oct 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2593 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on correction to cell re-selection requirement for satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The scaling factors M1, M2, M3 and M4 has been agreed to relaxed in previous WF R4-2210610. However, the relaxed factors are not been captured in 4.2C.2.2 and 4.2C.2.3 The Table number is not updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the case of SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, for serving cell measurement, upon more than one SMTC, relax M1 to 2.5. For the case of SMTC periodicity (TSMTC) > 20 ms and DRX cycle < 0.64 second, for intra-frequency measurement, upon more than one SMTC, relax M2= 2. For UE configured with enhanced RRM flag, relax M2 to 2, M3 and M4 to 2.5 when SMTC periodicity > 40ms and DRX cycle < 0.64s Fixing editorial mistakes for the name of Table 4.2C.2.3-2, and 4.2C.2.3-2, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The editorial mistakes may lead to incorrect interpretation. The original M1-M4 value in above cases could not be supported by UE under satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2C.2.2 4.2C.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2215500 |  |  |  |  |  |  |  |  |

<Start of change>

#### 4.2C.2.2	Measurement and evaluation of serving cell

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in [clause TBD] for the serving cell at least once every M1*N1 DRX cycle; where:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, upon one SMTC configured at the UE,

-	M1=2.5 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, upon more than one SMTC configured at the UE,

-	otherwise M1=1.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

If the UE has evaluated according to Table 4.2C.2.2-1 in Nserv consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities. Additionally, if the UE is configured with ‘t-service’ [2], the UE should start measurements of the neighbour cells indicated by the serving cell before ‘t-service’ is reached according to the requirements provided in clause 4.2C.2.3 and 4.2C.2.4.

If the UE is not configured with‘t-Service’ [2] in the serving cell and if the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information for [10 s], the UE shall initiate cell selection procedures for the selected PLMN as defined in [clause TBD].

If the UE is configured with ‘t-Service’ in the serving cell then the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 when any of the following conditions is fulfilled:

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within 10 s since time instance T1 provided that ‘t-Service’ > T1 or

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within 10 s since the time instance ‘t-Service’.

-	Where, T1 is the time instance in seconds when the UE has determined that the serving cell does not fulfil the cell selection criterion S.

Table 4.2C.2.2-1: Nserv

| DRX cycle length [s] | Scaling Factor (N1) | Nserv [number of DRX cycles] |
| --- | --- | --- |
|  | FR1 |  |
| 0.32 | 1 | M1*N1*4 |
| 0.64 |  | M1*N1*4 |
| 1.28 |  | N1*2 |
| 2.56 |  | N1*2 |
| Note 1: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |  |

#### 4.2C.2.3	Measurements of intra-frequency NR cells

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the distance between UE and serving cell reference location is smaller than [threshold] if the [threshold] is configured and UE has location information, then the UE may not perform measurement of intra-frequency.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in [clause TBD] within Kmulti_SMTC * Tdetect,NR_Intra when that Treselection= 0 if the UE does not support [capability for enhanced requriements] or if the [NW configuration for enhanced requirements] is not enabled, or within Kmulti_SMTC * Kcarrier * Tdetect,NR_Intra_enh if the UE supports [capability for enhanced requirements] and the [NW configuration for enhanced requirements] is enabled. An intra frequency cell is considered to be detectable according to the conditions defined in [clause TBD] for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Kmulti_SMTC * Tmeasure,NR_Intra (see table 4.2C.2.3-1) if the UE does not support [capability for enhanced requirements] or if the [NW configuration for enhanced requirements] is not enabled, or every Kmulti_SMTC * Tmeasure,NR_Intra_enh (see table 4.2C.2.3-2) if the UE supports [capability for enhanced requiremetns] and the [NW configuration for enhanced requirements] is enabled, for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Kmulti_SMTC * Tmeasure,NR_Intra/2.

The parameter Kmulti_SMTC is the scaling factor for measurements of multiple SMTCs which correspond to different satellites [clause TBD].

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in [clause TBD] within Tevaluate,NR_Intra if the UE does not support [capability for enhanced requriements] or if the [NW configuration for enhanced requirements] is not enabled, or within Tevaluate,NR_Intra_enh if the UE supports [capability for enhanced requriements] and the [NW configuration for enhanced requirements] is enabled, when Treselection = 0 as specified in table 4.2C.2.3-1 or table 4.2C.2.3-2 provided that:

-	when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

-	when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in [clause TBD] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2 if the current serving cell is among them.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a nonzero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in [clause TBD], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
|  | FR1 |  |  |  |
| 0.32 | 1 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms upon 1 SMTC configured at the UE; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |  |  |  |

Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh

| DRX cycle length [s] | Tdetect,NR_Intra_enh [s] (number of DRX cycles) | Tmeasure,NR_Intra_enh [s] (number of DRX cycles) | Tevaluate,NR_Intra_enh [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | [3.2 x M2 (10 x M2)] Note 1 | [0.32 x M3 ([1] x M3)] Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | [6.4 (10)] | [0.64 (1)] | 1.92 (3) |
| 1.28 | [10.24 (8)] | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5 |  |  |  |

If [serving cell service time information] is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area regardless of whether the distance condition based on serving cell reference location is met [or the legacy Srxlev/Squal condition are met], and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of [serving cell service time information] is started to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger.

Ttrigger = max(Tdetect,NR_Intra, Kcarrier* Tdetect,NR_Inter),

where

-	Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell,

-	Tdetect,NR_Intra is HST intra-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2C.2.3-2,

-	Tdetect,NR_Inter is HST inter-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2C.2.4-2.

The requriements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the [UE capability], otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in Table 4.2C.2.3-1 and Table 4.2C.2.3-2 is expected.

Editor’s note: FFS whether to include side condition related to valid target satellite information

Editor’s note: FFS how to differentiate requirements between LEO and GEO systems

<End of change>