3GPP TSG-RAN4 Meeting #105	  R4-2218551

# Toulouse, France, 14th  – 18th Nov 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0776 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for Rel-18 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CHTTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | DC_R18_1BLTE_1BNR_2DL2UL-Core |  |  |  |  |  | Date: |  |  | 2022-11-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Completed DC configurations after RAN4#104-e, RAN4#104-bis-e and RAN4#105 are specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | EN-DC configuration specific requirements are specified. Endorsed big draft CR R4-2212905 in RAN4#104-e with below contents: --------------------------------------------------------------------------------------------- Completed NE-DC configurations from the TR (R4-2212907): Inter-band NE-DC in FR1: DC_3(n)AA with single switched UL only Some configurations are completed via draft CR approach. This CR covers the following endorsed draft CR. Endorsed draft CR in RAN4#104-e: R4-2214905 Draft CR for 38.101-3 to add DC_7A-7A_n28A R4-2214913 Draft CR for TS 38.101-3: Support of n77(3A) in DC_5A_n77, DC_7A_n77 and DC_7A-7A_n77 R4-2215082 draft CR for 38.101-3: Add new configurations Endorsed correction draft CR in RAN4#104-e: The following correction draft CR is assigned to be merged in this CR: R4-2213611 Draft CR for TS 38.101-3 on updates to delta RIB for DC configurations of 1 band LTE and 1 band NR band R4-2213616 Draft CR for TS 38.101-3 on updates to delta TIB for DC configurations of 1 band LTE and 1 band NR band --------------------------------------------------------------------------------------------- Completed EN-DC configurations in RAN4#104-bis-e and RAN4#105-e from the latest TR (R4-2218550): Inter-band EN-DC in FR1: DL DC_8B_n78A with UL DC_8A_n78A DL DC_3C_n28A with UL DC_3C_n28A DL DC_1A_n26A with UL DC_1A_n26A DL DC_3A_n26A with UL DC_3A_n26A DL DC_3C_n26A with UL DC_3A_n26A, DC_3C_n26A DL DC_7A_n26A with UL DC_7A_n26A DL DC_7C_n26A with UL DC_7A_n26A, DC_7C_n26A DL DC_28A_n20A with UL DC_28A_n20A DL DC_28A_n38A with UL DC_28A_n38A DL DC_8A_n38A with UL DC_8A_n38A DC_1A_n1A with single switched UL only Some configurations are completed via draft CR approach. This CR covers the following endorsed draft CR. Endorsed draft CR in RAN4#104-bis-e R4-2215371 DraftCR 38.101-3 Addition of 1 Band LTE and 1 Band NR EN-DC Combinations R4-2216630 Draft CR for TS 38.101-3 to add configuration DC_1A_n257(2G) R4-2216631 Draft CR for TS 38.101-3 to add configuration DC_7A_n257(2G) Endorsed draft CR in RAN4#105 R4-2220603 draftCR to 38.101-3: Addition of DC_40A_n40A R4-2220606 Draft CR for TS 38.101-3: Support of n78(A-C) in DC_1_n78, DC_3_n78, DC_5_n78, DC_7_n78 and DC_7-7_n78 R4-2218919 Draft CR 38.101-3 to add DC_26A_n258A/B/C/D/E/F/G/H/I/J/K/L/M Endorsed correction draft CR in RAN4#105: R4-2219756 Draft CR for TS 38.101-3 to update delta TIB template for NE-DC combinations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Completed DC configurations are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3B.1.3, 5.3B.1a (new), 5.5B.2a (new), 5.5B.3, 5.5B.4, 5.5B.5, 6.2B.1.1a (new), 6.2B.1.2, 6.2B.1.3, 6.2B.4.2.1a (new), 6.2B.4.2.3.1, 6.2B.4.2.3a, 6.4B.2.1a (new), 6.5B.3.3.2, 6.5B.5.1a (new), 7.3B.2.1a (new), 7.3B.2.3.2, 7.3B.2.3.5, 7.3B.3.2, 7.3B.3.3, 7.4B.1a (new), 7.5B.1a (new), 7.6B.2.1a (new), 7.6B.3.1a (new), 7.6B.4.1a (new), 7.7B.1a (new), 7.8B.2.1a (new), 7.9B.1a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

5.3B.1	Intra-band EN-DC in FR1

5.3B.1.1	General

The requirements for intra-band EN-DC in this specification are defined for EN-DC configurations with associated bandwidth combination sets.

For each EN-DC configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

5.3B.1.2	BCS for Intra-band contiguous EN-DC

For intra-band contiguous EN-DC, an EN-DC configuration is a single operating band supporting an intra-band contiguous EN-DC bandwidth class.

Bandwidth combination sets for intra-band contiguous EN-DC are specified in Table 5.3B.1.2-1. The EN-DC configurations and bandwidth combination sets in Table 5.3B.1.2-1 also apply to higher order EN-DC combinations that include inter-band and intra-band EN-DC on the downlink and inter-band EN-DC on the uplink. If no BCS is reported in the UE capabilities for an intra-band combination the default is that the UE supports BCS0.

Table 5.3B.1.2-1: EN-DC configurations and bandwidth combination sets defined for intra-band contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_(n)3AA | DC_(n)3AA4 | 5, 10, 15, 20 | 5,10,15,20, 25, 30 |  | 50 | 0 |
|  |  |  | 5,10,15,20, 25, 30 | 5, 10, 15, 20 |  |  |
| DC_(n)5AA | DC_(n)5AA4 | 5, 10 | 5, 10, 15, 20 |  | 25 | 0 |
|  |  |  | 5, 10, 15, 20 | 5, 10 |  |  |
| DC_(n)7AA | DC_(n)7AA4 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
|  |  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |
| DC_(n)12AA | DC_(n)12AA4 | 5, 10 | 5, 10 |  | 15 | 0 |
|  |  |  | 5, 10 | 5, 10 |  |  |
| DC_(n)38AA | DC_(n)38AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)41AA | DC_(n)41AA | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 20, 30, 40, 50, 60, 80,100 | 10 |  |  |
| DC_(n)41AB | DC_(n)41AA, DC_41A_n41A | 10 | 20+20 |  | 70 | 0 |
|  |  |  | 20+20 | 10 |  |  |
|  |  | 20 | 10+20 |  |  |  |
|  |  |  | 10+20 | 20 |  |  |
|  |  | 20 | 20+30 |  |  |  |
|  |  |  | 20+30 | 20 |  |  |
| DC_(n)41CA | DC_(n)41AA, DC_41A_n41A2 | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 140 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20+20 |  |  |
|  |  | 10+20 | 10, 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 10+20 |  |  |
| DC_(n)41DA | DC_(n)41AA, DC_41A_n41A2 | 20+20+20 | 40, 60, 80,100 |  | 160 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 40, 50, 60, 80,100 |  | 160 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 30, 40, 50, 60, 80,100 |  | 160 | 2 |
|  |  |  | 30, 40, 50, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+15 | 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 30, 40, 50, 60, 80,100 | 20+20+15 |  |  |
| DC_(n)48AA5 | DC_(n)48AA4 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)48CA5 | DC_(n)48AA4 DC_48A_n48A4 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)48DA5 | DC_(n)48AA4 DC_48A_n48A4 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_(n)66AA | DC_(n)66AA4 | 5, 10, 15, 20 | 5,10,15,20, 25, 30, 40 |  | 60 | 0 |
|  |  |  | 5,10,15,20, 25, 30, 40 | 5, 10, 15, 20 |  |  |
| DC_(n)71AA | DC_(n)71AA3 | 15 | 5 |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |
|  |  | 5 | 5, 10, 15 |  |  |  |
|  |  |  | 5 | 15 |  |  |
|  |  |  | 5, 10 | 10 |  |  |
|  |  |  | 5, 10, 15 | 5 |  |  |
|  |  | 5 | 5,10,15,20 |  | 253 | 1 |
|  |  | 10 | 5,10,15 |  |  |  |
|  |  | 15 | 5,10 |  |  |  |
|  |  |  | 5,10,15,20 | 5 |  |  |
|  |  |  | 5,10,15 | 10 |  |  |
|  |  |  | 5,10 | 15 |  |  |
|  | - | 5 | 5, 10, 15, 20 |  | 356 | 2 |
|  |  | 10 | 10, 15, 20 |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |
|  |  |  | 5, 10, 15, 20 | 5 |  |  |
|  |  |  | 10, 15, 20 | 10 |  |  |
|  |  |  | 15, 20 | 15 |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: For maximum DL aggregated bandwidth of 25 MHz the asymmetric UL and DL channel bandwidth combination of Table 5.3.6-1 in TS 38.101-1 [2] is used with a maximum UL contiguous aggregated bandwidth of 20 MHz. Furthermore, a restriction is imposed on bandwidth combinations so that only a subset of BCS1 is allowed to be used on the uplink, and this subset is equivalent to BCS0. NOTE4: Only single switched UL is supported. NOTE5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 6: Bandwidth Combination Set 2 only applies to intra-band EN-DC with uplink in n71 but not in Band 71, and paired with another E-UTRA band. |  |  |  |  |  |  |

5.3B.1.3	BCS for Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC, an EN-DC configuration is a single operating band supporting E-UTRA and NR carriers, where E-UTRA configuration is indicated by using E-UTRA CA bandwidth class as defined in TS 36.101 [4] and NR configuration is indicated by using NR CA bandwidth class as defined in TS 38.101-1 [2].

Requirements for intra-band non-contiguous EN-DC are defined for the EN-DC configurations and bandwidth combination sets specified in Table 5.3B.1.3-1. The EN-DC configurations and bandwidth combination sets in Table 5.3B.1.3-1 also apply to higher order EN-DC combinations that include inter-band and intra-band EN-DC on the downlink and inter-band EN-DC on the uplink.  If no BCS is reported in the UE capabilities for an intra-band combination the default is that the UE supports BCS0.

Table 5.3B.1.3-1: EN-DC configurations and bandwidth combination sets defined for intra-band non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_1A_n1A | DC_1A_n1A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_2A_n2A | DC_2A_n2A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_3A_n3A | DC_3A_n3A |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30 |  |  |  |
| DC_5A_n5A | DC_5A_n5A2 | 5, 10 | 5, 10, 15 |  | 20 | 0 |
| DC_7A_n7A3 | DC_7A_n7A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| DC_40A_n40A4 | DC_40A_n40A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20 |  |  |
| DC_41A_n41A | DC_41A_n41A | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 20, 30, 40, 50, 60, 80,100 | 10 |  |  |
| DC_41C_n41A | DC_41A_n41A | 20+20 | 40, 60, 80,100 |  | 140 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20 |  |  |
|  |  | 20+20 | 40, 50, 60, 80,100 |  | 140 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20 |  |  |
| DC_41D_n41A | DC_41A_n41A | 20+20+20 | 40, 60, 80,100 |  | 160 | 0 |
|  |  |  | 40, 60, 80,100 | 20+20+20 |  |  |
|  |  | 20+20+20 | 40, 50, 60, 80,100 |  | 160 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20+20+20 |  |  |
| DC_48A_n48A4 | DC_48A_n48A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 60 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | 5, 10, 15, 20 |  |  |
| DC_48A-48A_n48A4 | DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| DC_48C_n48A4 | DC_48A_n48A2 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48C Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_48D_n48A4 | DC_48A_n48A2 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 | 5, 10, 15, 20, 40 |  | 100 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48D Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-1 |  |  |
| DC_66A_n66A | DC_66A_n66A2 | 5, 10, 15, 20 | 5, 10, 15, 20, 40 |  | 50 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30, 40 |  | 60 | 1 |
|  |  |  | 5, 10, 15, 20, 25, 30, 40 | 5, 10, 15, 20 |  |  |
| DC_71A_n71A | DC_71A_n71A2 | 15 | 5 |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |
|  |  | 5 | 5, 10, 15 |  |  |  |
|  |  |  | 5 | 15 |  |  |
|  |  |  | 5, 10 | 10 |  |  |
|  |  |  | 5, 10, 15 | 5 |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 30 | 1 |
|  |  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |  |  |  |  |

Table 5.3B.1.3-2: EN-DC configurations and bandwidth combination sets defined for intra-band EN-DC mixed intra-band contiguous and non-contiguous EN-DC

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_48A-(n)48AA4,5 | DC_(n)48AA2 DC_48A_n48A2 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 | 5, 10, 15, 20, 40 |  | 80 | 0 |
|  |  |  | 5, 10, 15, 20, 40 | See CA_48A-48A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |
| NOTE 1: Void. NOTE 2: Only single switched UL is supported. NOTE 3: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 4: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 5: The UE supporting these configurations mixed intra-band continuous and non-continuous ENDC with at most two sub-blocks containing more than two component carriers indicates ‘both’ by IE intraBandENDC-Support. |  |  |  |  |  |  |

5.3B.1a	Intra-band NE-DC in FR1

5.3B.1a.1	General

The requirements for intra-band NE-DC in this specification are defined for NE-DC configurations with associated bandwidth combination sets.

For each NE-DC configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

5.3B.1a.2	BCS for Intra-band contiguous NE-DC

For intra-band contiguous NE-DC, an NE-DC configuration is a single operating band supporting an intra-band contiguous NE-DC bandwidth class.

Bandwidth combination sets for intra-band contiguous NE-DC are specified in Table 5.3B.1a.2-1. The NE-DC configurations and bandwidth combination sets in Table 5.3B.1a.2-1 also apply to higher order NE-DC combinations that include inter-band and intra-band EN-DC on the downlink and inter-band NE-DC on the uplink. If no BCS is reported in the UE capabilities for an intra-band combination the default is that the UE supports BCS0.

Table 5.3B.1a.2-1: NE-DC configurations and bandwidth combination sets defined for intra-band contiguous NE-DC

| NR – E-UTRA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink NE-DC configuration | Uplink NE-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_3(n)AA | DC_3(n)AA1 | 5, 10, 15, 20 | 5,10,15, 20, 25, 30 |  | 50 | 0 |
|  |  |  | 5,10,15, 20, 25, 30 | 5, 10, 15, 20 |  |  |
| NOTE 1: Only single switched UL is supported. |  |  |  |  |  |  |

<< Next Changes >>

5.5B.2	Intra-band contiguous EN-DC

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)3AA | DC_(n)3AA6 | Yes6 |
| DC_(n)5AA | DC_(n)5AA6 | Yes6 |
| DC_(n)7AA | DC_(n)7AA6 | Yes6 |
| DC_(n)12AA | DC_(n)12AA6 | Yes6 |
| DC_(n)38AA5 | DC_(n)38AA6 | Yes6 |
| DC_(n)41AA5 DC_(n)41AB5 DC_(n)41CA5 DC_(n)41DA5 | DC_(n)41AA DC_41A_n41A | Yes3 |
| DC_(n)48AA5 | DC_(n)48AA6 | Yes6 |
| DC_(n)48CA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)48DA5 | DC_(n)48AA6 DC_48A_n48A6 | Yes6 |
| DC_(n)66AA | DC_(n)66AA6 | Yes |
| DC_(n)71AA2 | DC_(n)71AA | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. Uplink DC_(n)71AA is not applicable to BCS2. NOTE 5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 6: Only single switched UL is supported |  |  |

5.5B.2a	Intra-band contiguous NE-DC

Table 5.5B.2a-1: Intra-band contiguous NE-DC configurations

| NE-DC configuration | Uplink NE-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_3(n)AA | DC_3(n)AA2 | Yes2 |
| NOTE 1: Uplink NE-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported |  |  |

5.5B.3	Intra-band non-contiguous EN-DC

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_1A_n1A | DC_1A_n1A5 | Yes5 |
| DC_2A_n2A | DC_2A_n2A5 | Yes5 |
| DC_3A_n3A | DC_3A_n3A | Yes7 |
| DC_5A_n5A | DC_5A_n5A5 | Yes5 |
| DC_7A_n7A6 | DC_7A_n7A5 | Yes5 |
| DC_40A_n40A3 | DC_40A_n40A5 | Yes5 |
| DC_41A_n41A3 DC_41C_n41A3 DC_41D_n41A3 | DC_41A_n41A | Yes4 |
| DC_48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48A-48A_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48C_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_48D_n48A3 | DC_48A_n48A5 | Yes5 |
| DC_66A_n66A | DC_66A_n66A5 | Yes5 |
| DC_66A-66A_n66A | DC_66A_n66A5 | Yes5 |
| DC_71A_n71A3 | DC_71A_n71A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void. NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 7: Single UL allowed due to potential emission issues and self-interference. |  |  |

Table 5.5B.3-2: Intra-band EN-DC configurations for mixed intra-band contiguous and non-contiguous EN-DC

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_48A-(n)48AA3 | DC_(n)48AA5 DC_48A_n48A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Void NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. |  |  |

5.5B.4	Inter-band EN-DC within FR1

5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_1A_n3A DC_1C_n3A | DC_1A_n3A DC_1C_n3A | DC_1_n3 |  |
| DC_1A_n5A | DC_1A_n5A | No |  |
| DC_1A_n7A DC_1A_n7B | DC_1A_n7A DC_1A_n7B | No |  |
| DC_1A-1A_n7A DC_1A-1A_n7B | DC_1A_n7A | No |  |
| DC_1A_n8A | DC_1A_n8A | No |  |
| DC_1A_n20A | DC_1A_n20A | No |  |
| DC_1A_n26A |  | DC_1A_n26A |  | No |  |  |  |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A-1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n38A DC_1C_n38A | DC_1A_n38A | No |  |
| DC_1A_n40A DC_1A_n40B | DC_1A_n40A | No |  |
| DC_1A_n41A7 | DC_1A_n41A | No |  |
| DC_1A_n50A | DC_1A_n50A | No |  |
| DC_1A_n51A | DC_1A_n51A | No |  |
| DC_1A_n71A DC_1A_n71B | DC_1A_n71A | No |  |
| DC_1A_n77A7 DC_1A_n77C7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n77(2A)7 DC_1A_n77(3A)7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n78A7 DC_1A_n78C7 | DC_1A_n78A | No | No |
| DC_1A_n78(2A)7 DC_1A_n78(A-C)7 | DC_1A_n78A | No | No |
| DC_1A-1A_n78A | DC_1A_n78A | No | No |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No | No |
| DC_2A_n5A | DC_2A_n5A | No |  |
| DC_2A-2A_n5A | DC_2A_n5A | No |  |
| DC_2A_n7A | DC_2A_n7A | No |  |
| DC_2A_n7(2A) | DC_2A_n7A | No |  |
| DC_2A_n12A | DC_2A_n12A | No |  |
| DC_2A_n25A11, 13, 20 | N/A | N/A |  |
| DC_2A_n28A | DC_2A_n28A | No |  |
| DC_2A_n30A | DC_2A_n30A | No |  |
| DC_2A-2A_n30A | DC_2A_n30A | No |  |
| DC_2A_n38A | DC_2A_n38A | No |  |
| DC_2A-2A_n38A | DC_2A_n38A | No |  |
| DC_2A_n41A DC_2A_n41C DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |  |
| DC_2A_n41(2A) | DC_2A_n41A | No |  |
| DC_2A-2A_n41A | DC_2A_n41A | No |  |
| DC_2A_n46A | DC_2A_n46A | No |  |
| DC_2A_n48A DC_2A_n48B | DC_2A_n48A | No |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n66(2A) | DC_2A_n66A | DC_2_n66 |  |
| DC_2A-2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n71A DC_2A_n71B DC_2C_n71A | DC_2A_n71A | No |  |
| DC_2A-2A_n71A | DC_2A_n71A | No |  |
| DC_2A_n77A DC_2A_n77C | DC_2A_n77A | DC_2_n77 |  |
| DC_2A_n77(2A) | DC_2A_n77A | DC_2_n77 |  |
| DC_2A-2A_n77A DC_2A-2A_n77C | DC_2A_n77A | DC_2_n77 |  |
| DC_2A-2A_n77(2A) | DC_2A_n77A | DC_2_n77 |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A_n78(2A) | DC_2A_n78A | DC_2_n78 |  |
| DC_2A-2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_3A_n1A DC_3C_n1A | DC_3A_n1A DC_3C_n1A | DC_3_n1 |  |
| DC_3A-3A_n1A | DC_3A_n1A | DC_3_n1 |  |
| DC_3A_n5A DC_3C_n5A | DC_3A_n5A | DC_3_n5 |  |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A DC_3C_n7B | DC_3A_n7A DC_3A_n7B DC_3C_n7A | No |  |
| DC_3A-3A_n7A DC_3A-3A_n7B | DC_3A_n7A | No |  |
| DC_3A_n8A | DC_3A_n8A | No |  |
| DC_3A-3A_n8A | DC_3A_n8A | No |  |
| DC_3A_n20A DC_3C_n20A | DC_3A_n20A | No |  |
| DC_3A_n26A DC_3C_n26A | DC_3A_n26A DC_3C_n26A | Yes |  |
| DC_3A_n28A DC_3C_n28A | DC_3A_n28A DC_3C_n28A | No |  |
| DC_3A_n34A | DC_3A_n34A | No |  |
| DC_3A_n38A DC_3C_n38A | DC_3A_n38A | No |  |
| DC_3A_n40A DC_3A_n40B | DC_3A_n40A | No |  |
| DC_3A_n41A7 DC_3C_n41A | DC_3A_n41A DC_3C_n41A | DC_3_n41 | No |
| DC_3A_n50A | DC_3A_n50A | No |  |
| DC_3A_n51A | DC_3A_n51A | No |  |
| DC_3A_n71A DC_3A_n71B | DC_3A_n71A | No |  |
| DC_3A_n77A7 DC_3A_n77C7 DC_3C_n77A7 | DC_3A_n77A DC_3C_n77A | DC_3_n77 | No |
| DC_3A_n77(2A)7 DC_3A_n77(3A)7 DC_3C_n77(2A)7 | DC_3A_n77A DC_3C_n77A | DC_3_n77 | No |
| DC_3A-3A_n77A7 | DC_3A_n77A | DC_3_n77 | No |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 | DC_3A_n78A DC_3C_n78A | DC_3_n78 | No |
| DC_3A_n78(2A)7 DC_3A_n78(A-C)7 DC_3C_n78(2A)7 | DC_3A_n78A DC_3C_n78A | DC_3_n78 | No |
| DC_3A-3A_n78A7, 21 | DC_3A_n78A21 | DC_3_n78 | No |
| DC_3A_n79A7 DC_3A_n79C7 DC_3C_n79A7 | DC_3A_n79A DC_3C_n79A | No | No |
| DC_4A_n2A | DC_4A_n2A | No |  |
| DC_4A_n5A | DC_4A_n5A | DC_4_n5 |  |
| DC_4A_n7A | DC_4A_n7A | No |  |
| DC_4A_n28A | DC_4A_n28A | No |  |
| DC_4A_n38A | DC_4A_n38A | No |  |
| DC_4A_n41A | DC_4A_n41A | No |  |
| DC_4A_n78A | DC_4A_n78A | No |  |
| DC_4A_n78(2A) | DC_4A_n78A | No |  |
| DC_5A_n1A | DC_5A_n1A | No |  |
| DC_5A_n2A DC_5B_n2A | DC_5A_n2A | No |  |
| DC_5A-5A_n2A | DC_5A_n2A | No |  |
| DC_5A_n3A | DC_5A_n3A | DC_5_n3 |  |
| DC_5A_n7A | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n7(2A) | DC_5A_n7A | DC_5_n7 |  |
| DC_5A_n12A | DC_5A_n12A | No |  |
| DC_5A_n30A | DC_5A_n30A | No |  |
| DC_5A_n38A | DC_5A_n38A | DC_5_n38 |  |
| DC_5A_n40A | DC_5A_n40A | No |  |
| DC_5A_n48A DC_5A_n48B | DC_5A_n48A | No |  |
| DC_5A_n66A DC_5B_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A-5A_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A_n77A DC_5A_n77C | DC_5A_n77A | No |  |
| DC_5A_n77(2A) DC_5A_n77(3A) | DC_5A_n77A | No |  |
| DC_5A_n71A | DC_5A_n71A | No |  |
| DC_5A_n78A7 DC_5A_n78C7 | DC_5A_n78A | No | No |
| DC_5A_n78(2A)7 DC_5A_n78(A-C)7 | DC_5A_n78A | No | No |
| DC_5A_n79A | DC_5A_n79A | No | No |
| DC_7A_n1A DC_7C_n1A | DC_7A_n1A DC_7C_n1A | No |  |
| DC_7A-7A_n1A | DC_7A_n1A | No |  |
| DC_7A_n2A DC_7C_n2A | DC_7A_n2A | No |  |
| DC_7A_n3A DC_7C_n3A | DC_7A_n3A DC_7C_n3A | No |  |
| DC_7A_n5A DC_7C_n5A | DC_7A_n5A DC_7C_n5A | DC_7_n5 |  |
| DC_7A-7A_n5A | DC_7A_n5A | DC_7_n5 |  |
| DC_7A_n8A | DC_7A_n8A | No |  |
| DC_7A-7A_n8A | DC_7A_n8A | No |  |
| DC_7A-7A_n78(2A)7 | DC_7A_n78A | No |  |
| DC_7A_n20A | DC_7A_n20A | No |  |
| DC_7A_n25A DC_7C_n25A | DC_7A_n25A | No |  |
| DC_7A_n26A DC_7C_n26A | DC_7A_n26A DC_7C_n26A | Yes |  |
| DC_7A-7A_n25A | DC_7A_n25A | No |  |
| DC_7A_n28A DC_7C_n28A | DC_7A_n28A DC_7C_n28A | No |  |
| DC_7A-7A_n28A | DC_7A_n28A | No |  |
| DC_7A_n40A | DC_7A_n40A | Yes |  |
| DC_7A_n51A | DC_7A_n51A | No |  |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |  |
| DC_7A-7A_n66A | DC_7A_n66A | No |  |
| DC_7A_n71A | DC_7A_n71A | No |  |
| DC_7A_n77A7 DC_7C_n77A | DC_7A_n77A | No |  |
| DC_7A_n77(2A) DC_7A_n77(3A) DC_7C_n77(2A) | DC_7A_n77A | No |  |
| DC_7A-7A_n77A7 | DC_7A_n77A | No |  |
| DC_7A-7A_n77(2A) DC_7A-7A_n77(3A) | DC_7A_n77A | No |  |
| DC_7A_n78A7 DC_7C_n78A7 DC_7A_n78C7 | DC_7A_n78A DC_7C_n78A | No |  |
| DC_7A_n78(2A)7 DC_7A_n78(A-C)7 DC_7C_n78(2A)7 | DC_7A_n78A DC_7C_n78A | No |  |
| DC_7A-7A_n78A7, 21 DC_7A-7A_n78C7 | DC_7A_n78A21 | No |  |
| DC_7A-7A_n78(A-C)7 | DC_7A_n78A | No |  |
| DC_7A_n79A DC_7A_n79C | DC_7A_n79A | No |  |
| DC_8A_n1A | DC_8A_n1A | No |  |
| DC_8A_n2A | DC_8A_n2A | DC_8_n2 |  |
| DC_8A_n3A | DC_8A_n3A | No |  |
| DC_8A_n7A | DC_8A_n7A | No |  |
| DC_8A_n20A | DC_8A_n20A | Yes |  |
| DC_8A_n28A | DC_8A_n28A | No |  |
| DC_8A_n34A | DC_8A_n34A | No |  |
| DC_8A_n38A | DC_8A_n38A | No |  |
| DC_8A_n39A | DC_8A_n39A | No |  |
| DC_8A_n40A7 | DC_8A_n40A | No |  |
| DC_8A_n41A7 DC_8A_n41C | DC_8A_n41A | No | No |
| DC_8A_n41(2A) | DC_8A_n41A | No | No |
| DC_8A_n77A7 | DC_8A_n77A | No | No |
| DC_8A_n77(2A)7 DC_8A_n77(3A)7 | DC_8A_n77A | No | No |
| DC_8A_n78A7 DC_8B_n78A7 | DC_8A_n78A | No | No |
| DC_8A_n78(2A)7 | DC_8A_n78A | No | No |
| DC_8A_n79A7 DC_8A_n79C | DC_8A_n79A DC_8A_n79C | No | No |
| DC_8A_n93A | DC_8A_n93A_ULSUP-TDM | N/A |  |
| DC_8A_n94A | DC_8A_n94A_ULSUP-TDM | N/A |  |
| DC_11A_n1A | DC_11A_n1A | No |  |
| DC_11A_n3A | DC_11A_n3A | No |  |
| DC_11A_n28A | DC_11A_n28A | No |  |
| DC_11A_n41A7 | DC_11A_n41A | No |  |
| DC_11A_n77A7 | DC_11A_n77A | No | No |
| DC_11A_n77(2A)7 DC_11A_n77(3A)7 | DC_11A_n77A | No | No |
| DC_11A_n78A7 | DC_11A_n78A | No | No |
| DC_11A_n78(2A) | DC_11A_n78A | No | No |
| DC_11A_n79A7 | DC_11A_n79A | No |  |
| DC_12A_n2A | DC_12A_n2A | No |  |
| DC_12A_n5A | DC_12A_n5A | No |  |
| DC_12A_n7A | DC_12A_n7A | No |  |
| DC_12A_n7(2A) | DC_12A_n7A | No |  |
| DC_12A_n25A | DC_12A_n25A | No |  |
| DC_12A_n30A | DC_12A_n30A | No |  |
| DC_12A_n38A | DC_12A_n38A | No |  |
| DC_12A_n41A | DC_12A_n41A | No |  |
| DC_12A_n66A | DC_12A_n66A | No |  |
| DC_12A_n66(2A) | DC_12A_n66A | No |  |
| DC_12A_n71A | DC_12A_n71A18,19 | DC_12_n71 |  |
| DC_12A_n77A | DC_12A_n77A | DC_12_n77 |  |
| DC_12A_n77(2A) | DC_12A_n77A | DC_12_n77 |  |
| DC_12A_n78A | DC_12A_n78A | DC_12_n78 |  |
| DC_12A_n78(2A) | DC_12A_n78A | DC_12_n78 |  |
| DC_13A_n2A | DC_13A_n2A | No |  |
| DC_13A_n5A | DC_13A_n5A | DC_13_n5 |  |
| DC_13A_n7A | DC_13A_n7A | No |  |
| DC_13A_n7(2A) | DC_13A_n7A | No |  |
| DC_13A_n25A | DC_13A_n25A | No |  |
| DC_13A_n48A DC_13A_n48B | DC_13A_n48A | No |  |
| DC_13A_n66A | DC_13A_n66A | No |  |
| DC_13A_n71A | DC_13A_n71A | No |  |
| DC_13A_n77A DC_13A_n77C | DC_13A_n77A | No |  |
| DC_13A_n78A | DC_13A_n78A | No |  |
| DC_13A_n78(2A) | DC_13A_n78A | No |  |
| DC_14A_n2A | DC_14A_n2A | No |  |
| DC_14A_n5A | DC_14A_n5A | DC_14_n5 |  |
| DC_14A_n30A | DC_14A_n30A | No |  |
| DC_14A_n66A | DC_14A_n66A | No |  |
| DC_14A_n77A | DC_14A_n77A | No |  |
| DC_14A_n77(2A) | DC_14A_n77A | No |  |
| DC_18A_n3A | DC_18A_n3A | No |  |
| DC_18A_n28A | DC_18A_n28A | No |  |
| DC_18A_n41A16 | DC_18A_n41A | No |  |
| DC_18A_n77A7 DC_18A_n77(2A)7 | DC_18A_n77A | No | No |
| DC_18A_n78A7 | DC_18A_n78A | No | No |
| DC_18A_n78(2A)7 | DC_18A_n78A | No | No |
| DC_20A_n91A | DC_20A_n91A_ULSUP-TDM | N/A |  |
| DC_20A_n92A | DC_20A_n92A_ULSUP-TDM | N/A |  |
| DC_18A_n79A7 | DC_18A_n79A | No |  |
| DC_19A_n1A | DC_19A_n1A | No |  |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |  |
| DC_19A_n77(2A)7 | DC_19A_n77A | No |  |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No | No |
| DC_19A_n78(2A)7 | DC_19A_n78A | No | No |
| DC_19A_n79A7 DC_19A_n79C7 | DC_19A_n79A | No | No |
| DC_20A_n1A | DC_20A_n1A | No |  |
| DC_20A_n3A | DC_20A_n3A | No |  |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |  |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |  |
| DC_20A_n28A8,11,13 | DC_20A_n28A | No |  |
| DC_20A_n38A | DC_20A_n38A | No |  |
| DC_20A_n41A | DC_20A_n41A | DC_20_n41 |  |
| DC_20A_n50A | DC_20A_n50A | No |  |
| DC_20A_n51A | DC_20A_n51A | No |  |
| DC_20A_n77A7 | DC_20A_n77A | No |  |
| DC_20A_n78A7 DC_20A_n78C7 | DC_20A_n78A | No |  |
| DC_20A_n78(2A)7 | DC_20A_n78A | No |  |
| DC_21A_n1A | DC_21A_n1A | No |  |
| DC_21A_n28A17 | DC_21A_n28A | DC_21_n28 |  |
| DC_21A_n77A7 DC_21A_n77C7 | DC_21A_n77A | No |  |
| DC_21A_n77(2A)7 | DC_21A_n77A | No |  |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No | No |
| DC_21A_n78(2A)7 | DC_21A_n78A | No | No |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No | No |
| DC_25A_n41A | DC_25A_n41A | No |  |
| DC_25A-25A_n41A | DC_25A_n41A | No |  |
| DC_25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A-25A_n77A | DC_25A_n77A | DC_25_n77 |  |
| DC_25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_25A-25A_n78A | DC_25A_n78A | DC_25_n78 |  |
| DC_26A_n25A | DC_26A_n25A | No |  |
| DC_26A_n41A | DC_26A_n41A | No |  |
| DC_26A_n77A7 | DC_26A_n77A | No |  |
| DC_26A_n78A7 | DC_26A_n78A | No |  |
| DC_26A_n79A7 | DC_26A_n79A | No |  |
| DC_28A_n1A | DC_28A_n1A | No |  |
| DC_28A_n2A | DC_28A_n2A | No |  |
| DC_28A_n3A | DC_28A_n3A | No |  |
| DC_28A_n5A | DC_28A_n5A | No |  |
| DC_28A_n7A DC_28A_n7B | DC_28A_n7A DC_28A_n7B | No |  |
| DC_28A_n51A | DC_28A_n51A | No |  |
| DC_28A_n8A | DC_28A_n8A | No |  |
| DC_28A_n20A8,11,13 | DC_28A_n20A | No |  |
| DC_28A_n38A | DC_28A_n38A | No |  |
| DC_28A_n40A | DC_28A_n40A | No |  |
| DC_28A_n41A7 | DC_28A_n41A | No |  |
| DC_28A_n50A | DC_28A_n50A | No |  |
| DC_28A_n66A | DC_28A_n66A | No |  |
| DC_28A_n77A7 DC_28A_n77C7 | DC_28A_n77A | No | No |
| DC_28A_n77(2A)7 | DC_28A_n77A | No | No |
| DC_28A_n78A7 DC_28A_n78C7 | DC_28A_n78A | No | No |
| DC_28A_n78(2A)7 | DC_28A_n78A | No | No |
| DC_28A_n79A7 DC_28A_n79C7 | DC_28A_n79A | No |  |
| DC_30A_n2A | DC_30A_n2A | No |  |
| DC_30A_n5A | DC_30A_n5A | No |  |
| DC_30A_n66A | DC_30A_n66A | No |  |
| DC_30A_n77A | DC_30A_n77A | No |  |
| DC_30A_n77(2A) | DC_30A_n77A | No |  |
| DC_38A_n1A | DC_38A_n1A | No |  |
| DC_38A_n3A | DC_38A_n3A | No |  |
| DC_38A_n8A | DC_38A_n8A | No |  |
| DC_38A_n28A | DC_38A_n28A | No |  |
| DC_38A_n78A7 | DC_38A_n78A | No |  |
| DC_38A_n79A DC_38A_n79C | DC_38A_n79A | No |  |
| DC_39A_n40A3 | DC_39A_n40A | No |  |
| DC_39A_n41A3 DC_39C_n41A3 DC_39A_n41C3 | DC_39A_n41A DC_39C_n41A | No | No |
| DC_39A_n78A5,7 | DC_39A_n78A | No |  |
| DC_39A_n79A7 DC_39A_n79C7 | DC_39A_n79A | No | No |
| DC_40A_n1A DC_40C_n1A | DC_40A_n1A | No |  |
| DC_40A_n41A3 DC_40A_n41C3 DC_40C_n41A3 | DC_40A_n41A | No |  |
| DC_40A_n41(2A)3 | DC_40A_n41A | No |  |
| DC_40A_n77A DC_40A_n77C | DC_40A_n77A | No |  |
| DC_40A_n78A DC_40C_n78A | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n78(2A) DC_40C_n78(2A) | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n79A7,12 DC_40A_n79C7,12 DC_40C_n79A7,12 | DC_40A_n79A | No | No |
| DC_41A_n1A DC_41C_n1A | DC_41A_n1A DC_41C_n1A | No | DC_41A_n1A DC_41C_n1A |
| DC_41A_n3A7 DC_41C_n3A7 | DC_41A_n3A DC_41C_n3A | No |  |
| DC_41A_n28A7 DC_41C_n28A7 | DC_41A_n28A DC_41C_n28A | No |  |
| DC_41A_n77A DC_41C_n77A | DC_41A_n77A DC_41C_n77A | No |  |
| DC_41A_n77(2A) DC_41C_n77(2A) | DC_41A_n77A DC_41C_n77A | No |  |
| DC_41A_n78A DC_41C_n78A DC_41D_n78A | DC_41A_n78A DC_41C_n78A | No |  |
| DC_41A_n78(2A) DC_41C_n78(2A) | DC_41A_n78A DC_41C_n78A | No |  |
| DC_41A_n79A6,7 DC_41A_n79C6,7 DC_41C_n79A6,7 | DC_41A_n79A DC_41C_n79A | No | No |
| DC_42A_n1A7 DC_42C_n1A7 | DC_42A_n1A DC_42C_n1A | No |  |
| DC_42A_n3A7 DC_42C_n3A7 | DC_42A_n3A DC_42C_n3A | DC_42_n3 |  |
| DC_42A_n28A7 DC_42C_n28A7 | DC_42A_n28A DC_42C_n28A | No |  |
| DC_42A_n51A | DC_42A_n51A | No |  |
| DC_42A_n77A3,4,9,11 DC_42A_n77C3,4,9,11 DC_42C_n77A3,4,9,11 DC_42C_n77C3,4,9,11 DC_42D_n77A3,4,9,11 DC_42D_n77C DC_42E_n77A3,4,9,11 DC_42E_n77C | N/A | N/A |  |
| DC_42A_n77(2A)3,4,9,11 DC_42C_n77(2A)3,4,9,11 | N/A | N/A |  |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42D_n78C3,4,9,11 DC_42E_n78A3,4,9,11 DC_42E_n78C3,4,9,11 | N/A | N/A |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42D_n79C9,15 DC_42E_n79A9,15 DC_42E_n79C9,15 | N/A | N/A |  |
| DC_46A_n77A2 | N/A | N/A |  |
| DC_46A_n78A2 DC_46C_n78A2 DC_46D_n78A2 DC_46E_n78A2 | N/A | N/A |  |
| DC_48A_n2A DC_48C_n2A DC_48D_n2A DC_48E_n2A | DC_48A_n2A | No |  |
| DC_48A_n5A DC_48C_n5A DC_48D_n5A DC_48E_n5A | DC_48A_n5A | No |  |
| DC_48A_n12A | DC_48A_n12A | No |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A | DC_48A_n25A | No |  |
| DC_48A_n46A DC_48B_n46A DC_48C_n46A DC_48D_n46A DC_48E_n46A DC_48A_n46B DC_48B_n46B DC_48C_n46B DC_48D_n46B DC_48E_n46B DC_48A_n46C DC_48B_n46C DC_48C_n46C DC_48D_n46C DC_48E_n46C DC_48A_n46D DC_48B_n46D DC_48C_n46D DC_48D_n46D DC_48E_n46D DC_48A_n46E DC_48B_n46E DC_48C_n46E DC_48D_n46E DC_48E_n46E | DC_48A_n46A DC_48B_n46A | No |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A DC_48E_n66A | DC_48A_n66A | No |  |
| DC_48A_n71A DC_48B_n71A DC_48C_n71A DC_48D_n71A | DC_48A_n71A | No |  |
| DC_48A-48A_n71A DC_48A-48A-48A_n71A | DC_48A_n71A | No |  |
| DC_48A_n77A3. 4. 9, 11 DC_48C_n77A3. 4. 9, 11 DC_48A_n77C3. 4. 9, 11 DC_48C_n77C3. 4. 9, 11 DC_48D_n77A3. 4. 9, 11 DC_48D_n77C3. 4. 9, 11 DC_48E_n77A3. 4. 9, 11 | N/A | N/A |  |
| DC_48A-48A_n77A | N/A | N/A |  |
| DC_48A-48A-48A_n77A | N/A | N/A |  |
| DC_66A_n2A DC_66B_n2A DC_66C_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A-66A-66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n5A DC_66B_n5A DC_66C_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A-66A-66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A_n7A | DC_66A_n7A | No |  |
| DC_66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A-66A_n7A | DC_66A_n7A | No |  |
| DC_66A-66A_n7(2A) | DC_66A_n7A | No |  |
| DC_66A_n12A | DC_66A_n12A | No |  |
| DC_66A_n25A | DC_66A_n25A | DC_66_n25 |  |
| DC_66A_n28A | DC_66A_n28A | No |  |
| DC_66A_n30A | DC_66A_n30A | No |  |
| DC_66A-66A_n30A | DC_66A_n30A | No |  |
| DC_66A_n38A | DC_66A_n38A | No |  |
| DC_66A-66A_n38A | DC_66A_n38A | No |  |
| DC_66A_n41A DC_66A_n41C | DC_66A_n41A | No |  |
| DC_66A_n41(2A) | DC_66A_n41A | No |  |
| DC_66A_n46A | DC_66A_n46A | No |  |
| DC_66A_n48A DC_66A_n48B | DC_66A_n48A | No |  |
| DC_66A-66A_n48A DC_66A-66A_n48B | DC_66A_n48A | No |  |
| DC_66A_n71A DC_66C_n71A DC_66A_n71B | DC_66A_n71A | No |  |
| DC_66A-66A_n71A | DC_66A_n71A | No |  |
| DC_66A_n77A DC_66A_n77C | DC_66A_n77A | DC_66_n77 |  |
| DC_66A_n77(2A) | DC_66A_n77A | DC_66_n77 |  |
| DC_66A-66A_n77A DC_66A-66A_n77C | DC_66A_n77A | DC_66_n77 |  |
| DC_66A-66A_n77(2A) | DC_66A_n77A | DC_66_n77 |  |
| DC_66A-66A-66A_n77A DC_66A-66A-66A_n77C | DC_66A_n77A | DC_66_n77 |  |
| DC_66A-66A-66A_n77(2A) | DC_66A_n77A | DC_66_n77 |  |
| DC_66A_n78A | DC_66A_n78A | No |  |
| DC_66A_n78(2A) | DC_66A_n78A | No |  |
| DC_66A-66A_n78A | DC_66A_n78A | No |  |
| DC_66A-66A_n78(2A) | DC_66A_n78A | No |  |
| DC_71A_n2A | DC_71A_n2A | No |  |
| DC_71A_n5A | DC_71A_n5A | No |  |
| DC_71A_n38A | DC_71A_n38A | No |  |
| DC_71A_n41A | DC_71A_n41A | No |  |
| DC_71A_n48A | DC_71A_n48A | No |  |
| DC_71A_n66A | DC_71A_n66A | No |  |
| DC_71A_n77A DC_71A_n77C | DC_71A_n77A | No |  |
| DC_71A_n78A | DC_71A_n78A | No |  |
| DC_71A_n78(2A) | DC_71A_n78A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 / 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also apply for any band combinations when DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset of a higher order band combination. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 or Band 48 is not used. NOTE 10: Void. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16, the power imbalance requirement defined in clause 7.6B.2.6 apply. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: The frequency range in band n41 is restricted for this band combination to 2595 – 2645 MHz. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 18: Only single switched UL is supported. NOTE 19: The implementation with 4 antennas is targeted for FWA form factor for this band combination. NOTE 20: The combination is not used alone as fallback mode of other band combinations in which UL in Band 2 is not used. NOTE 21: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |  |  |

<< Next Changes >>

5.5B.5	Inter-band EN-DC including FR2

5.5B.5.1	Inter-band EN-DC configurations including FR2 (two bands)

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n257A DC_1A_n257D DC_1A_n257E DC_1A_n257F DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M | DC_1A_n257A DC_1A_n257D DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M |
| DC_1A_n257(2A) DC_1A_n257(2G) | DC_1A_n257A DC_1A_n257G DC_1A_n257(2A) DC_1A_n257(2G) |
| DC_1A_n258A DC_1A_n258B DC_1A_n258C DC_1A_n258D DC_1A_n258E DC_1A_n258F DC_1A_n258G DC_1A_n258H DC_1A_n258I DC_1A_n258J DC_1A_n258K DC_1A_n258L DC_1A_n258M | DC_1A_n258A DC_1A_n258D DC_1A_n258G DC_1A_n258H DC_1A_n258I |
| DC_1A_n258(2A) | DC_1A_n258A DC_1A_n258(2A) |
| DC_2A_n257A DC_2C_n257A | DC_2A_n257A |
| DC_2A_n257(2A) | DC_2A_n257A |
| DC_2A-2A_n257A | DC_2A_n257A |
| DC_2A_n258A DC_2A_n258D DC_2A_n258G DC_2A_n258H DC_2A_n258O DC_2A_n258P DC_2A_n258Q | DC_2A_n258A DC_2A_n258D DC_2A_n258G DC_2A_n258H DC_2A_n258O DC_2A_n258P DC_2A_n258Q |
| DC_2A_n258(2A) DC_2A_n258(3A) DC_2A_n258(4A) DC_2A_n258(5A) | DC_2A_n258A |
| DC_2A_n260A DC_2A_n260D DC_2A_n260E DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2A_n260O DC_2A_n260P DC_2A_n260Q DC_2C_n260A | DC_2A_n260A DC_2A_n260D DC_2A_n260E DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A_n260(2A) DC_2A_n260(3A) DC_2A_n260(4A) DC_2A_n260(5A) DC_2A_n260(6A) DC_2A_n260(7A) DC_2A_n260(8A) DC_2A_n260(2D) DC_2A_n260(2G) DC_2A_n260(3G) DC_2A_n260(4G) DC_2A_n260(2H) DC_2A_n260(2O) DC_2A_n260(3O) DC_2A_n260(4O) DC_2A_n260(2P) DC_2A_n260(A-D) DC_2A_n260(A-G) DC_2A_n260(A-H) DC_2A_n260(A-O) DC_2A_n260(A-P) DC_2A_n260(A-Q) DC_2A_n260(A-2G) DC_2A_n260(A-2H) DC_2A_n260(A-2O) DC_2A_n260(A-2P) DC_2A_n260(A-3G) DC_2A_n260(A-3O) DC_2A_n260(3A-G) DC_2A_n260(3A-H) DC_2A_n260(3A-O) DC_2A_n260(2A-G) DC_2A_n260(2A-H) DC_2A_n260(2A-O) DC_2A_n260(2A-P) DC_2A_n260(2A-Q) DC_2A_n260(2A-2G) DC_2A_n260(2A-2H) DC_2A_n260(2A-2O) DC_2A_n260(3A-2O) DC_2A_n260(3A-P) DC_2A_n260(4A-O) DC_2A_n260(4A-P) DC_2A_n260(4A-2O) DC_2A_n260(5A-O) DC_2A_n260(G-H) DC_2A_n260(O-P) DC_2A_n260(O-Q) DC_2A_n260(P-Q) DC_2A_n260(A-G-H) DC_2A_n260(A-O-P) DC_2A_n260(A-O-Q) DC_2A_n260(A-P-Q) DC_2A_n260(2A-O-P) DC_2A_n260(2A-G-H) DC_2A_n260(3A-O-P) | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A-2A_n260A DC_2A-2A_n260G DC_2A-2A_n260H DC_2A-2A_n260I DC_2A-2A_n260J DC_2A-2A_n260K DC_2A-2A_n260L DC_2A-2A_n260M | DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M |
| DC_2A_n261A DC_2A_n261B DC_2A_n261C DC_2A_n261D DC_2A_n261E DC_2A_n261F DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M DC_2A_n261O DC_2A_n261P DC_2A_n261Q | DC_2A_n261A DC_2A_n261B DC_2A_n261C DC_2A_n261D DC_2A_n261E DC_2A_n261F DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M DC_2A_n261O DC_2A_n261P DC_2A_n261Q |
| DC_2A_n261(2A) DC_2A_n261(3A) DC_2A_n261(4A) DC_2A_n261(2I) DC_2A_n261(2H) DC_2A_n261(A-G) DC_2A_n261(A-J) DC_2A_n261(A-K) DC_2A_n261(A-L) DC_2A_n261(A-2G) DC_2A_n261(A-H) DC_2A_n261(A-I) DC_2A_n261(2A-G) DC_2A_n261(2A-I) DC_2A_n261(2A-H) DC_2A_n261(3A-G) DC_2A_n261(G-H) DC_2A_n261(G-I) DC_2A_n261(G-J) DC_2A_n261(2G) DC_2A_n261(H-I) DC_2A_n261(A-G-H) DC_2A_n261(A-G-I) | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I |
| DC_2A-2A_n261A DC_2A-2A_n261G DC_2A-2A_n261H DC_2A-2A_n261I DC_2A-2A_n261J DC_2A-2A_n261K DC_2A-2A_n261L DC_2A-2A_n261M | DC_2A_n261A DC_2A_n261G DC_2A_n261H DC_2A_n261I DC_2A_n261J DC_2A_n261K DC_2A_n261L DC_2A_n261M |
| DC_3A_n257A DC_3A_n257B DC_3A_n257C DC_3A_n257D DC_3A_n257E DC_3A_n257F DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_3C_n257A DC_3C_n257D DC_3C_n257E DC_3C_n257F DC_3C_n257G DC_3C_n257H DC_3C_n257I DC_3C_n257J DC_3C_n257K DC_3C_n257L DC_3C_n257M | DC_3A_n257A DC_3A_n257B DC_3A_n257D DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M DC_3C_n257A |
| DC_3A-3A_n257A DC_3A-3A_n257D DC_3A-3A_n257E DC_3A-3A_n257F DC_3A-3A_n257G DC_3A-3A_n257H DC_3A-3A_n257I DC_3A-3A_n257J DC_3A-3A_n257K DC_3A-3A_n257L DC_3A-3A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A_n258A DC_3A_n258B DC_3A_n258C DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K DC_3A_n258L DC_3A_n258M DC_3C_n258A DC_3C_n258B DC_3C_n258C DC_3C_n258D DC_3C_n258E DC_3C_n258F DC_3C_n258G DC_3C_n258H DC_3C_n258I DC_3C_n258J DC_3C_n258K DC_3C_n258L DC_3C_n258M | DC_3A_n258A DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3C_n258A DC_3C_n258G DC_3C_n258H DC_3C_n258I |
| DC_4A_n260A DC_4A_n260D DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q | DC_4A_n260A DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q |
| DC_4A_n260(2A) DC_4A_n260(3A) DC_4A_n260(4A) DC_4A_n260(5A) DC_4A_n260(6A) DC_4A_n260(7A) DC_4A_n260(8A) DC_4A_n260(2D) DC_4A_n260(2G) DC_4A_n260(3G) DC_4A_n260(4G) DC_4A_n260(2H) DC_4A_n260(2O) DC_4A_n260(2P) DC_4A_n260(3O) DC_4A_n260(4O) DC_4A_n260(A-D) DC_4A_n260(2A-D) DC_4A_n260(3A-D) DC_4A_n260(4A-D) DC_4A_n260(A-O) DC_4A_n260(2A-O) DC_4A_n260(A-D-O) DC_4A_n260(2A-D-O) DC_4A_n260(A-2O) DC_4A_n260(D-2O) DC_4A_n260(A-D-2O) DC_4A_n260(2A-D-2O) DC_4A_n260(A-2D) DC_4A_n260(2A-2D) DC_4A_n260(A-P) DC_4A_n260(2A-P) DC_4A_n260(A-2P) DC_4A_n260(2A-2P) DC_4A_n260(A-G) DC_4A_n260(2A-G) DC_4A_n260(A-2G) DC_4A_n260(2A-2G) DC_4A_n260(G-O) DC_4A_n260(2G-O) DC_4A_n260(A-G-O) DC_4A_n260(2A-G-O) DC_4A_n260(2A-G-2O) DC_4A_n260(2A-G-3O) DC_4A_n260(A-2G-O) DC_4A_n260(2A-2G-O) DC_4A_n260(2A-2G-2O) DC_4A_n260(A-H) DC_4A_n260(A-2H) DC_4A_n260(2A-H) DC_4A_n260(2A-2H) DC_4A_n260(2A-2O) DC_4A_n260(A-3O) DC_4A_n260(2A-3O) DC_4A_n260(A-4O) DC_4A_n260(2A-4O) DC_4A_n260(3A-O) DC_4A_n260(3A-2O) DC_4A_n260(3A-3O) DC_4A_n260(3A-G) DC_4A_n260(3A-2G) DC_4A_n260(4A-G) DC_4A_n260(4A-2G) DC_4A_n260(4A-O) DC_4A_n260(4A-2O) DC_4A_n260(D-2G) DC_4A_n260(2D-O) DC_4A_n260(G-2O) DC_4A_n260(2G-2O) DC_4A_n260(G-3O) DC_4A_n260(2G-3O) DC_4A_n260(G-4O) DC_4A_n260(2G-4O) DC_4A_n260(3G-O) DC_4A_n260(4G-O) DC_4A_n260(H-O) DC_4A_n260(2H-O) DC_4A_n260(A-P-Q) DC_4A_n260(3A-O-P) DC_4A_n260(A-Q) DC_4A_n260(P-Q) DC_4A_n260(2A-O-P) DC_4A_n260(3A-P) DC_4A_n260(A-O-P) DC_4A_n260(2A-3G) DC_4A_n260(2A-G-H) DC_4A_n260(2A-Q) DC_4A_n260(3A-2G-O) DC_4A_n260(3A-D-O) DC_4A_n260(3A-G-O) DC_4A_n260(3A-G-2O) DC_4A_n260(3A-H) DC_4A_n260(4A-G-O) DC_4A_n260(4A-P) DC_4A_n260(5A-G) DC_4A_n260(5A-O) DC_4A_n260(A-G-2O) DC_4A_n260(A-G-3O) DC_4A_n260(A-G-4O) DC_4A_n260(A-G-H) DC_4A_n260(A-H-O) DC_4A_n260(A-2G-2O) DC_4A_n260(A-2G-3O) DC_4A_n260(A-3G-O) DC_4A_n260(A-3G) DC_4A_n260(A-4G) DC_4A_n260(A-D-G) DC_4A_n260(A-O-Q) DC_4A_n260(D-G) DC_4A_n260(D-O) DC_4A_n260(G-H) DC_4A_n260(G-H-O) DC_4A_n260(O-P) DC_4A_n260(O-Q) | DC_4A_n260A DC_4A_n260G DC_4A_n260H DC_4A_n260O DC_4A_n260P DC_4A_n260Q |
| DC_4A_n261A DC_4A_n261D DC_4A_n261G DC_4A_n261H DC_4A_n261I DC_4A_n261J DC_4A_n261K DC_4A_n261L DC_4A_n261M | DC_4A_n261A DC_4A_n261D DC_4A_n261G DC_4A_n261H DC_4A_n261I |
| DC_4A_n261(2A) DC_4A_n261(3A) DC_4A_n261(4A) DC_4A_n261(2G) DC_4A_n261(2H) DC_4A_n261(2I) DC_4A_n261(A-D) DC_4A_n261(A-H) DC_4A_n261(A-2H) DC_4A_n261(A-D-H) DC_4A_n261(A-G) DC_4A_n261(A-G-H) DC_4A_n261(A-I) DC_4A_n261(A-2I) DC_4A_n261(G-I) DC_4A_n261(A-G-I) DC_4A_n261(A-H-I) DC_4A_n261(G-H) DC_4A_n261(H-I) DC_4A_n261(D-H) DC_4A_n261(2A-D) DC_4A_n261(2A-G) DC_4A_n261(2A-H) DC_4A_n261(2A-I) DC_4A_n261(A-2G) DC_4A_n261(A-D-G) DC_4A_n261(D-G) | DC_4A_n261A DC_4A_n261H DC_4A_n261I DC_4A_n261G |
| DC_5A_n257A DC_5A_n257D DC_5A_n257E DC_5A_n257F DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_5A_n257J DC_5A_n257K DC_5A_n257L DC_5A_n257M DC_5B_n257A | DC_5A_n257A DC_5A_n257D DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_5B_n257A |
| DC_5A-5A_n257A | DC_5A_n257A |
| DC_5A_n258A DC_5A_n258D DC_5A_n258G DC_5A_n258H DC_5A_n258O DC_5A_n258P DC_5A_n258Q | DC_5A_n258A DC_5A_n258D DC_5A_n258G DC_5A_n258H DC_5A_n258O DC_5A_n258P DC_5A_n258Q |
| DC_5A_n260A DC_5A_n260B DC_5A_n260C DC_5A_n260D DC_5A_n260E DC_5A_n260F DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5B_n260A | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5B_n260A |
| DC_5A_n260(2A) DC_5A_n260(3A) DC_5A_n260(4A) DC_5A_n260(5A) DC_5A_n260(6A) DC_5A_n260(7A) DC_5A_n260(8A) DC_5A_n260(9A) DC_5A_n260(10A) DC_5A_n260(A-I) DC_5A_n260(A-P-Q) DC_5A_n260(3A-O-P) DC_5A_n260(D-G) DC_5A_n260(D-H) DC_5A_n260(D-I) DC_5A_n260(D-O) DC_5A_n260(D-P) DC_5A_n260(D-Q) DC_5A_n260(E-O) DC_5A_n260(E-P) DC_5A_n260(E-Q) DC_5A_n260(G-I) DC_5A_n260(2G) DC_5A_n260(2H) DC_5A_n260(2O) DC_5A_n260(3O) DC_5A_n260(4O) DC_5A_n260(2P) DC_5A_n260(3P) DC_5A_n260(4P) DC_5A_n260(2A-O) DC_5A_n260(A-2O) DC_5A_n260(2A-G) DC_5A_n260(A-2G) DC_5A_n260(2A-2G) DC_5A_n260(2G-O) DC_5A_n260(2A-2G-O) DC_5A_n260(A-2H) DC_5A_n260(2A-H) DC_5A_n260(2A-2H) DC_5A_n260(2A-2O) DC_5A_n260(2A-3O) DC_5A_n260(A-4O) DC_5A_n260(2A-4O) DC_5A_n260(3A-2O) DC_5A_n260(3A-2G) DC_5A_n260(4A-G) DC_5A_n260(4A-2G) DC_5A_n260(4A-O) DC_5A_n260(4A-2O) DC_5A_n260(A-O) DC_5A_n260(A-G) DC_5A_n260(G-O) DC_5A_n260(A-G-O) DC_5A_n260(2A-G-O) DC_5A_n260(A-2G-O) DC_5A_n260(A-H) DC_5A_n260(A-3O) DC_5A_n260(3A-O) DC_5A_n260(3A-G) DC_5A_n260(2D) DC_5A_n260(3G) DC_5A_n260(4G) DC_5A_n260(A-D) DC_5A_n260(2A-D) DC_5A_n260(A-D-O) DC_5A_n260(2A-D-O) DC_5A_n260(D-2O) DC_5A_n260(A-D-2O) DC_5A_n260(2A-D-2O) DC_5A_n260(A-2D) DC_5A_n260(2A-2D) DC_5A_n260(A-P) DC_5A_n260(2A-P) DC_5A_n260(A-2P) DC_5A_n260(2A-2P) DC_5A_n260(3A-3O) DC_5A_n260(D-2G) DC_5A_n260(2D-O) DC_5A_n260(G-2O) DC_5A_n260(2G-2O) DC_5A_n260(G-3O) DC_5A_n260(2G-3O) DC_5A_n260(G-4O) DC_5A_n260(2G-4O) DC_5A_n260(3G-O) DC_5A_n260(4G-O) DC_5A_n260(H-O) DC_5A_n260(2H-O) DC_5A_n260(A-Q) DC_5A_n260(P-Q) DC_5A_n260(2A-4P) DC_5A_n260(2O-2P) DC_5A_n260(3A-P) DC_5A_n260(4A-4O) DC_5A_n260(4A-2Q) DC_5A_n260(6A-2O) DC_5A_n260(6A-2P) DC_5A_n260(6A-3O) DC_5A_n260(8A-2O) DC_5A_n260(2A-O-P) DC_5A_n260(2A-2G-2O) DC_5A_n260(2A-2O-2P) DC_5A_n260(2A-2O-2Q) DC_5A_n260(O-P) DC_5A_n260(A-O-P) DC_5A_n260(2A-2O-P-Q) DC_5A_n260(2A-2O-P) DC_5A_n260(2A-2O-Q) DC_5A_n260(2A-2Q) DC_5A_n260(2A-3G) DC_5A_n260(2A-3O-P) DC_5A_n260(2A-3O-Q) DC_5A_n260(2A-3P) DC_5A_n260(2A-G-2O) DC_5A_n260(2A-G-3O) DC_5A_n260(2A-G-H) DC_5A_n260(2A-O-2P) DC_5A_n260(2A-O-2Q) DC_5A_n260(2A-O-3P) DC_5A_n260(2A-O-P-Q) DC_5A_n260(2A-O-Q) DC_5A_n260(2A-P-Q) DC_5A_n260(2A-Q) DC_5A_n260(2O-2Q) DC_5A_n260(2O-P-Q) DC_5A_n260(2O-P) DC_5A_n260(2O-Q) DC_5A_n260(2Q) DC_5A_n260(3A-2G-O) DC_5A_n260(3A-2O-P) DC_5A_n260(3A-2O-Q) DC_5A_n260(3A-2P) DC_5A_n260(3A-2Q) DC_5A_n260(3A-3P) DC_5A_n260(3A-4O) DC_5A_n260(3A-D-O) DC_5A_n260(3A-D) DC_5A_n260(3A-G-2O) DC_5A_n260(3A-G-O) DC_5A_n260(3A-H) DC_5A_n260(3A-O-2P) DC_5A_n260(3A-O-2Q) DC_5A_n260(3A-O-P-Q) DC_5A_n260(3A-O-Q) DC_5A_n260(3A-P-Q) DC_5A_n260(3A-Q) DC_5A_n260(3O-P) DC_5A_n260(3O-Q) DC_5A_n260(4A-2P) DC_5A_n260(4A-3O) DC_5A_n260(4A-D) DC_5A_n260(4A-G-O) DC_5A_n260(4A-O-P) DC_5A_n260(4A-O-Q) DC_5A_n260(4A-P-Q) DC_5A_n260(4A-P) DC_5A_n260(4A-Q) DC_5A_n260(5A-2O) DC_5A_n260(5A-2P) DC_5A_n260(5A-3O) DC_5A_n260(5A-G) DC_5A_n260(5A-O-P) DC_5A_n260(5A-O) DC_5A_n260(5A-P) DC_5A_n260(5A-Q) DC_5A_n260(6A-O-P) DC_5A_n260(6A-O) DC_5A_n260(6A-P) DC_5A_n260(7A-2O) DC_5A_n260(7A-O) DC_5A_n260(7A-P) DC_5A_n260(8A-O) DC_5A_n260(9A-O) DC_5A_n260(A-2G-2O) DC_5A_n260(A-2G-3O) DC_5A_n260(A-2O-2P) DC_5A_n260(A-2O-2Q) DC_5A_n260(A-2O-P-Q) DC_5A_n260(A-2O-P) DC_5A_n260(A-2O-Q) DC_5A_n260(A-2Q) DC_5A_n260(A-3G-O) DC_5A_n260(A-3G) DC_5A_n260(A-3O-P) DC_5A_n260(A-3O-Q) DC_5A_n260(A-3P) DC_5A_n260(A-4G) DC_5A_n260(A-4P) DC_5A_n260(A-D-G) DC_5A_n260(A-E) DC_5A_n260(A-G-2O) DC_5A_n260(A-G-3O) DC_5A_n260(A-G-4O) DC_5A_n260(A-G-H) DC_5A_n260(A-H-O) DC_5A_n260(A-O-2P) DC_5A_n260(A-O-2Q) DC_5A_n260(A-O-3P) DC_5A_n260(A-O-P-Q) DC_5A_n260(A-O-Q) DC_5A_n260(G-H-O) DC_5A_n260(G-H) DC_5A_n260(O-2P) DC_5A_n260(O-2Q) DC_5A_n260(O-3P) DC_5A_n260(O-P-Q) DC_5A_n260(O-Q) | DC_5A_n260A DC_5A_n260G DC_5A_n260H DC_5A_n260O DC_5A_n260P DC_5A_n260Q |
| DC_5A-5A_n260A | DC_5A_n260A |
| DC_5A_n261A DC_5A_n261B DC_5A_n261C DC_5A_n261D DC_5A_n261E DC_5A_n261F DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M DC_5A_n261O DC_5A_n261P DC_5A_n261Q | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M |
| DC_5A_n261(2A) DC_5A_n261(2G) DC_5A_n261(3A) DC_5A_n261(4A) DC_5A_n261(5A) DC_5A_n261(6A) DC_5A_n261(7A) DC_5A_n261(8A) DC_5A_n261(D-G) DC_5A_n261(D-H) DC_5A_n261(D-I) DC_5A_n261(D-O) DC_5A_n261(D-P) DC_5A_n261(D-Q) DC_5A_n261(E-O) DC_5A_n261(E-P) DC_5A_n261(E-Q) DC_5A_n261(2H) DC_5A_n261(2I) DC_5A_n261(A-H) DC_5A_n261(A-I) DC_5A_n261(A-J) DC_5A_n261(A-K) DC_5A_n261(A-L) DC_5A_n261(A-D) DC_5A_n261(A-D-H) DC_5A_n261(A-D-2O) DC_5A_n261(A-G) DC_5A_n261(A-G-H) DC_5A_n261(G-I) DC_5A_n261(A-G-I) DC_5A_n261(A-H-I) DC_5A_n261(G-H) DC_5A_n261(G-J) DC_5A_n261(H-I) DC_5A_n261(A-2D) DC_5A_n261(A-2H) DC_5A_n261(A-2P) DC_5A_n261(A-2Q) DC_5A_n261(A-2I) DC_5A_n261(A-4G) DC_5A_n261(A-4O) DC_5A_n261(A-7O) DC_5A_n261(A-2G-2O) DC_5A_n261(A-3G-O) DC_5A_n261(2A-G) DC_5A_n261(2A-H) DC_5A_n261(2A-I) DC_5A_n261(3A-G) DC_5A_n261(2A-2G-O) DC_5A_n261(2A-2G) DC_5A_n261(2A-2O) DC_5A_n261(2A-3G) DC_5A_n261(2A-3O) DC_5A_n261(2A-4O) DC_5A_n261(2A-5O) DC_5A_n261(2A-6O) DC_5A_n261(2A-D-O) DC_5A_n261(2A-D) DC_5A_n261(2A-G-2O) DC_5A_n261(2A-G-O) DC_5A_n261(2A-O) DC_5A_n261(2A-P) DC_5A_n261(2A-Q) DC_5A_n261(2D) DC_5A_n261(2G-2O) DC_5A_n261(2G-O) DC_5A_n261(2O) DC_5A_n261(2P) DC_5A_n261(2Q) DC_5A_n261(3A-2G) DC_5A_n261(3A-2O) DC_5A_n261(3A-3O) DC_5A_n261(3A-4O) DC_5A_n261(3A-5O) DC_5A_n261(3A-D) DC_5A_n261(3A-G-O) DC_5A_n261(3A-O) DC_5A_n261(3G-O) DC_5A_n261(3G) DC_5A_n261(3O) DC_5A_n261(4A-2O) DC_5A_n261(4A-3O) DC_5A_n261(4A-4O) DC_5A_n261(4A-G) DC_5A_n261(4A-O) DC_5A_n261(4G) DC_5A_n261(4O) DC_5A_n261(5A-2O) DC_5A_n261(5A-3O) DC_5A_n261(5A-O) DC_5A_n261(5O) DC_5A_n261(6A-2O) DC_5A_n261(6A-O) DC_5A_n261(6O) DC_5A_n261(7A-O) DC_5A_n261(7O) DC_5A_n261(A-2G-O) DC_5A_n261(A-2G) DC_5A_n261(A-2O) DC_5A_n261(A-3G) DC_5A_n261(A-3O) DC_5A_n261(A-5O) DC_5A_n261(A-6O) DC_5A_n261(A-D-G) DC_5A_n261(A-D-O) DC_5A_n261(A-E) DC_5A_n261(A-G-2O) DC_5A_n261(A-G-O) DC_5A_n261(A-O-P) DC_5A_n261(A-O-Q) DC_5A_n261(A-O) DC_5A_n261(A-P-Q) DC_5A_n261(A-P) DC_5A_n261(A-Q) DC_5A_n261(D-2O) DC_5A_n261(G-2O) DC_5A_n261(G-O) DC_5A_n261(O-P) DC_5A_n261(O-Q) DC_5A_n261(P-Q) | DC_5A_n261A DC_5A_n261G DC_5A_n261H DC_5A_n261I |
| DC_7A_n257A DC_7A_n257D DC_7A_n257E DC_7A_n257F DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_7A_n257L DC_7A_n257M | DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A_n257(2A) DC_7A_n257(2G) | DC_7A_n257A DC_7A_n257G DC_7A_n257(2A) DC_7A_n257(2G) |
| DC_7A-7A_n257A DC_7A-7A_n257D DC_7A-7A_n257E DC_7A-7A_n257F DC_7A-7A_n257G DC_7A-7A_n257H DC_7A-7A_n257I DC_7A-7A_n257J DC_7A-7A_n257K DC_7A-7A_n257L DC_7A-7A_n257M | DC_7A_n257A DC_7A_n257D DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K |
| DC_7A_n258A DC_7A_n258B DC_7A_n258C DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7C_n258A DC_7C_n258B DC_7C_n258C DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_7C_n258J DC_7C_n258K DC_7C_n258L DC_7C_n258M | DC_7A_n258A DC_7A_n258B DC_7A_n258C DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M DC_7C_n258A DC_7C_n258B DC_7C_n258C DC_7C_n258D DC_7C_n258E DC_7C_n258F DC_7C_n258G DC_7C_n258H DC_7C_n258I DC_7C_n258J DC_7C_n258K DC_7C_n258L DC_7C_n258M |
| DC_7A_n258(2A) | DC_7A_n258A DC_7A_n258(2A) |
| DC_8A_n257A DC_8A_n257D DC_8A_n257E DC_8A_n257F DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K DC_8A_n257L DC_8A_n257M | DC_8A_n257A DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I |
| DC_8A_n258A DC_8A_n258B DC_8A_n258C DC_8A_n258D DC_8A_n258E DC_8A_n258F DC_8A_n258G DC_8A_n258H DC_8A_n258I DC_8A_n258J DC_8A_n258K DC_8A_n258L DC_8A_n258M | DC_8A_n258A |
| DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | DC_11A_n257A DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I |
| DC_12A_n257A | DC_12A_n257A |
| DC_12A_n258A DC_12A_n258D DC_12A_n258G DC_12A_n258H DC_12A_n258O DC_12A_n258P DC_12A_n258Q | DC_12A_n258A DC_12A_n258D DC_12A_n258G DC_12A_n258H DC_12A_n258O DC_12A_n258P DC_12A_n258Q |
| DC_12A_n260A DC_12A_n260D DC_12A_n260E DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_12A_n260O DC_12A_n260P DC_12A_n260Q | DC_12A_n260A DC_12A_n260D DC_12A_n260E DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M DC_12A_n260O DC_12A_n260P DC_12A_n260Q |
| DC_12A_n260(2A) DC_12A_n260(2G) DC_12A_n260(A-G) DC_12A_n260(A-H) DC_12A_n260(G-H) DC_12A_n260(A-I) DC_12A_n260(G-I) | DC_12A_n260A |
| DC_12A_n261A DC_12A_n261B DC_12A_n261C DC_12A_n261D DC_12A_n261E DC_12A_n261F DC_12A_n261G DC_12A_n261H DC_12A_n261I DC_12A_n261J DC_12A_n261K DC_12A_n261L DC_12A_n261M DC_12A_n261O DC_12A_n261P DC_12A_n261Q | DC_12A_n261A DC_12A_n261B DC_12A_n261C DC_12A_n261D DC_12A_n261E DC_12A_n261F DC_12A_n261G DC_12A_n261H DC_12A_n261I DC_12A_n261J DC_12A_n261K DC_12A_n261L DC_12A_n261M DC_12A_n261O DC_12A_n261P DC_12A_n261Q |
| DC_13A_n257A | DC_13A_n257A |
| DC_13A_n260A DC_13A_n260D DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260K DC_13A_n260L DC_13A_n260M DC_13A_n260O DC_13A_n260P DC_13A_n260Q | DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260I DC_13A_n260J DC_13A_n260K DC_13A_n260L DC_13A_n260M DC_13A_n260O DC_13A_n260P DC_13A_n260Q |
| DC_13A_n260(2A) DC_13A_n260(3A) DC_13A_n260(4A) DC_13A_n260(5A) DC_13A_n260(6A) DC_13A_n260(7A) DC_13A_n260(8A) DC_13A_n260(2D) DC_13A_n260(2G) DC_13A_n260(3G) DC_13A_n260(4G) DC_13A_n260(2H) DC_13A_n260(2O) DC_13A_n260(3O) DC_13A_n260(4O) DC_13A_n260(A-G) DC_13A_n260(A-2G) DC_13A_n260(A-P) DC_13A_n260(A-Q) DC_13A_n260(2A-G) DC_13A_n260(2A-H) DC_13A_n260(2A-2G) DC_13A_n260(2A-2H) DC_13A_n260(3A-G) DC_13A_n260(3A-O) DC_13A_n260(3A-P) DC_13A_n260(3A-2O) DC_13A_n260(4A-O) DC_13A_n260(4A-2O) DC_13A_n260(P-Q) DC_13A_n260(A-P-Q) DC_13A_n260(2A-O-P) DC_13A_n260(3A-O-P) DC_13A_n260(A-H) DC_13A_n260(A-2H) DC_13A_n260(2A-O) DC_13A_n260(A-O) DC_13A_n260(2A-P) DC_13A_n260(A-O-P) DC_13A_n260(O-P) DC_13A_n260(2A-2O) DC_13A_n260(A-2O) DC_13A_n260(G-H) DC_13A_n260(2A-G-H) DC_13A_n260(2A-Q) DC_13A_n260(2P) DC_13A_n260(3A-H) DC_13A_n260(4A-P) DC_13A_n260(5A-O) DC_13A_n260(A-2P) DC_13A_n260(A-3G) DC_13A_n260(A-3O) DC_13A_n260(A-D) DC_13A_n260(A-G-H) DC_13A_n260(A-O-Q) DC_13A_n260(O-Q) | DC_13A_n260A DC_13A_n260G DC_13A_n260H DC_13A_n260O DC_13A_n260P DC_13A_n260Q |
| DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261J DC_13A_n261K DC_13A_n261I DC_13A_n261L DC_13A_n261M | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I DC_13A_n261J DC_13A_n261K DC_13A_n261L DC_13A_n261M |
| DC_13A_n261(2A) DC_13A_n261(2G) DC_13A_n261(3A) DC_13A_n261(4A) DC_13A_n261(2H) DC_13A_n261(2I) DC_13A_n261(A-G) DC_13A_n261(A-K) DC_13A_n261(A-L) DC_13A_n261(A-2G) DC_13A_n261(A-H) DC_13A_n261(A-I) DC_13A_n261(A-J) DC_13A_n261(2A-G) DC_13A_n261(2A-H) DC_13A_n261(2A-I) DC_13A_n261(3A-G) DC_13A_n261(G-H) DC_13A_n261(G-I) DC_13A_n261(G-J) DC_13A_n261(H-I) DC_13A_n261(A-G-H) DC_13A_n261(A-G-I) | DC_13A_n261A DC_13A_n261G DC_13A_n261H DC_13A_n261I |
| DC_14A_n258A | DC_14A_n258A |
| DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M | DC_14A_n260A DC_14A_n260G DC_14A_n260H DC_14A_n260I DC_14A_n260J DC_14A_n260K DC_14A_n260L DC_14A_n260M |
| DC_18A_n257A DC_18A_n257D DC_18A_n257E DC_18A_n257F DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_18A_n257J DC_18A_n257K DC_18A_n257L DC_18A_n257M | DC_18A_n257A DC_18A_n257G DC_18A_n257H DC_18A_n257I |
| DC_19A_n257A DC_19A_n257D DC_19A_n257E DC_19A_n257F DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L DC_19A_n257M | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L DC_19A_n257M |
| DC_20A_n257A DC_20A_n257B DC_20A_n257C DC_20A_n257D DC_20A_n257E DC_20A_n257F DC_20A_n257G DC_20A_n257H DC_20A_n257I DC_20A_n257J DC_20A_n257K DC_20A_n257L DC_20A_n257M | DC_20A_n257A |
| DC_20A_n258A DC_20A_n258B DC_20A_n258C DC_20A_n258D DC_20A_n258E DC_20A_n258F DC_20A_n258G DC_20A_n258H DC_20A_n258I DC_20A_n258J DC_20A_n258K DC_20A_n258L DC_20A_n258M | DC_20A_n258A |
| DC_21A_n257A DC_21A_n257D DC_21A_n257E DC_21A_n257F DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_26A_n257A | DC_26A_n257A |
| DC_26A_n258A DC_26A_n258B DC_26A_n258C DC_26A_n258D DC_26A_n258E DC_26A_n258F DC_26A_n258G DC_26A_n258H DC_26A_n258I DC_26A_n258J DC_26A_n258K DC_26A_n258L DC_26A_n258M | DC_26A_n258A DC_26A_n258G DC_26A_n258H DC_26A_n258I |
| DC_28A_n257A DC_28A_n257D DC_28A_n257E DC_28A_n257F DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_28A_n257J DC_28A_n257K DC_28A_n257L DC_28A_n257M | DC_28A_n257A DC_28A_n257D DC_28A_n257G DC_28A_n257H DC_28A_n257I DC_28A_n257J DC_28A_n257K DC_28A_n257L DC_28A_n257M |
| DC_28A_n258A DC_28A_n258B DC_28A_n258C DC_28A_n258D DC_28A_n258E DC_28A_n258F DC_28A_n258G DC_28A_n258H DC_28A_n258I DC_28A_n258J DC_28A_n258K DC_28A_n258L DC_28A_n258M | DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I |
| DC_30A_n258A | DC_30A_n258A |
| DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M | DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M |
| DC_30A_n260(2A) DC_30A_n260(2G) DC_30A_n260(A-G) DC_30A_n260(A-H) DC_30A_n260(G-H) DC_30A_n260(A-I) DC_30A_n260(G-I) | DC_30A_n260A |
| DC_38A_n257A DC_38A_n257G DC_38A_n257H DC_38A_n257I DC_38A_n257J DC_38A_n257K DC_38A_n257L DC_38A_n257M | DC_38A_n257A DC_38A_n257G DC_38A_n257H DC_38A_n257I DC_38A_n257J DC_38A_n257K DC_38A_n257L DC_38A_n257M |
| DC_38A_n258A DC_38A_n258G DC_38A_n258H DC_38A_n258I DC_38A_n258J DC_38A_n258K DC_38A_n258L DC_38A_n258M | DC_38A_n258A DC_38A_n258G DC_38A_n258H DC_38A_n258I DC_38A_n258J DC_38A_n258K DC_38A_n258L DC_38A_n258M |
| DC_39A_n257A DC_39A_n257D DC_39A_n257E DC_39A_n257F DC_39A_n257G DC_39A_n257H DC_39A_n257I DC_39A_n257J DC_39A_n257K DC_39A_n257L DC_39A_n257M | DC_39A_n257A |
| DC_39A_n258A DC_39A_n258B DC_39A_n258C DC_39A_n258D DC_39A_n258E DC_39A_n258F DC_39A_n258G DC_39A_n258H DC_39A_n258I DC_39A_n258J DC_39A_n258K DC_39A_n258L DC_39A_n258M | DC_39A_n258A DC_39A_n258B DC_39A_n258C DC_39A_n258D DC_39A_n258E DC_39A_n258F DC_39A_n258G DC_39A_n258H DC_39A_n258I DC_39A_n258J DC_39A_n258K DC_39A_n258L DC_39A_n258M |
| DC_40A_n257A DC_40A_n257D DC_40A_n257E DC_40A_n257F DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M DC_40C_n257A DC_40C_n257D DC_40C_n257E DC_40C_n257F DC_40C_n257G DC_40C_n257H DC_40C_n257I DC_40C_n257J DC_40C_n257K DC_40C_n257L DC_40C_n257M | DC_40A_n257A DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A-40A_n257A DC_40A-40A_n257D DC_40A-40A_n257E DC_40A-40A_n257F DC_40A-40A_n257G DC_40A-40A_n257H DC_40A-40A_n257I DC_40A-40A_n257J DC_40A-40A_n257K DC_40A-40A_n257L DC_40A-40A_n257M | DC_40A_n257A DC_40A_n257G DC_40A_n257H DC_40A_n257I DC_40A_n257J DC_40A_n257K DC_40A_n257L DC_40A_n257M |
| DC_40A_n258A DC_40A_n258B DC_40A_n258C DC_40A_n258D DC_40A_n258E DC_40A_n258F DC_40A_n258G DC_40A_n258H DC_40A_n258I DC_40A_n258J DC_40A_n258K DC_40A_n258L DC_40A_n258M | DC_40A_n258A DC_40A_n258B DC_40A_n258C DC_40A_n258D DC_40A_n258E DC_40A_n258F DC_40A_n258G DC_40A_n258H DC_40A_n258I DC_40A_n258J DC_40A_n258K DC_40A_n258L DC_40A_n258M |
| DC_41A_n257A DC_41A_n257D DC_41A_n257E DC_41A_n257F DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41A_n257J DC_41A_n257K DC_41A_n257L DC_41A_n257M DC_41C_n257A DC_41C_n257D DC_41C_n257E DC_41C_n257F DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_41C_n257J DC_41C_n257K DC_41C_n257L DC_41C_n257M | DC_41A_n257A DC_41A_n257D DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41C_n257A DC_41C_n257D DC_41C_n257G DC_41C_n257H DC_41C_n257I |
| DC_41A_n258A DC_41A_n258B DC_41A_n258C DC_41A_n258D DC_41A_n258E DC_41A_n258F DC_41A_n258G DC_41A_n258H DC_41A_n258I DC_41A_n258J DC_41A_n258K DC_41A_n258L DC_41A_n258M | DC_41A_n258A DC_41A_n258B DC_41A_n258C DC_41A_n258D DC_41A_n258E DC_41A_n258F DC_41A_n258G DC_41A_n258H DC_41A_n258I DC_41A_n258J DC_41A_n258K DC_41A_n258L DC_41A_n258M |
| DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F DC_42C_n257G DC_42C_n257H DC_42C_n257I DC_42C_n257J DC_42C_n257K DC_42C_n257L DC_42C_n257M DC_42D_n257A DC_42D_n257D DC_42D_n257E DC_42D_n257F DC_42D_n257G DC_42D_n257H DC_42D_n257I DC_42D_n257J DC_42D_n257K DC_42D_n257L DC_42D_n257M DC_42E_n257A DC_42E_n257D DC_42E_n257E DC_42E_n257F DC_42E_n257G DC_42E_n257H DC_42E_n257I DC_42E_n257J DC_42E_n257K DC_42E_n257L DC_42E_n257M | DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F DC_42D_n257A DC_42D_n257D DC_42D_n257E DC_42D_n257F DC_42E_n257A DC_42E_n257D DC_42E_n257E DC_42E_n257F |
| DC_42A-42A_n257A DC_42A-42A_n257D DC_42A-42A_n257E DC_42A-42A_n257F DC_42A-42A_n257G DC_42A-42A_n257H DC_42A-42A_n257I DC_42A-42A_n257J DC_42A-42A_n257K DC_42A-42A_n257L DC_42A-42A_n257M | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M |
| DC_46A_n260A3 | N/A |
| DC_46A_n261A3 | N/A |
| DC_48A_n257A DC_48C_n257A | DC_48A_n257A DC_48C_n257A |
| DC_48A-48A_n257A | DC_48A_n257A |
| DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48A_n260J DC_48A_n260K DC_48A_n260L DC_48A_n260M DC_48C_n260A DC_48D_n260A | DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48C_n260A |
| DC_48A_n260(2A) DC_48C_n260(2A) DC_48D_n260(2A) DC_48A_n260(3A) DC_48C_n260(3A) DC_48D_n260(3A) DC_48A_n260(4A) DC_48C_n260(4A) DC_48D_n260(4A) | DC_48A_n260A DC_48A_n260G DC_48A_n260H DC_48A_n260I DC_48C_n260A |
| DC_48A-48A_n260A | DC_48A_n260A |
| DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48A_n261J DC_48A_n261K DC_48A_n261L DC_48A_n261M DC_48C_n261A DC_48D_n261A | DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48C_n261A |
| DC_48A_n261(A-G) DC_48A_n261(A-G-H) DC_48A_n261(A-G-I) DC_48A_n261(A-H) DC_48A_n261(A-I) DC_48A_n261(A-J) DC_48A_n261(A-K) DC_48A_n261(A-L) DC_48A_n261(A-2G) DC_48A_n261(G-H) DC_48A_n261(G-I) DC_48A_n261(G-J) DC_48A_n261(H-I) DC_48A_n261(2A) DC_48C_n261(2A) DC_48D_n261(2A) DC_48A_n261(3A) DC_48A_n261(2A-G) DC_48A_n261(2A-H) DC_48A_n261(2A-I) DC_48A_n261(2G) DC_48A_n261(2H) DC_48A_n261(4A) DC_48A_n261(3A-G) | DC_48A_n261A DC_48A_n261G DC_48A_n261H DC_48A_n261I DC_48C_n261A |
| DC_66A_n257A DC_66A_n257G DC_66A_n257H DC_66A_n257I DC_66A_n257J DC_66A_n257K DC_66A_n257L DC_66A_n257M DC_66C_n257A | DC_66A_n257A |
| DC_66A_n257(2A) DC_66A-66A_n257A | DC_66A_n257A |
| DC_66A_n258A DC_66A_n258D DC_66A_n258G DC_66A_n258H DC_66A_n258O DC_66A_n258P DC_66A_n258Q | DC_66A_n258A DC_66A_n258D DC_66A_n258G DC_66A_n258H DC_66A_n258O DC_66A_n258P DC_66A_n258Q |
| DC_66A_n258(2A) DC_66A_n258(3A) DC_66A_n258(4A) DC_66A_n258(5A) | DC_66A_n258A |
| DC_66A_n260A DC_66A_n260D DC_66A_n260E DC_66A_n260F DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A_n260(2A) DC_66A_n260(3A) DC_66A_n260(4A) DC_66A_n260(5A) DC_66A_n260(6A) DC_66A_n260(7A) DC_66A_n260(8A) DC_66A_n260(9A) DC_66A_n260(10A) DC_66A_n260(A-I) DC_66A_n260(D-G) DC_66A_n260(D-H) DC_66A_n260(D-I) DC_66A_n260(D-O) DC_66A_n260(D-P) DC_66A_n260(D-Q) DC_66A_n260(E-O) DC_66A_n260(E-P) DC_66A_n260(E-Q) DC_66A_n260(G-I) DC_66A_n260(2G) DC_66A_n260(2H) DC_66A_n260(2O) DC_66A_n260(3O) DC_66A_n260(4O) DC_66A_n260(2P) DC_66A_n260(3P) DC_66A_n260(4P) DC_66A_n260(2A-O) DC_66A_n260(A-2O) DC_66A_n260(2A-G) DC_66A_n260(A-2G) DC_66A_n260(2A-2G) DC_66A_n260(2G-O) DC_66A_n260(2A-2G-O) DC_66A_n260(A-2H) DC_66A_n260(2A-H) DC_66A_n260(2A-2H) DC_66A_n260(2A-2O) DC_66A_n260(2A-3O) DC_66A_n260(A-4O) DC_66A_n260(2A-4O) DC_66A_n260(3A-2O) DC_66A_n260(3A-2G) DC_66A_n260(4A-G) DC_66A_n260(4A-2G) DC_66A_n260(4A-O) DC_66A_n260(4A-2O) DC_66A_n260(A-O) DC_66A_n260(A-G) DC_66A_n260(G-O) DC_66A_n260(A-G-O) DC_66A_n260(2A-G-O) DC_66A_n260(A-2G-O) DC_66A_n260(A-H) DC_66A_n260(A-3O) DC_66A_n260(3A-O) DC_66A_n260(3A-O-P) DC_66A_n260(3A-P) DC_66A_n260(3A-G) DC_66A_n260(2D) DC_66A_n260(3G) DC_66A_n260(4G) DC_66A_n260(A-D) DC_66A_n260(2A-D) DC_66A_n260(A-D-O) DC_66A_n260(2A-D-O) DC_66A_n260(D-2O) DC_66A_n260(A-D-2O) DC_66A_n260(2A-D-2O) DC_66A_n260(2A-O-P) DC_66A_n260(A-2D) DC_66A_n260(2A-2D) DC_66A_n260(A-P) DC_66A_n260(A-P-Q) DC_66A_n260(2A-P) DC_66A_n260(A-2P) DC_66A_n260(2A-2P) DC_66A_n260(3A-3O) DC_66A_n260(D-2G) DC_66A_n260(2D-O) DC_66A_n260(G-H) DC_66A_n260(G-2O) DC_66A_n260(2G-2O) DC_66A_n260(G-3O) DC_66A_n260(2G-3O) DC_66A_n260(G-4O) DC_66A_n260(2G-4O) DC_66A_n260(3G-O) DC_66A_n260(4G-O) DC_66A_n260(H-O) DC_66A_n260(2H-O) DC_66A_n260(2A-2G-2O) DC_66A_n260(6A-2O) DC_66A_n260(8A-2O) DC_66A_n260(2A-2O-2P) DC_66A_n260(6A-3O) DC_66A_n260(4A-4O) DC_66A_n260(6A-2P) DC_66A_n260(2O-2P) DC_66A_n260(2A-4P) DC_66A_n260(4A-2Q) DC_66A_n260(2A-2O-2Q) DC_66A_n260(A-Q) DC_66A_n260(P-Q) DC_66A_n260(A-O-P) DC_66A_n260(O-P) DC_66A_n260(2A-2O-P-Q) DC_66A_n260(2A-2O-P) DC_66A_n260(2A-2O-Q) DC_66A_n260(2A-2Q) DC_66A_n260(2A-3G) DC_66A_n260(2A-3O-P) DC_66A_n260(2A-3O-Q) DC_66A_n260(2A-3P) DC_66A_n260(2A-G-2O) DC_66A_n260(2A-G-3O) DC_66A_n260(2A-G-H) DC_66A_n260(2A-O-2P) DC_66A_n260(2A-O-2Q) DC_66A_n260(2A-O-3P) DC_66A_n260(2A-O-P-Q) DC_66A_n260(2A-O-Q) DC_66A_n260(2A-P-Q) DC_66A_n260(2A-Q) DC_66A_n260(2O-2Q) DC_66A_n260(2O-P-Q) DC_66A_n260(2O-P) DC_66A_n260(2O-Q) DC_66A_n260(2Q) DC_66A_n260(3A-2G-O) DC_66A_n260(3A-2O-P) DC_66A_n260(3A-2O-Q) DC_66A_n260(3A-2P) DC_66A_n260(3A-2Q) DC_66A_n260(3A-3P) DC_66A_n260(3A-4O) DC_66A_n260(3A-D-O) DC_66A_n260(3A-D) DC_66A_n260(3A-G-2O) DC_66A_n260(3A-G-O) DC_66A_n260(3A-H) DC_66A_n260(3A-O-2P) DC_66A_n260(3A-O-2Q) DC_66A_n260(3A-O-P-Q) DC_66A_n260(3A-O-Q) DC_66A_n260(3A-P-Q) DC_66A_n260(3A-Q) DC_66A_n260(3O-P) DC_66A_n260(3O-Q) DC_66A_n260(4A-2P) DC_66A_n260(4A-3O) DC_66A_n260(4A-D) DC_66A_n260(4A-G-O) DC_66A_n260(4A-O-P) DC_66A_n260(4A-O-Q) DC_66A_n260(4A-P-Q) DC_66A_n260(4A-P) DC_66A_n260(4A-Q) DC_66A_n260(5A-2O) DC_66A_n260(5A-2P) DC_66A_n260(5A-3O) DC_66A_n260(5A-G) DC_66A_n260(5A-O-P) DC_66A_n260(5A-O) DC_66A_n260(5A-P) DC_66A_n260(5A-Q) DC_66A_n260(6A-O-P) DC_66A_n260(6A-O) DC_66A_n260(6A-P) DC_66A_n260(7A-2O) DC_66A_n260(7A-O) DC_66A_n260(7A-P) DC_66A_n260(8A-O) DC_66A_n260(9A-O) DC_66A_n260(A-2G-2O) DC_66A_n260(A-2G-3O) DC_66A_n260(A-2O-2P) DC_66A_n260(A-2O-2Q) DC_66A_n260(A-2O-P-Q) DC_66A_n260(A-2O-P) DC_66A_n260(A-2O-Q) DC_66A_n260(A-2Q) DC_66A_n260(A-3G-O) DC_66A_n260(A-3G) DC_66A_n260(A-3O-P) DC_66A_n260(A-3O-Q) DC_66A_n260(A-3P) DC_66A_n260(A-4G) DC_66A_n260(A-4P) DC_66A_n260(A-D-G) DC_66A_n260(A-E) DC_66A_n260(A-G-2O) DC_66A_n260(A-G-3O) DC_66A_n260(A-G-4O) DC_66A_n260(A-G-H) DC_66A_n260(A-H-O) DC_66A_n260(A-O-2P) DC_66A_n260(A-O-2Q) DC_66A_n260(A-O-3P) DC_66A_n260(A-O-P-Q) DC_66A_n260(A-O-Q) DC_66A_n260(G-H-O) DC_66A_n260(O-2P) DC_66A_n260(O-2Q) DC_66A_n260(O-3P) DC_66A_n260(O-P-Q) DC_66A_n260(O-Q) | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A-66A_n260A DC_66A-66A_n260G DC_66A-66A_n260H DC_66A-66A_n260I DC_66A-66A_n260J DC_66A-66A_n260K DC_66A-66A_n260L DC_66A-66A_n260M | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M |
| DC_66A-66A_n260(2A) DC_66A-66A_n260(2G) DC_66A-66A_n260(2H) DC_66A-66A_n260(3A) DC_66A-66A_n260(4A) DC_66A-66A_n260(5A) DC_66A-66A_n260(6A) DC_66A-66A_n260(A-G) DC_66A-66A_n260(A-H) DC_66A-66A_n260(A-2G) DC_66A-66A_n260(G-H) DC_66A-66A_n260(2A-G) DC_66A-66A_n260(2A-2G) DC_66A-66A_n260(3A-G) | DC_66A_n260A DC_66A_n260G DC_66A_n260H |
| DC_66A_n261A DC_66A_n261B DC_66A_n261C DC_66A_n261D DC_66A_n261E DC_66A_n261F DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M DC_66A_n261O DC_66A_n261P DC_66A_n261Q | DC_66A_n261A DC_66A_n261B DC_66A_n261C DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |
| DC_66A_n261(2A) DC_66A_n261(3A) DC_66A_n261(4A) DC_66A_n261(2G) DC_66A_n261(D-G) DC_66A_n261(D-H) DC_66A_n261(D-I) DC_66A_n261(D-O) DC_66A_n261(D-P) DC_66A_n261(D-Q) DC_66A_n261(E-O) DC_66A_n261(E-P) DC_66A_n261(E-Q) DC_66A_n261(2H) DC_66A_n261(2I) DC_66A_n261(A-H) DC_66A_n261(A-I) DC_66A_n261(A-J) DC_66A_n261(A-K) DC_66A_n261(A-L) DC_66A_n261(A-D) DC_66A_n261(A-D-H) DC_66A_n261(A-G) DC_66A_n261(A-G-H) DC_66A_n261(G-I) DC_66A_n261(G-J) DC_66A_n261(A-G-I) DC_66A_n261(A-H-I) DC_66A_n261(G-H) DC_66A_n261(H-I) DC_66A_n261(A-D-2O) DC_66A_n261(A-2D) DC_66A_n261(A-2G) DC_66A_n261(A-2G-2O) DC_66A_n261(A-3G-O) DC_66A_n261(A-4G) DC_66A_n261(A-2H) DC_66A_n261(A-2I) DC_66A_n261(A-4O) DC_66A_n261(A-7O) DC_66A_n261(A-2P) DC_66A_n261(A-2Q) DC_66A_n261(2A-G) DC_66A_n261(2A-H) DC_66A_n261(2A-I) DC_66A_n261(3A-G) DC_66A_n261(2A-2G-O) DC_66A_n261(2A-2G) DC_66A_n261(2A-2O) DC_66A_n261(2A-3G) DC_66A_n261(2A-3O) DC_66A_n261(2A-4O) DC_66A_n261(2A-5O) DC_66A_n261(2A-6O) DC_66A_n261(2A-D-O) DC_66A_n261(2A-D) DC_66A_n261(2A-G-2O) DC_66A_n261(2A-G-O) DC_66A_n261(2A-O) DC_66A_n261(2A-P) DC_66A_n261(2A-Q) DC_66A_n261(2D) DC_66A_n261(2G-2O) DC_66A_n261(2G-O) DC_66A_n261(2O) DC_66A_n261(2P) DC_66A_n261(2Q) DC_66A_n261(3A-2G) DC_66A_n261(3A-2O) DC_66A_n261(3A-3O) DC_66A_n261(3A-4O) DC_66A_n261(3A-5O) DC_66A_n261(3A-D) DC_66A_n261(3A-G-O) DC_66A_n261(3A-O) DC_66A_n261(3G-O) DC_66A_n261(3G) DC_66A_n261(3O) DC_66A_n261(4A-2O) DC_66A_n261(4A-3O) DC_66A_n261(4A-4O) DC_66A_n261(4A-G) DC_66A_n261(4A-O) DC_66A_n261(4G) DC_66A_n261(4O) DC_66A_n261(5A-2O) DC_66A_n261(5A-3O) DC_66A_n261(5A-O) DC_66A_n261(5A) DC_66A_n261(5O) DC_66A_n261(6A-2O) DC_66A_n261(6A-O) DC_66A_n261(6A) DC_66A_n261(6O) DC_66A_n261(7A-O) DC_66A_n261(7A) DC_66A_n261(7O) DC_66A_n261(8A) DC_66A_n261(A-2G-O) DC_66A_n261(A-2O) DC_66A_n261(A-3G) DC_66A_n261(A-3O) DC_66A_n261(A-5O) DC_66A_n261(A-6O) DC_66A_n261(A-D-G) DC_66A_n261(A-D-O) DC_66A_n261(A-E) DC_66A_n261(A-G-2O) DC_66A_n261(A-G-O) DC_66A_n261(A-O-P) DC_66A_n261(A-O-Q) DC_66A_n261(A-O) DC_66A_n261(A-P-Q) DC_66A_n261(A-P) DC_66A_n261(A-Q) DC_66A_n261(D-2O) DC_66A_n261(G-2O) DC_66A_n261(G-O) DC_66A_n261(O-P) DC_66A_n261(O-Q) DC_66A_n261(P-Q) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_66A-66A_n261A DC_66A-66A_n261G DC_66A-66A_n261H DC_66A-66A_n261I DC_66A-66A_n261J DC_66A-66A_n261K DC_66A-66A_n261L DC_66A-66A_n261M | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M |
| DC_66A-66A_n261(2A) DC_66A-66A_n261(2G) DC_66A-66A_n261(3A) DC_66A-66A_n261(4A) DC_66A-66A_n261(A-G) DC_66A-66A_n261(A-G-H) DC_66A-66A_n261(A-G-I) DC_66A-66A_n261(A-2G) DC_66A-66A_n261(A-H) DC_66A-66A_n261(A-I) DC_66A-66A_n261(A-J) DC_66A-66A_n261(A-K) DC_66A-66A_n261(A-L) DC_66A-66A_n261(G-H) DC_66A-66A_n261(G-I) DC_66A-66A_n261(G-J) DC_66A-66A_n261(H-I) DC_66A-66A_n261(2H) DC_66A-66A_n261(2A-G) DC_66A-66A_n261(2A-H) DC_66A-66A_n261(2A-I) DC_66A-66A_n261(3A-G) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_71A_n257A | DC_71A_n257A |
| DC_71A_n258A | DC_71A_n258A |
| DC_71A_n260A | DC_71A_n260A |
| DC_71A_n261A | DC_71A_n261A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations. NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured Pcell. |  |

<< Next Changes >>

6.2B	Transmitter power for DC

6.2B.1	UE maximum output power for DC

6.2B.1.1	Intra-band contiguous EN-DC

The following UE Power Classes define the total maximum output power for any transmission bandwidth(s) of the CG(s) configured.

The maximum output power is measured as the total maximum output power across the UE antenna connector(s). The period of measurement shall be at least one sub frame.

Table 6.2B.1.1-1: Maximum output power for EN-DC (continuous sub-blocks)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)3AA3 |  |  |  |  | 23 | +2/-3 |
| DC_(n)5AA3 |  |  |  |  | 23 | +2/-3 |
| DC_(n)7AA3 |  |  |  |  | 23 | +2/-3 |
| DC_(n)12AA3 |  |  |  |  | 23 | +2/-3 |
| DC_(n)71AA |  |  |  |  | 23 | +2/-3 |
| DC_(n)38AA3 |  |  |  |  | 23 | +2/-3 |
| DC_(n)41AA | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| DC_(n)48AA3 |  |  |  |  | 23 | +2/-3 |
| DC_(n)66AA3 |  |  |  |  | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which the band has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: Power Class 3 is the default power class unless otherwise stated. NOTE 3: Only single switched UL is supported. |  |  |  |  |  |  |

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

-	if the E-UTRA UL/DL configuration is 0 or 6; or

-	if the E-UTRA UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-	if the IE p-maxUE-FR1-r15 as defined in TS 36.331 [8] is provided and set to the maximum output power of the default power class or lower;

-	apply all requirements for the default power class, and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	if the UE does not support a power class with higher maximum output power than power class 2; or

-	if the E-UTRA UL/DL configuration is not 2 or 4 or 5; or

-	if the field of UE IE maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE IE maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [9] is provided and set to the maximum output power of the power class 2 or lower;

- apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	apply all requirements for the supported power class, and set the configured transmitted power class as specified in clause 6.2B.4;

6.2B.1.1a	Intra-band contiguous NE-DC

The following UE Power Classes define the total maximum output power for any transmission bandwidth(s) of the CG(s) configured.

The maximum output power is measured as the total maximum output power across the UE antenna connector(s). The period of measurement shall be at least one sub frame.

Table 6.2B.1.1a-1: Maximum output power for NE-DC (continuous sub-blocks)

| NE-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)3AA3 |  |  |  |  | 23 | +2/-3 |
| NOTE 1: If all transmitted resource blocks over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: Power Class 3 is the default power class unless otherwise stated. NOTE 3: Only single switched UL is supported. |  |  |  |  |  |  |

6.2B.1.2	Intra-band non-contiguous EN-DC

Table 6.2B.1.2-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n1A4 |  |  |  |  | 23 | +2/-3 |
| DC_2A_n2A4 |  |  |  |  | 23 | +2/-3 |
| DC_3A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n5A4 |  |  |  |  | 23 | +2/-3 |
| DC_7A_n7A4 |  |  |  |  | 23 | +2/-3 |
| DC_40A_n40A4 |  |  |  |  | 23 | +2/-3 |
| DC_48A_n48A4 |  |  |  |  | 23 | +2/-3 |
| DC_41A_n41A | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| DC_66A_n66A4 |  |  |  |  | 23 | +2/-3 |
| DC_71A_n71A4 |  |  |  |  | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which the band has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands are confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: Void. NOTE 3: Power Class 3 is the default power class unless otherwise stated. NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |

If UE supports a different power class than the default UE power class for EN-DC band combination, and the supported power class enables higher maximum output power than that of the default power class:

-	if the E-UTRA UL/DL configuration is 0 or 6; or

-	if the E-UTRA UL/DL configuration is 1 and special subframe configuration is 0 or 5; or

-	if the IE p-maxUE-FR1-r15 as defined in TS 36.331 [8] is provided and set to the maximum output power of the default power class or lower;

-	apply all requirements for the default power class, and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	if the UE does not support a power class with higher maximum output power than power class 2; or

-	if the E-UTRA UL/DL configuration is not 2 or 4 or 5; or

-	if the field of UE IE maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE IE maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 (The exact evaluation period is no less than one radio frame); or

-	if the IE P-Max as defined in TS 38.331 [9] is provided and set to the maximum output power of the power class 2 or lower;

- apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2B.4;

-	else

-	apply all requirements for the supported power class, and set the configured transmitted power class as specified in clause 6.2B.4;

6.2B.1.3	Inter-band EN-DC within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  | 23 | +2/-3 |
| DC_1A_n5A |  |  | 23 | +2/-3 |
| DC_1A_n7A |  |  | 23 | +2/-3 |
| DC_1A_n8A |  |  | 23 | +2/-3 |
| DC_1A_n20A |  |  | 23 | +2/-3 |
| DC_1A_n26A |  |  | 23 | +2/-3 |
| DC_1A_n28A |  |  | 23 | +2/-3 |
| DC_1A_n38A |  |  | 23 | +2/-3 |
| DC_1A_n40A |  |  | 23 | +2/-3 |
| DC_1A_n41A |  |  | 23 | +2/-3 |
| DC_1A_n50A |  |  | 23 | +2/-3 |
| DC_1A_n51A |  |  | 23 | +2/-3 |
| DC_1A_n71A |  |  | 23 | +2/-3 |
| DC_1A_n77A DC_1A_n84A_ULSUP-TDM_n77A |  |  | 23 | +2/-3 |
| DC_1A_n78A DC_1A_n84A_ULSUP-TDM_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_1A_n79A DC_1A_n84A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_1A_n80A |  |  | 23 | +2/-3 |
| DC_2A_n5A |  |  | 23 | +2/-3 |
| DC_2A_n7A |  |  | 23 | +2/-3 |
| DC_2A_n12A |  |  | 23 | +2/-3 |
| DC_2A_n25A |  |  | N/A | N/A |
| DC_2A_n28A |  |  | 23 | +2/-3 |
| DC_2A_n30A |  |  | 23 | +2/-3 |
| DC_2A_n38A |  |  | 23 | +2/-3 |
| DC_2A_n41A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_2A_n46A |  |  | 23 | +2/-3 |
| DC_2A_n48A |  |  | 23 | +2/-3 |
| DC_2A_n66A |  |  | 23 | +2/-3 |
| DC_2A_n71A |  |  | 23 | +2/-3 |
| DC_2A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_2A_n78A |  |  | 23 | +2/-3 |
| DC_3A_n1A DC_3C_n1A |  |  | 23 | +2/-3 |
| DC_3A_n5A DC_3C_n5A |  |  | 23 | +2/-3 |
| DC_3A_n7A DC_3A_n7B DC_3C_n7A |  |  | 23 | +2/-3 |
| DC_3A_n8A |  |  | 23 | +2/-3 |
| DC_3A_n20A |  |  | 23 | +2/-3 |
| DC_3A_n26A DC_3C_n26A |  |  | 23 | +2/-3 |
| DC_3A_n28A DC_3C_n28A |  |  | 23 | +2/-3 |
| DC_3A_n38A |  |  | 23 | +2/-3 |
| DC_3A_n40A |  |  | 23 | +2/-3 |
| DC_3A_n41A, DC_3C_n41A, DC_3C_n41A, | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_3A_n50A |  |  | 23 | +2/-3 |
| DC_3A_n51A |  |  | 23 | +2/-3 |
| DC_3A_n71A |  |  | 23 | +2/-3 |
| DC_3A_n77A DC_3C_n77A |  |  | 23 | +2/-3 |
| DC_3A_n78A DC_3C_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_3A_n79A DC_3C_n79A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n41 DC_3C_n80A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n77A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_3A_n82A |  |  | 23 | +2/-3 |
| DC_3A_n84A |  |  | 23 | +2/-3 |
| DC_4A_n2A |  |  | 23 | +2/-3 |
| DC_4A_n5A |  |  | 23 | +2/-3 |
| DC_4A_n7A |  |  | 23 | +2/-3 |
| DC_4A_n28A |  |  | 23 | +2/-3 |
| DC_4A_n38A |  |  | 23 | +2/-3 |
| DC_4A_n41A |  |  | 23 | +2/-3 |
| DC_4A_n78A |  |  | 23 | +2/-3 |
| DC_5A_n2A |  |  | 23 | +2/-3 |
| DC_5A_n7A |  |  | 23 | +2/-3 |
| DC_5A_n12A |  |  | 23 | +2/-3 |
| DC_5A_n30A |  |  | 23 | +2/-3 |
| DC_5A_n38A |  |  | 23 | +2/-3 |
| DC_5A_n40A |  |  | 23 | +2/-3 |
| DC_5A_n48A |  |  | 23 | +2/-3 |
| DC_5A_n66A |  |  | 23 | +2/-3 |
| DC_5A_n71A |  |  | 23 | +2/-3 |
| DC_5A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_5A_n79A |  |  | 23 | +2/-3 |
| DC_7A_n1A |  |  | 23 | +2/-3 |
| DC_7A_n2A |  |  | 23 | +2/-3 |
| DC_7A_n3A |  |  | 23 | +2/-3 |
| DC_7A_n5A DC_7C_n5A |  |  | 23 | +2/-3 |
| DC_7A_n8A |  |  | 23 | +2/-3 |
| DC_7A_n20A |  |  | 23 | +2/-3 |
| DC_7A_n25A |  |  | 23 | +2/-3 |
| DC_7A_n26A DC_7C_n26A |  |  | 23 | +2/-3 |
| DC_7A_n28A |  |  | 23 | +2/-3 |
| DC_7A_n40A |  |  | 23 | +2/-3 |
| DC_7A_n51A |  |  | 23 | +2/-3 |
| DC_7A_n66A |  |  | 23 | +2/-3 |
| DC_7A_n71A |  |  | 23 | +2/-3 |
| DC_7A_n77A |  |  | 23 | +2/-3 |
| DC_7A_n78A DC_7C_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_7A_n79A |  |  | 23 | +2/-3 |
| DC_7A_n80A |  |  | 23 | +2/-3 |
| DC_8A_n1A |  |  | 23 | +2/-3 |
| DC_8A_n2A |  |  | 23 | +2/-3 |
| DC_8A_n3A |  |  | 23 | +2/-3 |
| DC_8A_n7A |  |  | 23 | +2/-3 |
| DC_8A_n20A |  |  | 23 | +2/-3 |
| DC_8A_n28A |  |  | 23 | +2/-3 |
| DC_8A_n34A |  |  | 23 | +2/-3 |
| DC_8A_n38A |  |  | 23 | +2/-3 |
| DC_8A_n39A |  |  | 23 | +2/-3 |
| DC_8A_n40A |  |  | 23 | +2/-3 |
| DC_8A_n41A, |  |  | 23 | +2/-3 |
| DC_8A_n77A |  |  | 23 | +2/-3 |
| DC_8A_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_8A_n79A DC_8A_n79C |  |  | 23 | +2/-3 |
| DC_8A_n80A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n41 |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n79A |  |  | 23 | +2/-3 |
| DC_11A_n1A |  |  | 23 | +2/-3 |
| DC_11A_n3A |  |  | 23 | +2/-3 |
| DC_11A_n28A |  |  | 23 | +2/-3 |
| DC_11A_n41A |  |  | 23 | +2/-3 |
| DC_11A_n77A |  |  | 23 | +2/-3 |
| DC_11A_n78A |  |  | 23 | +2/-3 |
| DC_11A_n79A |  |  | 23 | +2/-3 |
| DC_12A_n2A |  |  | 23 | +2/-3 |
| DC_12A_n5A |  |  | 23 | +2/-3 |
| DC_12A_n7A |  |  | 23 | +2/-3 |
| DC_12A_n25A |  |  | 23 | +2/-3 |
| DC_12A_n30A |  |  | 23 | +2/-3 |
| DC_12A_n38A |  |  | 23 | +2/-3 |
| DC_12A_n41A |  |  | 23 | +2/-3 |
| DC_12A_n66A |  |  | 23 | +2/-3 |
| DC_12A_n71A7 |  |  | 23 | +2/-3 |
| DC_12A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_12A_n78A |  |  | 23 | +2/-3 |
| DC_13A_n2A |  |  | 23 | +2/-3 |
| DC_13A_n5A |  |  | 23 | +2/-3 |
| DC_13A_n7A |  |  | 23 | +2/-3 |
| DC_13A_n25A |  |  | 23 | +2/-3 |
| DC_13A_n48A |  |  | 23 | +2/-3 |
| DC_13A_n66A |  |  | 23 | +2/-3 |
| DC_13A_n71A |  |  | 23 | +2/-3 |
| DC_13A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_13A_n78A |  |  | 23 | +2/-3 |
| DC_14A_n2A |  |  | 23 | +2/-3 |
| DC_14A_n5A |  |  | 23 | +2/-3 |
| DC_14A_n30A |  |  | 23 | +2/-3 |
| DC_14A_n66A |  |  | 23 | +2/-3 |
| DC_14A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_18A_n3A |  |  | 23 | +2/-3 |
| DC_18A_n28A |  |  | 23 | +2/-3 |
| DC_18A_n41A |  |  | 23 | +2/-3 |
| DC_18A_n77A |  |  | 23 | +2/-3 |
| DC_18A_n78A |  |  | 23 | +2/-3 |
| DC_18A_n79A |  |  | 23 | +2/-3 |
| DC_19A_n1A |  |  | 23 | +2/-3 |
| DC_19A_n77A |  |  | 23 | +2/-3 |
| DC_19A_n78A |  |  | 23 | +2/-3 |
| DC_19A_n79A |  |  | 23 | +2/-3 |
| DC_20A_n1A |  |  | 23 | +2/-3 |
| DC_20A_n3A |  |  | 23 | +2/-3 |
| DC_20A_n7A |  |  | 23 | +2/-3 |
| DC_20A_n8A |  |  | 23 | +2/-3 |
| DC_20A_n38A |  |  | 23 | +2/-3 |
| DC_20A_n28A |  |  | 23 | +2/-3 |
| DC_20A_n41A |  |  | 23 | +2/-3 |
| DC_20A_n50A |  |  | 23 | +2/-3 |
| DC_20A_n51A |  |  | 23 | +2/-3 |
| DC_20A_n77A |  |  | 23 | +2/-3 |
| DC_20A_n80A |  |  | 23 | +2/-3 |
| DC_20A_n78A |  |  | 23 | +2/-3 |
| DC_20A_n82A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_20A_n83A |  |  | 23 | +2/-3 |
| DC_21A_n1A |  |  | 23 | +2/-3 |
| DC_21A_n28A |  |  | 23 | +2/-3 |
| DC_21A_n77A |  |  | 23 | +2/-3 |
| DC_21A_n78A |  |  | 23 | +2/-3 |
| DC_21A_n79A |  |  | 23 | +2/-3 |
| DC_25A_n41A |  |  | 23 | +2/-3 |
| DC_25A_n77A |  |  | 23 | +2/-3 |
| DC_25A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n25A |  |  | 23 | +2/-3 |
| DC_26A_n41A |  |  | 23 | +2/-3 |
| DC_26A_n77A |  |  | 23 | +2/-3 |
| DC_26A_n78A |  |  | 23 | +2/-3 |
| DC_26A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n1A |  |  | 23 | +2/-3 |
| DC_28A_n2A |  |  | 23 | +2/-3 |
| DC_28A_n3A |  |  | 23 | +2/-3 |
| DC_28A_n5A |  |  | 23 | +2/-3 |
| DC_28A_n7A DC_28A_n7B |  |  | 23 | +2/-3 |
| DC_28A_n8A |  |  | 23 | +2/-3 |
| DC_28A_n20A |  |  | 23 | +2/-3 |
| DC_28A_n38A |  |  | 23 | +2/-3 |
| DC_28A_n40A |  |  | 23 | +2/-3 |
| DC_28A_n41A |  |  | 23 | +2/-3 |
| DC_28A_n50A |  |  | 23 | +2/-3 |
| DC_28A_n51A |  |  | 23 | +2/-3 |
| DC_28A_n66A |  |  | 23 | +2/-3 |
| DC_28A_n77A |  |  | 23 | +2/-3 |
| DC_28A_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_28A_n79A |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n41A |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_30A_n2A |  |  | 23 | +2/-3 |
| DC_30A_n5A |  |  | 23 | +2/-3 |
| DC_30A_n66A |  |  | 23 | +2/-3 |
| DC_30A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_38A_n1A |  |  | 23 | +2/-3 |
| DC_38A_n3A |  |  | 23 | +2/-3 |
| DC_38A_n8A |  |  | 23 | +2/-3 |
| DC_38A_n28A |  |  | 23 | +2/-3 |
| DC_38A_n78A |  |  | N/A | N/A |
| DC_38A_n79A |  |  | 23 | +2/-3 |
| DC_39A_n40A |  |  | 23 | +2/-3 |
| DC_39A_n41A DC_39C_n41A | 265,8 | +2/-3 | 23 | +2/-3 |
| DC_39A_n78A |  |  | 23 | +2/-3 |
| DC_39A_n79A | 265,8 | +2/-3 | 23 | +2/-3 |
| DC_40A_n1A |  |  | 23 | +2/-3 |
| DC_40A_n41A DC_40C_n41A |  |  | 23 | +2/-3 |
| DC_40A_n77A |  |  | N/A | N/A |
| DC_40A_n78A |  |  | 23 | +2/-3 |
| DC_40C_n78A |  |  | 23 | +2/-3 |
| DC_40A_n79A |  |  | 23 | +2/-3 |
| DC_41A_n1A DC_41C_n1A |  |  | 23 | +2/-3 |
| DC_41A_n3A DC_41C_n3A |  |  | 23 | +2/-3 |
| DC_41A_n28A DC_41C_n28A |  |  | 23 | +2/-3 |
| DC_41A_n77A DC_41C_n77A |  |  | 23 | +2/-3 |
| DC_41A_n78A DC_41C_n78A |  |  | 23 | +2/-3 |
| DC_41A_n79A DC_41C_n79A | 265,8 | +2/-3 | 23 | +2/-3 |
| DC_42A_n1A DC_42C_n1A |  |  | 23 | +2/-3 |
| DC_42A_n3A DC_42C_n3A |  |  | 23 | +2/-3 |
| DC_42A_n28A DC_42C_n28A |  |  | 23 | +2/-3 |
| DC_42A_n51A |  |  | 23 | +2/-3 |
| DC_42A_n77A |  |  | N/A | N/A |
| DC_42A_n78A |  |  | N/A | N/A |
| DC_42A_n79A |  |  | N/A | N/A |
| DC_48A_n2A |  |  | 23 | +2/-3 |
| DC_48A_n5A |  |  | 23 | +2/-3 |
| DC_48A_n12A |  |  | 23 | +2/-3 |
| DC_48A_n25A |  |  | 23 | +2/-3 |
| DC_2A_n46A |  |  | 23 | +2/-3 |
| DC_48A_n66A |  |  | 23 | +2/-3 |
| DC_48A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n2A |  |  | 23 | +2/-3 |
| DC_66A_n5A |  |  | 23 | +2/-3 |
| DC_66A_n7A |  |  | 23 | +2/-3 |
| DC_66A_n12A |  |  | 23 | +2/-3 |
| DC_66A_n25A |  |  | 23 | +2/-3 |
| DC_66A_n28A |  |  | 23 | +2/-3 |
| DC_66A_n30A |  |  | 23 | +2/-3 |
| DC_66A_n38A |  |  | 23 | +2/-3 |
| DC_66A_n41A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_66A_n46A |  |  | 23 | +2/-3 |
| DC_66A_n48A |  |  | 23 | +2/-3 |
| DC_66A_n71A |  |  | 23 | +2/-3 |
| DC_66A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| DC_66A_n78A DC_66A-66A_n78A |  |  | 23 | +2/-3 |
| DC_66A_n86A_ULSUP-TDM_n78A |  |  | 23 | +2/-3 |
| DC_71A_n2A |  |  | 23 | +2/-3 |
| DC_71A_n5A |  |  | 23 | +2/-3 |
| DC_71A_n38A |  |  | 23 | +2/-3 |
| DC_71A_n41A |  |  | 23 | +2/-3 |
| DC_71A_n48A |  |  | 23 | +2/-3 |
| DC_71A_n66A |  |  | 23 | +2/-3 |
| DC_71A_n78A |  |  | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 7: Only single switched UL is supported. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

If a UE supports a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4;

else

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

6.2B.1.3a	Inter-band NE-DC within FR1

For inter-band NE-DC of E-UTRA and NR in FR1, the following UE power classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

Table 6.2B.1.3a-1: Maximum output power for inter-band NE-DC (two bands)

| NE-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n1A_28A | 23 | +2/-3 |
| DC_n3A_1A | 23 | +2/-3 |
| DC_n3A_8A | 23 | +2/-3 |
| DC_n8A_1A | 23 | +2/-3 |
| DC_n8A_3A | 23 | +2/-3 |
| DC_n28A_3A DC_n28A_3C | 23 | +2/-3 |
| DC_n28A_8A | 23 | +2/-3 |
| DC_n28A_20A | 23 | +2/-3 |
| DC_n28A_34A | 23 | +2/-3 |
| DC_n28A_39A | 23 | +2/-3 |
| DC_n28A_40A DC_n28A_40C | 23 | +2/-3 |
| DC_n41A_3A | 23 | +2/-3 |
| DC_n41A_8A | 23 | +2/-3 |
| DC_n41A_34A | 23 | +2/-3 |
| DC_n41A_39A | 23 | +2/-3 |
| DC_n41A_40A | 23 | +2/-3 |
| DC_n77A_1A | 23 | +2/-3 |
| DC_n77A_3A | 23 | +2/-3 |
| DC_n77A_8A | 23 | +2/-3 |
| DC_n78A_1A | 23 | +2/-3 |
| DC_n78A_3A | 23 | +2/-3 |
| DC_n78A_5A | 23 | +2/-3 |
| DC_n78A_7A | 23 | +2/-3 |
| DC_n78A_8A | 23 | +2/-3 |
| DC_n78A_26A | 23 | +2/-3 |

<< Next Changes >>

6.2B.4.2	ΔTIB,c for DC

6.2B.4.2.0	General

For the UE which supports inter-band EN-DC or NE-DC configuration, ΔTIB,c in Tables below applies where unless otherwise stated, the same ΔTIB,c is applicable to NR band(s) part for DC configurations which have the same NR operating band combination. Unless otherwise stated, ΔTIB,c is set to zero.

Unless ΔTIB,c  is specified for the NE-DC configuration, the specified ΔTIB,c  for the EN-DC configuration including same bands as the corresponding NE-DC configuration is applicable for the NE-DC configuration.

6.2B.4.2.1	Intra-band contiguous EN-DC

ΔTIB,c is not applicable for intra-band contiguous EN-DC.

6.2B.4.2.1a	Intra-band contiguous NE-DC

ΔTIB,c is not applicable for intra-band contiguous NE-DC.

6.2B.4.2.2	Intra-band non-contiguous EN-DC

ΔTIB,c is not applicable for intra-band non-contiguous EN-DC.

6.2B.4.2.3	Inter-band EN-DC within FR1

6.2B.4.2.3.1	ΔTIB,c for EN-DC two bands

Table 6.2B.4.2.3.1-1: ΔTIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_1_n3 | 0.3 | 0.3 |
| DC_1_n5 | 0.3 | 0.3 |
| DC_1_n7 DC_1-1_n7 | 0.5 | 0.6 |
| DC_1_n8 | 0.3 | 0.3 |
| DC_1_n20 | 0.3 | 0.3 |
| DC_1_n26 | 0.3 | 0.3 |
| DC_1_n28 | 0.3 | 0.6 |
| DC_1_n38 | 0.5 | 0.5 |
| DC_1_n40 | 0.5 | 0.5 |
| DC_1_n41 | 0.5 | 0.5 |
| DC_1_n50 | 0.5 | 0.5 |
| DC_1_n51 | 0.6 | 0.6 |
| DC_1_n71 | 0.3 | 0.3 |
| DC_1_n77 | 0.6 | 0.8 |
| DC_1_n78 | 0.3 | 0.8 |
| DC_2_n5 DC_2-2_n5 | 0.3 | 0.3 |
| DC_2_n7 | 0.5 | 0.5 |
| DC_2_n12 | 0.3 | 0.3 |
| DC_2_n28 | 0.3 | 0.3 |
| DC_2_n30 DC_2-2_n30 | 0.5 | 0.3 |
| DC_2_n38 DC_2-2_n38 | 0.5 | 0.9 |
| DC_2_n41 DC_2-2_n41 | 0.5 | 0.41 / 0.92 |
| DC_2_n48 | 0.6 | 0.8 |
| DC_2_n66 DC_2-2_n66 | 0.5 | 0.5 |
| DC_2_n71 DC_2-2_n71 | 0.3 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 0.6 | 0.8 |
| DC_2_n78 DC_2-2_n78 | 0.6 | 0.8 |
| DC_3_n1 | 0.3 | 0.3 |
| DC_3_n5 | 0.3 | 0.3 |
| DC_3_n8 DC_3-3_n8 | 0.3 | 0.3 |
| DC_3_n7 DC_3-3_n7 | 0.5 | 0.5 |
| DC_3_n20 | 0.3 | 0.3 |
| DC_3_n26 | 0.3 | 0.3 |
| DC_3_n28 | 0.3 | 0.3 |
| DC_3_n34 | 0.5 | 0.5 |
| DC_3_n38 | 0.5 | 0.5 |
| DC_3_n40 | 0.5 | 0.5 |
| DC_3-n41 | 0.5 | 0.33 / 0.84 |
| DC_3_n50 | 0.5 | 0.5 |
| DC_3_n51 | 0.3 | 0.3 |
| DC_3_n71 | 0.3 | 0.3 |
| DC_3_n77 DC_3-3_n77 | 0.6 | 0.8 |
| DC_3_n78 DC_3-3_n78 | 0.6 | 0.8 |
| DC_4_n2 | 0.5 | 0.5 |
| DC_4_n5 | 0.3 | 0.3 |
| DC_4_n7 | 0.5 | 0.5 |
| DC_4_n28 | 0.3 | 0.6 |
| DC_4_n38 | 0.5 | 0.8 |
| DC_4_n41 | 0.5 | 0.81 / 1.32 |
| DC_4_n78 | 0.6 | 0.8 |
| DC_5_n1 | 0.3 | 0.3 |
| DC_5_n2 DC_5-5_n2 | 0.3 | 0.3 |
| DC_5_n3 | 0.3 | 0.3 |
| DC_5_n7 | 0.3 | 0.3 |
| DC_5_n12 | 0.8 | 0.4 |
| DC_5_n30 | 0.3 | 0.3 |
| DC_5_n38 | 0.3 | 0.3 |
| DC_5_n40 | 0.3 | 0.3 |
| DC_5_n48 | 0.3 | 0.3 |
| DC_5_n66 DC_5-5_n66 | 0.3 | 0.3 |
| DC_5_n71 | 0.5 | 0.5 |
| DC_5_n77 | 0.6 | 0.8 |
| DC_5_n78 | 0.6 | 0.8 |
| DC_7_n1 DC_7-7_n1 | 0.6 | 0.5 |
| DC_7_n2 | 0.5 | 0.5 |
| DC_7_n3 | 0.5 | 0.5 |
| DC_7_n5 DC_7-7_n5 | 0.3 | 0.3 |
| DC_7_n8 DC_7-7_n8 | 0.3 | 0.6 |
| DC_7_n20 | 0.3 | 0.3 |
| DC_7_n25 | 0.5 | 0.5 |
| DC_7_n26 | 0.3 | 0.3 |
| DC_7_n28, DC_7-7_n28 | 0.3 | 0.3 |
| DC_7_n40 | 0.5 | 0.6 |
| DC_7_n51 | 0.3 | 0.3 |
| DC_7_n71 | 0.3 | 0.6 |
| DC_7_n66 DC_7-7_n66 | 0.5 | 0.5 |
| DC_7_n77 DC_7-7_n77 | 0.5 | 0.8 |
| DC_7_n78 DC_7-7_n78 | 0.5 | 0.8 |
| DC_7_n79 | 0.5 | 0.8 |
| DC_8_n1 | 0.3 | 0.3 |
| DC_8_n2 | 0.3 | 0.3 |
| DC_8_n3 | 0.3 | 0.3 |
| DC_8_n7 | 0.6 | 0.3 |
| DC_8_n20 | 0.4 | 0.4 |
| DC_8_n28 | 0.6 | 0.5 |
| DC_8_n34 | 0.3 | 0.3 |
| DC_8_n38 | 0.6 | 0.3 |
| DC_8_n39 | 0.3 | 0.3 |
| DC_8_n40 | 0.3 | 0.3 |
| DC_8_n41 | 0.3 | 0.3 |
| DC_8_n77 | 0.6 | 0.8 |
| DC_8_n78 | 0.6 | 0.8 |
| DC_11_n1 | 0.3 | 0.3 |
| DC_11_n3 | 0.8 | 0.9 |
| DC_11_n28 | 0.4 | 0.6 |
| DC_11_n41 | 0.3 | 0.3 |
| DC_11_n77 | 0.4 | 0.8 |
| DC_11_n78 | 0.4 | 0.8 |
| DC_12_n2 | 0.3 | 0.3 |
| DC_12_n5 | 0.4 | 0.8 |
| DC_12_n7 | 0.3 | 0.3 |
| DC_12_n25 | 0.3 | 0.3 |
| DC_12_n30 | 0.3 | 0.3 |
| DC_12_n38 | 0.3 | 0.3 |
| DC_12_n41 | 0.3 | 0.3 |
| DC_12_n66 | 0.8 | 0.3 |
| DC_12_n71 | 1 | 1 |
| DC_12_n77 | 0.5 | 0.8 |
| DC_12_n78 | 0.5 | 0.8 |
| DC_13_n2 | 0.3 | 0.3 |
| DC_13_n5 | 0.5 | 0.5 |
| DC_13_n7 | 0.5 | 0.5 |
| DC_13_n25 | 0.3 | 0.3 |
| DC_13_n48 | 0.3 | 0.3 |
| DC_13_n66 | 0.3 | 0.3 |
| DC_13_n71 | 0.5 | 0.5 |
| DC_13_n77 | 0.5 | 0.8 |
| DC_13_n78 | 0.5 | 0.8 |
| DC_14_n2 | 0.3 | 0.3 |
| DC_14_n5 | 0.5 | 0.5 |
| DC_14_n30 | 0.3 | 0.3 |
| DC_14_n66 | 0.3 | 0.3 |
| DC_14_n77 | 0.5 | 0.8 |
| DC_18_n3 | 0.3 | 0.3 |
| DC_18_n28 | 0.5 | 0.5 |
| DC_18_n41 | 0.3 | 0.33 |
| DC_18_n77 | 0.3 | 0.8 |
| DC_18_n78 | 0.3 | 0.8 |
| DC_19_n1 | 0.3 | 0.3 |
| DC_19_n77 | 0.3 | 0.8 |
| DC_19_n78 | 0.3 | 0.8 |
| DC_20_n1 | 0.3 | 0.3 |
| DC_20_n3 | 0.3 | 0.3 |
| DC_20_n7 | 0.3 | 0.3 |
| DC_20_n8 | 0.4 | 0.4 |
| DC_20_n28 | 0.5 | 0.5 |
| DC_20_n38 | 0.5 | 0.3 |
| DC_20_n41 | 0.3 | 0.3 |
| DC_20_n50 | 0.3 | 0.4 |
| DC_20_n51 | 0.5 | 0.5 |
| DC_20_n77 | 0.6 | 0.8 |
| DC_20_n78 | 0.6 | 0.8 |
| DC_21_n1 | 0.3 | 0.3 |
| DC_21_n28 | 0.4 | 0.3 |
| DC_21_n77 | 0.4 | 0.8 |
| DC_21_n78 | 0.4 | 0.8 |
| DC_25_n41, DC_25-25_n41 | 0.5 | 0.41 / 0.92 |
| DC_25_n77 DC_25-25_n77 | 0.6 | 0.8 |
| DC_25_n78 DC_25-25_n78 | 0.6 | 0.8 |
| DC_26_n25 | 0.3 | 0.3 |
| DC_26_n41 | 0.3 | 0.3 |
| DC_26_n77 | 0.3 | 0.8 |
| DC_26_n78 | 0.3 | 0.8 |
| DC_28_n1 | 0.6 | 0.3 |
| DC_28_n2 | 0.3 | 0.3 |
| DC_28_n3 | 0.3 | 0.3 |
| DC_28_n5 | 0.5 | 0.5 |
| DC_28_n7 | 0.3 | 0.3 |
| DC_28_n8 | 0.5 | 0.6 |
| DC_28_n20 | 0.5 | 0.5 |
| DC_28_n38 | 0.3 | 0.3 |
| DC_28_n40 | 0.3 | 0.3 |
| DC_28_n41 | 0.3 | 0.3 |
| DC_28_n50 | 0.3 | 0.4 |
| DC_28_n51 | 0.5 | 0.5 |
| DC_28_n66 | 0.6 | 0.3 |
| DC_28_n77 | 0.5 | 0.8 |
| DC_28_n78 | 0.5 | 0.8 |
| DC_30_n2 | 0.3 | 0.5 |
| DC_30_n5 | 0.3 | 0.3 |
| DC_30_n66 | 0.5 | 0.8 |
| DC_30_n77 | 0.5 | 0.8 |
| DC_38_n1 | 0.5 | 0.5 |
| DC_38_n3 | 0.5 | 0.5 |
| DC_38_n8 | 0.6 | 0.3 |
| DC_38_n28 | 0.3 | 0.3 |
| DC_38_n78 | - | 0.5 |
| DC_38_n79 | 0.3 | 0.8 |
| DC_39-n41 | 0.5 | 0.5 |
| DC_39_n78 | 0.3 | 0.8 |
| DC_39_n79 | 0.3 | 0.8 |
| DC_40_n1 | 0.5 | 0.5 |
| DC_40_n415 | 0.5 | 0.5 |
| DC_40_n77 | - | 0.5 |
| DC_40_n78 | - | 0.56 |
| DC_40_n79 | 0.3 | 0.8 |
| DC_41_n1 | 0.5 | 0.5 |
| DC_41_n3 | 0.33 / 0.84 | 0.5 |
| DC_41_n28 | 0.3 | 0.3 |
| DC_41_n77 | 0.3 | 0.8 |
| DC_41_n78 | 0.3 | 0.8 |
| DC_41_n79 | 0.3 | 0.8 |
| DC_42_n1 | 0.8 | 0.3 |
| DC_42_n3 | 0.8 | 0.6 |
| DC_42_n28 | 0.5 | 0.8 |
| DC_42_n51 | 0.6 | 0.8 |
| DC_48_n2 | 0.8 | 0.6 |
| DC_48_n5 | 0.3 | 0.3 |
| DC_48_n12 | 0.3 | 0.3 |
| DC_48_n25 | 0.8 | 0.6 |
| DC_48_n46 | 0.8 | - |
| DC_48_n66 | 0.8 | 0.6 |
| DC_48_n71 DC_48-48_n71 DC_48-48-48_n71 | 0.3 | 0.3 |
| DC_66_n2 DC_66-66_n2 DC_66-66-66_n2 | 0.5 | 0.5 |
| DC_66_n5, DC_66-66_n5, DC_66-66-66_n5 | 0.3 | 0.3 |
| DC_66_n7 DC_66-66_n7 | 0.5 | 0.5 |
| DC_66_n12 | 0.8 | 0.3 |
| DC_66_n25 | 0.5 | 0.5 |
| DC_66_n28 | 0.3 | 0.6 |
| DC_66_n30 DC_66-66_n30 | 0.5 | 0.8 |
| DC_66_n38 DC_66-66_n38 | 0.5 | 0.5 |
| DC_66_n41 | 0.5 | 0.81 / 1.32 |
| DC_66_n48 DC_66-66_n48 | 0.6 | 0.8 |
| DC_66_n71 DC_66-66_n71 | 0.3 | 0.3 |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 0.6 | 0.8 |
| DC_66_n78 DC_66-66_n78 | 0.6 | 0.8 |
| DC_71_n2 | 0.3 | 0.3 |
| DC_71_n5 | 0.5 | 0.5 |
| DC_71_n38 | 0.6 | 0.3 |
| DC_71_n41 | 0.6 | 0.3 |
| DC_71_n48 | 0.3 | 0.3 |
| DC_71_n66 | 0.3 | 0.3 |
| DC_71_n77 | 0.5 | 0.8 |
| DC_71_n78 | 0.5 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 - 2515 MHz. NOTE 5: Applicable for UE supporting inter-band EN-DC without simultaneous Rx/Tx. NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 7: “-” denotes ΔTIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

<< Next Changes >>

6.2B.4.2.3a	Inter-band NE-DC within FR1

Unless ΔTIB,c  is specified in this clause, the value of ΔTIB,c  for the correspondingly specified EN-DC configuration in clause 6.2B.4.2.3 is applicable.

Table 6.2B.4.2.3a-1: ΔTIB,c due to NE-DC(two bands)

| Inter-band NE-DC configuration |  | ΔTIB,c for NR band / E-UTRA band (dB)1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration2 |  |  |  |
| DC_n28_34 |  | 0.3 |  | 0.3 |  |
| DC_n28_39 |  | 0.3 |  | 0.3 |  |
| DC_n41_34 |  | 0.3 |  | 0.3 |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |  |  |  |

<< Next Changes >>

6.4B.2	Transmit modulation quality for DC

6.4B.2.1	Transmit modulation quality for Intra-band contiguous EN-DC

6.4B.2.1.1	Error Vector Magnitude

For the intra-band contiguous EN-DC with one component carrier per CG the EVM requirement applies with PRB allocation in one of the CG and the other CG unallocated.

The EVM requirements for each CG are according to clause 6.5.2 of TS 36.101 [4] for the MCG and 6.4.2 of TS 38.101-1 [2] for the SCG with EN-DC configured.

6.4B.2.1.2	Carrier leakage

The carrier leakage requirements for each CG are according to clause 6.5.2 of TS 36.101 [4] for the MCG and 6.4.2 of TS 38.101-1 [2] for the SCG with EN-DC configured.

6.4B.2.1.3	In-band emissions

For the MCG the in-band emission requirments in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 in TS 36.101 [4] apply within the aggregated transmission bandwidth configuration of the EN-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the MCG at the edge of the said aggregated transmission bandwidth configuration.

For the SCG the in-band emission requirements in Table 6.4.2.3-1 in TS 38.101-1 [2] apply within the aggregated transmission bandwidth configuration of the EN-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the SCG at the edge of the aggregated transmission bandwidth configuration.

6.4B.2.1a	Transmit modulation quality for Intra-band contiguous NE-DC

6.4B.2.1a.1	Error Vector Magnitude

For the intra-band contiguous NE-DC with one component carrier per CG the EVM requirement applies with PRB allocation in one of the CG and the other CG unallocated.

The EVM requirements for each CG are according to clause 6.4.2 of TS 38.101-1 [2] for the MCG and 6.5.2 of TS 36.101 [4] for the SCG with NE-DC configured.

6.4B.2.1a.2	Carrier leakage

The carrier leakage requirements for each CG are according to clause 6.4.2 of TS 38.101-1 [2] for the MCG and 6.5.2 of TS 36.101 [4] for the SCG with NE-DC configured.

6.4B.2.1a.3	In-band emissions

For the MCG the in-band emission requirements in Table 6.4.2.3-1 in TS 38.101-1 [2] apply within the aggregated transmission bandwidth configuration of the NE-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the MCG at the edge of the aggregated transmission bandwidth configuration.

For the SCG the in-band emission requirments in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 in TS 36.101 [4] apply within the aggregated transmission bandwidth configuration of the NE-DC bandwidth with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the SCG at the edge of the said aggregated transmission bandwidth configuration.

<< Next Changes >>

6.5B.3.3	Inter-band EN-DC within FR1

##### 6.5B.3.3.1	General spurious emissions

The general spurious emissions requirements specified in clause 6.6.3.1 of TS 36.101 [4], clause 6.5.3.1 of TS 38.101-1 [2] and TS 38.101-2 [3] apply for each component carrier. For the case of inter-band EN-DC with a single carrier per cell group, the general spurious emissions requirements also apply with both downlink carrier and both both uplink carriers active. Limits on configured maximum output power for the uplink according to clause 6.2B.4 apply.

NOTE:	The general spurious emission requirements with both uplink carriers active are allowed to be verified for only a single inter-band EN-DC configuration per NR band. Furthermore, the requirements are allowed to be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur.

Table 6.5B.3.3.1-1: (Void)

6.5B.3.3.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified EN-DC, for coexistence with protected bands. The requirements in Table 6.5B.3.3.2-1 apply on each component carrier with all component carriers are active.

NOTE:	For inter-band EN-DC with uplink assigned to one LTE band and one NR band the requirements in Table 6.5B.3.3.2-1 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur;

Table 6.5B.3.3.2-1: Requirements

| EN-DC Configuration |  |  | Spurious emission |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Protected band |  | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |  |
| DC_1_n3 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA band 22, 42, 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 67 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_1_n5 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3,34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA band 41, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_1_n7 |  |  | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5,16 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_1_n8 |  |  | E-UTRA Band 11, 20, 21, 28, 31, 32, 38, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1, 8, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_1_n20 |  |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 38, 42, 69 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 20, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |  |
| DC_1_n26 |  |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 31, 40, 42, 43, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 738 |  | 799 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 799 |  | 803 | -40 | 1 | 5 |  |  |
| DC_1_n28 |  |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5,16 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_1_n38 |  |  | E-UTRA Band 1, 3, 5, 8, 20, 22, 27, 28, 31, 32, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_1_n40 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | NR band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_1_n41 |  |  | E-UTRA Band 3, 4, 5, 8, 12, 13, 14, 17, 19, 20, 21, 24, 26, 27, 28, 29, 30, 31, 32, 42, 43, 44, 45, 50, 51, 52, 66, 67, 68, 71, 72, 73, 75, 76, 85 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | NR Band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 8 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 8 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 8, 20 |  |  |
|  |  |  | E-UTRA Band 11, 18, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_1_n50 |  |  | E-UTRA Band 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 29, 31, 38, 40, 41, 42, 43, 44, 48, 52, 66, 67, 68, 69, 72, 73, 85 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | NR Band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5,16 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_1_n51 |  |  | E-UTRA Band 7, 12, 13, 17, 20, 22, 27, 28, 29, 31, 38, 44, 48, 67, 68, 69, 72, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5, 2 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
|  |  |  | E-UTRA Band 5, 6, 8, 26, 30, 40, 41, 42, 43, 46 NR Band n77, n78, n79, |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_1_n71 |  |  | E-UTRA Band 1, 5, 26, |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_1_n77 DC_1_n84_ULSUP-TDM_n77 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |  |
| DC_1_n78 DC_1_n84_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |  |
| DC_1_n79 DC_1_n84_ULSUP-TDM_n79 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 26, 28, 34, 40, 41, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 8 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 8 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 8 |  |  |
| DC_1_n80 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76, NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 42, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_2_n5 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |  |
|  |  |  | E-UTRA Band 2, 25, 48 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 41, 43, 53 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_2_n7 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_2_n12 |  |  | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 53, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 25, 85 NR band n12 |  | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 4, 50, 51, 66, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_2_n28 |  |  | E-UTRA Band 5, 26, 27, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_2_n30 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 48 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_2_n38 |  |  | E-UTRA Band 4, 5, 12, 13, 14,17, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_2_n41 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 43, 48 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_2_n48 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_2_n66 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_2_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 66 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 48, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_2_n77 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_2_n78 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_3_n1 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA band 22, 42, 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5,16 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_3_n5 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3,34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 42, 52 Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_3_n7 |  |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 50, 51, 65, 67, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA band 22, 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_3_n8 |  |  | E-UTRA Band 1, 11, 20, 21, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67,68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 8 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |  |
|  |  |  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_3_n20 |  |  | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3 NR band n20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 38, 42, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |  |
| DC_3_n26 |  |  | E-UTRA Band 1, 5, 11, 18, 19, 21, 26, 31, 39, 40, 43, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41, 42 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_3_n28 |  |  | E-UTRA Band 1, 42, 43, 50, 51, 65, 74, 75, 76 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 13 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
| DC_3_n34 |  |  | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 38, 39, 40, 41, 43, 44, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76, 79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 22, 42, 52 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_3_n38 |  |  | E-UTRA Band 1, 5, 8, 20, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 22, 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_3_n40 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 43, 44. 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 42, 52 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_3_n41, DC_3_n80_ULSUP-TDM_n41 |  |  | E-UTRA Band 1, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_3_n50 |  |  | E-UTRA Band 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 27, 28, 29, 31, 38, 40, 41, 43, 44, 52, 67, 68, 69, 72, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 2, 4, 33, 34, 39, 42, 48, 65, 66 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 |  |  |  |
| DC_3_n51 |  |  | E-UTRA Band 7, 8, 12, 13, 17, 20, 27, 28, 31, 33, 38, 67, 68, 69, 72, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 1, 5, 6, 22, 26, 30, 34, 36, 40, 41, 42, 43, 44, 46, 48, 65, 71 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_3_n71 |  |  | E-UTRA Band 5, 26, |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 3, 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_3_n77 DC_3_n80_ULSUP-TDM_n77 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_3_n78 DC_3_n80_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_3_n79 DC_3_n80_ULSUP-TDM_n79 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 28, 34, 39, 40, 41, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_3_n82 |  |  | E-UTRA Band 1, 3 7, 8, 20,31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72,74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 22, 38, 42, 69 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_3_n84 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73,74, 75, 76 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_4_n2 |  |  | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 42, 43, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_4_n5 |  |  | Bands 1, 2, 3, 4, 5, 6, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85, n257 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |  |
|  |  |  | Bands 41, 42, 48, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 18, 19 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_4_n7 |  |  | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
| DC_4_n28 |  |  | E-UTRA Band 2, 5, 7, 25, 26, 27, 38, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74, NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
| DC_4_n38 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_4_n41 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42, 48 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_4_n78 |  |  | E-UTRA Band 5, 7, 26, 28, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_5_n1 |  |  | E-UTRA Band 5, 7, 8, 11, 18, 19, 21, 22, 26, 28, 31, 38, 40, 42, 43, 50, 51, 65, 73, 74 NR band n1 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA band 41, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_5_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 28, 29, 30, 42, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | NR Band n2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41, 43, 53 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_5_n3 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 21, 26, 28, 31, 38, 40, 43, 50, 51, 65, 73, 74 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 34 NR band n3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 42, 52 Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_5_n7 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 28, 29, 30, 31, 34, 40, 42, 43, 65, 66, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 26 |  | 859 | - | 869 | -27 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 14 |  |  |
| DC_5_n12 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 43, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Bands 4, 41, 42, 48, 50, 51, 66, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_5_n30 |  |  | E-UTRA Band 2, 4, 5, 7,12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41, 53 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_5_n38 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 28, 29, 30, 31, 34, 40, 42, 43, 50, 51, 65, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_5_n40 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 21, 28, 31, 34, 38, 42, 43, 45, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41, 52 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_5_n48 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 29, 30, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_5_n66 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41, 42, 48, 52, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_5_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_5_n77 |  |  | E-UTRA Band 2, 4, 12, 13, 14, 17, 25, 26, 28, 29, 30, 40, 65, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_5_n78 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 45, 65, 66, 70 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 7 |  |  |
| DC_5_n79 |  |  | Bands 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |  |
|  |  |  | Bands 41, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_7_n1 |  |  | Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76, n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5,16 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7,16 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7,16 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n2 |  |  | E-UTRA Band 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 2 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n3 |  |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA band 22, 42, 52 NR band n78, n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n5 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 26, 28, 29, 30, 31, 40, 42, 43, 50, 51, 65, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 6 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 6 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 14 |  |  |
| DC_7_n8 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 42, 43, 52 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n20 |  |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42, 52 NR band n78, n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_7_n25 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 27, 28, 29, 30, 42, 66, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 43 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 2 NR Band n25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n26 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 12, 13, 14, 17, 26, 29, 30, 31, 34, 40, 42, 43, 50, 51, 65, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_7_n28 |  |  | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 4, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR band n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n40 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 42, 43, 50, 51, 52, 65, 67, 68, 72, 74, 75, 76, 77, 78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n51 |  |  | E-UTRA Band 2, 3, 5, 8, 26, 30, 31, 32, 33, 34, 40, 48, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 1, 4, 12, 13, 14, 17, 20, 22, 23, 27, 28, 29, 42, 43, 44, 46, 65, 66, 67, 68 NR Band n77, n78, n79, |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_7_n66 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 30, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n77 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n78 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n79 |  |  | E-UTRA Band 1, 3, 5, 8, 28, 34, 40, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_7_n80 |  |  | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 42, NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_8_n1 |  |  | E-UTRA Band 20, 28, 31, 32, 38, 40, 50, 51, 65, 67, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1, 8, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_8_n2 |  |  | E-UTRA Band 2, 8, 28, 50, 51, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41, 42, 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_8_n3 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 8 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |  |
|  |  |  | E-UTRA band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |  |
|  |  |  | E-UTRA band 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3.12 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5. 12 |  |  |
| DC_8_n7 |  |  | E-UTRA Band 1, 2, 4, 5, 10, 12, 13, 14, 17, 20, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 45, 50, 51, 52, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_8_n20 |  |  | E-UTRA Band 1, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 7, 22, 38, 42, 43, 52, 69 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8, 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |  |
| DC_8_n28 |  |  | E-UTRA Band 20, 31, 34, 38, 40, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 65, 73, 74, 75, 76 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 11 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10, 12 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9, 12 |  |  |
| DC_8_n34 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 38, 39, 40, 45, 50, 51, 65, 67, 69,72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |  |
| DC_8_n38 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR Band n77, n78, n79 |  |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 22, 42, 43, 52 |  |  | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 |  |  | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range |  |  | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |
|  | Frequency range |  |  | 2645 | - | 2690 | -40 | 1 | 5, 22 |
| DC_8_n39 |  |  | E-UTRA Band 1, 28, 34, 40, 45, 50, 51, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | UTRA Band 22, 41, 42, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_8_n40 |  |  | E-UTRA Band 1, 5, 11, 18, 19, 20, 21, 26, 28, 31, 32, 33, 34, 38, 39,, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_8_n41, DC_8_n81_ULSUP-TDM_n41 |  |  | E-UTRA Band 1, 11, 21, 28, 34, 39, 45, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 42, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |  |
|  |  |  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_8_n77 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 7, 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |  |
| DC_8_n78 DC_8_n81_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 20, 28, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 7, 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 12 |  |  |
| DC_8_n79 DC_8_n81_ULSUP-TDM_n79 |  |  | E-UTRA Band 1, 8, 28, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3,41,42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_8_n80 |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 3, 7, 22, 41, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 13 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_8_93_ULSUP-TDM, DC_8_94_ULSUP-TDM |  |  | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |  |  |
|  |  |  | E-UTRA 8 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_11_n1 |  |  | E-UTRA Band 1, 11, 18, 19, 21, 27, 28, 40, 42, 65 NR Band n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_11_n3 |  |  | E-UTRA Band 1, 11, 18, 19, 21, 28, 34, 40, 65 NR band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 42 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_11_n28 |  |  | E-UTRA Band 3, 18, 19, 34, 40 NR band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 42, 65, 74 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_11_n41 |  |  | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 42, 65 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_11_n77 |  |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_11_n78 |  |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_11_n79 |  |  | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_12_n2 |  |  | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 53, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 12, 25, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 3 |  |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 4, 50, 51, 66, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_12_n5 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 43 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Bands 4, 41, 42, 48, 50, 51, 66, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_12_n66 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 25, 26, 27, 30, 41, 53, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 48, 50, 51, 66, 70 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_12_n7 |  |  | E-UTRA Band 2, 5, 7, 13, 14, 17, 26, 27, 30, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 50, 51,66 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_12_n25 |  |  | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 53, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 48, 66, 70. NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 2, 12, 25, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 15 |  |  |
| DC_12_n30 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 48, 66, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_12_n38 |  |  | E-UTRA Band 2, 5, 13. 14. 17, 27, 30, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 50, 51, 66 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_12_n41 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 4, 48, 50, 51, 66, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_12_n77 |  |  | E-UTRA Band 2, 5, 7. 13, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 66 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_12_n78 |  |  | E-UTRA Band 2, 5, 7. 13, 17, 25, 26, 41, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 66 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 12 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_13_n2 |  |  | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2,14, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 30, 48 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_13_n5 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 29, 48, 50, 51, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 26 |  | 859 | - | 869 | -27 | 1 |  |  |  |
|  |  |  | E-UTRA Band 24, 30, 41, 53 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_13_n7 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 17,25，26, 27, 29, 50, 51, 66，74, 85 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 30 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_13_n25 |  |  | E-UTRA Band 4, 5,12,13,17, 26, 29, 41, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2,14 NR Band n25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 30, 48 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_13_n48 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 24, 30 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_13_n66 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 30, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -35 | 0.00625 | 5 |  |  |
| DC_13_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 17, 26, 48, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 24, 25, 30, 41, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 14, 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_13_n77 |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 17, 25, 26, 29, 41, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 14 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 24, 30 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_14_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85, |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 2, 25, 48 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_14_n5 |  |  | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 41, 53 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_14_n30 |  |  | E-UTRA Band 2, 4, 5,12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_14_n77 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_14_n66 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85, |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 48 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_18_n28 |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 11 |  |  |
|  |  |  | E-UTRA Band 42, 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 3, 34, 40 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 799 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_18_n41 |  |  | E-UTRA Band 1, 3, 11, 19, 21, 28, 34, 42, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_13_n78 |  |  | E-UTRA Band 2, 5, 7, 12, 13, 25, 26, 41, 66 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 769 | - | 775 | -35 | 0.00625 | 5 |  |  |
|  |  |  | Frequency range |  | 799 | - | 805 | -35 | 0.00625 | 5 |  |  |
| DC_18_n3 |  |  | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 65 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_18_n77 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_18_n78 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_18_n79 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_19_n1 |  |  | E-UTRA Band 1, 11, 21, 28, 40, 42, 65, 74 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_19_n77 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_19_n78 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_19_n79 |  |  | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_20_n1 |  |  | E-UTRA Band 1, 3, 7, 8, 20, 22, 31, 32, 34, 40, 43, 50, 51, 65, 67, 68, 72, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 38, 42, 69 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |  |
| DC_20_n3 |  |  | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 20 E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 38, 42, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |  |
| DC_20_n7 |  |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42, 52 NR band n78, n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_20_n8 |  |  | E-UTRA Band 1, 28, 31, 32, 34, 65, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 7, 22, 38, 42, 43 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_20_n38 |  |  | E-UTRA Band 1, 3, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42, 52 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_20_n41 |  |  | E-UTRA Band 1, 2, 4, 24, 25, 30, 31, 32, 33, 34, 39, 43, 48, 50, 51, 65, 66, 70, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 8, 12, 13, 14, 17, 42, 44, 45, 52, 67, 68, 71, 85 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 9, 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 19 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 19 |  |  |
| DC_20_n28 DC_20_n83 |  |  | E-UTRA Band 3, 7, 8, 31, 34 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_20_n50 |  |  | E-UTRA Band 2, 3, 7, 12, 17, 31, 33, 39, 43, 48, 65, 67, 68, 72, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 4, 5, 8, 13, 34, 38, 40, 41, 42, 52, 69 NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |  |
| DC_20_n51 |  |  | E-UTRA Band 1, 3, 4, 8, 17, 22, 28, 29, 31, 40, 43, 48, 65, 66, 68, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 7, 25, 32, 33, 34, 35, 36, 37, 38, 39, 41, 42, 46, 69, 70 NR Band n77, n78, n79, |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_20_n77 |  |  | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 38, 69 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_20_n78, DC_20_n82_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 3, 7, 8, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 38, 69 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_20_n80 |  |  | E-UTRA Band 1, 7, 8, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76. NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 42, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_20A_91A_ULSUP-TDM, DC_20A_92A_ULSUP-TDM |  |  | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 20 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 38, 42, 69, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 758 | - | 788 | -50 | 1 |  |  |  |
| DC_21_n1 |  |  | E-UTRA Band 1, 18, 19, 28, 40, 42, 65 NR Band n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_21_n28 |  |  | E-UTRA Band 1, 42, 65, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 3, 18, 19, 34, 40 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
| DC_21_n77 |  |  | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_21_n78 |  |  | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_21_n79 |  |  | E-UTRA Band 1, 3, 18, 19, 21, 28, 34, 40, 42, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 2545 | - | 2575 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2645 | -50 | 1 |  |  |  |
| DC_25_n41 |  |  | E-UTRA Band 4, 5, 12, 13 , 14, 17, 24, 26, 27, 28, 29, 30, 42, 45, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 48 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_25_n77 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 29, 30, 41, 65, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_25_n78 |  |  | E-UTRA Band 2, 5, 7, 12, 13, 25, 26, 28, 41, 66 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_26_n25 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 42, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 41, 43, 48, 53 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_26_n41 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13 , 14, 17, 18, 19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 42, 43, 48, 50, 51, 65, 66, 70, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |  |
|  |  |  | Frequency range |  | 703 | - | 799 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
| DC_26_n77 |  |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 703 | - | 799 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_26_n78 |  |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 703 | - | 799 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_26_n79 |  |  | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 703 | - | 799 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 799 | - | 803 | -40 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 945 | - | 960 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_28_n1 |  |  | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 38, 40, 41, 72, 73 NR band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 22, 32, 42, 43, 50, 51, 52, 65, 74, 75, 76 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5,16 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_28_n2 |  |  | E-UTRA Band 5, 26, 27, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 1, 65 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_28_n3 |  |  | E-UTRA Band 1, 22, 42, 43, 50, 51, 65, 74, 75, 76, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 40, 41, 72, 73 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
| DC_28_n5 |  |  | E-UTRA Band 2, 3, 5, 7, 8, 14, 18, 19, 24, 25, 26, 30, 31, 34, 38, 40, 70, 71 |  | FDL_low | - | FDL_high | -50 |  |  |  |  |
|  |  |  | E-UTRA Band 4, 22, 32, 41, 42, 43, 45, 48, 50, 51, 52, 65, 66, 73, 74, 75, 76 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 11 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
| DC_28_n7 |  |  | E-UTRA Band 2, 3, 5, 8, 20, 26, 27, 31, 34, 40, 72 NR band n7 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76 NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_28_n8 |  |  | E-UTRA Band 20, 31, 34, 38, 40, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 3, 7, 22, 41, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
| DC_28_n20 |  |  | E-UTRA Band 3, 7, 8, 31, 34 | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 2 |  |
| DC_28_n38 | E-UTRA Band 2, 3, 5, 8, 20, 27, 31, 34, 40, 52, 72 NR Band n79 |  |  | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n77, n78 |  |  | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 |  |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range |  |  | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range |  |  | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range |  |  | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |
|  | Frequency range |  |  | 2645 | - | 2690 | -40 | 1 | 5, 22 |
| DC_28_n40 |  |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 27, 31, 34, 38, 41, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 1, 11, 21, 22, 32, 42, 43, 50, 51, 52, 65, 73, 74, 75, 76 NR Band, n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_28_n41 DC_28_n83_ULSUP-TDM_n41 |  |  | E-UTRA Band 4, 14, 18, 19, 20, 26, 27, 39, 42, 43, 48, 50, 51, 52, 65, 66, 71, 73 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 11 |  |  |
|  |  |  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 70, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 11, 21, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
| DC_28_n50 |  |  | E-UTRA Band 4, 40, 42, 43, 48, 65, 66, 73 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |  |
|  |  |  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 25, 26, 27, 31, 34, 38, 39, 41, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
| DC_28_n51 |  |  | E-UTRA Band 2, 3, 5, 7, 8, 25, 26, 31, 34, 38, 40, 41, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 20, 22, 24, 32, 42, 43, 45, 46, 65, 66, 71, 73 NR band n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 2, 9, 10 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
| DC_28_n66 |  |  | E-UTRA Band 2, 5, 7, 25, 26, 27, 38, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74, NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
| DC_28_n77 |  |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 |  |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
| DC_28_n78 DC_28_n83_ULSUP-TDM_n78 |  |  | E-UTRA Band 3, 5, 7, 8, 18, 19, 20, 26, 34, 39, 40, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 |  |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
| DC_28_n79 |  |  | E-UTRA Band 3, 5, 8, 18, 19, 34, 39, 40, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 |  |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
| DC_30_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 43, 48 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_30_n5 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 29, 30, 38, 48, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41, 53 NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_30_n66 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_30_n77 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_38_n1 |  |  |  | E-UTRA Band 1, 3, 5, 8, 20, 22, 27, 28, 31, 32, 34, 40, 42, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_38_n3 |  |  |  | E-UTRA Band 1, 5, 8, 20, 27, 28, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  |  | E-UTRA Band 22, 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_38_n8 |  |  |  | E-UTRA Band 1, 2, 4, 12, 13, 14, 17 20, 27, 28, 29, 30, 31, 32, 33, 34, 39, 40, 45, 50, 51, 65, 66, 67, 68, 72, 73, 74, 75, 76, 85, 87, 88 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  |  | E-UTRA band 3, 22, 42, 43, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  |  | E-UTRA Band 8 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  |  | E-UTRA Band 11, 21 |  | FDL_low | - | FDL_high | -50 | 1 | 12 |  |  |
|  |  |  |  | Frequency range |  | 860 | - | 890 | -40 | 1 | 5, 12 |  |  |
|  |  |  |  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 3, 12 |  |  |
|  |  |  |  | Frequency range |  | 2620 | - | 2645 | -15.5 | 5 | 2, 5, 7, 22 |  |  |
|  |  |  |  | Frequency range |  | 2645 | - | 2690 | -40 | 1 | 2, 5, 22 |  |  |
| DC_38_n28 |  |  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 52, 65, 66, 74, 75, 76, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_38_n78 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_38_n79 |  |  | E-UTRA Band 1, 3, 5, 8, 28, 34, 40, 42, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |  |  |
|  |  |  | Frequency range |  | 2645 | - | 2690 | -40 | 1 | 5, 22 |  |  |
| DC_39_n40 |  |  | E-UTRA Band 1, 8, 22, 26, 28, 34, 41, 42, 44, 45, 50, 51, 52, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1805 |  | 1855 | -40 | 1 | 18 |  |  |
|  |  |  | Frequency range |  | 1855 |  | 1880 | -15.5 | 5 | 5, 7, 18 |  |  |
| DC_39_n41 |  |  | E-UTRA Band 1, 8, 26, 28, 34, 42, 44, 45, 50, 51, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1805 | - | 1855 | -40 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1855 | - | 1880 | -15.5 | 5 | 5, 7, 19 |  |  |
| DC_39_n78 |  |  | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1805 | - | 1855 | -40 | 1 | 18 |  |  |
|  |  |  | Frequency range |  | 1855 | - | 1880 | -15.5 | 5 | 18 |  |  |
| DC_39_n79 |  |  | E-UTRA Band 1, 8, 28, 34, 40, 41, 44, 45 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1805 | - | 1855 | -40 | 1 | 18 |  |  |
|  |  |  | Frequency range |  | 1855 | - | 1880 | -15.5 | 5 | 18 |  |  |
| DC_40_n1 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | NR Band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_40_n41 |  |  | Bands 1, 3, 5, 8, 11, 18, 19, 21, 26, 27, 28, 34, 39, 42, 44, 45, 50, 51, 65, 73, 74, NR Band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_40_n77 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_40_n78 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_40_n79 |  |  | Bands 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 41, 42, 65, 74 NR band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_41_n1 |  |  | E-UTRA Band 1, 5, 8, 11, 18, 19, 21, 26, 27, 28, 40, 42, 44, 45, 50, 51, 52, 65, 73, 74 NR Band n78 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | NR Band n77, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_41_n3 |  |  | E-UTRA Band 1, 5, 8, 26, 27, 28, 34, 39, 44, 45, 50, 51, 65, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 42, 52 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_41_n28 |  |  | E-UTRA Band 4, 14, 18, 19, 20, 26, 27, 39, 42, 43, 48, 50, 51, 52, 65, 66, 71, 73 NR Band n77, n78, n79 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 1 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |  |  |
|  |  |  | E-UTRA Band 2, 3, 5, 8, 24, 25, 30, 31, 34, 70, 72 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 11, 21, 74, 75, 76 |  | FDL_low | - | FDL_high | -50 | 1 | 9, 10 |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |  |  |
| DC_41_n77 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 33, 34, 39, 44, 45, 73, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_41_n78 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 39, 44, 45, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_41_n79 |  |  | E-UTRA Band 1, 3, 5, 8, 11, 18, 19, 21, 26, 28, 34, 42, 44, 45, 65, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 40 |  | FDL_low | - | FDL_high | -40 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_42_n1 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3, 34 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1880 | - | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 | - | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 | - | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_42_n3 |  |  | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 67, 68, 69, 72, 73, 74, 75, 76 NR Band n79 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 3 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_42_n51 |  |  | E-UTRA Band 3, 8, 20, 25, 30, 31, 34, 39, 41, 73 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 1, 2, 4, 5, 6, 7, 12, 13, 14, 17, 23, 24, 26, 27, 28, 29, 32, 38, 40, 44, 46, 65, 66, 67, 68, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_42_n77 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_42_n78 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_42_n79 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_48_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_48_n5 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_48_n12 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 30, 41, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 50, 51, 66, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_48_n25 |  |  | E-UTRA Band 4, 5, 12, 13 , 14, 17, 24, 26, 29, 30, 41, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | Frequency range |  | 1884.5 | - | 1915.7 | -41 | 0.3 |  |  |  |
|  |  |  | Frequency range |  | 1880 |  | 1895 | -40 | 1 | 5, 16 |  |  |
|  |  |  | Frequency range |  | 1895 |  | 1915 | -15.5 | 5 | 5, 7, 16 |  |  |
|  |  |  | Frequency range |  | 1915 |  | 1920 | +1.6 | 5 | 5, 7, 16 |  |  |
| DC_48_n66 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_48_n71 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 50, 51, 53, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_48_n77 |  |  | N/A |  |  |  |  |  |  |  |  |
| DC_66_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 22, 42, 43, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_66_n5 |  |  | E-UTRA Band 1, 2, 3, 4, 5, 6, 7, 8, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 38, 40, 43, 45, 50, 51, 65, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41, 42, 48, 52, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_66_n7 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 2570 | - | 2575 | +1.6 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |  |  |
|  |  |  | Frequency range |  | 2595 | - | 2620 | -40 | 1 | 5, 6 |  |  |
| DC_66_n12 |  |  | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 50, 51, 66, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 12, 85 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_66_n25 |  |  | E-UTRA Band 4, 5, 7, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 38, 41, 50, 51, 53, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 25 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 43 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_66_n28 |  |  | E-UTRA Band 2, 5, 7, 25, 26, 27, 38, 41 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 4, 10, 42, 43, 50, 51, 66, 74, NR band n77, n78 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 470 | - | 694 | -42 | 8 | 5, 17 |  |  |
|  |  |  | Frequency range |  | 470 | - | 710 | -26.2 | 6 | 14 |  |  |
|  |  |  | Frequency range |  | 662 | - | 694 | -26.2 | 6 | 5 |  |  |
|  |  |  | Frequency range |  | 758 | - | 773 | -32 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 773 | - | 803 | -50 | 1 |  |  |  |
| DC_66_n30 |  |  | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 53, 66, 70, 71, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_66_n41 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 43, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42, 48, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_66_n38 |  |  | EUTRA 2, 4, 5, 12, 13,14,17, 25, 27, 28, 29, 30, 43, 50, 51, 66, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 42 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | Frequency range |  | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |  |  |
|  |  |  | Frequency range |  | 2645 | - | 2690 | -40 | 1 | 5, 22 |  |  |
| DC_66_n48 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_66_n71 |  |  | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 7, 22, 25, 41, 42, 48, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_66_n77 |  |  | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_66_n78, DC_66_n86_ULSUP-TDM_n78 |  |  | E-UTRA Band 1, 3, 5, 7, 8, 20, 26, 28, 34, 39, 40, 41, 65 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
| DC_71_n2 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 25, 41, 48, 70, NR Band n2, n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |  |
| DC_71_n5 |  |  | E-UTRA Band 4, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 NR Band n5 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_71_n38 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 30, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 29 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_71_n41 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 66, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA band 2, 25, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |  |
| DC_71_n48 |  |  | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 50, 51, 66, 71, 74, 85 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 25, 41, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| DC_71_n66 |  |  | E-UTRA Band 4, 5, 13, 14, 17, 24, 26, 27, 29, 30, 43, 50, 51, 66, 74 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 7, 22, 25, 41, 42, 48, 70, NR Band n77 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA Band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
| DC_71_n77 |  |  | E-UTRA Band 1, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 30, 39, 40, 44, 45, 50, 51, 53, 65, 66, 73, 74, 85, 103 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 2, 7, 25, 41, 34, 70 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
|  |  |  | E-UTRA band 71 |  | FDL_low | - | FDL_high | -50 | 1 | 5 |  |  |
|  |  |  | E-UTRA Band 29 |  | FDL_low | - | FDL_high | -38 | 1 | 5 |  |  |
|  |  |  | Frequency range |  | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |  |  |
| DC_71_n78 |  |  | E-UTRA Band 5, 26 |  | FDL_low | - | FDL_high | -50 | 1 |  |  |  |
|  |  |  | E-UTRA Band 41 |  | FDL_low | - | FDL_high | -50 | 1 | 2 |  |  |
| NOTE 1: FDL_low and FDL_high refer to each frequency band specified in Table 5.5-1 in TS 36.101 [4] or in Table 5.2-1 in TS 38.101-1 [2]. NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void NOTE 16: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: This requirement is only applicable for E-UTRA carriers with bandwidth confined within 1885 - 1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for E-UTRA carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for E-UTRA carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 19: Void NOTE 20: Void. NOTE 21: Void NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	To simplify the above Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

<< Next Changes >>

6.5B.5	Transmit intermodulation for DC

6.5B.5.1	Intra-band contiguous EN-DC

Unless otherwise stated, no transmit intermodulation requirements are applied for intra band contiguous EN DC.

6.5B.5.1a	Intra-band contiguous NE-DC

Unless otherwise stated, no transmit intermodulation requirements are applied for intra band contiguous NE-DC.

<< Next Changes >>

7.3B.2	Reference sensitivity for DC

7.3B.2.1	Intra-band contiguous EN-DC

For intra-band contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels. The reference sensitivity requirements apply with all uplink carriers and all downlink carriers active for EN-DC configuration and Uplink EN-DC configuration listed in Table 5.5B.2-1 and Table 5.5B.3-1, as supported by the UE. For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements apply with single uplink transmission. The downlink carrier(s) from the cell group with uplink shall be configured closer to the uplink operating band than any of the downlink carriers from the cell group without uplink.

Sensitivity degradation is allowed for Intra-band contiguous EN-DC configurations listed in Table 7.3B.2.1-1 the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3B.2.1-1 and E-UTRA and NR single carrier requriements do not apply.

Table 7.3B.2.1-1: Reference sensitivity (MSD) for intra-band contiguous EN-DC

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | [10 (RBstart = 150)] | 1865.0 | N/A |  |
| DC_(n)5AA | 5 | 826.5 | 5 | N/A | 871.5 | 5.2 | FDD |
|  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | 0 |  |
| DC_(n)5AA | 5 | 829 | 10 | N/A | 874 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | 0 |  |
| DC_(n)5AA | 5 | 844 | 10 | 25 (RBend = 49) | 889 | 0 |  |
|  | n5 | 831.5 | 15 | N/A | 876.5 | 3.1 |  |
| DC_(n)5AA | 5 | 831.5 | 5 | N/A | 876.5 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | 0 |  |
| DC_(n)5AA | 5 | 846.5 | 5 | 25 | 891.5 | 0 |  |
|  | n5 | 836.5 | 15 | N/A | 881.5 | 1 |  |
| DC_(n)5AA | 5 | 834 | 10 | N/A | 879 | 1.5 |  |
|  | n5 | 844 | 10 | 25 (RBend = 51) | 889 | 0 |  |
| DC_(n)5AA | 5 | 844 | 10 | 25 (RBend = 49) | 889 | 0 |  |
|  | n5 | 834 | 10 | N/A | 879 | 1.4 |  |
| DC_(n)12AA | 12 | 703.5 | 5 | N/A | 733.5 | 4.5 | FDD |
|  | n12 | 711 | 10 | 20 (RBend = 51) | 741 | 0 |  |
| DC_(n)12AA | 12 | 711 | 10 | 20 (RBend = 49) | 741 | 0 |  |
|  | n12 | 703.5 | 5 | N/A | 733.5 | 4.5 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | [21.3] | FDD |
|  | n71 | 673.0 | 20 | [5 (RBstart = 0)] | 627.0 | [0] |  |
| DC_(n)71AA3 | 71 | N/A | 15 | N/A | 639.5 | [5.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0)] | 624.5 | [0] |  |
| DC_(n)71AA4 | 71 | 680.5 | 5 | N/A | 639.5 | [6.8] |  |
|  | n71 | 670.5 | 151 | [5 (RBstart = 2)] | 627.0 | [0] |  |
| DC_(n)71AA5 | 71 | 680.5 | 5 | N/A | 634.5 | [6.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0) | 624.5 | [0] |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

7.3B.2.1a	Intra-band contiguous NE-DC

For intra-band contiguous NE-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels. The reference sensitivity requirements apply with all uplink carriers and all downlink carriers active for NE-DC configuration and Uplink NE-DC configuration listed in Table 5.5B.2a-1, as supported by the UE. For NE-DC configurations where uplink is not available in either the MCG or the SCG or for NE-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements apply with single uplink transmission. The downlink carrier(s) from the cell group with uplink shall be configured closer to the uplink operating band than any of the downlink carriers from the cell group without uplink.

Unless otherwise stated, for the intra-band contiguous NE-DC configurations that have an intra-band contiguous EN-DC defined configuration, sensitivity degradation is allowed if the corresponding intra-band contiguous EN-DC configuration is listed in Table 7.3B.2.1-1, and the requirement in Table 7.3B.2.1-1 applies.

<< Next Changes >>

7.3B.2.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) are specified in Table 7.3B.2.3.2-1 with uplink configuration of the agressor band (high) specified in Table 7.3B.2.3.2-2.

Table 7.3B.2.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n2 | 714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| 7 | n264,12 | [2.0] |  |  |  |  |  |  |  |  |  |  |
| n38 | 59 | N/A | N/A |  |  |  |  |  |  |  |  |  |
| n40 | 284 | 37.8 | 34.8 | 33 | 30.3 |  |  |  |  |  |  |  |
| n41 | 189 | N/A | N/A | N/A |  |  |  |  |  |  |  |  |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | 6.1 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |
| n77 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n78 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 78 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 122 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 132 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 142 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n77 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 25 | [6.1] | [5.0] | [4.0] | [3.7] |  |  |  |  |  |  |  |
| n77 | 282 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |
| n7711 | 292 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n78 | 408 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n78 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n79 | 114 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 192 | 29.5 | 26.5 | 24.7 |  |  |  |  |  |  |  |  |
| n79 | 214 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 262 | 27 | 24 | 22.2 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case. NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. NOTE 12: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-1a: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 141 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n77 | 121 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n772 | 291 | 34 | 31 |  |  |  |  |  |  |  |  |  |
| n78 | 282 | 31 | 28 | 26.2 | 25 |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 2: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n71 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| 2 | n71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n2 | 71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| 7 | n26 | 15 |  |  |  | 256 |  |  |  |  |  |  |  |
| n40 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n41 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n77 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n78 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 7 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n77 | 12 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 13 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 14 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 25 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n77 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n78 | 29 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n78 | 40 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n79 | 11 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [4] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 4: Unless otherwise stated, the UL resource blocks allocation is applied at the center of the channel bandwidth. The note applies to the entire table. NOTE 5: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. NOTE 6: RBstart = 75 |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< Next Changes >>

7.3B.2.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

7.3B.2.3.5.0	General

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex I the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [4] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [4] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [4], with parameters specified in Table 7.3B.2.3.5.1-1, Table 7.3B.2.3.5.1-1a, Table 7.3B.2.3.5.2-0 and Table 7.3B.2.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration |  | EUTRA or NR band |  |  | UL Fc (MHz) |  |  | UL/DL BW (MHz) |  |  | UL LCRB | DL Fc (MHz) |  | MSD (dB) |  |  | IMD order |  |
| DC_1_n3 |  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 | 2140 |  | 23 |  |  | IMD3 |  |
|  |  | n3 |  |  | 1760 |  |  | 5 |  |  | 25 | 1855 |  | N/A |  |  | N/A |  |
| DC_1C_n3 |  | 1C |  |  | 1950 1970 |  |  | 20 20 |  |  | 1 (RBstart=0) 1 (RBstart=67) | 2140 2160 |  | N/A |  |  | N/A |  |
|  |  | n3 |  |  | N/A |  |  | 5 |  |  | N/A | 1877.5 |  | 36 |  |  | IMD5 |  |
| DC_1A_n8A |  | 1 |  |  | 1965 |  |  | 5 |  |  | 25 | 2155 |  | 6.0 |  |  | IMD4 |  |
|  |  | n8 |  |  | 887.5 |  |  | 5 |  |  | 25 | 932.5 |  | N/A |  |  | N/A |  |
| DC_1A_n71A DC_1A_n71B |  | 1 |  |  | 1958 |  |  | 5 |  |  | 25 | 2148 |  | N/A |  |  | N/A |  |
|  |  | n71 |  |  | 668 |  |  | 5 |  |  | 25 | 622 |  | 15.1 |  |  | IMD3 |  |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), |  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 | 2140 |  | 29.8 |  |  | IMD23 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 32.54 |  |  |  |  |
|  |  | n77 |  |  | 4090 |  |  | 10 |  |  | 50 | 4090 |  | N/A |  |  | N/A |  |
| DC_1A_n77A, DC_1A_SUL_n77A-n84A, DC_1A_n77(2A), DC_1A_n77(3A), DC_1A_n78A, DC_1A_SUL_n78A-n84A, DC_1A_n78(2A) DC_1A_n78(A-C) |  | 1 |  |  | 1950 |  |  | 5 |  |  | 25 | 2140 |  | 8.0 |  |  | IMD43 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |
|  |  | n77, n78 |  |  | 3710 |  |  | 10 |  |  | 50 | 3710 |  | N/A |  |  | N/A |  |
| DC_2A_n46A |  | 2 |  |  | 1880 |  |  | 5 |  |  | 25 | 1960 |  | 12.0 |  |  | IMD3 |  |
|  |  | n46 |  |  | 5720 |  |  | 20 |  |  | 100 | 5720 |  | N/A |  |  | N/A |  |
| DC_2A_n48A |  | 2 |  |  | 1852.5 |  |  | 5 |  |  | 25 | 1932.5 |  | 12 |  |  | IMD4 |  |
|  |  | n48 |  |  | 3625 |  |  | 20 |  |  | 100 | 3625 |  | N/A |  |  | N/A |  |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) |  | 2 |  |  | 1855 |  |  | 5 |  |  | 25 | 1935 |  | 20 |  |  | IMD3 |  |
|  |  | n66 |  |  | 1775 |  |  | 5 |  |  | 25 | 2175 |  | N/A |  |  | N/A |  |
| DC_2A_n66A, DC_2A-2A_n66A DC_2A_n66(2A) |  | 2 |  |  | 1883.3 |  |  | 5 |  |  | 25 | 1963.3 |  | N/A |  |  | N/A |  |
|  |  | n66 |  |  | 1750 |  |  | 5 |  |  | 25 | 2150 |  | 4 |  |  | IMD5 |  |
| DC_2A_n77A DC_2A_n77(2A) DC_2A-2A_n77A DC_2A_n77(2A) DC_2A-2A_n77(2A) |  | 2 |  |  | 1855 |  |  | 5 |  |  | 25 | 1935 |  | 26 |  |  | IMD2 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |  |  |
|  |  | n77 |  |  | 3790 |  |  | 10 |  |  | 50 | 3790 |  | N/A |  |  | N/A |  |
|  |  | 2 |  |  | 1900 |  |  | 5 |  |  | 25 | 1980 |  | 8.0 |  |  | IMD4 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |
|  |  | n77 |  |  | 3720 |  |  | 10 |  |  | 50 | 3720 |  | N/A |  |  | N/A |  |
|  |  | 2 |  |  | 1885 |  |  | 5 |  |  | 25 | 1965 |  | 5 |  |  | IMD5 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7.74 |  |  |  |  |
|  |  | n77 |  |  | 3810 |  |  | 10 |  |  | 50 | 3810 |  | N/A |  |  | N/A |  |
| DC_2A_n78A DC_2A_n78(2A) DC_2A-2A_n78(2A) |  | 2 |  |  | 1855 |  |  | 5 |  |  | 25 | 1935 |  | 26 |  |  | IMD23 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |  |  |
|  |  | n78 |  |  | 3790 |  |  | 10 |  |  | 50 | 3790 |  | N/A |  |  | N/A |  |
|  |  | 2 |  |  | 1885 |  |  | 5 |  |  | 25 | 1965 |  | 8.0 |  |  | IMD43 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |
|  |  | n78 |  |  | 3690 |  |  | 10 |  |  | 50 | 3690 |  | N/A |  |  | N/A |  |
| DC_3_n1 |  | 3 |  |  | 1760 |  |  | 5 |  |  | 25 | 1855 |  | N/A |  |  | N/A |  |
|  |  | n1 |  |  | 1950 |  |  | 5 |  |  | 25 | 2140 |  | 23 |  |  | IMD3 |  |
| DC_3_n5 |  | 3 |  |  | 1771 |  |  | 10 |  |  | 50 | 1866 |  | 4 |  |  | IMD4 |  |
|  |  | n5 |  |  | 838 |  |  | 5 |  |  | 25 | 883 |  | N/A |  |  | N/A |  |
|  |  | 3 |  |  | 1721 |  |  | 10 |  |  | 50 | 1816 |  | N/A |  |  | N/A |  |
|  |  | n5 |  |  | 838 |  |  | 5 |  |  | 25 | 883 |  | 24 |  |  | IMD23 |  |
| DC_3A_n7A DC_3C_n7A |  | 3 |  |  | 1730 |  |  | 5 |  |  | 25 | 1825 |  | N/A |  |  | N/A |  |
|  |  | n7 |  |  | 2535 |  |  | 10 |  |  | 50 | 2655 |  | 10.2 |  |  | IMD4 |  |
| DC_3_n8 |  | n8 |  |  | 900 |  |  | 5 |  |  | 25 | 945 |  | 8 |  |  | IMD43 |  |
|  |  | 3 |  |  | 1755 |  |  | 10 |  |  | 50 | 1850 |  | N/A |  |  | N/A |  |
|  |  | n8 |  |  | 897.5 |  |  | 5 |  |  | 25 | 942.5 |  | N/A |  |  | N/A |  |
|  |  | 3 |  |  | 1747.5 |  |  | 10 |  |  | 50 | 1842.5 |  | 6.4 |  |  | IMD5 |  |
| DC_3A_n20A |  | 3 |  |  | 1775 |  |  | 5 |  |  | 25 | 1870 |  | 4 |  |  | IMD4 |  |
| DC_3C_n20A |  | n20 |  |  | 840 |  |  | 5 |  |  | 25 | 799 |  | N/A |  |  | N/A |  |
|  |  | 3 |  |  | 1735 |  |  | 5 |  |  | 25 | 1830 |  | N/A |  |  | N/A |  |
|  |  | n20 |  |  | 847 |  |  | 5 |  |  | 25 | 806 |  | 9 |  |  | IMD4 |  |
| DC_3A_n26A |  |  | 3 |  |  | 1771 |  |  | 10 | 50 |  |  | 1866 |  | 4 | IMD4 |  |  |
|  |  |  | n26 |  |  | 838 |  |  | 5 | 25 |  |  | 883 |  | N/A | N/A |  |  |
|  |  |  | 3 |  |  | 1721 |  |  | 10 | 50 |  |  | 1816 |  | N/A | N/A |  |  |
|  |  |  | n26 |  |  | 838 |  |  | 5 | 25 |  |  | 883 |  | 24 | IMD23 |  |  |
| DC_3C_n26A |  | 3 |  |  | 1720 |  |  | 20 |  |  | 1 (RBSTART=0) | 1815 |  | N/A |  |  | N/A |  |
|  |  |  |  |  | 1739.8 |  |  | 20 |  |  | 1 (RBSTART=99) | 1834.8 |  | N/A |  |  |  |  |
|  |  |  |  | n26 |  |  | 841.5 |  | 15 | 25(RBSTART=54) |  |  | 886.5 |  | 18.9 | IMD3 |  |  |
| DC_3C_n28A |  |  | n28 |  |  | 715.5 |  |  | 25 | 25(RBSTART=108) |  |  | 770.5 |  | 11 | 1st order triple beat α (TX22TX1) i.e. IMD3 |  |  |
|  |  |  | 3 |  |  | 1720 |  |  | 20 | 1 (RBSTART=0) |  |  | 1815 |  | N/A | N/A |  |  |
| DC_3A_n38A |  | 3 |  |  | 1712.8 |  |  | 5 |  |  | 25 | 1807.8 |  | 8.2 |  |  | IMD4 |  |
|  |  | n38 |  |  | 2616.7 |  |  | 10 |  |  | 50 | 2616.7 |  | N/A |  |  | N/A |  |
| DC_3A_n41A DC_3C_n41A DC_3A_SUL_n41A-n80A, DC_3C_SUL_n41A-n80A |  | 3 |  |  | 1740 |  |  | 5 |  |  | 25 | 1835 |  | 8.2 |  |  | IMD4 |  |
|  |  | n41 |  |  | 2657.5 |  |  | 10 |  |  | 50 | 2657.5 |  | N/A |  |  | N/A |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_n77(3A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3A_n78(A-C) DC_3C_n78A DC_3C_n78(2A) |  | 3 |  |  | 1740 |  |  | 5 |  |  | 25 | 1835 |  | 26 |  |  | IMD23 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |  |  |
|  |  | n77, n78 |  |  | 3575 |  |  | 10 |  |  | 50 | 3575 |  | N/A |  |  | N/A |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) |  | 3 |  |  | 1765 |  |  | 5 |  |  | 25 | 1860 |  | 8.0 |  |  | IMD43 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |
|  |  | n77, n78 |  |  | 3435 |  |  | 10 |  |  | 50 | 3435 |  | N/A |  |  | N/A |  |
| DC_4A_n2A |  | 2 |  |  | 1860 |  |  | 20 |  |  | 502 | 1940 |  | 5 |  |  | IMD3 |  |
|  |  | 4 |  |  | 1752.5 |  |  | 5 |  |  | 25 | 2152.5 |  | N/A |  |  | N/A |  |
|  |  | 2 |  |  | 1868.3 |  |  | 5 |  |  | 25 | 1948.3 |  | N/A |  |  | N/A |  |
|  |  | 4 |  |  | 1735 |  |  | 5 |  |  | 25 | 2135 |  | 5 |  |  | IMD5 |  |
| DC_4A_n5A |  | n5 |  |  | 838 |  |  | 5 |  |  | 25 | 883 |  | 30 |  |  | IMD23 |  |
|  |  | 4 |  |  | 1721 |  |  | 5 |  |  | 25 | 2121 |  | N/A |  |  | N/A |  |
| DC_4A_n7A |  | 4 |  |  | 1730 |  |  | 5 |  |  | 25 | 2130 |  | N/A |  |  | N/A |  |
|  |  | n7 |  |  | 2535 |  |  | 10 |  |  | 50 | 2655 |  | 15 |  |  | IMD4 |  |
| DC_5A_n3A |  | 5 |  |  | 838 |  |  | 5 |  |  | 25 | 883 |  | N/A |  |  | N/A |  |
|  |  | n3 |  |  | 1771 |  |  | 10 |  |  | 50 | 1866 |  | 4 |  |  | IMD4 |  |
|  |  | 5 |  |  | 838 |  |  | 5 |  |  | 25 | 883 |  | 24 |  |  | IMD23 |  |
|  |  | n3 |  |  | 1721 |  |  | 10 |  |  | 50 | 1816 |  | N/A |  |  | N/A |  |
| DC_5_n7 |  | n7 |  |  | 2547 |  |  | 10 |  |  | 50 | 2667 |  | N/A |  |  | N/A |  |
|  |  | 5 |  |  | 834 |  |  | 5 |  |  | 25 | 879 |  | 12 |  |  | IMD33 |  |
| DC_5_n38 |  | 5 |  |  | 844 |  |  | 5 |  |  | 25 | 889 |  | 12 |  |  | IMD33 |  |
|  |  | n38 |  |  | 2577 |  |  | 10 |  |  | 50 | 2577 |  | N/A |  |  | N/A |  |
| DC_5A_n66A |  | 5 |  |  | 838 |  |  | 5 |  |  | 25 | 883 |  | 30 |  |  | IMD23 |  |
|  |  | n66 |  |  | 1721 |  |  | 5 |  |  | 25 | 2121 |  | N/A |  |  | N/A |  |
| DC_5A_n77A8 DC_5A_n77(2A)8 DC_5A_n77(3A)8 |  | 5 |  |  | 844 |  |  | 5 |  |  | 25 | 889 |  | 8.3 |  |  | IMD4 |  |
|  |  | n77 |  |  | 3421 |  |  | 10 |  |  | 50 | 3421 |  | N/A |  |  | N/A |  |
|  |  | 5 |  |  | 826.5 |  |  | 5 |  |  | 25 | 871.5 |  | 5.5 |  |  | IMD5 |  |
|  |  | n77 |  |  | 4177.5 |  |  | 10 |  |  | 50 | 4177.5 |  | N/A |  |  | N/A |  |
| DC_5A_n78A DC_5A_n78(2A) DC_5A_n78(A-C) DC_5A_n78C |  | 5 |  |  | 844 |  |  | 5 |  |  | 25 | 889 |  | 8.3 |  |  | IMD4 |  |
|  |  | n78 |  |  | 3421 |  |  | 10 |  |  | 50 | 3421 |  | N/A |  |  | N/A |  |
| DC_7_n3 |  | 7 |  |  | 2535 |  |  | 10 |  |  | 50 | 2655 |  | 13 |  |  | IMD4 |  |
|  |  | n3 |  |  | 1730 |  |  | 5 |  |  | 25 | 1825 |  | N/A |  |  | N/A |  |
| DC_7_n5 |  | 7 |  |  | 2547 |  |  | 10 |  |  | 50 | 2667 |  | N/A |  |  | N/A |  |
|  |  | n5 |  |  | 834 |  |  | 5 |  |  | 25 | 879 |  | 12 |  |  | IMD33 |  |
| DC_7A_n20A |  | 7 |  |  | 2512 |  |  | 10 |  |  | 50 | 2632 |  | N/A |  |  | N/A |  |
|  |  | n20 |  |  | 851 |  |  | 5 |  |  | 25 | 810 |  | 12 |  |  | IMD33 |  |
| DC_7A_n26A DC_7C_n26A |  | 7 |  |  | 2547 |  |  | 10 |  |  | 50 | 2667 |  | N/A |  |  | N/A |  |
|  |  | n26 |  |  | 834 |  |  | 5 |  |  | 25 | 879 |  | 12 |  |  | IMD33 |  |
|  |  | 7 |  |  | 2567.5 |  |  | 5 |  |  | 25 | 2687.5 |  | 2.5 |  |  | IMD5 |  |
|  |  | n26 |  |  | 816.5 |  |  | 5 |  |  | 25 | 861.5 |  | N/A |  |  | N/A |  |
| DC_7_n40 |  | 7 |  |  | 2510 |  |  | 5 |  |  | 25 | 2630 |  | 23 |  |  | IMD3 |  |
|  |  | n40 |  |  | 2390 |  |  | 5 |  |  | 25 | 2390 |  | N/A |  |  | N/A |  |
| DC_7A_n66A DC_7A-7A_n66A DC_7C_n66A |  | 7 |  |  | 2535 |  |  | 10 |  |  | 50 | 2655 |  | 15 |  |  | 4th IMD |  |
|  |  | n66 |  |  | 1730 |  |  | 5 |  |  | 25 | 2130 |  | N/A |  |  | N/A |  |
| DC_7A_n77A DC_7A-7A_n77(2A) DC_7A-7A_n77(3A) DC_7A_n77(2A) DC_7A_n77(3A) DC_7C_n77A DC_7C_n77(2A) |  | 7 |  |  | 2540 |  |  | 5 |  |  | 25 | 2660 |  | 7.1 |  |  | IMD4 |  |
|  |  | n77 |  |  | 3870 |  |  | 10 |  |  | 50 | 3870 |  | N/A |  |  | N/A |  |
| DC_7_n79 |  | 7 |  |  | 2510 |  |  | 5 |  |  | 25 | 2630 |  | [8] |  |  | IMD4 |  |
|  |  | n79 |  |  | 4900 |  |  | 40 |  |  | 216 | 4900 |  | N/A |  |  | N/A |  |
| DC_8A_n1A |  | 8 |  |  | 887.5 |  |  | 5 |  |  | 25 | 932.5 |  | N/A |  |  | N/A |  |
|  |  | n1 |  |  | 1965 |  |  | 5 |  |  | 25 | 2155 |  | 6 |  |  | IMD4 |  |
| DC_8A_n3A |  | 8 |  |  | 900 |  |  | 5 |  |  | 25 | 945 |  | 8 |  |  | IMD43 |  |
|  |  | n3 |  |  | 1755 |  |  | 10 |  |  | 50 | 1850 |  | N/A |  |  | N/A |  |
|  |  | 8 |  |  | 897.5 |  |  | 5 |  |  | 25 | 942.5 |  | N/A |  |  | N/A |  |
|  |  | n3 |  |  | 1747.5 |  |  | 10 |  |  | 50 | 1842.5 |  | 6.4 |  |  | IMD5 |  |
| DC_8A_n20A |  | n20 |  |  | 849.5 |  |  | 5 |  |  | 25 | 808.5 |  | 25 |  |  | IMD33 |  |
|  |  | 8 |  |  | 890.5 |  |  | 5 |  |  | 25 | 935.5 |  | N/A |  |  | N/A |  |
|  |  | n20 |  |  | 847.5 |  |  | 5 |  |  | 25 | 806.5 |  | N/A |  |  | N/A |  |
|  |  | 8 |  |  | 892.5 |  |  | 5 |  |  | 25 | 937.5 |  | 25 |  |  | IMD33 |  |
| DC_8A_n38A |  | 8 |  |  | 887.5 |  |  | 5 |  |  | 25 | 932.5 |  | 8.1 |  |  | IMD5 |  |
|  |  | n38 |  |  | 2617.5 |  |  | 5 |  |  | 25 | 2617.5 |  | N/A |  |  | N/A |  |
| DC_8A_n41A DC_8A_SUL_n41A-n81A |  | 8 |  |  | 882.5 |  |  | 5 |  |  | 25 | 927.5 |  | 12.1 |  |  | IMD33 |  |
|  |  | n41 |  |  | 2685 |  |  | 10 |  |  | 50 | 2685 |  | N/A |  |  | N/A |  |
| DC_8A_n77A, DC_8A_n78A, DC_8B_n78A DC_8A_n78(2A), DC_8A_n77(3A), DC_8A_SUL_n78A-n81A |  | 8 |  |  | 897.5 |  |  | 5 |  |  | 25 | 942.5 |  | 8.3 |  |  | IMD4 |  |
|  |  | n77, n78 |  |  | 3635 |  |  | 10 |  |  | 50 | 3635 |  | N/A |  |  | N/A |  |
| DC_8A_n79A, DC_8A_n79C, DC_8A_SUL_n79A-n81A |  | 8 |  |  | 897.5 |  |  | 5 |  |  | 25 | 942.5 |  | 4.8 |  |  | IMD5 |  |
|  |  | n79 |  |  | 4532.5 |  |  | 40 |  |  | 216 | 4532.5 |  | N/A |  |  | N/A |  |
| DC_11A_n28A |  | 11 |  |  | 1430.5 |  |  | 5 |  |  | 25 | 1478.5 |  | N/A |  |  | N/A |  |
|  |  | n28 |  |  | 743 |  |  | 5 |  |  | 25 | 798 |  | 10.4 |  |  | IMD4 |  |
| DC_12A_n77A DC_12A_n77(2A) |  | 12 |  |  | 702 |  |  | 5 |  |  | 20 | 732 |  | 5.5 |  |  | IMD5 |  |
|  |  | n77 |  |  | 3540 |  |  | 10 |  |  | 50 | 3540 |  | N/A |  |  | N/A |  |
| DC_12A_n78A |  | 12 |  |  | 710 |  |  | 5 |  |  | 25 | 740 |  | 5.5 |  |  | IMD5 |  |
|  |  | n78 |  |  | 3580 |  |  | 10 |  |  | 50 | 3580 |  | N/A |  |  | N/A |  |
| DC_13A_n5A |  | 13 |  |  | 783 |  |  | 5 |  |  | 25 | 752 |  | N/A |  |  | N/A |  |
|  |  | n5 |  |  | 828 |  |  | 5 |  |  | 25 | 873 |  | 25 |  |  | IMD3 |  |
| DC_13A_n7A DC_13A_n7(2A) |  | 13 |  |  | 784.5 |  |  | 5 |  |  | 25 | 753.5 |  | N/A |  |  | N/A |  |
|  |  | n7 |  |  | 2520 |  |  | 40 |  |  | 216 | 2640 |  | 2.5 |  |  | IMD5 |  |
| DC_13A_n77A |  | 13 |  |  | 784.5 |  |  | 5 |  |  | 20 | 753.5 |  | 5.5 |  |  | IMD5 |  |
|  |  | n77 |  |  | 3891.5 |  |  | 10 |  |  | 50 | 3891.5 |  | N/A |  |  | N/A |  |
| DC_14A_n5A |  | 14 |  |  | 791 |  |  | 5 |  |  | 25 | 761 |  | N/A |  |  | N/A |  |
|  |  | n5 |  |  | 836 |  |  | 5 |  |  | 25 | 881 |  | 25 |  |  | IMD3 |  |
|  |  | 14 |  |  | 795.5 |  |  | 5 |  |  | 25 | 765.5 |  | 25 |  |  | IMD3 |  |
|  |  | n5 |  |  | 826.5 |  |  | 5 |  |  | 25 | 871.5 |  | N/A |  |  | N/A |  |
| DC_14A_n77A DC_14A_n77(2A) |  | 14 |  |  | 795.5 |  |  | 5 |  |  | 15 | 765.5 |  | 5.5 |  |  | IMD5 |  |
|  |  | n77 |  |  | 3947.5 |  |  | 10 |  |  | 50 | 3947.5 |  | N/A |  |  | N/A |  |
| DC_18A_n3A |  | 18 |  |  | 823 |  |  | 5 |  |  | 25 | 868 |  | N/A |  |  | N/A |  |
|  |  | n3 |  |  | 1721 |  |  | 5 |  |  | 25 | 1816 |  | 4 |  |  | IMD4 |  |
| DC_18A_n77A DC_18A_n78A |  | 18 |  |  | N/A |  |  | N/A |  |  | N/A | N/A |  | N/A |  |  | IMD4 |  |
|  |  | n77, n78 |  |  | N/A |  |  | N/A |  |  | N/A | N/A |  | N/A |  |  | N/A |  |
| DC_19A_n78A |  | 19 |  |  | N/A |  |  | N/A |  |  | N/A | N/A |  | N/A |  |  | IMD4 |  |
|  |  | n78 |  |  | N/A |  |  | N/A |  |  | N/A | N/A |  | N/A |  |  | N/A |  |
| DC_20A_n3A |  | 20 |  |  | 840 |  |  | 5 |  |  | 25 | 799 |  | N/A |  |  | N/A |  |
|  |  | n3 |  |  | 1775 |  |  | 5 |  |  | 25 | 1870 |  | 4 |  |  | IMD4 |  |
|  |  | 20 |  |  | 847 |  |  | 5 |  |  | 25 | 806 |  | 9 |  |  | IMD4 |  |
|  |  | n3 |  |  | 1735 |  |  | 5 |  |  | 25 | 1830 |  | N/A |  |  | N/A |  |
| DC_20A_n38A |  | 20 |  |  | N/A |  |  | N/A |  |  | N/A | N/A |  | N/A |  |  | IMD5 |  |
|  |  | n38 |  |  | N/A |  |  | N/A |  |  | N/A | N/A |  | N/A |  |  | N/A |  |
| DC_20_n7 |  | 20 |  |  | 851 |  |  | 5 |  |  | 25 | 810 |  | 12 |  |  | IMD33 |  |
|  |  | n7 |  |  | 2512 |  |  | 10 |  |  | 50 | 2632 |  | N/A |  |  | N/A |  |
| DC_20A_n8A |  | 20 |  |  | 849.5 |  |  | 5 |  |  | 25 | 808.5 |  | 25 |  |  | IMD3 |  |
|  |  | n8 |  |  | 892.5 |  |  | 5 |  |  | 25 | 937.5 |  | 25 |  |  | IMD3 |  |
| DC_20_n41 |  | 20 |  |  | 851 |  |  | 5 |  |  | 25 | 810 |  | 12.1 |  |  | IMD3 |  |
|  |  | n41 |  |  | 2512 |  |  | 10 |  |  | 50 | 2512 |  | N/A |  |  | N/A |  |
| DC_20_n41 |  | 20 |  |  | 841 |  |  | 5 |  |  | 25 | 800 |  | 8.1 |  |  | IMD5 |  |
|  |  | n41 |  |  | 2564 |  |  | 10 |  |  | 50 | 2564 |  | N/A |  |  | N/A |  |
| DC_20A_n77A, DC_20A_n78A DC_20A_n78C7, DC_20A_n78(2A), DC_20A_SUL_n78A-n82A |  | 20 |  |  | 850 |  |  | 5 |  |  | 25 | 809 |  | 11 |  |  | IMD4 |  |
|  |  | n77, n78 |  |  | 3359 |  |  | 10 |  |  | 50 | 3359 |  | N/A |  |  | N/A |  |
| DC_20A_n77A |  | 20 |  |  | 840 |  |  | 5 |  |  | 25 | 799 |  | 6.5 |  |  | IMD5 |  |
|  |  | n77 |  |  | 4159 |  |  | 10 |  |  | 50 | 4159 |  | N/A |  |  | N/A |  |
| DC_21A_n28A7 |  | 21 |  |  | 1450.4 |  |  | 5 |  |  | 25 | 1498.4 |  | 2.5 |  |  | IMD5 |  |
|  |  | n28 |  |  | 735.5 |  |  | 5 |  |  | 25 | 790.5 |  | N/A |  |  | N/A |  |
| DC_21A_n79A |  | 21 |  |  | 1457.5 |  |  | 5 |  |  | 25 | 1505.5 |  | 18.4 |  |  | IMD3 |  |
|  |  | n79 |  |  | 4420.5 |  |  | 40 |  |  | 216 | 4420.5 |  | N/A |  |  | N/A |  |
| DC_25A_n77A DC_25A-25A_n77A |  | 25 |  |  | 1855 |  |  | 5 |  |  | 25 | 1935 |  | 26 |  |  | IMD2 |  |
|  |  | n77 |  |  | 3790 |  |  | 10 |  |  | 50 | 3790 |  | N/A |  |  | N/A |  |
|  |  | 25 |  |  | 1900 |  |  | 5 |  |  | 25 | 1980 |  | 8 |  |  | IMD4 |  |
|  |  | n77 |  |  | 3720 |  |  | 10 |  |  | 50 | 3720 |  | N/A |  |  | N/A |  |
|  |  | 25 |  |  | 1885 |  |  | 5 |  |  | 25 | 1965 |  | 5 |  |  | IMD5 |  |
|  |  | n77 |  |  | 3810 |  |  | 10 |  |  | 50 | 3810 |  | N/A |  |  | N/A |  |
| DC_25A_n78A DC_25A-25A_n78A |  | 25 |  |  | 1855 |  |  | 5 |  |  | 25 | 1935 |  | 26 |  |  | IMD2 |  |
|  |  | n78 |  |  | 3790 |  |  | 10 |  |  | 50 | 3790 |  | N/A |  |  | N/A |  |
|  |  | 25 |  |  | 1885 |  |  | 5 |  |  | 25 | 1965 |  | 8 |  |  | IMD4 |  |
|  |  | n78 |  |  | 3690 |  |  | 10 |  |  | 50 | 3690 |  | N/A |  |  | N/A |  |
|  |  | 25 |  |  | 1875 |  |  | 5 |  |  | 25 | 1955 |  | 5 |  |  | IMD5 |  |
|  |  | n78 |  |  | 3790 |  |  | 10 |  |  | 50 | 3790 |  | N/A |  |  | N/A |  |
| DC_26A_n41A |  | 26 |  |  | 839 |  |  | 5 |  |  | 25 | 884 |  | 15.6 |  |  | IMD33 |  |
|  |  | n41 |  |  | 2562 |  |  | 10 |  |  | 50 | 2562 |  | N/A |  |  | N/A |  |
| DC_28_n50 |  | 28 |  |  | 730 |  |  | 10 |  |  | 50 | 775 |  | 15.3 |  |  | IMD 2 |  |
|  |  | n50 |  |  | 1500 |  |  | 10 |  |  | 50 | 1500 |  | N/A |  |  | N/A |  |
|  |  | 28 |  |  | 740 |  |  | 10 |  |  | 50 | 785 |  | 6 |  |  | IMD 4 |  |
|  |  | n50 |  |  | 1500 |  |  | 10 |  |  | 50 | 1500 |  | N/A |  |  | N/A |  |
|  |  | 28 |  |  | 740 |  |  | 10 |  |  | 50 | 785 |  | 0.5 |  |  | IMD 5 |  |
|  |  | n50 |  |  | 1500 |  |  | 10 |  |  | 50 | 1500 |  | N/A |  |  | N/A |  |
| DC_28A_n51A |  | 28 |  |  | 742.3 |  |  | 5 |  |  | 25 | 797.3 |  | 5 |  |  | IMD4 |  |
|  |  | n51 |  |  | 1429.5 |  |  | 5 |  |  | 25 | 1429.5 |  | N/A |  |  | N/A |  |
| DC_26A_n77A, DC_26A_n78A |  | 26 |  |  | 836.5 |  |  | 5 |  |  | 25 | 881.5 |  | 11.1 |  |  | IMD4 |  |
|  |  | n77, n78 |  |  | 3391 |  |  | 10 |  |  | 50 | 3391 |  | N/A |  |  | N/A |  |
| DC_28A_n77A, DC_28A_n78A, DC_28A_n78(2A), DC_28A_SUL_n78A-n83A |  | 28 |  |  | 705.5 |  |  | 5 |  |  | 25 | 760.5 |  | 5.5 |  |  | IMD5 |  |
|  |  | n77, n78 |  |  | 3582.5 |  |  | 10 |  |  | 50 | 3582.5 |  | N/A |  |  | N/A |  |
| DC_30A_n77A DC_30A_n77(2A) |  | 30 |  |  | 2310 |  |  | 5 |  |  | 25 | 2355 |  | 8.0 |  |  | IMD4 |  |
|  |  | n77 |  |  | 3487.5 |  |  | 10 |  |  | 50 | 3487.5 |  | N/A |  |  | N/A |  |
| DC_38A_n3A |  | n3 |  |  | 1713 |  |  | 5 |  |  | 25 | 1808 |  | 8.2 |  |  | IMD4 |  |
|  |  | 38 |  |  | 2617 |  |  | 5 |  |  | 25 | 2617 |  | N/A |  |  | N/A |  |
| DC_38A_n8A |  | 38 |  |  | 2617.5 |  |  | 5 |  |  | 25 | 2617.5 |  | N/A |  |  | N/A |  |
|  |  | n8 |  |  | 887.5 |  |  | 5 |  |  | 25 | 932.5 |  | 8.1 |  |  | IMD5 |  |
| DC_41A_n3A DC_41C_n3A |  | n3 |  |  | 1740 |  |  | 5 |  |  | 25 | 1835 |  | 8.2 |  |  | IMD4 |  |
|  |  | 41 |  |  | 2657.5 |  |  | 5 |  |  | 25 | 2657.5 |  | N/A |  |  | N/A |  |
| DC_42_n3 |  | 42 |  |  | 3575 |  |  | 10 |  |  | 50 | 3575 |  | N/A |  |  | N/A |  |
|  |  | n3 |  |  | 1740 |  |  | 5 |  |  | 25 | 1835 |  | 26 |  |  | 2nd3 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |  |  |
|  |  | 42 |  |  | 3435 |  |  | 10 |  |  | 50 | 3435 |  | N/A |  |  | N/A |  |
|  |  | n3 |  |  | 1765 |  |  | 5 |  |  | 25 | 1860 |  | 8.0 |  |  | 4th3 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |  |  |
| DC_42_n28 |  | 42 |  |  | 3582.5 |  |  | 10 |  |  | 50 | 3582.5 |  | N/A |  |  | N/A |  |
|  |  | n28 |  |  | 705.5 |  |  | 5 |  |  | 25 | 760.5 |  | 5.5 |  |  | IMD5 |  |
| DC_48A_n2A DC_48C_n2A DC_48D_n2A DC_48E_n2A |  | 48 |  |  | 3625 |  |  | 20 |  |  | 100 | 3625 |  | N/A |  |  | N/A |  |
|  |  | n2 |  |  | 1852.5 |  |  | 5 |  |  | 25 | 1932.5 |  | 12 |  |  | IMD4 |  |
| DC_48A_n12A |  | 48 |  |  | 3557.5 |  |  | 10 |  |  | 50 | 3557.5 |  | N/A |  |  | N/A |  |
|  |  | n12 |  |  | 705.5 |  |  | 5 |  |  | 25 | 735.5 |  | 5.5 |  |  | IMD5 |  |
| DC_48A_n25A DC_48C_n25A DC_48D_n25A |  | 48 |  |  | 3625 |  |  | 20 |  |  | 100 | 3625 |  | N/A |  |  | N/A |  |
|  |  | n25 |  |  | 1852.5 |  |  | 5 |  |  | 25 | 1932.5 |  | 12 |  |  | IMD4 |  |
| DC_48A_n66A DC_48C_n66A DC_48D_n66A |  | 48 |  |  | 3630 |  |  | 20 |  |  | 100 | 3630 |  | N/A |  |  | N/A |  |
|  |  | n66 |  |  | 1715 |  |  | 5 |  |  | 25 | 2115 |  | 4 |  |  | IMD5 |  |
| DC_66A_n2A, DC_66A-66A_n2A |  | 66 |  |  | 1775 |  |  | 5 |  |  | 25 | 2175 |  | N/A |  |  | N/A |  |
|  |  | n2 |  |  | 1855 |  |  | 5 |  |  | 25 | 1935 |  | 20 |  |  | IMD3 |  |
|  |  | 66 |  |  | 1750 |  |  | 5 |  |  | 25 | 2150 |  | 4 |  |  | IMD5 |  |
|  |  | n2 |  |  | 1883.3 |  |  | 5 |  |  | 25 | 1963.3 |  | N/A |  |  | N/A |  |
| DC_66A_n5A |  | n5 |  |  | 838 |  |  | 5 |  |  | 25 | 883 |  | 30 |  |  | IMD23 |  |
|  |  | 66 |  |  | 1721 |  |  | 5 |  |  | 25 | 2121 |  | N/A |  |  | N/A |  |
| DC_66A_n7A DC_66A-66A_n7A DC_66A_n7(2A) DC_66A-66A_n7(2A) |  | 66 |  |  | 1730 |  |  | 5 |  |  | 25 | 2130 |  | N/A |  |  | N/A |  |
|  |  | n7 |  |  | 2535 |  |  | 10 |  |  | 50 | 2655 |  | 15 |  |  | IMD4 |  |
| DC_66A_n25A |  | 66 |  |  | 1775 |  |  | 5 |  |  | 25 | 2175 |  | N/A |  |  | N/A |  |
|  |  | n25 |  |  | 1855 |  |  | 5 |  |  | 25 | 1935 |  | 20 |  |  | IMD3 |  |
|  |  | 66 |  |  | 1712.5 |  |  | 5 |  |  | 25 | 2112.5 |  | 23 |  |  | IMD3 |  |
|  |  | n25 |  |  | 1912.5 |  |  | 5 |  |  | 25 | 1992.5 |  | N/A |  |  | N/A |  |
|  |  | 66 |  |  | 1750 |  |  | 5 |  |  | 25 | 2150 |  | 4 |  |  | IMD5 |  |
|  |  | n25 |  |  | 1883.3 |  |  | 5 |  |  | 25 | 1963.3 |  | N/A |  |  | N/A |  |
| DC_66A_n46A |  | 66 |  |  | 1735 |  |  | 5 |  |  | 25 | 2135 |  | 12.0 |  |  | IMD3 |  |
|  |  | n46 |  |  | 5605 |  |  | 20 |  |  | 100 | 5605 |  | N/A |  |  | N/A |  |
| DC_66A_n48A |  | 66 |  |  | 1715 |  |  | 5 |  |  | 25 | 2115 |  | 4 |  |  | IMD5 |  |
|  |  | n48 |  |  | 3630 |  |  | 20 |  |  | 100 | 3630 |  | N/A |  |  | N/A |  |
| DC_66A_n71A |  | 66 |  |  | 1750 |  |  | 5 |  |  | 25 | 2150 |  | 5 |  |  | IMD4 |  |
|  |  | n71 |  |  | 675 |  |  | 5 |  |  | 25 | 629 |  | N/A |  |  | N/A |  |
| DC_66A_n77A DC_66A_n77(2A) DC_66A-66A_n77A DC_66A-66A_n77(2A) DC_66A-66A-66A_n77A DC_66A-66A-66A_n77(2A) |  | 66 |  |  | 1775 |  |  | 5 |  |  | 25 | 2175 |  | 31.0 |  |  | IMD2 |  |
|  |  | n77 |  |  | 3950 |  |  | 10 |  |  | 50 | 3950 |  | N/A |  |  | N/A |  |
|  |  | 66 |  |  | 1760 |  |  | 5 |  |  | 25 | 2160 |  | 5.0 |  |  | IMD5 |  |
|  |  | n77 |  |  | 3720 |  |  | 10 |  |  | 50 | 3720 |  | N/A |  |  | N/A |  |
| DC_66A_n78A |  | 66 |  |  | 1730 |  |  | 5 |  |  | 25 | 2150 |  | 5.0 |  |  | IMD5 |  |
|  |  | n78 |  |  | 3660 |  |  | 10 |  |  | 50 | 3660 |  | N/A |  |  | N/A |  |
| DC_71A_n38A |  | 71 |  |  | 665 |  |  | 5 |  |  | 25 | 619 |  | 11 |  |  | IMD4 |  |
|  |  | n38 |  |  | 2614 |  |  | 10 |  |  | 50 | 2614 |  | N/A |  |  | N/A |  |
| DC_71A_n41A |  | 71 |  |  | 666 |  |  | 5 |  |  | 25 | 620 |  | 11 |  |  | IMD4 |  |
|  |  | n41 |  |  | 2618 |  |  | 5 |  |  | 25 | 2618 |  | N/A |  |  | N/A |  |
| DC_71A_n66A |  | 71 |  |  | 675 |  |  | 5 |  |  | 25 | 629 |  | N/A |  |  | N/A |  |
|  |  | n66 |  |  | 1750 |  |  | 5 |  |  | 25 | 2150 |  | 5 |  |  | IMD4 |  |
| DC_71A_n77A8 |  | 71 |  |  | 671 |  |  | 5 |  |  | 25 | 625 |  | 5.5 |  |  | IMD5 |  |
|  |  | n77 |  |  | 3309 |  |  | 10 |  |  | 50 | 3309 |  | N/A |  |  | N/A |  |
| DC_71A_n78A |  | 71 |  |  | 681.5 |  |  | 5 |  |  | 25 | 635.5 |  | 5.5 |  |  | IMD5 |  |
| DC_71A_n78(2A) |  | n78 |  |  | 3361.5 |  |  | 10 |  |  | 50 | 3361.5 |  | N/A |  |  | N/A |  |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBstart = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5.1-1a: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 18.4 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD2 |
| DC_3A-3A_n78A | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD4 |
| DC_3A-3A_n78A | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_8A_n78A | 8 | 897.5 | 5 | 25 | 942.5 | 15.5 | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | N/A |
| DC_2A_n77A DC_2A-2A_n77A DC_2A_n77C DC_2A-2A_n77C DC_2A_n77(2A) | 2 | 1855 | 5 | 25 | 1935 | 32.10 | IMD2 |
|  |  |  |  |  |  | 34.852 |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 19.10 | IMD41 |
|  |  |  |  |  |  | 21.852 |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_5A_n77A3 DC_5A_n77C3 | 5 | 844 | 5 | 25 | 889 | 18.60 | IMD41 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_13A_n77A DC_13A_n77C | 13 | 782 | 5 | 20 | 751 | 15.37 | IMD5 |
|  | n77 | 3879 | 10 | 50 | 3879 | N/A | N/A |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C DC_66A_n77(2A) | 66 | 1775 | 5 | 25 | 2175 | 34.33 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 11.27 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_5A_n78A | 5 | 844 | 5 | 25 | 889 | 17.5 | IMD4 |
|  | n78 | 3421 | 10 | 52 | 3421 | N/A | N/A |
| DC_12A_n77A | 12 | 702 | 5 | 20 | 732 | 11.7 | IMD5 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_14A_n77A | 14 | 795.5 | 5 | 15 | 765.5 | 11.7 | IMD5 |
|  | n77 | 3947.5 | 10 | 50 | 3947.5 | N/A | N/A |
| DC_30A_n77A | 30 | 2310 | 5 | 25 | 2355 | 17.6 | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 | N/A | N/A |
| DC_28A_n78A | 28 | 705.5 | 5 | 25 | 760.5 | 11.7 | IMD5 |
|  | n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_71A_n77A3 | 71 | 681.5 | 5 | 25 | 635.5 | 11.4 | IMD5 |
|  | n77 | 3361.5 | 10 | 50 | 3361.5 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 3: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 4: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

7.3B.2.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.3.5.2-0: MSD test points for Pcell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A-(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

<< Next Changes >>

7.3B.3.2	Intra-band non-contiguous EN-DC

Table 7.3B.3.2-1: Intra-band non-contiguous EN-DC with one uplink configuration on E-UTRA for reference sensitivity (E-UTRA uplink carrier is closer to the NR downlink carrier than it is to the E-UTRA downlink carrier)

| DC configuration | Aggregated channel bandwidth |  | Wgap / (MHz) | UL E-UTRA allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA | NR |  |  |  |  |
| DC_3A_n3A | 5 MHz | 5 MHz | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 | FDD |
|  |  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 5 MHz | 10 MHz | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 5 MHz | 15 MHz | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 5 MHz | 20 MHz | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 5 MHz | 25 MHz | 25.0 < Wgap ≤ 45.0 | 121 | 3.2 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 5 MHz | 30 MHz | 20.0 < Wgap ≤ 40.0 | 121 | 3.0 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 10 MHz | 5 MHz | 30.0 < Wgap ≤ 60.0 | 12 (RBstart = 25) | 5.1 |  |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 10 MHz | 10MHz | 25.0 < Wgap ≤ 55.0 | 12 (RBstart = 25) | 4.3 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 10 MHz | 15 MHz | 20.0 < Wgap ≤ 50.0 | 12 (RBstart = 25) | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 10 MHz | 20 MHz | 15.0 < Wgap ≤ 45.0 | 12 (RBstart = 25) | 3.5 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 10 MHz | 25 MHz | 10.0 < Wgap ≤ 40.0 | 12 (RBstart = 25) | 3.2 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 10 MHz | 30 MHz | 5.0 < Wgap ≤ 35.0 | 12 (RBstart = 25) | 2.8 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15 MHz | 5 MHz | 25.0 < Wgap ≤ 55.0 | 12 (RBstart = 35) | 6.0 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 15 MHz | 10 MHz | 20.0 < Wgap ≤ 50.0 | 12 (RBstart = 35) | 4.7 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 15 MHz | 15 MHz | 15.0 < Wgap ≤ 45.0 | 12 (RBstart = 35) | 4.2 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 15 MHz | 20 MHz | 10.0 < Wgap ≤ 40.0 | 12 (RBstart = 35) | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 15 MHz | 25 MHz | 5.0 < Wgap ≤ 35.0 | 12 (RBstart = 35) | 3.5 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 15 MHz | 30 MHz | 0.0 < Wgap ≤ 30.0 | 12 (RBstart = 35) | 3.3 |  |
|  | 20 MHz | 5 MHz | 15.0 < Wgap ≤ 50.0 | 16 (RBstart = 50) | 6.5 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 20 MHz | 10 MHz | 10.0 < Wgap ≤ 45.0 | 16 (RBstart = 50) | 5.1 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 20 MHz | 15 MHz | 5.0 < Wgap ≤ 40.0 | 16 (RBstart = 50) | 4.5 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 20 MHz | 20 MHz | 0.0 < Wgap ≤ 35.0 | 16 (RBstart = 50) | 4.1 |  |
|  | 20 MHz | 25 MHz | 0.0 < Wgap ≤ 30.0 | 16 (RBstart = 50) | 3.8 |  |
|  | 20 MHz | 30 MHz | 0.0 < Wgap ≤ 25.0 | 16 (RBstart = 50) | 3.6 |  |
| DC_66A_n66A | NOTE 4 |  | NOTE 8 | NOTE 9 | 0 | FDD |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The E-UTRA uplink carrier is the aggressor and the ΔRIBNC applies to the NR DL carrier only. The E-UTRA uplink carrier shall be located as close as possible to the NR downlink carrier centre frequency. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: Void. NOTE 6: Void. NOTE 7: Void. NOTE 8: All applicable sub-block gap sizes. NOTE 9: The UL LTE allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1 in TS 36.101 [4]. |  |  |  |  |  |  |

Table 7.3B.3.2-2: Intra-band non-contiguous EN-DC with one uplink configuration on NR for reference sensitivity (NR uplink carrier is closer to the E-UTRA downlink carrier than it is to the NR downlink carrier)

| DC configuration | Aggregated bandwidth |  | Wgap / (MHz) | UL NR allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA |  |  |  |  |
| DC_1A_n1A | 5 MHz | 5 MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |
| DC_2A_n2A | 5MHz | 5MHz | 30.0 < Wgap ≤ 50.0 | 121 | 5.3 | FDD |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 5MHz | 10MHz | 25.0 < Wgap ≤ 45.0 | 121 | 4.4 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 5MHz | 15MHz | 20.0 < Wgap ≤ 40.0 | 121 | 4.2 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 5MHz | 20MHz | 15.0 < Wgap ≤ 35.0 | 121 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 251 | 0 |  |
|  | 10MHz | 5MHz | 15.0 < Wgap ≤ 45.0 | 121 | 5.9 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 10MHz | 10MHz | 10.0 < Wgap ≤ 40.0 | 121 | 4.6 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 10MHz | 15MHz | 5.0 < Wgap ≤ 35.0 | 121 | 4.1 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 10MHz | 20MHz | 0.0 < Wgap ≤ 30.0 | 121 | 4.0 |  |
|  | 15MHz | 5MHz | 10.0 < Wgap ≤ 40.0 | 12 (RBstart = 39) | 6.7 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 361 | 0 |  |
|  | 15MHz | 10MHz | 5.0 < Wgap ≤ 35.0 | 12 (RBstart = 39) | 5.4 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 361 | 0 |  |
|  | 15MHz | 15MHz | 0.0 < Wgap ≤ 30.0 | 12 (RBstart = 39) | 4.6 |  |
|  | 15MHz | 20MHz | 0.0 < Wgap ≤ 25.0 | 12 (RBstart = 39) | 4.2 |  |
|  | 20MHz | 5MHz | 0.0 < Wgap ≤ 35.0 | 16 (RBstart = 57) | 7.2 |  |
|  | 20MHz | 10MHz | 0.0 < Wgap ≤ 30.0 | 16 (RBstart = 57) | 5.8 |  |
|  | 20MHz | 15MHz | 0.0 < Wgap ≤ 25.0 | 16 (RBstart = 57) | 5.0 |  |
|  | 20MHz | 20MHz | 0.0 < Wgap ≤ 20.0 | 16 (RBstart = 57) | 4.6 |  |
| DC_3A_n3A | 5MHz | 5MHz | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 | FDD |
|  |  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 5MHz | 10MHz | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 5MHz | 15MHz | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 5MHz | 20MHz | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 10MHz | 5MHz | 30.0 < Wgap ≤ 60.0 | 12 (RBstart = 25) | 5.1 |  |
|  |  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 10MHz | 10MHz | 25.0 < Wgap ≤ 55.0 | 12 (RBstart = 25) | 4.3 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 10MHz | 15MHz | 20.0 < Wgap ≤ 50.0 | 12 (RBstart = 25) | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 10MHz | 20MHz | 15.0 < Wgap ≤ 45.0 | 12 (RBstart = 25) | 3.5 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 15MHz | 5MHz | 25.0 < Wgap ≤ 55.0 | 12 (RBstart = 35) | 6.0 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 15MHz | 10MHz | 20.0 < Wgap ≤ 50.0 | 12 (RBstart = 35) | 4.7 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 15MHz | 15MHz | 15.0 < Wgap ≤ 45.0 | 12 (RBstart = 35) | 4.2 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 15MHz | 20MHz | 10.0 < Wgap ≤ 40.0 | 12 (RBstart = 35) | 3.8 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 20MHz | 5MHz | 15.0 < Wgap ≤ 50.0 | 16 (RBstart = 50) | 6.5 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 20MHz | 10MHz | 10.0 < Wgap ≤ 45.0 | 16 (RBstart = 50) | 5.1 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 20MHz | 15MHz | 5.0 < Wgap ≤ 40.0 | 16 (RBstart = 50) | 4.5 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 20MHz | 20MHz | 0.0 < Wgap ≤ 35.0 | 16 (RBstart = 50) | 4.1 |  |
|  | 25MHz | 5MHz | 10.0 < Wgap ≤ 45.0 | 16 (RBstart = 60) | 7.4 |  |
|  |  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 25MHz | 10MHz | 5.0 < Wgap ≤ 40.0 | 16 (RBstart = 60) | 5.5 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 25MHz | 15MHz | 0.0 < Wgap ≤ 35.0 | 16 (RBstart = 60) | 4.9 |  |
|  | 25MHz | 20MHz | 0.0 < Wgap ≤ 30.0 | 16 (RBstart = 60) | 4.6 |  |
|  | 30MHz | 5MHz | 5.0 < Wgap ≤ 40.0 | 16 (RBstart = 75) | 8.3 |  |
|  |  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 30MHz | 10MHz | 0.0 < Wgap ≤ 35.0 | 16 (RBstart = 75) | 5.9 |  |
|  | 30MHz | 15MHz | 0.0 < Wgap ≤ 30.0 | 16 (RBstart = 75) | 5.5 |  |
|  | 30MHz | 20MHz | 0.0 < Wgap ≤ 25.0 | 16 (RBstart = 75) | 4.9 |  |
| DC_5A_n5A | 5 MHz | 5 MHz | NOTE 10 | 121 | 5.3 | FDD |
|  | 10 MHz | 5 MHz |  |  | 4.4 |  |
|  | 15 MHz | 5 MHz |  |  | 6.1 |  |
|  | 5 MHz | 10 MHz |  |  | 5.9 |  |
|  | 10 MHz | 10 MHz |  |  | 4.6 |  |
| DC_7A_n7A | 5MHz | 5MHz | 0< Wgap ≤ 60 | 25 | 0.0 | FDD |
|  | 5MHz | 10MHz | 0 < Wgap ≤ 55 | 25 | 0.0 |  |
|  | 5MHz | 15MHz | 0 < Wgap ≤ 50 | 25 | 0.0 |  |
|  | 5MHz | 20MHz | 0 < Wgap ≤ 45 | 25 | 0.0 |  |
|  | 10MHz | 5MHz | 30 < Wgap ≤ 55 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 30 | 50 | 0.0 |  |
|  | 10MHz | 10MHz | 25.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 25.0 | 50 | 0.0 |  |
|  | 10MHz | 15MHz | 20 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 20 | 50 | 0.0 |  |
|  | 10MHz | 20MHz | 15 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 15 | 50 | 0.0 |  |
|  | 15MHz | 5MHz | 20.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 15MHz | 10MHz | 20.0 < Wgap ≤ 45.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 15MHz | 15MHz | 15.0 < Wgap ≤ 40.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 15MHz | 20MHz | 10 < Wgap ≤ 35 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 10 | 501 | 0.0 |  |
|  | 20MHz | 5MHz | 25 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 25 | 451 | 0.0 |  |
|  | 20MHz | 10MHz | 20 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  |  | 0 < Wgap ≤ 20 | 451 | 0.0 |  |
|  | 20MHz | 15MHz | 15.0 < Wgap ≤ 35.0 | 361 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 20MHz | 20MHz | 15.0 < Wgap ≤ 30.0 | 321 | 0.0 |  |
|  |  |  | 0.0 < Wgap ≤ 15.0 | 451 | 0.0 |  |
| DC_71A_n71A | 5 MHz | 5 MHz | 5 < Wgap ≤ 25 | 5 | 4.0 | FDD |
|  |  |  | 0 < Wgap ≤ 5 | 20 | 0 |  |
|  | 10 MHz | 5 MHz | 5 < Wgap ≤ 20 | 5 (RBstart = 9) | 4.6 |  |
|  |  |  | 0 < Wgap ≤ 5 | 20 (RBstart = 9) | 2.3 |  |
|  | 10 MHz | 10 MHz | 5 < Wgap ≤ 15 | 5 (RBstart = 9) | 4.3 |  |
|  |  |  | 0 < Wgap ≤ 5 | 20 (RBstart = 9) | 3.2 |  |
|  | 15 MHz | 10 MHz | 5 < Wgap ≤ 10 | 5 (RBstart = 2) | 22.2 |  |
|  |  |  | 0 < Wgap ≤ 5 | 20 (RBstart = 19) | 5.2 |  |
|  | 20 MHz | 10 MHz | Wgap = 5 | 5 | [22.2] |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The NR uplink carrier is the aggressor and the ΔRIBNC applies to the E-UTRA DL carrier only. The NR uplink carrier shall be located as close as possible to the E-UTRA downlink carrier center frequency. The NR SCS should be the smallest SCS that is compatible with the highest uplink channel bandwidth. NOTE 4: All combinations of channel bandwidths defined in Table 5.3B.1.3-1. NOTE 5: Void. NOTE 6: Void. NOTE 7: Void. NOTE 8: Void. NOTE 9: Void.9 refers to the UL resource blocks shall be located at RBstart=75. NOTE 10: All applicable sub-block gap sizes. NOTE 11: Void. NOTE 12: Void. NOTE 13: Void. NOTE 14: Void. NOTE 15: Void. |  |  |  |  |  |  |

7.3B.3.3	Inter-band EN-DC within FR1

7.3B.3.3.1	ΔRIB,c for EN-DC in two bands

Table 7.3B.3.3.1-1: ΔRIB,c due to EN-DC(two bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_1_n28 | - | 0.2 |
| DC_1_n51 | - | 0.1 |
| DC_1_n77 | 0.2 | 0.5 |
| DC_1_n78 | - | 0.5 |
| DC_2_n30 DC_2-2_n30 | 0.4 | 0.5 |
| DC_2_n48 | 0.2 | 0.5 |
| DC_2_n66 DC_2-2_n66 | 0.3 | 0.3 |
| DC_2_n77 DC_2-2_n77 | 0.2 | 0.5 |
| DC_2_n78 DC_2-2_n78 | 0.2 | 0.5 |
| DC_3_n41 | - | 03 / 0.54 |
| DC_3_n51 | 0.2 | 0.2 |
| DC_3_n77 DC_3-3_n77 | 0.2 | 0.5 |
| DC_3_n78 DC_3-3_n78 | 0.2 | 0.5 |
| DC_4_n2 | 0.3 | 0.3 |
| DC_4_n28 | - | 0.2 |
| DC_4_n38 | 0.5 | 0.5 |
| DC_4_n41 | 0.5 | 0.51 / 12 |
| DC_4_n78 | 0.2 | 0.5 |
| DC_5_n3 | 0.2 | 0.2 |
| DC_5_n12 | 0.5 | 0.3 |
| DC_5_n77 | 0.2 | 0.5 |
| DC_5_n78 | 0.2 | 0.5 |
| DC_7_n8 DC_7-7_n8 | - | 0.2 |
| DC_7_n40 | - | 0.5 |
| DC_7_n51 | - | 0.2 |
| DC_7_n66 DC_7-7_n66 | 0.5 | 0.5 |
| DC_7_n71 | - | 0.2 |
| DC_7_n77 DC_7-7_n77 | - | 0.5 |
| DC_7_n78 DC_7-7_n78 | - | 0.5 |
| DC_7_n79 | - | 0.5 |
| DC_8_n7 | - | 0.2 |
| DC_8_n28 | 0.2 | 0.1 |
| DC_8_n77 | 0.2 | 0.5 |
| DC_8_n78 | 0.2 | 0.5 |
| DC_11_n3 | 0.3 | 0.5 |
| DC_11_n28 | - | 0.2 |
| DC_11_n77 | - | 0.5 |
| DC_11_n78 | - | 0.5 |
| DC_12_n5 | 0.3 | 0.5 |
| DC_12_n66 | 0.5 | - |
| DC_12_n71 | 0.8 | 0.8 |
| DC_12_n77 | 0.2 | 0.5 |
| DC_12_n78 | 0.2 | 0.5 |
| DC_13_n7 | 0.5 | 0.5 |
| DC_13_n77 | 0.2 | 0.5 |
| DC_13_n78 | 0.2 | 0.5 |
| DC_14_n77 | 0.2 | 0.5 |
| DC_18_n41 | - | 03 |
| DC_18_n77 | - | 0.5 |
| DC_19_n77 | - | 0.5 |
| DC_19_n78 | - | 0.5 |
| DC_20_n38 | 0.2 | - |
| DC_20_n51 | - | 0.2 |
| DC_20_n77 | - | 0.5 |
| DC_20_n78 | - | 0.5 |
| DC_21_n77 | - | 0.5 |
| DC_21_n78 | - | 0.5 |
| DC_25_n41 DC_25-25_n41 | - | 01 / 0.52 |
| DC_25_n77 DC_25-25_n77 DC_25-25_n77 | 0.2 | 0.5 |
| DC_25_n78 DC_25-25_n78 | 0.2 | 0.5 |
| DC_26_n77 | - | 0.5 |
| DC_26_n78 | - | 0.5 |
| DC_28_n1 | 0.2 | - |
| DC_28_n8 | 0.1 | 0.2 |
| DC_28_n51 | - | 0.2 |
| DC_28_n66 | 0.2 | - |
| DC_28_n77 | 0.2 | 0.5 |
| DC_28_n78 | 0.2 | 0.5 |
| DC_30_n66 | 0.5 | 0.4 |
| DC_30_n77 | - | 0.5 |
| DC_38_n78 | 0.4 | 0.5 |
| DC_38_n79 | - | 0.5 |
| DC_39_n40 | 0.3 | 0.3 |
| DC_39_n41 | 0.2 | 0.2 |
| DC_39_n78 | - | 0.5 |
| DC_39_n79 | - | 0.5 |
| DC_40_n77 | 0.4 | 0.5 |
| DC_40_n78 | 0.45 | 0.55 |
| DC_40_n79 | - | 0.5 |
| DC_41_n3 | 03 / 0.54 | - |
| DC_41_n77 | - | 0.5 |
| DC_41_n78 | - | 0.5 |
| DC_41_n79 | - | 0.5 |
| DC_42_n1 | 0.5 | - |
| DC_42_n3 | 0.5 | 0.2 |
| DC_42_n28 | 0.2 | 0.5 |
| DC_42_n51 | - | 0.2 |
| DC_48_n2 | 0.5 | 0.2 |
| DC_48_n25 | 0.5 | 0.2 |
| DC_48_n46 | 0.5 | - |
| DC_48_n66 | 0.5 | 0.2 |
| DC_66_n2 DC_66-66_n2 DC_66-66-66_n2 | 0.3 | 0.3 |
| DC_66_n7 DC_66-66_n7 | 0.5 | 0.5 |
| DC_66_n12 | 0.5 | - |
| DC_66_n25 | 0.3 | 0.3 |
| DC_66_n28 | - | 0.2 |
| DC_66_n30 DC_66-66_n30 | 0.5 | 0.4 |
| DC_66_n38 DC_66-66_n38 | 0.5 | 0.5 |
| DC_66_n41 | 0.5 | 0.51 / 12 |
| DC_66_n48 DC_66-66_n48 | 0.2 | 0.5 |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 0.2 | 0.5 |
| DC_66_n78 DC_66-66_n78 | 0.2 | 0.5 |
| DC_71_n38 | 0.2 | - |
| DC_71_n41 | 0.2 | - |
| DC_71_n77 | 0.2 | 0.5 |
| DC_71_n78 | 0.2 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. NOTE 3: Applicable for the frequency range of 2515 – 2690 MHz. NOTE 4: Applicable for the frequency range of 2496 – 2515 MHz. NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_1_n77 the band order from left to right is 1 and n77. |  |  |

<< Next Changes >>

7.4B	Maximum input level for DC in FR1

7.4B.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC maximum input level requirement and parameters are defined in Table 7.4B.1-1.

Table 7.4B.1-1: Maximum Input

| Power in Largest CC, E-UTRA or NR, dBm | X1 |
| --- | --- |
| Power in each other CC, dBm | X1 – 10*log10(NxSCSx/NySCSy) |
| NOTE 1: Power in Largest E-UTRA or NR bandwidth CC, listed in Table 7.4-1 [2] NOTE 2: Nx, SCSx is the number of RB's and Sub carrier spacing in the largest carrier bandwidth and could be E-UTRA or NR carrier NOTE 3: Ny, SCSy is the number of RB's in any other carrier. NOTE 4: Void. NOTE 5: Void. |  |

7.4B.1a	Intra-band contiguous NE-DC in FR1

Intra-band contiguous NE-DC maximum input level requirement and parameters are defined in Table 7.4B.1a-1.

Table 7.4B.1a-1: Maximum Input

| Power in Largest CC, E-UTRA or NR, dBm | X1 |
| --- | --- |
| Power in each other CC, dBm | X1 – 10*log10(NxSCSx/NySCSy) |
| NOTE 1: Power in Largest E-UTRA or NR bandwidth CC, listed in Table 7.4-1 [2] NOTE 2: Nx, SCSx is the number of RB's and Sub carrier spacing in the largest carrier bandwidth and could be E-UTRA or NR carrier NOTE 3: Ny, SCSy is the number of RB's in any other carrier. |  |

<< Next Changes >>

7.5B	Adjacent channel selectivity for DC in FR1

7.5B.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC ACS requirement and parameters are defined for test case 1 in Table 7.5B.1-1 and for test case 2 in Table 7.5B.1-2.

Table 7.5B.1-1: ACS test case 1

| EN-DC Aggregated Bandwidth, MHz | <=100 | >100, <=120 | >120, <=140 | >140, <=160 |
| --- | --- | --- | --- | --- |
| ACS, dB | X1 | 19.2 | 18.5 | 17.9 |
| Pinterferer, dBm | PI 2 | Aggregated power + 17.7 dB | Aggregated power + 17 dB | Aggregated power + 16.4dB |
| Pw in Transmission BW configuration, per CC, dBm | REFSENS +14dB |  |  |  |
| NOTE 1: X is ACS level at the specified EN-DC aggregated bandwidth from Table 7.5.1A-1 in TS 36.101 [4] NOTE 2: PI is from Table 7.5.1A-2 in TS 36.101 [4] NOTE 3: Jammer BW and offset is from Table 7.5.1A-2 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 4: Void. NOTE 5: Void. |  |  |  |  |

Table 7.5B.1-2: ACS test case 2

| EN-DC Aggregated Bandwidth, ENBW, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | -42.7 +10log10(NRB,c/ NRB_agg) | -42 +10log10(NRB,c/ NRB_agg) | -41.4 +10log10(NRB,c/ NRB_agg) |
| Pinterferer, dBm | -25 |  |  |  |
| NOTE 1: PW is wanted signal power level at the specified EN-DC aggregated Bandwidth from Table 7.5.1A-3 in TS 36.101 [4] NOTE 2: Jammer BW and offset is from Table 7.5.1A-3 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 3: Void. NOTE 4: Void. |  |  |  |  |

7.5B.1a	Intra-band contiguous NE-DC in FR1

Intra-band contiguous NE-DC ACS requirement and parameters are defined for test case 1 in Table 7.5B.1a-1 and for test case 2 in Table 7.5B.1a-2.

Table 7.5B.1a-1: ACS test case 1

| NE-DC Aggregated Bandwidth, MHz | <=100 | >100, <=120 | >120, <=140 | >140, <=160 |
| --- | --- | --- | --- | --- |
| ACS, dB | X1 | 19.2 | 18.5 | 17.9 |
| Pinterferer, dBm | PI 2 | Aggregated power + 17.7 dB | Aggregated power + 17 dB | Aggregated power + 16.4dB |
| Pw in Transmission BW configuration, per CC, dBm | REFSENS +14dB |  |  |  |
| NOTE 1: X is ACS level at the specified NE-DC aggregated bandwidth from Table 7.5.1A-1 in TS 36.101 [4] NOTE 2: PI is from Table 7.5.1A-2 in TS 36.101 [4] NOTE 3: Jammer BW and offset is from Table 7.5.1A-2 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier |  |  |  |  |

Table 7.5B.1a-2: ACS test case 2

| NE-DC Aggregated Bandwidth, ENBW, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | -42.7 +10log10(NRB,c/ NRB_agg) | -42 +10log10(NRB,c/ NRB_agg) | -41.4 +10log10(NRB,c/ NRB_agg) |
| Pinterferer, dBm | -25 |  |  |  |
| NOTE 1: PW is wanted signal power level at the specified NE-DC aggregated Bandwidth from Table 7.5.1A-3 in TS 36.101 [4] NOTE 2: Jammer BW and offset is from Table 7.5.1A-3 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier |  |  |  |  |

<< Next Changes >>

7.6B.2	In-band blocking for DC in FR1

7.6B.2.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC in-band blocking requirement and parameters are defined in Table 7.6B.2.1-1.

Table 7.6B.2.1-1: In-band blocking

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm |  | REFSENS + Aggregated BW specific value below |  |  |
|  | PW 1 | 16.8 | 17.5 | 18 |
| NOTE 1: PW is wanted signal power level at the specified EN-DC aggregated Bandwidth from Table 7.6.1.1A-1 in TS 36.101 [4] NOTE 2: Interferer values are specified from Table 7.6.1.1A-2 in TS 36.101 [4] NOTE 3: Jammer BW and offset is from Table 7.6.1.1A-1 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 4: Void. NOTE 5: Void. |  |  |  |  |

7.6B.2.1a	Intra-band contiguous NE-DC in FR1

Intra-band contiguous NE-DC in-band blocking requirement and parameters are defined in Table 7.6B.2.1a-1.

Table 7.6B.2.1a-1: In-band blocking

| NE-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm |  | REFSENS + Aggregated BW specific value below |  |  |
|  | PW 1 | 16.8 | 17.5 | 18 |
| NOTE 1: PW is wanted signal power level at the specified EN-DC aggregated Bandwidth from Table 7.6.1.1A-1 in TS 36.101 [4] NOTE 2: Interferer values are specified from Table 7.6.1.1A-2 in TS 36.101 [4] NOTE 3: Jammer BW and offset is from Table 7.6.1.1A-1 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier |  |  |  |  |

<< Next Changes >>

7.6B.3	Out-of-band blocking for DC in FR1

7.6B.3.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC out-of-band requirement and parameters are defined in Table 7.6B.3.1-1.

Table 7.6B.3.1-1: Out-of-band blocking

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [4] NOTE 2: Void. NOTE 3: Void. |  |  |  |  |

For Table 7.6.2.1A-2 from TS 36.101 [4] in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7B.1 Spurious response are applicable.

7.6B.3.1a	Intra-band contiguous NE-DC in FR1

Intra-band contiguous NE-DC out-of-band requirement and parameters are defined in Table 7.6B.3.1a-1.

Table 7.6B.3.1a-1: Out-of-band blocking

| NE-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| NOTE 1: Interferer values and offsets are specified from Table 7.6.2.1A-2 in TS 36.101 [4] |  |  |  |  |

For Table 7.6.2.1A-2 from TS 36.101 [4] in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7B.1 Spurious response are applicable.

<< Next Changes >>

7.6B.4	Narrow band blocking for DC in FR1

7.6B.4.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC narrow band blocking requirement and parameters are defined in Table 7.6B.4.1-1.

Table 7.6B.4.1-1: Narrow band blocking parameters

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 16 |  |  |  |
| PUW, dBm (CW) | -55 |  |  |  |
| NOTE 1: Jammer offset is from Table 7.6.3.1A-1 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 2: Void. NOTE 3: Void. NOTE 4: If NR carrier BW > 40 MHz, no narrow band blocking requirements apply when blocker is applied at the edge of the NR carrier. |  |  |  |  |

7.6B.4.1a	Intra-band contiguous NE-DC in FR1

Intra-band contiguous NE-DC narrow band blocking requirement and parameters are defined in Table 7.6B.4.1a-1.

Table 7.6B.4.1a-1: Narrow band blocking parameters

| NE-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 16 |  |  |  |
| PUW, dBm (CW) | -55 |  |  |  |
| NOTE 1: Jammer offset is from Table 7.6.3.1A-1 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 4: If NR carrier BW > 40 MHz, no narrow band blocking requirements apply when blocker is applied at the edge of the NR carrier. |  |  |  |  |

<< Next Changes >>

7.7B	Spurious response for DC in FR1

7.7B.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC spurious response requirement and parameters are defined in Table 7.7B.1-1.

Table 7.7B.1-1: Spurious Response Parameters

| EN-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| Pinterferer, dBm (CW) | -44 |  |  |  |
| NOTE 1: Void. NOTE 2: Void. |  |  |  |  |

7.7B.1a	Intra-band contiguous NE-DC in FR1

Intra-band contiguous NE-DC spurious response requirement and parameters are defined in Table 7.7B.1a-1.

Table 7.7B.1a-1: Spurious Response Parameters

| NE-DC Aggregated Bandwidth, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | REFSENS + Aggregated BW specific value below |  |  |  |
|  | 9 |  |  |  |
| Pinterferer, dBm (CW) | -44 |  |  |  |

<< Next Changes >>

7.8B	Intermodulation characteristics for DC in FR1

7.8B.1	General

7.8B.2	Wide band Intermodulation

7.8B.2.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC wide band intermodulation requirement and parameters are defined in Table 7.8B.2.1-1.

Table 7.8B.2.1-1: Wide band intermodulation

| EN-DC Aggregated Bandwidth, MHz | <=100 | >100, <=120 | >120, <=140 | >140, <=160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | REFSENS + Aggregated BW specific value below |  |  |
|  |  | 16.8 | 17.5 | 18.0 |
| Pinterferer 1, dBm (CW)2 | -46 |  |  |  |
| Pinterferer 2, dBm (Modulated)2 | -46 |  |  |  |
| NOTE 1: PW is wanted signal power level from Table 7.8.1A-1 in TS 36.101 [4] NOTE 2: Jammer BW and offsets is from Table 7.8.1A-1 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 3: Void. NOTE 4: Void. |  |  |  |  |

7.8B.2.1a	Intra-band contiguous NE-DC in FR1

Intra-band contiguous NE-DC wide band intermodulation requirement and parameters are defined in Table 7.8B.2.1a-1.

Table 7.8B.2.1a-1: Wide band intermodulation

| NE-DC Aggregated Bandwidth, MHz | <=100 | >100, <=120 | >120, <=140 | >140, <=160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | REFSENS + Aggregated BW specific value below |  |  |
|  |  | 16.8 | 17.5 | 18.0 |
| Pinterferer 1, dBm (CW)2 | -46 |  |  |  |
| Pinterferer 2, dBm (Modulated)2 | -46 |  |  |  |
| NOTE 1: PW is wanted signal power level from Table 7.8.1A-1 in TS 36.101 [4] NOTE 2: Jammer BW and offsets is from Table 7.8.1A-1 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier |  |  |  |  |

<< Next Changes >>

7.9B	Spurious emissions for DC in FR1

7.9B.1	Intra-band contiguous EN-DC in FR1

The requirement is defined in clause 7.9A.1 in TS 38.101-1 [2].

7.9B.1a	Intra-band contiguous NE-DC in FR1

The requirement is defined in clause 7.9A.1 in TS 38.101-1 [2].

<< End of changes >>