3GPP TSG RAN Meeting #98-e							                RP-223358

Electronic Meeting, December 12-16, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.893 to introduce the potential solutions.

Agenda Item:	9.2.2

Document for:	Approval

Introduction

Referring to the discussion paper [1], text proposals for introduction of potential solutions are provided in this paper to address the issues from regionally-defined subsets of an NR band.

- References

[1] RP-223357, “Discussion on UE support of regionally-defined subsets of an NR band”, Huawei, HiSilicon

Text proposals

- <<Start of TP for TR 38.893>>

- 5	Other Issues

- 5.1	Roaming UEs

In some countries it is neither the operator nor the base station that takes responsibility for the UE meeting regulations.  In the USA UE vendors can only allow their devices to operate in parts of the band where there are FCC regulations at the time of device certification.

UE vendors may choose to pursue certification for some countries but not others. In view of the regional regulatory differences described above, we recognize that there may be ambiguity with regards to exactly which regulatory requirements are supported in any given UE implementation.   It is important to understand how this ambiguity could affect regulatory compliance when UEs roam in countries in which they are not certified. Two scenarios are identified for clarification for feasibility of roaming:

The first scenario is if a UE is certified to operate only in part of a band (e.g. band n77 cases UE in Canada/US).  In this scenario, the UE is only allowed to operate within the subset of the band for which it is certified in the country where this certification is necessary.  Because the UE in addition to the above regulatory certification is also compliant to 3GPP requirements (e.g., for Band n77), it is required that the UE can support the entire band.  Therefore, this UE when roaming to a country which does not have any regional regulatory requirements shall be able to operate on the entirety of the band.  If this country does have regional requirements, the UE shall abide by them to operate in this country.

The second scenario is if a UE can support the frequency range of a band based on the 3GPP requirements but not certified for additional regional requirements in a subset of the band in a certain foreign country. This aspect has to be evaluated for each country based on prevailing regulation.  Whether the roaming UE is allowed to operate without certification from the country’s regulator is subject to each country’s laws. It’s also allowed that the roaming UE doesn’t attempt to access to and/or operate in the specific frequency range in specific country/region, if UE can’t guarantee whether the roaming UE is allowed to operate without certification from the country’s regulator

- 6	Possible solutions

- 6.1	Solution 1

- 6.2	Solution 2

- 6.X	Solution X: the approach of UE implementation for accessing to cell + new capability

Since network can’t know whether UE has been certified in the specific country in specific partial frequency range no matter whether UE report the capability (e.g. extendedBand-n77-r16 or extendedBand-n77-2-r17 or future new capabilities) for subsets of band, it’s natural that neither the operator nor the base station can take responsibility for the UE meeting regional regulations.

In addition, in SIB1, network has broadcast the enough information to UE, such as MCC code and initial BWP information (frequency range and frequency location). Before UE starts to trigger a random access (transmit UL signals), UE can make a good judgement on whether it can meet the regional regulation or not.

Because only UE know whether it has been certified based on the regional regulation and whether it’s legal or illegal to access to one specific frequency range in this specific region, from 3GPP perspective, it can be up to UE implementation to choose whether to trigger a random access after receiving enough SIB1 information.

Once UE access to this cell, network can check UE capabilities which are used to indicate the subsets of band. When the carrier frequency that UE access to is same to what UE indicate, there is no issue. When the carrier frequency that UE access to is different from what UE indicate by using the capabilities, it can also be up to network implementation to do the following behaviour.

1) To ignore the UE capability and schedule UE in current cell or in current cell with restriction of initial BWP frequency range.

2) To do cell switching by considering UE capability indication.

3) To reject the UE accessing to this cell.

4) Other options are allowed.

Since the network implementations above can work well and network don’t need to take responsibility for the UE meeting regulations, there is no need to design any new SIB IE or NS values to bar UE accessing one cell.

In total, this solution X has the following characteristic.

1) For UE initial access, it’s up to UE implementation whether to access to the specific frequency range in specific region considering the information in SIB1.

2) After accessing to the cell, UE can report the new capability which is used to indicate the subset of band. The details of this new capability can be further discussed in the potential WI phase. However, this general capability which is used to indicate the subsets of a band is only valid under the condition that it is reported by UE in the specific band and specific region.

- 6.Y	Solution Y: the approach of UE behaviour standardization + new capability

In clause 4 root cause, it is shown that some UE behaviours are unclear and incorrect due to the ambiguity of UE behaviour. Thus, in order to address this issue and come up with a general solution, one way is to standardize the UE behaviours. It’s necessary to standardize some UE behaviours based on the common understanding from 3GPP perspective to comply with the regulation. And these UE behaviours should be aligned with the regional regulations as soon as possible. It’s very important to avoid some UE behaviours’ issues from legacy UE and incorrect UE behaviours, if all the UE behaviours specified in the specification are very clear.

The solution Y was proposed with the following characteristics.

1) To standard UE behaviour for UE whether to access to the specific frequency range in specific region considering the information in SIB1.

2) After accessing to the cell, UE can report the new capability which is used to indicate the subset of band. The details of this new capability can be further discussed in the potential WI phase. However, this general capability which is used to indicate the subsets of a band is only valid under the condition that it is reported by UE in the specific band and specific region.

- <<End of TP for TR 38.893>>