3GPP TSG-RAN2 Meeting #120	R2-2213286

# Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1866 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Multiple CSI Subframe Sets on CQI-ReportPeriodicScell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_euCA-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2022-11-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN1 initially agreed that if a UE is configured to report for more than one CSI subframe set then parameter sets for subframe set 1 and subframe set 2 should be configured. However, RRC signalling does not support to separate configuration of single CSI subframe set and multiple CSI subframe sets regarding cqi-pmi-ConfigIndexDormant/ cqi-pmi-ConfigIndex2Dormant and ri-ConfigIndexDormant /ri-ConfigIndex2Dormant. In reply LS from RAN1 (R2-2209320), It is answered that RAN1 will remove ‘cqi-pmi-ConfigIndex2Dormant’ and ‘ri-ConfigIndex2Dormant’ in TS 36.213 for Rel-15/16 and RAN1 kindly requests RAN2 to introduce the RRC parameters ‘cqi-pmi-ConfigIndex2Dormant’ and ‘ri-ConfigIndex2Dormant’ in Rel-17. For supporting to separate configuration of single CSI subframe set and multiple CSI subframe sets for Dormant SCell, the new UE capability is required from Rel-17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description for the new UE capability (i.e. csi-SubframeSet2ForDormantSCell-r17) in Physical layer parameter, subclause 4.3.4. This field defines whether the UE supports second CSI subframe set for periodic CSI reporting for dormant serving cells. This field is only applicable for UEs supporting TDD. A UE that indicates support of this field shall also indicate support for dormantSCellState-r15. Impact analysis Impacted architecture options: EUTRA Impacted functionality: Support of Multiple CSI Subframe Sets on CQI-ReportPeriodicScell Inter-operability: If the UE is implemented according to the CR while the network is not, there are no inter-operability problem. If the network is implemented according to the CR while the UE is not, there are no inter-operability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Separate configuration of single CSI subframe set and multiple CSI subframe is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ...TS 36.331 CR#4899 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

4.3.4	Physical layer parameters

4.3.4.xyz	csi-SubframeSet2ForDormantSCell-r17

This field defines whether the UE supports second CSI subframe set for periodic CSI reporting for dormant serving cells. This field is only applicable for UEs supporting TDD. A UE that indicates support of this field shall also indicate support for dormantSCellState-r15.

END OF CHANGE