3GPP TSG RAN2 Meeting #120	R2-2213361

# Toulouse, France, 14th - 18th November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | 0110 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data volume calculation for DAPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to current specification, at uplink data switching, the PDCP entity for AM DRBs performs the retransmission from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed. In this case, the data volume for the PDCP SDUs to be retransmitted shall be reported to the target cell. However, in the current specification, the PDCP SDUs to be retransmitted are not calcauated for data volume at uplink data switching. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At uplink data switching, the PDCU SDUs to be retransmitted are calcuated for the data volume. Impact analysis Impacted 5G architecture options: NR standalone Impacted functionality: Data volume calculation Inter-operability: 1. 1. If the network is implemented according to the CR and the UE is not, the UE may report the incorrect information for buffer status report. Then, the network provides the uplink grant based on the incorrect information for buffer status report. 2. If the UE is implemented according to the CR and the network is not, no inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect information for buffer status report is reported during DAPS handover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | [MCC]:Coversheet revision: WI code updated to NR_Mob_enh-Core. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

## 5.6	Data volume calculation

For the purpose of MAC buffer status reporting, the transmitting PDCP entity shall consider the following as PDCP data volume:

-	the PDCP SDUs for which no PDCP Data PDUs have been constructed;

-	the PDCP Data PDUs that have not been submitted to lower layers;

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

<End of modification>