3GPP TSG-RAN WG2 Meeting #120	R2-2213185

# Toulouse, France, November 14th – 18th 2022	was R3-226917

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0359 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on direct data forwarding for SN initiated CPC to TS37.340 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (ZTE, CATT, Huawei, Ericsson, Intel Corporation) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 37.340, the following descriptions can be found, they could be understood as direct data forwarding is supported in stage2 specification. - In case multiple candidate SNs are prepared, the MN includes a list of Target SN ID and list of data forwarding addresses to the source SN. However, current TS37.340 has not included the description of direct data forwarding for SN initiated CPC to TS37.340. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add early data forwarding description for SN initiated CPC procedure. Impact Analysis: This CR has an isolated impact towards the previous version of the specification, and will not cause non-backward compatible issue. This CR only has an impact on the support of direct data forwarding in case multiple T-SNs are prepared for SN initiated inter-SN CPC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For SN initiated CPC, its description in stage 2 specification is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.1, 10.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R3-224266, R3-226244. R3-226886 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-226917. |  |  |  |  |  |  |  |  |

## 10.5	Secondary Node Change (MN/SN initiated)

### 10.5.1	EN-DC

<Skip unrelated part>

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for CPC configuration and CPC execution.

The SN initiated conditional SN change procedure may also be initiated by the source SN, to modify the existing SN-initiated CPC configuration, or to trigger the release of the candidate SN by cancellation of all the prepared PSCells at the candidate SN and releasing the CPC related UE context at the candidate SN.

Figure 10.5.1-4: Conditional SN Change – SN initiated

Figure 10.5.1-4 shows an example signalling flow for the Conditional Secondary Node Change initiated by the SN:

1.	The source SN initiates the conditional SN change procedure by sending SgNB Change Required message which contains a CPC initiation indication. The message also contains candidate SN ID(s) information and may include the SCG configuration (to support delta configuration), and contains the measurement results related to the candidate SN(s). The message also includes a list of proposed PSCell candidates recommended by the source SN, including execution conditions, the upper limit for the number of PSCells that can be prepared by the candidate SN(s), and may also include the SCG measurement configurations for CPC (e.g. measurement ID(s) to be used for CPC).

2/3.	The MN requests each target candidate SN to allocate resources for the UE by means of the SgNB Addition procedure(s) including a CPC initiation indication, and the measurements results related to the candidate SN and indicating a list of proposed PSCell candidates received from the source SN, but not including execution conditions. Within the list of PSCells suggested by the source SN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the MN) and, for each prepared PSCell, the candidate SN decides SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SgNB Addition Request Acknowledge message. If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN includes the indication of full or delta RRC configuration, and the list of prepared PSCell IDs to the MN. The candidate SN can either accept or reject each of the candidate cells suggested by the source SN, i.e. it cannot configure any alternative candidates.

NOTE 6a:	In case the candidate SN includes the indication of the full RRC configuration, the MN performs release of the SN terminated radio bearer configuration and release and add of the NR SCG configuration part towards the UE in the conditional configuration.

4/5.	The MN may indicate the candidate PSCells accepted by the candidate SN(s) to the source SN via SgNB Modification Request message before it configures the UE e.g., when not all candidate PSCells were accepted by the candidate SN. If the MN does not send such indication, step 4 and 5 are skipped. If requested,the source SN sends an SgNB Modification Request Acknowledge message and if needed, provides an updated measurement configurations and/or the execution conditions for CPC to the MN.

6.	The MN sends to the UE an RRCConnectionReconfiguration message including the CPC configuration, i.e. a list of RRCConnectionReconfiguration* messages and associated execution conditions, in which each RRCConnectionReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCConnectionReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required conditional measurements.

7.	The UE applies the RRCConnectionReconfiguration message received in step 6, stores the CPC configuration and replies to the MN with an RRCConnectionReconfigurationComplete message, which can include an NR RRC RRCReconfigurationComplete*** message. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

8.	If an NR RRC response message is included, the MN informs the source SN with the NR RRCReconfigurationComplete*** message via SgNB Change Confirm message. If step 4 and 5 are skipped, the MN will indicate the candidate PSCells accepted by the candidate SN(s) to the source SN in the SgNB Change Confirm message.

The MN sends the SgNB Change Confirm message towards the source SN to indicate that CPC is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the candidate SN, the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding. In case multiple candidate SNs are prepared, the MN includes a list of Target SgNB ID and list of data forwarding addresses to the source SN.

NOTE 6b:	The Data Forwarding Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some PDCP SDUs if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional PSCell change.

9a-9d.	The source SN may send the SgNB Modification Required message to trigger an update of CPC execution condition and/or corresponding SCG measurement configuration for CPC for the UE if any. In such case in step 9b and 9c, the MN reconfigures the UE as in step 6 and 7.

10.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies the RRCConnectionReconfiguration* message corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete* message, including the NR RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

11a-11b.	The MN triggers the MeNB initiated SgNB Release procedure to inform source SN to stop providing user data to the UE, and provide the address of the SN of the selected candidate PSCell and if applicable, start late data forwarding.

12a-12c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected candidate PSCell via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SgNB Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

13.	The UE synchronizes to the PSCell indicated in the RRCConnectionReconfiguration* message applied in step 10.

14.	For SN terminated bearers using RLC AM, the source SN sends the SN Status Transfer message, which the MN sends then to the SN of the selected candidate PSCell, if needed.

15.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding message from the MN.

16.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 7:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SgNB may send the report when the transmission of the related bearer is stopped.

17-21.	If applicable, a path update is triggered by the MN.

22.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

### 10.5.2	MR-DC with 5GC

<Skip unrelated part>

SN initiated conditional SN Change

The SN initiated conditional SN change procedure is used for CPC configuration and CPC execution.

The SN initiated conditional SN change procedure may also be initiated by the candidate SN, to modify the existing CPC configuration, or to trigger the release of the candidate SN by cancellation of all the prepared PSCells at the candidate SN and releasing the CPC related UE context at the candidate SN.

Figure 10.5.2-4: Conditional SN change procedure - SN initiated

Figure 10.5.2-4 shows an example signalling flow for the conditional SN Change initiated by the SN:

1.	The source SN initiates the conditional SN change procedure by sending the SN Change Required message, which contains a CPC initiation indication. The message also contains candidate node ID(s) and may include the SCG configuration (to support delta configuration), and contains the measurements results which may include cells that are not CPC candidates. The message also includes a list of proposed PSCell candidates recommended by the source SN, including execution conditions, the upper limit for the number of PSCells that can be prepared by the candidate SN(s), and may also include the SCG measurement configurations for CPC (e.g. measurement ID(s) to be used for CPC).

2/3.	The MN requests each candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure(s), including a CPC initiation indication, and the measurements results which may include cells that are not CPC candidates received from the source SN to the candidate SN, and indicating a list of proposed PSCell candidates received from the source SN, but not including execution conditions. Within the list of PSCells suggested by the source SN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the MN) and, for each prepared PSCell, the candidate SN decides SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SgNB Addition Request Acknowledge message. If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN includes the indication of full or delta RRC configuration, and the list of prepared PSCell IDs to the MN. The candidate SN can either accept or reject each of the candidate cells suggested by the source SN, i.e., it cannot configure any alternative candidates.

4/5.	The MN may indicate the candidate PSCells accepted by the candidate SN to the source SN via SN Modification Request message before it configures the UE, e.g., when not all candidate PSCells were accepted by the candidate SN. If the MN does not send such indication, step 4 and 5 are skipped. If requested, the source SN sends an SN Modification Request Acknowledge message and if needed, provides an updated measurement configurations and/or the execution conditions to the MN.

6.	The MN sends to the UE an RRCReconfiguration message including the CPC configuration, i.e. a list of RRCReconfiguration* messages and associated execution conditions, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCReconfiguration* message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required conditional measurements.

7.	The UE applies the RRCReconfiguration message received in step 6, stores the CPC configuration and replies to the MN with an RRCReconfigurationComplete message, which can include an NR RRCReconfigurationComplete*** message. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

8.	If an SN RRC response message is included, the MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message. If step 4 and 5 are skipped, the MN will indicate the candidate PSCells accepted by the candidate SN(s) to the source SN in the SN Change Confirm message.

The MN sends the SN Change Confirm message towards the source SN to indicate that CPC is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the candidate SN, the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding. In case multiple candidate SNs are prepared, the MN includes a list of Target SN ID and list of data forwarding addresses to the source SN.

NOTE 6a:	The Xn-U Address Indication procedure may further be invoked to indicate to the source SN to stop already initiated early data forwarding for some PDCP SDUs if they are no longer subject to data forwarding due to the modification or cancellation of the prepared conditional PSCell change.

9a-9d.	The source SN may send the SN Modification Required message to trigger an update of CPC execution condition and/or corresponding SCG measurement configuration for CPC for the UE if any. In such case in step 9b and 9c, the MN reconfigures the UE as in step 6 and 7.

10.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell.

11a-11c.	The MN triggers the MN initiated SN Release procedure to inform the source SN to stop providing user data to the UE, and triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell and if applicable, starts late data forwarding.

12a-12c.	If the RRC connection reconfiguration procedure was successful, the MN informs the SN of the selected PSCell candidate via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message. The MN sends the SN Release Request message(s) to cancel CPC in the other candidate SN(s), if configured. The other candidate SN(s) acknowledges the release request.

13.	The UE synchronizes to the PSCell indicated in the RRCReconfiguration* message applied in step 10.

14.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer message, which the MN sends then to the SN of the selected candidate PSCell, if needed.

15.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the data forwarding address related information from the MN.

16.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 6:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

17-21.	If applicable, a PDU Session path update procedure is triggered by the MN.

22.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

## ----------End of the Change--------------