3GPP TSG-RAN2 WG2 Meeting #120	R2-2213307

# Toulouse, France, 14th – 18th November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | 0025 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on capturing L2M agreements in TS 38.314 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At RAN2#119-e meeting, RAN2 made the following agreements: Agreements: - RAN2 agree that for a consistent calculation of the total RAN delay per split DRB per UE, a set of formulas for total RAN delay calculation are needed. RAN2 has already agreed that the total RAN delay calculation can be used for QoS monitoring and MDT purposes. - Total RAN delay calculation formula for split DRB in RAN node is to be specified in Rel 17, for the following scenarios: PDCP duplication is not enabled during the delay measurement period PDCP duplication was enabled during the delay measurement period - Total RAN delay calculation formula for split DRB is to be discussed in Rel-18, for the following scenario: PDCP duplication was enabled per packet basis i.e., some packets transmitted with duplication and some packets without duplication in a delay measurement period. So this CR is to capture these agreements in TS 38.314. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following delay measurements are introduced: - DL Delay Calculation per Split-DRB per UE when PDCP duplication is enabled for entire measurement period - UL Delay Calculation per Split-DRB per UE when PDCP duplication is enabled for entire measurement period - DL Delay Calculation per Split-DRB per UE when PDCP duplication is not enabled for entire measurement period - UL Delay Calculation per Split-DRB per UE when PDCP duplication is not enabled for entire measurement period Impact analysis Impacted functionality: Layer 2 measurements Inter-operability analysis: There is no inter-operability issue as the CR only impacts network side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The previous RAN2 agreements are missing in this specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.x (New), 4.2.1.x.1 (New), 4.2.1.x.2 (New), 4.2.1.x.3 (New), 4.2.1.x.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.1.x Total RAN Delay in Split-DRB Scenario

4.2.1.x.1 DL Delay Calculation per Split-DRB per UE when PDCP duplication is enabled for entire measurement period

The objective of this measurement is to calculate total DL RAN delay per split-DRB per UE when PDCP duplication is enabled for the entire measurement period, for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol layer: PDCP

Table 4.2.1.x.1-1: Definition for total DL delay for DL PDCP duplicated packets per Split-DRB per UE

| Definition | Total DL delay for DL PDCP duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total DL delay for a Split-DRB when PDCP duplication is enabled for the entire measurement period. Detailed Definition: M_DL(T,drbid)=min(M_DLMCG(T,drbid) , M_DLSCG(T,drbid)) , where explanations can be found in the table 4.2.1.x.1-2 below. |
| --- | --- |

Table 4.2.1.x.1-2: Parameter description for total DL delay for DL PDCP duplicated packets per Split-DRB per UE

| M_DL(T,drbid) | Total DL Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| M_DLMCG(T,drbid) | Total DL Delay on the MCG per split DRB per UE during the time period T. |
| M_DLSCG(T,drbid) | Total DL Delay on the SCG per split DRB per UE during the time period T. |
| min(M_DLMCG(T,drbid) , M_DLSCG(T,drbid)) | Minimum of the total DL delays between MCG and SCG legs per Split-DRB per UE during the time period T. |

4.2.1.x.2 UL Delay Calculation per Split-DRB per UE when PDCP duplication is enabled for entire measurement period

The objective of this measurement is to calculate total UL RAN delay per split-DRB per UE when PDCP duplication is enabled for the entire measurement period, for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol layer: PDCP

Table 4.2.1.x.2-1: Definition for total UL delay for UL PDCP duplicated packets per Split-DRB per UE

| Definition | Total UL delay for UL PDCP duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total UL delay for a Split-DRB when PDCP duplication is enabled for the entire measurement period. Detailed Definition: M_UL(T,drbid)=min(M_ULMCG(T,drbid) , M_ULSCG(T,drbid)) , where explanations can be found in the table 4.2.1.x.2-2 below. |
| --- | --- |

Table 4.2.1.x.2-2: Parameter description for total UL delay for UL PDCP duplicated packets per Split-DRB per UE

| M_UL(T,drbid) | Total UL Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| M_ULMCG(T,drbid) | Total UL Delay on the MCG per split DRB per UE during the time period T. |
| M_ULSCG(T,drbid) | Total UL Delay on the SCG per split DRB per UE during the time period T. |
| min(M_ULMCG(T,drbid) , M_ULSCG(T,drbid)) | Minimum of the total UL delays between MCG and SCG legs per Split-DRB per UE during the time period T. |

4.2.1.x.3 DL Delay Calculation per Split-DRB per UE when PDCP duplication is not enabled for entire measurement period

The objective of this measurement is to calculate total DL RAN delay per split-DRB per UE when PDCP duplication is not enabled for the entire measurement period, for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol layer: PDCP

Table 4.2.1.x.3-1: Definition for total DL delay for DL PDCP non-duplicated packets per Split-DRB per UE

| Definition | Total DL delay for DL PDCP non-duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total DL delay for a Split-DRB when PDCP duplication is not enabled for the entire measurement period. Detailed Definition: M_DL(T,drbid)=I_DLMCG(T,drbid)×M_DLMCG(T,drbid)+I_DLSCG(T,drbid)×M_DLSCG(T,drbid)I_DL(T,drbid) I_DLT,drbid=I_DLMCG(T,drbid)+I_DLSCG(T,drbid), where explanations can be found in the table 4.2.1.x.3-2 below. |
| --- | --- |

Table 4.2.1.x.3-2: Parameter description for total DL delay for DL PDCP non-duplicated packets per Split-DRB per UE

| M_DL(T,drbid) | Total DL Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| M_DLMCG(T,drbid) | Total DL Delay on the MCG per split DRB per UE during the time period T. |
| M_DLSCG(T,drbid) | Total DL Delay on the SCG per split DRB per UE during the time period T. |
| I_DLT,drbid | Total number of DL PDCP non-duplicated packets sent per Split-DRB per UE during the time period T. |
| I_DLMCG(T,drbid) | Total number of DL PDCP non-duplicated packets sent on the MCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |
| I_DLSCG(T,drbid) | Total number of DL PDCP non-duplicated packets sent on the SCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |

4.2.1.x.4 UL Delay Calculation per Split-DRB per UE when PDCP duplication is not enabled for entire measurement period

The objective of this measurement is to calculate total UL RAN delay per split-DRB per UE when PDCP duplication is not enabled for the entire measurement period, for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501 [4].

Protocol layer: PDCP

Table 4.2.1.x.4-1: Definition for total UL delay for UL PDCP non-duplicated packets per Split-DRB per UE

| Definition | Total UL delay for UL PDCP non-duplicated packets per Split-DRB per UE. This measurement is applicable for EN-DC and SA. This measurement provides the total UL delay for a Split-DRB when PDCP duplication is not enabled for the entire measurement period. Detailed Definition: M_UL(T,drbid)=I_ULMCG(T,drbid)×M_ULMCG(T,drbid)+I_ULSCG(T,drbid)×M_ULSCG(T,drbid)I_UL(T,drbid) I_ULT,drbid=I_ULMCG(T,drbid)+I_ULSCG(T,drbid), where explanations can be found in the table 4.2.1.x.4-2 below. |
| --- | --- |

Table 4.2.1.x.4-2: Parameter description for total UL delay for UL PDCP non-duplicated packets per Split-DRB per UE

| M_UL(T,drbid) | Total UL Delay per Split-DRB per UE during the time period T. |
| --- | --- |
| M_ULMCG(T,drbid) | Total UL Delay on the MCG per split DRB per UE during the time period T. |
| M_ULSCG(T,drbid) | Total UL Delay on the SCG per split DRB per UE during the time period T. |
| I_ULT,drbid | Total number of UL PDCP non-duplicated packets sent per Split-DRB per UE during the time period T. |
| I_ULMCG(T,drbid) | Total number of UL PDCP non-duplicated packets sent on the MCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |
| I_ULSCG(T,drbid) | Total number of UL PDCP non-duplicated packets sent on the SCG leg per Split-DRB per UE during the time period T, as defined in Table 4.2.1.8.1-1. |