3GPP TSG-RAN WG2 Meeting #120	 R2-2213280

# Toulouse, France, 14 – 18 November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3698 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections related to FeatureCombinationPreambles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon, MediaTek, LG Electronics Inc., ZTE, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core, NR_Slice-Core, NR_redcap-Core, NR_cov_enh-Core |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Change 1: According to the current TS 38.331, if the ssb-SharedRO-MaskIndex-r17 field is configured within FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA, it indicates a subset of ROs configured within this RACH-ConfigCommonTwoStepRA. However, when the FeatureCombinationPreambles is part of legacy 2-step RA configuration (i.e., rach-ConfigCommonTwoStepRA) and the ROs configured in rach-ConfigCommonTwoStepRA are shared with legacy 4-step RA configuration indicated by msgA-SSB-SharedRO-MaskIndex-r16, the ssb-SharedRO-MaskIndex-r17 field cannot be applied to the subset of ROs configured within this RACH-ConfigCommonTwoStepRA, since we do not extend or change the definition of mask index in TS 38.321. This situation is depicted in the figure below: The configuration as in the blue highlighted part above is not possible, since in case of shared ROs, 2-step RACH configuration does not provide a separate PRACH index. However, the current field description is saying that the mask index refers to PRACH configurations from 2-step RA configuration which is not possible for the shared RO case: ssb-SharedRO-MaskIndex Mask index (see TS 38.321 [3]). Indicates a subset of ROs where preambles are allocated for this feature combination. If this field is configured within FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA, it indicates a subset of ROs configured within this RACH-ConfigCommonTwoStepRA. This field is configured when there is more than one RO per SSB. If the field is absent, all ROs configured in RACH-ConfigCommon or RACH-ConfigCommonTwoStepRA containing this FeatureCombinationPreambles are shared. An example is shown in the below figure. When the ROs for Rel-16 2-step RACH configured by RACH-ConfigCommonTwoStepRA are shared with the ROs for Rel-16 4-step RACH (e.g. by setting the msgA-SSB-SharedRO-MaskIndex-r16 to 10 to indicate that every odd PRACH occasion of SSB is allowed for Rel-16 2-step RACH), if the network wants to indicate a subset of ROs (e.g. the RO#3) using ssb-SharedRO-MaskIndex-r17 field for a certain Rel-17 feature, the network should set the ssb-SharedRO-MaskIndex-r17 field to 3 according to the TS 38.321 clause 7.4 (i.e. the PRACH occasion with index 3 of SSB is allowed for the certain Rel-17 feature). Then actually in this case, the the ssb-SharedRO-MaskIndex-r17 in FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA is used to indicate the subset of ROs configured for 4-step RACH instead of 2-step RACH. Hence, it is important to clarify the field description by separating the shared RO and separate RO cases. Change 2: The field featureCombinationPreambles is used to define a partition of the preambles of a RACH configuraiton that are dedicated for a certain purpose, e.g. for SDT, or for Msg1-indications for RedCap UEs. The network may configure a RedCap-specific initial uplink BWP and may configure RA resources for this BWP which are not overlapping with any other RA resources. If the network does not intend to dedicate those preambles to some other feature combination, it is clear that all UEs performing random access using those preambles will be RedCap UEs and there is no need for the network to configure a FeatureCombinationPreambles for these RA resources. However, the current specification mandates the network to configure a featureCombinationPreambles even for this case due to this condition: The field is mandatory present if the RACH-ConfigCommon is included in initialUplinkBWP-RedCap or in an AdditionalRACH-Config. Otherwise, it is optional, Need R. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: Clarified that if the ssb-SharedRO-MaskIndex-r17 field is configured within FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA, in case of separate ROs for 4-step and 2-step random access, this field indicates a subset of ROs configured for 2-step random access, and in case of shared ROs, it indicates the subset of ROs configured for 4-step random access. Change 2: Change the condition AdditionalRACH-AndRedCap such that featureCombinationPreambles is optional even in case the RACH-ConfigCommon is included in initialUplinkBWP-RedCap. And the name is updated accordingly. Impact Analysis Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: Msg1-indications for RedCap Inter-operability: For Change 1: - If the UE is implemented according to the CR while the network is not, the the UE and the network may use/assume different parameters settings which may lead to RACH performance degradation or failure. - If the network is implemented according to the CR while the UE is not, the the UE and the network may use/assume different parameters settings which may lead to RACH performance degradation or failure. For Change 2: 1. If the network is implemented according to the CR and the UE is not, the UE may expect a featureCombinationPreambles even if it is not provided. But this should not cause any inter-operability issues since the UE would for this case not do Msg1-indications for RedCap. 2. If the UE is implemented according to the CR and the network is not, no inter-operability issues are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Change 1: The ssb-SharedRO-MaskIndex-r17 may be misinterpreted or cannot be used in some cases. Change 2: The network is forced to configure (and the UE is forced to apply) an unnecssary featureCombinationPreambles even in situations where this information is unnecessary. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Incorporated change 1 from R2-2212196. And modified the formulation of the condition compared to the original CR. Rev 2: Polishing of the field description for ssb-SharedRO-MaskIndex. |  |  |  |  |  |  |  |  |

### 6.3.2	Radio resource control information elements

#### –	FeatureCombinationPreambles

The IE FeatureCombinationPreambles associates a set of preambles with a feature combination. For parameters which can be provided in this IE, the UE applies this field value when performing Random Access using a preamble in this featureCombinationPreambles, otherwise the UE applies the corresponding value as determined by applicable Need Code, e.g. Need S. On a specific BWP, there can be at most one set of preambles associated with a given feature combination per RA Type (i.e. 4-step RACH or 2-step RACH).

FeatureCombinationPreambles information element

-- ASN1START

-- TAG-FEATURECOMBINATIONPREAMBLES-START

FeatureCombinationPreambles-r17 ::=   SEQUENCE {

featureCombination-r17                FeatureCombination-r17,

startPreambleForThisPartition-r17     INTEGER (0..63),

numberOfPreamblesPerSSB-ForThisPartition-r17 INTEGER (1..64),

ssb-SharedRO-MaskIndex-r17            INTEGER (1..15)                                           OPTIONAL, -- Need S

groupBconfigured-r17                  SEQUENCE {

ra-SizeGroupA-r17                     ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB-r17          ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA-r17        INTEGER (1..64)

}                                                                                               OPTIONAL, -- Need R

separateMsgA-PUSCH-Config-r17         MsgA-PUSCH-Config-r16                                     OPTIONAL, -- Cond MsgAConfigCommon

msgA-RSRP-Threshold-r17               RSRP-Range                                                OPTIONAL, -- Need R

rsrp-ThresholdSSB-r17                 RSRP-Range                                                OPTIONAL, -- Need R

deltaPreamble-r17                     INTEGER (-1..6)                                           OPTIONAL, -- Need R

...

}

-- TAG-FEATURECOMBINATIONPREAMBLES-STOP

-- ASN1STOP

FeatureCombinationPreambles field descriptions

deltaPreamble Power offset between msg3 or msgA-PUSCH and RACH preamble transmission. If configured, this parameter overrides msg3-DeltaPreamble or msgA-DeltaPreamble, Actual value = field value * 2 [dB] (see TS 38.213 [13], clause 7.1). If msgA-DeltaPreamble is configured in separateMsgA-PUSCH-Config-r17, this field is absent.

featureCombination Indicates which combination of features that the preambles indicated by this IE are associated with. The UE ignores a RACH resource defined by this FeatureCombinationPreambles if any feature within the featureCombination is not supported by the UE or if any of the spare fields within the featureCombination is set to true.

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on (see TS 38.321 [3], clause 5.1.2).

msgA-RSRP-Threshold The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the concerned feature combination in the BWP. If configured, this parameter overrides msgA-RSRP-Threshold-r16. If absent, the UE applies msgA-RSRP-Threshold-r16, if configured

numberOfPreamblesPerSSB-ForThisPartition It determines how many consecutive preambles are associated to the Feature Combination starting from the starting preamble(s) per SSB.

numberOfRA-PreamblesGroupA It determines how many consecutive preambles per SSB are associated to Group A starting from the starting preamble(s). The remaining preambles associated to the Feature Combination are associated to Group B

ra-SizeGroupA Transport Blocks size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.2). If this feature combination preambles are associated to a RACH-ConfigCommon-twostepRA, this field correspond to ra-MsgA-SizeGroupA, otherwise it corresponds to ra-Msg3SizeGroupA.

rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE. If this parameter is included in FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA, it corresponds to msgA-RSRP-ThresholdSSB, as defined in TS 38.321 [3]. If this parameter is included in FeatureCombinationPreambles which is included in RACH-ConfigCommon, it it corresponds to rsrp-ThresholdSSB, as defined in TS 38.321 [3].

separateMsgA-PUSCH-Config If present it specifies how the 2-step RACH preambles identified by this FeatureCombinationPreambles are mapped to a PUSCH slot separate from the one defined in MsgA-ConfigCommon-r16. If the field is absent, the UE should apply the corresponding parameter in the RACH-ConfigCommonTwoStepRA of the BWP which includes the FeatureCombinationPreambles IE.

ssb-SharedRO-MaskIndex Mask index (see TS 38.321 [3]). Indicates a subset of ROs where preambles are allocated for this feature combination. If this field is configured within FeatureCombinationPreambles which is included in RACH-ConfigCommonTwoStepRA, in case of separate ROs are configured for 4-step and 2-step random access, this field indicates a subset of ROs configured within this RACH-ConfigCommonTwoStepRA, and in case shared ROs are used for 4-step and 2-step random access, it indicates the subset of ROs configured within RACH-ConfigCommon, which are the subset of ROs configured for 2-step random access. This field is configured when there is more than one RO per SSB. If the field is absent, all ROs configured in RACH-ConfigCommon or RACH-ConfigCommonTwoStepRA containing this FeatureCombinationPreambles are shared.

startPreambleForThisPartition It defines the first preamble associated with the Feature Combination. If the UE is provided with a number N of SSB block indexes associated with one PRACH occasion, and N<1, the first preamble in each PRACH occasion is the one having the same index as indicated by this field. If N>=1, N blocks of preambles associated with the Feature Combination are defined, each having start index + startPreambleForThisPartition, where n refers to SSB block index (see TS 38.213 [13], clause 8.1).

| Conditional Presence | Explanation |
| --- | --- |
| MsgAConfigCommon | The field is optionally present, Need S, if FeatureCombinationPreambles is included in RACH-ConfigCommonTwoStepRA. Otherwise, it is absent. If the field is absent in FeatureCombinationPreambles included in RACH-ConfigCommonTwoStepRA, the UE applies MsgA-PUSCH-Config included in the corresponding MsgA-ConfigCommon. |

[Omitted unchanged parts]

#### –	RACH-ConfigCommon

The IE RACH-ConfigCommon is used to specify the cell specific random-access parameters.

RACH-ConfigCommon information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMON-START

RACH-ConfigCommon ::=               SEQUENCE {

rach-ConfigGeneric                  RACH-ConfigGeneric,

totalNumberOfRA-Preambles           INTEGER (1..63)                                                     OPTIONAL,   -- Need S

ssb-perRACH-OccasionAndCB-PreamblesPerSSB   CHOICE {

oneEighth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                   ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                     ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                         ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                        INTEGER (1..16),

eight                                       INTEGER (1..8),

sixteen                                     INTEGER (1..4)

}                                                                                                       OPTIONAL,   -- Need M

groupBconfigured                    SEQUENCE {

ra-Msg3SizeGroupA                   ENUMERATED {b56, b144, b208, b256, b282, b480, b640,

b800, b1000, b72, spare6, spare5,spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB            ENUMERATED { minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA          INTEGER (1..64)

}                                                                                                       OPTIONAL,   -- Need R

ra-ContentionResolutionTimer            ENUMERATED { sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64},

rsrp-ThresholdSSB                       RSRP-Range                                                      OPTIONAL,   -- Need R

rsrp-ThresholdSSB-SUL                   RSRP-Range                                                      OPTIONAL,   -- Cond SUL

prach-RootSequenceIndex                 CHOICE {

l839                                    INTEGER (0..837),

l139                                    INTEGER (0..137)

},

msg1-SubcarrierSpacing                  SubcarrierSpacing                                               OPTIONAL,   -- Cond L139

restrictedSetConfig                     ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

msg3-transformPrecoder                  ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

...,

[[

ra-PrioritizationForAccessIdentity-r16  SEQUENCE {

ra-Prioritization-r16                   RA-Prioritization,

ra-PrioritizationForAI-r16              BIT STRING (SIZE (2))

}                                                                                                       OPTIONAL,   -- Cond InitialBWP-Only

prach-RootSequenceIndex-r16             CHOICE {

l571                                    INTEGER (0..569),

l1151                                   INTEGER (0..1149)

}   OPTIONAL   -- Need R

]],

[[

ra-PrioritizationForSlicing-r17         RA-PrioritizationForSlicing-r17                          OPTIONAL,   -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17     SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL -- Cond AdditionalRACH

]]

}

-- TAG-RACH-CONFIGCOMMON-STOP

-- ASN1STOP

RACH-ConfigCommon field descriptions

featureCombinationPreamblesList Specifies a series of preamble partitions each associated to a combination of features and 4-step RA. The network does not configure this list to have more than 16 entries.

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on. (see TS 38.321 [3], clause 5.1.2)

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz If absent, the UE applies the SCS as derived from the prach-ConfigurationIndex in RACH-ConfigGeneric (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]). The value also applies to contention free random access (RACH-ConfigDedicated), to SI-request and to contention-based beam failure recovery (CB-BFR). But it does not apply for contention free beam failure recovery (CF-BFR) (see BeamFailureRecoveryConfig).

msg3-transformPrecoder Enables the transform precoder for Msg3 transmission according to clause 6.1.3 of TS 38.214 [19]. If the field is absent, the UE disables the transformer precoder (see TS 38.213 [13], clause 8.3).

numberOfRA-PreamblesGroupA The number of CB preambles per SSB in group A. This determines implicitly the number of CB preambles per SSB available in group B. (see TS 38.321 [3], clause 5.1.1). The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB.

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1). The value range depends on whether L=839 or L=139 or L=571 or L=1151. The length of the root sequence corresponding with the index indicated in this IE should be consistent with the one indicated in prach-ConfigurationIndex in the RACH-ConfigDedicated (if configured). If prach-RootSequenceIndex-r16 is signalled, UE shall ignore the prach-RootSequenceIndex (without suffix). For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139

ra-ContentionResolutionTimer The initial value for the contention resolution timer (see TS 38.321 [3], clause 5.1.5). Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes, and so on.

ra-Msg3SizeGroupA Transport Blocks size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.2).

ra-Prioritization Parameters which apply for prioritized random access procedure on any UL BWP of SpCell for specific Access Identities (see TS 38.321 [3], clause 5.1.1a).

ra-PrioritizationForAI Indicates whether the field ra-Prioritization-r16 applies for Access Identities. The first/leftmost bit corresponds to Access Identity 1, the next bit corresponds to Access Identity 2. Value 1 indicates that the field ra-Prioritization-r16 applies otherwise the field does not apply (see TS 23.501 [32]).

ra-PrioritizationForSlicing Parameters which apply to configure prioritized CBRA 4-step random access type for slicing.

rach-ConfigGeneric RACH parameters for both regular random access and beam failure recovery.

restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets, see TS 38.211 [16], clause 6.3.3.1.

rsrp-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]).

rsrp-ThresholdSSB-SUL The UE selects SUL carrier to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). The value applies to all the BWPs and all RACH configurations.

ssb-perRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEighth corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). See TS 38.213 [13].

totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free 4-step or 2-step random access in the RACH resources defined in RACH-ConfigCommon, excluding preambles used for other purposes (e.g. for SI request). If the field is absent, all 64 preambles are available for RA. The setting should be consistent with the setting of ssb-perRACH-OccasionAndCB-PreamblesPerSSB, i.e. it should be a multiple of the number of SSBs per RACH occasion.

| Conditional Presence | Explanation |
| --- | --- |
| AdditionalRACH | The field is mandatory present if the RACH-ConfigCommon is included in an AdditionalRACH-Config. On an initialUplinkBWP-RedCap where a non-RedCap feature shall be indicated, this field is mandatory present with at least one FeatureCombinationPreambles indicating only RedCap and the other indicating RedCap and the non-RedCap feature(s). Otherwise, it is optional, Need R. |
| InitialBWP-Only | This field is optionally present, Need R, if this BWP is the initial BWP of SpCell. Otherwise, the field is absent. |
| L139 | The field is mandatory present if prach-RootSequenceIndex L=139, otherwise the field is absent, Need S. |
| SUL | The field is mandatory present in rach-ConfigCommon in initialUplinkBWP if supplementaryUplink is configured in ServingCellConfigCommonSIB or if supplementaryUplinkConfig is configured in ServingCellConfigCommon; otherwise, the field is absent. This field is not configured in additionalRACH-Config. |

#### –	RACH-ConfigCommonTwoStepRA

The IE RACH-ConfigCommonTwoStepRA is used to specify cell specific 2-step random-access type parameters.

RACH-ConfigCommonTwoStepRA information element

-- ASN1START

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-START

RACH-ConfigCommonTwoStepRA-r16 ::=                   SEQUENCE {

rach-ConfigGenericTwoStepRA-r16                      RACH-ConfigGenericTwoStepRA-r16,

msgA-TotalNumberOfRA-Preambles-r16                   INTEGER (1..63)                                    OPTIONAL, -- Need S

msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16   CHOICE {

oneEighth                                            ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneFourth                                            ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

oneHalf                                              ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

one                                                  ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32,n36,n40,n44,n48,n52,n56,n60,n64},

two                                                  ENUMERATED {n4,n8,n12,n16,n20,n24,n28,n32},

four                                                 INTEGER (1..16),

eight                                                INTEGER (1..8),

sixteen                                              INTEGER (1..4)

}                                                                                                                   OPTIONAL, -- Cond 2StepOnly

msgA-CB-PreamblesPerSSB-PerSharedRO-r16              INTEGER (1..60)                                                OPTIONAL, -- Cond SharedRO

msgA-SSB-SharedRO-MaskIndex-r16                      INTEGER (1..15)                                                OPTIONAL, -- Need S

groupB-ConfiguredTwoStepRA-r16                       GroupB-ConfiguredTwoStepRA-r16                                 OPTIONAL, -- Need S

msgA-PRACH-RootSequenceIndex-r16                     CHOICE {

l839                                                 INTEGER (0..837),

l139                                                 INTEGER (0..137),

l571                                                 INTEGER (0..569),

l1151                                                INTEGER (0..1149)

}                                                                                                                   OPTIONAL, -- Cond 2StepOnly

msgA-TransMax-r16                                    ENUMERATED {n1, n2, n4, n6, n8, n10, n20, n50, n100, n200}     OPTIONAL, -- Need R

msgA-RSRP-Threshold-r16                              RSRP-Range                                                     OPTIONAL, -- Cond 2Step4Step

msgA-RSRP-ThresholdSSB-r16                           RSRP-Range                                                     OPTIONAL, -- Need R

msgA-SubcarrierSpacing-r16                           SubcarrierSpacing                                              OPTIONAL, -- Cond 2StepOnlyL139

msgA-RestrictedSetConfig-r16                         ENUMERATED {unrestrictedSet, restrictedSetTypeA,

restrictedSetTypeB}                                OPTIONAL, -- Cond 2StepOnly

ra-PrioritizationForAccessIdentityTwoStep-r16        SEQUENCE {

ra-Prioritization-r16                                RA-Prioritization,

ra-PrioritizationForAI-r16                           BIT STRING (SIZE (2))

}                                                                                                                   OPTIONAL, -- Cond InitialBWP-Only

ra-ContentionResolutionTimer-r16                     ENUMERATED {sf8, sf16, sf24, sf32, sf40, sf48, sf56, sf64}     OPTIONAL, -- Cond 2StepOnly

...,

[[

ra-PrioritizationForSlicingTwoStep-r17               RA-PrioritizationForSlicing-r17              OPTIONAL, -- Cond InitialBWP-Only

featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 OPTIONAL  -- Cond AdditionalRACH

]]

}

GroupB-ConfiguredTwoStepRA-r16 ::=                       SEQUENCE {

ra-MsgA-SizeGroupA                                   ENUMERATED {b56, b144, b208, b256, b282, b480, b640, b800,

b1000, b72, spare6, spare5, spare4, spare3, spare2, spare1},

messagePowerOffsetGroupB                             ENUMERATED {minusinfinity, dB0, dB5, dB8, dB10, dB12, dB15, dB18},

numberOfRA-PreamblesGroupA                           INTEGER (1..64)

}

-- TAG-RACH-CONFIGCOMMONTWOSTEPRA-STOP

-- ASN1STOP

RACH-ConfigCommonTwoStepRA field descriptions

featureCombinationPreamblesList Specifies a series of preamble partitions each associated to a combination of features and 2-step RA. The network does not configure this list to have more than 16 entries.

groupB-ConfiguredTwoStepRA Preamble grouping for 2-step random access type. If the field is absent then there is only one preamble group configured and only one msgA PUSCH configuration.

msgA-CB-PreamblesPerSSB-PerSharedRO Number of contention-based preambles used for 2-step RA type from the non-CBRA 4-step type preambles associated with each SSB for RO shared with 4-step type RA. The number of preambles for 2-step RA type shall not exceed the number of preambles per SSB minus the number of contention-based preambles per SSB for 4-step type RA. The possible value range for this parameter needs to be aligned with value range for the configured SSBs per RACH occasion in ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon. The field is only applicable for the case of shared ROs with 4-step type random access.

msgA-PRACH-RootSequenceIndex PRACH root sequence index. If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e., not within AdditionalRACH-Config), the UE applies the value in field prach-RootSequenceIndex in RACH-ConfigCommon in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE applies the corresponding value of prach-RootSequenceIndex in RACH-ConfigCommon in the same AdditionalRACH-Config. When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access. For FR2-2, only the following values are applicable depending on the used subcarrier spacing: 120 kHz: L=139, L=571, and L=1151 480 kHz: L=139, and L=571 960 kHz: L=139

msgA-RestrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets for 2-step random access type preamble. If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config), the UE applies the value in field restrictedSetConfig in RACH-ConfigCommon in the configured BWP. If the field is absent in RACH-ConfigCommonTwoStepRA in AdditionalRACH-Config, the UE applies the value of restrictedSetConfig in RACH-ConfigCommon in the same AdditionalRACH-Config. When both 2-step and 4-step type random access is configured, this field is only configured for the case of separate ROs between 2-step and 4-step type random access.

msgA-RSRP-Threshold The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the BWP.

msgA-RSRP-ThresholdSSB UE may select the SS block and corresponding PRACH resource for path-loss estimation and (re)transmission based on SS blocks that satisfy the threshold (see TS 38.213 [13]).

msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB The meaning of this field is twofold: the CHOICE conveys the information about the number of SSBs per RACH occasion. Value oneEight corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. The ENUMERATED part indicates the number of Contention Based preambles per SSB. Value n4 corresponds to 4 Contention Based preambles per SSB, value n8 corresponds to 8 Contention Based preambles per SSB, and so on. The total number of CB preambles in a RACH occasion is given by CB-preambles-per-SSB * max(1, SSB-per-rach-occasion). If the field is not configured in RACH-ConfigCommonTwoStepRA which is configured directly within a BWP (i.e. not within AdditionalRACH-Config) and both 2-step and 4-step are configured for the BWP, the UE applies the value in the field ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon. If the field is not configured in AdditionalRACH-Config and both 2-step and 4-step are configured in AdditionalRACH-Config, the UE applies the value in the field ssb-perRACH-OccasionAndCB-PreamblesPerSSB in RACH-ConfigCommon in the same AdditionalRACH-Config. The field is not present when RACH occasions are shared between 2-step and 4-step type random access in the BWP.

msgA-SSB-SharedRO-MaskIndex Indicates the subset of 4-step type ROs shared with 2-step random access type for each SSB. This field is configured when there is more than one RO per SSB. If the field is absent, and 4-step and 2-step has shared ROs, then all ROs are shared.

msgA-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2). Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 60 or 120 kHz FR2-2: 120, 480, or 960 kHz. If the field is absent, the UE applies the SCS as derived from the msgA-PRACH-ConfigurationIndex in RACH-ConfigGenericTwoStepRA (see tables Table 6.3.3.1-1, Table 6.3.3.1-2, Table 6.3.3.2-2 and Table 6.3.3.2-3, TS 38.211 [16]) in case of 2-step only BWP, otherwise the UE applies the same SCS as Msg1 derived from RACH-ConfigCommon. The value also applies to contention free 2-step random access type (RACH-ConfigDedicated).

msgA-TotalNumberOfRA-Preambles Indicates the total number of preambles used for contention-based and contention-free 2-step random access type when ROs for 2-step are not shared with 4-step. If the field is absent, and 2-step and 4-step does not have shared ROs, all 64 preambles are available for 2-step random access type.

msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step random access (see TS 38.321 [3], clauses 5.1.1). This field is only applicable when 2-step and 4-step RA type are configured and switching to 4-step type RA is supported. If the field is absent, switching from 2-step RA type to 4-step RA type is not allowed.

ra-ContentionResolutionTimer The initial value for the contention resolution timer for fallback RAR in case no 4-step random access type is configured (see TS 38.321 [3], clause 5.1.5). Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes, and so on. If both 2-step and 4-step random access type resources are configured on the BWP, then this field is absent.

ra-Prioritization Parameters which apply for prioritized random access procedure on any UL BWP of SpCell for specific Access Identities (see TS 38.321 [3], clause 5.1.1a).

ra-PrioritizationForAI Indicates whether the field ra-Prioritization-r16 applies for Access Identities. The first/leftmost bit corresponds to Access Identity 1, the next bit corresponds to Access Identity 2. Value 1 for an Access Identity indicates that the field ra-Prioritization-r16 applies, otherwise the field does not apply.

ra-PrioritizationForSlicingTwoStep Parameters which apply to configure prioritized CBRA 2-step random access type for slicing.

rach-ConfigGenericTwoStepRA 2-step random access type parameters for both regular random access and beam failure recovery.

GroupB-ConfiguredTwoStepRA field descriptions

messagePowerOffsetGroupB Threshold for preamble selection. Value is in dB. Value minusinfinity corresponds to –infinity. Value dB0 corresponds to 0 dB, dB5 corresponds to 5 dB and so on. (see TS 38.321 [3], clause 5.1.1).

numberOfRA-PreamblesGroupA The number of CB preambles per SSB in group A for idle/inactive or connected mode. The setting of the number of preambles for each group should be consistent with msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB or msgA-CB-PreamblesPerSSB-PerSharedRO if configured.

ra-MsgA-SizeGroupA Transport block size threshold in bits below which the UE shall use a contention-based RA preamble of group A. (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| 2Step4Step | The field is mandatory present if both 2-step random access type and 4-step random access type are configured in the BWP, otherwise the field is not present. The field is mandatory present in msgA-ConfigCommon field in AdditionalRACH-Config if both 2-step random access type and 4-step random access type are configured for the same feature combination in the BWP. |
| 2StepOnlyL139 | The field is mandatory present if msgA-PRACH-RootSequenceIndex L=139 and no 4-step random access type is configured, otherwise the field is absent, Need S. |
| 2StepOnly | The field is mandatory present in msgA-ConfigCommon field in BWP-UplinkCommon if rach-ConfigCommon field is absent in this BWP-UplinkCommon, otherwise the field is optionally present in msgA-ConfigCommon field in BWP-UplinkCommon, Need S. The field is mandatory present in msgA-ConfigCommon field in AdditionalRACH-Config if rach-ConfigCommon field is absent in this AdditionalRACH-Config, otherwise the field is optionally present in msgA-ConfigCommon field in AdditionalRACH-Config, Need S. |
| AdditionalRACH | The field is mandatory present if the RACH-ConfigCommon is included in an AdditionalRACH-Config. On an initialUplinkBWP-RedCap where a non-RedCap feature shall be indicated, this field is mandatory present with at least one FeatureCombinationPreambles indicating only RedCap and the other indicating RedCap and the non-RedCap feature(s). Otherwise, it is optional, Need R. |
| InitialBWP-Only | This field is optionally present, Need R, if this BWP is the initial BWP of SpCell. Otherwise, the field is absent. |
| SharedRO | The field is mandatory present if the 2-step random access type occasions are shared with 4-step random access type, otherwise the field is not present. |