3GPP TSG RAN meeting #98-e								RP-222935

Electronic Meeting, December 12-16, 2022

## Status Report to TSG

Agenda item:			9.2.10

| WI / SI Name | Study on XR Enhancements for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_XR_enh |  |  |  |  |
| Unique ID | 940087 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-220285 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2023 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 80% | Core part: N/A | Performance Part: N/A | Testing part: |  |

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Benoist Sébire |
|  | Company | Nokia |
|  | Email | benoist.sebire@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

The TR is not complete and only version 1.0.0 is submitted for information to this meeting. Note this is the first version submitted to RAN as it could not be submitted last time since it was not 60% complete then.

Since both SA2 and SA4 extended their studies by one quarter, RAN2 also needs one more quarter to conclude the study on XR awarenes. The TR will be submitted for approval then.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #110bis-e (October 2022)

XR specific power saving techniques

Agreement

For enhancement of CDRX to align with XR traffic periodicity (i.e., Issue 1-1)

- Prioritize semi-static solutions

  - FFS: Whether dynamic solutions will be also needed

Conclusion

“Retransmission-less CG for UL pose transmission (Item 3.3-5)” is a RAN2 issue, leave the discussion to RAN2, RAN1 does not further investigate the issue.

- Note: how to capture evaluation results and findings will be separately discussed

Conclusion

RAN1 does not further study jitter handling by LP-WUS (Item 2.2-4) in Rel-18 XR SI

- Note: how to capture evaluation results and findings will be separately discussed

Conclusion

In addition to the values for jitter in Table 5.1-2 in TR 38.838, the following statistical parameters for jitter can also be optionally evaluated in Rel-18 XR SI.

- Note: This optional assumption is not applicable to the evaluation of 90 FPS and above

| Parameter | unit | Optional value for evaluation |
| --- | --- | --- |
| Mean | ms | 0 |
| STD | ms | 5 |
| Truncation range | ms | [-8, 8] |

Conclusion

RAN1 deprioritizes DCP indicated SSSG switching (Item 3.3-4)

- Note: how to capture evaluation results and findings will be separately discussed

XR-specific capacity enhancements techniques

Agreement

To study whether/how the enhanced CG candidate techniques are necessary and beneficial for improving XR capacity, focus at least on the following techniques:

- Dynamic indication of the unused CG PUSCH occasion(s) or resource(s) by the UE

- Increase CG PUSCH transmission occasions in a duration

Conclusion

No further discussion in RAN1 for Rel-18 XR to extend the support of legacy single DCI scheduling multi-PDSCHs for FR2-2, to other SCS in FR1/FR2-1.

Conclusion

The capacity gain performance results in R1-2208661, R1-2209658 and R1-2209198 corresponding to enhancements based on multi-PDSCH scheduling by a single DCI are captured in XR SI TR

Conclusion

Study on enhancement for CBG based HARQ-ACK feedback reporting is down-priorotized in RAN1 XR SI.

Conclusion

The following proposed enhancements techniques to improve XR capacity performance are down-priorotized in RAN1 XR SI:

- (P3-5-3) Study on PHR enhancement based on XR traffic arrival periodicity or UL pose periodicity.

- (P3-5-4) Study mechanism of packet dropping based on the PDB requirement, to avoid resource waste due to the out-of-date packets.

Agreement

- For further study the mechanisms to enable HARQ retransmission of a TB on a different cell than the cell of the initial TB transmission for CA operation on TDD cells, consider at least the following:

  - Capacity performance evaluation results

  - Complexity analysis and RAN2 impact

Conclusion

- Study of soft HARQ-ACK and Delta MCS in RAN1 XR SI for improving XR capacity is down-priortized.

- Note: The corresponding capacity gain performance results in R1-2210003, R1-2208377 and R1-2203607 are captured in XR SI TR.

Conclusion

- Study on enhanced CQI based on CBG transmission, and study on enhanced CQI based on DMRS for improving XR capacity are down-priortized in RAN1 XR SI.

- Note: The corresponding capacity gain performance results in R1-2208402 and R1-2209536 are captured in XR SI TR.

Conclusion

Study on Cooperative MIMO via DL interference probing based on SRS enhancement for improving XR capacity is down prioritized in RAN1 XR SI.

Note: The corresponding capacity gain performance results in R1-2208377 are captured in XR SI TR.

Conclusion

No consensus to continue study on differentiation of XR multiple flows based on CG enhancement in RAN1 XR SI.

Conclusion

No consensus to continue study of multi-bits SR mechanisms for capacity improvement of XR traffic in RAN1 XR SI.

RAN1 #111 (November 2022)

XR specific power saving techniques

Conclusion

There is no consensus on the following proposals in Table 1 for CDRX enhancements and Table 2 for PDCCH monitoring enhancements.

Table 1: CDRX enhancements

Proposal for CDRX enhancements

Proposal 2.2: support dynamic periodicity alignment between CDRX and XR traffic

Proposal 2.3-1: support additional active time by UE extending DRX active time if UE does not receive XR data within current active time

Proposal 2.3-2: support additional active time by gNB using dynamic signaling such as a DCI to trigger additional On Duration if the data packet arrives after the On Duration expires

Proposal 2.4: support non-uniform PMOs within CDRX On Duration

Proposal 2.5: support two-stage CDRX On Duration

Proposal 2.6-1: support early stopping of On-Duration timer by inactivity timer expiration

Proposal 2.6-2: support early stopping of On-Duration timer after a time window after the reception of XR data

Proposal 2.7: support multiple active CDRX configurations

Proposal 2.8: support dynamic grant enhancement with XR-specific pre-scheduling

Proposal 2.9: support SPS+DG with UE power saving scheme

Proposal 2.10: Support XR-specific plyaoutDelayForMediaStartup for XR UE power saving enhancement

Table 2: PDCCH monitoring adaptation enhancements

Proposals for PDCCH monitoring adaptation enhancements

Proposal 3.1: support PDCCH monitoring resume if UE transmits NACK after PDCCH skipping starts

Proposal 3.2-1: support PDCCH skipping duration enhancements by additional PDCCH skipping durations (>3)

Proposal 3.2-2: support PDCCH skipping duration enhancements by PDCCH skipping till the start of next potential data arrival

Proposal 3.3-1: support non-scheduling DCI based PDCCH skipping indication

Proposal 3.3-2: support continuous PDCCH skipping, i.e., UE continuously skips the PDCCH MOs until the DCI is successfully decoded at the time of packet arrival

Proposal 3.4: support an implicit SSSG at the start of drx-OnDuration and another SSSG applies when a PDCCH for data traffic is received, with search space set monitoring pattern aligned with DRX cycle

XR-specific capacity enhancements techniques

Agreement

TPs for XR TR in R1-2212732 is endorsed in principle.

Conclusion

- The capacity gain performance results in R1-2210907 (Huawei/HiSilicon) corresponding to enhancements based on UL delay aware scheduling are captured in XR SI TR.

Conclusion

- The capacity gain performance results in R1-2211906 (ZTE/Sanechips) and R1-2211175 (CATT) corresponding to BSR enhancements are captured in XR SI TR.

Conclusion

- The capacity gain performance results in R1-2211175 (CATT) corresponding to enhanced CG+DG scheme are captured in XR SI TR.

  - Note: SU-MIMO is used for baseline and MU-MIMO for the proposed enhancement.

Conclusion

- The capacity gain performance results in R1-2211175 (CATT) corresponding to XR-specific playoutDelayForMediaStartup enhancement scheme are captured in XR SI TR.

Agreement

Support dynamic indication of the unused CG PUSCH occasion(s) based on UCI (e.g., CG-UCI or a new UCI) by the UE

Agreement

Support multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

Conclusion

No consensus on the following:

Support dynamic indication of the unused CG PUSCH resource(s) based on UCI (e.g., CG-UCI or a new UCI) by the UE

Conclusion 3-1:

- Deprioritize discussion on HARQ retransmission of a TB on a different cell than the cell of the initial TB transmission for CA operation on TDD cells in XR agenda.

Conclusion 5-5:

- Deprioritize in RAN1, the study of XR-specific playoutDelayForMediaStartup for XR awareness scheduling to improve XR capacity enhancement.

Conclusion 4-1:

- The capacity gain performance results in R1-2210907 and R1-2212254 corresponding to enhancements on RRM measurement are captured in XR SI TR.

Conclusion

No consensus on the following:

Enhancements on RRM to relax scheduling restriction for at least for intra-frequency RRM without MGs in FR2 and for inter-frequency RRM with MGs

Agreement

The following TP for section 5.3.1 is endorsed in principle for TR 38.835:

The following enhancements for configured grant based transmission are recommended: - Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration; - Dynamic indication of unused CG PUSCH occasion(s) based on UCI (e.g., CG-UCI or a new UCI) by the UE. The corresponding capacity performance evaluation results are available in Annex B.1.6. The evaluation results for other proposed and studied capacity enhancement schemes are available in Annex B.1.

#### 2.1.2	Remaining Open issues

In accordance of the SID:

-	There is no remaining open issue.

## 2.2	RAN2

#### 2.2.1	Agreements

Agreements from RAN2#119bis-e meeting:

-	From RAN2 viewpoint, the following information would be useful for PDU set handling in UL and DL:

-	Semi-static information (from CN to RAN): At least PSER and PSDB;

-	Dynamic information: At least identifying which PDU belongs to which data burst/PDU set is also needed, including means to determine at least PDU set boundaries.

-	Capture the models 1a/b, 2a/b (from R2-2209777) in TR and indicate what is possible in current specifications and how. FFS how LCH options work in each case

-	SDAP maps each data packet in a PDU set to a single PDCP SDU, as in legacy (i.e. each PDU is only mapped to a single SDU).

-	HARQ and RLC re-/transmissions for XR traffic are done as in legacy (i.e. they are not based on XR PDU sets).

-	For UE transmitter, the PDCP discard should be performed per PDU set basis.

-	For UE transmitter, the PDCP discard is managed per SDU for PDU set, the PDCP entity discards all PDCP SDUs associated with the PDU set.

-	At least RRC pre-configuration and switching of configurations of DRX could be considered for enhancements of XR power saving. Other solutions are not precluded and can be further discussed.

-	Introduce new BS table(s) to reduce the quantisation errors (e.g. for high bit rates). FFS how new BSR tables are created and how they impact BSR formats (can be discussed in WI phase).

-	Delay information consists of at least “remaining time”.

-	RAN2 considers a delay information is useful for XR. FFS if dynamic reporting from UE to network (e.g. via BSR) is needed, or whether PSDB is sufficient. If we have delay information, it needs to distinguish how much data is buffered for which delay value. Stage-3 details (e.g. what’s contained, how the triggering is done) can be discussed in the WI phase.

-	If we have delay information reporting, RAN2 aims to define how the UE determines the “remaining time” in the delay information.

-	Current CG configurations can be reused for UL XR traffic. FFS if enhancements are needed (RAN1 is already discussing something). RAN2 can discuss this in the next meeting.

-	RAN2 can discuss potential enhancement to provide some assistant information on UL XR traffic for CG configurations at the gNB. FFS whether TSCAI can already provide all necessary information.

-	RAN2 discuss whether additional traffic or QoS related information on downlink traffic beyond what has been agreed by SA2 needs to be provided to RAN for UE power savings.

-	RAN2 study what traffic and QoS related information on uplink traffic (e.g. counterpart of what has been agreed by SA2) should be provided to RAN for UE power savings and how the information may be provided to RAN.

-	Capture in TR that traffic parameters and Jitter are semi-static info.

-	Can capture also SA2 agreements related to how they impact RAN2.

Agreements from RAN2#120 meeting:

-	N1N excluded.

-	Splitting DRB into multiple LCH (DC like) FFS.

-	Should try to understand why we would need to treat PDU sets differently over the radio and why different PDU sets are muxed over same flows. Also need to understand need for reordering. LS to SA2/SA4 sent in R2-2213351.

-	Agree that UE identifies PDU Sets / Bursts.

-	In-band marking not needed. Further information considered if BSR is not enough.

-	Handling of discard FFS.

-	Regarding making LCP delay aware:

-	If delay-aware LCP is introduced, need the ability to turn it off;

-	SRBs not impacted.

-	Not considered further unless fundamental issues are identified.

-	RAN2 to support timer-based discarding of UL transmit side of PDCP PDU/SDUs of a PDU set. FFS how this is modelled in PDCP specification, can be discussed in WI phase.

-	RAN2 aims to allow XR frame rates that correspond to non-integer periodicities in at least semi-static manner (e.g. RRC). Details can be left to WI phase.

-	RAN2 thinks we need one or more additional BSR table(s) for XR. FFS whether these are static (=specified) or dynamic (e.g. generated, differs according to some RRC parameter), can be discussed in WI phase.

-	RAN2 will introduce data volume information associated with delay information (e.g. remaining time) in a MAC CE. FFS if this is extension of BSR or new format. FFS how to do that (e.g. what exactly is reported) and how to ensure this information is up-to-date e.g. considering UL scheduling delay.

-	RAN2 needs to discuss additional BSR triggering conditions to allow timely availability of buffer status information at gNB. This can be discussed in WI phase.

-	RAN2 sees some benefit from CG to XR services. RAN2 will address enhancements triggered by RAN1 work. -	RAN2 agrees some assistance information can be beneficial (e.g. periodicity, packet size). RAN2 assumes baseline could be TSCAI (pending SA2 conclusions), can discuss during WI phase whether something additional is needed on top of that. If any assistance information is needed, its definition should be standardized.

-	RAN2 thinks all information may not be always available at UE application.

#### 2.2.2	Open Issues

In accordance of the SID:

-	Finish identifying mechanisms for XR-awareness.

## 3.	Detailed progress in SA WGs since last TSG meeting (for all involved WGs)

## 3.1	SA2

The SA2 status report is available in S2-2211189. The study was completed at SA2#154.

## 4.	References

TR 38.835

Latest version in R2-2213229.

RAN1#109-e

- R1-2204673, TR 38.835 Skeleton for Study on XR enhancements for NR, Rapporteur (Nokia)

- R1-2205053, Work plan for Rel-18 SI on XR enhancements for NR, Qualcomm Incorporated

- R1-2205329, TR 38.835 Skeleton for Study on XR enhancements for NR, Rapporteur (Nokia)

- R1-2205419, [Draft] LS on draft TR 38.835 skeleton, Nokia

- R1-2205420, LS on draft TR 38.835 skeleton, RAN1, Nokia

- R1-2205443, LS on draft TR 38.835 skeleton, RAN1, Nokia

- R1-2203131, Discussion on XR-specific power saving techniques, Huawei, HiSilicon

- R1-2203348, Discussion on XR specific power saving techniques, Spreadtrum Communications

- R1-2203484, UE Power saving techniques for XR, CATT

- R1-2203585, Discussion on XR specific power saving enhancements, vivo

- R1-2203606, Discussion on XR specific power saving techniques, ZTE, Sanechips

- R1-2203638, Discussion on power saving enhancements for XR, Ericsson

- R1-2203666, Discussion on XR enhancement for NR, China Telecom

- R1-2203744, Considerations on power saving techniques for XR, Sony

- R1-2203927, Considerations on XR-specific Power Savings, Samsung

- R1-2203940, Discussion on XR specific power saving techniques, NEC

- R1-2204028, Discussion on XR specific power saving techniques, OPPO

- R1-2204123, Discussion on XR specific power saving enhancements, InterDigital, Inc.

- R1-2204177, XR specific power saving techniques, TCL Communication Ltd.

- R1-2204264, Views on XR specific power saving techniques, Apple

- R1-2204326, Discussion on XR-specific power saving techniques, CMCC

- R1-2204400, Discussion on XR specific power saving techniques, NTT DOCOMO, INC.

- R1-2204414, XR-specific power saving techniques, Lenovo

- R1-2204444, Discussion on XR specific power saving techniques, ITRI

- R1-2204633, Discussion on XR-specific power saving techniques, LG Electronics

- R1-2204655, Discussion on power saving techniques for XR, ETRI

- R1-2204674, Discussion on XR-specific power saving enhancements, Nokia, Nokia Shanghai Bell

- R1-2204698, On XR specific power saving techniques, MediaTek Inc.

- R1-2204818, Discussion on power saving enhancements for XR applications, Intel Corporation

- R1-2205054, Power saving techniques for XR, Qualcomm Incorporated

- R1-2205055, Moderator Summary#1 on XR specific power saving techniques, Qualcomm Incorporated

- R1-2205176, Power saving techniques for XR, Qualcomm Incorporated

- R1-2205410, Moderator Summary#2 on XR specific power saving techniques, Moderator (Qualcomm)

- R1-2205411, Moderator Summary#3 on XR specific power saving techniques, Moderator (Qualcomm)

- R1-2205412, Final Moderator Summary on XR specific power saving techniques, Moderator (Qualcomm)

- R1-2205413, Draft Reply LS on UE Power Saving for XR and Media Services, Moderator (Qualcomm)

- R1-2205530, Draft Reply LS on UE Power Saving for XR and Media Services, Moderator (Qualcomm)

- R1-2205531, Reply LS on UE Power Saving for XR and Media Services, RAN1, Qualcomm

- R1-2203065, XR Capacity Evaluation and Enhancements, FUTUREWEI

- R1-2203132, Discussion on XR-specific capacity  enhancements techniques, Huawei, HiSilicon

- R1-2203349, XR capacity consideration, Spreadtrum Communications

- R1-2203485, NR enhancement for XR capacity improvement, CATT

- R1-2203586, Discussion on XR specific capacity enhancements, vivo

- R1-2203607, Discussion on XR specific capacity enhancements techniques, ZTE, Sanechips

- R1-2203639, Discussion on capacity enhancements for XR, Ericsson

- R1-2203689, Discussion on XR-specific capacity enhancements, NEC

- R1-2203745, Considerations on capacity enhancements techniques for XR, Sony

- R1-2203928, Considerations on XR Capacity Improvements, Samsung

- R1-2203934, Discussion on XR specific capacity improvement techniques, Panasonic

- R1-2204029, Discussion on XR specific capacity enhancements techniques, OPPO

- R1-2204124, Discussion on XR specific capacity enhancements, InterDigital, Inc.

- R1-2204129, Discussion on XR specific capacity enhancements techniques, III

- R1-2204178, XR-specific capacity enhancements techniques, TCL Communication Ltd.

- R1-2204265, Views on XR specific capacity enhancements techniques, Apple

- R1-2204327, Discussion on XR-specific capacity enhancements techniques, CMCC

- R1-2204401, Discussion on XR specific capacity improvement enhancements, NTT DOCOMO, INC.

- R1-2204415, XR-specific capacity enhancement techniques, Lenovo

- R1-2204634, Discussion on XR-specific capacity enhancement techniques, LG Electronics

- R1-2204656, Discussion on capacity enhancements techniques for XR, ETRI

- R1-2204675, Discussion on XR-specific capacity enhancements, Nokia, Nokia Shanghai Bell

- R1-2204699, On XR specific capacity improvement enhancements, MediaTek Inc.

- R1-2204759, Discussion on potential SPS enhancements for XR, CEWiT

- R1-2204819, Discussion on capacity enhancements for XR applications, Intel Corporation

- R1-2205056, Capacity enhancement techniques for XR, Qualcomm Incorporated

- R1-2205072, Discussion on XR-specific capacity enhancements techniques, FGI

- R1-2205265, FL Summary#1 – Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2205266, FL Summary#2 – Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2205267, FL Summary#3 – Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2205268, FL Summary#4 – Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2203486, XR awareness scheduling and QoS control, CATT

- R1-2203587, Discussion on other aspects for XR specific RAN enhancements, vivo

- R1-2203608, Consideration about XR services, ZTE, Sanechips

- R1-2203640, Discussion on XR-Awareness, Ericsson

- R1-2204125, Discussion on XR-Awareness, InterDigital, Inc.

- R1-2204266, Considerations on enhancements for XR, Apple

- R1-2204635, Other aspects of XR enhancements for NR, LG Electronics

- R1-2204676, Performance results of XR-related enhancements, Nokia, Nokia Shanghai Bell

- R1-2204820, Views on XR specific RAN enhancement in QoS, III

- R1-2204908, Discussion on XR-specific capacity and power issues based on SA2 outcome, Huawei, HiSilicon

RAN2#119

- R2-2206917, LS on draft TR 38.835 skeleton (R1-2205443; contact: Nokia), RAN1

- R2-2206923, Reply LS on UE Power Saving for XR and Media Services (R1-2205531; contact: Qualcomm), RAN1

- R2-2206964, LS on QoS support with PDU Set granularity (S2-2201803; contact: Intel), SA2

- R2-2206966, LS on UE Power Saving for XR and Media Services (S2-2203418; contact: Nokia), SA2

- R2-2206969, LS Reply on QoS support with PDU Set granularity (S4-220505; contact: Qualcomm), SA4

- R2-2207042, Draft reply LS on UE power savings for XR and media services, Qualcomm Incorporated

- R2-2207043, Draft reply LS on RAN feedback for low latency, Qualcomm Incorporated

- R2-2207371, Work Plan for Rel-18 SI on XR Enhancements for NR, Nokia, Qualcomm (Rapporteurs)

- R2-2207372, XR TR Structure, Nokia (Rapporteur)

- R2-2207373, TR 83.835 v001, Nokia (Rapporteur)

- R2-2207374, TR 83.835 v002, Nokia (Rapporteur)

- R2-2207375, XR Overview TP, Nokia (Rapporteur)

- R2-2207376, Draft LS to SA4 on Pose Information for XR, Nokia (Rapporteur)

- R2-2208316, Discussion of SA2 LS on UE Power Saving for XR and Media Services, Meta Ireland

- R2-2207044, XR-awareness in RAN, Qualcomm Incorporated

- R2-2207117, XR awareness: RAN2 areas of interest, assumptions, and inputs to SA2 LS, Intel Corporation

- R2-2207118, Solution Directions for XR Specific Differentiated Traffic Handling and Packet Dropping, Intel Corporation

- R2-2207197, Discussion on XR-awareness, NTT DOCOMO, INC.

- R2-2207210, Discussing on XR-awareness in RAN, Xiaomi Communications

- R2-2207366, Discussion on XR-awareness, TCL Communication

- R2-2207377, XR Awareness in SA2, Nokia (Rapporteur)

- R2-2207429, Considerations on XR-awareness, QoS-metrics, and XR-specific traffic handling, Apple

- R2-2207489, Discussion on XR-awareness, InterDigital, Inc.

- R2-2207508, XR requirements and issues, CATT

- R2-2207680, Discussion on RAN awareness of XR traffic characteristics, Spreadtrum Communications

- R2-2207697, Discusion of XR awareness in RAN, Lenovo

- R2-2207756, Discussion on XR-awareness, vivo

- R2-2207761, Discussion on XR-awareness, III

- R2-2207780, Discussion on XR-awareness, KT Corp.

- R2-2207801, Discussion on XR-awareness in RAN, OPPO

- R2-2207831, Considerations on XR awareness, Sony

- R2-2207893, XR-awareness techniques, Google Inc.

- R2-2207926, First steps for XR handling, Vodafone GmbH

- R2-2207980, RAN level protocol enhancements for XR awareness, ZTE Corporation, Sanechips

- R2-2207991, Views on XR-specific handling at RAN, Huawei, HiSilicon

- R2-2207998, On RAN awareness of XR traffic characteristics, MediaTek Inc.

- R2-2208021, Draft LS on first steps for XR handling, Vodafone GmbH

- R2-2208223, RAN behaviour for XR-awareness QoS, ETRI

- R2-2208259, Discussion on XR awareness, Samsung

- R2-2208313, Discussion on XR-Awareness RAN , Meta Ireland

- R2-2208321, Discussion on XR-awareness, LG Electronics Inc.

- R2-2208443, Consideration on XR-awareness in RAN, CMCC

- R2-2208618, Discussion on XR traffic characteristics, Futurewei

- R2-2208677, Discussion on XR-awareness, Ericsson

- R2-2206986, Discussion on XR-specific power saving, FGI

- R2-2206996, Discussion on CDRX enhancement for XR, OPPO

- R2-2207045, Power saving enhancements for XR, Qualcomm Incorporated

- R2-2207084, Consideration on CDRX enhancement for XR, KDDI Corporation

- R2-2207119, Study of C-DRX enhancements for XR traffic, Intel Corporation

- R2-2207171, Discussion on XR power saving, III

- R2-2207211, Discussing on XR-specific power saving, Xiaomi Communications

- R2-2207294, C-DRX enhancement for XR-specific power saving, NEC Telecom MODUS Ltd.

- R2-2207368, Discussion on XR-specific power saving, TCL Communication

- R2-2207409, Discussion on XR-specific power saving techniques, DENSO CORPORATION

- R2-2207430, Power Saving for Periodical XR Traffics, Apple

- R2-2207490, Discussion on XR-specific power saving, InterDigital, Inc.

- R2-2207509, Consideration on power saving for XR services, CATT

- R2-2207569, DRX enhancement for power saving in XR, LG Electronics Inc.

- R2-2207673, Discussion on power saving in XR, Spreadtrum Communications

- R2-2207757, Discussion on XR-specific power saving, vivo

- R2-2207832, Considerations on XR specific C-DRX power saving enhancements, Sony

- R2-2207846, Discussion on power saving scheme for XR, Samsung

- R2-2207864, XR-specific power saving techniques, Google Inc.

- R2-2207877, Discussion on Power saving enhancements, Lenovo

- R2-2207888, Discussion on XR-specific power saving techniques, Huawei, HiSilicon

- R2-2207979, Power Saving enhancements for XR, ZTE Corporation, Sanechips

- R2-2207999, C-DRX enhancements for XR, MediaTek Inc.

- R2-2208019, XR power saving RAN1 study overview and suggestions for RAN2 focus, Nokia, Nokia Shanghai Bell (Rapporteur)

- R2-2208020, XR Power Saving enhancements, Nokia, Nokia Shanghai Bell

- R2-2208295, Draft Reply LS on UE Power Saving for XR and Media Services , Nokia

- R2-2208440, Discussion on XR-specific power saving, CMCC

- R2-2208620, Impacts of XR traffics on UE power saving, Futurewei

- R2-2208680, Discussion on power saving enhancements for XR, Ericsson

- R2-2207050, Capacity enhancements for XR, Qualcomm Israel Ltd.

- R2-2207173, Discussion on the UL enhancement for XR, ITRI

- R2-2207212, Discussing on XR-specific capacity improvements, Xiaomi Communications

- R2-2207295, XR-specific capacity improvements, NEC Telecom MODUS Ltd.

- R2-2207367, Discussion on XR-specific capacity improvements, TCL Communication

- R2-2207378, XR Capacity Improvements, Nokia, Nokia Shanghai Bell

- R2-2207410, Discussion on XR-specific capacity improvements, DENSO CORPORATION

- R2-2207431, Capacity Enhancement based on XR PDU Set Characteristics, Apple

- R2-2207491, Discussion on XR-specific capacity improvements, InterDigital, Inc.

- R2-2207510, XR-specific Capacity Improvement, CATT

- R2-2207674, Some improvements on XR capacity, Spreadtrum Communications

- R2-2207719, XR-specific capacity improvements, MediaTek Beijing Inc.

- R2-2207758, Discussion on XR Capacity Enhancements, vivo

- R2-2207762, Discussion on XR-specific capacity improvements, III

- R2-2207785, Discussion on XR capacity improvements, KT Corp.

- R2-2207802, Discussion on XR-specific capacity improvements, OPPO

- R2-2207833, Considerations on XR specific capacity improvements, Sony

- R2-2207878, Discussion on XR-specific capacity enhancements, Lenovo

- R2-2207921, XR-specific capacity improvements, Google Inc.

- R2-2207978, Capacity enhancements of XR support in RAN, ZTE Corporation, Sanechips

- R2-2208232, Scheduling method for XR packets, ETRI

- R2-2208302, Discussion on XR-specific capacity improvement, Samsung

- R2-2208401, Discussion on Capacity enahancement for XR, LG Electronics Inc.

- R2-2208417, Support for XR-specific scheduler enhancements, AT&T

- R2-2208422, Discussion on XR-specific capacity improvements, CMCC

- R2-2208498, Discussion on XR-specific capacity enhancements techniques, Huawei, HiSilicon

- R2-2208621, Layer 2 based XR capacity enhancement, Futurewei

- R2-2208676, XR capacity enhancements, Ericsson

RAN1#110

- R1-2207831, Moderator Summary#1 on XR specific power saving techniques, Moderator (Qualcomm Incorporated)

- R1-2207832, Moderator Summary#2 on XR specific power saving techniques, Moderator (Qualcomm Incorporated)

- R1-2207833, Final Moderator Summary on XR specific power saving techniques, Moderator (Qualcomm Incorporated)

- R1-2205843, XR specific power saving techniques, TCL Communication Ltd.

- R1-2205877, Discussion on XR-specific power saving techniques, Huawei, HiSilicon

- R1-2205916, Discussion on power saving enhancements for XR, Ericsson

- R1-2206007, Discussion on XR specific power saving techniques, Spreadtrum Communications

- R1-2206061, Discussion on XR specific power saving enhancements, vivo

- R1-2206105, Discussion on XR power saving techniques, III

- R1-2206131, Considerations on power saving techniques for XR, Sony

- R1-2206225, XR-specific power saving enhancements, Nokia, Nokia Shanghai Bell

- R1-2206244, Discussion on XR specific power saving techniques, NEC

- R1-2206328, Discussion on XR specific power saving techniques, OPPO

- R1-2206384, UE Power saving techniques for XR, CATT

- R1-2206436, Discussion on XR specific power saving techniques, Panasonic

- R1-2206495, Power saving techniques for XR, Rakuten Mobile, Inc

- R1-2206518, XR-specific power saving techniques, Lenovo

- R1-2206601, Discussion on XR specific power saving techniques, Intel Corporation

- R1-2206629, Discussions on techniques for XR Power Saving, Xiaomi

- R1-2206702, Discussion on XR specific power saving enhancement for NR, China Telecom

- R1-2206846, Considerations on XR-specific Power Savings, Samsung

- R1-2206931, Discussion on XR-specific power saving techniques, CMCC

- R1-2206959, Discussion on power saving techniques for XR, ETRI

- R1-2206965, On XR-specific power saving techniques, Google Inc.

- R1-2207008, On XR specific power saving techniques, MediaTek Inc.

- R1-2207042, Discussion on XR-specific power saving techniques, LG Electronics

- R1-2207061, Evaluation on XR specific power saving techniques, ZTE, Sanechips

- R1-2207253, Power saving techniques for XR, Qualcomm Incorporated

- R1-2207263, Discussion on XR specific power saving techniques, InterDigital, Inc.

- R1-2207351, XR specific power saving techniques, Apple

- R1-2207426, Discussion on XR specific power saving techniques, NTT DOCOMO, INC.

- R1-2207860, Discussion on XR specific power saving enhancements, Moderator (vivo)

- R1-2205751, XR Capacity Evaluation and Enhancements, FUTUREWEI

- R1-2205844, XR-specific capacity enhancements techniques, TCL Communication Ltd.

- R1-2205878, Discussion on XR-specific capacity  enhancements techniques, Huawei, HiSilicon

- R1-2205917, Discussion on capacity enhancements for XR, Ericsson

- R1-2206008, Discussion on XR specific capacity enhancements techniques, Spreadtrum Communications

- R1-2206062, Discussion on XR specific capacity enhancements, vivo

- R1-2206132, Discussion on XR-specific capacity enhancements, Sony

- R1-2206226, XR-specific capacity enhancements, Nokia, Nokia Shanghai Bell

- R1-2206245, Discussion on XR-specific capacity enhancements, NEC      Withdrawn

- R1-2206329, Discussion on XR specific capacity enhancements techniques, OPPO

- R1-2206385, NR enhancement for XR capacity improvement, CATT

- R1-2206475, Discussion on XR-specific capacity enhancements, NEC

- R1-2206519, XR-specific capacity enhancement techniques, Lenovo

- R1-2206602, Discussion on XR specific capacity enhancement techniques, Intel Corporation

- R1-2206703, Discussion on XR specific capacity enhancement for NR, China Telecom

- R1-2206847, Considerations on XR Capacity Improvements, Samsung

- R1-2206932, Discussion on XR-specific capacity enhancements techniques, CMCC

- R1-2206960, Discussion on SPS and CG enhancements for XR capacity improvement, ETRI

- R1-2206964, On XR-specific capacity enhancements techniques, Google Inc.

- R1-2207009, On XR specific capacity improvement enhancements, MediaTek Inc.

- R1-2207043, Discussion on XR-specific capacity enhancement techniques, LG Electronics

- R1-2207062, XR specific capacity enhancements, ZTE, Sanechips

- R1-2207077, Discussion on XR specific capacity enhancements, CEWiT

- R1-2207095, Disscusion on XR-specific capacity enhancements techniques, FGI

- R1-2207254, Capacity enhancement techniques for XR, Qualcomm Incorporated

- R1-2207264, Discussion on XR-specific capacity enhancements techniques, InterDigital, Inc.

- R1-2207301, Discussion on XR-specific capacity improvements, Rakuten Mobile, Inc

- R1-2207352, XR-specific capacity enhancements techniques, Apple

- R1-2207427, Discussion on XR specific capacity improvement enhancements, NTT DOCOMO, INC.

- R1-2207820, Moderator Summary#1 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2207821, Moderator Summary#2 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2207822, Moderator Summary#3 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2207823, Moderator Summary#4 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2207718, Discussion on XR specific capacity enhancements, vivo

RAN2#119bis

- R2-2209552, Work Plan for Rel-18 SI on XR Enhancements for NR, Nokia, Qualcomm (Rapporteurs)

- R2-2209553, SA2 Status for XR, Nokia (Rapporteur)

- R2-2209554, SA4 Status for XR, Nokia (Rapporteur)

- R2-2211041, LS on XR and Media Services (S2-2209979; contact: vivo), SA2

- R2-2210005, Discussion on handling and usage of PDU sets and data bursts related information in RAN2, Samsung R&D Institute India

- R2-2210008, Discussion on PDU-Sets handling, KT Corp.

- R2-2210021, Discussion on PDU Set awareness, OPPO

- R2-2210108, Considerations on PDU Set handling, Fujitsu

- R2-2210201, Handling of XR PDU sets in RAN, Huawei, HiSilicon

- R2-2210213, Considerations on XR awarness, Sony

- R2-2210360, Discussion on PDU Sets and Data Bursts for XR, Google Inc.

- R2-2210381, Discussion XR-Awareness for XR services, Meta

- R2-2210508, Considerations on PDU sets and Data bursts in RAN, CMCC

- R2-2210593, Discussion on PDU sets and data bursts, LG Electronics Inc.

- R2-2210603, Discussion on PDU Sets mapping to DRBs, TCL Communication

- R2-2210619, Discussion on PDU set parameters for XR-awareness, III

- R2-2210628, Discussion on PDU sets and data bursts, NTT DOCOMO, INC.

- R2-2210689, Discussion on PDU Set and Data Burst, Ericsson

- R2-2209414, On mapping PDU Sets for XR, Futurewei

- R2-2209450, Discuss on PDU Sets, Qualcomm Incorporated

- R2-2209467, PDU sets characterization and mapping, CATT

- R2-2209485, Discussion on PDU sets and data bursts for XR awareness, vivo

- R2-2209555, PDU Set Identification Details, Nokia, Nokia Shanghai Bell

- R2-2209631, DRB mapping for XR traffic, Intel Corporation

- R2-2209635, XR related information for awareness in RAN, Intel Corporation

- R2-2209644, PDU-set to DRB mapping for XR, ZTE Corporation, Sanechips

- R2-2209668, Discussion on QoS support with PDU Set granularity, Xiaomi Communications

- R2-2209686, Discussion on PDU sets and data bursts, InterDigital, Inc.

- R2-2209698, Support for XR-aware scheduling, AT&T

- R2-2209777, PDU Sets and Mapping of QoS flows and DRBs for XR, Apple

- R2-2209846, Discussion on PDU Set for XR-awareness, NEC Corporation

- R2-2209873, Number of DRBs for XR, VODAFONE Group Plc

- R2-2209937, Discussion on PDU sets and data burst awareness in RAN, Lenovo

- R2-2209987, Discussion on XR-awareness info, Spreadtrum Communications

- R2-2210013, Discussion on LCP impact, Samsung

- R2-2210022, Discussion on PDU prioritization, OPPO

- R2-2210046, Discussion on the LCP enhancements for XR, ITRI

- R2-2210202, Discussion about XR-awareness impacts on LCP, Huawei, HiSilicon

- R2-2210361, Discussion on PDU prioritization, Google Inc.

- R2-2210507, Impact on PDU Prioritization by XR Awareness, CMCC

- R2-2210536, Discussion on traffic prioritization of XR traffic, Beijing Xiaomi Mobile Software

- R2-2210560, Discussion on the prioritization for XR, LG Electronics Inc.

- R2-2210620, Discussion on PDU prioritization for XR-awareness, III

- R2-2210649, On PDU prioritisation, MediaTek Inc.

- R2-2210688, Discussion on PDU Prioritization, Ericsson

- R2-2209451, Discussion on PDU prioritization, Qualcomm Incorporated

- R2-2209468, Prioritization of XR traffic, CATT

- R2-2209486, Discussion on PDU prioritization for XR awareness, vivo

- R2-2209556, LCP Impacts for XR, Nokia, Nokia Shanghai Bell

- R2-2209632, Handling and in-sequence delivery of XR packets with different priorities, Intel Corporation

- R2-2209646, PDU-set prioritization for XR, ZTE Corporation, Sanechips

- R2-2209687, Discussion on PDU prioritization, InterDigital, Inc.

- R2-2209778, Enhancements for Traffic Prioritization in XR, Apple

- R2-2209889, Discussion on PDU prioritization, Lenovo

- R2-2209990, Some LCP enhancements based on the traffic awareness, Spreadtrum Communications

- R2-2210023, Discussion on PDU discard, OPPO

- R2-2210203, Discussion on PDU discarding for XR traffic, Huawei, HiSilicon

- R2-2210362, Discussion on PDUs Discarding, Google Inc.

- R2-2210371, Discussion on PDU discard for XR video traffic, Futurewei

- R2-2210375, PDU Set Handling, Meta

- R2-2210506, Considerations on PDU Discarding of XR Traffic, CMCC

- R2-2210559, Discussion on the discard and retransmission for XR, LG Electronics Inc.

- R2-2210627, Discussion on PDU discard, NTT DOCOMO, INC.

- R2-2210650, On the need and impact of PDU discard in the RAN, MediaTek Inc.

- R2-2210687, Discussion on PDU Discard, Ericsson

- R2-2210815, TP to 38.835, Nokia

- R2-2210814, TP to 38.835, Nokia

- R2-2209452, Discussion on PDU discard, Qualcomm Incorporated

- R2-2209469, PDU Discard for XR Services, CATT

- R2-2209487, Discussion on PDU discard for XR awareness, vivo

- R2-2209557, PDU Discard for XR, Nokia, Nokia Shanghai Bell

- R2-2209586, PDU Set and PDCP Discard for XR, Samsung

- R2-2209633, Packet discard optimizations for XR traffic, Intel Corporation

- R2-2209645, PDU-set discard functionality for XR, ZTE Corporation, Sanechips

- R2-2209669, Discussing on PDU discarding of XR traffic, Xiaomi Communications

- R2-2209688, Discussion on PDU discard, InterDigital, Inc.

- R2-2209779, Enhancements for PDU Discarding in XR, Apple

- R2-2209888, Discussion on PDU discarding, Lenovo

- R2-2209993, PDU discard of XR traffic, Spreadtrum Communications

- R2-2210009, DRX enhancement for power saving in XR, LG Electronics Inc.

- R2-2210061, Discussion on power saving scheme for XR, Samsung

- R2-2210144, Discussion on DRX enhancements for XR-specific power saving, CMCC

- R2-2210186, DRX enhancements for XR, Nokia, Nokia Shanghai Bell

- R2-2210189, Candidate Solutions on C-DRX Enhancements, NEC Telecom MODUS Ltd.

- R2-2210214, Considerations on XR specific C-DRX power saving enhancements, Sony

- R2-2210359, DRX Enhancement for XR, Google Inc.

- R2-2210501, C-DRX enhancements for XR-specific power saving, DENSO CORPORATION

- R2-2210651, C-DRX enhancements for XR, MediaTek Inc.

- R2-2210690, Discussion on RAN2-specific CDRX aspects, Ericsson

- R2-2210692, Discussion on solutions for DRX cycle mismatch and jitter, Ericsson

- R2-2210705, Discussion on DRX enhancements for XR-specific power saving, III

- R2-2209453, DRX enhancements for XR, Qualcomm Incorporated

- R2-2209470, DRX Enhancements to Address Cycle Mismatch, CATT

- R2-2209471, Serving XR traffic with minimum power consumption, CATT

- R2-2209488, Discussion on DRX enhancements for XR power saving, vivo

- R2-2209502, On DRX enhancements for handling non-integer traffic periodicity, Futurewei

- R2-2209511, Discussion on CDRX enhancement for XR based on outputs from RAN1, OPPO

- R2-2209512, Discussion on CDRX enhancement for Power saving, OPPO

- R2-2209515, Analysis on XR traffic characteristics for C-DRX enhancement, Huawei, HiSilicon

- R2-2209516, Further discussion on C-DRX enhancements for XR, Huawei, HiSilicon

- R2-2209634, C-DRX enhancements for XR traffic, Intel Corporation

- R2-2209649, DRX enhancements for XR, ZTE Corporation, Sanechips

- R2-2209670, Discussing on XR-specific C-DRX enhancements, Xiaomi Communications

- R2-2209689, Discussion on DRX enhancements, InterDigital, Inc.

- R2-2209780, On C-DRX Enhancement for HARQ Handling in XR, Apple

- R2-2209938, Discussion of DRX enhancement, Lenovo

- R2-2210010, Enhancement in legacy power saving for XR, LG Electronics Inc.

- R2-2210062, Discussion on XR-awareness for power saving scheme design, Samsung

- R2-2210145, Discussion on XR-specific power saving, CMCC

- R2-2210187, Multiple CG configurations for XR, Nokia, Nokia Shanghai Bell

- R2-2209454, Non-DRX power saving enhancements for XR, Qualcomm Incorporated

- R2-2209455, Information to RAN for UE power savings, Qualcomm Incorporated

- R2-2210825, Information to RAN for UE power savings, Qualcomm Incorporated

- R2-2209489, XR specific information for RAN power saving, vivo

- R2-2209648, Other Power Saving enhancements for XR, ZTE Corporation, Sanechips

- R2-2209690, Discussion on PDCCH monitoring enhancements, InterDigital, Inc.

- R2-2209781, XR-Specific Power Saving for Configured Scheduling, Apple

- R2-2209939, Discussion of PDCCH monitoring enhancement, Lenovo

- R2-2209982, Discussion on power saving in XR, Spreadtrum Communications

- R2-2210024, Discussion on feedback enhancement, OPPO

- R2-2210047, Discussion on the UE feedback enhancements for XR, ITRI

- R2-2210150, Consideration on BSR enhancement for XR, CMCC

- R2-2210191, Feedback Enhancements for Capacity Improvement, NEC Telecom MODUS Ltd.

- R2-2210215, Considerations on BSR, Sony

- R2-2210502, Discussion on UE feedback enhancements for XR capacity, DENSO CORPORATION

- R2-2210537, Discussion on BSR enhancement for XR-specific capacity improvement, Huawei, HiSilicon

- R2-2210599, Discussion on BSR enahancement for timing information in XR, LG Electronics Inc.

- R2-2210621, Discussion on Feedback enhancements for XR-specific capacity improvements, III

- R2-2210686, Discussion on BSR enhancements, Ericsson

- R2-2210816, Report of [AT119bis-e][207][XR] BSR enhancements for XR (NN), NN

- R2-2210817, TP to 38.835 on BSR enhancements for XR, NN

- R2-2209456, UE feedback enhancements for capacity improvement, Qualcomm Incorporated

- R2-2209472, BSR enhancement for XR capacity, CATT

- R2-2209490, Discussion on feedback enhancements for XR-specific capacity improvements, vivo

- R2-2209517, Discussion on buffer status report for XR, Google Inc.

- R2-2209558, BSR for XR, Nokia, Nokia Shanghai Bell

- R2-2209591, BSR enhancement for XR capacity, MediaTek Inc.

- R2-2209636, Enhancements to Buffer Status Reporting for XR Traffic, Intel Corporation

- R2-2209650, UE feedback enhancements for XR capacity, ZTE Corporation, Sanechips

- R2-2209672, Discussing on UE feedback enhancements for XR capacity, Xiaomi Communications

- R2-2209691, Discussion on XR-specific feedback enhancements, InterDigital, Inc.

- R2-2209782, BSR Enhancements for XR, Apple

- R2-2209828, Discussion on BSR enhancements for XR, Samsung

- R2-2209890, Discussion on UE Feedback enhancements, Lenovo

- R2-2209983, Some feedback enhancements on XR capacity, Spreadtrum Communications

- R2-2210025, Discussion on scheduling enhancement, OPPO

- R2-2210151, Consideration on scheduler enhancement for XR, CMCC

- R2-2210216, Considerations on XR specific capacity improvements, Sony

- R2-2210358, Scheduling Enhancement for XR, Google Inc.

- R2-2210483, Discussion on CG enhancement, Samsung

- R2-2210541, Discussion on scheduling enhancement for XR traffic, Huawei, HiSilicon

- R2-2210600, Discussion on Scheduling enahancement for XR, LG Electronics Inc.

- R2-2210604, Further discussion on DG for XR uplink traffic transmission, TCL Communication

- R2-2210691, Discussion on Scheduling enhancements, Ericsson

- R2-2209457, Scheduling enhancements for capacity improvement, Qualcomm Incorporated

- R2-2209473, Discussion on CG enhancements, CATT

- R2-2209491, Discussion on scheduling enhancements XR-specific capacity improvements, vivo

- R2-2209559, Capacity Enhancements for XR, Nokia, Nokia Shanghai Bell

- R2-2209592, Scheduling enhancement for XR capacity, MediaTek Inc.

- R2-2209647, Scheduling enhancements for XR, ZTE Corporation, Sanechips

- R2-2209673, Discussing on XR-specific scheduling enhancements, Xiaomi Communications

- R2-2209692, Discussion on scheduling enhancements, InterDigital, Inc.

- R2-2209783, Considerations of Scheduling Enhancement for XR, Apple

- R2-2209907, Scheduling and measurement gap enhancements for XR traffic, Intel Corporation

- R2-2209940, Discussion of scheduling enhancement, Lenovo

- R2-2209991, Some enhancements on XR scheduling, Spreadtrum Communications

- R2-2209994, Enhancement to measurement gap, Spreadtrum Communications

RAN2#120

- R2-2211138, LS on XR and Media Services (S2-2209979; contact: vivo), SA2

- R2-2211490, Reply LS to SA2 on XR, vivo

- R2-2211595, Work Plan for Rel-18 SI on XR Enhancements for NR, Nokia, Qualcomm (Rapporteurs)

- R2-2211596, SA2 Status for XR, Nokia, Qualcomm (Rapporteurs)

- R2-2212189, Discussion on network exposure of congestion level of RAN node, Huawei, HiSilicon

- R2-2212908, TR 38.835 v031, Nokia (Rapporteur)

- R2-2212989, Reply LS to SA2 on XR, vivo

- R2-2213226, Reply LS to SA2 on XR, RAN2

- R2-2213229, TR 38.835 v031, Nokia (Rapporteur)

- R2-2211177, Discussions on PDU Sets, Qualcomm Incorporated

- R2-2211378, DRB mapping for XR specific requirement, Intel Corporation

- R2-2211436, XR awareness for PDU sets and bursts, CATT

- R2-2211437, On the PDU set mapping options, CATT

- R2-2211491, Discussion on XR awareness and per-QoS flow/DRB congestion, vivo

- R2-2211524, PDU set to DRB mapping for XR, ZTE Corporation, Sanechips

- R2-2211584, Discussion on QoS support with PDU Set granularity, Xiaomi Communications

- R2-2211597, Mapping of PDU Set, QoS Flow and DRB, Nokia, Nokia Shanghai Bell

- R2-2211718, PDU Set based QoS, Apple

- R2-2211848, Discussions on L2 structure of XR, Fujitsu

- R2-2211957, Discussion on PDU Set awareness, OPPO

- R2-2211995, Discussion on PDU sets mapping model, NTT DOCOMO, INC.

- R2-2212039, Discussion on PDU sets and data burst awareness in RAN, Lenovo

- R2-2212163, Discussion on PDU sets and data bursts, Spreadtrum Communications

- R2-2212188, Further discussion on PDU set handling, Huawei, HiSilicon

- R2-2212329, Discussion on PDU Sets and Data Bursts for XR, Google Inc.

- R2-2212471, Discussion on PDU sets and data bursts, InterDigital, Inc.

- R2-2212534, Discussion on PDU Set for XR-awareness, NEC Corporation

- R2-2212608, Discussion on Uplink XR-Awareness for XR services, Meta USA

- R2-2212649, Discussion on PDU set to DRB mapping, Samsung

- R2-2212695, Discussion on PDU set mapping for XR-awareness, III

- R2-2212704, Considerations on PDU sets and Data bursts in RAN, CMCC

- R2-2212852, Discussion on XR awareness and PDU Set, LG Electronics Inc.

- R2-2212889, Discussion on PDU Sets and Data Burst, Ericsson

- R2-2212993, [DRAFT] LS on PDU Set Handling, Nokia

- R2-2213225, LS on PDU Set Handling, Nokia

- R2-2213351, LS on PDU Set Handling, RAN2

- R2-2211178, Discussion on PDU prioritization, Qualcomm Incorporated

- R2-2211379, Enhancements to provide differentiated XR handling, Intel Corporation

- R2-2211438, Considerations on PDU Prioritization, CATT

- R2-2211492, Discussion on PDU prioritization for XR awareness, vivo

- R2-2211526, PDU-set prioritization for XR, ZTE Corporation, Sanechips

- R2-2211585, Discussion on traffic prioritization of XR traffic, Xiaomi Communications

- R2-2211598, LCP Impacts for XR, Nokia, Nokia Shanghai Bell

- R2-2211719, Enhancements for Traffic Prioritization in XR, Apple

- R2-2211923, Considerations on XR PDU prioritization, Sony

- R2-2211958, Discussion on PDU prioritization, OPPO

- R2-2212130, Discussion on PDU prioritization, Lenovo

- R2-2212190, Discussion about XR-awareness impacts on LCP, Huawei, HiSilicon

- R2-2212205, Discussion on LCP impact, Samsung

- R2-2212330, Discussion on PDU prioritization, Google Inc.

- R2-2212472, Discussion on PDU prioritization, InterDigital, Inc.

- R2-2212703, Impact on PDU Prioritization by XR Awareness, CMCC

- R2-2212759, Discussion on the prioritization for XR, LG Electronics Inc.

- R2-2212888, Discussion on PDU Prioritization, Ericsson

- R2-2212899, On potential impacts to LCP mechanisms for XR, Futurewei

- R2-2211179, Discussion on PDU discard, Qualcomm Incorporated

- R2-2211380, Packet discard for XR traffic, Intel Corporation

- R2-2211439, PDU Discard of XR services, CATT

- R2-2211493, Discussion on PDU discard for XR awareness, vivo

- R2-2211525, PDU-set discard functionality for XR, ZTE Corporation, Sanechips

- R2-2211587, Discussing on PDU discarding of XR traffic, Xiaomi Communications

- R2-2211599, PDU Discard for XR, Nokia, Nokia Shanghai Bell

- R2-2211720, Packet Discarding and Reordering Enhancements for XR, Apple

- R2-2211859, On PSDB and PDU discard, MediaTek Inc.

- R2-2211924, Considerations on XR PDU discard, Sony

- R2-2211959, Discussion on PDU discard, OPPO

- R2-2211993, Discussion on PDU discard, NTT DOCOMO, INC.

- R2-2212098, PDU Set and PDCP Discard Handling, Samsung R&D Institute India

- R2-2212129, Discussion on PDU discarding, Lenovo

- R2-2212164, PDU discard of XR traffic, Spreadtrum Communications

- R2-2212191, Discussion on PDU discarding for XR traffic, Huawei, HiSilicon

- R2-2212331, Discussion on PDUs Discarding, Google Inc.

- R2-2212473, Discussion on PDU discard, InterDigital, Inc.

- R2-2212537, Discussion on PDU discard for XR awareness, NEC Corporation

- R2-2212582, Discussion on PDU Discard, Meta USA

- R2-2212702, Considerations on PDU Discarding of XR Traffic, CMCC

- R2-2212758, Discussion on the discard and retransmission, LG Electronics Inc.

- R2-2212887, Discussion on PDU Discard, Ericsson

- R2-2211180, DRX enhancements for XR, Qualcomm Incorporated

- R2-2211278, Further discussion on C-DRX enhancements for XR, Huawei, HiSilicon

- R2-2211297, Discussion on CDRX enhancement for XR service, OPPO

- R2-2211298, Discussion on CDRX enhancement for Power saving, OPPO

- R2-2211381, C-DRX enhancements for XR traffic, Intel Corporation

- R2-2211426, Considerations on XR jitter handling, KDDI Corporation

- R2-2211440, Enhancements for XR Power Saving, CATT

- R2-2211494, Discussion on DRX enhancements for XR power saving, vivo

- R2-2211529, DRX enhancements for XR, ZTE Corporation, Sanechips

- R2-2211588, Discussing on XR-specific C-DRX enhancements, Xiaomi Communications

- R2-2211715, DRX Enhancements for XR, Apple

- R2-2211775, DRX enhancements for XR, Nokia, Nokia Shanghai Bell

- R2-2211860, C-DRX enhancements for XR, MediaTek Inc.

- R2-2211925, Considerations on XR specific C-DRX power saving enhancements, Sony

- R2-2212040, Discussion of DRX enhancement, Lenovo

- R2-2212237, Candidate solutions on C-DRX enhancement, NEC

- R2-2212249, On DRX enhancements for handling non-integer traffic periodicity, Futurewei

- R2-2212332, DRX Enhancement for XR, Google Inc.

- R2-2212474, Discussion on DRX enhancements, InterDigital, Inc.

- R2-2212579, DRX enhancement for power saving in XR, LG Electronics Inc.

- R2-2212631, Discussion on DRX enhancements, CMCC

- R2-2212770, C-DRX enhancements for XR-specific power saving, DENSO CORPORATION

- R2-2212812, Discussion on power saving scheme for XR, Samsung

- R2-2212886, Discussion on DRX enhancements, Ericsson

- R2-2211181, Non-DRX power saving enhancements for XR, Qualcomm Incorporated

- R2-2211277, Analysis on XR traffic characteristics for C-DRX enhancement, Huawei, HiSilicon

- R2-2211382, Information in RAN for XR traffic and congestion, Intel Corporation

- R2-2211495, Uplink XR Traffic Information for Power Saving, vivo

- R2-2211528, Other Power Saving enhancements for XR, ZTE Corporation, Sanechips

- R2-2211721, PDU Set Parameters and Descriptors, Apple

- R2-2211776, QoS related information in Uplink, Nokia, Nokia Shanghai Bell

- R2-2212041, Discussion of other power saving enhancement, Lenovo

- R2-2212171, Discussion on power saving in XR, Spreadtrum Communications

- R2-2212172, Align the uplink and downlink transmission for XR, Spreadtrum Communications

- R2-2212206, Discussion on power saving impact of packet discard operation, Samsung

- R2-2212475, Discussion on other XR power enhancements, InterDigital, Inc.

- R2-2212580, Information on uplink traffic for power saving, LG Electronics Inc.

- R2-2212632, Discussion on Information for UE power saving, CMCC

- R2-2212891, Discussion on UL and DL traffic information for power saving, Ericsson

- R2-2211182, UE feedback enhancements for capacity improvement, Qualcomm Incorporated

- R2-2211275, BSR feedback enhancements for XR, Dell Technologies

- R2-2211319, Discussion on multi-modal synchronization for XR, TCL Communication Ltd.

- R2-2211383, Enhancements to Buffer Status Reporting for XR traffic, Intel Corporation

- R2-2211394, Discussion on BSR enhancements for XR, Samsung

- R2-2211441, Further consideration on BSR, CATT

- R2-2211496, Discussion on feedback enhancements for XR-specific capacity improvements, vivo

- R2-2211530, fFeedback enhancements for XR capacity, ZTE Corporation, Sanechips

- R2-2211590, Discussing on UE feedback enhancements for XR capacity, Xiaomi Communications

- R2-2211600, BSR for XR, Nokia, Nokia Shanghai Bell

- R2-2211716, Considerations for BSR Enhancements, Apple

- R2-2211926, Considerations on BSR, Sony

- R2-2211960, Discussion on feedback enhancement, OPPO

- R2-2211975, Discussion on BSR enhancement for XR-specific capacity improvement, Huawei, HiSilicon

- R2-2212139, Discussion of UE feedback enhancements, Lenovo

- R2-2212173, BSR enhancement on XR, Spreadtrum Communications

- R2-2212235, BSR enhancements for XR, NEC

- R2-2212318, BSR enhancement for XR capacity, MediaTek Inc.

- R2-2212476, Discussion on XR-specific feedback enhancements, InterDigital, Inc.

- R2-2212517, Discussion on BSR enhancements, Futurewei

- R2-2212636, Enhancement on BSR for XR-specific capacity improvement, CMCC

- R2-2212715, Discussion on Feedback enhancements for XR-specific capacity improvements, III

- R2-2212771, Discussion on UE feedback enhancements for XR capacity, DENSO CORPORATION

- R2-2212783, draft Reply LS on XR and Media Services on Network exposure, Xiaomi Communications

- R2-2212787, Discussion on BSR enhancement for delay information in XR, LG Electronics Inc.

- R2-2212885, Discussion on BSR enhancements, Ericsson

- R2-2211183, Scheduling enhancements for capacity improvement, Qualcomm Incorporated

- R2-2211276, CG scheduling enhancements for XR, Dell Technologies

- R2-2211384, Scheduling enhancements for XR traffic, Intel Corporation

- R2-2211442, Further consideration on XR-specific capacity improvement, CATT

- R2-2211497, Discussion on scheduling enhancements XR-specific capacity improvements, vivo

- R2-2211527, Scheduling enhancements for XR, ZTE Corporation, Sanechips

- R2-2211592, Discussing on XR-specific scheduling enhancements, Xiaomi Communications

- R2-2211601, Capacity Enhancements for XR, Nokia, Nokia Shanghai Bell

- R2-2211717, Configured Scheduling and UE-Assistance Information for XR, Apple

- R2-2211927, Considerations on XR specific capacity improvements, Sony

- R2-2211928, UL Scheduling enhancement for XR traffic and evaluation results, Sony

- R2-2211952, Discussion on SR configuration for XR uplink traffic transmission, TCL Communication

- R2-2211961, Discussion on scheduling enhancement, OPPO

- R2-2212042, Discussion of scheduling enhancement, Lenovo

- R2-2212174, Scheduling enhancement on XR, Spreadtrum Communications

- R2-2212236, UE assistance information for CG configuration at gNB, NEC

- R2-2212319, Scheduling enhancement for XR capacity, MediaTek Inc.

- R2-2212333, Scheduling Enhancement for XR, Google Inc.

- R2-2212477, Discussion on scheduling enhancements, InterDigital, Inc.

- R2-2212637, Enhancement on CG for XR-specific capacity improvement, CMCC

- R2-2212650, Discussion on UE Assistance Information for CG configuration, Samsung

- R2-2212788, Discussion on XR-specific Scheduling enahancement, LG Electronics Inc.

- R2-2212890, Discussion on Scheduling enhancements, Ericsson

- R2-2212936, Discussion on scheduling enhancements, NTT DOCOMO, INC.

RAN1 #110bis-e

- R1-2210337, Moderator Summary#1 on XR specific power saving techniques, Moderator (Qualcomm Incorporated)

- R1-2210338, Moderator Summary#2 on XR specific power saving techniques, Moderator (Qualcomm Incorporated)

- R1-2208401, Discussion on power saving enhancements for XR, Ericsson

- R1-2208420, Discussion on XR-specific power saving techniques, Huawei, HiSilicon

- R1-2208566, Discussion on XR specific power saving techniques, Spreadtrum Communications

- R1-2208660, Discussion on XR specific power saving enhancements, vivo

- R1-2208781, Discussion on XR specific power saving enhancement for NR, China Telecom

- R1-2208862, Discussion on XR specific power saving techniques, OPPO

- R1-2208952, UE Power saving techniques for XR, CATT

- R1-2208999, XR specific power saving techniques, TCL Communication Ltd.

- R1-2209069, Discussion on XR specific power saving techniques, Intel Corporation

- R1-2209112, Considerations on XR specific power saving techniques, Sony

- R1-2209128, XR-specific power saving techniques, Lenovo

- R1-2209197, Evaluation on XR specific power saving techniques, ZTE, Sanechips

- R1-2209263, Discussions on techniques for XR Power Saving, xiaomi

- R1-2209354, Discussion on XR-specific power saving techniques, CMCC

- R1-2209387, Discussion on XR specific power saving techniques, Panasonic

- R1-2209410, Discussion on power saving techniques for XR, ETRI

- R1-2209427, Discussion on XR specific power saving techniques, NEC

- R1-2209456, Discussion on XR-specific power saving techniques, LG Electronics

- R1-2209517, On XR specific power saving techniques, MediaTek Inc.

- R1-2209535, XR-specific power saving enhancements, Nokia, Nokia Shanghai Bell

- R1-2209597, XR specific power saving techniques, Apple

- R1-2209619, Power saving techniques for XR, Rakuten Symphony

- R1-2209636, On XR-specific power saving techniques, Google Inc.

- R1-2209657, Discussion on XR specific power saving techniques , InterDigital, Inc.

- R1-2209748, Considerations on Power Savings for XR, Samsung

- R1-2209919, Discussion on XR specific power saving techniques, NTT DOCOMO, INC.

- R1-2210002, Power saving techniques for XR, Qualcomm Incorporated

- R1-2210084, Further Discussion on XR power saving, III

- R1-2210410, Moderator Summary#1 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2210411, Moderator Summary#2 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2210412, Moderator Summary#3 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2210413, Moderator Summary#4 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2208377, XR Capacity Evaluation and Enhancements, FUTUREWEI

- R1-2208402, Discussion on capacity enhancements for XR, Ericsson

- R1-2208421, Discussion on XR-specific capacity enhancements techniques, Huawei, HiSilicon

- R1-2208661, Discussion on XR specific capacity enhancements, vivo

- R1-2208782, Discussion on XR specific capacity enhancement for NR, China Telecom

- R1-2208863, Discussion on XR specific capacity enhancements techniques, OPPO

- R1-2208953, NR enhancement for XR capacity improvement, CATT

- R1-2209000, XR-specific capacity enhancements techniques, TCL Communication Ltd.

- R1-2209070, Discussion on XR specific capacity enhancement techniques, Intel Corporation

- R1-2209113, Considerations on XR-specific capacity enhancements, Sony

- R1-2209129, XR-specific Capacity Enhancement Techniques, Lenovo

- R1-2209156, Discussion on XR-specific capacity enhancements, NEC

- R1-2209198, XR specific capacity enhancements, ZTE, Sanechips

- R1-2209355, Discussion on XR-specific capacity enhancements techniques, CMCC

- R1-2209388, Discussion on XR capacity enhancement techniques , Panasonic

- R1-2209457, Discussion on XR-specific capacity enhancement techniques, LG Electronics

- R1-2209518, On XR specific capacity improvement enhancements, MediaTek Inc.

- R1-2209536, XR-specific capacity enhancements, Nokia, Nokia Shanghai Bell

- R1-2209598, XR-specific capacity enhancements techniques, Apple

- R1-2209620, Discussion on XR-specific capacity improvements, Rakuten Symphony

- R1-2209642, On XR-specific capacity enhancements techniques, Google Inc.

- R1-2209658, Discussion on XR-specific capacity enhancements techniques, InterDigital, Inc.

- R1-2209749, Considerations on Capacity Improvements for XR, Samsung

- R1-2209920, Discussion on XR specific capacity improvement enhancements, NTT DOCOMO, INC.

- R1-2210003, Capacity enhancement techniques for XR, Qualcomm Incorporated

RAN1 #111

- R1-2212698, Moderator Summary#1 on XR specific power saving techniques, Moderator (Qualcomm Incorporated)

- R1-2212699, Moderator Summary#2 on XR specific power saving techniques, Moderator (Qualcomm Incorporated)

- R1-2212700, Moderator Summary#3 on XR specific power saving techniques, Moderator (Qualcomm Incorporated)

- R1-2212701, Final Moderator Summary on XR specific power saving techniques, Moderator (Qualcomm Incorporated)

- R1-2210906, Discussion on XR-specific power saving techniques, Huawei, HiSilicon

- R1-2210922, Discussion on power saving enhancements for XR, Ericsson

- R1-2211024, Discussion on XR specific power saving enhancements, vivo

- R1-2211174, UE Power saving techniques for XR, CATT

- R1-2211246, Discussion on XR specific power saving techniques, Spreadtrum Communications

- R1-2211341, Discussions on techniques for XR Power Saving, xiaomi

- R1-2211389, Discussion on XR specific power saving techniques, Intel Corporation

- R1-2211490, Discussion on XR specific power saving techniques, OPPO

- R1-2211536, Discussion on XR specific power saving enhancement for NR, China Telecom

- R1-2211539, XR specific power saving techniques, TCL Communication Ltd.

- R1-2211551, XR-specific power saving enhancements, Nokia, Nokia Shanghai Bell

- R1-2211566, Discussion on XR-specific power saving techniques, ETRI

- R1-2211624, Discussion on XR-specific power saving techniques, Sony

- R1-2211653, Discussion on XR specific power saving techniques, Panasonic

- R1-2211697, Discussion on XR-specific power saving techniques, CMCC

- R1-2211752, Discussion on XR specific power saving techniques, NEC

- R1-2211781, XR-specific power saving techniques, Lenovo

- R1-2211826, On XR specific power saving techniques, Apple

- R1-2211842, Discussion on XR specific power saving techniques, InterDigital, Inc.

- R1-2211905, Evaluation on XR specific power saving techniques, ZTE, Sanechips

- R1-2211920, Further Discussion on XR power saving, III

- R1-2212000, Discussion on XR specific power saving techniques, NTT DOCOMO, INC.

- R1-2212062, Considerations on Power Savings for XR, Samsung

- R1-2212134, Power saving techniques for XR, Qualcomm Incorporated

- R1-2212253, On XR specific power saving techniques, MediaTek Inc.

- R1-2212305, Discussion on XR-specific power saving techniques, LG Electronics

- R1-2212387, On XR-specific power saving techniques, Google Inc.

- R1-2212606, Moderator Summary#1 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2212607, Moderator Summary#2 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2212608, Moderator Summary#3 - Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2212609, Moderator Summary#4 (Final) – Study on XR Specific Capacity Improvements, Moderator (Ericsson)

- R1-2210849, XR Capacity Evaluation and Enhancements, FUTUREWEI

- R1-2210907, Discussion on XR-specific capacity enhancements techniques, Huawei, HiSilicon

- R1-2210923, Discussion on capacity enhancements for XR, Ericsson

- R1-2211025, Discussion on XR specific capacity enhancements, vivo

- R1-2211175, NR enhancement for XR capacity improvement, CATT

- R1-2211415, Discussion on XR specific capacity enhancement techniques, Intel Corporation

- R1-2211491, Discussion on XR specific capacity enhancements techniques, OPPO

- R1-2211540, XR-specific capacity enhancements techniques, TCL Communication Ltd.

- R1-2211552, XR-specific capacity enhancements, Nokia, Nokia Shanghai Bell

- R1-2211625, XR-specific capacity enhancements and evaluation results, Sony

- R1-2211654, Discussion on XR capacity enhancement techniques, Panasonic

- R1-2211698, Discussion on XR-specific capacity enhancements techniques, CMCC

- R1-2211782, XR-specific Capacity Enhancement Techniques, Lenovo

- R1-2211827, On XR-specific capacity enhancements techniques, Apple

- R1-2211843, Discussion on XR-specific capacity enhancements techniques, InterDigital, Inc.

- R1-2211906, XR specific capacity enhancements, ZTE, Sanechips

- R1-2212001, Discussion on XR specific capacity improvement enhancements, NTT DOCOMO, INC.

- R1-2212063, Considerations on Capacity Improvements for XR, Samsung

- R1-2212135, Capacity enhancement techniques for XR, Qualcomm Incorporated

- R1-2212254, On XR specific capacity improvement enhancement, MediaTek Inc.

- R1-2212275, Discussion on XR-specific capacity enhancements techniques, KT Corp.

- R1-2212306, Discussion on XR-specific capacity enhancement techniques, LG Electronics

- R1-2212365, Discussion on XR-specific capacity enhancements, NEC

- R1-2212386, On XR-specific capacity enhancements techniques, Google Inc.