3GPP TSG-RAN5 Meeting #98	R5-231058

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1573 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of reference sensitivity for 21A_n28A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Requirement in 7.3B.2.3 for DC_21A_n28A is added to 38.101-3 but 38.521-3 does not contain it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce requirement in 7.3B.2.3 for DC_21A_n28A to 38.521-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement in 7.3B.2.3 for DC_21A_n28A cannot be declared. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

7.3B.2	Reference sensitivity for EN-DC

7.3B.2.0	Minimum Conformance Requirements of Reference sensitivity for EN-DC

<Unchanged sections skipped>

7.3B.2.0.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

7.3B.2.0.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.0.3.1-1 with uplink configuration of the aggressor band (low) specified in Table 7.3B.2.0.3.1-2.

Table 7.3B.2.0.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1, 3 | n772, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 2 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | n782, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 3 | n782, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 5 | n786,7 |  | 10.5 | 8.9 | 7.8 |  |  | 5.4 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| 5 | n776, 7, 17 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 | 2.8 | 2.3 | 2.1 | 1.4 |
|  | n774, 5, 17 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 8 | n418,9 |  | 13 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 |  | 4.3 | 3.9 | 3.5 |
| 8 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| 8 | n794,5 |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| 12 | n668,9,10 | 10 | 7.5 | 6.2 | 5.5 |  |  | 2.4 |  |  |  |  |  |  |
| 12 | n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 13 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 18, 19 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 28 | n512,13 | 27.8 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n513 | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 | 7.3B.2.0.3.1-1 |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 20 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| n28 | 2116 | N/A | N/A | N/A |  |  |  |  |  |  |  |  |  |  |
| 26 | n418,9 |  | 10.3 | 8.4 | 7.4 |  |  | 5 | 4.3 | 3.9 |  | 3.1 | 2.7 |  |
| 26 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| n28 | 18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
| n71 | 211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
|  | 212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
| n71 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |  |
| 66 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 | n782, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n66 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_66_n78, and DC_25_n77. NOTE 14: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 from TS 36.101-1 apply unless otherwise specified). NOTE 15: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 16: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL. This band is subject to 2nd harmonic fall in B21 also which MSD is not specified. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

7.3B.2.0.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

7.3B.2.0.3.5.0	General

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex M  the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [5] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [5] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [5], with parameters specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.0.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.0.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration |  | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1_n3 |  | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  |  | n3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
| DC_1A_n8A |  | 1 | 1965 | 5 | 25 | 2155 | 6.0 | IMD4 |
|  |  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
| DC_1A_n77A |  | 1 | 1950 | 5 | 25 | 2140 | 29.8 | IMD23 |
| DC_1A_SUL_n77A- |  |  |  |  |  |  | 32.54 |  |
| n84A |  | n77, n78 | 4090 | 10 | 50 | 4090 | N/A | N/A |
| DC_1A_n77A |  | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
| DC_1A-SUL_n77A-n84A |  |  |  |  |  |  | 10.74 |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A |  | n77, n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_1A_n78A, |  | 1 | 1950 | 5 | 25 | 2140 | 8.0 | IMD43 |
| DC_1A_SUL_n78A- |  |  |  |  |  |  | 10.74 |  |
| n84A |  | n78 | 3710 | 10 | 25 | 3710 | N/A |  |
| DC_2A_n66A |  | 2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  |  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_2A_n66A |  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
|  |  | n66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
| DC_2A_n77A DC_2A-2A_n77A |  | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  |  |  |  |  |  |  | 28.74 |  |
|  |  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  |  | 2 | 1900 | 5 | 25 | 1980 | 8.0 | IMD4 |
|  |  |  |  |  |  |  | 10.74 |  |
|  |  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  |  | 2 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  |  |  |  |  |  |  | 7.74 |  |
|  |  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_2A_n78A |  | 2 | 1855 | 5 | 25 | 1935 | 26 | IMD23 |
|  |  |  |  |  |  |  | 28.74 |  |
|  |  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  |  | 2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD43 |
|  |  |  |  |  |  |  | 10.74 |  |
|  |  | n78 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_3_n1 |  | 3 | 1760 | 5 | 25 | 1855 | N/A | N/A |
|  |  | n1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  |  | 3 | 1771 | 10 | 50 | 1866 | 4 | IMD4 |
| DC_3_n5 |  | n5 | 838 | 5 | 25 | 883 | N/A | N/A |
|  |  | 3 | 1721 | 10 | 50 | 1816 | N/A | N/A |
|  |  | n5 | 838 | 5 | 25 | 883 | 24 | IMD23 |
| DC_3A_n7A |  | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_3C_n7A |  | n7 | 2535 | 10 | 50 | 2655 | 10.25 | IMD4 |
| DC_3A_n8A |  | n8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
|  |  | 3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  |  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  |  | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_3A_n41A |  | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
| DC_3C_n41A |  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | IMD4 |
| DC_3A_SUL_n41A-n80A, |  | 3 | 1740 | 5 | 25 | 1835 | 8.2 | IMD4 |
| DC_3C_SUL_n41A-n80A |  | n41 | 2657.5 | 10 | 52 | 2657.5 | N/A | N/A |
| DC_3A_n77A |  | 3 | 1740 | 5 | 25 | 1835 | 26 | IMD23 |
| DC_3A_SUL_n77A-n80A DC_3A_n78A |  |  |  |  |  |  | 28.74 |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  | n77, n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n77A |  | 3 | 1765 | 5 | 25 | 1860 | 8.0 | IMD43 |
| DC_3A_SUL_n77A-n80A, DC_3A_n78A |  |  |  |  |  |  | 10.74 |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  | n77, n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_5A_n66A |  | 5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  |  | n66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
| DC_5A_n78A |  | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  |  | n78 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_5A_n77A8 |  | 5 | 844 | 5 | 25 | 889 | 8.3 | IMD4 |
|  |  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
|  |  | 5 | 826.5 | 5 | 25 | 871.5 | 5.5 | IMD5 |
|  |  | n77 | 4177.5 | 10 | 50 | 4177.5 | N/A | N/A |
| DC_7_n3 |  | 7 | 2535 | 10 | 50 | 2655 | 13 | IMD4 |
|  |  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
| DC_7_n5 |  | 7 | 2547 | 10 | 50 | 2667 | N/A | N/A |
|  |  | n5 | 834 | 5 | 25 | 879 | 12 | IMD33 |
| DC_7A_n77A |  | 7 | 2540 | 5 | 25 | 2660 | 7.1 | IMD4 |
|  |  | n77 | 3870 | 10 | 50 | 3870 | N/A | N/A |
| DC_7A_n66A |  | 7 | 2535 | 10 | 50 | 2655 | 15 | 4th IMD |
| DC_7A-7A_n66A DC_7C_n66A |  | n66 | 1730 | 5 | 25 | 2130 | N/A | N/A |
| DC_8A_n1A |  | 8 | 887.5 | 5 | 25 | 932.5 | N/A | N/A |
|  |  | n1 | 1965 | 5 | 25 | 2155 | 6 | IMD4 |
|  |  | 8 | 900 | 5 | 25 | 945 | 8 | IMD43 |
| DC_8A_n3A |  | n3 | 1755 | 10 | 50 | 1850 | N/A | N/A |
|  |  | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  |  | n3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
| DC_8A_n20A | n20 |  | 849.5 | 5 | 25 | 808.5 | 25 | IMD33 |
|  | 8 |  | 890.5 | 5 | 25 | 935.5 | N/A | N/A |
|  | n20 |  | 847.5 | 5 | 25 | 806.5 | N/A | N/A |
|  | 8 |  | 892.5 | 5 | 25 | 937.5 | 25 | IMD33 |
| DC_8A_n41A |  | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
|  |  | n41 | 2685 | 10 | 50 | 2685 | N/A | N/A |
| DC_8A_SUL_n41A- |  | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | IMD33 |
| n81A |  | n41 | 2685 | 10 | 52 | 2685 | N/A | N/A |
| DC_8A_n77A DC_8A_n78A |  | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | IMD4 |
| DC_8A-n79C, DC_8A-SUL_n78A-n81A |  | n77, n78 | 3635 | 10 | 50 | 3635 | N/A | H4 |
| DC_12_n78 |  | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  |  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_13A_n77A |  | 13 | 784.5 | 5 | 20 | 753.5 | 5.5 | IMD5 |
|  |  | n77 | 3891.5 | 10 | 50 | 3891.5 | N/A | N/A |
|  |  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_20A_n3A |  | n3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  |  | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  |  | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
| DC_8A_n79A |  | 8 | 897.5 | 5 | 25 | 942.5 | 4.8 | IMD5 |
| DC_8A-SUL_n79A-n81A |  | n79 | 4532.5 | 40 | 216 | 4532.5 | N/A | N/A |
| DC_18A_n77A DC_18A_n78A |  | 18 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  |  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_12A_n77A |  | 12 | 702 | 5 | 20 | 732 | 5.5 | IMD5 |
|  |  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_19A_n78A |  | 19 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  |  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_20_n7 | 20 |  | 851 | 5 | 25 | 810 | 12 | IMD33 |
|  | n7 |  | 2512 | 10 | 50 | 2632 | N/A | N/A |
| DC_20A_n8A |  | 20 | 849.5 | 5 | 25 | 808.5 | 21 | IMD3 |
|  |  | n8 | 892.5 | 5 | 25 | 937.5 | 21 | IMD3 |
| DC_20A_n77A DC_20A_n78A, |  | 20 | 850 | 5 | 25 | 809 | 11 | IMD4 |
| DC_20A-SUL_n78A-n82A |  | n77, n78 | 3359 | 10 | 50 | 3359 | N/A | N/A |
| DC_20A_n77A |  | 20 | 840 | 5 | 25 | 799 | 6.5 | IMD54 |
|  |  | n77 | 4159 | 10 | 50 | 4159 | N/A | N/A |
| DC_21A_n28A7 |  | 21 | 1450.4 | 5 | 25 | 1498.4 | 2.5 | IMD5 |
|  |  | n28 | 735.5 | 5 | 25 | 790.5 | N/A | N/A |
| DC_21A_n79A |  | 21 | 1457.5 | 5 | 25 | 1505.5 | 18.4 | IMD3 |
|  |  | n79 | 4420.5 | 40 | 216 | 4420.5 | N/A | N/A |
| DC_26A_n41A |  | 26 | 839 | 5 | 25 | 884 | 15.6 | IMD3 |
|  |  | n41 | 2562 | 10 | 50 | 2562 | N/A | N/A |
| DC_28A_n51A |  | 28 | 742.3 | 5 | 25 | 797.3 | 5 | IMD 4 |
|  |  | n51 | 1429.5 | 5 | 25 | 1429.5 | N/A | N/A |
| DC_25A_n77A DC_25A-25A_n77A |  | 25 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  |  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  |  | 25 | 1900 | 5 | 25 | 1980 | 8 | IMD4 |
|  |  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
|  |  | 25 | 1885 | 5 | 25 | 1965 | 5 | IMD5 |
|  |  | n77 | 3810 | 10 | 50 | 3810 | N/A | N/A |
| DC_26A_n77A, |  | 26 | 836.5 | 5 | 25 | 881.5 | 11.1 | IMD4 |
| DC_26A_n78A |  | n77, n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |
| DC_28A_n77A, |  | 28 | 705.5 | 5 | 25 | 760.5 | 5.5 | IMD5 |
| DC_28A_n78A, DC_28A-SUL_n78A-n83A |  | n77, n78 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |
| DC_48A_n66A |  | 48 | 3630 | 20 | 100 | 3630 | N/A | N/A |
|  |  | n66 | 1715 | 5 | 25 | 2115 | 4 | IMD5 |
|  |  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
| DC_66A_n2A |  | n2 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
|  |  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  |  | n2 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n5A |  | n5 | 838 | 5 | 25 | 883 | 30 | IMD23 |
|  |  | 66 | 1721 | 5 | 25 | 2121 | N/A | N/A |
|  |  | 66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  |  | n25 | 1855 | 5 | 25 | 1935 | 20 | IMD3 |
| DC_66A_n25A |  | 66 | 1712.5 | 5 | 25 | 2112.5 | 23 | IMD3 |
|  |  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  |  | 66 | 1750 | 5 | 25 | 2150 | 4 | IMD5 |
|  |  | n25 | 1883.3 | 5 | 25 | 1963.3 | N/A | N/A |
| DC_66A_n71A |  | 66 | 1750 | 5 | 25 | 2150 | 5 | IMD4 |
|  |  | n71 | 675 | 5 | 25 | 629 | N/A | N/A |
| DC_66A_n77A DC_66-66_n77A DC_66-66-66_n77A |  | 66 | 1775 | 5 | 25 | 2175 | 31.0 | IMD2 |
|  |  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  |  | 66 | 1760 | 5 | 25 | 2160 | 5.0 | IMD5 |
|  |  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_66A_n78A |  | 66 | 1730 | 5 | 25 | 2130 | 5.0 | IMD5 |
|  |  | n78 | 3660 | 5 | 50 | 3660 | N/A | N/A |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>