3GPP TSG-RAN5 Meeting #98	R5-231685

# Athens, Greece, 27th Feburary – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1579 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to R17 Configuration for DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas ADT, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some EN-DC configurations are missing in the clause 5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added following Inter-band EN-DC including FR1 configurations as at least one test cases in clause 6 or 7 has covered the requirement: - DC_71A_n2A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EN-DC configuration could be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5B.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-231183 with correcting tdoc # in CR cover |  |  |  |  |  |  |  |  |

## {Start of changes}

<Unchanged Sections Skipped>

## 5.5B	Configuration for DC

<Unchanged Sections Skipped>

### 5.5B.1	General

The operating bands and bandwidth classes are specified for operation with EN-DC, NGEN-DC, NE-DC or NR-DC configured. The EN-DC, NE-DC or NGEN-DC band combinations include at least one E-UTRA operating band.

For EN-DC or NE-DC configurations indicated by column "Single Uplink allowed" (e.g., problematic band combinations as defined in TS 38.306 [16]) in tables in this clause the UE may indicate capability of not supporting simultaneous dual and triple uplink operation due to possible intermodulation interference to its own primary downlink channel bandwidth of PCell or PSCell if the intermodulation order is 2 or if the intermodulation order is 3 for the combinations when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz.

In the case for EN-DC or NE-DC configurations listed in tables in this clause for which the intermodulation products caused by the dual and triple uplink operation fall into the receive band but do not interfere with its own primary downlink channel bandwidth of PCell or PSCell as defined in Annex I the UE is mandated to operate in dual and triple uplink mode. Single Uplink is also allowed for certain band combinations where intermodulation or reverse intermodulation products could create difficulty for meeting emission requirements.

For EN-DC combinations of order 3 or higher, "Single Uplink allowed" UL configurations captured in Table 5.5B.2-1, Table 5.5B.3-1, and Table 5.5B.4-1 apply.

Non-contiguous resource allocation and almost contiguous allocation are not applicable for E-UTRA or NR carrier part of intra-band EN-DC configuration.

If multiple UL DC configurations are listed for multiple DL DC configurations, valid uplink configurations are such that uplink does not have more carriers than downlink.

Non-contiguous resource allocation and almost contiguous allocation are not applicable for E-UTRA or NR carrier part of intra-band EN-DC configuration.

If the mandatory simultaneous Rx/Tx capability applies for a lower order DC configuration, when the applicable lower order DC configuration is a band pair in a higher order DC configuration, the mandatory simultaneous Rx/Tx capability also applies for the band pair in the higher order DC configuration.

### 5.5B.2	Intra-band contiguous EN-DC

Supported channel bandwidths for E-UTRA operating bands are defined in TS 36.521-1 [10] and for NR operating bands in TS 38.521-1 [8].

Table 5.5B.2-1: Intra-band contiguous EN-DC configurations

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_(n)41AA5 DC_(n)41CA5 | DC_(n)41AA | Yes3 |
| DC_(n)41CA5 | DC_41A_n41A | Yes3 |
| DC_(n)71AA2 | DC_(n)71AA | No4 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Requirements in this specification apply for NR SCS of 15 kHz only. NOTE 3: Single UL allowed due to potential emission issues, not self-interference. NOTE 4: For UE(s) supporting dynamic power sharing it is mandatory to do dual simultaneous UL. For UE(s) not supporting dynamic power sharing single UL is allowed. NOTE 5: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. |  |  |

### 5.5B.3	Intra-band non-contiguous EN-DC

Supported channel bandwidths for E-UTRA operating bands are defined in TS 36.521-1 [10] and for NR operating bands in TS 38.521-1 [8].

Table 5.5B.3-1: Intra-band non-contiguous EN-DC configurations

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_41A_n41A3 DC_41C_n41A3 DC_41D_n41A3 | DC_41A_n41A | Yes4 |
| DC_66A_n66A | DC_66A_n66A5 | Yes5 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported in Rel-15 NOTE 3: The minimum requirements only apply for non-simultaneous Tx/Rx between all carriers. NOTE 4: Single UL allowed due to potential emission issues, not self-interference. NOTE 5: Only single switched UL is supported. NOTE 6: Reserve. NOTE 7: Single UL allowed due to potential emission issues and self-interference. |  |  |

### 5.5B.4	Inter-band EN-DC within FR1

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.521-1 [10] and for NR operating bands and CA configurations in TS 38.521-1 [8] and present document.

#### 5.5B.4.1	Inter-band EN-DC configurations within FR1 (two bands)

Table 5.5B.4.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_1A_n3A | DC_1A_n3A | DC_1_n3 |  |
| DC_1A_n5A | DC_1A_n5A | No |  |
| DC_1A_n7A | DC_1A_n7A | No |  |
| DC_1A_n28A | DC_1A_n28A | No |  |
| DC_1A_n77A7 | DC_1A_n77A | DC_1_n77 | No |
| DC_1A_n78A7 DC_1A_n78C7 | DC_1A_n78A | No | No |
| DC_1A-1A_n78A | DC_1A_n78A | No |  |
| DC_1A_n79A7 DC_1A_n79C7 | DC_1A_n79A | No | No |
| DC_2A_n5A | DC_2A_n5A | No |  |
| DC_2A_n41A DC_2C_n41A | DC_2A_n41A DC_2C_n41A | No |  |
| DC_2A_n66A | DC_2A_n66A | DC_2_n66 |  |
| DC_2A_n71A | DC_2A_n71A | No |  |
| DC_2A_n77A | DC_2A_n77A | DC_2_n77 |  |
| DC_2A_n78A | DC_2A_n78A | DC_2_n78 |  |
| DC_3A_n1A | DC_3A_n1A | DC_3_n1 |  |
| DC_3A_n5A | DC_3A_n5A | DC_3_n5 |  |
| DC_3A_n7A | DC_3A_n7A | No |  |
| DC_3A_n28A | DC_3A_n28A | No |  |
| DC_3A_n41A7 | DC_3A_n41A | DC_3_n41 | No |
| DC_3A_n77A7 | DC_3A_n77A | DC_3_n77 | No |
| DC_3A_n78A7 DC_3A_n78C7 DC_3C_n78A7 | DC_3A_n78A | DC_3_n78 | No |
| DC_3A_n79A7 DC_3A_n79C7 | DC_3A_n79A | No | No |
| DC_5A_n2A | DC_5A_n2A | No |  |
| DC_5A_n66A | DC_5A_n66A | DC_5_n66 |  |
| DC_5A_n77A | DC_5A_n77A | No |  |
| DC_5A_n78A7 DC_5A_n78C7 | DC_5A_n78A | No | No |
| DC_7A_n1A | DC_7A_n1A | No |  |
| DC_7A_n3A | DC_7A_n3A | No |  |
| DC_7A_n5A | DC_7A_n5A | DC_7_n5 |  |
| DC_7A_n28A | DC_7A_n28A | No |  |
| DC_7A_n66A DC_7C_n66A | DC_7A_n66A | No |  |
| DC_7A_n78A7 DC_7C_n78A7 | DC_7A_n78A | No |  |
| DC_7A-7A_n78A7, 21 | DC_7A_n78A21 | No |  |
| DC_8A_n1A | DC_8A_n1A | No |  |
| DC_8A_n3A | DC_8A_n3A | No |  |
| DC_8A_n20A | DC_8A_n20A | Yes |  |
| DC_8A_n41A7 | DC_8A_n41A | No | No |
| DC_8A_n77A7 | DC_8A_n77A | No | No |
| DC_8A_n78A7 | DC_8A_n78A | No | No |
| DC_11A_n77A7 | DC_11A_n77A | No | No |
| DC_11A_n78A7 | DC_11A_n78A | No | No |
| DC_11A_n79A7 | DC_11A_n79A | No |  |
| DC_12A_n5A | DC_12A_n5A | No |  |
| DC_12A_n66A | DC_12A_n66A | No |  |
| DC_12A_n78A | DC_12A_n78A | DC_12_n78 |  |
| DC_13A_n2A | DC_13A_n2A | No |  |
| DC_13A_n66A | DC_13A_n66A | No |  |
| DC_13A_n77A | DC_13A_n77A | No |  |
| DC_14A_n2A | DC_14A_n2A | No |  |
| DC_14A_n66A | DC_14A_n66A | No |  |
| DC_18A_n77A7 | DC_18A_n77A | No | No |
| DC_18A_n78A7 | DC_18A_n78A | No | No |
| DC_18A_n79A7 | DC_18A_n79A | No |  |
| DC_19A_n1A | DC_19A_n1A | No |  |
| DC_19A_n77A7 DC_19A_n77C7 | DC_19A_n77A | No |  |
| DC_19A_n78A7 DC_19A_n78C7 | DC_19A_n78A | No | No |
| DC_19A_n79A7 | DC_19A_n79A | No | No |
| DC_20A_n1A | DC_20A_n1A | No |  |
| DC_20A_n3A | DC_20A_n3A | No |  |
| DC_20A_n7A | DC_20A_n7A | DC_20_n7 |  |
| DC_20A_n8A | DC_20A_n8A | DC_20_n8 |  |
| DC_20A_n28A8,11,13 | DC_20A_n28A | No |  |
| DC_20A_n78A7 | DC_20A_n78A | No |  |
| DC_21A_n1A | DC_21A_n1A | No |  |
| DC_21A_n28A17 | DC_21A_n28A | DC_21_n28 |  |
| DC_21A_n77A7 | DC_21A_n77A | No |  |
| DC_21A_n78A7 DC_21A_n78C7 | DC_21A_n78A | No | No |
| DC_21A_n79A7 DC_21A_n79C7 | DC_21A_n79A | No | No |
| DC_25A_n41A | DC_25A_n41A | No |  |
| DC_26A_n41A | DC_26A_n41A | No |  |
| DC_26A_n77A7 | DC_26A_n77A | No |  |
| DC_26A_n78A7 | DC_26A_n78A | No |  |
| DC_26A_n79A7 | DC_26A_n79A | No |  |
| DC_28A_n3A | DC_28A_n3A | No |  |
| DC_28A_n5A8 | DC_28A_n5A | No |  |
| DC_28A_n7A | DC_28A_n7A | No |  |
| DC_28A_n77A7 | DC_28A_n77A | No | No |
| DC_28A_n78A7 DC_28A_n78C7 | DC_28A_n78A | No | No |
| DC_28A_n79A7 | DC_28A_n79A | No |  |
| DC_30A_n5A | DC_30A_n5A | No |  |
| DC_30A_n66A | DC_30A_n66A | No |  |
| DC_38A_n78A7 | DC_38A_n78A | No |  |
| DC_39A_n41A3 DC_39C_n41A3 | DC_39A_n41A DC_39C_n41A | No | No |
| DC_39A_n79A7 | DC_39A_n79A | No | No |
| DC_40A_n1A | DC_40A_n1A | No |  |
| DC_40A_n41A3 | DC_40A_n41A | No |  |
| DC_40A_n78A DC_40C_n78A | DC_40A_n78A DC_40C_n78A | No |  |
| DC_40A_n79A7,12 DC_40C_n79A7,12 | DC_40A_n79A | No | No |
| DC_41A_n77A DC_41C_n77A | DC_41A_n77A | No |  |
| DC_41A_n78A DC_41C_n78A | DC_41A_n78A | No |  |
| DC_41A_n79A6,7 | DC_41A_n79A | No | No |
| DC_42A_n77A3,4,9,11 DC_42C_n77A3,4,9,11 DC_42D_n77A3,4,9,11 | N/A | N/A |  |
| DC_42A_n78A3,4,9,11 DC_42A_n78C3,4,9,11 DC_42C_n78A3,4,9,11 DC_42C_n78C3,4,9,11 DC_42D_n78A3,4,9,11 DC_42E_n78A3,4,9,11 | N/A | N/A |  |
| DC_42A_n79A9,15 DC_42A_n79C9,15 DC_42C_n79A9,15 DC_42C_n79C9,15 DC_42D_n79A9,15 DC_42E_n79A9,15 | N/A | N/A |  |
| DC_48A_n5A | DC_48A_n5A | No |  |
| DC_48A_n66A | DC_48A_n66A | No |  |
| DC_66A_n2A | DC_66A_n2A | DC_66_n2 |  |
| DC_66A_n5A | DC_66A_n5A | DC_66_n5 |  |
| DC_66A_n41A | DC_66A_n41A | No |  |
| DC_66A_n71A | DC_66A_n71A | No |  |
| DC_66A_n78A | DC_66A_n78A | No |  |
| DC_71A_n2A | DC_71A_n2A | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured PCell. NOTE 3: The minimum requirements apply only when there is non-simultaneous Tx/Rx operation between E-UTRA and NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order EN-DC configuration. NOTE 4: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addition to intra-band non-contiguous EN-DC. The intra-band requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 5: The frequency range above 3600 MHz for Band n78 is not used in this combination. NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 8: The frequency range in band n28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. NOTE 9: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. NOTE 10: Void. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16, the power imbalance requirement defined in clause 7.6B.2.6 of 38.101-3 [4] apply. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 12: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [25] and clause 7.32.2.12 of 36.133 [12] respectively. NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. Same restrictions are applied to related higher order configurations. NOTE 16: Reserved. NOTE 17: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This restriction applies also for these band combinations when applicable EN-DC configuration is part of a higher order EN-DC configuration. Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1. NOTE 18: Reserved. NOTE 19: Reserved. NOTE 20: Reserved. NOTE 21: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |  |  |

#### 5.5B.4.2	Inter-band EN-DC configurations within FR1 (three bands)

Table 5.5B.4.2-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n28A | DC_1A_n28A DC_3A_n28A |
| DC_1A-3A_n77A5 | DC_1A_n77A DC_3A_n77A |
| DC_1A-3A_n78A5DC_1A-3C_n78A5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-1A-3A_n78A DC_1A-1A-3C_n78A | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3C_n78(2A)5 | DC_1A_n78A DC_3A_n78A DC_3C_n78A |
| DC_1A-3A_n79A5 | DC_1A_n79A DC_3A_n79A |
| DC_1A-5A_n78A5 DC_1A-5A_n78C5 | DC_1A_n78A DC_5A_n78A |
| DC_1A-1A-5A_n78A | DC_1A_n78A DC_5A_n78A |
| DC_1A-7A_n3A | DC_1A_n3A DC_7A_n3A |
| DC_1A-7A_n28A5 | DC_1A_n28A DC_7A_n28A |
| DC_1A-7A_n78A5 | DC_1A_n78A DC_7A_n78A |
| DC_1A-8A_n3A | DC_1A_n3A DC_8A_n3A |
| DC_1A-8A_n78A5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-8A_n78(2A)5 | DC_1A_n78A DC_8A_n78A |
| DC_1A-19A_n77A5 | DC_1A_n77A DC_19A_n77A |
| DC_1A-19A_n78A5 DC_1A-19A_n78C5 | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A_n79A5 DC_1A-19A_n79C5 | DC_1A_n79A DC_19A_n79A |
| DC_1A-20A_n3A | DC_1A_n3A DC_20A_n3A |
| DC_1A-20A_n8A | DC_1A_n8A DC_20A_n8A |
| DC_1A-20A_n28A6,11,12 | DC_1A_n28A DC_20A_n28A |
| DC_1A-20A_n78A5 | DC_1A_n78A DC_20A_n78A |
| DC_1A-21A_n77A5 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A_n78A5 DC_1A-21A_n78C5 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A_n79A5 DC_1A-21A_n79C5 | DC_1A_n79A DC_21A_n79A |
| DC_1A-28A_n3A | DC_1A_n3A DC_28A_n3A |
| DC_1A-28A_n5A6 | DC_1A_n5A DC_28A_n5A |
| DC_1A_n28A-n78A5 | DC_1A_n28A DC_1A_n78A |
| DC_1A-28A_n79A DC_1A-28A_n79C | DC_1A_n79A DC_28A_n79A |
| DC_1A-41A_n28A5 DC_1A-41C_n28A5 | DC_1A_n28A DC_41A_n28A DC_41C_n28A |
| DC_1A-41A_n77A | DC_1A_n77A DC_41A_n77A |
| DC_1A-41A_n78A | DC_1A_n78A DC_41A_n78A |
| DC_1A-42A_n77A10,11 DC_1A-42C_n77A10,11 DC_1A-42D_n77A10,11 | DC_1A_n77A |
| DC_1A-42A_n78A10,11 DC_1A-42A_n78C10,11 DC_1A-42C_n78A10,11 DC_1A-42C_n78C10,11 DC_1A-42D_n78A10,11 DC_1A-42E_n78A10,11 | DC_1A_n78A |
| DC_1A-42A_n79A DC_1A-42A_n79C DC_1A-42C_n79A DC_1A-42C_n79C DC_1A-42D_n79A DC_1A-42E_n79A | DC_1A_n79A |
| DC_1A_n77A-n79A | DC_1A_n77A DC_1A_n79A |
| DC_1A_n78A-n79A | DC_1A_n78A DC_1A_n79A |
| DC_2A-5A_n66A | DC_2A_n66A DC_5A_n66A |
| DC_2A-12A_n66A | DC_2A_n66A DC_12A_n66A |
| DC_2A-13A_n77A | DC_2A_n77A DC_13A_n77A |
| DC_2A-14A_n2A | DC_2A_n2A2 DC_14A_n2A |
| DC_2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-2A-14A_n66A | DC_2A_n66A DC_14A_n66A |
| DC_2A-30A_n66A | DC_2A_n66A DC_30A_n66A |
| DC_2A-66A_n5A | DC_2A_n5A DC_66A_n5A |
| DC_2A-66A_n41A | DC_2A_n41A DC_66A_n41A |
| DC_2A-66A_n71A | DC_2A_n71A DC_66A_n71A |
| DC_2A-66A_n77A | DC_2A_n77A DC_66A_n77A |
| DC_2A-(n)71AA | DC_2A_n71A DC_(n)71AA |
| DC_3A_n1A-n78A5 | DC_3A_n1A DC_3A_n78A |
| DC_3A_n1A-n79A5 | DC_3A_n1A DC_3A_n79A |
| DC_3A-5A_n78A5 DC_3A-5A_n78C5 | DC_3A_n78A DC_5A_n78A |
| DC_3A-7A_n5A | DC_3A_n5A DC_7A_n5A |
| DC_3A-7A_n28A | DC_3A_n28A DC_7A_n28A |
| DC_3A-7A_n78A5 DC_3C-7A_n78A5 | DC_3A_n78A DC_7A_n78A |
| DC_3A-8A_n1A | DC_3A_n1A DC_8A_n1A |
| DC_3A-8A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3A-8A_n78A5 | DC_3A_n78A DC_8A_n78A |
| DC_3A-18A_n77A | DC_3A_n77A DC_18A_n77A |
| DC_3A-8A_n78(2A) | DC_3A_n78A DC_8A_n78A |
| DC_3A-18A_n78A | DC_3A_n78A DC_18A_n78A |
| DC_3A-19A_n77A5 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A_n78A5 DC_3A-19A_n78C5 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A_n79A5 DC_3A-19A_n79C5 | DC_3A_n79A DC_19A_n79A |
| DC_3A-20A_n1A | DC_3A_n1A DC_20A_n1A |
| DC_3A-20A_n28A5,6,11,12 | DC_3A_n28A DC_20A_n28A |
| DC_3A-20A_n78A5 | DC_3A_n78A DC_20A_n78A |
| DC_3A-21A_n77A5 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A_n78A5 DC_3A-21A_n78C5 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A_n79A5 DC_3A-21A_n79C5 | DC_3A_n79A DC_21A_n79A |
| DC_3A-28A_n78A5 | DC_3A_n78A DC_28A_n78A |
| DC_3A_n28A-n78A5 | DC_3A_n28A DC_3A_n78A |
| DC_3A-40A_n1A | DC_3A_n1A DC_40A_n1A |
| DC_3A-41A_n28A5 DC_3A-41C_n28A5 | DC_3A_n28A DC_41A_n28A DC_41C_n28A |
| DC_3A-41A_n41A | DC_3A_n41A |
| DC_3A-41A_n77A DC_3A-41C_n77A | DC_3A_n77A DC_41A_n77A |
| DC_3A-41A_n77(2A) | DC_3A_n77A DC_41A_n77A |
| DC_3A-42A_n77A10,11 DC_3A-42C_n77A10,11 DC_3A-42D_n77A10,11 | DC_3A_n77A |
| DC_3A-42A_n78A10,11 DC_3A-42A_n78C10,11 DC_3A-42C_n78A10,11 DC_3A-42C_n78C10,11 DC_3A-42D_n78A10,11 DC_3A-42E_n78A10,11 | DC_3A_n78A |
| DC_3A-42A_n79A DC_3A-42A_n79C DC_3A-42C_n79A DC_3A-42C_n79C DC_3A-42D_n79A DC_3A-42E_n79A | DC_3A_n79A |
| DC_3A_n77A-n79A | DC_3A_n77A DC_3A_n79A |
| DC_3A_n78A-n79A | DC_3A_n78A DC_3A_n79A |
| DC_5A-7A_n78A | DC_5A_n78A DC_7A_n78A |
| DC_5A-30A_n66A | DC_5A_n66A DC_30A_n66A |
| DC_7A_n5A-n78A DC_7C_n5A-n78A | DC_7A_n5A DC_7C_n5A DC_7A_n78A DC_7C_n78A |
| DC_7A-8A_n1A | DC_7A_n1A DC_8A_n1A |
| DC_7A-8A_n3A | DC_7A_n3A DC_8A_n3A |
| DC_7A-20A_n1A | DC_7A_n1A DC_20A_n1A |
| DC_7A-20A_n3A | DC_7A_n3A DC_20A_n3A |
| DC_7A-20A_n8A | DC_7A_n8A DC_20A_n8A |
| DC_7A-20A_n28A6,11,12 | DC_7A_n28A DC_20A_n28A |
| DC_7A-20A_n78A5 | DC_7A_n78A DC_20A_n78A |
| DC_7A-28A_n3A | DC_7A_n3A DC_28A_n3A |
| DC_7A-28A_n5A6 | DC_7A_n5A DC_28A_n5A |
| DC_7A-28A_n78A5 | DC_7A_n78A DC_28A_n78A |
| DC_7A_n28A-n78A5 DC_7C_n28A-n78A | DC_7A_n28A DC_7A_n78A DC_7C_n28A DC_7C_n78A |
| DC_12A-30A_n66A | DC_12A_n66A DC_30A_n66A |
| DC_13A-66A_n77A | DC_13A_n77A DC_66A_n77A |
| DC_18A-41A_n3A | DC_18A_n3A DC_41A_n3A |
| DC_18A-41A_n77A DC_18A-41C_n77A | DC_18A_n77A DC_41A_n77A DC_41C_n77A |
| DC_18A-41A_n78A DC_18A-41C_n78A | DC_18A_n78A DC_41A_n78A DC_41C_n78A |
| DC_19A_n1A-n78A5 | DC_19A_n1A DC_19A_n78A |
| DC_19A_n1A-n79A5 | DC_19A_n1A DC_19A_n79A |
| DC_19A-21A_n77A5 | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A_n78A5 DC_19A-21A_n78C5 | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A_n79A5 DC_19A-21A_n79C5 | DC_19A_n79A DC_21A_n79A |
| DC_19A-42A_n77ADC_19A-42C_n77A | DC_19A_n77A |
| DC_19A-42A_n78A DC_19A-42C_n78A DC_19A-42A_n78C DC_19A-42C_n78C | DC_19A_n78A |
| DC_19A-42A_n79A DC_19A-42C_n79A DC_19A-42A_n79C DC_19A-42C_n79C | DC_19A_n79A |
| DC_19A_n77A-n79A | DC_19A_n77A DC_19A_n79A |
| DC_19A_n78A-n79A | DC_19A_n78A DC_19A_n79A |
| DC_20A_n28A-n78A5,6,11,12 | DC_20A_n28A DC_20A_n78A |
| DC_20A_n76A-n78A5 | DC_20A_n78A |
| DC_21A_n1A-n78A5 | DC_21A_n1A DC_21A_n78A |
| DC_21A_n1A-n79A5 | DC_21A_n1A DC_21A_n79A |
| DC_21A-42A_n77A10,11 DC_21A-42C_n77A10,11 | DC_21A_n77A |
| DC_21A-42A_n78A10,11 DC_21A-42A_n78C10,11 DC_21A-42C_n78A10,11 DC_21A-42C_n78C10,11 | DC_21A_n78A |
| DC_21A-42A_n79A DC_21A-42A_n79C DC_21A-42C_n79A DC_21A-42C_n79C | DC_21A_n79A |
| DC_21A_n77A-n79A | DC_21A_n77A DC_21A_n79A |
| DC_21A_n78A-n79A | DC_21A_n78A DC_21A_n79A |
| DC_28A_n7A-n78A | DC_28A_n7A DC_28A_n78A |
| DC_28A-42A_n78A10,11 | DC_28A_n78A |
| DC_41A-42A_n78A10,11 | DC_41A_n78A |
| DC_66A-(n)71AA | DC_66A_n71A DC_(n)71AA |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Only single switched UL is supported. NOTE 3: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band for Band 46 is paired with the uplink operating band (external E-UTRA band) of the carrier aggregation configuration that is supporting the configured PCell. NOTE 4: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier can be up to 140us and placed in SUL resources. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 7: Void. NOTE 8: Reserved. NOTE 9: Reserved. NOTE 10: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 12: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

#### 5.5B.4.3	Inter-band EN-DC configurations within FR1 (four bands)

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A_n28A | DC_1A_n28A DC_3A_n28A DC_7A_n28A |
| DC_1A-3A-7A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A |
| DC_1A-3A-8A_n78A2 | DC_1A_n78A DC_3A_n78A DC_8A_n78A |
| DC_1A-3A-19A_n77A2 | DC_1A_n77A DC_3A_n77A DC_19A_n77A |
| DC_1A-3A-19A_n78A2 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A_n79A2 | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n28A3,7,8 | DC_1A_n28A DC_3A_n28A DC_20A_n28A |
| DC_1A-3A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_20A_n78A |
| DC_1A-3A-21A_n77A2 | DC_1A_n77A DC_3A_n77A DC_21A_n77A |
| DC_1A-3A-21A_n78A2 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A_n79A2 | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-3A-28A_n78A2 | DC_1A_n78A DC_3A_n78A DC_28A_n78A |
| DC_1A-3A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A |
| DC_1A-3A-42A_n78A6,7 DC_1A-3A-42C_n78A6,7DC_1A-3A-42D_n78A6,7 | DC_1A_n78A DC_3A_n78A |
| DC_1A-3A-42A_n79A DC_1A-3A-42C_n79ADC_1A-3A-42D_n79A | DC_1A_n79A DC_3A_n79A |
| DC_1A-7A-20A_n28A3,7,8 | DC_1A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-7A-20A_n78A2 | DC_1A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-7A-28A_n78A | DC_1A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-7A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A |
| DC_1A-19A-21A_n77A | DC_1A_n77A DC_19A_n77A DC_21A_n77A |
| DC_1A-19A-21A_n78A2 DC_1A-19A-21A_n78C2 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A_n79A DC_1A-19A-21A_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_1A-19A-42A_n77A6,7 DC_1A-19A-42C_n77A6,7 | DC_19A_n77A |
| DC_1A-19A-42A_n78A DC_1A-19A-42A_n78C DC_1A-19A-42C_n78A DC_1A-19A-42C_n78C | DC_1A_n78A DC_19A_n78A |
| DC_1A-19A-42A_n79A DC_1A-19A-42A_n79C DC_1A-19A-42C_n79A DC_1A-19A-42C_n79C | DC_1A_n79A DC_19A_n79A |
| DC_1A-20A_n28A-n78A2,3,7,8 | DC_1A_n28A DC_1A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-21A-42A_n77A6,7 DC_1A-21A-42C_n77A6,7 | DC_1A_n77A DC_21A_n77A |
| DC_1A-21A-42A_n78A6,7 DC_1A-21A-42A_n78C6,7 DC_1A-21A-42C_n78A6,7 DC_1A-21A-42C_n78C6,7 | DC_1A_n78A DC_21A_n78A |
| DC_1A-21A-42A_n79A DC_1A-21A-42A_n79C DC_1A-21A-42C_n79A DC_1A-21A-42C_n79C | DC_1A_n79A DC_21A_n79A |
| DC_2A-7A-13A_n66A DC_2A-7C-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A-7A-13A_n66A | DC_2A_n66A DC_7A_n66A DC_13A_n66A |
| DC_2A-7A-66A_n66A DC_2A-7C-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7A-7A-66A_n66A | DC_2A_n66A DC_7A_n66A DC_66A_n66A4 |
| DC_2A-7C-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-7A-7A-66A_n78A | DC_2A_n78A DC_7A_n78A DC_66A_n78A |
| DC_2A-13A-66A_n77A | DC_2A_n77A DC_13A_n77A DC_66A_n77A |
| DC_2A-14A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A-66A_n2A | DC_2A_n2A4 DC_14A_n2A DC_66A_n2A |
| DC_2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-2A-14A-66A_n66A | DC_2A_n66A DC_14A_n66A DC_66A_n66A4 |
| DC_2A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_66A_n5A DC_66A_n77A |
| DC_2A-66A-(n)71AA | DC_2A_n71A DC_66A_n71A DC_(n)71AA |
| DC_3A-7A-20A_n8A | DC_3A_n8A DC_7A_n8A DC_20A_n8A |
| DC_3A-7A-20A_n28A3,7,8 | DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_3A-7A-20A_n78A2 | DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_3A-7A-28A_n78A2 | DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_3A-7A_n28A-n78A2 | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A |
| DC_3A-19A-21A_n78A2 DC_3A-19A-21A_n78C2 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A_n79A2 DC_3A-19A-21A_n79C2 | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-19A-42A_n77A6,7 DC_3A-19A-42C_n77A6,7 | DC_3A_n77A DC_19A_n77A |
| DC_3A-19A-42A_n78A6,7 DC_3A-19A-42A_n78C6,7 DC_3A-19A-42C_n78A6,7 DC_3A-19A-42C_n78C6,7 | DC_3A_n78A DC_19A_n78A |
| DC_3A-19A-42A_n79A2 DC_3A-19A-42A_n79C2 DC_3A-19A-42C_n79A2 DC_3A-19A-42C_n79C2 | DC_3A_n79A DC_19A_n79A |
| DC_3A-20A_n28A-n78A2,3,7,8 | DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-21A-42A_n77A6,7 DC_3A-21A-42C_n77A6,7 | DC_3A_n77A DC_21A_n77A |
| DC_3A-21A-42A_n78A6,7 DC_3A-21A-42A_n78C6,7 DC_3A-21A-42C_n78A6,7 DC_3A-21A-42C_n78C6,7 | DC_3A_n78A DC_21A_n78A |
| DC_3A-21A-42A_n79A DC_3A-21A-42A_n79C DC_3A-21A-42C_n79A DC_3A-21A-42C_n79C | DC_3A_n79A DC_21A_n79A |
| DC_7A-20A_n28A-n78A2,3,7,8 | DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_13A-66A_n2A-n77A | DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A |
| DC_19A-21A-42A_n77A6,7 DC_19A-21A-42C_n77A6,7 | DC_19A_n77A DC_21A_n77A |
| DC_19A-21A-42A_n78A6,7 DC_19A-21A-42A_n78C6,7 DC_19A-21A-42C_n78A6,7 DC_19A-21A-42C_n78C6,7 | DC_19A_n78A DC_21A_n78A |
| DC_19A-21A-42A_n79A DC_19A-21A-42A_n79C DC_19A-21A-42C_n79A DC_19A-21A-42C_n79C | DC_19A_n79A DC_21A_n79A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Only single switched UL is supported. NOTE 5: UL carrier shall be supported in Band 2 or band 66 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 8: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

#### 5.5B.4.4	Inter-band EN-DC configurations within FR1 (five bands)

Table 5.5B.4.4-1: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A-20A_n28A3,6,7 | DC_1A_n28A DC_3A_n28A DC_7A_n28A DC_20A_n28A |
| DC_1A-3A-7A-20A_n78A2 | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_20A_n78A |
| DC_1A-3A-7A_n28A-n78A2 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A |
| DC_1A-3A-7A-28A_n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A |
| DC_1A-3A-19A-42A_n78A5,6 DC_1A-3A-19A-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_19A_n78A |
| DC_1A-3A-19A-42A_n79A DC_1A-3A-19A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_19A_n79A |
| DC_1A-3A-20A_n28A-n78A2,3,6,7 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-3A-21A-42A_n78A5,6 DC_1A-3A-21A-42C_n78A5,6 | DC_1A_n78A DC_3A_n78A DC_21A_n78A |
| DC_1A-3A-21A-42A_n79A DC_1A-3A-21A-42C_n79A | DC_1A_n79A DC_3A_n79A DC_21A_n79A |
| DC_1A-7A-20A_n28A-n78A2,3,6,7 | DC_1A_n28A DC_1A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_1A-19A-21A-42A_n78A5,6 DC_1A-19A-21A-42A_n78C5,6 DC_1A-19A-21A-42C_n78A5,6 DC_1A-19A-21A-42C_n78C5,6 | DC_1A_n78A DC_19A_n78A DC_21A_n78A |
| DC_1A-19A-21A-42A_n79A DC_1A-19A-21A-42A_n79C DC_1A-19A-21A-42C_n79A DC_1A-19A-21A-42C_n79C | DC_1A_n79A DC_19A_n79A DC_21A_n79A |
| DC_3A-7A-20A_n28A-n78A2,3,6,7 | DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| DC_3A-19A-21A-42A_n78A5,6 DC_3A-19A-21A-42C_n78A5,6 | DC_3A_n78A DC_19A_n78A DC_21A_n78A |
| DC_3A-19A-21A-42A_n79A5,6 DC_3A-19A-21A-42C_n79A5,6 | DC_3A_n79A DC_19A_n79A DC_21A_n79A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 4: Reserved. NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

#### 5.5B.4.5	Inter-band EN-DC configurations within FR1 (six bands)

Table 5.5B.4.5-1: Inter-band EN-DC configurations within FR1 (six bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A-20A_n28A-n78A2,3,5,6 | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_7A_n28A DC_7A_n78A DC_20A_n28A DC_20A_n78A |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 3: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 4: Reserved NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec between overlapping or partially overlapping DL bands contained in different cell groups. |  |

## {End of changes}