3GPP TSG-RAN5 Meeting #98	R5-231507

Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0518 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to clause A.21 Activation and deactivation of Supplementary Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on discussion paper R5-231176, inclusion of DNS server IP address in PDU session establishment Accept message is needed for supplementary servies test cases involving communication between UE and XCAP server. But when run on a device with multi PDU configuration, these test cases may fail due to unexpected service request procedures from UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Note in Clause A.21. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity will remain when test case failure is observed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, A.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-231507 is the final version of R5-231189 with below changes: 1. Updated References to include TS 36.523-3. 2. Changed wording in clause A.21 |  |  |  |  |  |  |  |  |

<Start of modified section>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[3]	3GPP TS 34.229-2: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[4]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[5]	3GPP TS 34.229-4: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 4: Enabler for IP multimedia applications testing".

[6]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[7]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[8]	ETSI ES 201 873-1: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language".

[9]	ETSI ES 201 873-2: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 2: TTCN-3 Tabular Presentation Format (TFT)".

[10]	ETSI TR 201 873-3: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 3: TTCN-3 Graphical Presentation Format (GFT)".

[11]	ETSI ETS 300 406: "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

[12]	ISO/IEC 9646-1: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[13]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[14]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3".

[15]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[16]	3GPP TS 33.203: "3G security;Access security for IP-based services".

[17]	IETF RFC 3329: "Security Mechanism Agreement for the Session Initiation Protocol (SIP)".

[18]	IETF RFC 3680: "A Session Initiation Protocol (SIP) Event Package for Registrations".

[19]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[20]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[21]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[22]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[23]	IETF RFC 2617: "HTTP Authentication: Basic and Digest Access Authentication".

[24]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[25]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[26]	3GPP TS 24.237: "IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) Service Continuity".

[27]	3GPP TS 23.003: "Numbering, addressing and identification".

[28]	IETF RFC 6665: “SIP-Specific Event Notification”.

[29]	IETF RFC 3312: “Integration of Resource Management and SIP”.

[30]	IETF RFC 3262: “Reliability of Provisional Responses in the Session Initiation Protocol (SIP)”.

[31]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

[32]	3GPP TS 24.610: "Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem".

[33]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[34]	3GPP TS 24.606: "Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem".

[35]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem".

[36]	3GPP TS 24.629: "Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem".

[37]	IETF RFC 4028: "Session Timers in the Session Initiation Protocol (SIP)".

[38]	IETF RFC 4566: "SDP: Session Description Protocol".

[39]	IETF RFC 7462: "URNs for the Alert-Info Header Field of the Session Initiation Protocol (SIP) ".

[40]	IETF RFC 3891: "The Session Initiation Protocol (SIP) "Replaces" Header".

[41]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[42]	IETF RFC 3326: "The Reason Header Field for the Session Initiation Protocol (SIP)".

[43]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[44]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[45]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[46]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[47]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[48]	3GPP TS 38.509: "5GS; Special conformance testing functions for User Equipment (UE)".

[49]	IETF RFC 6228: "Session Initiation Protocol (SIP) Response Code for Indication of Terminated Dialog".

[50]	IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[51]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Networks".

[52]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common
Implementation Conformance Statement (ICS) proforma".

[53]	3GPP TS 36.523-3: "LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Test suites”

<End of modified section>

<Start of modified section>

# A.21	Activation and deactivation of Supplementary Services / 5GS

Generic test procedure for signalling between UE and XCAP server to activate or deactivate a supplementary service.

Test procedure:

0a)	Pre-configurations:

-	The UE is in state 3N-A, registered to IMS on an IMS PDU session and at least one PDU session for Internet active according to TS 38.508-1 [21], clause 4.5.3.

NOTE:	All XCAP signalling shall occur on the Internet PDU session according to NG.114 [31], Annex C.3. It is recommended that px_MobileDataOn (TS 36.523-3 [53]) is set to false when executing test cases using this generic test procedure.

0b)	At the SS an HTTP server is established at port 80 to simulate the XCAP server.

NOTE:	TLS is not a test requirement i.e. the UE uses port 80 to access XCAP and BSF servers and SS does not redirect the UE to use HTTPS (port 443).

1)	Activation of the specific Supplementary Service is triggered at the UE with appropriate MMI command.

2)	The UE sends an initial HTTP request to the SS.

3)	In case of HTTP Digest XCAP authentication when the UE does not provide correct authorization credentials within its initial request:

3a)	the SS shall challenge the UE by sending a “401 Unauthorized” response to it. 
When the UE supports GBA for XCAP authentication and GBA shall be used according to test requirements or test configuration, the SS shall indicate bootstrapped security association is required as specified in TS 24.109 [43] clause 5.2.4 and the generic procedure A.22 shall be applied.

3b)	the UE repeats the HTTP request including a valid digest response in the authorization header.
The SS shall check the digest response taking into account the user’s prearranged password for (pure) HTTP digest authentication, or, for GBA,  being derived from the key material (Ks) using key derivation function as specified in 3GPP TS 33.220 [44].

4)	The SS sends a 200 (OK) response

5)	Optionally UE and SS exchange a sequence of additional HTTP requests and responses. In this sequence the UE may query the contents of the simservs document or selected parts of it. 
In general the HTTP requests are responded with a 200 “Ok” response but in case of a GET request to a non-existing node the SS shall respond with a 404 “File Not Found”.

6)	The simservs document is checked according to specific test requirements.

7)	Deactivation of supplementary service is triggered at the UE with appropriate MMI command.

8)	UE and SS exchange a sequence of HTTP requests and responses. In this sequence the UE may query the contents of the simservs document or selected parts of it.
In general the HTTP requests are responded with a 200 “Ok” response but in case of a GET request to a non-existing node the SS shall respond with a 404 “File Not Found”.

9)	The simservs document is checked according to specific test requirements.

Expected sequence:

| Step | Direction |  | Message/Procedure | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | Make the UE attempt activation of supplementary service |  |
| 2 |  |  | Initial HTTP Request | NOTE 1 |
| 3 |  |  | EXCEPTION: steps 3a and 3b describe behaviour in case of HTTP Digest XCAP authentication when the UE does not provide correct authorization credentials within its initial request |  |
| 3a |  |  | HTTP Response: “401 Unauthorized” |  |
|  |  |  | EXCEPTION: By default, when the UE supports GBA for XCAP authentication, GBA shall be used according to the generic test procedure A.22. NOTE: See TS 34.229-2 [3] for cases where the default does not apply. | (optional) GBA authentication at BSF server |
| 3b |  |  | HTTP Request with valid authorization credentials | The SS checks the digest response |
| 4 |  |  | HTTP Response: “200 OK” |  |
| 5 |  |  | EXCEPTION: steps 5a and 5b describe further optional message exchange between the UE and the SS; steps 5a and steps 5b can be repeated several times this exchange of information is considered to be finished when there is no further HTTP request sent by the UE within 20 seconds after the previous request |  |
| 5a |  |  | HTTP Request | NOTE 1 |
| 5b |  |  | HTTP Response: “200 OK” or “404 File Not Found” | NOTE 3 |
| 6 |  |  | Check: Does the simservs document stored in the SS contain the information supplied by the UE as required by the test requirements of the specific test case? | This is done by fetching the whole simservs document from the XCAP server and checking its content against the respective XML file (according to the XSD definitions for the respective supplementary service) |
| 7 |  |  | Make the UE attempt deactivation of supplementary service |  |
| 8 |  |  | EXCEPTION: steps 8a and 8b describe the mandatory message exchange between the UE and the SS which can be repeated several times; this exchange of information is considered to be finished when there is no further HTTP request sent by the UE within 20 seconds after the previous request |  |
| 8a |  |  | HTTP Request | NOTE 1 |
| 8b |  |  | HTTP Response: “200 OK” or “404 File Not Found” | NOTE 3 |
| 9 |  |  | Check: Does the simservs document stored in the SS contain the information supplied by the UE as required by the test requirements of the specific test case? | This is done by fetching the whole simservs document from the XCAP server and checking its content against the respective XML file (according to the XSD definitions for the respective supplementary service) |
| NOTE 1: The HTTP requests sent by the UE are processed by an XCAP server implementation at the SS to modify or query the contents of the simservs document. NOTE 2: Void. NOTE 3: “404 File Not Found” is sent as response for a GET request to a non-existing node |  |  |  |  |

Specific Message Contents

HTTP Requests sent by the UE (step 2, 3b, 5a, 8a)

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 2616 [46] |
| Method |  | GET, PUT, DELETE |  |  |
| Request-URI |  | XCAP URI referring to the simservs document as specified in RFC 4825 [45]; the document selector of such XCAP URI consists of - Configured XCAP root URI - simservs.ngn.etsi.org - users - same public user id as the default public user identity received in P-Associated-URI header in 200 OK for REGISTER (NOTE 4). - simservs.xml (in this order, separated by a slash); According to RFC 4825 [45] the node selector of the XCAP URI shall identify a valid part of a simservs document or whole document itself (NOTE 2). |  |  |
| Version |  | HTTP 1.1 |  |  |
| User-Agent | A1 |  |  | RFC 2616 [46] |
| Product token |  | 3gpp-gba |  | TS 24.109 [43] |
| Authorization |  | present in case of HTTP Digest XCAP authentication in the initial request or in the request following the “401 Unauthorized” response Digest |  | RFC 2617 [16] RFC 3310 [17] |
| username | NOT A2 | As configured in the UE (NOTE 5). Same public user id as the default public user identity received in P-Associated-URI header in 200 OK for REGISTER (NOTE 6). |  |  |
| username | A2 | B-TID as obtained from GBA authentication |  |  |
| realm |  | same value as received in the realm directive in the WWW Authenticate header sent by SS |  |  |
| nonce |  | same value as in WWW-Authenticate header sent by SS |  |  |
| opaque |  | same value as sent by the SS in “401 Unauthorized” |  |  |
| digest-uri |  | same URI as used in Request-URI |  |  |
| qop-value |  | auth |  |  |
| cnonce-value |  | value assigned by UE affecting the response calculation |  |  |
| nonce-count |  | 1 |  |  |
| response | NOT A2 | response calculated by UE using prearranged password |  |  |
| response | A2 | response calculated by UE using password derived from the key material of the GBA authentication according to Generic key derivation as specified in Annex B.3 of TS 33.220 [44] using static string “gba-me” as parameter P0 in Annex B.3. |  |  |
| algorithm |  | MD5 |  |  |
| Content-Type |  | present for HTTP PUT method |  | RFC 2616 [46] |
| media-type |  | application/vnd.etsi.simservs+xml or application/xcap-el+xml or application/xcap-att+xml (NOTE 3) |  |  |
| Message-body |  | present for HTTP PUT method: XML fragment of given node |  | RFC 2616 [46] RFC 4825 [45] |
| NOTE 1: Any other headers are ignored. NOTE 2: The SS shall check and make sure that the syntax of the node selector expressions is in compliance to clause 6.2 of RFC 4825 [45]. NOTE 3: the media-type depends on the kind of node being accessed by the Request-URI: document, element or attribute (see RFC 4825 [45]). NOTE 4: According A.12/38 3GPP TS 34.229-2 [3]. NOTE 5: Shall be present if A.12/37 3GPP TS 34.229-2 [3] is yes. NOTE 6: Shall be present if A.12/37 3GPP TS 34.229-2 [3] is no. |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | UE supports GBA authentication |
| A2 | GBA authentication shall be applied (according to test requirements or test configuration) |

HTTP Responses (step 4, 5b, 8b2) – normal case

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 2616 [46] |
| Version |  | HTTP 1.1 |  |  |
| Code |  | 200 |  |  |
| Reason |  | OK |  |  |
| Server |  |  |  | RFC 2616 [46] |
| product |  | XCAP-Server |  |  |
| Date |  |  |  | RFC 2616 [46] |
| HTTP-date |  | valid date according to RFC 2616 [46] section 3.3.1 |  |  |
| ETag |  |  |  | RFC 2616 [46] |
| entity-tag |  | hextring: value starting with "478fb2358f700" and incremented after each PUT operation |  |  |
| Content-Type |  | present for HTTP GET method |  | RFC 2616 [46] |
| media-type |  | application/vnd.etsi.simservs+xml or application/xcap-el+xml or application/xcap-att+xml (NOTE 1) |  |  |
| Content-Length |  |  |  | RFC 2616 [46] |
| value |  | length of the message body |  |  |
| Message-body |  | present for GET method: XML fragment of given node |  | RFC 2616 [46] RFC 4825 [45] |
| NOTE 1: the media-type depends on the kind of node being accessed with the HTTP GET method: document, element or attribute (see RFC 4825 [45]). |  |  |  |  |

HTTP Responses (step 5b, 8b) – Response for GET request to a non-existing node

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 2616 [46] |
| Version |  | HTTP 1.1 |  |  |
| Code |  | 404 |  |  |
| Reason |  | File Not Found |  |  |
| Server |  |  |  | RFC 2616 [46] |
| product |  | XCAP-Server |  |  |
| Date |  |  |  | RFC 2616 [46] |
| HTTP-date |  | valid date according to RFC 2616 [46] section 3.3.1 |  |  |

HTTP Response (step 3a) for HTTP Digest XCAP authentication

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 2616 [46] |
| Version |  | HTTP 1.1 |  |  |
| Code |  | 401 |  |  |
| Reason |  | Unauthorized |  |  |
| Server |  |  |  | RFC 2616 [46] |
| product |  | XCAP-Server |  |  |
| Date |  |  |  | RFC 2616 [46] |
| HTTP-date |  | valid date according to RFC 2616 [46] section 3.3.1 |  |  |
| WWW-Authenticate |  |  |  | RFC 2616 [46] |
| realm | NOT A1 | home domain name as stored in EFDOMAIN or home domain name derived from the IMSI |  |  |
| realm | A1 | containing two parts delimited by "@" (see TS 24.109 [43] clause 5): - 3GPP-bootstrapping - home domain name as stored in EFDOMAIN or home domain name derived from the IMSI |  |  |
| algorithm |  | MD5 |  |  |
| qop-value |  | auth |  |  |
| nonce |  | Base 64 encoding of RAND and AUTN |  |  |
| opaque |  | arbitrary value (to be returned by the UE in subsequent request) |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | UE supports GBA authentication and GBA authentication shall be applied (according to test requirements or test configuration) |

<End of modified section>